2705 West Lake Drive

Taylor, Texas 76574

(512) 248-6800

HTTP://WWW.ERCOT.COM

COMS

Details on the Settlement of Combined Cycle Plants

V2.0

March 13, 2008

Revision History

| Revision | Comments | Date | Author |
| --- | --- | --- | --- |
| V0.92 | Initial version for Market Participant comment | 10-24-07 | Kenneth Ragsdale |
| V0.93 | Incorporated comments from Market Participants | 11-19-07 | Kenneth Ragsdale |
| V0.94 | Incorporated additional comments from Market Participants | 12-13-07 | Kenneth Ragsdale |
| V0.95 | Updated to reflect TPTF discussion on 12/18/07 | 12-18-07 |  |
| v1.0 | Incremented to v 1.0 to reflect TPTF approval from 12-18-07 | 1-02-08 |  |
| V1.1 | Updated to reflect changes to Clawback factors determination | 2-19-08 |  |
| V1.2 | Updated to reflect changes proposed at 2-21-08 TPTF meeting (Clawback and Operation at a configuration different than the DAM award) | 2-22-08 |  |
| V1.3 | Updated to incorporate TPTF feedback at 3-5-08 TPTF meeting | 3-05-08 |  |
| v2.0 | Incremented to reflect TPTF approval at 3-5-08 TPTF meeting. | 3-13-08 |  |

Table of Contents

1.	CCP Configurations	5

2.	Registration of CC’s in Siebel and NMMS	6

3.	Settlement Point Issues	7

4.	Transition Matrix	7

5.	Startup Offers	8

6.	Minimum Energy Offers	12

7.	Energy Offers	12

8.	COP	12

9.	High Level Settlement Issues	12

10.	Average Incremental Energy Costs	13

11.	Eligibility	13

12.	Eligibility for DAM Settlement	15

13.	RUC Settlement	16

14.	Minimum and Maximum Resource Prices to be used in CRR Hedge Value Calculations	17

15.	Dual Grid Combined Cycle Plants	17

Details on the Settlement of Combined Cycle Plants

This paper covers the details on the settlement of Combined Cycle Plants (CCP).  A CCP cannot be a Split Generation Resource (SGR).  The details on the settlement of SGR’s are covered in another paper.  For most of the issues, several different solutions were considered, however this paper deliberately does not review the options discussed ---- but instead is focused on presenting the final approach and the details of the approach that was selected.

# CCP Configurations

Within MMS each CCP configuration is specified as a separate logical generating unit. The Network Constrained Unit Commitment (NCUC) optimization considers these logical resources according to possible transitions between specified CCP configurations.

For example, a Combined Cycle Plant (CCP) consists of three Gas Turbines (GT) and one Steam Turbine (ST) and the following four configurations are registered:

- 1 GT only

- 2 GTs without ST

- 2 GTs with ST

- 3 GTs with ST.

It is assumed that at lest two GTs must be online for some time before ST starts.

As such, the CCP is registered as a set of four configurations:

| CCP Registration |  | CCP Configurations |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D |
| Physical Units | GT1 |  |   |   |    |
|  | GT2 |  |  |  |  |
|  | GT3 |  |  |  |  |
|  | ST |  |  |  |  |
| Configuration Type |  | 1GT | 2GTs | 2GTs + ST | 3GTs + ST |

A. 	Any 1 GT (single cycling)

B. 	Any 2 GTs (single cycling)

C. 	Any 2 GTs + ST (combined cycling)

D. 	All 3 GTs + ST (combined cycling).

Each CCP configuration is treated as a generating resource within MMS. Each configuration has its own set of operating parameters, min/max up/down time limits, and Three-Part Supply Offers to be optimized within MMS.

The configurations for the train are provided to ERCOT by the Resource Entity through the RARF.

# Registration of CC’s in Siebel and NMMS

The following naming convention shall be used.

CC Configuration Id = 14 characters

Generation Site Code- 8 characters (station/site code name already used in zonal)

Combined Cycle Train- 4 characters (_CC# to differentiate between multiple trains at a single site)

Configuration- 2 characters (_# to specify the number of the unique configuration)

Example: Lost Pines Combined Cycle Train, 1 site, 3 units, 3 configurations submitted by the Resource Entity

GenSite Id =      LOSTPI (already in ERCOT system)

Train Id =           _CC1     (only one train onsite, composed of 3 units)

Config/Mode =   _1

In this example,3 configurations are registered:

LOSTPI_CC1_1

LOSTPI_CC1_2

LOSTPI_CC1_3

And each unit keeps it’s zonal unit code for nodal:

LOSTPI_LOSTPGT1

LOSTPI_LOSTPGT2

LOSTPI_LOSTPST1

The following will be in Siebel:

LOSTPI_CC1

LOSTPI_LOSTPGT1

LOSTPI_LOSTPGT2

LOSTPI_LOSTPST1

The following will be in NMMS:

LOSTPI_CC1

LOSTPI_CC1_1

LOSTPI_CC1_2

LOSTPI_CC1_3

LOSTPI_LOSTPGT1

LOSTPI_LOSTPGT2

LOSTPI_LOSTPST1

The following will be in S&B (including Data Agg):

LOSTPI_CC1

LOSTPI_CC1_1

LOSTPI_CC1_2

LOSTPI_CC1_3

LOSTPI_LOSTPGT1

LOSTPI_LOSTPGT2

LOSTPI_LOSTPST1

S&B needs each configuration in L* in order to provide verifiable costs for each configuration.

Data Aggregation only needs to provide RTMG at the plant level.

Each CCP will have a “Logical Resource Node” for the plant.  This Logical Resource Node for the plant is where the CCP makes its offers and CRR’s are bought and sold.

NMMS will be the source system for the Logical Resource Node for the CCP.

# Settlement Point Issues

Each CCP will have a “Logical Resource Node”.  This “Logical Resource Node” represents the aggregated Resource Nodes of the physical generating units that compose the CCP.  LMP will be calculated for the CCP at it “Logical Resource Node” using aggregate shift factor.  This aggregate LMP should be used for CCP settlements.

NMMS, MMS and S&B need to work out the details of the modeling and implementation.

# Transition Matrix

The transitions between configurations can be illustrated as follows:

Startup

GT1

Conf C

2GTs

+ST

Conf D

3GTs

+ST

Conf B

2GTs

Conf A

1GT

All

OFF

Startup

GT2

Startup

GT3+ST

Startup

GT1&2

Startup

ST

Startup

GT3

Represented configurations and allowed transitions can be specified in a matrix form:

| TRANSITION MATRIX |  | Transition To Configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | OFF | A | B | C | D |
| Transition From Configuration | OFF |  |  |  |  |  |
|  | A |  |  |  |  |  |
|  | B |  |  |  |  |  |
|  | C |  |  |  |  |  |
|  | D |  |  |  |  |  |

 - Upward transition

 - Downward transition

The transition matrix is a part of Registration/Master File data.

# Startup Offers

The Startup Offers are submitted for each CCP configuration. The configuration startup offers represent the sum of startup costs for all physical units that are online in that configuration.

Startup Offers are functions of hot/intermediate/cold state of whole CCP. When the last unit within CCP is shut down the CCP cooling time counter is started. When the first unit is started then cooling time for whole CCP is stopped. The warmth state of whole CCP at that point of time is used to determine Startup Offers for all configurations. The transition costs are determined using the Startup Offers correspondent to warmth state of whole CCP.

The transition costs are defined and optimized as a function of hot/intermediate/cold states. The warmth state is determined by hours at which whole CCP was off-line.

The transition costs for upward transitions are calculated as the difference between startup costs of after and before configurations, i.e.:

Transition Costs (beforeafter) =

MAX {0; Startup Costs (after) – Startup Costs (before)}

The transition costs for configurations that CCP can start are equal to startup costs for these configurations.

MMS will provide to S&B the warmth state (hot, intermediate or cold) for after CCP configuration for each start and each transition. The above formula will be used by S&B to calculate startup costs.

For example, let the Startup Offers for the CPP configurations equal as follows:

| Startup Offers |  |  |  |
| --- | --- | --- | --- |
| A | B | C | D |
| $1000 - hot $1100 - int $1200 - cold | $2000 - hot $2200 - int $2400 - cold | $3500 – hot $4000 – int $4500 – cold | $4600 – hot $5100 – int $5700 - cold |

Note that hot/intermediate/cold states refer to warmth state of whole CCP, i.e. they are the same for all CCP configurations.

Let hot-to-intermediate time for CCP equal 2 hours and intermediate-to-cold time equal 5 hours. If the whole CCP was offline for 3 hours then CCP is in intermediate state. Then the intermediate state is selected for all CCP configurations.

Therefore, the transitions from one configuration to another are optimized using only Startup Offer values correspondent to the intermediate state:

| Optimized Startup Offers |  |  |  |
| --- | --- | --- | --- |
| A | B | C | D |
| $1100 – int | $2200 - int | $4000 – int | $5100 – int |

In this case, upward transition costs are constant values and can be calculated as follows:

Transition OFFA =

Startup Offer for A - Startup Offer Costs for OFF =

$1000 – $0 = $1000 = Startup costs of GT1

Transition OFFB =

Startup Offer for B - Startup Offer Costs for OFF =

$2200 – $0 = $2200 = Startup Costs of GT1&GT2

Transition AB =

Startup Offer for B - Startup Offer Costs for A =

$2200 – $1100 = $1100 = Startup Costs of GT2

Transition BC =

Startup Offer for C - Startup Offer for B =

$4000 – $2200 = $1800 = Startup Costs of ST

Transition BD =

Startup Offer for D - Startup Offer for B =

$5100 - $2200 = $2900 = Startup Costs of GT3 + ST

Transition CD =

Startup Offer for D - Startup Offer for C =

$5100 - $4000 = $1100 = Startup Costs of GT3.

| Transition Cost Functions |  | Transition To Configuration |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D |
| Transition From Configuration | OFF | $1100 GT1int | $1100 GT1int + $1100 GT2int |  |  |
|  | A |  | $1100 GT2int |  |  |
|  | B |  |  | $1800 STint | $1100 GT3int + $1800 Stint |
|  | C |  |  |  | $1100 GT3int |
|  | D |  |  |  |  |

The downward transition costs are equal to zero in all cases.

MMS will provide to S&B the start type (warmth state) for each start and each transition.

Configuration 1 can be made up of different GT’s but still be configuration 1.  The COP for configuration 1 should show the correct HSL and LSL.

Generic costs will be used for CCP’s with no approved verifiable costs on file.   Since there are only two Generic start types for Combined Cycle Plants represented in the Nodal Protocols (Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps) the following process will be used.  Resource Parameter data is submitted through the RARF for three types of starts (hot, intermediate and cold).  Start types of hot and intermediate (as determined by MMS) will use the generic cost for the entry in the Nodal Protocols for “less than 5 hours offline”.  A cold start type will use the entry for “5 + hours offline”.

The following protocols changes (shown in red) are envisioned.

Startup Cost

All costs incurred by a Generation Resource in starting up and reaching breaker close in dollars per start.  For a Combined Cycle Configuration with a steam unit, the Startup Cost is all costs incurred in starting up and reaching LSL of the configuration, excluding fuel associated with the metered generation after breaker close and before LSL.

Startup Offer

Represents an offer for all costs incurred by a Generation Resource in starting up and reaching breaker close in dollars per start.  For a Combined Cycle Configuration with a steam unit, the Startup Offer represents an offer for all costs incurred in starting up and reaching LSL of the configuration, excluding fuel associated with the metered generation after breaker close and before LSL.

The amount of time to start up the Combined Cycle Configuration should be the time required to get to LSL of the configuration.

# Minimum Energy Offers

The Minimum Energy Offers can be submitted for each CPP configuration on hourly basis. These costs represent production costs at LSL of given CCP configuration.

# Energy Offers

There should be an Energy Offer for each configuration.  SCED will only consider the Energy Offer for the configuration shown in the telemetered resource status.

# COP

There should be a COP for each configuration.  Only one configuration should be shown in the COP as being on-line when the plant is online.

# High Level Settlement Issues

All of the Resource level Charge Types will be ultimately calculated for the plant.  Configuration information will have to be used to perform the calculations.  The following will be provided at the plant level:

DAMMWAMT, RUCMWAMT, RUCCBAMT, RUCDCAMT, BPDAMT, EMREAMT, VSSVARAMT, VSSEAMT, BSSAMT, RMRSBAMT, RMREAMT, RMRAAMT, and RMRNPAMT.

Also, the following Data Aggregation cuts will be at the plant level:

RTMG and GSPLITPER.

The telemetered Resource Status will show the plant’s configuration.  It is assumed that the S&B system will be provided one configuration for each 15 minute interval.  In other words, the transformation of 4 sec data into 15 minute data will be performed upstream to S&B, most likely in EMS.  The rule is that the configuration for the 15 minute interval is the configuration that the plant was in for most of the 15 minute interval.  The telemetered value is limited to only registered configurations.

# Average Incremental Energy Costs

Settlements will receive an indicator that contains the configuration the Combined Cycle Resource was awarded in the DAM, committed in RUC, or operated in during Real-Time for VSS.

Only one configuration will be valid for any 15-minute Settlement Interval.  Only one configuration is valid for each 15-minute settlement interval within a DAM or RUC-Committed hour.

All submitted offer curves and/or output schedules will be supplied to Settlements.

Every configuration will have a corresponding COP and therefore a corresponding LSL/DALSL and HSL/DAHSL.

AIEC for Combined Cycle Resources will be calculated at a configuration level.

# Eligibility

Overlapping DAM and RUC-Committed Hours

Eligibility will handle overlapping DAM and RUC commitments for a CCP in the same way it handles overlapping DAM and RUC commitments for non CCP Resources.

As with non CCP Resources the DAM commitment will always be the earliest issued commitment. In the case of overlapping RUC commitments, the last RUC process that results in a RUC instruction to a configuration will determine the RUC commitment.

Overlapping RUC and RUC-De-committed Hours

Eligibility will handle RUC-Decommitments that overlap with a DAM commitment or RUC commitment for a CCP the same way it handles the overlap for non CCP Resources.

If the RUC De-commitment occurred after the RUC commitment was issued, then both the RUCD and the RUC will be altered to <3>, not a valid commitment.  If the RUC commitment occurred after the RUC De-commitment, only the RUCD would be altered to reflect an ineligible commitment.

Is the CCP  eligible for  Startup compensation?

Eligibility will determine if the CCP is eligible to receive compensation for a start; (ie if there is valid start, “off-to-on” or a valid transition, “transition from one configuration to another configuration”).  Four flags will be created from Eligibility, DAMSUFLAG, DAMTFLAG, RUCSUFLAG and RUCTFLAG, all at the CCP level.  The RUCSUFLAG and the DAMSUFLAG data will identify eligible “off-to-on” starts. (1=eligible start, 0=no start, or start not eligible).  Eligibility will also provide, for DAM and RUC settlements, a new data element that identifies transitions that are eligible for startup costs. A Plant that receives a DAM or RUC commitment could be eligible for either a SU or a Transition, but not both for the same action. The new data elements will be the DAMTFLAG and the RUCTFLAG, which will designate a Resource eligible for SU transition compensation. This data will be at the CCP level.

Eligibility for RUC Startup

The same rules are applied to CCPs to determine eligibility for startup compensation that are utilized for non CCP Resources. The plant must be off for five minutes in the six hour window prior to the instruction and must generate in the instructed configuration for at least one minute during the commitment period.

Transitions will be validated by confirming that the plant generated in the committed configuration for at least one minute during the commitment period.

Note:  If a CCP is instructed to go to configuration 2 and never gets there but gets to configuration 1, this is seen as a failed attempt and will not get paid startup.  However, if the QSE comes in (with a dispute) and says they spent x dollars attempting to get to configuration 2, this amount will be considered and reviewed and could be paid.  (See Verifiable Cost Manual).

Determination of Startup type (hot, intermediate, or cold)

The STARTYPE will be at the CCP level and will be provided to Settlements by MMS, as is the process for non CCP Resources.

Eligibility for RUC De-commitment

Eligibility provides for RUC-Decommitment settlement, the RUCDFLAG and RUCDSTARTTYPE.  Both of these data elements will be at the CCP level.

The same rules for other units apply to CCP’s.  In other words, eligibility for decommitment payment is only valid when the plant is shown in the QSE’s COP to be on-line through the end of the day.

For example, if the CCP receives a RUC-Decommitment to go down to configuration 2 from configuration 3, it is entitled the start up cost to go from configuration 2 to 3 at the end of the decommitted period, only if they were planning (and showed in their COP) to be at a configuration higher than 2 through midnight at the time the decommitment was issued.

A Resource that is brought on by a RUC-Commitment is not eligible for a RUC-Decommitment payment.

The method to determine the start-type to be paid to the plant for the de-commitment is consistent with the method used for all other Resources.   The Resource Parameters (or configuration parameters in the case of a combined-cycle plant) and the length of the de-commitment are used to determine the start-type.

# Eligibility for DAM Settlement

The data sent from MMS to settlements for a CCP that has been awarded in the DAM will include which configuration was selected through the DAM process.  Settlements will use the data associated with this configuration in the DAM Make-Whole Settlement calculations.

The same rules are applied to CCPs to determine eligibility for startup compensation that are utilized for non CCP Resources. The plant must be off for five minutes during the adjustment period and must generate for at least one minute during the award period.

Transitions will be validated by confirming that the plant generated  for at least one minute during the award period.

A CCP will be Eligible for Energy compensation if they generate for at least one minute during the hour of the award. Eligibility will provide a DAMWENEFLAG at the CCP level.

# RUC Settlement

For RUC settlement of CCPs in Make-Whole, Clawback and Decommitment, the following calculations will be calculated for each configuration: VERISU, VERIME, SUPR and MEPR.  All other RUC calculations will be calculated for the plant.

RUC Make-Whole Payment and RUC Clawback Charge

Assumption:  RUC commitments will be issued to the QSE for the CCP and will state which configuration is committed.

RUCHR will be calculated at the CCP level by combining all RUC-Commitments for each configuration for the CCP.

RUC calculations that only calculate within RUC-Committed hours (RUCG, RUCMEREV and RUCEXRR) will use the corresponding data for the configuration the CCP is committed to be in, regardless of what is seen in Real-Time.  This configuration data would include SUPR, MEPR, LSL and RTAIEC.  It is expected that Compliance will monitor CCP behavior to insure they follow commitments timely and accurately.

RUC calculations that are calculated only for QSE Clawback intervals (RUCEXRQC) will use the corresponding data for the configuration in which the CCP operates in real-time.  This configuration data would include MEPR, LSL and RTAIEC.

Clawback Factors (RUCCBFR and RUCCBFC) will be based on the CCP, not the configuration.  This means that if any configuration is offered into the DAM for the CCP, then the RUCCBAMT is calculated for the CCP with an offer-submitted status.

RUCMWAMT and RUCCBAMT will be calculated at the CCP level.

RUC Decommitment Payment

De-commitment is when ERCOT instructs the CCP to go off-line or move to a configuration with a lower LSL.  ERCOT will send out a RUC Decommitment instruction only to maintain the reliability of the ERCOT system as stated in Section 5 of the Nodal Protocols.

Assumption:  RUC-Decommitments will be specific and will state “go completely off” or “go from configuration 3 to configuration 2”. For example: The RUCD should contain <from config>_<to config> or <from config>_<off> in the data element.   This should not be in the comments field.

NCDCHR will be calculated at the CCP level by combining all RUC-Decommitments for each configuration for the CCP.

RUC-Decommitment settlement will use the corresponding data for the decommitted configuration.  This configuration data would include SUPR, MEPR and LSL.

RUCDCAMT will be calculated at the CCP level.

# Minimum and Maximum Resource Prices to be used in CRR Hedge Value Calculations

The minimum and maximum Resource Prices to be used in the CRR hedge value calculation at the Logical Resource Node for a CCP shall be determined by considering the all the configurations of the plant.  The size of the CCP is determined by the capacity of the largest simple cycle combustion turbine in the train of all of the configurations shown in registration and submitted in the RARF.  It does not matter which configuration is shown as available or on-line.

# Dual Grid Combined Cycle Plants

ERCOT has several Combined Cycle Plants that can be interconnected to ERCOT and another grid at the same time.  An example is a plant with two GT’s interconnected to ENTERGY while the ST is interconnected to ERCOT.  It is expected that the QSE telemeters the configuration of the plant in ERCOT and indicates through “adjusted” high and low limits what is available to ERCOT.  SCED will use that data as input and provide Base Points for the plant.  The Base Points reflect the output that is expected to be injected into ERCOT from the plant.  The metering of the output of the CCP should provide measurement of only the output that is provided to ERCOT.  For consideration in the DAM and RUC, the QSE should show only those configurations that are available for ERCOT.  In the end, Settlements should see the configurations that the plant was in and instructed to go to, and use that information for settlement.