TPTF Discussion originally from 02/22/2008

Follow-up from TPTF request to add Load Resource Flag to AS Trades Submissions

Responsive Reserve Services can be provided by:

Generation Resources

Controllable Load Resources (CLR)

Non-CLR Load Resources

Limitation of RRS from non-CLR Load Resources is 50% for total RRS

No limits of RRS from CLR (but ERCOT has coding to allow it to be limited if needed in future)

Protocols do not specify tracking mechanics in submission for RRS from the three resource types (Gen, CLR, non-CLR Load Resource)

- AS Offers  into DAM/SASM - Resource specific (so ERCOT tracks by Resource Type)

- Self-Arranged AS   flag recently added to delineate RRS from CLR/non-CLR Load

- AS Trades- do not reflect if RRS from CLR or non-CLR Load Resources

No comments received, but additional issue surfaced at TPTF 030508-

Discussion of negative AS Self-Arrangement

- Current MMS only allows self-arranged AS from QSEs with ERCOT assigned obligation

- Current MMS does not allow self-arranged AS less than zero

4.4.7.1	Self-Arranged Ancillary Service Quantities

(1)	A QSE may self-arrange all or a portion thereof, but not to exceed, the Ancillary Service Obligation allocated to it by ERCOT.  If a QSE elects to self-arrange Ancillary Service capacity, then ERCOT shall not pay the QSE for the Self-Arranged Ancillary Service Quantities for the portion that meets its Ancillary Service Obligation.

(2)	The QSE must indicate before 1000 in the Day-Ahead the Self-Arranged Ancillary Service Quantities, by service, so ERCOT can determine how much Ancillary Service capacity, by service, needs to be obtained through the DAM.

(3)	At or after 1000 in the Day-Ahead, a QSE may not change its Self-Arranged Ancillary Service Quantities unless ERCOT opens a Supplemental Ancillary Service Market.

(4)	Before 1430 in the Day-Ahead, all Self-Arranged Ancillary Service Quantities must be represented by physical capacity, either by Generation Resources or Load Resources, or backed by Ancillary Service Trades.

(5)	When a QSE chooses to self-arrange all or a portion of its Ancillary Service Obligations, it commits to the following conditions:

(a)	The QSE may self-arrange Regulation Up Service (Reg-Up), Regulation Down Service (Reg-Down), Responsive Reserve Service (RRS), and Non-Spin;

(b)	The QSE may provide all or part of its Self-Arranged Ancillary Service Quantity from one or more Resources it represents;

(c)	The QSE may provide all or a part of its Self-Arranged Ancillary Service Quantity through an Ancillary Service Trade;

(d)	The additional Self-Arranged Ancillary Service Quantity specified by the QSE in response to a Supplemental Ancillary Service Market notice by ERCOT to obtain additional Ancillary Services in the Adjustment Period cannot be more than the additional Ancillary Service amount allocated by ERCOT to that QSE, as stated in the SASM notice, and cannot be changed once committed to ERCOT;

(e)	If a QSE does not self-arrange all of its Ancillary Service Obligation, ERCOT shall procure the remaining amount of the Ancillary Service Obligation for the QSE; and

(f)	For self-arranged Responsive Reserve, the QSE will indicate the quantity of the service that is provided from Generation Resources, Controllable Load Resources (CLR) or non-CLR Load Resources.

4.4.7.3	Ancillary Service Trades

(1)	An Ancillary Service Trade is the information for a QSE-to-QSE transaction that transfers an obligation to provide Ancillary Service capacity between a buyer and a seller.

(2)	An Ancillary Service Trade that is reported to ERCOT by 1430 in the Day-Ahead changes the Ancillary Service Supply Responsibility of the buyer and seller in the DRUC process. An Ancillary Service Trade that is reported to ERCOT after 1430 in the Day-Ahead changes the Ancillary Service Supply Responsibility of the buyer and seller in any applicable HRUC process, the deadline for which is after the trade is submitted.

(3)	As soon as practicable, ERCOT shall notify each QSE through the Messaging System of any of its Ancillary Service Trades that are invalid Ancillary Service Trades.  The QSE may correct and resubmit any invalid Ancillary Service Trade, but the reporting time of the trade is determined by when the validated Ancillary Service Trade was submitted and not when the original invalid Ancillary Service Trade was submitted.

4.4.7.3.1	Ancillary Service Trade Criteria

(1)	Each Ancillary Service Trade must be reported by a QSE and must include the following information:

(a)	The buying QSE;

(b)	The selling QSE;

(c)	The type of Ancillary Service;

(d)	The quantity in MW;

(e)	The first and last hours of the trade; and

(f)	For Responsive Reserve, the QSE will indicate the quantity of the service that is provided from Generation Resources, Controllable Load Resources (CLR) or non-CLR Load Resources.

(2)	An Ancillary Service Trade must be confirmed by both the buyer and seller to be considered valid and to be used in an ERCOT process.

4.4.7.3.2	Ancillary Service Trade Validation

(1)	A valid Ancillary Service Trade is an Ancillary Service Trade that ERCOT has determined meets the criteria listed in Section 4.4.7.3.1, Ancillary Service Trade Criteria.  Only one confirmed Ancillary Service Trade is allowed for the same buying and selling QSEs for each type of Ancillary Service for each hour.

(2)	When an Ancillary Service Trade is reported to ERCOT, ERCOT shall notify both the buying and selling QSEs by using the Messaging System if available and the MIS Certified Area.

(3)	ERCOT shall continuously validate Ancillary Service Trades and continuously display on the MIS Certified Area information that allows any QSE named in an Ancillary Service Trade to view its confirmed and unconfirmed Ancillary Service Trades.

(4)	The QSE that first reports the Ancillary Service Trade to ERCOT is deemed to have confirmed the Ancillary Service Trade unless it subsequently affirmatively rejects it.  The QSE that first reports an Ancillary Service Trade may reject, edit, or delete an Ancillary Service Trade that its counterpart has not confirmed.  The counterpart is deemed to have confirmed the Ancillary Service Trade when it submits an identical Ancillary Service Trade.  After both the buyer and seller have confirmed an Ancillary Service Trade, either party may reject it at any time, but the rejection is effective only for any ERCOT process for which the deadline for reporting Ancillary Service Trades has not yet passed.

4.4.7.4	Ancillary Service Supply Responsibility

(1)	A QSE’s Ancillary Service Supply Responsibility is the net amount of Ancillary Service capacity that the QSE is obligated to deliver to ERCOT, by hour and service type, from Resources represented by the QSE. The Ancillary Service Supply Responsibility is the difference in MW, by hour and service type, between the amounts specified in (a) and (b) defined as follows:

(a)	The sum of:

(i)	the QSE’s Self-Arranged Ancillary Service Quantity;

(ii)	the total (in MW) of Ancillary Service Trades for which the QSE is the seller; plus

(iii)	Awards to the QSE of Ancillary Service Offers in the DAM; and

(b)	The total Ancillary Service Trades for which the QSE is the buyer.

(2)	The QSE is also responsible for maintaining the limit of the total Responsive Reserve Service reflected in the COP of its non-CLR Load Resource to remain within the capacity that ERCOT received from the QSE in self-arrangement, the awarded amount from ERCOT in the DAM and SASM, and the net of the AS Trades. This limit is the difference in MW by hour, between the amounts specified in (a) and (b) defined as follows:

(a)	The sum of:

(i)	the QSE’s Self-Arranged Responsive Reserve Service Quantity from Load Resource excluding CLR;

(ii)	the total (in MW) of Responsive Reserve Service Trades for which the QSE is the seller and the resource is Load Resource excluding CLR; plus

(iii)	Awards to the QSE of Responsive Reserve Service Offers for Load Resource excluding CLR in the DAM; and

(b)	The total Ancillary Service Trades for which the QSE is the buyer and the resource is Load Resource excluding CLR.

(3)	If the percentage of Responsive Reserve Service from CLR is also specified by ERCOT, the QSE is also responsible for maintaining the limit of the total Responsive Reserve Service reflected in the COP of its CLR Load Resource to remain within the capacity that ERCOT received from the QSE in self-arrangement, the awarded amount from ERCOT in the DAM and SASM, and the net of the AS Trades.  This limit is the difference in MW by hour, between the amounts specified in (a) and (b) defined as follows

(a)	The sum of:

(i)	the QSE’s Self-Arranged Responsive Reserve Service Quantity from CLR;

(ii)	the total (in MW) of Responsive Reserve Service Trades for which the QSE is the seller and the resource is CLR; plus

(iii)	Awards to the QSE of Responsive Reserve Service Offers for CLR in the DAM; and

(b)	The total Ancillary Service Trades for which the QSE is the buyer and the resource is CLR.

(4)	A QSE may only use a Resource to provide its Ancillary Service during non-RUC-Committed Intervals.

(5)	By 1430 in the Day-Ahead, the QSE must notify ERCOT, in the QSE’s COP, which Resources represented by the QSE will provide the Ancillary Service capacity necessary to meet the QSE’s Ancillary Service Supply Responsibility, specified by Resource, hour, and service type. The DAM Ancillary Service awards are Resource-specific; the QSE must include those DAM awards in its COP, and the QSE may not change that Resource-specific DAM award information until after 1600 under the conditions set out in Section 3.9, Current Operating Plan (COP).

(6)	Section 6.4.8.1.3, Replacement of Ancillary Service Due to Failure to Provide, specifies what happens if the QSE fails on its Ancillary Service Supply Responsibility.