Implementation of DGTF Recommendation No. 2

The DGTF Recommendation No. 2, Large Distributed Renewable Generation.

listed two combinations which should be treated as Large Renewable Generation:

- The renewable generator has a capacity of 50 – 2,000 kW

- The renewable generator has a capacity < 2,000 kW and the ESI ID is assigned to BUSIDRRQ

The task force recommendation for these combinations related to metering was:

- Require IDR metering capable of separately measuring load (in-flow) and generation (out-flow)

- Alternatively, allow the DG owner to pursue registration and settlement as a traditional generation resource

If metering option 2 is selected, the settlement metering section of Protocols applies.  Option 1, on the other hand, probably requires a Protocol Section 18 modification to the IDR Requirement language.

Consider, for example, an ESI ID currently assigned to BUSLOLF that installs a 50 kW PV generator.  Adoption of the DGTF Recommendation would result in requiring installation of 2-way IDR metering.  Not adopting the recommendation would allow NIDR metering of load and IDR metering of generation at a single premise, which may be seen as an undesirable metering setup.

Suggestion to discuss this at PWG, and, if the group has consensus to require 2-way IDR metering, to draft PRR language to that effect.  Since there is no 150 day notice associated with this change, it would not require fast-tracking, but before PWG is done with DG we should make a decision on how to handle this issue.