| LPGRR Number | 029 | LPGRR Title | Modification of Time-Of-Use Schedule Approval Process |
| --- | --- | --- | --- |

| Load Profiling Guide (LPG) Sections Requiring Revision | 16, Supplemental Load Profiling 16.1, Load Profiling for Time-Of-Use Schedules 16.1.1, Existing TOU Schedules 16.1.2, Establishing New TOU Schedules 16.1.2.1, Timing of New TOU Schedule Implementation 16.1.3, Chunking Profiling Methodology Description 16.1.4, Evaluation of the Chunking Profiles 16.1.4.1, Assessment of Chunking as a General Method Base on Profiles from Other Areas 16.1.4.2, Assessment of the Chunking Method Using ERCOT Load Research Data 16.1.4.3, Assessment of Chunking Method Base on Characteristics of the TOU and NOTOU Populations 16.1.5, Revisions to TOU Profile Methods if Changes Are Needed |
| --- | --- |
| Proposed Effective Date | To be decided. |
| Priority & Ranking Assigned | Not applicable. |
| Revision Description | This Load Profiling Revision Request (LPGRR) provides language cleanup and removes the requirement for Public Utility Commission of Texas (PUCT) approval of new Time-Of-Use Schedules (TOUS). |
| Overall Market Benefit | Reduce Market Participant cost for TOUS approval. |
| Overall Market Impact | Not known. |
| Consumer Impact | Not known. |
| Procedural History | - On 11/27/07, LPGRR-029 was posted. - On 2/28/08, the Profiling Working Group (PWG) considered LPGRR-029 - On 4/23/08, the PWG considered LPGRR-029. |
| PWG Decision | On 2/28/08, the PWG was in consensus to table LPGRR-029. On 4/23/08, the PWG was in consensus to recommend to the COPS approval of LPGRR-029 as modified by the PWG. |
| PWG Discussion | On 2/28/08, the PWG decided to focus its efforts on PRR-756 and LPGRR-030. On 4/23/08, the PWG reviewed LPGRR-029 and made changes to it. |

| Sponsor |  |
| --- | --- |
| Name | Ernest Podraza |
| E-mail Address | Ernest.podraza@directenergy.com |
| Company | Direct Energy |
| Company Address | 12 Greenway Plaza Suite 600, Houston Texas 77046 |
| Phone Number | 713-877-3517 |
| Fax Number | 713-877-3559 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| None. |  |

Quantitative Impacts and Benefits

| Assumptions | 1 | none |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | none | none |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | reduce MP cost for TOU Schedule approval | none |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

Proposed Guide Language Revision

# 16	SUPPLEMENTAL LOAD PROFILING

(1)	Protocol Section 18.7, Supplemental Load Profiling, requires that supplemental Load Profiles be developed for programs or pricing schemes that encourage a Demand response to price in the retail market.  A Demand Response (DR) program is designed to alter Load shape.  For such programs, methods other than Adjusted Static Methodology are necessary.  The supplemental Load Profiling Methodologies described in this section of the Load Profiling Guide (LPG) are intended only for DR programs or pricing schemes.  Use of these methodologies for other applications requires approval of the TAC.

(2)	The ERCOT Protocols allow Premises with Time-Of-Use (TOU) capable meters to be settled by a profiling method known as chunking, which is described below in Section 16.1.2, Chunking Profiling Methodology Description.  Only those Premises with TOU metered energy can utilize this capability.  The Protocols require that Direct Load Control (DLC) programs shall be profiled using Representative Interval Data Recorder (RIDR) profiles based on statistically representative Load research samples (Protocol Section 18.7.2, Load Profiling of ESI IDs Under Direct Load Control).  Other supplemental profiles (Protocol Section 18.7.3, Other Load Profiling) are limited to segments that are subject to pricing schemes designed to encourage DR.  The appropriate methodology for other supplemental profiling shall be determined based on the characteristics of the DR program.

## 16.1	Load Profiling for Time-Of-Use Schedules

(1)	Subject to the following restrictions (Protocol Section 18.7.1, Load Profiling of Time-Of-Use Metered ESI ID) all Competitive Retailers (CRs) have the right to offer Time-Of-Use Schedules (TOUS) in all Transmission and/or Distribution Service Provider (TDSP) service territories:

(a)	Within each TDSP service territory, a CR may offer only TOUSs that are listed in Appendix D, Profile Decision Tree.

(b)	Implementation of any new TOUS is subject to the ERCOT and Texas Standard Electronic Transaction (TX SET) change control process.

(2)	The right to use TOUS, subject to restrictions (a) and (b) stated above, applies in both investor-owned TDSP service territories, in Non Opt-In Entities (NOIEs) territories if they opt in, and in any new TDSP territories.  For purposes of TOUS management, all TOUS for NOIE or TDSP territories that opt into the market shall be considered new TOUS and therefore subject to the new TOUS process in Section 16.1.1, Establishing New Time-Of-Use Schedules.

### 16.1.1	Establishing New Time-Of-Use Schedules

(1)	Available TOUS are listed in Appendix D, Profile Decision Tree.  Any request for a new TOUS shall be submitted as a Load Profiling Guide Revision Request (LPGRR) in accordance with the process set forth in the Section 2, Load Profiling Guide Revision Process.

(2)	Currently, the ERCOT settlement system can only accommodate TOUS that have up to four TOU periods (e.g., Off-Peak, Mid-Peak, On-Peak, Super-Peak).  Requests for new TOUS that have four or less buckets can be implemented in ERCOT systems within seven Business Days of approval.  Requests for TOUS with more than these four buckets will require significant system changes and will subject the LPGRR requesting the new TOUS to the project prioritization process within ERCOT to determine their implementation time.

(3)	Since TOUS also impact TDSP systems and these systems vary in their ability to support these schedules, ERCOT Staff and the Profiling Working Group (PWG) will coordinate closely with TDSP(s) impacted by a new proposed TOUS.  Each impacted TDSP will be requested to submit comments to the LPGRR regarding the system impacts and time frame required to implement the requested TOUS.  The PWG will incorporate this time frame in the expected implementation date for the LPGRR.

(4)	ERCOT shall issue a market notice once the new TOUS has been incorporated into ERCOT systems, and also once the affected TDSP’s have implemented the new TOUS and notified ERCOT Load Profiling by sending an email to ERCOTLoadProfilingDepartment@ercot.com that the TOUS is ready.  At this point CR’s may begin to request that the TOU meters be installed.

- 16.1.2	Chunking Profiling Methodology Description

The chunking method of Load Profiling for TOU Customers means that a profile is applied to the Customer’s consumption data for a meter reading period.  However, the energy for each TOU period in the Load Profile is scaled so that it is equal to the metered energy (kWh) for the TOU period.  For each TOU period within a meter read cycle, the metered consumption during the TOU period is allocated to time intervals within the TOU period in proportion to the Load Profile level at each interval in that period.

### 16.1.3	Evaluation of the Chunking Profiles

(1)	Load Profiles that are adjusted by chunking shall be evaluated as part of the general evaluation procedures described in Section 6, Load Profiling Methodology and Section 8, Load Profile Models.  Evaluation of the effects of chunking shall be included in the assessment of Unaccounted for Energy (UFE) described in Section 8.3, Evaluating Profile Models Without Current Load Research Data.  If Load research data are available for a sample of TOU Customers, these data could also be used in the evaluation using methods discussed in Section 8.3.

(2)	Additional procedures that may be used to assess the adequacy of chunking include the following:

(a)	Assess chunking as a general method based on profiles from other areas;

(b)	Assess chunking using Load research data collected in the ERCOT service territory; and

(c)	Assess chunking using data on Customer characteristics in the ERCOT service territory.

(3)	These procedures are described below.

#### 16.1.3.1	Assessment of Chunking as a General Method Based on Profiles 							from Other Areas

(1)	This assessment evaluates chunking as a general methodology, not necessarily specific to particular ERCOT Load Profiles.  Such an assessment could be conducted using Load Profiles from another source service area.  To be used in this analysis, the source service area shall have separate profiles based on separate Load research samples for a TOU class and a corresponding Non-Time-Of-Use (NOTOU) class.

(2)	The assessment compares the source service area’s TOU profile based on the TOU sample to a chunked profile created by applying the chunking method to the source service area’s corresponding profile.  To create the chunked profile, the TOUS applicable from this TOU class in the source service area are used, with the chunking procedures described in Section 16.1.2.

(3)	The two profiles are compared using the comparison methods of Section 8.  In the terminology of Section 8, the first profile, based on the actual Load research data from the TOU class, is treated as the proposed profile.  The second, chunked profile is the existing profile.  That is, the second profile is the profile the existing ERCOT methodology would use for the TOU class, if these classes were in ERCOT.

(4)	Such comparisons may be made for several different TOU classes, with corresponding NOTOU classes, in different regions, depending on available Load Profiles from other areas.  Substantial differences between the two profiles for many of the classes studied would indicate problems with the general approach.  Substantial similarities between the two profiles for most classes would indicate that the general method is reasonable.

#### 16.1.3.2	Assessment of the Chunking Method Using ERCOT Load Research Data

(1)	This assessment relies on a limited ERCOT Load research sample to evaluate the TOU profiles developed by chunking.  The goal is to compare two profiles, the existing TOU profile developed by the chunking method to:

(a)	The corresponding non-chunked profile.

(b)	The profile for the same population of TOU ESI IDs developed from a direct Load research sample of that population.

(2)	For purposes of this assessment, ERCOT may implement a limited Load research sample from each ERCOT TOUS and segment to be studied.  The Sampling criteria for each Profile Type and schedule do not have to adhere to the Sampling guidelines established in Section 15.2.3, Criteria of Standards since these samples are not being used for settlement purposes.

(4)	For each segment and TOUS sampled, ERCOT shall determine the average Load for each hour of the study period from the Load research sample data.  This Load research based profile shall then be compared to the existing chunked profile, using the comparison methods of Section 8.  In the terminology of Section 8, the chunked profile is the existing profile, and the profile developed from the Load research sample is the proposed profile.

(5)	Substantial differences between the two profiles for many of the classes studied would indicate problems with the general approach.  Substantial similarities between the two profiles for most classes would indicate that the general method is reasonable.  The results might also indicate that the method is adequate for some classes but not for others.

#### 16.1.3.3	Assessment of Chunking Method Based on Characteristics of the 						Time-Of-Use and Non-Time-Of-Use Populations

(1)	This assessment is less direct than the previous two assessment methods.  The goal is to determine whether behavioral or operational differences between the TOU and NOTOU Customers are large enough to create substantial differences between the true Load shape for the TOU group and the chunked profile.

(2)	For this assessment, ERCOT may examine data on appliance/equipment use patterns for ERCOT TOU and NOTOU Customers.   Such data may be obtained from appliance saturation studies conducted by TDSPs, if available, or from a new survey conducted by ERCOT.  If little difference is found between TOU and NOTOU Customers in the types of equipment in place and timing of its use, the chunking method may be considered adequate.

(3)	If substantial differences are found, ERCOT may develop rough adjustments to the NOTOU profile that reflect these differences.  Such adjustments would require estimated end-use Load shapes, which may be provided by TDSPs if available, or obtained from commercial databases.

(4)	The adjusted profile would then be chunked to provide a new estimate of the TOU profile.  This new TOU profile would then be compared with the original TOU profile, using the comparison methods of Section 8.

(5)	In the terminology of Section 8, the original chunked profile is the existing profile, and the new profile developed from chunking the adjusted NOTOU profile is the proposed profile.  However, this new profile based on rough adjustments would not in fact replace the existing TOU profile if substantial differences are found.  Rather, these differences would be taken as an indication that the chunking method is inadequate for this segment.

(6)	Likewise, if substantial differences are found by this method for several segments, the chunking methodology as a whole may be questioned.  Conversely, if several segments are examined and no substantial differences are found, the general chunking methodology is supported.

### 16.1.4	Revisions to Time-Of-Use Profile Methods if Changes Are Needed

If the current chunking is determined to be an inadequate methodology for profiling TOU Customers, the change to any other profiling method for these Customers would require the TAC approval in accordance with the Protocol Section 18.2, Methodology.  The primary alternative that would be considered is lagged dynamic Load Profiling.  Other alternatives may be proposed.