| NPRR Number |  | NPRR Title | Section 7 Cleanup |
| --- | --- | --- | --- |
| Date Posted |  | May 06, 2008 |  |
| Protocol Section(s) Requiring Revision (Include Section No. and Title) |  | 7.3.1 Flowgates 7.7.3 Allocation of McCamey Flowgate Rights (MCFRIs) 7.9.1.4 Payments for FGR Settled in DAM 7.9.2.4 Payments for FGR Settled in Real-Time 7.9.1.5 Payments and Charges for PTP Obligations with Refund Settled in DAM 7.9.1.6 Payments for PTP Options with Refund Settled in DAM 7.9.2.3 Payments for NOIE PTP Options with Refund Settled in Real-Time 7.9.2.5 Payments and Charges for PTP Obligations with Refund in Real-Time |  |
| Requested Resolution (Normal or Urgent, and justification for Urgent status) |  |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) adds clarifications to the flowgate definition and the variables that are used in flowgate settlements. This NPRR proposes corrections to flowgate formulae and clarifying variables and subscripts. The NPRR also proposes revisions that streamline the OBLRACT and OPTRACT calculations, but maintain the intent of the original calculations. With this revision less data is required from MMS and a fairly complicated calculation is avoided. This revision came about during interface discussions with MMS. |  |
| Reason for Revision |  | Clarifies and corrects flowgate settlements calculation Simplifies the calculation of OBLRACT and OPTRACT. |  |
| Overall Market Benefit |  | To have the Protocols accurately reflect flowgate settlements. Easier implementation without a compromise in the result of the calculation. |  |
| Overall Market Impact |  |  |  |
| Consumer Impact |  |  |  |
| Credit Implications (Yes or No, and summary of impact) |  |  |  |
| Reason for Revision (from Transition Plan Task Force (TPTF) Charter Scope) |  | (1) Revisions resulting from Commission orders; (2) Clarifications of Protocol language that do not change the intent or technical specifications of the Protocols; (3) Correction of technical errors or processes that are found to not be technically feasible; (4) Revisions to the Protocols necessary to implement the results of the value engineering analysis or to otherwise avoid severe cost impacts; or (5) Other (describe): Corrects flowgate settlements formula and adds clarifications to the description of flowgate variables. |  |
| TPTF Review (Yes or No, and summary of conclusion) |  |  |  |

Quantitative Impacts and Benefits

Instructions:  To allow for comprehensive NPRR consideration and development of the Cost Benefit Analysis (CBA), please fill out each block below completely and provide as much detailed information as possible.  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to the PRR.

| Assumptions | 1 | e.g.: Key assumptions used in estimating market cost and/or benefit |  |
| --- | --- | --- | --- |
|  | 2 | Dependencies on other projects or other timing requirements |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | e.g.: Cost per MP to implement | e.g.: $10,000 each for 50 QSEs |
|  | 2 | Add’l staff required per MP | 1.5 FTE each for 6 TDSPs @ $65/hour |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | e.g.: Reduced MP costs | e.g.: 2 FTE reduction for 25 CRs @ $65/hour |
|  | 2 | Enhanced MP efficiency | 2 hour savings per day for 50 generators @$65 |
|  | 3 | Reduced congestion cost | 0.5% reduction in total congestion cost |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | e.g.: Benefits that are difficult to quantify |  |
|  | 2 | Benefits that are not certain but relatively likely |  |
|  | 3 | Customer service impacts, cash flow impacts, transaction speed, etc. |  |
|  | 4 |  |  |
| Other Comments | 1 | e.g.: Thoughts on ERCOT systems impacts |  |
|  | 2 | Potential manual workarounds or delivery options |  |
|  | 3 | Other comments of value to PRS, TAC and the Board of Directors |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Mandy Bauld/Sharon Wang |
| E-mail Address | Abauld@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6455 |
| Cell Number |  |
| Market Segment |  |

| Market Rules Staff Contact |  |
| --- | --- |
| Name |  |
| E-Mail Address |  |
| Phone Number |  |

Proposed Protocol Language Revision

7.3.1	Flowgates

7.3.1.1	Process for Defining Flowgates

Flowgates where ERCOT offers FGRs may only be created by an amendment to Section 7.3.1.2, Defined Flowgates. ERCOT shall post the list of all flowgates available for FGRs on the MIS Public Area. If there is any change in the designation of flowgates, ERCOT shall provide notice to all Market Participants as soon as practicable.

7.3.1.2	Defined Flowgates

McCamey Area flowgates are the only flowgates where FGRs are available in ERCOT as specified in Section 7.7, Congestion Management in McCamey Area.

7.3.1.3	  Flowgate Definition

A McCamey Area flowgate represents a predefined directional network element, i.e. each flowgate shall consist of one directional network element. The network element could be a Transmission Element or a Transmission Generic Constraint (a bundle of Transmission Elements).

An Impact Normalization Factor (INF) should be specified for each contingency (including the null contingency or base case).  The INF is equal to one in the base case by definition.    In contingency cases the INF shall be calculated as the ratio of the contingency rating to the normal rating of the network element of the flowgate.

7.7.3	Allocation of McCamey Flowgate Rights (MCFRIs)

(1)	ERCOT shall determine the “Capacity Impact” of each McCamey Area WGR eligible for MCFRIs on each McCamey Area flowgate by multiplying the maximum rated output for the WGR times its Shift Factor from the base case transmission model for the corresponding McCamey Area flowgate relative to the Load-weighted average Shift Factor of all Electrical Buses in ERCOT.

(2)	Prior to each CRR Auction, ERCOT shall allocate the lesser of 90% of the limit for each McCamey Area flowgate adjusted for impact of previously allocated PCRRs and MCFRIs for the corresponding auction as MCFRIs or the sum of the Capacity Impact for each McCamey Area WGR to each McCamey Area WGR in proportion to its Capacity Impact as a percentage of the sum of all Capacity Impacts for McCamey Area WGRs for the corresponding flowgate.  All commercial and committed WGRs in the McCamey Area must be included in the analysis, but MCFRIs may be allocated only to WGRs that are in commercial operation.  The determination of available McCamey Area flowgate capacity must account for reduced flowgate element capacities resulting from CRRs already sold or allocated.

(3)	ERCOT shall allocate MCFRIs to each McCamey Area WGR as described in (2) above no later than 25 days prior to an annual auction, no later than ten Business Days prior to a monthly auction, and by no later than one hour prior to the DAM

7.9.1.4	Payments for FGRs Settled in DAM

(1)	If an FGR is competitive, i.e., all directional network elements associated with the FGR are Competitive Constraints, ERCOT shall pay the owner of the FGR an amount equal to the sum of the Shadow Price of the hour for each directional network element associated with the FGR for each contingency (including the null contingency or base case) normalized to the impact of the principal network element of the FGR (the normal rating of which is used to determine the total MW amount for the flowgate).  The payment to each CRR Owner for its FGRs determined by the principal network element of each flowgate for a given hour is calculated as follows:

DAFGRAMT o, f	=	(-1) * DAFGRTP o, f

Where:

DAFGRTP o, f	=	DAFGRPR f * DAFGR o, f

DAFGRPR f	=	Σe∈f( Σc INF f, e, c * DASP e, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAFGRAMT o, f | $ | Day-Ahead FGR Amount per CRR Owner per flowgateThe payment to CRR Owner o of the flowgate f settled in DAM, for the hour. |
| DAFGRTP o, f | $ | Day-Ahead FGR Target Payment per CRR Owner per flowgate—The target payment for CRR Owner o’s flowgate f settled in the DAM, for the hour. |
| DAFGRPR f | $/MW per hour | Day-Ahead FGR Price per flowgateThe DAM price of the flowgate f for the hour. |
| DASP e, c | $/MW per hour | Day-Ahead Shadow Price per element per constraintThe DAM Shadow Price on the directional network element e, for constraint c, for the hour. |
| INF f, e, c | none | Impact Normalization Factor per flowgate per element per constraint The parameter that reflects the normalized impact on the directional network element e of flowgate f for each constraint c relative to the impact on the principal directional network element of flowgate f for the base case. |
| DAFGR o, f | MW | Day-Ahead FGR per CRR Owner per flowgateThe CRR Owner o’s total number of FGRs determined by the principal element of flowgate f settled in the DAM for the hour. |
| o | none | A CRR Owner. |
| f | none | A flowgate. |
| e | none | A predefined directional network element. |
| c | none | A constraint for each contingency (including the null contingency and base case) |
| ef | none | The directional network element e that belongs to the flowgate f. |

(2)	If an FGR is non-competitive, i.e., one or more directional network elements associated with the FGR are Non-Competitive Constraints, the FGR payment may be reduced due to transmission elements that are oversold in previous CRR auctions.  The payment for MCFRI, when it is not designated as a Competitive Constraint, is calculated in paragraph (3); the payment for any other FGR, when it is non-competitive, will be specified upon introduction of the FGR.

(3)	The payment to each CRR Owner for its MCFRI for a given hour, when MCFRI is not designated as a Competitive Constraint, is calculated as follows:

DAFGRAMT o, MCFRI	=	(-1) * Max ((DAFGRTP o, MCFRI –         DAFGRDA o, MCFRI), Min (DAFGRTP o, MCFRI, DAFGRHV o, MCFRI))

Where:

The target payment:

DAFGRTP o, MCFRI	=	DAFGRPR MCFRI * DAFGR o, MCFRI

DAFGRPR MCFRI	=Σe∈MCFRI( Σc INF MCFRI, e, c * DASP e, c)

The derated amount:

DAFGRDA o, MCFRI	=	FGRDRPR MCFRI * DAFGR o, MCFRI

FGRDRPR MCFRI	=	Σe∈MCFRI( Σc( INF MCFRI, e, c * DASP e, c * DRF e, c))

The hedge value:

DAFGRHV o ,MCFRI	=	DAFGRHVPR MCFRI * DAFGR o, MCFRI

DAFGRHVPR MCFRI	=	Max (0, (DAWALBEP – MINRESPR j, MCFRI - Σe∉MCFRIΣc (DASP e, c * (DASFMCWGRS e, c - DAWASFLB e, c))) / (DASFMCWGRS e=MCFRI principal element, cBase Case - DAWASFLB e=MCFRI principal element, cBase Case))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAFGRAMT o, f | $ | Day-Ahead FGR Amount per CRR Owner per flowgateThe payment to CRR Owner o of the FGRs associated with flowgate f settled in DAM, for the hour. |
| DAFGRTP o, f | $ | Day-Ahead FGR Target Payment per CRR Owner per flowgate—The target payment for CRR Owner o’s flowgate f settled in the DAM, for the hour. |
| DAFGRHV o, f | $ | Day-Ahead FGR Hedge Value per CRR Owner per flowgate—The hedge value of CRR Owner o’s flowgate f settled in the DAM, for the hour. |
| DAFGRDA o, f | $ | Day-Ahead FGR Derated Amount per CRR Owner per flowgate—The derated amount of CRR Owner o’s flowgate f settled in the DAM, for the hour. |
| DAFGRPR f | $/MW per hour | Day-Ahead FGR Price per flowgateThe DAM price of the flowgate f for the hour. |
| FGRDRPR f | $/MW per hour | FGR Deration Price per flowgate—The deration price of the flowgate f for the hour. |
| INF f, e, c | none | Impact Normalization Factor per flowgate per element per constraintThe parameter that reflects the normalized impact on the directional network element e of flowgate f for each constraint c relative to the impact on the principal flowgate element of flowgate f for the base case. |
| DASP e, c | $/MW per hour | Day-Ahead Shadow Price per element per constraintThe DAM Shadow Price on the directional network element e, for constraint c, for the hour. |
| DAFGR o, f | MW | Day-Ahead FGR per CRR Owner per flowgateThe CRR Owner o’s total number of FGRs determined by the principal element of the flowgate f settled in the DAM for the hour. |
| DRF e, c | none | Deration Factor per element per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASFLB e, c | none | Day-Ahead Weighted Average Shift Factor of Load Buses per element per constraint—The Day-Ahead weighted average Shift Factor for all load buses on the directional network element e, for constraint c, in the hour. |
| DASFMCWGRS e, c | none | Day-Ahead Shift Factor of McCamey WGRs per element per constraint—The Day-Ahead McCamey Area WGR maximum rated output weighted Shift Factor on the directional network element e, for constraint c, in the hour. |
| DAFGRHVPR f | $/MWh | Day-Ahead FGR Hedge Value Price per flowgate—The Day-Ahead hedge price of the flowgate f, for the hour. |
| MINRESPR j, MCFRI | $/MWh | Minimum Resource Price—The lowest Minimum Resource Price for the Resources located at the source j of MCFRI. |
| DAWALBEP | $/MWh | Day-Ahead Weighted Average Load Bus Energy Price—The weighted average DAM energy price of all load buses for the hour. |
| o | none | A CRR Owner. |
| f | none | A flowgate; in this application f = MCFRI. |
| e | none | A predefined directional network element. |
| c | none | A constraint for each contingency (including the null contingency and base base). |
| j | none | A source Settlement Point |
| eMCFRI | none | The directional network element e that belongs to MCFRI. |
| eMCFRI | none | The directional network element e that doesn’t belong to MCFRI. |
| cBase Case | none | The constraint c that is under the Base Case. |

(4)	The total of the payments to each CRR Owner for the Operating Hour of all its FGRs settled in the DAM is calculated as follows:

DAFGRAMTOTOT o	=	ΣfDAFGRAMT o, f

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAFGRAMTOTOT o | $ | Day-Ahead FGR Amount Owner Total per CRR Owner—The total payment to CRR Owner o of all its FGRs settled in the DAM, for the hour. |
| DAFGRAMT o, f | $ | Day-Ahead FGR Amount per CRR Owner per flowgate—The payment to CRR Owner o of the FGRs associated with flowgate f settled in DAM, for the hour. |
| o | none | A CRR Owner. |
| f | none | A flowgate. |

7.9.2.4	Payments for FGRs in Real-Time

(1)	In the event that ERCOT is unable to execute the DAM, ERCOT shall pay the owner of the FGR an amount based on the time-weighted Shadow Price of each SCED interval for each directional network element associated with the FGR for each contingency (including the null contingency or base case) normalized to the impact of the principal network element of the FGR (the normal rating of which is used to determine the total MW amount for the FGR).  The payment to each CRR Owner for its FGRs determined by the principal flowgate element of each flowgate for a given hour is calculated as follows:

NDRTFGRAMT o, f	=	(-1) * NDRTFGRTP o, f

Where:

NDRTFGRTP o, f	=	NDRTFGRPR f * DAFGR o, f

NDRTFGRPR f	= 	ye∈f(INFf,e∗cRTSPe,c,y)∗TLMPyyTLMPyye∈f(cINFf,e,c∗RTSPe,c,y)∗TLMPyyTLMPy

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| NDRTFGRAMT o, f | $ | No DAM Real-Time FGR Amount per CRR Owner per flowgateThe payment to CRR Owner o of the flowgate f settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |
| NDRTFGRTP o, f | $ | No DAM Real-Time FGR Target Payment per CRR Owner per flowgate—The target payment for CRR Owner o’s flowgate f settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |
| NDRTFGRPR f | $/MW per hour | No DAM Real-Time FGR Price per flowgateThe Real-Time price of the flowgate f when ERCOT is unable to execute DAM, for the hour. |
| RTSP e, c, y | $/MW per hour | Real-Time Shadow Price per element per constraint per SCED intervalThe Real-Time Shadow Price on the directional network element e, for constraint c, in the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| INF f, e ,c | none | Impact Normalization Factor per flowgate per element per constraintThe parameter that reflects the normalized impact on the flowgate element e for each constraint c relative to the impact on the principal flowgate element of flowgate f for the base case. |
| DAFGR o, f | MW | Day-Ahead FGR per CRR Owner per flowgateThe CRR Owner o’s total number of FGRs determined by the principal element of flowgate f settled in the DAM for the hour. See Section 7.9.1.4, Payments for FGRs Settled in DAM. |
| o | none | A CRR Owner. |
| f | none | A flowgate. |
| e | none | A predefined directional network element. |
| c | none | A constraint for each contingency (including the null contingency and base case). |
| ef | none | The directional network element e that belongs to the flowgate f. |
| y | none | A SCED interval in the hour. |

(2)	If ERCOT is unable to execute the DAM, the total of the payments to each CRR Owner for the Operating Hour of all its FGRs settled in Real-Time is calculated as follows:

NDRTFGRAMTOTOT o	=	ΣfNDRTFGRAMT o, f

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| NDRTFGRAMTOTOT o | $ | No DAM Real-Time FGR Amount Owner Total per CRR Owner—The total payment to CRR Owner o of all its FGRs settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |
| NDRTFGRAMT o, f | $ | No DAM Real-Time FGR Amount per CRR Owner per flowgateThe payment to CRR Owner o of the flowgate f settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |
| o | none | A CRR Owner. |
| f | none | A flowgate. |

7.9.1.5	Payments and Charges for PTP Obligations with Refund Settled in DAM

(1)	Except as specified otherwise in paragraph (2) below, ERCOT shall pay the owner of a PTP Obligation with Refund the difference in the Day-Ahead Settlement Point Prices between the sink Settlement Point and the source Settlement Point, subject to a charge for refund, when the price difference is positive, as described in the item (e) (i) of Section 7.4.2, PCRR Allocation Terms and Conditions.

(2)	The payment of PTP Obligations with Refund may be further reduced due to transmission elements that are oversold in previous CRR auctions.

(3)	The payment or charge to each CRR Owner for a given Operating Hour of PTP Obligations with Refund with each pair of source and sink Settlement Points settled in the DAM is calculated as follows:

If the PTP Obligation with Refund has a non-positive value, i.e., (DAOBLPR (j, k)  0), then

DAOBLRAMT o, (j, k)	=	(-1) * DAOBLRTP o, (j, k)

If the PTP Obligation with Refund has a positive value, i.e., (DAOBLPR (j, k) > 0), then

DAOBLRAMT o, (j, k)	=	(-1) * Max (DAOBLRTP o, (j, k) - DAOBLRDA, Min (DAOBLRTP, DAOBLRHV))

Where:

The target payment:

DAOBLRTP o, (j, k)	=	DAOBLPR (j, k) * Min (DAOBLR o, (j, k), OBLRACT o, (j, k))

DAOBLPR (j, k)	=	DASPP k - DASPP j

OBLRACT o, (j, k)	=	 ΣyΣr(OBLROF o, r * RESACT r *ΣyOBLRF o , r, (j, k) )

If (A valid OS r, y exists for all SCED intervals within the hour)

RESACT r  	 =    	Σy(  OS r, y * TLMP y) / (ΣyTLMP y)

OS r, y

Otherwise

RESACT r

= 	TGFTH r

The derated amount:

DAOBLRDA o, (j, k)	=	OBLDRPR (j, k) * Min (DAOBLR o, (j, k),  OBLRACT o, (j, k))

OBLDRPR (j, k)	=	Σc(Max (0, DAWASF j, c – DAWASF k, c) *     DASP c * DRF c)

The hedge value:

DAOBLRHV o, (j, k)	=	DAOBLHVPR (j, k) * Min (DAOBLR o, (j, k), OBLRACT o, (j, k))

If the source, j, is a Load Zone or Hub and the sink, k, is a Resource Node,

DAOBLHVPR (j, k)	=	Max (0, MAXRESPR k – DASPP j)

If the source, j, is a Resource Node and the sink, k, is a Load Zone or Hub,

DAOBLHVPR (j, k)	=	Max (0, DASPP k – MINRESPR j)

The above variables are defined as follows:

| Variable |  | Unit |  | Definition |
| --- | --- | --- | --- | --- |
| DAOBLRAMT o, (j, k) |  | $ |  | Day-Ahead Obligation with Refund Amount per CRR Owner per pair of source and sinkThe payment to CRR Owner o for the PTP Obligation with Refund with the source j and the sink k, settled in the DAM, for the hour. |
| DAOBLRTP o, (j, k) |  | $ |  | Day-Ahead Obligation with Refund Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Obligations with Refund, with the source j and the sink k, settled in the DAM, for the hour. |
| DAOBLRHV o, (j, k) |  | $ |  | Day-Ahead Obligation with Refund Hedge Value per CRR Owner per source and sink pair—The hedge value of CRR Owner o’s PTP Obligations with Refund, with the source j and the sink k, settled in the DAM, for the hour. |
| DAOBLRDA o, (j, k) |  | $ |  | Day-Ahead Obligation with Refund Derated Amount per CRR Owner per source and sink pair—The derated amount of CRR Owner o’s PTP Obligations with Refund, with the source j and the sink k, settled in the DAM, for the hour. |
| DAOBLPR (j, k) |  | $/MW per hour |  | Day-Ahead Obligation PriceThe DAM price of a PTP Obligation with the source j and the sink k, for the hour. |
| DASPP j |  | $/MWh |  | Day-Ahead Settlement Point Price at sourceThe DAM Settlement Point Price at the source Settlement Point j for the hour. |
| DASPP k |  | $/MWh |  | Day-Ahead Settlement Point Price at sinkThe DAM Settlement Point Price at the sink Settlement Point k for the hour. |
| OBLDRPR (j, k) |  | $/MW per hour |  | Obligation Deration Price per source and sink pair—The deration price of a PTP Obligation with the source j and the sink k, for the hour. |
| DASP c |  | $/MW per hour |  | Day-Ahead Shadow Price per constraint—The DAM Shadow Price of the constraint c for the hour. |
| DRF c |  | none |  | Deration Factor per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASF j, c |  | none |  | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and the directional network element for constraint c, in the hour. |
| DAWASF k, c |  | none |  | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and the directional network element for constraint c, in the hour. |
| DAOBLHVPR (j, k) |  | $/MWh |  | Day-Ahead Obligation Hedge Value Price per source and sink pair—The Day-Ahead hedge price of a PTP Obligation with the source j and the sink k, for the hour. |
| MINRESPR j |  | $/MWh |  | Minimum Resource Price for source—The lowest Minimum Resource Price for Resources located at the source Settlement Point j. |
| MAXRESPR k |  | $/MWh |  | Max Resource Price for sink—The highest Maximum Resource Price for Resources located at the sink Settlement Point k. |
| DAOBLR o, (j, k) |  | MW |  | Day-Ahead Obligation with Refund per CRR Owner per pair of source and sink The number of CRR Owner o’s PTP Obligations with Refund with the source j and the sink k settled in DAM for the hour. |
| OBLRACT o, (j, k) |  | MW |  | Obligation with Refund Actual usage per CRR Owner per pair of source and sink—CRR Owner o’s actual usage for the PTP Obligations with Refund with the source j and the sink k, for the hour. |
| RESACT r |  | MW |  | Resource Actual per Resource per Hour—The time-weighted average of the Output Schedule of Resource r (if a valid OS exists) or the telemetered output of Resource r, for the hour. |
| OBLROF o, r |  | none |  | Obligation with Refund Ownership Factor per CRR Owner per resource—The factor showing the percentage usage of Resource r for CRR Owner o’s PTP Obligations with Refund. Its value is 1, if only one CRR Owner has acquired PCRRs under the refund provision using this Resource r. |
| OS r, y |  | MW |  | Output Schedule per resource per SCED interval—The Output Schedule submitted to ERCOT for Resource r for the SCED interval y. |
| TGFTH r |  | MWh |  | Telemetered Generation for the Hour per Resource per Hour—The telemetered generation of Generation Resource r, for the hour. |
| OBLRF o ,r, (j, k) |  | none |  | Obligation with Refund Factor per CRR Owner per Resource associated with pair of source and sink—The ratio of CRR Owner o’s Resource r’s capacity allocated to the PTP Obligations with Refund with the source j and sink k to the same CRR Owner’s total capacity for the Resource r nominated for all the PCRRs under the refund provision with the same source j. |
| TLMP y |  | second |  | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| o |  | none |  | A CRR Owner. |
| y |  | none |  | A SCED interval in the hour. |
| r |  | none |  | A Resource. |
| j |  | none |  | A source Settlement Point. |
| k |  | none |  | A sink Settlement Point. |
| c |  | none |  | A constraint associated with a directional network element for the hour. |

(4)	The net total payment or charge to each CRR Owner for the Operating Hour of all its PTP Obligations with Refund settled in the DAM is calculated as follows:

DAOBLRAMTOTOT o	=	DAOBLRCROTOT o + DAOBLRCHOTOT o

Where:

DAOBLRCROTOT o	=	ΣjΣkMin (0, DAOBLRAMT o, (j, k))

DAOBLRCHOTOT o	=	ΣjΣkMax (0, DAOBLRAMT o, (j, k))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOBLRAMTOTOT o | $ | Day-Ahead Obligation with Refund Amount Owner Total per CRR Owner—The net total payment or charge to CRR Owner o for all its PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRCROTOT o | $ | Day-Ahead Obligation with Refund Credit Owner Total per CRR Owner—The total payment to CRR Owner o for its PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRCHOTOT o | $ | Day-Ahead Obligation with Refund Charge Owner Total per CRR Owner—The total charge to CRR Owner o for its PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRAMT o, (j, k) | $ | Day-Ahead Obligation with Refund Amount per CRR Owner per pair of source and sink—The payment or charge to CRR Owner o for the PTP Obligations with Refund with the source j and the sink k settled in the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

7.9.1.6	Payments for PTP Options with Refund Settled in DAM

(1)	Except as specified otherwise in paragraph (2) below, ERCOT shall pay the owner of a PTP Option with Refund the difference in the DAM Settlement Point Prices between the sink Settlement Point and the source Settlement Point, if positive, subject to a charge for refund, as described in the item (e) (i) of Section 7.4.2, PCRR Allocation Terms and Conditions.

(2)	The payment of PTP Options with Refund may be further reduced due to transmission elements that are oversold in previous CRR auctions.

(3)	The payment to each CRR Owner for a given Operating Hour of its PTP Options with Refund with each pair of source and sink Settlement Points settled in the DAM is calculated as follows:

DAOPTRAMT o, (j, k)	=	(-1) * Max ((DAOPTRTP o, (j, k) –       DAOPTRDA o, (j, k)), Min (DAOPTRTP o, (j, k), DAOPTRHV o, (j, k)))

Where:

The target payment:

DAOPTRTP o, (j, k)	=	DAOPTPR (j, k) * Min (DAOPTR o, (j, k), OPTRACT o, (j, k)  * DAOPTR o, (j, k) /        (DAOPTR o, (j, k) + RTOPTR o, (j, k)))

DAOPTPR (j, k)	=	Max (0, DASPP k – DASPP j)

OPTRACT o, (j, k)	=	 ΣyΣr(OPTROF o, r * RESACT rΣy* OPTRF o, r, (j, k))

If (A valid OS r, y exists for all SCED intervals within the hour)

RESACT r	=	Σy( OS r, y * TLMP y) / (ΣyTLMP y)

Otherwise

RESACT r   = 	TGFTH r

The derated amount:

DAOPTRDA o, (j, k)	=	OPTDRPR (j, k) * Min (DAOPTR o, (j, k),    OPTRACT o, (j, k)  * DAOPTR o, (j, k) /        (DAOPTR o, (j, k) + RTOPTR o, (j, k)))

OPTDRPR (j, k)	=	Σc(Max (0, DAWASF j, c – DAWASF k, c) * DASP c * DRF c)

The hedge value:

DAOPTRHV o, (j, k)	=	DAOPTHVPR (j, k) * Min (DAOPTR o, (j, k), OPTRACT o, (j, k) * DAOPTR o, (j, k) /         (DAOPTR o, (j, k) + RTOPTR o, (j, k)))

DAOPTHVPR (j, k)	=	Max (0, DASPP k – MINRESPR j)

The above variables are defined as follows:

| Variable |  |  | Unit |  | Definition |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAOPTRAMT o, (j, k) |  |  | $ |  | Day-Ahead Option with Refund Amount per CRR Owner per pair of source and sinkThe payment to CRR Owner o for its PTP Options with Refund with the source j and the sink k, settled in the DAM, for the hour. |  |  |
| DAOPTRTP o, (j, k) |  |  | $ |  | Day-Ahead Option with Refund Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in the DAM, for the hour. |  |  |
| DAOPTRHV o, (j, k) |  |  | $ |  | Day-Ahead Option with Refund Hedge Value per CRR Owner per source and sink pair—The hedge value of CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in the DAM, for the hour. |  |  |
| DAOPTRDA o, (j, k) |  |  | $ |  | Day-Ahead Option with Refund Derated Amount per CRR Owner per source and sink pair—The derated amount of CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in the DAM, for the hour. |  |  |
| DAOPTPR (j, k) |  |  | $/MW per hour |  | Day-Ahead Option Price per pair of source and sinkThe DAM price of the PTP Option with the source j and the sink k, for the hour. |  |  |
| DASPP j |  |  | $/MWh |  | Day-Ahead Settlement Point Price at sourceThe DAM Settlement Point Price at the source Settlement Point j, for the hour. |  |  |
| DASPP k |  |  | $/MWh |  | Day-Ahead Settlement Pont Price at sinkThe DAM Settlement Point Price at the sink Settlement Point k, for the hour. |  |  |
| DAOPTR o, (j, k) |  |  | MW |  | Day-Ahead Option with Refund per CRR Owner per pair of source and sinkThe number of CRR Owner o’s PTP Options with Refund with the source j and the sink k, settled in DAM, for the hour. |  |  |
| RTOPTR o, (j, k) |  |  | MW |  | Real-Time Option with Refund per CRR Owner per pair of source and sink—The number of CRR Owner o’s PTP Options with Refund with the source j and the sink k settled in Real-Time, for the hour. |  |  |
| OPTRACT o, (j, k) |  |  | MW |  | Option with Refund Actual usage per CRR Owner per pair of source and sink—CRR Owner o’s actual usage for the PTP Options with Refund with the source j and the sink k, for the hour. |  |  |
| RESACT r |  |  | MW |  | Resource Actual per Resource per Hour—The time-weighted average of the Output Schedule of Resource r (if a valid OS exists) or the telemetered output of Resource r, for the hour. |  |  |
| OPTROF o, r |  |  | none |  | Option with Refund Ownership Factor per CRR Owner per resource—The factor showing the percentage usage of Resource r for CRR Owner o’s PTP Options with Refund. Its value is 1, if only one CRR Owner has acquired PCRRs under the refund provision using this Resource r. |  |  |
| OS r, y |  |  | MW |  | Output Schedule per resource per SCED interval—The Output Schedule submitted to ERCOT for Resource r for the SCED interval y. |  |  |
| TGFTH r |  |  | MWh |  |  | Telemetered Generation for the Hour per Resource per Hour—The telemetered generation of Generation Resource r, for the hour. |  |
| OPTRF o ,r, (j, k) |  |  | none |  | Option with Refund Factor per CRR Owner per Resource associated with pair of source and sink—The ratio of CRR Owner o’s Resource r’s capacity allocated to the PTP Options with Refund with the source j and sink k to the same CRR Owner’s total capacity for the Resource r nominated PCRRs under the refund provision with the same source j. |  |  |
| TLMP y |  |  | second |  | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |  |  |
| OPTDRPR (j, k) |  |  | $/MW per hour |  | Option Deration Price per source and sink pair—The deration price of a PTP Option with the source j and the sink k, for the hour. |  |  |
| DASP c |  |  | $/MW per hour |  | Day-Ahead Shadow Price per constraint—The DAM Shadow Price of the constraint c for the hour. |  |  |
| DRF c |  |  | none |  | Deration Factor per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |  |  |
| DAWASF j, c |  |  | none |  | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and the directional network element for constraint c, in the hour. |  |  |
| DAWASF k, c |  |  | none |  | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and the directional network element for constraint c, in the hour. |  |  |
| DAOPTHVPR (j, k) |  |  | $/MWh |  | Day-Ahead Option Hedge Value Price per pair of source and sink—The Day-Ahead hedge price of a PTP Option with the source j and the sink k, for the hour. |  |  |
| MINRESPR j |  |  | $/MWh |  | Minimum Resource Price for source—The lowest Minimum Resource Price for Resources located at the source Settlement Point j. |  |  |
| o |  |  | none |  | A CRR Owner. |  |  |
| y |  |  | none |  | A SCED interval in the hour. |  |  |
| r |  |  | none |  | A Resource. |  |  |
| j |  |  | none |  | A source Settlement Point. |  |  |
| k |  |  | none |  | A sink Settlement Point. |  |  |
| c |  |  | none |  | A constraint associated with a directional network element for the hour. |  |  |

(4)	The total payment to each NOIE CRR Owner for the Operating Hour of all its PTP Options with Refund settled in the DAM is calculated as follows:

DAOPTRAMTOTOT o	=	ΣjΣkDAOPTRAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAOPTRAMTOTOT o | $ | Day-Ahead Option with Refund Amount Owner Total per CRR Owner—The total payment to NOIE CRR Owner o for all its PTP Options with Refund settled in the DAM, for the hour. |
| DAOPTRAMT o, (j, k) | $ | Day-Ahead Option with Refund Amount per CRR Owner per pair of source and sink—The payment to NOIE CRR Owner o for the PTP Options with Refund with the source j and the sink k settled in the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

7.9.2.3	Payments for NOIE PTP Options with Refund Settled in Real-Time

(1)	Except as specified in paragraphs (2) and (3) below, ERCOT shall pay the NOIE that owns a PTP Option with Refund that was allocated to that NOIE as a PCRR and that was, declared before DAM execution by the NOIE to be settled in Real-Time but not cleared in the DAM, for the MW quantity up to the pro-rata actual usage based on the positive difference in Real-Time Settlement Point Price between the sink and the source.

(2)	The payment of PTP Options with Refund may be further reduced due to transmission elements that are oversold in previous CRR auctions.

(3)	When the DAM is not executed, ERCOT shall pay the NOIE owner of each PTP Option with Refund that was allocated to that NOIE as a PCRR, for the quantity up to the actual usage based on the positive difference in Real-Time Settlement Point Prices between the sink Settlement Point and the source Settlement Point. ERCOT shall not reduce the PTP Options with Refund payment as specified in paragraph (2) above due to transmission elements that are oversold in previous CRR auctions. The payment to each NOIE CRR Owner for a given Operating Hour of its PTP Options with Refund each pair of source and sink Settlement Points is calculated as follows:

NDRTOPTRAMT o, (j, k)	=	(-1) * NDRTOPTRTP o, (j, k)

Where:

The target payment if ERCOT is unable to execute the DAM:

NDRTOPTRTP o, (j, k)	=	RTOPTRPR (j, k) * Min (DAOPTR o, (j, k),  OPTRACT o, (j, k))

(4)	When the DAM is executed, the payment to each NOIE CRR Owner for a given Operating Hour of the PTP Options with Refund with each pair of source and sink Settlement Points settled in Real-Time is calculated as follows:

RTOPTRAMT o, (j, k)	=	(-1) * Max ((RTOPTRTP o, (j, k) – RTOPTRDA o, (j, k)), Min (RTOPTRTP o, (j, k), RTOPTRHV o, (j, k)))

Where:

The target payment:

RTOPTRTP o, (j, k)	=	RTOPTPR (j, k) * Min (RTOPTR o, (j, k),       (OPTRACT o, (j, k) * RTOPTR o, (j, k) /               (RTOPTR o, (j, k) + DAOPTR o, (j, k))))

RTOPTPR (j, k)	=	Σ4i=1Max (0, RTSPP k, i – RTSPP j, i) / 4

OPTRACT o, (j, k)	=	 ΣyΣr(OPTROF o, r * RESACT rΣy * OPTRF o, r, (j, k))

If (A valid OS r, y exists for all SCED intervals within the hour)

RESACT r	=	Σy(OS r, y* TLMP y) / (ΣyTLMPy)

Otherwise

RESACT r	=	TGFTH r

The derated amount:

RTOPTRDA o, (j, k)	=	OPTDRPR (j, k) * Min (RTOPTR o, (j, k),         (OPTRACT o, (j, k) * RTOPTR o, (j, k) /               (RTOPTR o, (j, k) + DAOPTR o, (j, k))))

OPTDRPR (j, k)	=	Σc(Max (0, DAWASF j, c – DAWASF k, c) * DASP c * DRF c)

The hedge value:

RTOPTRHV o, (j, k)	=	RTOPTHVPR (j, k) * Min (RTOPTR o, (j, k),   (OPTRACT o, (j, k) * RTOPTR o, (j, k) /               (RTOPTR o, (j, k) + DAOPTR o, (j, k))))

RTOPTHVPR (j, k)	=	Max (0, RTSPP k – MINRESPR j)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTOPTRAMT o, (j, k) | $ | Real-Time Option with Refund Amount per CRR Owner per pair of source and sink—The payment to CRR Owner o of the PTP Options with Refund with the source j and the sink k, settled in Real-Time, for the hour. |
| NDRTOPTRAMT o, (j, k) | $ | No DAM Real-Time Option with Refund Amount per CRR Owner per pair of source and sink—The payment to CRR Owner o of the PTP Options with Refund with the source j and the sink k, settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |
| RTOPTRTP o, (j, k) | $ | Real-Time Option with Refund Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in Real-Time, for the hour. |
| NDRTOPTRTP o, (j, k) | $ | No DAM Real-Time Option with Refund Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |
| RTOPTRHV o, (j, k) | $ | Real-Time Option with Refund Hedge Value per CRR Owner per source and sink pair—The hedge value of CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in Real-Time, for the hour. |
| RTOPTRDA o, (j, k) | $ | Real-Time Option with Refund Derated Amount per CRR Owner per source and sink pair—The derated amount of CRR Owner o’s PTP Options with Refund, with the source j and the sink k, settled in Real-Time, for the hour. |
| RTOPTPR (j, k) | $/MW per hour | Real-Time Option Price per pair of source and sink—The Real-Time price of the PTP Options with the source j and the sink k, for the hour. |
| RTSPP j, i | $/MWh | Real-Time Settlement Point Price at source per interval—The Real-Time Settlement Point Price at the source j for the 15-minute Settlement Interval i. |
| RTSPP k, i | $/MWh | Real-Time Settlement Point Price at sink per interval—The Real-Time Settlement Point Price at the sink k for the 15-minute Settlement Interval i. |
| OPTRACT o, (j, k) | MW | Option with Refund Actual usage per CRR Owner per pair of source and sink—CRR Owner o’s actual usage for the PTP Options with Refund with the source j and the sink k, for the hour. |
| RESACT r | MW | Resource Actual per Resource per Hour—The time-weighted average of the Output Schedule of Resource r (if a valid OS exists) or the telemetered output of Resource r, for the hour. |
| OPTROF o, r | none | Option with Refund Ownership Factor per CRR Owner per Resource—The factor showing the percentage usage of Resource r for CRR Owner o’s PTP Options with Refund. Its value is 1, if only one CRR Owner uses this Resource for PCRRs under the refund provision. |
| OS r, y | MW | Output Schedule per resource per SCED interval—The Output Schedule submitted to ERCOT for Resource r for the SCED interval y. |
| TGFTH r | MWh | Telemetered Generation for the Hour per Resource per Hour—The telemetered generation of Generation Resource r, for the hour. |
| OPTRF o ,r, (j, k) | none | Option with Refund Factor per CRR Owner per Resource associated with pair of source and sink—The ratio of CRR Owner o’s Resource r’s capacity allocated to the PTP Options with Refund with the source j and sink k to the same CRR Owner’s total capacity for the Resource r nominated for all the PCRRs under the refund provision with the same source j. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| RTOPTR (j, k) | MW | Real-Time Option with Refund per pair of source and sink—The number of the CRR Owner’s PTP Options with Refund with the source j and the sink k, settled in Real-Time, for the hour. |
| DAOPTR o, (j, k) | MW | Day-Ahead Option with Refund per CRR Owner per pair of source and sink—The number of CRR Owner o’s PTP Options with Refund settled in the DAM for the hour. |
| OPTDRPR (j, k) | $/MW per hour | Option Deration Price per source and sink pair—The deration price of a PTP Option with the source j and the sink k, for the hour. |
| DASP c | $/MW per hour | Day-Ahead Shadow Price per constraint—The DAM Shadow Price of the constraint c for the hour. |
| DRF c | none | Deration Factor per constraint— The deration factor of the constraint c for the hour, equal to the MW amount by which the constraint is oversold divided by the total MW amount of the positive impacts on the constraint of all CRRs existing prior to DAM execution. |
| DAWASF j, c | none | Day-Ahead Weighted Average Shift Factor at source per constraint—The Day-Ahead Shift Factor for the source Settlement Point and the directional network element for constraint c, in the hour. |
| DAWASF k, c | none | Day-Ahead Weighted Average Shift Factor at sink per constraint—The Day-Ahead Shift Factor for the sink Settlement Point and the directional network element for constraint c, in the hour. |
| RTOPTHVPR (j, k) | $/MWh | Real-Time Option Hedge Value Price per source and sink pair—The Real-Time hedge price of a PTP Option with the source j and the sink k, for the hour. |
| MINRESPR j | $/MWh | Minimum Resource Price for source—The lowest Minimum Resource Price for Resources located at the source Settlement Point j. |
| o | none | A CRR Owner. |
| r | none | A Resource. |
| y | none | A SCED interval in the hour. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| c | none | A constraint associated with a directional network element for the hour. |

(5)	The total payment to each NOIE CRR Owner for the Operating Hour of all its PTP Options with Refund settled in Real-Time is calculated as follows:

RTOPTRAMTOTOT o	=	ΣjΣkRTOPTRAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTOPTRAMTOTOT o | $ | Real-Time Option with Refund Amount Owner Total per CRR Owner—The total payment to NOIE CRR Owner o for all its PTP Options with Refund settled in Real-Time, for the hour. |
| RTOPTRAMT o, (j, k) | $ | Real-Time Option with Refund Amount per CRR Owner per pair of source and sink—The payment to NOIE CRR Owner o for the PTP Options with Refund with the source j and the sink k settled in Real-Time, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(6)	If ERCOT is unable to execute the DAM, the total payment to each NOIE CRR Owner for the Operating Hour of all its PTP Options with Refund settled in Real-Time is calculated as follows:

NDRTOPTRAMTOTOT o	=	ΣjΣkNDRTOPTRAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| NDRTOPTRAMTOTOT o | $ | No DAM Real-Time Option with Refund Amount Owner Total per CRR Owner—The total payment to NOIE CRR Owner o for all its PTP Options with Refund settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |
| NDRTOPTRAMT o, (j, k) | $ | No DAM Real-Time Option with Refund Amount per CRR Owner per pair of source and sink—The payment to NOIE CRR Owner o for the PTP Options with Refund with the source j and the sink k settled in Real-Time when ERCOT is unable to execute the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

7.9.2.5	Payments and Charges for PTP Obligations with Refund in Real-Time

(1)	In the event that ERCOT is unable to execute the DAM, ERCOT shall pay or charge the NOIE owner of a PTP Obligation with Refund, for the quantity up to the actual usage based on the difference in the Real-Time Settlement Point Prices between the sink Settlement Point and the source Settlement Point.  The payment or charge to each NOIE CRR Owner for a given Operating Hour of its PTP Options with Refund each pair of source and sink Settlement Points in Real-Time is calculated as follows:

NDRTOBLRAMT o, (j, k)	=	(-1) * NDRTOBLRTP o, (j, k)

Where:

The target payment:

NDRTOBLRTP o, (j, k)	=	RTOBLRPR (j, k) * Min (DAOBLR o, (j, k), OBLRACT o, (j, k))

RTOBLPR (j, k)	=	Σ4i=1(RTSPP k, i – RTSPP j, i) / 4

OBLRACT o, (j, k)	=	ΣyΣr(OBLROF o, r * RESACT r Σy * OBLRF o, r, (j, k))

If (A valid OS r, y exists for all SCED intervals within the hour)

RESACT r		=	Σy(OS r, y* TLMP y) / (ΣyTLMP y)

Otherwise

RESACT r	=	TGFTH r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| NDRTOBLRAMT o, (j, k) | $ | No DAM Real-Time Obligation with Refund Amount per CRR Owner per pair of source and sinkThe payment to CRR Owner o for the PTP Obligation with Refund with the source j and the sink k, settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour. |
| NDRTOBLRTP o, (j, k) | $ | No DAM Real-Time Obligation with Refund Target Payment per CRR Owner per source and sink pair—The target payment for CRR Owner o’s PTP Obligations with Refund, with the source j and the sink k, settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour. |
| RTOBLPR (j, k) | $/MW per hour | Real-Time Obligation Price—The Real-Time price of the PTP Obligation, for the hour. |
| RTSPP j, i | $/MWh | Real-Time Settlement Point Price at source per interval—The Real-Time Settlement Point Price at the source j for the 15-minute Settlement Interval i. |
| RTSPP k, i | $/MWh | Real-Time Settlement Point Price at sink per interval—The Real-Time Settlement Point Price at the sink k for the 15-minute Settlement Interval i. |
| DAOBLR o, (j, k) | MW | Day-Ahead Obligation with Refund per CRR Owner per pair of source and sink The number of CRR Owner o’s PTP Obligations with Refund with the source j and the sink k settled in DAM for the hour. See Section 7.9.1.5, Payments and Charges for PTP Obligations with Refund Settled in DAM. |
| OBLRACT o, (j, k) | MW | Obligation with Refund Actual usage per CRR Owner per pair of source and sink—CRR Owner o’s actual usage for the PTP Obligations with Refund with the source j and the sink k, for the hour. |
| RESACT r | MW | Resource Actual per Resource per Hour—The time-weighted average of the Output Schedule of Resource r (if a valid OS exists) or the telemetered output of Resource r, for the hour. |
| OBLROF o, r | none | Obligation with Refund Ownership Factor per CRR Owner per resource—The factor showing the percentage usage of Resource r for CRR Owner o’s PTP Obligations with Refund. Its value is 1, if only one CRR Owner has acquired PCRRs under the refund provision using this Resource r. |
| OS r, y | MW | Output Schedule per resource per SCED interval—The Output Schedule submitted to ERCOT for Resource r for the SCED interval y. |
| TGFTH r | MWh | Telemetered Generation for the Hour per Resource per Hour—The telemetered generation of Generation Resource r, for the hour. |
| OBLRF o ,r, (j, k) | none | Obligation with Refund Factor per CRR Owner per Resource—The ratio of CRR Owner o’s Resource r’s capacity allocated to the PTP Obligations with Refund with the source j and sink k to the same CRR Owner’s total capacity for the Resource r nominated for all the PCRRs under the refund provision with the same source j. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the hour. |
| o | none | A CRR Owner. |
| y | none | A SCED interval in the hour. |
| r | none | A Resource. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |

(2)	If ERCOT is unable to execute the DAM, the net total payment or charge to each CRR Owner for the Operating Hour of all its PTP Obligations with Refund settled in Real-Time is calculated as follows:

NDRTOBLRAMTOTOT o	= ΣjΣk NDRTOBLRAMT o, (j, k)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| NDRTOBLRAMTOTOT o | $ | No DAM Real-Time Obligation with Refund Amount Owner Total per CRR Owner—The net total payment or charge to CRR Owner o for all its PTP Obligations with Refund settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour. |
| NDRTOBLRAMT o, (j, k) | $ | No DAM Real-Time Obligation with Refund Amount per CRR Owner per pair of source and sinkThe payment to CRR Owner o for the PTP Obligation with Refund with the source j and the sink k, settled in Real-Time, when ERCOT is unable to execute the DAM, for the hour. |
| o | none | A CRR Owner. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |