| PRR Number | xxx | PRR Title | Use of WGRPP as Planned Operating Level in Day-Ahead Resource Plan for WGRs |
| --- | --- | --- | --- |
| Date Posted |  | May 1x, 2008 |  |
| Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms 4.4.15, QSE Resource Plans |  |
| Requested Resolution |  | Urgent - In order to incorporate the wind-powered generation forecast into Replacement Reserve Service (RPRS) studies as quickly as possible for capacity planning purposes. |  |
| Revision Description |  | This Protocol Revision Request (PRR) allows Qualified Scheduling Entities (QSEs) representing Wind-powered Generation Resources (WGR) to use an ERCOT provided Wind-powered Generation Resource Power Potential (WGRPP) forecast as the planned operating level in the Resource Plan. |  |
| Reason for Revision |  | Historically, ERCOT has not provided a wind-powered generation forecast for either the total ERCOT System or for the individual WGRs. Also, when performing RPRS studies, the planned operating level in the Resource Plans for WGRs is treated in the same manner as all other Resources. WGRs will continue to be treated in the same fashion; however, this PRR will allow ERCOT to perform Day Ahead RPRS procurement using a conservative forecast of the output from WGRs. |  |
| Overall Market Benefit |  | Allows ERCOT to have more visibility when performing capacity studies. |  |
| Overall Market Impact |  | May affect the amount of procurement of RPRS. |  |
| Consumer Impact |  | None. |  |
| Credit Implications (Yes or No, and summary of impact) |  | None. |  |
| Relevance to Nodal Market |  | Yes, the wind-powered generation forecasts are the same as those forecasts going to be produced per the Nodal Protocols. |  |
| Nodal Protocol Section(s) Requiring Revision |  | None. |  |

| Quantitative Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | None. | Not known. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | May improve reliability | Not known. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | John Dumas |
| E-mail Address | jdumas@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6998 |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Giriraj Sharma |
| E-Mail Address | gsharma@ercot.com |
| Phone Number | 512-248-6759 |

Proposed Protocol Language Revision

2.1	Definitions

Weather Zone

A geographic region in which climatological characteristics are similar for all areas within such region.

Wholesale Customers

Non-Opt-in entities receiving service at wholesale points of delivery from an LSE other than themselves.

Wind-powered Generation Resource (WGR)

A Generation Resource that is powered by wind.

Wind-powered Generation Resource Production Potential (WGRPP)

The generation in MWh per hour from a WGR that could be generated from all available units of that Resource allocated from the 80% probability of exceedance of the Total ERCOT Wind Power Forecast.

2.2	Acronyms

TEWPF	Total ERCOT Wind Power Forecast

VT	Voltage Transformer

WGR	Wind-powered Generation Resource

WGRPP	Wind-powered Generation Resource Production Potential

WSCC	Western System Coordinating Council

4.4.15 QSE Resource Plans

Each QSE that represents a Resource will present a Resource Plan to ERCOT at 1600.  These Resources may be specific Generation Resources and/or LaaRs.  The Resource Plan capacity should be sufficient to accommodate the combined quantity of energy and Ancillary Services scheduled by that QSE from the Resources that the QSE represents.  The Resource Plan shall indicate the availability of the Resources represented by the QSE, including a lead-time status code, and the planned operating level of each Resource, for each hour of the Operating Day.  The Resource Plan shall indicate the HOL and LOL, and HSL and LSL by Resource.  A Resource may be listed as unavailable to ERCOT if the Resource’s capacity has been committed to markets in regions outside of ERCOT.  ERCOT shall use other Resource Dispatch options to maintain system reliability prior to Dispatching a Generation Resource below its LOL.  ERCOT shall request Qualifying Facilities (QF), hydro units, and/or nuclear to operate below their LOL only after other Resource Dispatch options have been exhausted.

ERCOT shall produce renewable production potential forecasts for Wind-powered Generation Resources (WGRs) to be used as the planned operating level in the Resource Plan during Replacement Reserve Service procurements.  The WGR Production Potential (WGRPP) is an hourly 80% probability of exceedance forecast of energy production for each WGR.  ERCOT shall use a probabilistic Total ERCOT Wind Power Forecast (TEWPF) and select the forecast that the actual total ERCOT WGR production is expected to exceed 80% of the time (80% probability of exceedance forecast).  To produce the WGRPP, ERCOT will allocate the TEWPF 80% probability of exceedance forecast to each WGR such that the sum of the individual WGRPP forecasts equal the TEWPF forecast.  ERCOT shall produce these forecasts using information provided by WGRs to their QSEs including meteorological information, WGR power production curves and SCADA.  ERCOT shall provide forecasts for each WGR to the QSEs representing WGRs and shall deliver the forecasts before Replacement Reserve Service procurements to allow the QSEs to update the WGR Resource Plans.  QSEs shall use the ERCOT-provided forecasts for WGRs as the planned operating level for the 1600 Resource Plan and prior to running a Replacement Reserve Service market in the Adjustment Period.  The QSE may submit a lower operating level than the WGRPP forecast in the WGR Resource Plan if the WGR has communicated that it will be unavailable or operating at a reduced capability during an Operating Period which the forecast did not anticipate.  QSEs representing WGRs shall update their Resource Plans and schedules to reflect the expected wind-powered generation production after the close of the Replacement Reserve Service market.  The energy schedules submitted for WGRs should correspond with the Resource Plan scheduled energy output in order for Real Time balancing and the operator entered offset to perform properly.

QSEs shall use best efforts, consistent with Good Utility Practice, to continually update their Resource Plans to reflect the current and anticipated operating conditions of the Resources.  ERCOT will monitor the performance of QSEs with respect to the submission of accurate Resource Plans in accordance with the measures established in Section 4.10, Resource Plan Performance Metrics.  ERCOT will work with individual QSEs as necessary to improve the individual QSE performance.