| RMGRR Number |  | RMGRR Title | Out-flow Power from Distributed Renewable Generation Facilities |
| --- | --- | --- | --- |
| Date Posted |  |  |  |
| Retail Market Guide (RMG) Section(s) Requiring Revision (Include Section No. and Title) |  | New Section: 7.14 Out-flow Power from Distributed Renewable Generation Facilities |  |
| Protocol Section(s) Requiring Revision, if any |  | See PRR756 (approved by ERCOT Board on 5-20-08) |  |
| Requested Resolution (Normal or Urgent, and justification for Urgent status) |  | Normal |  |
| Revision Description |  | The new section describes the process by which owners of distributed renewable generation (DRG) may receive credit (via their REP’s QSE) for out-flow power delivered to the grid. Most importantly, this section describes the metering that is required and how the monthly out-flow generation value will be transported on the TX SET Monthly Usage and Historical Usage transactions without changing the TX SET Implementation Guides i.e. no “TX SET change”. In addition, this section will also broadly describe the settlement processes that will be used to account for out-flow power. These settlement processes are detailed in PRR756 and LPGRR030. |  |
| Reason for Revision |  | HB 3693, PURA 39.914 and 39.916, PUCT Subst. Rule §25.213 |  |
| Overall Market Benefit |  | Compliance with Rules and Law |  |
| Overall Market Impact |  | See PRR756 Cost Benefit Analysis |  |
| Consumer Impact |  | Benefits DRG owners who want to sell out-flow power |  |

Quantitative Impacts and Benefits

Instructions:  To allow for comprehensive RMGRR consideration and development of the Cost Benefit Analysis (CBA), please fill out each block below completely and provide as much detailed information as possible.  Wherever possible, please include reasons, explanations, and cost/benefit analyses pertaining to this RMGRR.

| Assumptions | 1 | All impacts are captured in the CBA for PRR756. |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Rob Bevill on behalf of TX SET |
| E-mail Address | Rob.Bevill@greenmountain.com |
| Company | Green Mountain Energy Company |
| Phone Number | 512-691-6134 |
| Cell Number | 512-657-0237 |
| Market Segment | Independent REP |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | JLevine@ercot.com |
| Phone Number | 512-248-6464 |

Proposed Guide Language Revision

  - Out-flow Power from Distributed Renewable Generation Facilities

Beginning on January 1, 2009, customers who have on-site distributed renewable generation facilities with a capacity of less than 50 kW, as defined in PURA §39.904, may sell out-flow power that is delivered onto the distribution network.  REPs (via their QSEs) can receive a wholesale settlement adjustment for this out-flow energy, according to the process and requirements outlined below.

    - Primary Requirements for Receiving a Settlement Adjustment for Out-flow Power

In order for ERCOT to provide the wholesale settlement adjustment for out-flow power; the following requirements must be met;

      - A current and valid Interconnection Agreement must be in place with the TDSP, as described in PUCT Sub. Rules 25.211 and 25.212.

      - Customer requested metering that measures and reports separately consumption from the distribution network and out-flow from the customer’s side of the meter to the distribution network

      - The ESI ID must be assigned to a distributed renewable generation load profile (see the ERCOT Load Profiling Guide for further information on load profile requirements)

7.14.5        TDSP Communication to ERCOT and the REP of Record of technical information from Distributed Renewable Generation Interconnection Agreements.

In order to assign a distributed renewable generation load profile, ERCOT requires that the information below from the interconnection agreement between the TDSP and the retail customer be provided.  Once the TDSP establishes an interconnection agreement with a distributive renewable generation owner the TDSP shall complete the spreadsheet in Appendix J** and provide the spreadsheet to ERCOT and to the REP Relations Primary and Secondary contacts of the REP of Record within ten Retail Business Days of the effective date of the interconnection agreement.

| Affirmation from the TDSP that customer has signed an approved Interconnection Agreement with the TDSP |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information as requested on the ERCOT approved form to include: |  |  |  |  |  |  |  |  |  |  |
| a. | ESI ID |  |  |  |  |  |  |  |  |  |
| b. | Generation type, e.g. PV, wind, etc. |  |  |  |  |  |  |  |  |  |
| c. | Interconnection Agreement effective date |  |  |  |  |  |  |  |  |  |
| d. | Meter with kWh gen and kWh load measurement capability installed? (Y/N) |  |  |  |  |  |  |  |  |  |
| e. | Current Profile ID |  |  |  |  |  |  |  |  |  |
| f. | Total inverter capacity (if applicable and available) |  |  |  |  |  |  |  |  |  |
| g. | The inverter's published peak efficiency rating (if applicable and available) |  |  |  |  |  |  |  |  |  |
| h. | If PV renewable generation is present the following information will be required: |  |  |  |  |  |  |  |  |  |
|  | I. | Number of modules |  |  |  |  |  |  |  |  |
|  | II. | Size of modules (watts) |  |  |  |  |  |  |  |  |
|  | III. | Total PV generation capacity (DC) |  |  |  |  |  |  |  |  |
| i. | If wind renewable generation is present the following information will be required: |  |  |  |  |  |  |  |  |  |
|  | I. | Number of turbines |  |  |  |  |  |  |  |  |
|  | II. | Size of turbine (watts) |  |  |  |  |  |  |  |  |
|  | III. | Total wind generation capacity (kW) |  |  |  |  |  |  |  |  |
| j. | If renewable generation other than PV or wind is present the following information will be required: |  |  |  |  |  |  |  |  |  |
|  | I. | Number and type of units |  |  |  |  |  |  |  |  |
|  | II. | Size of units (watts) |  |  |  |  |  |  |  |  |
|  | III. | Total generation capacity (kW) |  |  |  |  |  |  |  |  |

**The information communicated in Appendix XX should be considered Proprietary Customer Information

    - Metering Required for Measurement and Settlement of Out-flow Power

In order for out-flow to be measured, the premise must have metering that measures in-flow and out-flow of electricity separately at the point of common coupling.  Customers choosing to have their out-flow measured shall contact their TDSP to request the necessary metering if they have not already done so in conjunction with their interconnection activities.  TDSP charges may apply for the cost of the metering.  See PUCT Subst. Rule 25.213, Metering for Distributed Renewable Generation, for further details.

    - Transmittal of Out-flow Power Meter Data

The generation value (kWh) will be transmitted on the 867_03 Monthly Usage and the 867_02 Historical Usage transactions in the REF~JH~I segment (REF = Meter Role, PTD = Non-Interval Detail).  As of January 1, 2009, the REF~JH~I (REF = Meter Role, PTD = Non-Interval Detail) segment is dedicated for out-flow values only.  For instances where there has been no generation, the segment should either be omitted or included and populated with zero (“0”).  In the absence of a meter that measures out-flow, the REF~JH~I shall not be included on the TX SET 867_02 or 867_03 transactions.

7.14.4             ERCOT Processing of Meter Data for Out-flow Power

ERCOT will process out-flow values for settlement received for any ESI ID, provided that the ESI ID is also assigned to a distributed renewable generation load profile.  Any Monthly Usage transaction received by ERCOT that contains a value for out-flow in the REF~JH~I for an ESI ID that is not assigned to a distributed renewable generation load profile will be processed but the out-flow value will be ignored by ERCOT for settlement purposes.

For a detailed description of the wholesale settlement impact of out-flow values, see Protocol sections 11.4.4.2 Load Reduction for Excess Photovoltaic Generation and 11.4.4.3 Load Reduction for Excess Non-Photovoltaic Generation.