| NPRR Number | 107 | NPRR Title | Nodal Emergency Interruptible Load Service (EILS) |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Recommended Action |  | Approval |
| Date of Decision |  | May 22, 2008 |  |  |
| Protocol Sections Requiring Revision |  | Section 2, Definitions and Acronyms Section 3.14, Contracts for Reliability Resources Section 3.14.3, Emergency Interruptible Load Service (EILS) (New) Section 6.5.8.1, Deployment of Emergency Interruptible Load Service (EILS) (New) Section 6.5.9, Emergency Operations Section 6.5.9.4, Emergency Electric Curtailment Plan Section 6.5.9.7, Emergency Interruptible Load Service (EILS) (New) Section 6.6.11, Emergency Interruptible Load Service (EILS) Settlement (New) Section 8.1.3, Emergency Interruptible Load Service (EILS) Performance Criteria and Testing Requirements (New) Section 9.14.5, Resettlement of Emergency Interruptible Load Service (New) (subsequent subsections in Section 9.14 renumbered) Section 16.13, Registration of Emergency Interruptible Load Service (EILS) Resources (New) Section 22 -- Attachment 22K, Standard Form Emergency Interruptible Load Service Agreement (Reformatted) |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) modifies the ERCOT Nodal Protocols to transition the Emergency Interruptible Load Service (EILS) to the Nodal Market. EILS is deployed during an Emergency Electric Curtailment Plan (EECP), after all available Generation and Load Resources have been deployed. In various cases, certain language has been modified slightly to enhance clarity. Changes to EILS deployment are made to be consistent with the split deployment of Load Resources in Step 2 of EECP, by grouping the EILS Loads into two approximately equal groups if the total amount of available EILS equals or exceeds 500 MW. This split is necessary to support Security Constrained Economic Dispatch (SCED) deployments every 5 minutes. |  |  |
| Overall Market Benefit |  | Ensures continuing ERCOT compliance with PUCT Substantive Rules. |  |  |
| Overall Market Impact |  | Up to $50 million per year may be uplifted to Qualified Scheduling Entities (QSEs) representing Load. Pursuant to PUC Subst. R. 25.507, EILS costs are capped at $50 million per year. |  |  |
| Consumer Impact |  | The impact to Consumers depends upon duration and terms of the electric supply contracts and the degree to which Load Serving Entities (LSEs) pass uplifted costs onto the Consumers. |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - NPRR107 was posted on 2/29/08. - On 3/20/08, PRS referred NPRR107 to the Transition Plan Task Force (TPTF). - On 4/16/08, ERCOT Staff comments were posted. - On 5/9/08, TPTF comments were posted. - On 5/22/08, PRS considered this NPRR. |  |  |
| PRS Decision |  | On 3/20/08, PRS unanimously voted to refer NPRR107 to the TPTF for review. All Market Segments were present for the vote. On 5/22/08, PRS voted to recommend approval of NPRR107 as revised by TPTF comments. The motion passed with one opposing vote from the Independent Power Marketer (IPM) Market Segment and four abstentions from the Municipally Owned Utility (MOU), Investor Owned Utility (IOU), Independent Generator (IG), IPM, and Independent Retail Electric Provider (IREP) Market Segments. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 3/20/08, PRS noted that TPTF had not reviewed NPRR107. On 5/22/08, ERCOT Staff explained that this NPRR is intended to carry the EILS program forward in to the Nodal Market. Some participants opined that taking away capacity payment is not a sufficient deterrent for under-performance and that the penalty is not comparable to the penalties faced by Generation Resources. |  |  |

| ERCOT/Market Segment Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 | Firm Load shed is undesirable due to societal costs, including business losses. |  |
|  | 2 | EILS reduces the probability of needing to shed firm load. |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | Potential Settlement Obligation to QSEs on a Load Ratio Share basis will increase. | Cost of service– depends on bids received. Cost Cap of program increased from $20 million to $50 million. |
|  | 2 | Implementation costs for QSEs offering EILS (legal, technology, communications, etc.) | EILS Resource Entities will need to invest in equipment to enable them respond as required to provide service. QSEs wishing to represent EILS Resources will need to invest in systems to allow communications with such Resources. |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Reduced likelihood of firm load shedding. | Difficult to quantify. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | Stated goal of the Commission is for this program to attract more demand response from all market segments, which is considered an important feature of the energy-only market design. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 | For the foreseeable future, this Service will continue to be administered manually by ERCOT staff. No capital project is currently required or contemplated. However, as the service matures, ERCOT Staff will continuously review its internal processes and procedures for administering EILS with an eye toward potential system changes that could improve efficiency and reduce costs. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT Staff 041608 | Offered revisions to clarify a Settlement and compliance provision relating to availability requirements for EILS Resources assigned to the alternate baseline. |
| TPTF 050908 | Offered revisions to conform the EILS language in the zonal Protocols to the nomenclature used in the Nodal Protocols. |

| Original Sponsor |  |
| --- | --- |
| Name | Paul Wattles |
| Company | ERCOT |
| Market Segment | n/a |

Proposed Protocol Language Revision

Emergency Interruptible Load Service (EILS)

A special emergency service consistent with subsection (a) of PUCT Subst. Rule §25.507 used during an EECP  to reduce Load and assist in maintaining or restoring ERCOT System frequency EILS is not an Ancillary Service.

EILS Contract Period

A time frame during which ERCOT may procure EILS in an amount no greater than 1000 MW.

EILS Time Period

Blocks of hours in an EILS Contract Period in which EILS Loads are contractually committed to provide EILS.  EILS Time Periods are specific to an EILS Contract Period and shall be defined by ERCOT in the Request for Proposal for that EILS Contract Period.

EILS Self-Provision

The act by a QSE to meet its Load Ratio Share (LRS) of the total EILS procurement by designating Load to act as an EILS Load.   A QSE self-providing EILS shall submit EILS Offers at a price of zero dollars.

EILS Load

A Load or aggregation of Loads that is contracted to provide EILS.

2.2 Acronyms

EILS	Emergency Interruptible Load Service

[…]

3.14	Contracts for Reliability Resources

ERCOT shall procure Resources that are used to provide Reliability Must-Run Service, Black Start Service, and Emergency Interruptible Load Service (EILS) through Agreements.

3.14.3	Emergency Interruptible Load Service (EILS)

(1)	ERCOT shall procure EILS for EILS Contract Periods.  The standing EILS Contract Periods are as follows:

(a) 	June through September;

(b) 	October through January; and

(c) 	February through May.

(2)	ERCOT may restructure EILS Contract Periods in order to facilitate additional Load participation in EILS.  ERCOT shall provide Notice of any changes to the standing EILS Contract Periods no less than 90 days prior to the start date of that EILS Contract Period.

(3)	ERCOT will request EILS Offers prior to each EILS Contract Period.  ERCOT may procure additional EILS at any time.

(4)	EILS Offers may be submitted to ERCOT only by Qualified Scheduling Entities (QSEs) capable of receiving Verbal Dispatch Instructions (VDIs) on behalf of represented EILS Loads.  A QSE may submit offers on behalf of multiple EILS Loads for any EILS Contract Period.

(5)	ERCOT shall solicit EILS Offers.  QSEs on behalf of EILS Load may submit offers for one or more EILS Time Periods within a Contract Period.  An EILS Offer is specific to a Time Period.  In submitting a offer, a QSE and the EILS Load are committing to provide EILS for that Time Period if selected.

(6)	The minimum amount of EILS that may be offered in an offer to ERCOT is one megawatt (MW).  EILS Loads may be aggregated to reach the one MW offer requirement.

(7)	An offer to provide EILS shall include:

(a)	The name of the QSE representing the EILS Load;

(b)	The name of the Entity supplying the EILS Loads;

(c)	A description of the Load(s) which will provide EILS if selected, including name(s) and ESI ID(s);

(d)	The EILS Time Period for which the offer is submitted;

(e)	A dollars per MW price for the capacity offer unless the offer is for EILS Self-Provision;

(f)	The quantity of capacity for which the offer price is effective, expressed in whole number MWs;

(g)	The minimum base Load, in MW, for each ESI ID in the EILS Load, defined as that level of Load the EILS Load is unwilling to interrupt in an EILS deployment event;

(h)	For EILS Loads that are not metered by a dedicated ESI ID in a competitive choice area of the ERCOT Region, including those situated in territories served by Non-Opt-In Entities (NOIEs) or within private use networks, the most recently available 12 months of IDR data in a format specified by ERCOT;

(i)	QSEs opting for EILS Self-Provision must provide ERCOT with the maximum amount of capacity they plan to provide through this option before ERCOT begins to accept EILS Offers;

(j)	A QSE opting for EILS Self-Provision may also offer separate capacity into EILS in the form of a priced offer in the same manner as any other QSE.

(k)	Affirmation that the capacity being offered into EILS is not capacity that is separately obligated to respond during an EECP event, and receiving a separate reservation payment for such obligation, occurring in the contracted EILS Time Period.

(8)	QSEs may self-provide EILS.  Self-providing QSEs must adhere to the following steps for offering EILS in an EILS Contract Period:

(a)	A QSE electing to self-provide part or all of its EILS obligation for an EILS Contract Period shall provide ERCOT with the following, while adhering to a schedule published by ERCOT:

(i)	The maximum MW of Capacity it is willing to offer through EILS Self-Provision, per Time Period;

(ii)	A Proxy Load Ratio Share specific to the Time Period.  “Proxy Load Ratio Share” shall be a number between zero and one and determined by the self-providing QSE to represent its estimate of its final Load Ratio Share (LRS) to be used in EILS Settlement.

(b)	After receiving EILS Self-Provision information, ERCOT will award offers for additional MWs of EILS capacity such that EILS capacity procured through EILS Offers and the combined maximum MW capacity for EILS Self-Provision do not exceed 1000 MW.

(c)	If the total amount of EILS capacity procured through offers and EILS Self-Provision equals 1000 MW, a QSE shall not change its EILS Self-Provision capacity obligation.

(d)	If the total amount of EILS capacity procured through offers and EILS Self-Provision is less than 1000 MW, ERCOT shall provide QSEs offering EILS Self-Provision their adjusted estimated obligation based on their Proxy Load Ratio Shares.  A QSE may then reduce its EILS Self-Provision capacity to a number no lower than the lowest number represented in the following three options:

(i)	Option 1 -- The capacity of MW procured by ERCOT through offers divided by one (1) minus the Sum of EILS Self-Provision Proxy Load Ratio Shares multiplied by the QSE’s Proxy Load Ratio Share, as expressed in the following formula:

(Total_OFFERProcqc(tp) / (1-∑ProxSPLRSqc(tp)))*ProxSPLRSqc(tp)

| Variable | Unit | Description |
| --- | --- | --- |
| q | None | QSE |
| c | None | EILS Contract Period |
| tp | None | Hours in an EILS Time Period |
| Total_OFFERProcqc(tp) | MW | Total contracted capacity for an EILS Load for the EILS Time Period |
| ProxSPLRSqc(tp) | % | The value reported by each QSE of its estimated Load Ratio Share for EILS for the EILS Time Period |

- Option 2 -- The sum of the capacity procured by ERCOT and the capacity self-provided multiplied by the QSE’s Proxy Load Ratio Share, as expressed in the following formula:

(Total_OFFERProcqc(tp) + ∑OfferedSPqc(tp))*ProxSPLRSqc(tp)

| Variable | Unit | Description |
| --- | --- | --- |
| Q | None | QSE |
| C | None | EILS Contract Period |
| Tp | None | Hours in an EILS Time Period |
| Total_OFFERProcqc(tp) | MW | Total contracted capacity for an EILS Load for the EILS Time Period |
| OfferedSPqc(tp) | MW | The capacity offered by a QSE for Self-Provision for the EILS Time Period, as communicated to ERCOT prior to ERCOT procuring competitive EILS Offers. |
| ProxSPLRSqc(tp) | % | The value reported by each QSE of its estimated Load Ratio Share for EILS for the EILS Time Period |

(iii)	Option 3 -- The QSE’s declared maximum MW offer self-provided capacity.

(e)	A QSE with reduced EILS Self-Provision capacity may reduce the commitment(s) of specific EILS Loads by providing notification to ERCOT.  Such notification must be received by ERCOT within two Business Days following ERCOT’s notification to the QSE of its reduced obligation.

(f)	If a QSE reduces its EILS commitment according to these procedures, it will not be obligated to pay EILS charges so long as the amount of its Self-Provision capacity remains equal to or greater than its final LRS of the total EILS capacity procured through offers and EILS Self-Provision, as described in Section 6.6.11, Emergency Interruptible Load Service Capacity Payments and Settlement, item (1), and so long as all self-provided EILS Loads meet their availability and performance obligations as described in Section 8.1.3.1, Performance Criteria for EILS Loads.

(9)	ERCOT shall not procure more than $50 million of EILS in any 12 month period beginning on February 1st and ending on January 31st (“EILS Cap”).  ERCOT may determine cost limits for each EILS Contract Period in order to ensure that the EILS Cap is not exceeded.  In order to minimize the cost of EILS, ERCOT may reject any offer it determines to be unreasonable or outside the parameters of an acceptable offer.  ERCOT shall establish a written process for determining the cost limits for each EILS Contract Period and for the reasonableness of offers.

(10)	ERCOT shall reduce the EILS Cap by the value of the amount of EILS Self-Provision.  ERCOT shall value EILS Self-Provision at the weighted average cost per MW of the EILS procured multiplied by the total MW of EILS Self-Provision during each relevant EILS Time Period and EILS Contract Period.

(11)	The maximum amount of EILS for which ERCOT may contract in an EILS Contract Period is 1,000 MW for each EILS Time Period.

(12)	ERCOT shall evaluate each offer to determine the actual capacity an EILS Load is capable of providing and may limit any award to that EILS Load based on the results of the evaluation.

(13)	ERCOT shall select EILS Loads for each EILS Time Period to serve during an EILS Contract Period based upon least cost offer per MW of capacity offer based upon the payment as offer for selected EILS Loads; provided that ERCOT may consider geographic location and its affect on zonal or local Congestion in selecting EILS Load.  ERCOT may prorate awards when there are more MWs available at a given price than ERCOT can procure, if acceptable to the offering QSE.  An EILS Offer may declare a minimum amount of MW that the EILS Load is willing to provide and, if pro-ration would result in an award below that amount, the offer will be excluded from the EILS procured.

(14)	QSEs representing selected EILS Loads, except for Load designated for EILS Self-Provision, will be entitled to payment as offered, subject to adjustment, pursuant to the Protocols. Deployment of EILS Loads will not result in additional payments other than any Load Imbalance payments received.

(15)	QSEs representing EILS Loads selected to provide EILS shall execute a Standard Form EILS Agreement, as provided in Section 22K, Standard Form Emergency Interruptible Load Service Agreement, for each committed EILS Time Period and EILS Contract Period.

(16)	An EILS Load shall be subject to a maximum of two Dispatch Instructions per EILS Contract Period.  Additionally, an EILS Load shall be subject to a maximum of eight hours of Dispatch Instructions per EILS Contract Period, unless an EILS deployment is still in effect when the eighth hour lapses (in which case the EILS Load must follow the Dispatch Instruction until ERCOT releases the EILS Load).

(17)	Unless ERCOT has received a notice of unavailability in a format prescribed by ERCOT, ERCOT shall assume that such a contracted EILS Load is fully available for receiving Dispatch Instructions.

(18)	EILS Loads shall meet the following technical requirements:

(a)	Each EILS Load must have an ESI ID or other unique service identifier, as defined by ERCOT.  Each individual EILS Load must have an installed Interval Data Recorder (IDR) meter or equivalent, subject to ERCOT approval, dedicated to the Load providing EILS.  ERCOT shall analyze 15-minute interval meter data for each EILS Load for purposes of offer analysis, availability and performance measurement.  EILS Loads behind a NOIE meter point shall arrange, preferably with the NOIE Transmission Service Provider, to provide ERCOT with 15-minute interval meter data subject to ERCOT’s specifications and approval.  EILS Loads behind a Private Use Network’s settlement meter point shall provide ERCOT 15-minute interval meter data subject to ERCOT’s specifications and approval.  Notwithstanding the aforementioned, ERCOT may accept offers or self-provision offers from QSEs representing non-IDR metered aggregated EILS Loads if the QSE submits and ERCOT approves a statistically valid alternative to universal IDR metering for measurement and verification consistent with industry best practices.  ERCOT shall publish guidelines and schedules for submittal of such alternatives.

(b)	An EILS Load must be capable of reducing its Load by its contracted capacity relevant to its assigned baseline methodology within ten minutes of an ERCOT Dispatch Instruction to its QSE and must be able to maintain such reduced capacity level for the entire period of the Dispatch Instruction and shall not return to normal operations until released to do so by ERCOT.  The ERCOT EILS Dispatch Instruction to a NOIE opting for EILS Self-Provision which is also dynamically scheduling shall be considered an instructed deviation so the NOIE is not penalized for keeping Generation Resources On-line.

(c)	Any QSE representing an EILS Load must be capable of communicating with its EILS Loads within the prescribed time constraints for deployment of EILS.

(d)	Committed EILS Loads are responsible for communicating any material changes in availability status to the QSE representing the EILS Load and to ERCOT, irrespective of whether the change in availability is scheduled with ERCOT as described in paragraph (5) of Section 8.1.3.1, Performance Criteria for EILS Loads.

(e)	EILS Loads deployed for EILS must be able to return to their contracted operating level for providing EILS for any committed hours within ten hours following a release Dispatch Instruction.

(f)	EILS Loads and their QSEs are subject to qualification based on ERCOT’s evaluation of their historic meter data and, if applicable, their historic performance in providing other comparable ERCOT services.  EILS Loads and their QSEs are subject to performance and testing requirements as described in Section 8.1.3, EILS Performance Criteria and Testing Requirements.

(g)	EILS Loads are not subject to the modeling, telemetry and Resource Plan requirements of other Resources.

(19)	The contracted capacity of EILS Loads may not be used to provide Ancillary Services during the contracted EILS Time Period of the contracted EILS Contract Period.    Nothing herein shall be construed to limit passive (voluntary) load response, provided the EILS performance requirements as described in Section 8.1.3.1, EILS Performance Criteria and Testing Requirements, are met.

(20)	ERCOT will review the effectiveness and benefits of the EILS every12 months from the start of the program and report its findings to TAC.

(21)	Within ten days of the  announcement of awards for EILS foran upcoming Contract Period, ERCOT shall post on the MIS PublicArea   the number of MW procured per EILS Time Period, the number of EILS Loads selected and the projected total cost of EILS for that Contract Period.

6.5.9	Emergency Operations

(1)	ERCOT, based on ERCOT System reliability needs, may issue a Dispatch Instruction requiring a Resource to move to a specific output level (“Emergency Base Point”).

(2)	A Qualifying Facility (QF) may only be ordered Off-Line in the case of an ERCOT-declared Emergency Condition with imminent threat to the reliability of the ERCOT System.  ERCOT may only Dispatch a QF below its LSL when ERCOT has declared an Emergency Condition and the QF is the only Resource that can provide the necessary relief.

(3)	ERCOT shall honor all Resource operating parameters in Dispatch Instructions under normal conditions and Emergency Conditions.  During Emergency Conditions, ERCOT may verbally request QSEs to operate its Resources outside normal operating parameters.  If such request is received by a QSE, the QSE shall discuss the request with ERCOT in good faith and may choose to comply with the request.

(4)	A QSE may not self-arrange for Ancillary Services procured in response to Emergency Conditions.

6.5.9.4	Emergency Electric Curtailment Plan

(1)	At times it may be necessary to reduce ERCOT System Demand because of a temporary decrease in available electricity supply.  To provide orderly, predetermined procedures for curtailing Demand during such emergencies, ERCOT shall initiate and coordinate the implementation of the Emergency Electric Curtailment Plan (EECP) following the steps set forth below in Section 6.5.9.4.2, EECP Steps.

(2)	The goal of the EECP is to provide for maximum possible continuity of service while maintaining the integrity of the ERCOT System to reduce the chance of cascading Outages.

(3)	ERCOT’s operating procedures must meet the following goals:

(a)	Use of market processes to the fullest extent practicable without jeopardizing the reliability of the ERCOT System;

(b)	Use of Responsive Reserve Services, other Ancillary Services, and Emergency Interruptible Load Service (EILS) to the extent permitted by ERCOT System conditions;

(c)	Maximum use of ERCOT System capability;

(d)	Maintenance of station service for nuclear-powered Generation Resources;

(e)	Securing startup power for Generation Resources;

(f)	Operation of Generation Resources during loss of communication with ERCOT; and

(g)	Restoration of service to Loads in the manner defined in the Operating Guides.

(4)	ERCOT is responsible for coordinating with QSEs, TSPs, and DSPs to monitor ERCOT System conditions, initiating the EECP steps, notifying all QSEs, and coordinating the implementation of the EECP steps while maintaining transmission security limits.

(5)	ERCOT, at management’s discretion, may at any time issue an appeal through the public news media for voluntary energy conservation.

(6)	During the EECP, ERCOT has the authority to obtain energy from non-ERCOT Control Areas using the DC Ties or by using Block Load Transfers (BLTs) to move load to non-ERCOT Control Areas.  ERCOT maintains the authority to curtail energy schedules flowing into or out of the ERCOT System across the DC Ties in accordance with NERC scheduling guidelines.

(7)	Some of the EECP steps are not applicable if transmission security violations exist. There may be insufficient time to implement all EECP steps in sequence, however, to the extent practicable, ERCOT shall use Ancillary Services that QSEs have made available in the market to maintain or restore reliability.

(8)	ERCOT may immediately implement Step 4 of the EECP any time the steady-state system frequency is below 59.8 Hz and shall immediately implement Step 4 any time the steady-state frequency is below 59.5 Hz.

(9)	Percentages for Step 4 Load shedding will be based on the previous year’s TSP peak Loads, as reported to ERCOT, and must be reviewed by ERCOT and modified annually as required.

6.5.9.4.1	General Procedures Prior to EECP Operations

(1)	Prior to declaring EECP Step 1 detailed in Section 6.5.9.4.2, EECP Steps, ERCOT may perform the following operations consistent with Good Utility Practice:

(a)	Provide Dispatch Instructions to QSEs for specific Resources to operate at an Emergency Base Point to maximize Resource deployment so as to increase Responsive Reserve levels on other Resources;

(b)	Commit available Resources as necessary that can respond in the timeframe of the emergency.  Such commitments will be settled using the HRUC process;

(c)	Start RMR units available in the time frame of the emergency.  RMR units should be loaded to full capability;

(d)	Issue Dispatch Instructions to QSEs to suspend any ongoing ERCOT required generating unit or Resource performance testing;

(e)	Utilize available resources providing Non-Spin Reserve Service as required; and

(f)	ERCOT shall use the PRC to determine the appropriate Emergency Notification and EECP Steps.

(2)	In addition, ERCOT may issue an appeal through the public news media for voluntary load reduction if authorized by the ERCOT Chief Executive Officer or its designee based on an evaluation of existing and anticipated system conditions.

6.5.9.4.2	EECP Steps

(1)	Step 1 - Maintain a sum total of 2,300 MW that results from adding the amount of ERCOT Physical Responsive Capability (PRC) MW (Section 6.5.7.5, Ancillary Services Capacity Monitor) and the amount of RRS MW which is supplied from Load Resources.

(a)	ERCOT will:

(i)	Notify the Southwest Power Pool Security Coordinator;

(ii)	Initiate manual HRUC Dispatch Instructions to Generation Resources available and off-line that can perform within the expected timeframe of the emergency;

- Use available DC Tie import capacity not already being used and inquire about availability of BLTs;

(b)	QSEs will notify ERCOT of any Resources uncommitted but available in the timeframe of the emergency.

(2)	Step 2 - Maintain a sum total of 1,750 MW that results from adding the amount of ERCOT Physical Responsive Capability (PRC) MW (Section 6.5.7.5) and the amount of RRS MW which is supplied from Load Resources.

(a)	In addition to the measures associated with Step 1, ERCOT:

(i)	Will instruct TSPs and DSPs to reduce Customers load by using distribution voltage reduction measures, if deemed beneficial by the TSP or DSP;

(ii)	Will instruct QSEs to deploy Responsive Reserve that is supplied from Load Resources (controlled by high-set under-frequency relays) in accordance with the following:

(A)	Instruct QSEs to deploy half of the Responsive Reserve that is supplied from Load Resources (controlled by high-set under-frequency relays) by instructing the QSE representing the specific Load Resource to interrupt Group 1 Load Resources providing Responsive Reserve.  QSEs shall deploy Load Resources according to the group designation and will be given some discretion to deploy additional Load Resources from Group 2 if Load Resource operational considerations require such.  ERCOT shall issue notification of the deployment via XML message.  ERCOT shall follow this XML notification with a Hotline VDI, which shall initiate the ten-minute deployment period.

(B)	At the discretion of the ERCOT Operator, instruct QSEs to deploy the remaining Responsive Reserve that is supplied from Load Resources (controlled by high-set under-frequency relays) by instructing the QSE representing the specific Load Resource to interrupt Group 2 Load Resources providing Responsive Reserve. ERCOT shall issue notification of the deployment via XML message.  ERCOT shall follow this XML notification with a Hotline VDI, which shall initiate the ten-minute deployment period.

(C)	The ERCOT Operator may deploy both of the groups of Load Resources providing Responsive Reserves at the same time.  ERCOT shall issue notification of the deployment via XML message.  ERCOT shall follow this XML notification with a Hotline VDI, which shall initiate the ten-minute deployment period; and

(D)	ERCOT shall post a list of Load Resources on the MIS Certified Area immediately following the DRUC for each QSE with a Load Resource obligation which may be deployed to interrupt under paragraph (A), Group 1 and paragraph (B), Group 2.  ERCOT shall develop a process for determining which individual Load Resource to place in Group 1 and which to place in Group 2.  ERCOT Procedures shall select Group 1 and Group 2 based on a random sampling of individual Load Resources.  At ERCOT’s discretion, ERCOT may deploy all Load Resources at any given time during EECP Step 2.

(iii)	With the approval of the affected Non-ERCOT control area, may instruct TSPs or DSPs to implement BLTs, which transfer load from the ERCOT control area to Non-ERCOT control areas.  Use of a BLT will be defined in the ERCOT Operating Guides.

(iv)	Unless such a media appeal is already in effect, shall issue an appeal through the public news media for voluntary energy conservation.

(b)	Confidentiality requirements reagarding  transmission operations and system capacity information will be lifted, as needed to  restore reliability.

(3)	Step 3 – Maintain system frequency at 60 Hz.  Following deployment of the measures associated with Steps 1 and 2, ERCOT will deploy  available contracted EILS Loads, via a single Verbal Dispatch Instruction (VDI) to the all-QSE hotline; as follows:

(a)	If less than 500 MW of EILS is available for deployment, ERCOT shall deploy all EILS Loads as a single block.

(b)	If the amount of EILS available for deployment equals or exceeds 500 MW, ERCOT may deploy EILS Loads as a single block or may deploy EILS Loads sequentially in two groups of approximately equal size as designated by ERCOT.  For a sequential group deployment, ERCOT shall instruct QSEs to deploy Group 1 immediately and to deploy Group 2 at a specified time in the future.    ERCOT shall develop a process for determining which individual EILS Loads to place in Group 1 and which to place in Group 2.  ERCOT Procedures shall select Group 1 and Group 2 based on a random sampling of individual EILS Loads.

(c)	QSEs shall instruct the EILS Loads to curtail Load consistent with their commitments.

(d)	EILS may be deployed at any time in a Settlement Interval.

(e)	Once ERCOT has deployed EILS, EILS Loads shall remain reduced until ERCOT specifically releases the EILS deployment via a VDI to the all-QSE hotline.

(f)	Unless scheduled to go off-line, due either to an EILS Time Period transition or a previously scheduled period of unavailability, an EILS Load deployed for EILS shall return to its committed operating level as soon as practical following an ERCOT recall.  All EILS Load shall return to normal within ten hours of being recalled.

(4)	Step 4 - Maintain system frequency at 59.8 Hz or greater.

(a)	In addition to measures associated with Steps 1, 2 and 3, ERCOT will direct all TSPs and DSPs or their agents to shed firm load, in 100 MW blocks, as documented in the ERCOT Operating Guides in order to maintain a steady state system frequency of 59.8 Hz.

(b)	In addition to measures associated with Steps 1, 2 and 3, TSPs and DSPs will keep in mind the need to protect the safety and health of the community and the essential human needs of the citizens.  Whenever possible, TSPs and DSPs shall not manually drop load connected to under-frequency relays during the implementation of the Emergency Electric Curtailment Plan.

6.5.9.4.2	Restoration of Market Operations

ERCOT shall continue EECP until sufficient offers are received and deployed by ERCOT to eliminate the conditions requiring the EECP and normal SCED operations are restored.  ERCOT shall release EILS Loads after the restoration of Responsive Reserves .  Intermittent solutions of SCED do not set new LMPs until ERCOT declares that the EECP is no longer needed.

[ repeat of prior sections]

6.6.11	Emergency Interruptible Load Service (EILS) Capacity

6.6.11.1  EILS Capacity Payments

(1)	ERCOT shall pay for each EILS Contract Period, the QSEs representing EILS Loads as follows:

EILqce(tp) = -1 * OFFERPriceqce(tp) * OFFERValueqce(tp) * AvailFactorqce(tp)* EILFactorqce(tp) * TPh

QSE_EILqc(tp) = ∑ EILqce(tp)

Total_OFFERValueqc(tp) = ∑ OFFERValueqce(tp)

| Variable | Unit | Description |
| --- | --- | --- |
| q | None | QSE |
| c | None | EILS Contract Period |
| e | None | Individual EILS Load |
| tp | None | Hours in an EILS Time Period |
| TPh | None | Number of hours in an EILS Time Period |
| AvailFactorqce(tp) | % | EILS availability factor for an EILS Time Period, as calculated (and revised if necessary) in Section 8.1.3.1, Performance Criteria for EILS Loads |
| OFFERPriceqce(tp) | $ per MW | Contracted Offer Price for an EILS Load for an EILS Time Period |
| OFFERValueqce(tp) | MW | Contracted capacity for an EILS Load for an EILS Time Period |
| Total_OFFERValueqc(tp) | MW | Total contracted EILS capacity for a QSE in an EILS Time Period |
| EILFactorqce(tp) | % | EILS event performance factor for an EILS Time Period, as described in Section 8.1.3.1, Performance Criteria for EILS Loads |
| EILqce(tp) | $ | EILS total payment for an EILS Load for an EILS Time Period |
| QSE_EILqc(tp) | $ | EILS total payment to a QSE for an EILS Time Period |

(2)	ERCOT shall assess the settlement payment for EILS for each EILS Contract Period on the Initial Settlement Statement for an Operating Day no later than 70 calendar days after the last Operating Day of the EILS Contract Period.  For dispute purposes, ERCOT and QSEs shall use the Operating Day of the Settlement Statement on which the EILS payment appears. The timeliness of a dispute concerning EILS pursuant to Section 9.5.2, Notice, shall be determined by the Operating Day of the Settlement Statement on which the EILS payment appears.

6.6.11.2	EILS Capacity Charge

(1)	ERCOT shall allocate costs for an EILS Contract Period based on the Load Ratio Share (LRS) of each QSE during each EILS Time Period in an EILS Contract Period.  A QSE’s LRS for an EILS Time Period shall be the QSE’s total Load for the EILS Time Period divided by the total ERCOT Load in the EILS Time Period.

(2)	If a QSE opts for EILS Self-Provision, the QSE’s LRS for an EILS Time Period shall be the QSE’s total Load for the EILS Time Period, divided by the total ERCOT Load in the EILS Time Period.  The QSE’s LRS for an EILS Time Period is then compared to the amount of EILS Self-Provision by the QSE for an EILS Time Period.

(a)	If the QSE’s EILS Self-Provision amount is equal to the QSE’s LRS multiplied by the total contracted amount of EILS for an EILS Time Period, the QSE’s obligation for that Time Period is zero.

(b)	If the QSE’s EILS Self-Provision amount is greater than the QSE’s LRS multiplied by the total contracted amount of EILS for an EILS Time Period, the QSE’s obligation for that Time Period is zero.

(c)	If the QSE’s EILS Self-Provision amount is less than the QSE’s LRS multiplied by the total contracted amount of EILS for an EILS Time Period, the QSE’s obligation for that Time Period is the difference between the EILS Self-Provision amount and the QSE’s LRS.

(3)	ERCOT shall calculate each QSE’s EILS capacity charge as follows:

LAEILqc(tp) = EILPqc(tp) * EILOqc(tp)

Where:

EILOqc(tp) = Max[0,(LRSqc(tp) *  (Total_OFFERValueqc(tp) + ∑(SPqc(tp))) - SPqc(tp)]

EILPqc(tp) = ∑(QSE_EILqc(tp)) / ∑EILOqc(tp)

SPqc(tp) =  [(SPCqc(tp)) * AvailFactorqce(tp)* EILFactorqce(tp)]

| Variable | Unit | Description |
| --- | --- | --- |
| q | None | QSE |
| c | None | EILS Contract Period |
| tp | None | Hours in an EILS Time Period |
| n | None | The number of committed EILS Loads for a QSE in an EILS Time Period |
| i | None | An index number used to identify a QSE’s EILS Load for an EILS Time Period |
| EILOqc(tp) | MW | EILS Net Obligation per QSE per hour for an EILS Time Period |
| EILPqc(tp) | $ | Offer Price for the EILS Load for an EILS Time Period |
| SPqc(tp) | MW | EILS provided by the QSE through EILS Self-Provision for an EILS Time Period |
| Total_OFFERValueqc(tp) | MW | Total Capacity for an EILS Load contracted for EILS for an EILS Time Period |
| SPCqc(tp) | MW per Hour | Self providing QSE’s commitment for an EILS Time Period |
| AvailFactorqce(tp) | % | Availability factor for an EILS Load in an EILS Time Period as calculated (and revised if necessary) pursuant to Section 8.1.3.1, Performance Criteria for EILS Loads |
| EILFactorqce(tp) | % | Event performance factor for an EILS Load in an EILS Time Period as calculated pursuant to Section 8.1.3.1, Performance Criteria for EILS Loads |
| QSE_EILqc(tp) | $ | EILS total payments to QSEs for an EILS Time Period |
| LRSqc(tp) | % | EILS Load Ratio Share for the QSE for an EILS Time Period |
| LAEILqc(tp) | $ | EILS charge for the QSE for an EILS Time Period |

(4)	ERCOT shall assess the EILS capacity charge as determined above for each EILS Time Period in an EILS Contract Period on the Initial Settlement Statement for an Operating Day no later than 70 calendar days after the last Operating Day of the EILS Contract Period.  For dispute purposes, ERCOT and QSEs shall use the Operating Day of the Settlement Statement on which the EILS payment appears. The timeliness of a dispute concerning EILS pursuant to Section 9.5.2, Notice, shall be determined by the Operating Day of the Settlement Statement on which the EILS obligation appears.

[…]

8.1.3 	Emergency Interruptible Load Service (EILS) Performance  and Testing

8.1.3.1	Performance Criteria for EILS Loads

- (1)	EILS Loads’ performance will be evaluated based on both their availability during a Contract Period and their performance during an EILS deployment event.  As part of its evaluation of each EILS Offer during an EILS procurement process, ERCOT will assign each EILS Load, including each ESI ID participating in an aggregated EILS Load, to a unique baseline.  The baseline has three purposes:

- (a)	To verify or establish an EILS Load’s maximum offer capacity;

- (b)	To provide the basis for ERCOT to determine the EILS Load’s availability during its committed hours in a Contract Period; and

- (b)	To verify the EILS Load’s performance, as compared to its contracted capacity, during an EILS deployment event.

- (2)	EILS Default Baseline:

- (a)	As part of its offer evaluation process, ERCOT will initially attempt to assign each EILS Load offer to a default baseline formula.  The default baseline formula is designed to predict the interval Load based on variables which will include an ESI ID’s historic Load data, weather, time of day and other relevant calendar information.  ERCOT may use other data variables in the default baseline formula at ERCOT’s sole discretion, if ERCOT determines the additional data will enhance the accuracy of the default baseline.  Development of the default baseline for each EILS Load will be consistent with practices used in developing ERCOT Load Profile models.  The methodology for developing the default baseline formula will be documented and published on the ERCOT web site.

- (b)	ERCOT shall establish the default baseline for an aggregated EILS Load by adding the default baselines of the individual ESI IDs in the aggregation.   The performance of an aggregated EIL Resource shall be verified by ERCOT at the EILS Load level.

- (c)	ERCOT will develop a default baseline for each EILS Load by analyzing 15-minute interval usage data for the most recent 12-month period available.  ERCOT may use additional historic data at its sole discretion.  If ERCOT uses historical data pulled from its own systems to develop the default baseline for an EILS Load, ERCOT shall upon request provide the historical data used to the Entity responsible for that EILS Load.

- (d)	Based on ERCOT’s analysis of data in establishing a default baseline for an EILS Load, ERCOT may reduce the amount of capacity an EILS Load may be awarded in a given EILS Contract Period.

- (e)	Upon request, ERCOT shall provide default baseline analysis results for an EILS Load to the Entity representing that EILS Load.

- (3)	Alternate Baseline:

- (a)	ERCOT may assign an EILS Load to a single alternate baseline formula if ERCOT determines that the EILS Load cannot be assigned to the default baseline with sufficient accuracy.

- (b)	Under the alternate baseline formula, ERCOT shall calculate an EILS Load’s average (mean) IDR-metered Load (MWh) over the most recent available 12-month period, with an emphasis on the months corresponding to the upcoming Contract Period.  ERCOT will validate the MW capacity offer for each EILS Load for the applicable EILS Time Period, based upon the difference between this average Load calculation (MWh) and the EILS Load’s declared minimum base Load (MWh).  In selecting an EILS Load with an alternate baseline, ERCOT may award the lesser of the MW offer or the MW capacity validated by ERCOT.  When deployed by ERCOT, the EILS Load assigned to an alternate baseline shall curtail down to its minimum base Load or below, regardless of how much actual Load the EILS Load has online at the time of deployment.

- (4)	All ESI IDs within an aggregated EILS Load must be assigned to the same baseline methodology (either the default baseline or the alternate baseline).

- (5)	End of Contract Period Availability Review and Capacity Payment Adjustments:

- (a)	Within 45 days after the end of an EILS Contract Period, ERCOT will complete an availability review for each EILS Load that was contracted for that EILS Contract Period.  In its availability review, ERCOT will determine an availability factor for each EILS Load in that EILS Contract Period.

- (b)	ERCOT will determine the availability factor for an EILS Resource.  An availability factor of 95% or greater for an EILS Resource shall result in no reduction in capacity payment for the EILS Resource (i.e., ERCOT shall set the availability factor at 100%), and the EILS Resource will be deemed to have complied with its availability requirements.  If an EILS Resource’s availability factor for an EILS Contract Period falls below 95%, ERCOT shall set the EILS Resource’s availability factor at its actual availability factor, and the EILS Resource will be deemed to have failed to meet its availability requirements which may result in a capacity payment adjustment.  The calculations to determine the availability factor to be used for Settlement purposes are as follows:

AvailFactorqce(tp)r = 1 if AvailFactorqce(tp) >= .95, else AvailFactorqce(tp)r = AvailFactorqce(tp)

| Variable | Unit | Description |
| --- | --- | --- |
| q | None | QSE |
| c | None | EILS Contract Period |
| e | None | Individual EILS Resource |
| tp | None | EILS Time Period |
| AvailFactorqce(tp) | % | Availability factor for an EILS Resource for an EILS Time Period, as calculated pursuant to the following subsections |
| AvailFactorqce(tp)r | % | Revised availability factor for an EILS Resource for an EILS Time Period |

- (c)	For an EILS Resource assigned to the default baseline, ERCOT will calculate its availability factor as follows:

- (i)	ERCOT will consider the EILS Load to have been available for any hour in a contracted Time Period in which the EILS Load’s IDR-metered Load was greater than 95% of its contracted EILS MW capacity (offer capacity plus declared minimum base Load); otherwise, the EILS Load will be considered unavailable for that hour.  The availability factor will be the ratio of the number of hours the EILS Resource was available during the EILS Contract Period divided by the total hours in the EILS Contract Period.

- (ii) 	Notwithstanding the foregoing, in determining availability factor, ERCOT will consider the EILS Load to have been available for any of the following contracted hours:

- (A)	Any hours for which the EILS Load’s QSE notified ERCOT, in a format prescribed by ERCOT, of the EILS Load’s unavailability at least five Business Days in advance, up to a maximum of two percent of the total contracted hours in the EILS Contract Period;

- (B) 	Any hours in which an EECP was in effect, starting with initiation of Step 1 and including the full EILS recovery period, if applicable; and

- (C)	Any hours following the second EILS deployment in an EILS Contract Period.

- (d)	For an EILS Load assigned to the alternate baseline, ERCOT will calculate its availability factor as follows:

- (i)	ERCOT shall divide the EILS Load’s actual average Load per hour (excluding its declared minimum base Load, if any) for the contracted hours in the EILS Time Period EILS Load’s contracted MW offer  provided that the availability factor shall not be greater than one.

- (ii)	In determining the EILS Load’s average actual Load,

- (A)	ERCOT shall exclude from the average any hours for which the EILS Load’s QSE notified ERCOT, in a format prescribed by ERCOT, of the EILS Load’s unavailability at least five Business Days in advance, up to a maximum of two percent of the total contracted hours in the EILS Contract Period;

- (B)	Any hours in which an EECP was in effect, starting with initiation of Step 1 and including the full EILS recovery period, if applicable; and

- (C)	Any hours following the second EILS deployment in an EILS Contract Period.

- (iii)	The calculations for the alternate baseline availability factor are as follows:

AvailFactorqce(tp) =  MIN (1, (AV(tp)/(h*OFFERValueqce(tp))))

| Variable | Unit | Description |
| --- | --- | --- |
| Q | None | QSE |
| C | None | EILS Contract Period |
| E | None | Individual EILS Load |
| Tp | None | EILS Time Period |
| H | Hour | hour |
| AVtp | MWh | Average Load per hour for an EILS Load in a contracted EILS Time Period, excluding declared minimum base Load |
| OFFERValueqce(tp) | MW | An EILS Load’s contracted capacity for an EILS Time Period |
| AvailFactorqce(tp) | Percent | Availability factor for an EILS Load for an EILS Time Period |

(d)	In the event an EILS Load that is part of a QSE’s EILS Self-Provision obligation fails to meet its availability requirement for a Contract Period, ERCOT shall adjust the EILS Load’s QSE’s Settlement obligation to reflect the actual availability factor by modifying the term “SP” in Section 6.6.11, Emergency Interruptible Load Service Settlement, paragraph (2).  An EILS Load that is part of a QSE’s EILS Self-Provision that achieves an availability factor of 0.95 or greater shall be considered to have met its availability requirement.

- (6)	EILS Loads’ Compliance During an EILS Deployment Event and Capacity Payment Adjustments:

(a)	Upon ERCOT’s issuance of a Verbal Dispatch Instruction (VDI) during EECP Step 3 requesting EIL deployment, EILS Loads assigned to the default baseline must curtail at least 95% of available contracted capacity within ten (10) minutes of receiving the VDI and must stay at or below that level until released by ERCOT.  EILS Loads assigned to the alternate baseline must curtail to their declared minimum base Load or below, and must stay at or below that level until released by ERCOT.

(b)	ERCOT shall measure each EILS Load’s compliance with this requirement through analysis of f15-minute IDR data from each ESI ID.  ERCOT will determine an event performance factor for each EILS Load based upon this analysis.

(c)	For an EILS Load assigned to a default baseline, the event performance factor (EILFactor) is computed as the arithmetic average of the EILS Interval Performance Factors (EIPF) for the entire curtailment period.  An EIPF is computed for the EILS Load for each of the 15-minute intervals during which an EIL curtailment is required. For an interval, EIPFi is computed as follows:

EIPFi = Max(Min(((Base_MWhi – Actual_MWhi) / (IntFraci ×

OFFERValue)),1),0)

Where:

| Variable | Unit | Description |
| --- | --- | --- |
| i | None | Interval |
| IntFraci | None | Interval fraction for that EILS Load for that interval |
| Base_MWhi | MWh | Aggregated sum of the baseline MWh values estimated by ERCOT for all ESI IDs in the EILS Load for that interval |
| Actual_ MWhi | MWh | Aggregated sum of the actual MWh values for all ESI IDs in the EILS Load for that interval |
| OFFERValue | MWh | The EILS Load’s contracted capacity |

and where IntFraci corresponds to the fraction of time for that interval for which the curtailment period is in effect and is computed as follows:

IntFraci =  (CEndTi - CBegTi) / 15

Where:

| Variable | Unit | Description |
| --- | --- | --- |
| i | None | Interval |
| IntFraci | None | Interval fraction for that EILS Load for that interval |
| CBegTi | Minutes | If the curtailment begins after the start of that interval, the time in minutes from the beginning of that interval to the beginning of the curtailment period, otherwise it is zero (0) |
| CEndTi | Minutes | If the curtailment ends during that interval, the time in minutes from the beginning of that interval to the end of the curtailment period, otherwise it is 15 |

- (d)	For an EILS Load assigned to an alternate baseline, the EILFactor is computed as the arithmetic average of the EILS Interval Performance Factors (EIPF) for the entire curtailment period. An EIPF is computed for the EILS Load for each of the 15-minute intervals during which an EIL curtailment is required. For an interval, EIPFi is computed as follows:

- (i)	For the first interval in the curtailment period,

- If Actual_MWhi = 0 then EIPFi = 1,

- Otherwise

EIPFi = Min((((1-IntFraci)×Actual_MWhi-1 + (IntFraci × Min_MWh)) / Actual_MWhi),1)

- (ii)	For the last interval in the curtailment period,

- If Actual_MWhi = 0 then EIPFi = 1,

- Otherwise

EIPFi = Min((((1-IntFraci)×Actual_MWhi+1 + (IntFraci × Min_MWh)) / Actual_MWhi),1)

(iii)	For all other intervals in the curtailment period

- If Actual_MWhi = 0 then EIPFi = 1,

Otherwise

EIPFi = Min(Min_MWh / Actual_MWhi),1)

Where:

| Variable | Unit | Description |
| --- | --- | --- |
| i | None | Interval |
| IntFraci | None | Interval fraction for that EILS Load for that interval |
| Min_MWh | MWh | Aggregated sum of the minimum base Load in MWh values contracted for all ESI IDs in the EILS Load for that interval |
| Actual_ MWhi | MWh | Aggregated sum of the actual MWh values for all ESI IDs in the EILS Load for that interval |

(iv)	Where IntFraci corresponds to the fraction of time for that interval for which the curtailment period is in effect, computed as follows:

IntFraci =  (CEndTi - CBegTi) / 15

Where:

| Variable | Unit | Description |
| --- | --- | --- |
| i | None | Interval |
| IntFraci | None | Interval fraction for that EILS Load for that interval |
| CBegTi | Minutes | If the curtailment begins after the start of that interval, the time in minutes from the beginning of that interval to the beginning of the curtailment period, otherwise it is zero |
| CEndTi | Minutes | If the curtailment ends during that interval, the time in minutes from the beginning of that interval to the end of the curtailment period, otherwise it is 15 |

- (e)	In the event that an EILS Load does not meet its performance obligations according to the appropriate methodology described above, ERCOT may, in its sole discretion, adjust the EILS Load’s event performance factor to reflect the severity of the failure.  The event performance factor for an EILS Load may be any number from zero to one, inclusive.  An EILS Load that achieves an event performance factor of 0.95 or greater shall be considered to have met its performance obligations for that event.

- (f)	In any EILS Contract Period in which ERCOT has issued one or more EILS deployments, if an EILS Load meets its event performance requirements as described in this subsection for all EILS deployments, ERCOT shall establish the EILS Load’s availability factor at the availability factor calculated by ERCOT or 50%, whichever is greater.  For any such EILS Load, ERCOT shall apply the calculations below for an EILS Contract Period in which there was at least one EECP event in which ERCOT deployed EILS:

AvailFactorqce(tp)r = MAX(.5, AvailFactorqce(tp))

- Where:

| Variable | Unit | Description |
| --- | --- | --- |
| q | None | QSE |
| c | None | EILS Contract Period |
| e | None | Individual EILS Load |
| tp | None | EILS Time Period |
| r | None | Revised pursuant to Subparagraph (e) or (f) above |
| AvailFactorqce(tp) | % | Availability factor for the EILS Load for the EILS Time Period as calculated by ERCOT pursuant to Subsection (5) above |
| AvailFactorqce(tp)r | % | Revised EILS availability factor for the EILS Load for the EILS Time Period |

- In the event an EILS Load that is part of a QSE’s EILS Self-Provision obligation does not meet its performance requirement in an EILS deployment event, ERCOT shall adjust the Settlement obligation of the QSE representing the EILS Load to reflect the actual event performance factor by modifying the term “SP” in Section 6.6.11, Emergency Interruptible Load Service Capacity Payments and Settlement, paragraph (5).  An EILS Load that is part of a QSE’s EILS Self-Provision that achieves an event performance factor of 0.95 or greater shall be considered to have met its performance requirement for that EILS event.

8.1.3.2		Testing of EILS Loads

- (1)	ERCOT may conduct an unannounced Load-shedding test of any EILS Load at any time during an EILS Contract Period in which the EILS Load is contracted to provide EILS.  Any such test will not be counted as one of the EILS Load’s two maximum deployments in a Contract Period.  A Load-shedding test shall be deemed to be successful if the EILS Load meets its event performance criteria as defined in paragraph (6) of Section 8.1.3.1, Performance Criteria for EILS Loads.  An EILS Load that successfully completes an ERCOT-conducted Load-shedding test shall not be subject to an additional Load-shedding test for at least 365 days.  In addition, an EILS Load that meets its performance obligations during any EILS deployment event shall not be subject to a full Load-shedding test for at least the following 365 days.

- (2)	ERCOT may conduct an unannounced Load-shedding test of a Load or aggregation of Loads that has been suspended from participation in EILS pursuant to Subsection 8.1.3.3, Suspension of Qualification of EILS Loads and/or their QSEs.  ERCOT will conduct such a test only after the QSE representing the Load or aggregation has communicated to ERCOT a request for reinstatement of the suspended EILS Load.

8.1.3.3	Suspension of Qualification of EILS Loads and/or their QSEs

(1)	If ERCOT determines that an EILS Load has failed to meet its performance or availability obligations, ERCOT shall suspend the EILS Load’s qualification to participate in EILS for six months.  If ERCOT determines that a QSE representing an EILS Load has failed to meet its EILS obligations under these Protocols, ERCOT shall suspend the QSE’s ability to represent EILS Loads for six months.  Such suspension may be based on performance during an EILS deployment, the amount of capacity available, the number of hours available, or a combination of the above.  ERCOT may consider mitigating factors such as equipment failures and Force Majeure Events in determining whether to assess such penalties.  If the EILS Load or QSE is actively providing EILS at the time it failed to meet its obligations, the six-month suspension period shall commence at the end of the current Contract Period.  If the EILS Load or QSE is not providing EILS at the time it failed to meet its obligations, the six-month suspension period shall begin immediately upon the finding.

(2)	If ERCOT suspends qualification of an EILS Load or a QSE representing an EILS Load due to failure to meet performance obligations, ERCOT may revoke and recoup all payments for the EILS Contract Period associated with that EILS Load.

(3)	ERCOT may reinstate an EILS Load’s eligibility to offer into EILS only after the EILS Load satisfactorily performs a Load-shedding test conducted by ERCOT, as described in Section 8.1.3.2, Testing of EILS Loads.

8.1.3.4	ERCOT Data Collection for EILS

(1)	ERCOT will collect all data necessary to analyze offers, Self-Provision offers, and all availability and performance obligations of EILS Loads and their QSEs under the Protocols.  QSEs and EILS Loads they represent are required to provide any data to ERCOT that ERCOT may require, as specified by ERCOT.

(2)	ERCOT shall post to the MIS Certified Area a summary of each QSEs EILS Load availability and performance for each Contract Period.

[….]

9.14.5 	Resettlement of Emergency Interruptible Load Service (EILS)

ERCOT shall issue a Resettlement Statement for the EILS Contract Period for any approved EILS disputes no later than 120 calendar days after the date that the EILS Contract Period was initially settled.  All disputes for an EILS Contract Period will be due sixty (60) calendar days after the date that the EILS Contract Period was initially settled.

9.14.6	Disputes for Operations Decisions

Settlement Statement or Invoice Recipients may not dispute a Settlement Statement or Invoice due to a decision made by ERCOT in its operation of the ERCOT System, unless the Market Participant alleged the decision violated these Protocols.  Inquiries or disputes concerning such decisions, Protocols, or Operating Guides must be handled through the Protocol change process set forth in Section 21.

9.14.7	Reporting Capability for Disputes

ERCOT shall post daily to the MIS Public Area a report of the status of all disputes.  The report must include:

(a)	Filed date;

(b)	Disputing Entity;

(c)	Dispute ID number;

(d)	Dispute status;

(e)	Date of last status change;

(f)	Charge Type;

(g)	Amount disputed; and

(h)	Amount awarded.

[….]

16.13 	Registration of Emergency Interruptible Loads

(1)	EILS Loads shall register with ERCOT by completing and signing Appendix A (Acknowledgement by EILS Load Owning or Controlling Entity) of the Standard Form Emergency Interruptible Load Service (EILS) Supplement to QSE Agreement (Section 22K of the Protocols).

ERCOT Protocols

Section 22

Attachment K

Standard Form

Emergency Interruptible Load Service (EILS) Agreement

December 11, 2007

Standard Form Emergency Interruptible Load Service (EILS)

Supplement to QSE Agreement

Between

(Name of Participant)

and

Electric Reliability Council of Texas, Inc.

This Supplement to QSE Agreement (“Supplement”), effective as of [START DATE] (“Start Date”), is entered into by and between [PARTICIPANT’s NAME], a Qualified Scheduling Entity in the ERCOT Region (“QSE” or “Participant”) and Electric Reliability Council of Texas, Inc., a Texas non-profit corporation (“ERCOT”).

Recitals

WHEREAS:

- The Public Utility Commission of Texas (“PUCT”) instituted its Substantive Rule 25.507, “Electric Reliability Council of Texas (ERCOT) Emergency Interruptible Load Service” (“EILS Rule”) providing for the creation of a special emergency service known as Emergency Interruptible Load Service (“EILS”); and

- Participant is a QSE in the ERCOT Region and has executed a Standard Form Qualified Scheduling Entity Agreement (“QSE Agreement”) with ERCOT; and

- Participant is the QSE representing an entity or entities owning or controlling EILS Load(s) that will be obligated to provide EILS; and

- Participant and ERCOT wish to supplement the QSE Agreement between Participant and ERCOT to provide for Participant to represent EILS Loads wishing to participate in the EILS; and

- The Parties enter into this Supplement in order to establish the terms and conditions by which ERCOT and Participant will discharge their respective duties and responsibilities with respect to EILS.

Agreements

NOW, THEREFORE, in consideration of the mutual covenants and promises contained herein, ERCOT and Participant (the “Parties”) hereby agree as follows:

- All terms and conditions of the QSE Agreement between Participant and ERCOT remain in full force and effect.

- In addition to its obligations under the QSE Agreement with ERCOT, Participant will submit offers for EILS on behalf of the entities set forth in Appendix A for a particular Contract Period as described in a Request for Proposal issued by ERCOT.

- Participant and ERCOT will abide by and comply with the EILS Rule as well as all ERCOT Protocols and Technical Requirements concerning EILS.

- Participant and ERCOT agree that each award of EILS will be confirmed by a terms and conditions sheet (“Term Sheet”) provided by ERCOT to Participant in a form substantially the same as the form attached hereto as Appendix B.

- Either Party may terminate this Supplement by providing thirty days notice to the other Party; provided, however, no termination of this Supplement will be effective before the end of an EILS Contract Period for which ERCOT has already issued a Term Sheet to Participant.

- This Supplement may be executed in two or more counterparts, each of which is deemed an original but all constitute one and the same instrument.

SIGNED, ACCEPTED, AND AGREED TO by each undersigned signatory who, by signature hereto, represents and warrants that s/he has full power and authority to execute this Supplement.

Electric Reliability Council of Texas, Inc.:

By: ____________________________________________________

Printed Name: ___________________________________________

Title: ___________________________________________________

Date: ___________________________________________________

Participant:

By: _____________________________________________________

Printed Name:____________________________________________

Title: ___________________________________________________

Date: ___________________________________________________

Appendix A

To Supplement to QSE Agreement

The Entity that owns or controls the following EILS Load hereby acknowledges that it understands and will comply with PUC Subst. Rule 25.507 and the ERCOT Protocols relating to EILS.

| Acknowledgement by EILS Load Owning or Controlling Entity |  |
| --- | --- |
| Company Name |  |
| Authorized Representative Name |  |
| Title |  |
| Address Line1 |  |
| Address Line 2 |  |
| City/State/ZIP |  |
| Phone |  |
| Email Address |  |

[ENTITY NAME]

By:_______________________________________________

[Signature]

Printed Name:______________________________________

Its:_______________________________________________

[Title]

Date:__________________________________________

|  | 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

EILS Load Offer is awarded or accepted.

Appendix B

Supplement to QSE Agreement for EILS Loads

This Term Sheet is subject to the QSE Agreement and EILS Supplement thereto between {QSE Name} and Electric Reliability Council of Texas, Inc., dated MM/DD/YY.  The terms of this Term Sheet are binding on both parties.

| EMERGENCY INTERRUPTIBLE LOAD SERVICE TERM SHEET |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QSE Name |  |  | QSE DUNS Number |  |  |  |
| Contract Period |  |  |  |  |  |  |
| EILS Load Type |  |  |  |  |  |  |
| Baseline Methodology (Default or Alternate) |  |  |  |  |  |  |
| EILS Time Period (from RFP) |  |  |  |  |  |  |
| AWARD AMOUNTS |  |  |  |  |  |  |
| Interruptible Capacity (MW) |  |  |  |  |  |  |
| Price ($/MW) to be applied to each hour of the contract period |  |  |  |  |  |  |
| Minimum Base Load (MW) associated with this EILS Load that will not be interrupted as part of the EILS deployment |  |  |  |  |  |  |

A. Individual EILS Load Data

| ESI ID or unique service identifier assigned to meter (MUST BE IDR) |  |
| --- | --- |
| Load Owner/Operator |  |
| Load Common Name |  |
| Meter Reading Entity (TDSP) |  |
| Meter Reading Entity DUNS Number |  |
| Load within a NOIE Service Area (Y or N) |  |
| Load within a Private Use Network (Y or N) |  |

B. Aggregated EILS Load Data

| ESI ID or unique service identifier assigned to meter (MUST BE IDR) | Resource Owner or Operator | Resource Common Name | Meter Reading Entity (TDSP) | Meter Reading Entity DUNS Number | Within a NOIE Service Area | Within a Private Use Network |
| --- | --- | --- | --- | --- | --- | --- |

C.  Non-IDR Aggregated EILS Load Data

|  | Description of EILS Load (Aggregation population) |  |  |
| --- | --- | --- | --- |
|  | Resource Owner/Operator |  |  |
| Resource Common Name |  |  |  |
|  | Meter Reading Entity (TDSP) |  |  |
|  | Meter Reading Entity DUNS Number |  |  |
|  | Load within a NOIE Service Area (Y or N) |  |  |
|  | Load within a Private Use Network (Y or N) |  |  |

Unless otherwise indicated, capitalized terms in this Agreement have the meanings ascribed to them in the ERCOT Protocols.