ERCOT Nodal Protocols

Section 13:  Transmission and Distribution Losses

Updated: August 1, 2007

(Effective upon the Nodal Protocol Transition Plan’s Texas Nodal Market Implementation Date as prescribed by zonal Protocol Section 21.12, Process for Transition to Nodal Market Protocol Sections)

13	Transmission and Distribution Losses	13-1

13.1	Overview	13-1

13.1.1	Responsibility for Transmission and Distribution Losses	13-1

13.1.2	Calculation of Losses for Settlement	13-2

13.2	Transmission Losses	13-2

13.2.1	Forecasted Transmission Loss Factors	13-2

13.2.2	Deemed Actual Transmission Loss Factors	13-2

13.2.3	Transmission Loss Factor Calculations	13-2

13.2.4	Monthly Transmission Loss Factor Calculation	13-3

13.2.5	Loss Monitoring	13-3

13.3	Distribution Losses	13-3

13.3.1	Loss Factor Calculation	13-4

13.3.2	Loss Monitoring	13-5

13.4	Special Loss Calculations for Settlement and Analysis	13-5

13.4.1	Deemed Actual Transmission Losses for NOIEs	13-5

13.4.2	Deemed Actual Transmission Losses for UFE Analysis	13-6

# Transmission and Distribution Losses

13.1	Overview

This section sets forth the method for calculating Transmission and Distribution Losses (T&D Losses) and responsibilities of ERCOT, Qualified Scheduling Entities (QSEs), Transmission Service Providers (TSPs) and Distribution Service Providers (DSPs) with respect to T&D Losses.

13.1.1	Responsibility for Transmission and Distribution Losses

(1)	T&D Losses are the responsibility of the QSE representing the Competitive Retailer’s Load.  The QSE will schedule the necessary amount of energy to cover the Competitive Retailer’s Load plus the applicable T&D Losses.  ERCOT shall allocate T&D Losses to Load at the appropriate aggregate level as part of the data aggregation process to calculate the Load obligation of QSEs for settlement purposes.

(2)	ERCOT shall forecast Transmission Loss Factors (TLFs) and post them to the MIS Public Area by 0600 of the Day Ahead period.  ERCOT shall forecast the ERCOT-wide TLFs as a percentage of Load for each Settlement Interval of the Operating Day.  By the close of business on the day following the Operating Day, ERCOT shall also calculate TLFs for each Settlement Interval using the actual system Load for that Settlement Interval and shall post the resulting deemed actual TLFs to the settlement system and the MIS Public Area.

(3)	ERCOT shall forecast Settlement Interval Distribution Loss Factors (DLFs) and post them to the MIS Public Area by 0600 of the Day Ahead period.  ERCOT shall forecast the Settlement Interval DLFs as a percentage of Load for each Settlement Interval of the Operating Day.  On the day following the Operating Day, ERCOT shall also calculate Settlement Interval DLFs using actual system Load for that Settlement Interval and post the resulting deemed actual Settlement Interval DLFs to the settlement system and the MIS Public Area.

(4)	Distribution loss coefficients, and the calculation methodology from which they are derived, will be subject to audit by ERCOT for accurate and consistent application.  Non-Opt-in Entities (NOIE) with Interval Data Recorders at the settlement point of delivery are not required to provide Distribution loss coefficients and calculation methodology.

(5)	In the special case where there are distribution facilities upstream from a wholesale NOIE settlement IDR, that NOIE settlement IDR will be compensated for line and transformer losses between the IDR and the ERCOT Transmission Grid to account for the Distribution Losses.  The NOIE will be then treated as a transmission level NOIE.  Calculations are subject to review by ERCOT.  Since loss compensation is included in the wholesale settlement IDR, the TSP and/or DSP providing upstream wheeling facilities may need to offer wholesale wheeling tariffs excluding the losses that have already been compensated for.

13.1.2	Calculation of Losses for Settlement

ERCOT shall use the deemed actual Settlement Interval DLFs applicable to each ESI ID and the deemed actual Settlement Interval TLFs when adjusting aggregated Load for losses to determine the QSE total Load obligations.

13.2	Transmission Losses

13.2.1	Forecasted Transmission Loss Factors

(1)	The forecasted TLF for each interval in the Operating Day shall be a linear interpolation or extrapolation using the on-peak and the off-peak TLFs and the corresponding forecast of ERCOT System Load during the same interval to calculate the loss factors.

(2)	At 0600 of the Day Ahead period, ERCOT shall forecast a TLF for each Settlement Interval of the Operating Day and post on the MIS Public Area the forecasted TLFs which correspond to the Operating Day forecast.  The source of the on-peak and off-peak losses are the ERCOT load flow base cases for the applicable month.

13.2.2	Deemed Actual Transmission Loss Factors

(1)	ERCOT shall determine the deemed actual TLF for each interval in the Operating Day, by use of a linear interpolation or extrapolation using the on-peak and the off-peak TLFs corresponding to the actual ERCOT System  Load during the interval.

(2)	The day after the Operating Day, ERCOT shall calculate deemed actual TLFs for each Settlement Interval of the Operating Day and publish the TLFs to be used in settlement calculations.

(3)	ERCOT shall use the TLFs corresponding to the on-peak and off-peak base case ERCOT System Loads during the applicable months as the basis for the ERCOT-wide deemed actual TLFs.  ERCOT will post TLFs to the MIS Public Area by 0600 two days after the Operating Day.

13.2.3	Transmission Loss Factor Calculations

The following formulas shall be used to translate the monthly on-peak and off-peak TLFs into Settlement Interval TLFs.

TLFi	=	(MSC * SIELi) + MIC

| Variable | Unit | Description |
| --- | --- | --- |
| i | none | Interval |
| TLFi | none | Transmission Loss factor for a settlement interval |
| SIELi | MWh | Settlement Interval ERCOT System Load (forecasted or actual) |
| MSC | none | Monthly Slope Coefficient |
| MIC | none | Monthly Intercept Coefficient |

And

MSC 	=	(MONLF – MOFFLF)/(MONL-MOFFL)

MIC	=	[(MOFFLF*MONL)-(MONLF*MOFFL)]/(MONL-MOFFL)

| Variable | Unit | Description |
| --- | --- | --- |
| MONLF | none | Monthly on-peak percent loss factor |
| MOFFLF | none | Monthly off-peak percent loss factor |
| MONL | none | Monthly on-peak Load value |
| MOFFL | none | Monthly off-peak Load value |

13.2.4	Monthly Transmission Loss Factor Calculation

(1)	Monthly on-peak and off-peak TLFs are derived from the monthly updated ERCOT on-peak and off-peak load flow base cases analysis by ERCOT.  Base cases reflect the most current data on the transmission system and Generation Resource dispatch.  The ERCOT Transmission Grid topology and related Generation Resource dispatch in the base cases are the critical factors in calculating losses.

(2)	ERCOT shall calculate monthly TLFs by dividing ERCOT monthly case transmission losses (60 kV system and higher) by the ERCOT monthly base Load adjusted (reduced) for self serve Load modeled in the case.  The resulting loss factors are expressed as a percentage of Load.

(3)	ERCOT shall post to the MIS Public Area monthly TLFs 30 days prior to the start of the month.  The posting will include monthly on-peak and off-peak cases for 18 months in the future.

13.2.5	Loss Monitoring

ERCOT shall monitor Transmission Losses annually and will investigate any abnormal loss factors.  ERCOT and TSPs shall use the cost of losses as one criterion in evaluating the need for transmission additions.

13.3	Distribution Losses

(1)	By October 30th of each year for the next calendar year, or two months prior to the posting of any update to the approved Distribution loss coefficients, codes, or calculation, each Distribution Service Provider (DSP), except NOIEs, shall calculate and provide ERCOT the Annual Distribution loss coefficients to be applied to distribution voltage level Loads in its area of certification.  ERCOT shall review and approve the DLF calculation methodology used by each DSP prior to use of the loss coefficients for settlement purposes.  If the DLF calculation methodology does not conform with ERCOT’s interpretation of the Protocol criteria in this subsection, ERCOT will work with the Distribution Service Provider to correct the deficiency.  Until deficiencies are resolved, the last approved Distribution loss coefficients and the calculation methodology will be posted, and the last approved Distribution loss coefficients shall be used for settlement.  A DSP may only submit a change to the DLF calculation methodology annually or when a change in a DSP service area warrants an update to the approved DLF methodology based on the DSP internal evaluation.

(2)	The DSP shall assign a Distribution loss code to each ESI ID.  A maximum of five Distribution loss codes may be submitted for each DSP based upon ERCOT approved parameters, such as service voltages or number of transformations.

(3)	The following standards will be used to identify the Distribution loss code applicable to each ESI ID:

- T = Transmission connected Customers (no Settlement Interval DLF applied)

- A through E = TDSP defined Customer segment(s)

(4)	The DSPs, except NOIEs, are obligated to provide Distribution loss coefficients  to ERCOT.  ERCOT will post the Distribution loss coefficients and calculation methodology, for each DSP.

(5)	Distribution loss information submitted by the DSP shall include:

(a)	The annual Distribution loss coefficients (F1, F2, and F3) for each Distribution loss code; and

(b)	The methodology upon which the calculation of the coefficients (F1, F2, and F3) was made.

13.3.1	Loss Factor Calculation

(1)	ERCOT shall use the Distribution loss coefficients submitted by the DSP to calculate the Settlement Interval DLFs.  Settlement Interval DLFs will be calculated from the data provided by DSPs as follows using the following equation:

SILFi = F1 * (SIELi/AAL) + F2 + F3 / (SIELi/AAL)

| Variable | Unit | Description |
| --- | --- | --- |
| i |  | interval |
| SILFi |  | Settlement Interval DLF |
| SIELi |  | Settlement Interval ERCOT System Load (forecasted or actual) |
| AAL |  | Annual Interval Average ERCOT System Load. The AAL is calculated using the total ERCOT Load stated in the most recent settlement during the period beginning on September 1 and ending August 31. ERCOT will provide the AAL to DSPs that are obligated to provide Distribution loss coefficients and calculation methodology to ERCOT, by September 15th of each year. |
| F1 , F2 , F3 |  | Distribution Loss coefficients determined by the Distribution Service Provider to allow calculation of its SILF from ERCOT System Load |

(2)	ERCOT shall use the deemed actual Settlement Interval DLFs calculated for each Settlement Interval of the Operating Day for settlement purposes.

13.3.2	Loss Monitoring

Distribution loss coefficients and the calculation methodology from which they are derived for all DSPs, except for NOIEs, will be submitted to ERCOT and will be subject to audit for accuracy and consistency of application.

13.4	Special Loss Calculations for Settlement and Analysis

13.4.1	Deemed Actual Transmission Losses for NOIEs

(1)	All QSEs representing Load, including NOIEs, will be responsible for Transmission Losses allocated in the manner described in these Protocols.  Those Entities using transmission tie line meters to determine Load will adjust the net meter readings to remove calculated Transmission Losses behind the meter in order to determine the Load responsibility of the Entity.  ERCOT will provide to settlement the calculation of the losses behind the meters, for each interval, using actual system conditions for that interval.

(2)	The deemed actual Transmission Losses for NOIEs shall be a linear interpolation or extrapolation between the monthly on-peak and the monthly off-peak NOIE TLFs corresponding to the actual NOIE metered Load in the interval.

(3)	ERCOT shall calculate monthly NOIE TLFs corresponding to the on-peak and off-peak base case system loads during each of the subsequent 18 calendar months as the basis for the NOIE TLFs.  NOIE monthly loss factors will be calculated in the same manner as the loss factors are calculated for the ERCOT-wide TLFs.

13.4.2	Deemed Actual Transmission Losses for UFE Analysis

(1)	ERCOT shall adjust Net Generation data used for UFE analysis zones that contains Transmission Facilities behind any metering points to the ERCOT-wide TLF.  This adjustment requires reducing the Net Generation by the calculated deemed actual MWh of Transmission Losses and adding back the ERCOT-wide TLF translated into a MWh value.  ERCOT shall provide the calculation of the deemed actual Transmission Losses behind the UFE zonal meters, for each interval, to settlement using actual system conditions for that interval.

(2)	The deemed actual Transmission Losses  for UFE analysis zones shall be a linear interpolation or extrapolation between the monthly on-peak and the monthly off-peak UFE analysis zone TLFs corresponding to the actual UFE analysis zone metered Load in the interval.

(3)	ERCOT shall calculate monthly UFE analysis zones TLFs corresponding to the on-peak and off-peak base case UFE zone system Loads during each of the subsequent 18 calendar months as the basis for the UFE analysis zone TLFs.  UFE analysis zone monthly loss factors will be calculated in the same manner as the loss factors are calculated for the ERCOT-wide TLFs.