Procedure for Identifying Resource Nodes

Updated: June 16, 2008

This procedure is the guiding document for ERCOT, in conjunction with the Market Participants, to identify Resource Nodes.

The following procedure delineates the steps necessary to incorporate a Resource Node into the Network Operations Model.  The Resource Node will subsequently be used in all the associated ERCOT models, applications, and processes.

Procedure:

- At the designated appropriate time period as determined by Nodal Protocols 3.10 and associated ERCOT business processes, a Resource Entity (RE) must submit a Resource Asset Registration Form (RARF) that includes a detailed electrical one-line drawing of the Generation Facility.  The ERCOT Business process indicates that the RARF will be presented to the Network Modeling Group (NMG) within ERCOT.

- The Network Modeling Group will utilize the “Principles for Resource Node Definition” located in Appendix A to determine the Resource Node parameters.

- The NMG will provide documentation back to the RE clearly indicating the Resource Node parameters.

- The RE will have five (5) Business Days to accept or reject the suggested Resource Node parameters.

- If there are any disagreements with the Resource Node parameters, ERCOT and the RE shall work together to reach consensus.

- If consensus can not be reached by ERCOT and the RE, the Wholesale Market Subcommittee (WMS) or appropriate WMS working group shall help guide the decision.

- Upon a consensus between ERCOT and the RE, the Resource Node parameters will be implemented in the Network Operations Model.

- The normal timeline for this procedure shall not exceed 20 Business Days after the submission date of a validated RARF submittal that includes a detailed electrical one-line drawing.

- In the event of disagreement issues that may extend beyond the 20 Business Days, no Resource Node parameters will be entered into the Network Operations Model.  This may have an affect on CRR Models and associated CRR activities regarding the Generation Resource  in question.

- The Resource Node parameters, associated electrical one-line drawings, and other relevant data shall be posted on the MIS Secure Area and will be available to all Market Participants.

# Appendix A

PRINCIPLES FOR RESOURCE NODE DEFINITION

- Network Operations Model

  - Annual/Monthly CRR Auctions use network model as close as possible to the Network Operations Model.

- MMS and EMS use the same Network Operations Model for both commercial and operational purposes.

- Breakers between the Connectivity Nodes and the Resource Node are closed as default so that Resource Nodes and associated Connectivity Nodes appear energized.

- Transmission Element outages, as defined in the ERCOT Protocols, are submitted into Outage Scheduler and posted before DAM submission, i.e. de-energized Resource Nodes (Settlement Points) are known in advance of DAM submission.

- Connectivity Nodes

  - Connectivity Node represents the Electrical Bus where physical generator is connected.

  - Generator power output is injected at the Connectivity Node.

  - More then one Resource can be connected to the same Connectivity Node.

- Resource Nodes

  - Resource Node Definition

  - Resource Node represents the Electrical Bus or the logical construct that defines the location of a Settlement Point.

  - Resource Nodes include Generation Resource Nodes, CCP Logical Resource Nodes, CCU Resource Nodes and PUN Resource Nodes.

  - Generation Resource Node represents the Settlement Point for ERCOT and PUN Generation Resources. The 3PO’s, DAM Energy Only offers, AS offers and DAM energy bids as well as CRR offers and bids can be submitted and settled at a Generation Resource Node.

  - CCP Logical Resource Node represents the Settlement Point for 3PO’s for Combined Cycle Plant (CCP) configurations. Only 3PO’s, and AS offers for CCP configurations can be submitted and settled at a CCP Logical Resource Node.

  - CCU Resource Node represents the Settlement Point for the Combined Cycle Unit (CCU). Only DAM Energy Only offers, DAM energy bids and CRR bids and offers can be submitted and settled at a CCU Resource Node.

  - PUN Resource Node represents the Settlement Point at the PUN interconnection to ERCOT. Only DAM Energy Only offers, DAM energy bids and CRR bids and offers can be submitted and settled at a PUN Resource Node.

  - Multiple Generation Resources can be mapped to the same Resource Node, i.e. offers from different Generation Resources can be settled at the same Settlement Point.

  - Generation Resource can be mapped only to one Resource Node, i.e. offers from the Generation Resources can be settled only at one Settlement Price.

  - The Resource Nodes for “single” Resources and for Resources that are a component of a Combined Cycle Plant shall be identified prior to the identification of the PUN Resource Nodes.  Once those Resource Nodes have been located, the PUN Resource Nodes shall be located for PUN Resources that are not co-located with an existing Resource Node.

  - Resource Nodes shall not be located at the DC-ties.  (The DC-ties are Load Zones.)

  - Resource Nodes shall not be located at the Block Load Transfer buses.

  - Resource Nodes shall not be located for Load Resources.

  - Do not identify or locate Resource Nodes for “non-modeled” generators.  (Only modeled generators with the required EPS meters installed can have a Resource Node.)

  - Resource Node Location

    - First Fork Rule: Locate Resource Node at the first Electrical Bus with alternate paths starting from Generator Connectivity Node. Parallel network paths do not count as alternate paths.  If the Generation Resource is within a Private Use Network that has only one interconnection to the ERCOT Transmission Grid, locate the Resource Node at the Electrical Bus that is the interconnection point of the PUN to the ERCOT Transmission Grid.

    - EPS Meter Rule: Locate Resource Node close to EPS meter location, i.e. where energy is effectively metered.

    - Ownership Rule: Locate Resource Node at the Electrical Bus that essentially is the ERCOT interconnection point (if practical).

    - De-Energization Rule: Locate Resource Node at Electrical Bus that is less often de-energized, if alternate choices exist. LMPs for de-energized Resource Nodes are calculated using heuristic rules.

    - Transmission Constraint Rule: Transmission Elements between Resource Node and associated Connectivity Nodes cannot be constrained.

    - Generic Constraint Rule: Resources mapped into the same Resource Node can not be separated into different generic constraints.

    - Publicity Rule: Market Participants need to know where the Resource Nodes are located.

- Combined Cycle Plant (CCP) Modeling

  - CCP Logical Resource Node

- Each CCP Configuration for a train represents a CCP Logical Generation Resource.

- Each CCP Logical Generation Resource is mapped to a CCP Logical Resource Node.  All CCP Logical Generation Resources, i.e. all CCP Configurations for a train are mapped to the same CCP Logical Resource Node.

- Each CCP train has its own CCP Logical Resource Node, i.e. CCP Logical Generation Resources for different CCP trains are mapped to different CCP Logical Resource Nodes.

- Each CCP Logical Resource Node is a Settlement Point.

- CCP Logical Resource Nodes are used only for Resource specific 3PO’s and AS offers for CCP configurations.

  - CCU Resource Node

- Combined Cycle Unit (CCU) Resource Nodes are mapped to a CCP Logical Resource Node.

- A CCU Resource Node is the Electrical Bus determined by above rules (First Fork and others) starting from the Connectivity Node of the physical CCP train Resources.

- A CCU Resource Node is a Settlement Point.

- Only DAM Energy Only offers, DAM energy bids and CRR offers and bids can be submitted at CCU Resource Nodes.

  - CCP/CCU Resource Node Processing

- CRR cleared quantities are injected at Electrical Buses of CCU Resource Nodes.

- DAM LMP for CCU Resource Node is used as Settlement Price for CRR bids and offers that sink or source at CCU Resource Node.

- In DAM, energy for CCP Logical Resource is distributed to Connectivity Nodes of physical CCP Resources proportionally to the Resource capacities that are online in selected CCP configuration.

- In DAM, Shift Factor for CCP Logical Resource Node is calculated as capacity weighted average of Shift Factors for CCU Resource Nodes using the Resource capacities that are online in the selected CCP configuration as weights.

- DAM LMP for CCP Logical Resource Node is equal to weighted average of DAM LMPs at CCU Resource Nodes using the Resource capacities that are online in selected CCP configuration as weights.

- DAM LMP for CCP Logical Resource Node is used as Settlement Price for CCP 3PO’s.

- In RTM, Shift Factor for CCP Logical Resource Node is calculated as weighted average of Shift Factors for CCU Resource Nodes using the telemetered outputs of CCU Resources that are online in current CCP configuration as weights.

- RTM LMP for CCP Logical Resource Node is calculated as weighted average of LMPs at CCU Resource Nodes using telemetered outputs of CCU Resources that are online in current CCP configuration as weights.

- RTM Settlement Price for the CCP Logical Resource Node is the Base Point or time weighted average of RTM LMPs at Logical Resource Node.

- PUN Modeling

  - PUN Resource Node

- PUN Resource Node represents the Electrical Bus where an EPS meter is effectively located that is measuring the flow at a point of interconnection with ERCOT.

- PUN Resource Node is a Settlement Point.

- PUN Resource Node can not have mapped PUN Generation Resources.

- There can be several PUN Resource Nodes for one PUN.

- Only CRR and DAM Energy Only Offers can be submitted at PUN Resource Node.

- For DAM Energy Only Offers, power is injected at the Electrical Bus of the PUN Resource Node.

- Cleared quantities are settled at PUN Resource Node Settlement Prices.

  - Resource Nodes for PUN Generation Resource

  - Connectivity Node for PUN Generation Resource represents the Electrical Bus where physical Resource is connected.

  - Generator power outputs are injected at Connectivity Nodes.

  - Resource Node for PUN Generation Resource represents the Electrical Bus where Settlement Point for PUN Generation Resource is located.

  - Resource Node for PUN Generation Resource is defined using First Fork Rule.

  - A Resource Node for a PUN Generation Resource is a Settlement Point.

  - PUN energy offers represent net in respect to PUN self-served load.

  - 3PO and AS offers can be submitted for PUN Generation Resource for the excess capacity and energy not used to serve the PUN self serve load.

  - DAM Resource specific Offers for PUN Generation Resources are settled at Settlement Prices at Resource Nodes for PUN Generation Resources.

  - Constraints within in a PUN can be monitored but will not be enforced by DAM, RUC and SCED.

  - CCP Modeling within PUN

  - CCP trains within PUN are treated in the same way as any CCP within ERCOT.

  - CCP trains within PUN do not have CCU Resource Nodes.

- Settlement Points

- Settlement Point is a Resource Node, Load Zone or Hub.

- Resource Nodes include Generation Resource Nodes, CCP Logical Resource Nodes, CCU Resource Nodes and PUN Resource Nodes.

- Generation Resource Nodes within ERCOT as well as within PUN are Settlement Points.

- DAM Clearing and Settlements

- CRR offers and bids can be submitted using any Settlement Point (except CCP Logical Resource Nodes) as a CRR source and sink.

- Annual/Monthly CRRs acquired at de-energized Settlement Points will not be considered by SFT function.

- DAM Energy Only Offers can be submitted at any Settlement Point (except CCP Logical Resource Nodes).

- DAM Resource Specific energy offers that are submitted are mapped to  a Generation Resource Node or a CCP Logical Resource Node only.

- DAM Energy Bids can be submitted at Load Zones, Hubs, Generation Resource Nodes, CCU Resource Nodes and PUN Resource Nodes, i.e. at any Settlement Point except CCP Logical Resource Nodes.

- DAM/SASM AS Offers are Generation/Load Resource specific, not Settlement Point specific.

- DAM scheduling determines hourly quantities for CRR, energy and AS offers and bids.

- DAM pricing determines hourly LMPs for all Settlement Points.  For purposes of CRR clearing, a Settlement Point Price will be administratively determined by ERCOT at each de-energized CRR source or sink.

- DAM Settlements is based on DAM quantities and DAM Settlement Point Prices.

- RTM Clearing and Settlements

  - SCED dispatch determines Base Points for Generation Resources.

  - SCED pricing determines LMPs for all Generation Resource Nodes, CCP Logical Resource Nodes, CCU Resource Nodes, PUN Resource Nodes and all EPS meter locations.

  - RTM determines 15-minute Settlement Prices for each Settlement Point and each EPS meter location.  These prices are the Base Point weighted and time weighted average  of the Real-Time LMPs.

  - RTM Settlements uses 15-minute RTM Settlement Point Prices (prices at Settlement Points) and Settlement Prices (prices at EPS meter locations).

  - RTM Energy Settlement for the measured output from the Generation Resources uses the prices at the EPS meter locations as specified in section 6.6.3.

  - For purposes of CRR clearing, a Settlement Point Price will be administratively determined by ERCOT at each de-energized CRR source or sink.

- Summary of Allowed Activities

|  | ACTIVITIES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Settlement Points | 3PO | AS Offer | DAM Energy Only Offers | DAM Energy Bid | CRR Offers and Bids | QSE to QSE Transaction |
| Resource Node | Yes | Yes | Yes | Yes | Yes | Yes |
| CCU Resource Node | No | No | Yes | Yes | Yes | Yes |
| PUN Resource Node | No | No | Yes | Yes | Yes | Yes |
| CCP Logical Resource Node | Yes | Yes | No | No | No | No |

Note that Resource Specific offers (3PO and AS offers) are made for the Resource and the submittal does NOT specify a Resource Node.