| NPRR Number | 112 | NPRR Title | Emergency Base Point Price Revision |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Approved |
| Date of Decision |  | July 15, 2008 |  |  |
| Nodal Protocol Section Requiring Revision |  | 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT |  |  |
| Effective Date |  | Effective upon the Nodal Protocol Transition Plan’s Texas Nodal Market Implementation Date, as prescribed by zonal Protocol Section 21.12, Process for Transition to Nodal Market Protocol Sections. |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) clarifies the calculation of the Emergency Base Point Price per QSE per Resource by interval (EBPPR) variable. |  |  |
| Overall Market Benefit |  | The proposal establishes consistency in Energy Offer Curve extension methodology between the Market Management System (MMS) and Settlements. It also reduces the Charge for Emergency Power Increases (LAEMREAMT). |  |  |
| Overall Market Impact |  | None. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR112 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - NPRR112 was posted on 3/5/08. - On 2/29/08, the Transition Plan Task Force (TPTF) voting ballot was posted. - On 3/20/08, PRS considered NPRR112. - On 4/16/08, the Impact Analysis was posted. - On 4/18/08, PRS reviewed the Impact Analysis and the PRS Recommendation Report. - On 5/8/08, TAC considered NPRR112. - On 6/10/08, a second TPTF voting ballot was posted. - On 6/12/08, TPTF comments were posted. - On 6/19/08, PRS again considered NPRR112. - On 6/26/08, TAC considered NPRR112. - On 7/15/08, the ERCOT Board considered NPRR112. |  |  |
| PRS Decision |  | On 3/20/08, PRS voted to recommend approval of NPRR112 as submitted. The motion passed with three abstentions from the Consumer and the Independent Power Marketer (IPM)(2) Market Segments. All Market Segments were present for the vote. On 4/18/08, PRS unanimously voted to endorse the Recommendation Report and forward the document to TAC. Then PRS unanimously voted to endorse the Impact Analysis and forward the document to TAC. All Market Segments were present for the votes. On 6/19/08, PRS voted to recommend approval of NPRR112 as revised by TPTF comments. The motion passed unanimously with all Market Segments present. |  |  |
| Summary of PRS Discussion |  | On 3/20/08, PRS noted that the Consumers and IPM Market Segments abstained from the Transition Plan Task Force (TPTF) vote. ERCOT Staff explained that this NPRR corrects the gap when an Offer falls outside the emergency base point curve. Consumer participants questioned why an Entity would not submit an Energy Offer Curve up to the cap or High Dispatch Limit (HDL). ERCOT Staff responded that this NPRR represents the methodology chosen by the TPTF and is consistent with how the Security-Constrained Economic Dispatch (SCED) mitigates. There was no discussion preceding the PRS vote on 4/18/08. On 6/19/08, participants noted that TAC remanded this NPRR to address concerns raised by the Consumer Market Segment and the Independent Market Monitor (IMM). TPTF has revised the language to ensure that the Emergency Base Point Price will not exceed the Mitigated Offer Cap. |  |  |
| TAC Decision |  | On 5/8/08, the TAC Chair remanded NPRR112 to PRS to review and reconcile inconsistencies between the graphs and the text in the relevant Nodal Protocol language. On 6/26/08, TAC voted to recommend approval of NPRR112 as recommended by PRS. The motion passed with four opposing votes from the Consumer Market Segment and two abstentions from the Independent Retail Electric Provider (IREP) Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 5/8/08, ERCOT Staff stated that after review of the proposed Nodal Protocol language revision, it appeared that there were inconsistencies between the graphs and the text in the relevant Nodal Protocol language. ERCOT Staff recommended remanding NPRR112 to PRS in order to resolve these inconsistencies. On 6/26/08, some participants questioned whether this NPRR creates the correct incentives and whether there is any reason for Resources to submit offers that are not up to the System Wide Offer Cap. Proponents of the NPRR responded that there needs to be scarcity pricing mechanisms in an energy-only market; any form of economic withholding would be subject to review by regulatory bodies and the IMM. |  |  |
| Board Decision |  | On 7/15/08, the ERCOT Board approved NPRR112 as recommended by TAC. |  |  |

ERCOT/Market Segment Impacts and Benefits

| Assumptions | 1 | Formulas should accurately the intent of the Protocols and accuracy enhances transparency of the market. |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| TPTF 022108 | TPTF endorsed NPRR112 (see TPTF Ballot posted as an NPRR112 key document). |
| TPTF 061208 | Offered revisions to correct pricing issues. |

| Original Sponsor |  |
| --- | --- |
| Name | Bill Barnes |
| Company | ERCOT |
| Market Segment | Not applicable |

Proposed Protocol Language Revision

6.6.9.1	Payment for Emergency Power Increase Directed by ERCOT

(1)	If the Emergency Base Point issued to a Generation Resource is higher than the SCED Base Point immediately before the Emergency Condition, then ERCOT shall pay the QSE an additional compensation for the Resource at its Resource Node Settlement Point.  The payment for a given 15-minute Settlement Interval is calculated as follows:

EMREAMT q, r, p	=	(-1) * EMREPR q, r, p * EMREq, r, p

Where:

EMREPR q, r, p	=	Max (0, EBPWAPR q, r, p – RTSPP p)

EBPWAPR q, r, p	=	Σy(EBPPR q, r, p, y * EBP q, r, p, y * TLMP y) /

Σy(EBPq, r, p, y * TLMP y)

EMRE q, r, p	=	Max (0, Min (AEBP q, r, p * ¼ RTMG q,r,  pr) – ¼ * BP q, r, p)

AEBP q, r, p		=	Σy(EBP q, r, p, y * TLMPy / 3600)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EMREAMT q, r, p | $ | Emergency Energy Amount per QSE per Settlement Point per Resource—The payment to QSE q as additional compensation for the additional energy produced by Generation Resource r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| EMREPR q, r, p | $/MWh | Emergency Energy Price per QSE per Settlement Point per Resource—The compensation rate for the additional energy produced by Generation Resource r at Resource Node p represented by QSE q in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| EMRE q, r, p | MWh | Emergency Energy per QSE per Settlement Point per Resource—The additional energy produced by Generation Resource r at Resource Node p represented by QSE q in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| EBPWAPR q, r, p | $/MWh | Emergency Base Point Weighted Average Price per QSE per Settlement Point per Resource—The weighted average of the energy prices corresponding with the Emergency Base Points on the Energy Offer Curve for Resource r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval. |
| BP q, r, p | MW | Base Point per QSE per Settlement Point per Resource—The Base Point of Resource r at Resource Node p represented by QSE q from the SCED prior to the Emergency Condition. |
| AEBP q, r, p | MW | Aggregated Emergency Base Point— The Generation Resource’s aggregated Emergency Base Point, for the 15-minute Settlement Interval. |
| EBP q, r, p, y | MW | Emergency Base Point per QSE per Settlement Point per Resource by interval—The Emergency Base Point of Resource r at Resource Node p represented by QSE q for the Emergency Base Point interval or SCED interval y. If a Base Point instead of an Emergency Base Point is effective during the interval y, its value equals the Base Point. |
| EBPPR q, r, p, y | $/MWh | Emergency Base Point Price per QSE per Settlement Point per Resource by interval – The average incremental energy cost calculated per the Energy Offer Curve for the output levels between the SCED Base Point immediately before the Emergency Condition and the Emergency Base Point of Resource r at Resource Node p represented by QSE q for the Emergency Base Point interval or SCED interval y |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| RTMG q, r, p | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource—The metered generation of Resource r at Resource Node p represented by QSE q in Real-Time for the 15-minute Settlement Interval. |
| TLMP y | second | Duration of Emergency Base Point interval or SCED interval per interval—The duration of the portion of the Emergency Base Point interval or SCED interval y within the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |
| y | none | An Emergency Base Point interval or SCED interval that overlaps the 15-minute Settlement Interval. |

(2)	The extension of the Energy Offer Curve is used to calculate the Emergency Base Point Price.  If the Emergency Base Point MW value is greater than the largest MW value on the Energy Offer Curve submitted by the QSE for the Resource, then the Energy Offer Curve is extended to the Emergency Base Point MW value with a $/MWh value that is the Mitigated Offer Cap (pursuant to Section 4.4.9.4.1, Mitigated Offer Cap) for the highest MW output on the Energy Offer Curve submitted by the QSE for the Resource.

(3)	The total additional compensation to each QSE for emergency power increases of Generation Resources for the 15-minute Settlement Interval is calculated as follows:

EMREAMTQSETOT q	=	ΣrΣpEMREAMT q, r, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EMREAMTQSETOT q | $ | Emergency Energy Amount QSE Total per QSEThe total of the payments to QSE q as additional compensation for emergency power increases of the non-RMR Generation Resources represented by this QSE for the 15-minute Settlement Interval. |
| EMREAMT q, r, p | $ | Emergency Energy Amount per QSE per Settlement Point per Resource—The payment to QSE q as additional compensation for the additional energy produced by Generation Resource r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. |
| q | None | A QSE. |
| p | None | A Resource Node Settlement Point. |
| r | None | A Generation Resource. |