2705 West Lake Drive

Taylor, Texas 76574

(512) 248-6800

HTTP://WWW.ERCOT.COM

SIG0011 Net Generation

Sai Moorty

06/11/2008

Revision History

| Revision | Comments | Date | Author |
| --- | --- | --- | --- |
| V1.0 | Initial Version | 04/15/2008 | Sai Moorty |
| V2.0 | Updated to include implementation plan. Solution design complete. | 05/20/2008 | Sai Moorty |
| V3.0 | Removed implementation verification | 06/11/2008 | Tobi Brenner |

Table of Contents

1 - Problem Statement	4

2 - Solution Design	5

3 – Implementation Activities	5

4 – Comments	5

# 1 - Problem Statement

The definition of Net Generation from a generation resource and its use by ERCOT systems needs clarity. The need for clarity is mainly for PUN facilities with self-serve load and generation facilities serving associated loads (i.e. station auxiliary loads) from a common switchyard that meets the  400 yard rule defined in Protocol 10.3.2.3 (3).

The two definitions of Net Generation from a generation resource are listed below:

Definition 1: This definition is being used for Go-Live by All Nodal Systems: Net Generation for a Generation Resource is defined as:

Net Generation = Gross Generation Unit Output – (Generation Unit Auxiliary Load+ the amount of station auxiliary load assigned to the Generation Unit, as determined by the QSE/Resource Entity) – (the amount of PUN self-serve load assigned to the Generation Unit, as determined by the QSE/Resource Entity in accordance with the guidelines described in SIG0002, Private Use Networks).

Definition 2: This is an alternative definition that is NOT going to be implemented for Go-Live: Net Generation for a generator is defined as:

Net Generation = Gross – (Unit Auxiliary Load) – (amount of station auxiliary load as assigned to the Generation Unit by the QSE/Resource Entity).

The protocol sections that provide related information are:

- Section 2: Net Generation: “Gross generation less station auxiliary Load or other internal unit power requirements metered or adjusted to the point of interconnection with the ERCOT Transmission Grid at the common switchyard”.

- Section 3.10.7.2 (8):Modeling of Resources and Transmission Loads: “Loads associated with a Generation Resource in a common switchyard as defined in Section 10.3.2.3, Generation Netting for ERCOT Polled Settlement Meters, and served through a transformer owned by the Generation Entity is treated as an auxiliary Load and must be netted first against any generation meeting the requirements under Section 10.3.2.3, Generation Netting for ERCOT Polled Settlement Meters.”

- Section 3.10,7.3 (4): Modeling of Private use Networks: “ERCOT shall ensure the Network Operations Model properly models the physical effect of the loss of generators and transmission elements on the ERCOT Transmission Grid equipment loading, voltage, and stability.”

- Section 6.5.5.2 (a): Operational Data Requirements:” Net real power (in MW) as measured by installed power metering or as calculated in accordance with ERCOT Operating Guides based on metered gross real power and conversion constants determined by the Resource Entity and provided to ERCOT as a result of Section 3.7, Resource Parameters. Net real power represents the actual generation of a Resource for all real power dispatch purposes, including use in SCED, determination of HASL, HDL, LDL and LASL, and is consistent with telemetered HSL and LSL; “

- Section 10.3.2.3: Generation Netting for EPS Meters:

Definition 1 is consistent with Nodal protocols except for item 3) above for RUC.

# 2 - Solution Design

Definition 1 for Net Generation will be implemented by ALL Nodal systems for Go-Live.

Key Driving Points:

- Definition 1 for Net Generation will be implemented by ALL Nodal systems for Go-Live.

- Net Generation represents the output of Generation Resources injected  into the ERCOT Grid and NOT the physical Net Output of the Generation Resource.

- Definition 1 requires that the QSE or Resource Entity that is responsible for a facility utilizing a net metering arrangement or a PUN, with self-serve Load, manage the provisioning of data for these Generation Resources as net injections (Surplus >0) into the ERCOT Grid for:

- Telemetry to ERCOT of Net Generation.

- Receipt of Base points from ERCOT and translating them to appropriate controls signals to control generation resource output.

- Submission of resource specific data to represent ONLY the net injection (surplus>0) into the ERCOT grid for COP, Three-Part-Offers, Energy Only Offers, AS Offers.

Design:

Refer to SIG PUN paper (SIG0002) and apply same logic to Self Serve Load facilities as for PUN except that for Self Serve facilities, a PUN resource node should not be defined.

# 3 – Implementation Activities

Refer to Implementation Activities of SIG PUN paper (SIG0002) and apply same logic to Self Serve Load facilities as for PUN.

# 4 – Comments

Please note that all implementation details will be maintained in the integrated schedule.