2705 West Lake Drive

Taylor, Texas 76574

(512) 248-6800

HTTP://WWW.ERCOT.COM

SIG 0020 Rounding of MW Values

Version 5.3

Kenneth Ragsdale

9/18/2008

Revision History

| Revision | Comments | Date | Author |
| --- | --- | --- | --- |
| V1.0 | Initial Version | 4/20/2008 | Kenneth Ragsdale |
| V2.0 | Incorporated changes from CRR and also incorporated significant change to use a granularity of tenths of MW in almost all cases. Incorporated Implementation dates. Solution design complete. | 5/20/2008 | Kenneth Ragsdale and Tobi Brenner |
| V3.0 | Removed implementation schedule and verification | 6/11/2008 | Tobi Brenner |
| V4.0 | Incorporated comments from Market Participants received after the 7-21-08 release. | 8/19/2008 | Kenneth Ragsdale |
| V5.0 | Incorporated comments from ERCOT internal discussion held 8-29-08. | 8/29/2008 | Kenneth Ragsdale |
| V5.1 | Updated after discussions with ABB | 9/15/2008 | Sai Moorty |
| V5.2 | A few minor additions in preparation for internal meeting. | 9/15/2008 | Kenneth Ragsdale |
| V5.3 | Added a section for EMS and MIS. Made a few other changes identified at the 9-17-08 internal meeting. | 9/182008 | Kenneth Ragsdale |

Table of Contents

1 - Problem Statement	4

2 - Solution Design	4

3 – Implementation Activities	7

4 – Comments	7

# 1 - Problem Statement

The Nodal Protocols and the approved MMS requirements are silent on the subject of how many significant figures are to be used in submissions and awards of Ancillary Services and Energy Offers and Bids.  CRR is the only system which specifically describes the granularity of measurement.  This specification was changed in the Protocols for PCRR allocations and CRR auctions from whole MWs to 0.1 MWs in NPRR012.

A policy on rounding and how many significant figures should be used for each particular situation needs to be developed.  This policy would need to be embraced and followed by the various ERCOT systems and the Market Participants.

Background:

In Zonal, the market systems were deliberately designed in order to generate market awards to the nearest MW.  Currently in the Nodal market there is the capability for Market Participants to submit offers and receive awards in decimals from 0.1 to 0.001.  NPRR 012 was approved in July 2007 and specified the following for CRR:

Quantities are measured in MWs, with granularity of tenths of MWs (0.1 MW)

ERCOT shall allocate all PCRRs in quantities truncated to the nearest tenth MW (0.1 MW).

ERCOT shall award CRRs in quantities truncated to the nearest tenth MW (0.1 MW).

# 2 - Solution Design

The policy should provide detailed information on the number of significant figures to be used for any quantities associated with obligations, offers, bids, awards and QSE to QSE transactions.  Additionally, the policy should clearly describe any kind of rounding or truncating that needs to be performed and when that rounding or truncating should be executed during the calculation time line.  The policy should also describe the expectation from Market Participant submittals and the input screens seen by Market Participants.

Policy summary:  All quantities for obligations, bids, offers, awards, and output schedules shall be provided in tenths of MWs (0.1 MW).ERCOT shall provide awarded and allocated quantities with a granularity of tenths of MWs and these values shall be the amount after rounding to the first digit to the right of the decimal point.  All final dollar amount calculations performed by the Settlement system shall be rounded to the nearest penny, as necessary in the last step of any string of calculations.  Standard rounding shall be performed.  If the first digit to be dropped is less than 5, the last retained digit is not changed.  If the first digit to be dropped is greater than or equal to 5, the last digit retained is increased by 1.

This policy has been adopted based on the precedence set by NPRR012 and also because in most instances quantities measured in tenths of MWs reflect an appropriate level of precision for the obligations, allocations and awards.

| Function Name | Design Decisions |
| --- | --- |
| Registration | - Generation Resource Parameters (e.g. HRL, LRL, etc) for Registration of Resources shall be submitted in quantities of tenths of MWs or MWhs, |
| S&B | - Load Ratio Shares (LRS) calculated by Lodestar to be used to calculate AS obligations shall be provided with as many significant figures that are in Lodestar. (This can be many significant figures, for example more than 9.) - Settlement calculations shall be performed as specified in the Requirement documents. Rounding to the nearest penny shall be performed only in the last step of a calculation. |
| MMS | - Ancillary Services Obligations shall be issued to the Market Participants in quantities of tenths of MWs. The obligation amount for each QSE shall be the amount after rounding to the first digit to the right of the decimal point. All calculated Obligations less than 1 MW (configurable 1 or 0.1) will be discarded. There will be a configuration parameter that will set the minimum AS obligation for a QSE and the current default setting shall be 1 MW. The configuration parameter shall be 1 MW or 0.1 MW. An NPRR is required before the configuration parameter of 0.1 MW can be used. Some testing may be done with the configuration parameter set to 0.1 MW before an NPRR is written or approved. If the configuration parameter is set to 0.1 MW the minimum amount of Obligation per QSE will be 0.1 MW and calculated Obligations less than 0.1 MW will be discarded. - Acquisition of slightly more or less than the desired A/S ERCOT-wide service than the target amount due to rounding is acceptable. For example, if the target was to acquire 2300 MW of Responsive Reserve, ERCOT would discard all QSE Obligations less than 1 MW resulting in a total that is less than the 2300 MW target. The amount procured is the difference between the target (ERCOT-wide System Requirement) and the Self Arranged amount. After rounding the Awards for each QSE to tenths of MWs, the total for ERCOT may be a little more or a little less than 2300 MW. - Ancillary Services Self Arranged Quantities shall be submitted in quantities of tenths of MWs. The minimum amount that may be self arranged per QSE is 1 MW (configurable 1 or 0.1). There will be a configuration parameter that will set the minimum AS Self Arranged amount that can be submitted by a QSE and the current default setting shall be 1 MW. The configuration parameter shall be 1 MW or 0.1 MW. Note that for RRS Self Arranged - each subtype from generation, controllable and non-controllable load resources must be zero or greater than or equal to this configuration parameter. An NPRR is required before the configuration parameter of 0.1 MW can be used. Some testing may be done with the configuration parameter set to 0.1 MW before an NPRR is written or approved. - Ancillary Services Offers in the DAM or SASM shall be submitted in quantities of tenths of MWs. The minimum amount per Resource for each Ancillary Service product that may be offered is one MW. (See section 4.4.7.2.1 paragraph (4).) There will be a configuration parameter that will set the minimum AS Offer submitted for DAM or SASM and the current default setting shall be 1 MW. The configuration parameter shall be 1 MW or 0.1 MW. An NPRR is required before the configuration parameter of 0.1 MW can be used. Some testing may be done with the configuration parameter set to 0.1 MW before an NPRR is written or approved. - Ancillary Services Trades shall be submitted in quantities of tenths of MWs. - Ancillary Services Awards shall be rounded to the first digit after the decimal point resulting in quantities of tenths of MWs. Resource specific awards can be less than 1 MW. - Energy and Capacity QSE to QSE trades shall be submitted in quantities of tenths of MWs. - Energy Offers and Bids shall be submitted in quantities of tenths of MWs. The minimum amount per Resource for each Energy Offer Curve that may be offered is one MW. (See section 4.4.9.3.1 paragraph (3). The minimum amount for each DAM Energy-Only Offer Curve that may be offered is one MW. (See section 4.4.9.5.1 paragraph (3). The minimum amount for each DAM Energy Bid that may be bid is one MW. (See section 4.4.9.6.1 paragraph (2). There will be a configuration parameter that will set the minimum Energy Offer submitted (Resource specific or DAM Energy Only) and the current default setting shall be 1 MW. The configuration parameter shall be 1 MW or 0.1 MW. An NPRR is required for each of the three sections identified above before the configuration parameter of 0.1 MW can be used. Some testing may be done with the configuration parameter set to 0.1 MW before an NPRR is written or approved. - Output Schedules submitted by QSEs shall be submitted in quantities of tenths of MWs. - Energy Offer and Bid awards shall be rounded to the first digit after the decimal point resulting in quantities of tenths of MWs. - COP information shall be submitted in quantities of tenths of MWs. - Generation Resource Parameters (e.g. HRL, LRL, etc) for Registration of Resources shall be submitted in quantities of tenths of MWs or MWhs, - Displays and interfaces seen by Market Participants shall reflect this policy. - Displays and interfaces seen by ERCOT Operators shall reflect this policy. Phased implementation: Initially no changes to the displays. Upon review on a display by display basis, individual displays will be changed through display formatting only. Note: this activity may complete post go-live. - Internal calculations performed by the MMS system should not use rounded or truncated values. All rounding/truncating should be performed as part of post processing. |
| CMM | - No changes required |
| CRR | - CRR offers, bids, trades, allocations and awards shall be in quantities of tenths of MWs. - Displays and interfaces shall reflect this policy. |
| EIP | - The EIP layer should not round or truncate values determined and provided by MMS post processing. |
| EMS | - Emergency Base Points from EMS shall be rounded to the first digit after the decimal point. Base Points and LMP’s from MMS will be passed through EMS with no changes. |
| MIS | - Information provided by the Market Information System shall conform to the upstream source systems. |

# 3 – Implementation Activities

| MS | Milestone Description | Assigned to | Pred’r MS |
| --- | --- | --- | --- |
| 1 | Registration; Confirm the RARF form and Siebel do not have limitations. | Hope Parrish | N/A |
| 2 | S&B; Confirm that Lodestar has been coded as specified in this policy. | Hope Parrish | N/A |
| 3 | MMS; Determine and complete changes necessary to the ABB system in order to follow the policy. | Murray Nixon | N/A |
| 4 | CRR; Confirm that the CRR system has the ability to accept and generate values to the tenth of a MW. | Beth Garza | N/A |
| 5 | EIP; Confirm that the EIP layer does not round or truncate any of the values provided by the Market Participants or MMS post processing. | Stephen Kerr | N/A |
| 6 | EMS; Confirm that EMS is prepared to follow the policy | David Hackett | N/A |
| 7 | MIS; Confirm that MIS is prepared to follow the policy | Brett Economides | N/A |

# 4 – Comments

Please note that all implementation details will be maintained in the integrated schedule.