10/7/2008 11:32 AM
BUSINESS REQUIREMENTS 
Page 6 of 6

Project 5 Retail Market Transactions


	ID
	REQUIREMENT
	BP
	Recommended Timelines
	TX SET

	BR- 001
	Ability  for the TDSP to notify ERCOT and REP of Record via an automated transaction that an ESIID has a provisioned advanced meter 

Note:  This will apply to the backlog of meters that have already been provisioned as well as meters that get provisioned going forward

Possible Solutions:  Indicate the presence of an AMS meter on an 814_20 

Recommendation – meter type we would add AMS code in front of existing k1 – kh meter type (AMS k1) REF~MT~Meter Type.  This will require change control to make the graybox corrections.

Assumption:  ERCOT does not and /or will not settle on meter type in the REF~MT of the 814_20 to settle the ESI ID as an AMS meter.

AMS meter type on an 814_20 represented a provisioned a provisioned AMS meter according to the PUCT rule.
REF~MT~AMS_KHMON

 REF~MT~AMS_COMBOMON

Types of consumption

AMS K1 Kilowatt Demand (kw)

AMS k2 Kilvolt amperers Reactive

Ams K3

Ams K4 Kilovolt amperes 

AMS KH kilowatt hour

Metering intervals reported for billing purposes

Nnn number of mins form 001 -999

Day daily

Mon monthly

Example: 

KHMON – Kilowatt hours per month

K1015 kilowatt per 15


	002.01
	· n/a
	TX SET PRR

	BR- 001.01
	Ability for the TDSP enrollment responses to indicate the presence of an AMS meter 

Enrollment Transactions impacted: Provided details of each trans.
· 814_20

· 814_04 Switch CR 
· 814_05 CR Gaining

· 814_14 MT (CR) 

· 814_22 CSA 

· 867_03 monthly only (not used when BPT07 = F) 

· PTD = SU or PL (would add AMS code)
Assumption – Provisioned – AMS meter types will be indicated in the non IDR meter type segments of the 867_03 monthly usage trans

Note: BPT01 (did not get note here) 

	002.02 002.03
	· n/a
	TX SET

	BR- 001.02
	Ability to indicate on monthly usage transaction that the meter is an AMS meter
· 867_03 monthly only 
· Meter type we would add new AMS code in front of existing K1 –KH meter type (i.e... AMS K1) in the 
· REF~MT~meter type.  This will require a change control to make graybox corrections.


	002.08
	· n/a
	TX SET

	BR- 002
	Ability for any REP to identify whether any ESIID has an AMS meter or not on an on demand on going basis

Possible Solution:  Ability for ERCOT to display AMS meter indicator in the TDSP ESIID extracts and on the TML 
May require an SCR instead of a PRR since this is not indentified specifically in the protocols.

· SCR would be written to update the current TDSP ESI ID extracts and display on the TML that includes to include the REF~MT~meter type pulled from the 814_20.
· TX set graybox shows if there are mult meter types on one ESI ID then only one meter type will be sent – examples AMS_ COMBO

· If for any reason ERCOT receives more than one metere type in the same trans then ERCOT should reject the 814_20.

	002.01
	· n/a
	PRR 



	BR- 003A
	The ability for the TDSP to disconnect AMS meters remotely for non pay.   


	002.05
	· n/a
	N/A

	BR- 003B
	The ability for the TDSP to reconnect AMS meters remotely.   

Johnny – will this affect the timeline on TX SET..  k.scott – tariff timeline..kmiller – may want to market test (flight)..
	002.05
	· n/a
	N/A


	BR- 003.01
	The ability for the TDSP to reconnect AMS meters for customers on a prepay product within one hour of notice from the Rep of Record (commission workshop to be scheduled)
There may be a need for 650_01 should carry a prepay flag. Within 1 hour of the notice

Commission is working a prepay rule and customer protection..

Discussion on the ability to identify a prepay customer.   

Depending upon the rule outcome, if it is determined that prepay reconnects are on a different timeline than other reconnects TX SET will need to revised the 650_01 transaction to support a pre-pay notification from CR.
Update to Retail Market Guide will be needed to support market processes for pre-pay customers.

	002.05
	· n/a
	TBD


	BR- 003.02
	Ability to identify an ESIID as a prepay customer

(commission workshop to be scheduled)
If this is a need for the TDSP, why would this business requirement be necessary?  Would it be the same as BR-003-01?  This is unclear to TX SET as to what the requirement is addressing and to whom.

Depending upon the rule outcome, if it is determined that prepay reconnects are on a different timeline than other reconnects TX SET will need to revised the 650_01 transaction to support a pre-pay notification from CR.

	002.05
	· n/a
	TBD

	BR- 004
	Ability to reduce the Rep of Record advance notice required to be given to a TDSP for disconnect due to non pay for ESIID with AMS
TX SET determined the existing 650s transaction will support same day DNP/Reconnect and the two advance notice could be removed with tariffs and/or customer protection rules which is outside of TX SET responsibility.


	002.05
	· Same business day if received before 3 p.m. 


	RMG TARIFF PUC RULE 

	BR- 005
	Ability for the TDSP to reduce the time for disconnect and reconnect once a request is received from the REP of Record
TX SET determined the existing 650s transaction supports the request for same day reconnects requests, with reference to disconnects for non-pay see response BR-004.
Field Operational Day 

	002.05
	· Reconnect – Within 1 business hour if received by 5 p.m. on a business day

· DNP – Same business day if received before 3 p.m.


	RMG TARIFF

PUC RULE

	BR- 006
	Ability for Reps to do their own meter reads over the TDSP web portal or FTP site until there is a common web portal for the market 
If FTP Site, TX SET would be able to develop file format and data elements once those items have been defined and finalized by AMIT.
If TDSP Web Portal, TX SET would be willing to develop RMG section with applicable information. (Intermediate - solution at this time)


	002.06
	· n/a
	RMG 

	BR- 007
	Ability for the TDSP to reduce the 2 business day advance notification requirement for move out on an AMS meter once a request is received from the REP of Record.
TX SET determined the existing 814_24 MVO transactions will support same day request and the two advance notice could be removed with tariffs and/or customer protection rules which is outside of TX SET responsibility.


	002.010
	· 1 business day advance notice 
	RMG TARIFF PUC RULE

	BR- 008
	Ability to perform real-time enrollment switching for AMS meters (see task 148)  Creed – clarify if it is either enrollment or meter switching… 
	
	· TBD
	

	BR- 001
	
	
	· 
	


	Solutions Cube Group LLC ® ©

Irving, Texas 
	AMS Project

Business Requirements - Project 5 - Retail Transactions.doc



