Advanced Metering Interim Solution Use Cases

| Use Case Number: 08 | Related Business Requirements: BR-006 BR-006.01 BR-006.02 |  | Date: 10/16/2008 | Version: v.01 |
| --- | --- | --- | --- | --- |
| High Level Description |  | ERCOT will settle ESI IDs that receive AMI interval data on that data. Any NIDR data received on those ESI IDs will be excluded from the settlement process. If insufficient data exists for those ESI IDs, they will be settled on an applicable profile. |  |  |
| Requirement |  | BR-006: Ability to settle residential ESIIDs on actual 15 minute data for those ESIIDs that have an AMS meter BR-006.01: Ability to settle the residential class ESIIDs with an AMS meter on a default 15 minute applicable profile (residential / business) profile when insufficient / no historical data is available for the ESIID. BR-006.02: Ability to exclude 867_03 from the ERCOT settlement process for ESIIDs which are included in the LSE file sent by the TDSP (non-interval 867_03). |  |  |
| Pre-Conditions/Trigger |  | - ESI ID is set up in ERCOT’s system to receive interval data. |  |  |
| Scenario 1: TDSP sends only IDR data for ESI ID. |  | - TDSP sends interval data for the ESI ID. - ERCOT validates, accepts, and loads IDR data for the ESI ID. - ERCOT uses the IDR data in the settlement process. |  |  |
| Scenario 2: TDSP sends both IDR and NIDR data for ESI ID. |  | - TDSP sends a NIDR 867_03 for the ESI ID. ERCOT validates, accepts, and forwards the 867_03 to the forwarding CR. - TDSP sends interval data for the same ESI ID. - ERCOT validates, accepts, and loads IDR data for the ESI ID. - ERCOT uses the IDR data in the settlement process. - The NIDR data is excluded from settlement. |  |  |
| Scenario 3: TDSP sends only NIDR data for ESI ID. |  | - TDSP sends a NIDR 867_03 for the ESI ID. ERCOT validates, accepts, and forwards the 867_03 to the forwarding CR. - Since ERCOT does not have IDR data for the ESI ID, the ESI ID is settled based on the applicable profile. - The NIDR data is excluded from settlement even though no IDR data exists for the ESI ID. |  |  |
| Scenario 3: TDSP sends both IDR and NIDR data for ESI ID. ERCOT rejects the IDR data. |  | - TDSP sends interval data for the ESI ID. - ERCOT validates and rejects the IDR data for the ESI ID. - TDSP sends a NIDR 867_03 for the ESI ID. ERCOT validates, accepts, and forwards the 867_03 to the forwarding CR. - Since ERCOT does not have IDR data for the ESI ID, the ESI ID is settled based on the applicable profile. - The NIDR data is excluded from settlement even though no IDR data exists for the ESI ID. |  |  |