Advanced Metering Interim Solution Use Cases

| Use Case Number: 16 | Related Business Requirements: BR-004 |  | Date: 11/3/2008 | Version: .01 |
| --- | --- | --- | --- | --- |
| High Level Description |  | TDSPs will provide 15 minute interval data for AMS meters on their FTP sites. |  |  |
| Requirement |  | BR-004: Ability for the TDSPs to provide raw non-VEE 15 minute ESIID KWH usage load and generation information daily for REPs on a day after basis through an FTP site. See TSK-128 Note: • The information in the FTP by ESIID will support the ability for REPs to identify which ESIIDs have AMS provisioned meters • REPs will have access to their customer’s AMS meter data within 30 days after the meters are provisioned. |  |  |
| Assumptions |  | - DEV process is currently based on 727 extract. Once IDR usage data is removed from the 727 extract, DEV issues on that interval data will be based on the data sent to ERCOT and not the data picked up from the FTP site. This data can be obtained from ERCOT’s web services. (Note: This only applies to DEV issues on the interval data.) |  |  |

|  | Oncor | AEP | CNP | TNMP |
| --- | --- | --- | --- | --- |
| When will raw data be posted? | N/A | N/A | 12:00:00 (noon) following calendar day | Unknown at this time |
| When will VEE’s data be posted to FTP site? | 12:00:00 (noon) following calendar day | 12:00:00 (noon) following calendar day | On the 4th day | Unknown at this time |
| When will VEE’d data be sent to ERCOT? | 12:00:00 (noon) following calendar day | 12:00:00 (noon) following calendar day | On the 4th day | Unknown at this time |
| What if a day is missing some intervals? | Gaps will be estimated w/status of E prior to posting | Gaps will be estimated w/status of E prior to posting | File will be posted with NULL values for any gaps. | Unknown at this time |
| What if a day is missing all intervals? | Gaps will be estimated w/status of E prior to posting | Gaps will be estimated w/status of E prior to posting | File will be posted with all NULL values | Unknown at this time |

Operating Day 7/21

12:00 (noon) following day:

- Oncor posts VEE’d data and sends to ERCOT

- AEP posts VEE’d data and sends to ERCOT

- CNP posts RAW data

7/22

7/23

7/24

7/25

7/26

7/27

7/28

7/29

12:00 (noon) 4th day

- CNP posts VEE’d data and sends to ERCOT

- ERCOT Initial Settlement