AMIT Interim Solution Use Case 16

Advanced Metering Interim Solution Use Cases
	Use Case Number: 16
	Related Business Requirements:

BR-004


	Date:  11/3/2008
	Version: .01

	

	High Level Description
	TDSPs will provide 15 minute interval data for AMS meters on their FTP sites.

	Requirement
	BR-004:  Ability for the TDSPs to provide raw non-VEE 15 minute ESIID KWH usage load and generation information daily for REPs on a day after basis through an FTP site. 

See TSK-128

Note:

• The information in the FTP by ESIID will support the ability for REPs to identify which ESIIDs have AMS provisioned meters

• REPs will have access to their customer’s AMS meter data within 30 days after the meters are provisioned.

	Assumptions
	1. DEV process is currently based on 727 extract.  Once IDR usage data is removed from the 727 extract, DEV issues on that interval data will be based on the data sent to ERCOT and not the data picked up from the FTP site.  This data can be obtained from ERCOT’s web services.  (Note:  This only applies to DEV issues on the interval data.)  


Reorganize this into time order.
	
	Oncor
	AEP
	CNP
	TNMP

	When will raw data be posted?
	N/A
	N/A
	12:00:00 (noon) following calendar day
	Unknown at this time

	When will VEE’d data be posted to FTP site?
	12:00:00 (noon) following calendar day
	12:00:00 (noon) following calendar day
	On the 4th day at 12:00:0 (noon)
	Unknown at this time

	When will VEE’d data be sent to ERCOT?
	12:00:00 (noon) following calendar day
	12:00:00 (noon) following calendar day
	On the 4th day at 12:00:0 (noon)
	Unknown at this time

	What if a day is missing 8 or less contiguous intervals as of Day 0?  (2 hours or less)
	Gaps will be estimated w/status of E prior to posting – linear interpolation
	Unknown at this time
	File will be posted with NULL values for any gaps.
	Unknown at this time

	What if a day is missing 8 or less contiguous intervals as of Day 4?  (2 hours or less)
	Gaps will be estimated w/status of E prior to posting – linear interpolation
	Unknown at this time
	Gaps will be estimated.
	Unknown at this time

	What if a day is missing more than 8 intervals as of Day 0?
	Gaps will be estimated w/status of E prior to posting – proxy day lookup or profile methodology
	Unknown at this time
	File will be posted with NULL values for any gaps.
	Unknown at this time

	What if a day is missing more than 8 intervals as of Day 4?
	No new file until actual data is received.
	Unknown at this time
	Gaps will be estimated.
	Unknown at this time

	What if a day is missing all intervals?
	Gaps will be estimated w/status of E prior to posting
	File will be posted with all NULL values
	File will be posted with all NULL values
	Unknown at this time


[image: image1.emf] 

7/21 

[image: image2.emf] 

7/21 

[image: image3.emf] 

7/21 

[image: image4.emf] 

7/2 2  

[image: image5.emf] 

7/21 


7/23�





Operating Day 7/21





7/22





7/26�





12:00 (noon) following day:


Oncor posts VEE’d data and sends to ERCOT


AEP posts VEE’d data and sends to ERCOT


CNP posts RAW data





7/24





7/25





7/27�





7/28�





7/29�





12:00 (noon) 4th day


CNP posts VEE’d data and sends to ERCOT





ERCOT Initial Settlement








4 of 4

