| PRR Number | 777 | PRR Title | WGR QSE Metric Correction |
| --- | --- | --- | --- |
| Date |  | December 2, 2008 |  |
| Submitter’s Information |  |  |  |
| Name |  | John Dumas |  |
| E-mail Address |  | jdumas@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | (512) 248-3195 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable. |  |

Comments

Protocol Revision Request (PRR) 763, Use of WGRPP as Planned Operating Level in Day Ahead Resource Plan for WGRs, requires all Wind-powered Generation Resources (WGRs) to use the ERCOT provided Wind-powered Generation Resource Production Potential (WGRPP) forecast as the planned operating level in the Resource Plan during Replacement Reserve Service (RPRS) procurements.  This change enables ERCOT to procure capacity for the next Operating Day based upon a conservative wind forecast.  The change has made the wind-only Qualified Scheduling Entity (QSE) schedules no longer meaningful in the Day Ahead process.  However, the wind-only QSEs can update their High Sustainable Limits (HSLs) with a value that represents their forecast and aligns their Resource Plan HSLs with their energy schedules.  Protocol Section 4.1.1, Day Ahead Scheduling Process, states that ERCOT will verify that Resource Plans are sufficient to meet QSEs energy schedules.  ERCOT proposes a change that would verify that all QSEs have sufficient capacity in their Resource Plans to meet their energy schedules.

| Overall Market Benefit |  |
| --- | --- |
| Overall Market Impact |  |
| Consumer Impact |  |

Revised Proposed Protocol Language

4.10.5	Day Ahead Zonal Schedule Measure

The “Day Ahead Zonal Schedule Measure” compares each QSE’s energy schedule to the QSE’s aggregated HSLs at the time a Day Ahead Schedule validation, as described in Section 4.7, Validation and Correction of Schedule Data, is run and approved.  The QSE’s energy schedule and the aggregated HSLs for all twenty-four (24) hours of the next day are recorded at the time of the Day Ahead Schedule validation.  The greatest of the four (4)15-minute intervals in an hour is selected to represent the QSE’s energy schedule.  The HSLs for all Resources are aggregated by QSE for each hour to create the QSE’s aggregated HSLs.  If multiple Day Ahead Schedule validations are run on a particular day, only the first approved Day Ahead Schedule validation is used.  Only hours when the energy schedule is greater than zero (0) MW are considered in this measure.

An Occurrence is recorded for a given hour if the QSE’s energy schedule plus Ancillary Service Obligations are greater than the aggregated HSLs.  Only one (1) Occurrence can be recorded per hour per QSE.  To determine the QSE Measure Score for the Day Ahead Zonal Schedule Measure, Occurrences are summed for every hour in a given month, and divided by the total number of hours in that month where the QSE’s energy schedule for a particular hour is greater than zero (0) MW.

4.10.6	Adjustment Period Zonal Schedule Measure

The “Adjustment Period Zonal Schedule Measure” compares each QSE’s zonal energy schedule to the aggregated planned operating level for that Congestion Zone before the start of the Operating Hour.  Each QSE’s zonal energy schedule used to calculate this measure is taken at the end of the Adjustment Period.  The last Resource Plan submitted before the start of the Operating Hour but after the end of the Adjustment Period is used.  The QSE’s zonal energy schedules for each fifteen (15) minute interval in an hour are averaged over the entire hour to create the QSE’s average zonal energy schedule.  The planned operating level for all Resources in a Congestion Zone are aggregated by QSE for each hour to create the QSE’s aggregated planned operating

level.  Only hours when the zonal energy schedule is greater than zero (0) MW are considered in this measure.

An Occurrence is recorded for a Congestion Zone for a given hour if the QSE’s zonal energy schedule and the aggregated planned operating level for that Congestion Zone differ by the greater of two percent (2%) of the zonal energy schedule or one (1) MW.  Only one (1) Occurrence can be recorded per Congestion Zone per hour per QSE.  To determine the QSE Measure Score for the Adjustment Period Zonal Schedule Measure, Occurrences are summed for all Congestion Zones for every hour in a given month, and divided by the number of Congestion Zones multiplied by the total number of hours in that month when the QSE’s zonal energy schedule in a Congestion Zone for a particular hour is greater than zero (0) MW.

Any hour where a QSE updated its Resource Plan before the start of or during the Operating Hour but after the end of the Adjustment Period and failed to pass this measure for that interval will be excluded in the calculation of this measure.