LSE File Format Specifications:

PR80027 Advanced Metering Interim Project

Version 1.1

| Date | Version | Description | Author(s) |
| --- | --- | --- | --- |
| 12/8/2008 | 1.0 | Final version distributed to Market. | Catherine Meiners |
| 2/6/2008 | 1.1 | Corrected typo on row 00000004. Date format should be YYYYMMDDHHMMSS. Added title page and versioning information. | Catherine Meiners |

LSE File Definition

- File Size

In order to process data records most efficiently, ERCOT recommends that files contain between 10,000 and 50,000 data records and that the files be zipped prior to PGP encryption and compression.  The combination of zipping and encrypting will result in a significant savings in internet bandwidth, decryption time, and storage.

| File Size in Data Records | Raw Size | PGP Encrypted Only | Zipped Only | Both Zipped and Encrypted |
| --- | --- | --- | --- | --- |
| 10,000 | 18,022.40 KB | 140 KB | 124 KB | 2.28 KB |
| 25,000 | 45,158.40 KB | 344 KB | 308 KB | 3.64 KB |
| 50,000 | 90,316.80 KB | 686 KB | 616 KB | 5.81 KB |

- File Naming Convention

The recommended file naming convention is <DUNS><ReportName><DateTime><Counter>.lse <.optional data>in addition to any application file naming conventions used in transmitting the file.  For example, “999999999IntervalData20081227113001123.lse<.optional data>” where:

| Element | Explanation | Format |
| --- | --- | --- |
| DUNS | TDSP DUNS Number | Numeric (9 or 13) |
| ReportName | ‘IntervalData’ | Alphanumeric (12) |
| DateTime | File transmission date/time stamp | Datetime format = ccyymmddhhmmss |
| Counter | Counter with no specified value | Numeric (3) |
| .lse | Value of .lse in file name |  |
| <.optional data> | Any optional data, if necessary | Cannot contain csv |

At a minimum the filename must contain .lse after decryption otherwise the file will be rejected by ERCOT.  The filename cannot contain .csv after decryption.  Files will be sent with a NAESB input-format of “FF”.

- Definition of inbound file:

Note:  The correct number of commas must be included even if optional elements are not provided.

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row One |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000001 | Must be 00000001 | Numeric (8) |
| 2 | ESI ID | Mandatory |  |  | Alphanumeric (64) |
| 3 | Channel | Mandatory | 1 4 | Indicates type of data contained in the detailed rows. 1 = generation 4 = load | Numeric (1) |
| 4 | Start Time | Mandatory |  |  | YYYYMMDDHHMMSS (24-hour) (14) |
| 5 | Stop Time | Mandatory |  |  | YYYYMMDDHHMMSS (24-hour) (14) |
| 6 | DST Participation | Mandatory | Y | Y = DST Participant Data must be DST adjusted and must be in local prevailing time | Alphanumeric (1) |
| 7 | Invalid Record Flag | Mandatory | N | N indicates that the data is VEE’d and does not need to be pre-validated. | Alphanumeric (1) |

Note:  The correct number of commas must be included even if optional elements are not provided.

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row Two |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000002 | Must be 00000002 | Numeric (8) |
| 2 | Meter Start Reading | Optional |  | Meter Start Reading Not used by ERCOT Default = 0 | Non-negative numeric Max = 999999999999999.9999 00:00:00 |
| 3 | Meter Stop Reading – Register Read for the day | Optional |  | Meter Stop Reading Not used by ERCOT Default = 0 | Non-negative numeric Max = 999999999999999.9999 23:59:59 |
| 4 | Meter Multiplier | Conditional |  | Must be included if meter start and meter stop is included. Not used by ERCOT Default = 0 | Non-negative numeric Max = 999999999999999.9999 |
| 5 | Empty value | Mandatory | No value provided | Must be Null. See example. | The correct number of commas must be included. |
| 6 | Pulse multiplier | Optional |  | Not used by ERCOT Default = 0 | Non-negative numeric Max = 999999999999999.9999 |
| 7 | Empty value | Mandatory | No value provided | Must be Null. See example. | The correct number of commas must be included. |
| 8 | Seconds Per Interval | Mandatory | 900 | 900 = 15 minute intervals | Numeric (3) |
| 9 | Lodestar Unit of Measure | Mandatory | 01 | 01 = kWh | Numeric (2) |
| 10 | Basic Unit Code | Optional |  | Not used by ERCOT Default = 1 | Positive numeric. Max 9999. |
| 11 | Time Zones West of GMT | Optional |  | Not used by ERCOT Default = -1 | Numeric. Min = -1 Max = 47 |
| 12 | Population | Optional |  | Not used by ERCOT Default = 0.0 | Positive numeric. Max = 999999999999999.9999 |
| 13 | Weight | Optional |  | Not used by ERCOT Default = 0.0 | Max = 999999999999999.9999 |
| 14 | Time Zone Standard Name | Mandatory | CST | Not used by ERCOT | Alphanumeric (3) |

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row Three |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000003 | Must be 00000003 | Numeric (8) |
| 2 | Descriptor | Mandatory |  | Unique tran id | Alphanumeric (80) |

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row Four |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000004 | Must be 00000004 | Numeric (8) |
| 2 | Timestamp | Mandatory |  | Timestamp of read. This value will determine which read will ‘win’ for a day if there are multiple reads. | YYYYMMDDHHMMSS |
| 3 | Origin | Mandatory | M | M=Metered | Alphanumeric (1) |

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row Thirty |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000030 | Must be 00000030 | Numeric (8) |
| 2 | Name Value Pairs | Mandatory | ATTRIBUTE_VALUE_PAIRS | Must be ATTRIBUTE_VALUE_PAIRS | Alphanumeric (21) |
| 3 | MRE DUNS Number | Mandatory | MRE=<required MRE DUNS> | Insert MRE DUNS Number | MRE= + Numeric (9 or 13) |
| 4 | TDSP DUNS Number | Mandatory | Sender=<required Sender DUNS> | Insert Sender DUNS Number | Sender= + Numeric (9 or 13) |
| 5 | ERCOT DUNS Number | Mandatory | Receiver= 183529049 | Must contain ERCOT’s DUNS number. | Receiver= + Numeric (9) |
| 6 | CR DUNS Number | Attribute is Mandatory Value is optional | REP=<optional CR DUNS> | Insert CR DUNS number The attribute (REP=) is required. The value (the CR DUNS number) is optional. Not Used by ERCOT | REP= + Numeric (9 or 13) |

Note:  The correct number of commas must be included even if optional elements are not provided.

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Detailed Record |  |  |  |  |  |
| 1 | Sort code | Mandatory | 10000000 through 10000024 | Each row must contain four 15 minute interval sets. For a 92 interval day, the data records will go through row 22 (10000022). For a 96 interval day the data records will go through row 23 (10000023). For a 100 interval day will go through row 24 (10000024). | Numeric (8) |
| 2 | Interval value | Mandatory |  |  | Numeric Maximum of 3 significant digits to the right of the decimal. Must be a positive value. |
| 3 | Lodestar Status Code | Mandatory | A = actual E = estimate | Indicates whether the interval is an actual or estimate. | Alphanumeric (1) |
| 4 | Empty value | Mandatory | No value provided | Must be Null. See example. | The correct number of commas must be included. |

Example of complete data record:

00000001,100000000000000,4,20080510000000,20080510235900,Y,N

00000002,0,0,0,,0,,900,01,01,-1,0.0,0.0,CST

00000003,UNIQUETRANID0123456789

00000004,20080519112825,M

00000030,ATTRIBUTE_VALUE_PAIRS,MRE=666666666,Sender=666666666,Receiver=183529049,REP=111111111

10000000,68.29,A,,69.17,A,,67.99,A,,67.99,A,,

10000001,66.81,E,,66.52,E,,67.11,E,,67.11,E,,

10000002,67.11,A,,66.52,A,,66.81,A,,66.82,A,,

10000003,69.47,A,,107.73,A,,230.76,A,,228.11,A,,

10000004,69.47,A,,107.73,A,,230.76,A,,228.11,A,,

10000005,231.94,A,,228.4,A,,230.17,A,,233.7,A,,

10000006,25.18,A,,21.95,A,,246.36,A,,247.83,A,,

10000007,259.61,A,,283.45,A,,303.16,A,,313.76,A,,

10000008,325.83,A,,336.72,A,,350.26,A,,354.38,A,,

10000009,358.5,A,,30.27,A,,361.74,A,,366.45,A,,

10000010,365.86,A,,369.39,A,,369.69,A,,368.21,A,,

10000011,371.45,A,,39.69,A,,367.63,A,,369.39,A,,

10000012,371.15,A,,369.68,A,,368.51,A,,369.98,A,,

10000013,370.57,A,,373.51,A,,373.22,A,,369.39,A,,

10000014,370.86,A,,369.98,A,,368.81,A,,365.57,A,,

10000015,364.39,A,,34.68,A,,36.8,A,,284.03,A,,

10000016,309.64,A,,308.76,A,,302.88,A,,296.1,A,,

10000017,286.68,A,,279.91,A,,24.29,A,,189.55,A,,

10000018,190.43,A,,161.59,A,,10.68,A,,114.79,A,,

10000019,108.9,A,,107.14,A,,101.55,A,,98.61,A,,

10000020,94.78,A,,4.49,A,,95.66,A,,92.42,A,,

10000021,84.77,A,,80.94,A,,80.94,A,,80.35,A,,

10000022,80.35,A,,80.35,A,,7.29,A,,77.41,A,,

10000023,77.71,A,,76.82,A,,77.41,A,,7.82,A,,