Verifiable Cost Manual

Version: 0.10

Status: Active

Approval Date:

# Table of Contents

Table of Contents	2

Revisions and Approval Information	4

Approval Date:	4

Effective Date:	4

Revision History:	4

Introductory Information	5

Purpose	5

Intended Audience	5

Global Definitions	5

Section 1:  An Introduction to Verifiable Costs	7

Verifiable Cost Flowcharts	9

Chart 1:  Verifiable Startup Cost Data: Calculations and Dependencies	9

Chart 2:  Verifiable Minimum Energy Cost Data:  Calculations and Dependencies	10

Chart 3:  Real Time Mitigation Data:  Calculations and Dependencies	11

Section 2: General Rules of Verifiable Costs	12

Generally Applicable Rules	12

Additional Rules for Combined Cycle Plants	14

Additional Rules for Split Generation Resources	15

Additional Rules for Submitting Emission Costs	16

Section 3:  Verifiable Startup Costs	23

Verifiable Startup Costs Policies	23

Submitting Startup Costs	24

Startup Fuel Consumption	24

Fuel Type Percentages	25

Non-Fuel Startup Costs	26

Start Type Descriptions	26

Table 1:  Startup Verifiable Cost Data Requirements per Resource per start type	27

Section 4:  Verifiable Minimum Energy Costs	28

Verifiable Minimum Energy Cost Policies	28

Submitting Minimum Energy Costs	28

Minimum Energy Fuel	29

Fuel Type Percentages at LSL	29

Non-Fuel Minimum Energy Cost	29

Verifiable Operation Minimum Energy Costs	30

Verifiable Maintenance Minimum Energy Costs	30

Table 2:  Minimum Energy Cost Data Requirements per Resource	31

Section 5:  Mitigated Offers and Verifiable Costs	32

Mitigated Offer Cap Policies	32

Submitting Data for Mitigated Offers	32

Variable O&M	32

Fuel Type Percentages above LSL	33

Verifiable IHR	33

Table 3:  Real Time Mitigation Data	33

Section 6:  Verifiable Heat Rates	34

Verifiable Heat Rate Policies	34

Submission of Heat Rate Curve Data	34

Input-Output Curve	35

Incremental Heat Rate Curve	35

Average Heat Rate Curve	36

Section 7:  Forced Outages of a RUC-Committed Resource	37

Policy for Costs Resulting from a Forced Outage	37

Submission of Forced Outage Cost Data	37

Section 8:    Operating and Maintenance Cost Guidelines	39

Operating and Maintenance Cost Policies	39

Verifiable Operating Costs	39

Verifiable Maintenance Costs	39

Event Specific Verifiable O&M Costs	40

Options for Submitting Verifiable O&M Costs	40

Option 1:  Submission of O&M costs with Documentation and Calculated Using a Resource Specific Methodology	40

Option 2:  Submission of O&M costs with Documentation and Calculated Using the Methodology Described in Appendix 1	41

Additional Conditions Applicable to Option 1 and Option 2	41

Conditions Applicable to All Types of Resources	41

Conditions Applicable to Combustion Turbines	43

Conditions Applicable to Fossil Thermal Resources	44

Maintenance Period Conditions	44

Operating and Maintenance Cost Submissions Types	44

Section 9:  Timelines Applicable to the Submission and Review of Verifiable Costs	45

Submission and Approval of Verifiable Cost Data	45

Timeline Applicable to the Submission and Approval of Verifiable Costs	46

Chart 4:  Verifiable Cost Submission Timeline	48

Policy for Rescinding Previously Approved Verifiable Costs	50

Timeline Applicable to Rescinding Approval of Verifiable Costs	50

Chart 5:  Timeline Applicable to Rescinding Approval of Verifiable Costs	53

Appealing Verifiable Cost Rejections	55

Verifiable Cost Appeal Process and Timeline	55

First Level of Appeal	55

Second Level of Appeal	56

Appendix 1A:  Methodology for Determining Maintenance Costs of Nuclear and Fossil Steam Units	58

Method of Calculating Total Maintenance Costs using 1998 as the Operating Year	58

Appendix 1B:  Methodology for Determining the Maintenance Costs of CT and CCP Units	59

Example Calculation of Total Maintenance Dollars using Operating Year of 1998	59

How to Calculate Equivalent Service Hours (ESH)	59

How to Calculate Equivalent Hourly Maintenance Cost (EHMC)	60

Applying Equivalent Hours Maintenance Costs to Calculate Maintenance Rates:	60

Example Calculation of the Maintenance Adder for a Combustion Turbine	61

Appendix 2:  Auxiliary Data	62

Figure 1:  Emission Factors	62

Figure 2:  Handy-Whitman Index	63

Figure 3:  Data to Support Verifiable Costs	64

Appendix 3:  Operating and Maintenance Cost Categories	65

Maintenance Cost Categories	74

Operating Cost Categories	74

Table 4:  O&M Categories applicable to all generating plants	74

Table 5:  O&M Categories applicable to Coal-Fired Generating Plants	75

Table 6:  O&M Categories applicable to Combustion Turbine/Combined-Cycle 	75

Table of References	82

# Revisions and Approval Information

### Approval Date:

### Effective Date:

### Revision History:

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 10/07/07 | 0.01 | Verifiable Cost Manual – Initial Public Draft | Ino Gonzalez and Scott Meyerott |
| 10/12/07 | 0.02 | Updated to remove all provisions that were inconsistent or outside the scope of the Nodal Protocols – Presented at Verifiable Cost Workshop | Ino Gonzalez and Scott Meyerott |
| 10/29/07 | 0.03 | Version presented to SDAWG on 11/02/08. This update - Includes decisions made at the Verifiable Cost Workshop; - Incorporates feedback received from market participants; and - Has been edited for clarity and mistakes. | Ino Gonzalez and Scott Meyerott |
| 11/09/07 | 0.04 | This update - Removes Maintenance categories listed in previous versions; - Allows Maintenance categories defined by NERC; - Removes the requirement that a PE sign off on submitted Heat Rate Curves; and - Renames “Actual Verifiable Costs” to “Event Specific Verifiable Costs”; and - Has been edited for clarity and mistakes. | Ino Gonzalez and Scott Meyerott |
| 11/19/07 | 0.05 | This version includes general edits, flowcharts illustrating the review process, and clarifications to various timelines. | Ino Gonzalez and Scott Meyerott |
| 12/26/07 | 0.06 | This version includes general edits and changes made by the TPTF VC Task Force. | Ino Gonzalez and Scott Meyerott |
| 3/8/08 | 0.07 | This version includes general edits and changes made by the TPTF VC Task Force. | Ino Gonzalez and Scott Meyerott |
| 4/9/08 | 0.08 | This version includes general edits and changes made by ERCOT based on comments from TPTF | Ino Gonzalez |
| 5/16/08 | 0.09 | This version includes how emission costs should be calculated and is based on discussions with the VC Subgroup. | Ino Gonzalez John Edwards Dr. Jim guy |
| 2/10/2009 | 0.10 | This version includes updates on rules for emissions, PPAs and the 5-RUC deployment requirements. | Ino Gonzalez Dr. Jim Guy |

# Introductory Information

This Manual details the rules and procedures pertaining to the Verifiable Cost Process in the Texas Nodal Market.  If any provision in this document is in conflict with the Nodal Protocols, then the Protocols prevail to the extent of the inconsistency.  See section 1.1 Paragraph 5 of the Nodal Protocols.

### Purpose

This Manual provides an overview of ERCOT’s Verifiable Cost Process.  It defines Verifiable Costs and also delimits which costs are properly included within that definition.  This Manual details the format and the manner in which various types of Verifiable Cost data are to be submitted to ERCOT.  It also details the processes used by ERCOT to evaluate submitted Verifiable Cost data and also the considerations bearing on ERCOT’s decision to approve or reject that data.  This document establishes the procedures to be used when disputing ERCOT’s treatment or rejection of submitted data.  Lastly, this document details the various timelines that apply to the submission, review, approval, update, and dispute of Verifiable Costs.

### Intended Audience

This Manual is primarily intended to be used by those submitting Verifiable Cost data or disputing ERCOT’s treatment of submitted data.  This Manual, however, is also intended for anybody wishing to learn more about the rules and procedures applicable to the Verifiable Cost Process in the Texas Nodal Market.

### Global Definitions

Where this Manual uses the generic phrase “Verifiable Costs,” it is intended to refer to the sum of any applicable, Verified Operating and Maintenance Costs and any appropriate, Verified Fuel Costs.  ERCOT itself calculates Fuel Costs, but does so using Fuel Consumption data that have been submitted and verified.  Thus, the Fuel Cost component implied by the term “Verifiable Costs” should be interpreted to mean whichever of the following is contextually appropriate:

- Fuel Consumption per-start (MMBtu/start)

- Fuel Consumption per-hour at LSL  (MMBtu/hr)

- Fuel Consumption as determined from submitted heat rate (a measure of generator efficiency) data

The following are several abbreviations that are used throughout this Manual and the intended meaning of each:

- “AHR Curve” denotes Average Heat Rate Curve

- “CCP” denotes Combined Cycle Plant

- “FIP” denotes Fuel Index Price

- “FOP” denotes Fuel Oil Price

- “IHR Curve” denotes Incremental Heat Rate Curve.

- “I/O Curve” denotes Input-Output Curve

- “LSL” denotes Low Sustained Limit

- “HSL” denotes High Sustained Limit

- “Manual” refers to this document, ERCOT’s Verifiable Cost Manual

- “MMBtu” denotes one-million British Thermal Units

- “O&M costs” denotes Operations and Maintenance costs.

- “QSE” denotes Qualifying Scheduling Entity

- “RUC” denotes the Reliability Unit Commitment

- “SGR” denotes Split Generation Resource

- “VOM" denotes Variable O&M

- “VCMS” denotes Verifiable Cost Management System

- “lb” denotes Pounds-Mass

- “WMS” Wholesale Market Subcommittee

- “LEL” denotes Low Emergency Limit

- “HEL” denotes High Emergency Limit

- “PPA” denotes Power Purchase and Tolling Agreements

# Section 1:  An Introduction to Verifiable Costs

This section of ERCOT’s Verifiable Cost Manual provides an introduction to Verifiable Costs and explains what they represent.

Verifiable Costs are a significant component in the Texas Nodal Market.  They are used by ERCOT in various calculations and internal processes, including the replacement of generic generator-specific cost caps and to provide QSEs with forms of cost recovery that would otherwise be unavailable.

Verifiable Costs are used as a proxy for the costs a Resource incurs while operating.  These costs are determined for, and allocated according to, the three discrete stages of the power generation cycle: startup to LSL, operation at LSL, and operation above LSL.  While Verifiable Costs are calculated uniquely for each of these stages, Verifiable Costs in all stages are a function of two types of costs: Fuel Costs and Operation and Maintenance (“O&M”) costs.

The Fuel Cost component of Verifiable Costs is intended to capture the fuel costs a Resource incurs while generating power during a given period.  This component of Verifiable Costs is itself a function of fuel price ($/MMBtu) and the verifiable fuel consumption rate (either MMBtu/start or MMBtu/hr).  When ERCOT calculates the fuel component of the Resource-specific offer caps, ERCOT uses the Fuel Index Price (“FIP”) and the Fuel Oil Price (“FOP”) and the corresponding percentages of each fuel submitted in the Offer. .  . QSEs should not  include fuel prices with their Verifiable Cost submissions.  See Equations 1 and 2 of Appendix 5 for a description of the equations used by ERCOT to calculate Resource-specific offer caps.

The O&M component of Verifiable Costs is intended to capture various other non-fuel, incremental (marginal) costs a Resource incurs while generating power in a given period.  The O&M component of Verifiable Costs is calculated by a QSE and then verified by ERCOT using data submitted.  Both the data and the methods used to calculate these costs will vary, depending, for example, on the costs intended to be captured or on the different internal record-keeping and cost-allocation schemes implemented by Resource Entities.  Additional detail is provided in subsequent sections of this Manual.

ERCOT calculates Verifiable Costs using data specific to a Resource and that is submitted by the QSE representing the Resource.  Submitted data is only valid for a single Resource (i.e., it is Resource-specific).

The process of calculating Verifiable Costs begins when a QSE submits, in the appropriate manner, cost and/or Resource performance data which is permitted by this document and the Nodal Protocols.  ERCOT will subsequently review the submitted data and will approve (“verify”) it if ERCOT finds the data to be sufficiently accurate and documented.  Only after approving submitted data does ERCOT use it as a basis for calculating Verifiable Costs.

Once approved, Verifiable Cost data are used prospectively to calculate Verifiable Costs.  An important exception to this rule pertains to Event Specific Verifiable Costs.  Event Specific Verifiable Costs are documented costs a Resource actually incurred during a specific event and, thus, exclude amortized or discounted future costs.  Event Specific Verifiable Costs are only relevant to Section 7: Forced Outages of a RUC-Committed Resource.  Event Specific Verifiable Costs are applied retroactively (i.e., used when resettling the Operating Day on which an applicable event occurred) and only apply to the event from which they arose (i.e., must be resubmitted for all similar events and will not be used for “normal” Verifiable Costs).  The phrase “Event Specific Verifiable Costs” is underlined and italicized within this Manual in an effort to increase clarity, reduce confusion, and emphasize the contrast between Verifiable Costs and Event Specific Verifiable Costs.

### Verifiable Cost Flowcharts

#### Chart 1:  Verifiable Startup Cost Data: Calculations and Dependencies

#### Chart 2:  Verifiable Minimum Energy Cost Data:  Calculations and Dependencies

#### Chart 3:  Real Time Mitigation Data:  Calculations and Dependencies

# Section 2: General Rules of Verifiable Costs

This section of ERCOT’s Verifiable Cost Manual provides general rules that apply to Verifiable Cost submissions.

### Generally Applicable Rules

The use of Verifiable Costs, rather than generic amounts capped according to Resource Category, is optional for a QSE that does not receive RUC instructions for a Resource. A QSE receiving RUC instructions for a Resource is required to submit Verifiable Cost data in various circumstances as described in paragraph (1) of Section 5.6.1 of the Protocols.  For  QSEs opting to submit Verifiable Cost data and also for those required to submit it, the following rules apply:

- Verifiable Cost data must be submitted by the QSE representing the Resource

- Data must be submitted that allows for the verification of startup costs for each start type (cold, intermediate, and hot).

- A QSE is not considered to have submitted Verifiable Cost data unless data are submitted for all startup types (cold, intermediate, and hot) and for minimum energy costs.

- ERCOT will not approve Verifiable Cost data for a Resource unless the Resource Entity has registered the Resource with ERCOT.

- In the event that multiple Resources are tied to a fuel header, each QSE submitting fuel consumption data must provide the individual meter values if available.  Otherwise,,the QSE must provide ERCOT with the aggregate meter values and the relative percentages of fuel consumption during the relevant stage of the generation cycle (startup, operation at LSL, and operation above LSL).

- All references to Heat Rate Curves and Heat Rate data within this document refer to Net Output.

- For use in Real Time Mitigation, QSEs have the option of submitting, but none are required to submit, Incremental Heat Rate (IHR) curves, with the exception of RMR Resources, the incremental O&M costs of generating power at greater than LSL output.  Submission of this information is also optional for QSEs that have received RUC Commitments.

- ERCOT will not separately approve Verifiable Startup Costs and Verifiable Minimum Energy Costs.  Both must be submitted and approved together.

- If fuel consumption rates vary by season, QSEs must submit such seasonal costs in accordance with the procedure describe herein.

- All submissions of Verifiable Cost data must include an affidavit that avers to the data’s accuracy and authenticity and that is signed by an officer of theQSE representing the Resource.

- All Verifiable Costs submitted must represent “true” costs and must be submitted in sufficient detail to allow ERCOT to validate the data.

- If future versions of the Nodal Protocols alter the amount of Nodal Surcharge, ERCOT will automatically adjust the relevant portions of Resources’ previously approved Verifiable Costs.

- Verifiable Cost documentation must be submitted to ERCOT electronically, as is described within Section 9:  Timelines Applicable to the Submission and Review of Verifiable Costs.  However, if ERCOT requests additional documentation for which electronic submission would be impractical, e.g., physical manuals, such documents may be submitted by mail or other courier.

- ERCOT has authority to review and reject the Verifiable Costs it previously approved if a QSE is provided with sufficient notice.  Rescinded approval will only affect future Operating Days.  See Section 10:  Timeline for Rescinding Approval of Verifiable Costs for additional information.

- Resource  may not register as both an SGR and a CCP; therefore, ERCOT will not accept Verifiable Cost data for an SGR that is also a CCP.

- The QSE must create and submit to ERCOT a graphical representation of the generator’s fuel consumption, MMBtu/hr, MW and MMBtu/MWh versus time, as is shown by Figure 3 below.  This must also indicate the number of hours it takes the Resource to reach LSL and to shutdown.  The required graphical representations for each individual share of a Split Generation Resource must be identical.

- The  QSE must provide the High Sustained Limit (HSL) and the Low Sustained Limit (LSL) of the Resource in its submittal of Verifiable Cost data.

- ERCOT's Settlements & Billing group may share Resource-specific Heat Rate and O&M data with ERCOT’s Planning group for the purpose of performing economic planning studies.  ERCOT will share this information on a "need to know" basis and will continue to treat the information as Protected Information pursuant to the ERCOT Protocols.

### Additional Rules for Combined Cycle Plants

In addition, the following apply specifically to combined cycle units:

- Verifiable Costs will only be verified for CCP configurations that are registered with ERCOT.  If a QSE wishes to submit Verifiable Cost data for a CCP configuration, the QSE must submit costs for all units that make up that registered configuration.

- If the Resource includes “Alternate” Units in its Resource Registration, for the purpose of determining verifiable costs for the registered configuration, ERCOT will create a composite unit by averaging the verifiable cost data of the non-Alternate Unit(s) with the verifiable cost data of the Alternate unit(s).  To determine the composite unit ERCOT will weight the data from the units using the respective HSL ratings of the Resources.  See Equation 3 of Appendix 5 for a description of the equation used by ERCOT to determine the composite unit.

3QSEs submitting Verifiable Cost data for startup or operation at LSL must submit fuel and O&M data for each configuration that has been registered.   QSEs submitting Verifiable Cost data for operation above LSL may submit O&M data for each configuration at which the plant will be operated.

- Input-Output (“I/O”) curves need only be submitted on a per-configuration basis.  If the costs incurred by individual units are determined by apportioning per-configuration costs, there must be a reasonable financial and/or engineering basis for doing so and it must be described in the QSE’s Verifiable Cost submission.

### Additional Rules for Split Generation Resources

- Entities representing Split Generation Resources (“SGRs”) may select a Management QSE to provide ERCOT with the facility’s total Verifiable Costs for the entire facility or for the Verifiable Costs of each respective Resource entity which makes up the SGR .  Alternatively,  QSEs may be designated for each individual Resource entity to submit that Resource entity’s unit individual verifiable cost data which are then aggregated into the facility’s total costs.  Verifiable Costs and their associated cost caps may be different for each owner.

- The Management QSE must be chosen from one of the QSEs representing the SGR.  Only for purposes related to the submission of total Verifiable Costs, a QSE designated Management QSE, is responsible for representing the interests of, and serving as the point-of-contact for, all entities associated with the SGR.

- The Management QSE must notify ERCOT in writing that it will act as representative for the other SGR owners.  The letter must be signed by all other SGR owners and a contact person must be listed for each.

- Individual unit SGR costs must be consistent with the total Verifiable Costs of the generation facility.  All owners of a facility should therefore participate in preparing Verifiable Cost data for submission by the Management QSE.  ERCOT may compare both the individual units’ costs and the SGR’s total Verifiable Costs with other similar situated Generation Resources to determine the reasonability of the submitted costs.

- Each QSE must determine the individual Verifiable Costs of each SGR unit share and ensure that they are appropriately included in Verifiable Cost submissions made by the Management QSE, if appropriate.

- The individual verifiable startup fuel (MMBtu/Start) for each SGR owner shall be equal to that owner’s ownership percentage multiplied by the total facility startup fuel.

- The verifiable fuel rate for operation at LSL (MMBtu/Hr) for each SGR owner shall be equal to that owner’s ownership percentage multiplied by the total fuel rate for operation at LSL.

- Verifiable operations and maintenance costs for startup and operation at LSL may be different for each SGR owner.

- ERCOT may separately review and approve a Verifiable Cost submission made by an individual SGR owner.  However, ERCOT retains the right to require all SGR owners submit their individual Verifiable Costs before ERCOT approves any individual cost submissions.

- Individual owners of an SGR may utilize different methodologies to determine their individual costs.  However, costs must not be included in the Management QSE’s submission more than once and, if multiple owners submit fractional shares of a cost, the sum of those shares must equal the facility’s associated cost.

- All owners must submit the same percentages of each type of fuel consumed during startup and operation at LSL.

### Additional Rules for Submitting Emission Costs

As a trading market develops pertaining to emissions limits at a state and or regional level, the costs associated to comply with emission restrictions may be eligible to be recovered and be part of the Verifiable Cost methodology. At the appropriate time, any market participant may propose a methodology to the Verifiable Cost Working Group to recuperate the emission costs, which will be addressed in the Verifiable Cost manual.

Verifiable cost data may include the cost of purchasing emission credits but only to the extent necessary to meet environmental regulations associated with the operation of the specific unit.  ERCOT will not approve emission costs of any type unless they are sufficiently documented.  When submitting emission costs the following procedures apply:

- QSEs submitting emission costs per-start must do so for each start type, cold, hot and intermediate.   ERCOT will calculate Verifiable Startup Emission Costs ($/start) for a Resource by multiplying the verified per-start fuel consumption rate (MMBtu/start) by the verified emission rate (lb/MMBtu) and by the relevant emission price ($/lb). See Equation 4 of Appendix 5 for a description of the equation used by ERCOT to calculate Verifiable Startup Emission Costs

- Emission costs incurred while operating the Resource at the Minimum-Energy level or above LSL are calculated on a $/MWh basis.  ERCOT will calculate Verifiable emission costs ($/MWh) at LSL by multiplying the verified fuel consumption rate (MMBtu/MWh) at LSL by the verified emission rate (lb/MMBtu) and by the relevant emission price ($/lb).  See Equation 5 of Appendix 5 for a description of the equation used by ERCOT to calculate Verifiable Emission Costs at LSL

- Special considerations regarding NOx regulations

Non-attainment Area for NOx in Houston-Galveston-Brazoria

- Use Argus or Platts published average monthly index price as currently described in the VC Manual

- For verifying the emission rates, the QSE will submit the historic calendar annual average for the unit specific emission rates reported to TCEQ and or EPA by April 30 of the applicable year, if deemed necessary by the QSE.

- .

Emission prices for SO2 and NOx will be obtained by ERCOT and will be based on monthly index prices selected by ERCOT that are generally accepted in the industry and regularly published.  ERCOT will use monthly indices that are published as near to the end of each month as possible..    However, if appropriate monthly indices are not available ERCOT will calculate monthly indices using the arithmetic average of the available daily or weekly index numbers.

ERCOT will disclose to Market Participants the source of its selected price indices, along with descriptions of the nature and derivation of the indices as available from the publishers of those indices.  In the event that an ERCOT selected index becomes unavailable or unsuitable for the intended purpose, ERCOT will select a substitute index source.  .  ERCOT will notify Market Participants of any change in the index, along with a description of the nature and derivation of the substitute index and a summary of the reasons for the change, thirty days prior to the beginning of its use.  However, in the event that 30 days notice cannot be given for any reason, ERCOT will notify Market Participants as far prior to use as practical.

On a monthly basis, ERCOT will recalculate each Resource’s emission costs for SO2 and NOx utilizing the emission prices taken from the indices described above. The recalculated emission costs will replace the emission costs in the previously approved Verifiable Costs.

ERCOT emission cost calculations for each Resource will be completed by and the results made available to QSEs by the 3rd Business Day following the date of publication of the last of the index numbers being used for the month.  The effective date for use of these new emission costs will be the 5th Business Day following publication.

Additional Rules for Submission of Power Purchase and Tolling Agreements

The Protocols allow Power Purchase and Tolling Agreements (PPAs) to be considered as documentation of Verifiable Costs in lieu of Resource-specific data submissions.  ERCOT will cap the O&M costs for startup and for Minimum Energy Operation (at LSL).  The PPA cap will be determined by ERCOT using a Reference Resource with similar characteristics as the Resource underlying the PPA.  When making the assessment of caps, ERCOT will generally compare only the O&M costs stated in the PPA with the Verifiable O&M costs of the Reference Resource.

The QSE representing the PPA Resource is permitted to submit the actual fuel rates at Startup and Minimum Energy Operation without caps to establish the fuel component of Verifiable Costs as set forth in the Verifiable Cost Manual.

When there is no or insufficient data for comparable Combined Cycle (CCP) resources, ERCOT will attempt to construct a Reference Resource using a “building block” approach.  ERCOT will select from the available pool of approved Combustion Turbines and Steam Turbines that are comparable to components of the PPA Resource configuration.  If the building block approach fails to select a suitable Reference Resource, Resource-specific Generic Costs will be used in lieu of Verifiable Costs until such time when ERCOT can develop a Reference Resource as described herein. All newly established Reference Resources will be created on a monthly basis as described in this Manual..

Specific Guidelines for Inclusion of Power Purchase and Tolling Agreements in the Verifiable Costs Process.

The following principles will govern how ERCOT will process PPAs and how QSEs submitting PPAs as documentation for Verifiable Costs must prepare their submittals to ERCOT.

- Only PPAs covering registered Resources will be accepted by ERCOT.  The identity of the Resource or Resources covered by the PPA must be specified in the PPA or declared by the QSE in supporting documentation.

- Only QSEs offering Three-Part Supply Offers for a specific Resource covered by a PPA may submit that PPA as Verifiable Costs documentation.

- A QSE submitting a PPA as Verifiable Cost documentation must represent one-hundred percent (100%) of the underlying Resource’s capacity.

- QSEs must submit to ERCOT a non-redacted copy of the complete PPA with all attachments.

- The PPA must clearly define the start and end dates (effective dates) for the PPA.  Only PPAs:  (a) signed prior to July 16, 2008 and (b) not between affiliates, subsidiaries or partners will be accepted as Verifiable Cost documentation.

- If a PPA does not clearly state separately the cost of Start-Up and Minimum Energy level operation, the QSE may provide to ERCOT additional documentation describing the methodology on how these costs are calculated and allocated to each cost category, startup and minimum energy levels.

- QSEs representing PPAs must specify costs for each Start type (cold, intermediate or hot).  ERCOT recognizes that aircraft derivative gas turbines may have Start-up costs by start type that are identical, or nearly identical.  QSEs representing this type of PPA Resource may submit cost information accordingly, or may submit differing costs with supporting documentation.   For non-aircraft derivative turbines  the ERCOT expectation is that intermediate and hot start costs will bear a relationship to cold start costs of approximately 70% and 50%, respectively, as follows:

  - Cost of hot start = Cost of cold start x 0.5

  - Cost of intermediate start = Cost of cold start x 0.7

QSEs representing non-aircraft derivative turbines with PPAs  may submit costs by start type that deviate substantially from these relative weights, but must include supporting documentation.

- Fuel costs for PPAs will be based upon fuel use rates of the Resources covered by the PPA, with no caps, whenever possible.  This will not always be possible, as when the QSE does not submit separate values for fuel cost and O&M cost, or when ERCOT does not approve a submitted fuel use rate.  ERCOT procedures in these cases will be described later in this document.

- All PPA based Verifiable O&M Costs will be capped at the level of a Reference Resource.  Caps will generally, but not always, apply only to O&M on PPAs, as noted in the preceding paragraph.  Caps on Verifiable Costs will apply only to PPA covered Resources.

- The selection of the Reference Resource used to determine O&M cost caps shall be made from a group of comparable specific Resources.  Resources in this group must not be subject to any PPA, and they must have ERCOT approved Verifiable Costs.  The criteria for comparability are defined in a succeeding paragraph.  The Reference Resource shall be the Resource within that group with the highest ERCOT approved Verifiable O&M Costs.

- Reference Resources will be selected and Verifiable O&M Cost caps will be calculated and applied separately for Cold start, Hot start, Intermediate start, and LSL operation.  Thus, any given PPA may be compared to four different Reference Resources to establish its status for the succeeding month as “capped’ or “not capped” and four separate cap values for Verifiable Costs will be calculated and applied during a month. See Appendix 3 for examples of the process for determining Cost Caps for PPA Resources.

- The selection of the Reference Resource for each of the four VC types (Cold, Hot, Intermediate Startup and Operation at LSL) will be done on the basis of the highest ERCOT approved Verifiable O&M Costs for a comparable non-PPA Resource for that VC type.

- The group of Resources used to establish the Reference Resource for a given PPA and VC type will be selected on the basis of the following criteria:

  - Same Resource technology

  - For CCP Resources, only other CCP Resources with similar configurations will be included.

  - Same primary fuel type

  - High Sustained Limit (HSL) must be within plus or minus 30% of the HSL of the PPA’s underlying Resource

  - Commercial operation date must be within 5 years of the commercial operation date of the PPA’s underlying Resource

- Reference Resources for each type of Verifiable Cost will be selected after the end of each month when fuel and emission credit price indices are available.  The selection of the Reference Resources from among the groups of comparable Resources will be based upon monthly averages for fuel price indices and emission credit indices for the previous month combined with fuel volumes, emission volumes, and O&M costs as previously approved by ERCOT for those Resources.

- Selected Reference Resources by Verifiable cost type will be compared on a monthly basis to PPA Verifiable cost type to determine whether the PPA will be subject to one or more of the caps for the succeeding month.  The status of a PPA Verifiable cost type as “capped” or “not capped”, as determined monthly, will continue unchanged throughout the succeeding month until determination of Reference Resources and caps has been re-accomplished.

- For a PPA that has been determined to be capped for the month, the Reference Resource values for fuel volumes, if applicable, and O&M costs (including emission credit costs if applicable) will be substituted for the PPA values for the purpose of calculating cap values.  This principle applies both to PPAs with separate values for fuel cost and O&M cost and to PPAs with only one non-differentiated cost shown.  For a PPA that has been determined to not be capped for the month, values used to determine payment will be the fuel volumes taken from the underlying Resource and O&M costs taken from the PPA, as described below.

- The Verifiable Cost values used during the month, for a Resource with a PPA that has furnished approved fuel volume data, will be based upon interval by interval calculations.  These calculations will use, for a Resource that:

  - has been capped for the month:

    - the underlying Resource fuel volumes,

    - the daily Fuel Index Price,

    - Reference Resource emission volumes if applicable,

    - the monthly average emission credit indices if applicable, and

    - the Reference Resource O&M costs.

Thus, the fuel part of verifiable costs will vary on a daily basis, but the verifiable costs cap on O&M costs will vary on a monthly basis, and the O&M costs will be equal to the costs calculated for the Reference Resources for the same start-up and LSL operation levels.

  - has not been capped for the month:

    - the underlying Resource fuel volumes,

    - the daily Fuel Index Price,

    - the underlying Resource emission volumes if applicable,

    - the monthly average emission credit indices if applicable,

    - and PPA O&M costs.

Thus, the fuel part of verifiable costs will vary on a daily basis, the O&M part of VC will vary on a monthly basis and the O&M costs will not be higher during the month than the verifiable costs that would have been calculated for the Reference Resources for the same start-up and LSL operation levels.

- In the event that Verifiable Costs for comparable Resources do not exist at the time of the filing of Verifiable Cost documentation for a PPA, ERCOT will use the PPA’s underlying Resource actual fuel volumes submitted by the QSE for startup and operation at LSL and the Resource Category Startup Offer Generic Costs as the cap for the O&M portion of the startup costs until ERCOT receives and has approved comparable Resource Specific Verifiable Costs.  The Generic cost for operation at LSL includes no O&M cost.  Thus, when no Reference Resource is available, zero O&M cost will be added to the fuel cost at LSL. .

- If the Verifiable Cost of a Reference Resource for a PPA changes, the cap for that PPA may also change, on a monthly basis.

- Only fuel and O&M costs related to Start-Up and operation at LSL will be included in Verifiable Costs for a PPA. If multiple fuels are used for Startup and/or LSL operation, the QSE must provide estimated percentages of each fuel to be used and support such estimates with sufficient data.  For the purpose of calculating the reference unit, ERCOT will utilize the higher fuel percentage submitted with VC.

- If a Resource uses one fuel to start-up and bring the Resource to LSL and a different fuel for continuing operation, the QSE must submit its start-up Verifiable Cost information on the basis of the start-up fuel making it clear in its submission this circumstance. ERCOT will make its Verifiable Cost determinations separately on the basis of the fuel used for start-up fuel and the fuel used for operation at LSL.

- Recovery of fixed costs as Verifiable Costs is prohibited by the Protocols and therefore not recoverable under this procedure.

- Recovery of capacity costs under PPAs as Verifiable Costs is prohibited by the Protocols and therefore not recoverable under this procedure.

- QSEs choosing to submit PPAs and choosing to submit Incremental Heat Rate (IHR) Curves for Real Time Mitigation (above LSL) must calculate these curves from the underlying Resource’s actual Input-Output (I/O) equation as set forth in the Verifiable Cost Manual.  ERCOT will not accept Heat Rate values as stated in the PPA at face value unless documentation is submitted showing that the Heat Rate curve is derived from actual data generated from the Resource’s operation and that it is consistent with Section 6:  Verifiable Heat Rates of the Verifiable Cost Manual.

- QSEs choosing to submit Variable O&M costs above LSL must submit O&M costs (including the costs of emission credits) of the PPA for operations above LSL but these are capped at the average cost of the Reference Resource.

- The selection of the Reference Resource for operations above LSL will be done on the basis of the highest simple average ERCOT approved Verifiable O&M Costs, above LSL, for all comparable Resources without a PPA, for that verifiable cost type.  The PPA O&M cap will be set equal to the O&M costs of the Reference Resource for the entire PPA Resource IHR range.

- If energy supplied under a PPA is provided by more than one Resource or by more than one combined cycle train configuration, the QSE must provide Verifiable Cost information for each Resource and configuration.

- Upon the expiration of a PPA submitted as Verifiable Costs documentation, the Three-Part Supply Offer Cap will be based on Generic cost and will be effective one (1) Calendar Day after the expiration of the PPA, unless the QSE has submitted and ERCOT has approved new Resource-Specific Verifiable Costs prior to the expiration of the PPA, or another PPA replaces the one that expired.

- QSEs representing Resources covered by a PPA submitted for verifiable cost documentation may include the Nodal surcharge as a separate and additional cost element for the determination of verifiable cost, but only if the Surcharge is not included as part of another cost element submitted.

- The timeline that applies to ERCOT for reviewing PPAs may differ from the timeline for Verifiable Costs submissions without PPAs, depending on the complexity of the PPAs and ERCOT’s ability to review the PPAs as rapidly as a Resource Specific submission.

- ERCOT shall treat all PPAs as Protected Information.

# Section 3:  Verifiable Startup Costs

This section of ERCOT’s Verifiable Cost Manual describes policies and procedures that relate to the submission of Verifiable Startup Costs.

### Verifiable Startup Costs Policies

- The components of startup costs are fuel consumption rates and incremental O&M costs.  These are to be derived by applying financial and/or engineering analysis to manufacturer data, operational data, or the results of recent tests on a Resource.

- Startup fuel consumption rates (MMBtu/start) must be submitted for all startup types (cold, intermediate, and hot) for a QSE to be considered as having submitted Verifiable Startup Costs.  However, if a Resource does not have a distinct start type which is analogous to intermediate startup (or if there are no costs specific to an intermediate start), the QSE must assign a value to the intermediate startup costs equal to the hot startup costs.

- Submitted startup fuel consumption rates and O&M costs will be reviewed by ERCOT and, upon approval, will be used prospectively to calculate Verifiable Startup Costs.

- The Verifiable Startup Cost ($/start) for a Resource is the verified per-start fuel consumption rate (MMBtu/start) multiplied by the relevant fuel price ($/MMBtu) plus the verified O&M costs for the Resource. See Equation 6 of Appendix 5 for a description of the equation used by ERCOT to calculate Verifiable Startup Cost.

- Fuel consumed from breaker open to shutdown may be included as a Verifiable Startup Cost.

### Submitting Startup Costs

Verifiable Startup Costs for a Resource represent a proxy for all of the costs incurred in order to bring a generator online and make it available to produce power.  Only the costs incurred from startup through LSL and from breaker open to shutdown are permissible startup costs. .

The nature of the data that QSE may submit is set forth below and also partially within Section 5 of the Protocols, which defines Verifiable Startup Costs as:

- Fuel consumption rate per start (MMBtu/start) multiplied by a resource category generic fuel price (FIP, FOP, or $1.50 per MMBtu, as applicable); and

- Unit-specific verifiable operation and maintenance expenses.

#### Startup Fuel Consumption

Fuel consumed during a startup is defined as the fuel consumed from first fire through LSL plus the fuel consumed from breaker open through shutdown. (Including auxiliary boiler fuel and auxiliary-equipment fuel or electrical power requirements), excluding normal plant heating.

It is expected that the amount of fuel consumed will be different for each of the three start types.  If available, historical data must be used to determine the typical amount of fuel consumed per start for each start type.  This typical per-start fuel consumption is to be determined in accordance with the following rules:

- When possible, startup fuel consumption rates are to be based on the amount of fuel a Resource has historically consumed per-start.

- If available, submitted historical usage data should, for each start type, include fuel consumption rates for the lesser of the last 10 starts or every start within the past three years.  For each start type, the QSE shall submit the historical fuel consumption data, an average amount of historical fuel consumption, and the fuel consumption rate that the Resource believes represents the current startup fuel consumption rate.

- If a Resource does not have the historical fuel consumption rates for each start described above, the QSE must submit the aforementioned data that it does possess and may also include per-start fuel consumption rates based on manufacturer suggested values or tests which are ERCOT approved.

- If a QSE submits historical startup fuel consumption on a per hour basis (MMBtu/hour), it must also provide proof of the average number of hours it requires to reach LSL for each startup type.

- In its sole discretion, however, ERCOT may choose not to accept the  QSE’s submitted per-start fuel consumption rates if ERCOT determines that they do not represent a Resource’s “true” startup fuel consumption or that they have not been proven in sufficient detail.

- Historical fuel consumption rates must be based on documented metered reads when available.

#### Fuel Type Percentages

For each start type, the QSE must provide documentation establishing the respective ratios of gas, oil, and solid fuel consumed during the startup through LSL sequence. Historical and/or manufacturer suggested ratios are to be submitted as percentages and in accordance with the manner of submitting startup fuel consumption data, detailed above.  For each start type, the QSE must calculate andsubmit:

- total fuel consumption per-start (MMBtu/start); and

- the ratio of each type of fuel consumed to the total amount of fuel consumed per startup.

QSEs with approved fuel consumption ratios for the associated Resource are to submit updated data to ERCOT if they subsequently use a different fuel type during startup and if they also anticipate doing so for any substantial period of time, whether due to fuel market conditions or otherwise.

#### Non-Fuel Startup Costs

Verifiable Non-Fuel Startup Costs represent a proxy for all non-fuel costs that a Resource incurs during the startup through LSL and breaker open through shutdown sequence.   The costs that ERCOT considers in calculating this proxy is limited to incremental operation and maintenance costs (Verifiable O&M) that can reasonably be said to result from the Resource starting up.  Verifiable O&M Costs are defined to include incremental emission costs, for more information see Section 2: General Rules of Verifiable Costs.   ERCOT will not approve fixed costs or capital costs of any type nor insufficiently documented costs.

To be included as a Verifiable O&M Cost of Startup, O&M costs must be submitted in accordance with Section 8:  Operating and Maintenance Cost Guidelines.  ERCOT will not approve submitted O&M startup costs if the amounts or the methods used to calculate them do not coincide with other O&M costs a QSE has submitted, unless there is a reasonable, documented reason for doing so.  For example, startup operating costs might be different because there are greater chemical, water or emissions costs during the startup sequence.  Also, it might be reasonable to multiply an hours-based maintenance cost by the amount of time it takes to complete a startup to LSL sequence.  Additionally, if maintenance costs are allocated on a per-start basis, it might be reasonable for the maintenance component of verifiable startup costs to differ per start type.

#### Start Type Descriptions

The following is a general description of startup costs per start type:

Hot Startup Cost

Hot startup cost is the expected cost to start a Resource, which is in the "hot" condition. Hot conditions vary unit by unit, but in general, a steam unit is hot through an overnight shutdown.

Intermediate Startup Cost

Intermediate startup cost is the expected cost to start a Resource that has recently been online and for which neither hot nor cold conditions are applicable.

Cold Startup Cost

Cold startup cost is the expected cost to start a Resource which is in the "cold" condition.  Cold conditions vary unit by unit, but in general, a steam unit is cold after a two or three-day shutdown.

#### Table 1:  Startup Verifiable Cost Data Requirements per Resource per start type

| Input Data | Description |
| --- | --- |
| AFCRS (MMBtu/Start) | Fuel consumption rate per start |
| GASPERSU (%) | Gas fuel consumed per start as a percentage of total per start fuel consumption |
| OILPERSU (%) | Oil fuel consumed per start as a percentage of total per start fuel consumption |
| SFPERSU (%) | Solid fuel consumed per start as a percentage of total per start fuel consumption |
| O&M ($/start) | Verifiable O&M expenses per start |

# Section 4:  Verifiable Minimum Energy Costs

This section of ERCOT’s Verifiable Cost Manual describes the policies and procedures that relate to the submission of Verifiable Minimum Energy Costs.

### Verifiable Minimum Energy Cost Policies

- The components of Minimum Energy Costs are fuel consumption rate and incremental O&M costs.  These are to be derived by applying financial and/or engineering analysis to manufacturer data, operational data, or the results of recent tests performed on the Resource.

- QSEs must submit the rate at which fuel is consumed during operation at LSL (MMBtu/hour).

- QSEs must submit the LSL level of output (MW) expected during normal operations.  This LSL should also represent the LSL a QSE expects to submit in future COPs.

- If Minimum Energy fuel consumption rates vary by season, QSEs must submit such seasonal rates in accordance with the procedure describe herein.

- Submitted minimum energy fuel consumption rates and O&M costs will be reviewed by ERCOT and, upon approval, will be used prospectively to calculate Verifiable Minimum Energy Costs.

- The Verifiable Minimum Energy Cost ($/MWh) for a Resource is the verified fuel consumption rate at LSL (MMBtu/hr) divided by the Resource’s LSL (MW), multiplied by the relevant fuel price ($/MMBtu), plus the verified O&M ($/MWh) at LSL for the Resource. See Equation 7 of Appendix 5 for a description of the equation ERCOT will use to calculate Verifiable Minimum Energy Cost.

### Submitting Minimum Energy Costs

Verifiable Minimum Energy Costs are the verifiable costs a Generation Resource incurs while operating at the LSL.  The Verifiable Minimum Energy Cost shall also include the Nodal Implementation Surcharge ($/MWh) if properly included in the Verifiable Cost submission.  The nature of the data that QSE may submit is set forth below and within Section 5 of the Protocols, which defines Verifiable Minimum Costs as:

1)	The unit-specific verifiable minimum-energy costs for a Resource are:

(a) 	Actual fuel cost to operate the unit at LSL;

(b) 	Variable operation and maintenance expenses;

(c) 	Nodal implementation surcharges to operate the unit at LSL.

#### Minimum Energy Fuel

Minimum Energy Fuel is defined as fuel consumed by a Resource while operating at the Low Sustained Limit, including auxiliary equipment fuel or electrical power requirements but excluding normal plant heating.  Minimum Energy Fuel is to be determined in accordance with the following:

- QSEs must submit resource-specific Input-Output (I/O) curves (MMBtu/Hr), including their corresponding equation coefficients, for the period from which their verifiable cost data was derived.  ERCOT will calculate Average Heat Rate Curves utilizing these I/O equations.  If fuel consumption at LSL varies by season, QSEs must submit seasonal I/O curves.  More information on heat rates is provided below in Section 6:  Verifiable Heat Rates.

- If available, submitted historical usage data should include fuel consumption rates for the last 25 periods a Resource operated at the LSL.

- If a Resource does not have the historical fuel consumption rates for all of the deployments described above, the  QSE must submit the aforementioned data that it does possess and may also include a fuel consumption rate at LSL (MMBtu/hour) based on manufacturer suggested values or actual tests which are ERCOT approved.

- In its sole discretion, however, ERCOT may choose not to accept the Resource’s fuel consumption rate at LSL submitted by the QSE if ERCOT determines that it does not represent a Resource’s “true” fuel consumption at LSL or that it has not been proven in sufficient detail.

#### Fuel Type Percentages at LSL

QSEs must provide documentation establishing the respective ratios of gas, oil, and solid fuel consumed during operation at the Low Sustained Limit.  Historical and/or manufacturer suggested ratios are to be submitted as percentages and in accordance with the manner of submitting minimum energy fuel consumption data, detailed above.

#### Non-Fuel Minimum Energy Cost

Verifiable Non-Fuel Minimum Energy Costs represent a proxy for all non-fuel costs that a Resource incurs while operating at LSL.  These costs need to be submitted in terms of $/MWh.  ERCOT will not approve fixed costs or capital costs of any type nor insufficiently documented costs.  The costs that ERCOT considers in calculating this proxy are limited to:

- Incremental operation and maintenance costs (Verifiable O&M) that can reasonably be said to result from operation at LSL; and

- The Nodal implementation surcharge ($/MWh).

Verifiable Non-fuel Minimum Energy O&M Costs are defined to include incremental emission costs, for more information see Section 2: General Rules of Verifiable Costs.

#### Verifiable Operation Minimum Energy Costs

To be included as a Verifiable Minimum Energy Cost, operating costs must be submitted in accordance with Section 8:  Operating and Maintenance Cost Guidelines.  Emission costs may be included as a cost of operating a generator at minimum energy.  ERCOT will not approve submissions for the operating cost component of Verifiable Minimum Energy Costs if the amounts or the method used to calculate them do not coincide with other Verifiable Operating Costs a Resource has submitted, unless there is a reasonable, documented reason for doing so.

#### Verifiable Maintenance Minimum Energy Costs

To be included as a Verifiable Minimum Energy Costs, maintenance costs may be submitted in accordance with Section 8:  Operating and Maintenance Cost Guidelines.  ERCOT will not approve submissions for the maintenance cost component of Verifiable Minimum Energy Costs if the amounts or the method used to calculate them do not coincide with other Verifiable Maintenance Costs a Resource has submitted, unless there is a reasonable, documented reason for doing so.

#### Table 2:  Minimum Energy Cost Data Requirements per Resource

| Input Data | Description |
| --- | --- |
| VFCLSL (MMBtu/MWh) | Verified fuel consumption at LSL |
| GASPERME (%) | Gas fuel consumed during operation at LSL as a percentage of total LSL fuel consumption |
| OILPERME (%) | Oil fuel consumed during operation at LSL as a percentage of total LSL fuel consumption |
| SFPERME (%) | Solid fuel consumed during operation at LSL as a percentage of total LSL fuel consumption |
| LSL (MW) | Low Sustained Limit Output |
| VOMLSL ($/MWh) | Variable O&M expenses at LSL |

# Section 5:  Mitigated Offers and Verifiable Costs

This section of ERCOT’s Verifiable Cost Manual pertains to the use of Verifiable Costs for the purpose of calculating Mitigated Offer Caps.  It describes the policies and procedures relating to the submission of Verifiable Incremental Heat Rate and Variable O&M data.

### Mitigated Offer Cap Policies

Under Section 6.5 of the Nodal Protocols, Energy Offer Curves are subject to mitigation in Real-Time Operations by Security Constrained Economic Dispatch (“SCED”).  That is, Energy Offers may be capped at the Mitigated Offer Cap when a Mitigated Offer Cap is greater than its related Reference LMP.  Pursuant to Section 4.4, ERCOT will create a Mitigated Offer Cap curve such that each point on the curve is the greater of (1) FIP or FOP multiplied by a constant or (2) the Resource’s Verifiable Incremental Heat Rate multiplied by FIP or FOP, plus VOM multiplied by a constant.  Thus, the first product (default or generic Cap) will be used unless a QSE submits (and ERCOT approves) the required Verifiable Cost data and if the submitted data yields a greater Mitigated Offer Cap.

### Submitting Data for Mitigated Offers

QSEs may submit two types of resource-specific Verifiable Cost data for inclusion in the calculation of Mitigated Offer Caps: Verifiable O&M data and Verifiable Incremental Heat Rate (“IHR”) data.  Submission of this data, for use in calculating Mitigated Offer Caps, is optional, except for Resources under a Reliability-Must-Run (RMR) contract which must submit to ERCOT an Incremental Heat Rate Curve calculated in a manner as described in Section 6:  Verifiable Heat Rates of this Manual.  A  QSE that submits Variable O&M Cost data for use in calculating Mitigated Offers must also submit IHR Curve data.  However, a QSE may submit IHR Curve data without submitting corresponding VOM data.  To be verified by ERCOT, submitted data must also comply with the relevant requirements set forth below.

#### Variable O&M

For purposes of calculating Mitigated Offer Caps, Variable O&M costs are limited to the incremental O&M costs a Resource incurs while operating above LSL.  Variable O&M costs will only be verified if both the type of cost and its method of calculation are permissible under Section 8:  Operating and Maintenance Cost Guidelines of this document.  Submitted data is to be in terms of dollars per Megawatt-hour ($/MWh) and must be a single average value for the entire   ) range( LSL to HSL) of the IHR Curve submitted.  Emission costs may be included as an additional component of the Variable O&M cost.  For more information see Section 2: General Rules of Verifiable Costs.

#### Fuel Type Percentages above LSL

QSEs that submit Incremental Heat Rates must also provide documentation establishing the ratios of gas, oil, and solid fuel that is consumed during operation above LSL.  The same ratios will apply to all output levels between LSL (or LEL) and HSL (or HEL).

#### Verifiable IHR

If a QSE submits Verifiable Variable O&M costs, the QSE must also submit Verifiable IHR Curves in accordance with Section 6: Verifiable Heat Rates to complete the submission of Verifiable Costs for Mitigated Offer Caps.  Additionally, submitted IHR Curve data must be calculated directly from the I/O equation, as described in that same section.  IHR Curve data must:

- Consist of at least 2, but no more than 10, pairs of MMBtu/MWh and MW points; and

- Include the B, C and D coefficients calculated from the I/O equation (i.e., I/O = A + Bx + Cx2 + Dx3).

#### Table 3:  Real Time Mitigation Data

| Input Data | Description |
| --- | --- |
| Incremental Heat Rate Data Points (MMBtu/MWh, MW) | Points along the Incremental Heat Rate Curve – Up to 10 pairs |
| VOM ($/MWh) | Variable O&M expenses above LSL; one average value along the entire IHR Curve |
| GASPROL (%) | Gas fuel consumed during operation over LSL as a percentage |
| OILPEROL (%) | Oil fuel consumed during operation over LSL as a percentage |
| SFPEROL (%) | Solid fuel consumed during operation over LSL as a percentage |

# Section 6:  Verifiable Heat Rates

This section of ERCOT’s Verifiable Cost Manual describes the policies and procedures for submitting Heat Rate information.

### Verifiable Heat Rate Policies

Heat rate curve data is utilized within the Nodal Verifiable Cost Process because it can be used, in conjunction with the market price of fuel, to provide a proxy for the actual fuel costs of operating a Resource.

In the ERCOT Nodal market, there are two types of Resource-specific Heat Rate Curve data that QSEs may submit: Incremental Heat Rate and Average Heat Rate data.  Average Heat Rate data must be submitted with Verifiable Minimum Energy Cost submissions.  Incremental Heat Rate data may be included with data submitted for use in calculating Mitigated Offer Caps.  ERCOT, however, will not approve any submitted heat rate data unless the QSE also develops and submits Resource-specific heat-input versus power-output curves (“I/O Curves”).  That is because ERCOT uses I/O Curves as a basis for verifying heat rate curves.

The first of the aforementioned curves, the Incremental Heat Rate (“IHR”) Curve, is defined as the first derivative of the I/O Curve.  It provides the amount of thermal energy used by a Resource to achieve an incremental change in electrical energy output (i.e., the amount of thermal energy used to produce an additional unit of output).  The second, the Average Heat Rate (“AHR”) Curve provides the average amount of energy used by a Resource to provide each unit of power output.  The AHR Curve is determined by dividing input in fuel by the power it generates for various levels of generation.

### Submission of Heat Rate Curve Data

All Heat Rate Curves and Heat Rate data must be submitted in terms of Net Output.  QSEs have two options for submitting Heat Rate Curves and Heat Rate data: submit the results of tests actually performed on the generating unit or submit the manufacturer suggested Heat Rate values.  If a QSE has reason to believe that the physical characteristics of a generator have changed so that previously submitted Heat Rate Curves overstate fuel consumption or understate efficiency, the QSE must notify ERCOT and must submit updated Heat Rate data.

Finally, submitted data relating to these curves must comply with the relevant requirements set forth below in order to be approved by ERCOT.

Input-Output Curve

For each Resource for which Heat Rate Curve data are submitted, the QSE shall submit to ERCOT the I/O Curve in accordance with the following:

- Resource total heat (or fuel) I/O curves based on design or data from comparable units, modified by available actual unit test data

- The actual data for the total heat (or fuel) I/O curve must include minimum and maximum load points and at least two intermediate load points

- I/O curves are to be fitted from data using either manual or computer techniques

- Submitted I/O Curves are to be defined by the third-order equation: y = A + Bx + Cx2 + Dx3

  - Where: x = Output in MW; y = Input in MMBtu/hr; and A-D = coefficients that define the equation

  - All values (y, x, A, B, C, D) must be provided to ERCOT

  - All coefficients (A, B, C, D) must be nonzero.

- If during the construction of the I/O Curve a Resource believes the Second-Order equation (i.e. d = 0) better defines the curve for this specific Resource, the QSE must submit to ERCOT a written explanation detailing the reason and process used to create such a curve.

- In its sole discretion, however, ERCOT may choose not to accept a QSE’s submitted I/O equation if ERCOT determines that it does not represent a Resource’s “true” heat rate curve or if the curve has not been proven in sufficient detail.

ERCOT will verify this I/O Curve and will use it a basis for verifying the IHR and AHR Curves.

#### Incremental Heat Rate Curve

All submitted IHR Curve data must:

- be determined, either mathematically, or directly from the relevant Resource’s I/O Curve;

- provide a monotonic, non-decreasing function;

- be submitted in terms of MMBtu/MWh;

- consist of at least 2, but no more than 10, pairs of IHR (MMBtu/MWh) and Output (MW) points; and

- be defined as the I/O Curve’s first derivative: IHR = dy/dx = B + 2Cx +3Dx2, unless it’s based on a second order equation as described in (5) above.

- Where: x = Output in MW; y = Input in MMBtu/MWh; and B-D = coefficients that define the equation

- All values (dy/dx, x, B, C, D) must be provided to ERCOT

If a generator does not have a monotonic and non-decreasing IHR, a QSE may instead submit both of the following:

  - their actual IHR curve; and may submit

  - a representative monotonic, non-decreasing curve that has been derived from the actual IHR.  This representative curve is subject to the above paragraph.

#### Average Heat Rate Curve

All submitted AHR Curve data must:

- be determined, either mathematically or graphically, directly from the relevant Resource’s I/O Curve;

- be submitted in terms of MMBtu/MWh;

- and Output (MW) points; and

- be defined as the I/O Curve divided by the output (x): AHR = y/x = ( A + Bx + Cx2 + Dx3) / x

- Where: x = Output in MW; y = Input in MMBtu/MWh; and A-D = coefficients that define the equation

- All values (y/x, x, A, B, C, D) must be provided to ERCOT

# Section 7:  Forced Outages of a RUC-Committed Resource

This section of ERCOT’s Verifiable Cost Manual describes the policies and procedures for submitting Event Specific Verifiable Cost data relating to a Forced Outage of a RUC Committed Resource.

### Policy for Costs Resulting from a Forced Outage

Pursuant to Section 5.6 of the Protocols, if, before reaching breaker close, a RUC-committed Resource experiences a Forced Outage resulting from startup failure, ERCOT shall include the Resource’s Event Specific Verifiable Costs in the QSE’s RUC Guarantee, limited to the lesser of:

- costs that qualify as normal startup expenses, including fuel and operation and maintenance expenses, incurred before the event that caused the Forced Outage; and

2.  the respective QSE’s Startup Offer in the RUC.

### Submission of Forced Outage Cost Data

For purposes of calculating Event Specific Verifiable Costs resulting from the Forced Outage of a RUC committed Resource, a QSE may submit the actual costs it incurred as an unavoidable and direct result of the forced outage.  All submitted costs must be sufficiently documented so as to allow ERCOT to verify them.  Only actual, incremental costs relevant to the startup attempt will be approved.  Fixed costs will not be accepted.

In addition to the above restrictions, fuel and O&M costs are subject to the following:

- QSEs should submit O&M costs in terms of dollars for the attempted start ($/attempted start).  These O&M costs must be calculated using the methodology described in Section 8:  Maintenance and Operations Cost Guidelines, but only actual, documented costs relevant to the start-up attempt should be submitted.

- QSEs may submit the amount of fuel actually consumed for the attempted start (MMBtu/attempted start).

- The ERCOT Settlement and Billing system will not automatically calculate a payment to a QSE representing a Resource that experiences a failed startup attempt.  QSEs representing a Resource that experienced a failed start-up attempt must submit a settlement dispute in accordance with the dispute process outlined in Section 9.14 Settlement and Billing Dispute Process of the Nodal Protocols in order to receive payment.  In addition to the standard information required by the dispute form located on the ERCOT Market Information System (MIS), the dispute should clearly indicate:

- The Dispatch Instruction associated with the Forced Outage;

- The total Event Specific Verifiable Costs of attempting to provide the service; and

- A description of the supporting documentation to be subsequently provided to ERCOT

A QSE seeking to have additional Forced Outage costs, which are not specifically discussed in this section, included in their RUC Guarantee must seek to have them approved by the ERCOT Board of Directors during an Executive Session at the next regularly scheduled meeting.  Requests must be presented to the Board in person by a representative of the company that incurred the costs.  Should the Board approve the inclusion of such costs in the RUC Guarantee, the submitting company must draft and submit a Nodal Protocol Revision Request (NPRR), in accordance with Protocol Section 21, Process for Protocol Revision.  Therefore, to ensure subsequent NPRRs accurately capture what the Board did (and did not) approve and clearly provide what is required of similar cost submissions in the future, initial requests to the Board of Directors must include a typewritten statement of the lobbying company’s position.  This statement must:

- define all additional costs sought and do so in a manner that is not specific to the lobbying company (e.g., “insurance deductibles”); and

- describe the documentation necessary to establish the accrual of such costs.

# Section 8:    Operating and Maintenance Cost Guidelines

This section of ERCOT’s Verifiable Cost Manual describes the policies and procedures relating to the submission of Operating and Maintenance Costs.

### Operating and Maintenance Cost Policies

Manuals drafted by the manufacturer of a generating unit and that relate to O&M should be submitted to ERCOT if a QSE submits O&M cost data for use in the Verifiable Cost process.  These manuals are necessary for ERCOT to analyze such data.

#### Verifiable Operating Costs

Operating costs are the non-fuel costs incurred while a Resource is operated, whether incurred during startup or at different output levels.  These costs may include, for example, labor costs, the cost of consumables and non-durable goods, costs related to emissions, water costs, etc.

ERCOT will only approve operating costs that are incremental costs; fixed operating costs will be rejected.  Additionally, ERCOT will only approve submitted operating costs to the extent that they reasonably represent the incremental operating costs at the stage of the power generation cycle for which they are submitted (startup, operation at LSL, or operation above LSL).  For example, ERCOT might approve the wages paid to on-call, hourly employees that are needed to help start a Resource.  On the other hand, ERCOT would not approve labor costs that are incurred regardless of an incremental deployment (such as those associated with salaried, on-call employees).  The reasonableness of an allocation is highly dependent on individual circumstances; thus, ERCOT will make this determination on a case-by-case basis.  In making this determination, ERCOT will consider factors such as whether a Resource’s operating costs relate to starting the unit or running at different operating levels, follow industry standards, and coincide with manufacturer expectations.

#### Verifiable Maintenance Costs

Maintenance costs are those costs associated with the maintenance, repair, inspection, and upkeep of generation resources, as well as their parts and equipment.  Maintenance costs will only be approved to the extent that they represent the maintenance costs resulting from an incremental period of usage.

Verifiable Maintenance Costs are a proportionate share of future maintenance costs; the allocation of maintenance costs not yet borne must be based upon Resource usage that is reasonably anticipated to occur during the interim.  For example, maintenance expenses might be allocated using hours-based, starts-based, or equivalent-operating-hours criteria

In considering whether or not to approve submitted maintenance costs, ERCOT will consider factors such as whether the costs and frequency of maintenance anticipated by a Resource coincides with manufacturer expectations; whether the starts and usage anticipated by a resource is practical, or is likely given the type of generator and general market conditions, or coincides with historical data; etc.  Prospective opportunity costs, such as the loss of generating capacity during maintenance, are not verifiable and, therefore, will not be approved by ERCOT.

#### Event Specific Verifiable O&M Costs

Event Specific Verifiable O&M Costs are limited to provable O&M costs that are actually incurred as a direct result of the specific events detailed in Section 7: Forced Outage of a RUC-Committed Resource.

O&M costs incurred during this event (to the extent they exceed O&M costs incurred during any similar, normal operations) must not be incorporated when Resources calculate their incremental Verifiable O&M Costs.  Because these added O&M costs are incurred as a result of anomalous and infrequent events; it is improper to include them when allocating O&M costs to incremental generation.  QSEs, however, may submit Event Specific Verifiable O&M Costs by following the dispute process outlined in Section 9.14 Settlement and Billing Dispute Process in the Nodal Protocols.

### Options for Submitting Verifiable O&M Costs

QSEs may submit Verifiable O&M Costs (but neither Event Specific Verifiable O&M Costs nor Fuel Costs) using either of the following two methods:

#### Option 1:  Submission of O&M costs with Documentation and Calculated Using a Resource Specific Methodology

- QSEs must submit the O&M costs of starting up ($/start) for each start type, cold, hot and intermediate, calculated according to a specific methodology that the Resource previously requested and which ERCOT approved.

- QSEs must submit the O&M costs of operating at LSL ($/MWh) calculated according to a specific methodology that the Resource previously requested and which ERCOT approved.

- QSEs may submit the O&M costs of operating above LSL ($/MWh) calculated according to a specific methodology that the Resource previously requested and which ERCOT approved.

- The amounts and methods used to calculate both the operation and maintenance components of Verifiable O&M Costs must be clearly detailed.

- Any methodology must meet the guidelines of ERCOT’s Verifiable Cost Manual.

- ERCOT may rescind its prior approval of Verifiable Costs submitted under this Option at any time, but it must have a material, factual basis for doing so.  For example, ERCOT may rescind its approval if it subsequently determines the Resource-specific method of calculating O&M was inherently flawed.

#### Option 2:  Submission of O&M costs with Documentation and Calculated Using the Methodology Described in Appendix 1

- QSEs must submit O&M costs that result from starting for each type, cold, hot and intermediate ($/start).

- QSEs must submit O&M costs that result from operation at the LSL ($/MWh).

- QSEs may submit O&M costs that result from operation above LSL ($/MWh).

- QSEs must submit a detailed account of the amounts used to calculate these costs and the manner in which they applied the methodology set forth in Appendix1.

- ERCOT may rescind its prior approval of Verifiable Costs submitted under this Option at any time, but it must have a material, factual basis for doing so.  For example, ERCOT would rescind its approval of part (or all) of the labor costs associated with a scheduled outage upon learning that some (or all) of the workers were salaried.

### Additional Conditions Applicable to Option 1 and Option 2

O&M Costs submitted to ERCOT under either Option 1 or Option 2 above are subject to additional conditions, as set for below.

#### Conditions Applicable to All Types of Resources

- All O&M costs must be incremental; that is, they may not include fixed or capital costs.

- Submitted O&M documentation must be Resource specific.  Exceptions may be granted for Combined-Cycled Resources on a case-by-case basis.

- O&M costs must not include fuel costs.

- Both individual component costs and the final aggregated O&M values must be clearly detailed and shown as $/start or $/MWh, as applicable.

- Maintenance costs are to be calculated taking into account all available maintenance history (limited by the number of years in a maintenance period where applicable) regardless of unit ownership.

- QSEs must submit operating costs (including water, chemical, emission credits, consumables, etc) per-start ($/start) and per hour of operation at LSL ($/MWh).

- QSEs that include water consumption as part of their operating costs must include documentation indicating the number of gallons used and the output level and duration of operation during which the water was used. The price(s) paid for water ($/gal) must also be documented and submitted.

- QSEs including emission credits as part of their operating costs must provide ERCOT a description of the credits and the methodology used to allocate their costs to O&M.

- All consumables included in O&M must be clearly documented and include both the quantities used and prices paid.

- QSEs must provide a description of all components included as operating costs and the reason for their inclusion.

- If a QSE feels that a unit modification or required change in operating procedures will affect the unit's maintenance cost, revised maintenance costs must be submitted to ERCOT for approval.

- A QSE may submit only one value ($/MWh) for the O&M cost of operation at LSL.

- 13.A QSE may submit only one single average value ($/MWh)              for the O&M cost of operation above LSL.

14.  A QSE with a currently effective Long Term Service Contract (LTSA) for the overhaul and maintenance work on a Resource may include the costs described within that LTSA as Verifiable O&M costs if the following conditions are met:

  - The maintenance costs to be included are incremental costs and are otherwise consistent with the definitions and uses of those costs within this Manual.

  - A dollar value for each component of the variable maintenance costs is specifically set by the LTSA.

  - Cost curves showing variable O&M values versus time are submitted.

  - ERCOT can verify the incremental or variable maintenance costs ($/MWh) or ($/start) in the LTSA.

  - LTSA contracts must be submitted directly to ERCOT for review and approval and must be currently effective.

#### Conditions Applicable to Combustion Turbines

- The Maintenance Cost component of starting a Combustion Turbine Resource ($/start) or operating at or above LSL ($/MWh) may be calculated and applied on a "per equivalent service hour" (ESH) basis as described in Appendix 1B. If this option is chosen, the calculation can be based on maintenance expenses in a Maintenance Period chosen by the Resource.

- Resources that determine their costs on an ESH basis may use either the methodology described below in Appendix 1B or another methodology approved by ERCOT to determine their ESH values.

- The maintenance cost component for a given period must be stated as the total dollars spent per period.

- QSEs must submit the total number of running hours during the maintenance period.

- QSEs must submit the total number of starts during the maintenance period.

- QSEs must submit the average time in hours spent starting up, shown per start type if possible, in the maintenance period.

- A QSE may include manufacturer recommended values in ESH calculations, if approved by ERCOT in advance.

- QSEs must calculate the maintenance costs per hour of operation in the maintenance period ($/hr) as the total maintenance cost divided by the ESH, unless ERCOT approves another methodology in advance.

- QSEs must calculate the maintenance cost ($/hour) times the average length of start in hours to arrive at an average maintenance cost per start ($/start).

- QSEs must divide the calculated maintenance cost ($/hour) by the LSL (MW) to determine the average maintenance cost per hour of operation at LSL.

- All O&M costs calculated using equivalent service hours (ESH) must clearly show how the ESH are calculated.

#### Conditions Applicable to Fossil Thermal Resources

- QSEs must submit the methodology that was used to determine their O&M costs ($/start or $/MWh) for a maintenance period using either a Resource-specific methodology or the one described in Appendix 1a.

- Any methodology used to determine O&M costs ($/start or $/MWh) for the maintenance period should also describe the costs on a $/MMBtu basis.

#### Maintenance Period Conditions

When calculating maintenance costs, a 10-year historical period should be used. If data are not available for the previous 10 years, then the length of the maintenance period is the length of time for which the data are available.   QSEs may determine maintenance costs on an annual basis and then normalize them for the entire 10 year (or less) maintenance period.  In addition, QSEs must utilize the Cost Escalation Factors derived by ERCOT, see Figure 2 below, to escalate historical maintenance costs to present value.

ERCOT will assess all submitted O&M costs for reasonability.  When making this assessment for submissions with a maintenance period less than 10 years, ERCOT will consider Original Equipment Manufacturers (OEM) recommendations and similar Resources’ costs.

### Operating and Maintenance Cost Submissions Types

Appendix 4:  Operating and Maintenance Cost Categories provides a list of the types of O&M costs that QSEs may use when calculating O&M data.

# Section 9:  Timelines Applicable to the Submission and Review of Verifiable Costs

This is the section of ERCOT’s Verifiable Cost Manual that details the processes and procedures that apply to the submission and approval of Verifiable Costs.

### Submission and Approval of Verifiable Cost Data

A QSE initiates the Verifiable Cost Process by submitting their costs to ERCOT in a manner to be determined.  In the Nodal market, Verifiable Costs take effect three (3) Business Days after the day they are approved by ERCOT.  They will only be used prospectively.

A separate process applies to the submission of Event Specific Verifiable Costs, which must be submitted in accordance with the dispute process outlined in Section 9.14, Settlement and Billing Dispute Process of the Nodal Protocols.  Event Specific Verifiable Costs take effect upon approval.  They will be applied retroactively and will be used to resettle the Operating Day on which the costs were incurred.  Furthermore, Event Specific Verifiable Costs are only valid for the specific event for which they are initially submitted; they will not be reused for any similar, subsequently occurring events.

When a QSE initiates an iteration of the Verifiable Cost Process, the following must be submitted to ERCOT:

- the Verifiable Cost Template with all relevant fields completed;

- electronic copies of supporting documentation and any required affidavits and signatures;

- a list of any physical supporting documentation that is being sent to ERCOT; and

- any other relevant materials that this document requires to be submitted.

Note: The timeline within this section pertains to Verifiable Costs, but not to Event Specific Verifiable Costs.

A QSE may submit Verifiable Costs at any time, but it must submit those costs no later than 30 days after the QSE receives five (5)  RUC instructions for that Resource in a calendar year..

If the RUC process instructs a QSE to start a Resource 5 times within a calendar year, ERCOT will notify the QSE to update its Verifiable Cost data, but ERCOT will not make such a request more frequently than annually.  ERCOT will also notify a QSE to update its Verifiable Cost data if at least five years have passed since ERCOT last approved the Resource’s Verifiable Cost data.  If a QSE does not submit updated data within 30 days of being notified to do so under either of these scenarios, ERCOT will determine payments using the lower of the following until updated costs are approved:

- Resource Category Startup Generic and Resource Category Minimum-Energy Generic Caps; and

- the Resource’s most recently approved Verified Startup and Minimum Energy Costs (except for Event Specific Verifiable Costs which are submitted and applied to a specific Operating Day).

### Timeline Applicable to the Submission and Approval of Verifiable Costs

Verifiable Costs submitted to ERCOT after implementation of the Texas Nodal Market will be reviewed based on the following timeline:

- Once a Resource’s verifiable cost data is submitted, ERCOT will review the data for completeness and accuracy and to ensure the submission has met all of the requirements described within the guidelines of ERCOT’s Verifiable Cost Manual.

- Within fifteen (15) Business Days from the date on which the data is submitted, ERCOT will inform the submitting QSE that:

  - the review is complete and the amounts that ERCOT is able to approve are provided to the QSE;

  - the review is complete and the submission was rejected,

  - the submission was incomplete, or

  - additional supporting documentation must be submitted.

- A QSE must submit any additional information ERCOT requests no later than thirty (30) Business Days after ERCOT first requests it.  Multiple submissions of data are permissible, but a complete submission, which includes all information requested, must be made within thirty Business Days of ERCOT”s request.

- Upon receiving additional documentation, ERCOT will acknowledge receipt and review it for completeness within ten (10) Business Days.  If ERCOT receives all requested information, ERCOT will make a decision to approve or reject Verifiable Costs within the same ten (10) Business Days.

- At any time during the approval process, ERCOT may request additional information from a QSE.  All requests must be fulfilled within thirty (30) Business Days.

- From the time that ERCOT’s final decision is provided , the QSEs has five (5) Business Days to either accept or reject any amounts approved by ERCOT.  If the QSE rejects the amounts calculated by ERCOT, generic costs and caps will be used.  Failure to accept or reject ERCOT’s approvable amounts within these five Business Days will be deemed as acceptance.

- If ERCOT does not approve a Resource’s Verifiable Costs, the QSE may file an appeal in accordance with the procedure described in Section 11:  Appealing Rejected Verifiable Costs.

- ERCOT’s deadline for reviewing Verifiable Costs submissions (15 or 10 Business Days) applies from the date ERCOT receives a submission.  ERCOT will not postpone review until a QSE’s submission deadline has passed.

#### Chart 4:  Verifiable Cost Submission Timeline

Section 10:  Timeline for Rescinding Approval of Verifiable Costs

### Policy for Rescinding Previously Approved Verifiable Costs

ERCOT has authority to, at any time, review and reject the Verifiable Costs that it has previously approved.  ERCOT must, however, be judicious and impartial when rescinding its approval.  Furthermore, ERCOT must not rescind Verifiable Costs unless there is a material, factual basis that justifies rescission.  For example, rescission would be justified upon ERCOT determining: the submitted data was inaccurate based on public filings or comparisons with substantially similar generators; a type of O&M cost submitted was a fixed cost (or a portion thereof); changed Nodal Market policies disallow or limit certain costs, accounting practices, or calculation methodologies; etc.  ERCOT must approach these types of situations with the aim of rescinding approval only to the extent necessary to eliminate all inaccurate cost data.

Rescission of approval does not affect the timing of Verifiable Cost updates mandated under Section 5.6.1 of the Nodal Protocols.

If the entire verifiable cost of a Resource (Fuel and O&M) is rescinded, then the payment to the QSE will be based on the Resource Category Generic Caps.

If ERCOT only rescinds approval for a specific type of costs (or reduces the approved amount), ERCOT will recalculate the Resource’s Verifiable Costs using the corrected data, other unchanged amounts, and the previous calculation methodology.  The QSE has the option to reject ERCOT’s recalculated amount.  Generic costs will apply to Resources that reject the amounts recalculated by ERCOT.

Finally, a decision by ERCOT to rescind approval does not immediately take effect.  ERCOT must first provide an affected QSE with notice and then permit that entity a chance to respond.  During this interim period, the unaltered Verifiable Costs will still be in use.

### Timeline Applicable to Rescinding Approval of Verifiable Costs

The process of rescinding Verifiable Costs proceeds according to the following timeline:

- Once ERCOT decides to initiate rescinding a Resource’s Verifiable Costs, ERCOT must provide the QSE with notice of:

  - the impending rescission;

  - the amounts and or categories that will no longer be approved;

  - the specific reason(s) for rescission;

  - the date on which notice is being provided; and

  - the last date a Resource may submit explanatory information.

- A QSE has ten (10) Business Days after the day on which they receive notice to provide ERCOT with any documents, justifications, and other information that supports inclusion of the cost in question.  Not submitting supporting information or making untimely submission is deemed as acquiescence.

- After the passing of the last day a QSE can submit explanatory information, ERCOT has seven (7) Business Days to review all submissions and to reach a decision.

- ERCOT will inform QSEs whether or not it will be rescinding its prior approval within one (1) Business Day of reaching its decision.

- If ERCOT is sufficiently persuaded by a QSE’s supplementary information and determines that no costs had been improperly included, ERCOT will not update the stored Verifiable Costs.

- If a QSE did not submit any information, if a QSE submitted information that was unpersuasive, or if a QSE submitted information that only persuaded ERCOT to approve a portion of the costs in question, ERCOT must recalculate and update the Resource’s Verifiable Costs.  These recalculated amounts will be communicated to the QSE at the same time that ERCOT informs the QSE of its decision.

- Once ERCOT’s final decision and recalculated amounts are available, QSEs have five (5) Business Days to accept or reject any amounts approved by ERCOT.  If a QSE rejects the amounts recalculated by ERCOT, generic costs will be used.  Failure to accept or reject ERCOT’s approvable amounts within these five Business Days will be deemed as acceptance.

Rejection of ERCOT’s recalculated amounts will not affect the timing of Verifiable Cost updates mandated under Section 5.6.1 of the Nodal Protocols.

- Any changes to a Resource’s Verifiable Costs—whether due to recalculation or replacement with generics—take effect three (3) Business Days after a QSE accepts them.

Any QSE wishing to appeal a rescission of their Verifiable Costs must follow the procedures provided in Section 11:  Appealing Rejected Verifiable Costs.

Chart 5:  Timeline Applicable to Rescinding Approval of Verifiable Costs

Section 11:  Appealing Rejected Verifiable Costs

This section of ERCOT’s Verifiable Cost Manual describes the policies and procedures that apply when a QSE wishes to appeal a rejection of their Verifiable Costs.

### Appealing Verifiable Cost Rejections

If ERCOT denies a QSE’s Verifiable Cost submission (or a portion thereof) or if ERCOT rescinds its prior approval of a QSE’s Verifiable Cost submission (or a portion thereof), the QSE may appeal ERCOT’s decision.

If any additional costs are approved as a result of the appeal process, the additional costs will only be used going forward.  The newly approved costs will not be used to resettle prior Operating Days.  If the inclusion of the newly approved costs represents a shift in ERCOT’s Verifiable Cost doctrine, this Manual will be updated and, if necessary, Nodal Protocol Revision Requests will be drafted.  These updates will inform Market Participants of the doctrinal shift, will allow similarly situated QSEs to submit analogous costs, and, thus, will ensure that ERCOT approves Verifiable Costs without bias.

### Verifiable Cost Appeal Process and Timeline

There are [undetermined] levels of review available to QSEs wishing to appeal a rejection of their Verifiable Costs.  They are as follow:

#### First Level of Appeal

- QSEs initiate appeals of rejected Verifiable Costs.  This is done by submitting an appeal to ERCOT.

- Appeals must be submitted no more than twenty (20) Business Days after ERCOT informs a QSE of its final decision and the rejection.  If a QSE does not submit an appeal within this 20 Business Day period, the QSE is deemed to have accepted the rejection.

- A Review Meeting between the appealing QSE and ERCOT must be scheduled to occur within twenty (20) Business Days of a QSE initiating an appeal.  This Review Meeting may be held over the phone.  When scheduling a Review Meeting, QSEs must name a primary and an alternate representative.  The QSE’s representative will meet with [TBD ERCOT employee(s)], who will represent ERCOT and who is outside of the Settlements and Billing Department.

- Both ERCOT and the appealing QSE must submit a position paper to each other at least two (2) Business Days before the Review Meeting.  These papers are to:

  - explain why a cost or a portion thereof should or should not be included in the QSE’s Verifiable Costs;

  - include copies of documentation that proves the costs were incurred; and

  - reference sections within the Nodal Protocols, this Manual, or other controlling documents that supports inclusion/exclusion of the disputed costs.

- The goal of this first level of appeal is to reach an agreement and understanding between ERCOT and the QSE as quickly as possible by providing dialog between the QSE and a representative of ERCOT that is external to the Settlements and Billing department.

  - If [TBD ERCOT employee(s)], finds a QSE’s arguments and position paper to be persuasive, ERCOT will update the Verifiable Costs to the extent approved by the representative(s).  The inclusion of these costs will take effect not more than three (3) Business Days after approval by ERCOT’s representative(s).

  - If ERCOT’s representative(s) is unpersuaded by the QSE, the rescission or rejection stands.

  - If at any time it becomes apparent from discussions between the QSE and ERCOT that no agreement will be reached or that the positions of the QSE and ERCOT are diametrically opposed, this level of review should quickly be terminated to allow the QSE to elevate their appeal to the next level.

- Review at this level must terminate within 50 Business Days after an appeal is submitted.  If no agreement is reached or if ERCOT’s representative does not make a decision in this time, the appeal is deemed denied and the rejection/rescission stands.

#### Second Level of Appeal

This level of appeal is only available to QSEs that unsuccessfully appealed ERCOT’s treatment of their Verifiable Costs through the first level of appeal.  That is, this level of appeal is only open to QSEs that timely appealed Settlements and Billing’s rejection of their costs but were unable to persuade ERCOT’s representative to approve inclusion of a cost (or the extent requested).

A QSE may only appeal the costs or amounts that were rejected by ERCOT’s representative in the first level of appeal.  If ERCOT’s representative and a QSE reach an understanding in the first level of appeal, their agreed upon treatment of a cost is not appealable.  For example, if a QSE and ERCOT’s representative agree to include 50% of disputed cost “A” but do not agree as to the treatment of disputed cost “B”, a QSE may only appeal for the inclusion of cost “B”.  On the other hand, if a QSE never assents to the partial inclusion of cost “A”, the QSE   may appeal to have all of cost “A” included.

- QSEs initiate this second level of appeal by submitting an appeal to ERCOT through MIS.

- Appeals must be submitted no more than ten (10) Business Days after the termination of the first level of appeal, as determined by the status shown on the MIS. If a QSE does not submit an appeal within this 10 Business Day period, the QSE loses their right to appeal.

- Appeal at this level is determined by [undetermined party—possibly IMM or ERCOT’s unaffiliated members of BoD].  The [undetermined party] will make a decision at [undetermined place/time—next board meeting?  a meeting with ERCOT and the QSE within 60 days?].

- To aid [undetermined party] in making their decision, they will receive the position papers and meeting notes from the first level of appeal.  Additionally, representatives of ERCOT and the QSE must attend the [undetermined place/meeting] to answer any questions that they [undetermined party] may have.

- If the [undetermined party] approves the inclusion of any additional costs or amounts, ERCOT will update the QSE’s Verifiable Costs to the extent approved.  The inclusion of these costs will take effect not more than three (3) Business Days after approval by the [undetermined party].

- If [undetermined party] denies inclusion of the appealed costs or amounts, ERCOT’s the rescission or rejection stands.

# Appendix 1A:  Methodology for Determining Maintenance Costs of Nuclear and Fossil Steam Units

### Method of Calculating Total Maintenance Costs using 1998 as the Operating Year

TMD = (A97 * F97) + (A96 * F96) …. + (A88 * F88)

TFuel = Fuel97 + Fuel96…. + Fuel 88

TSD = (B97  * F97) + (B96 * F96) … +  (B88 * F88)

TS = S97 + S96…. + S88

MA98 = TMD / TFuel

SMA98 = TSD / TS

Where:

- TMD  =  Total Maintenance Dollars in period

- A  =  Total Maintenance spent in period

- B  =  Total Maintenance costs that are startup related

- F  =  Escalation Factor for a particular year, as approved by ERCOT and based on escalation factors shown in Figure 2.

- TFuel  =  Total fuel burn or heat released (for same years used in TMD)

- Fuel  =  Total annual fuel consumed (expressed in tons, barrels, gallons, etc.) total annual heat released (expressed in MBTUs, etc.)

- TS  =  Total Starts

- TSD  =  Total Start maintenance Dollars

- S  =  Number of Starts per year

- MA  =  Maintenance Cost Adder ($/unit of fuel or heat)

- SMA  =  Start Maintenance Cost Adder ($/start-up)

Total Maintenance Dollars (TMD) plus Total Start maintenance Dollars (TSD) must be included with a Verifiable Cost submission and will be reviewed by ERCOT.

# Appendix 1B:	Methodology for Determining the Maintenance Costs of CT and CCP Units

### Example Calculation of Total Maintenance Dollars using Operating Year of 1998

TMD = (C97 * F97) + (C96 * F96) … + (C88 * F88)

Where:

- C  =  Total dollars spent on maintenance during a particular year

- F  =  Escalation Factor for a particular year, as approved by ERCOT and based on the Cost Escalation Factors as shown in Figure 2.

Note: TMD must be calculated for the same historical period as Equivalent Service Hours.

### How to Calculate Equivalent Service Hours (ESH)

ESH = (A * number of starts) + Z hours + (B * Y hours)

Where:

- A  =  Cyclic starting factor (A = 5.0 for aircraft - type CT's; A = 10.0 for industrial - type CT's)

For example, the incremental maintenance charged to one start on an industrial - type CT is equivalent to the incremental maintenance attributable to ten hours of base load operation.

- B  =  Cyclic peaking factor (B = 3.0 for all CT's)

This means that the additional incremental maintenance charged to the incremental energy between base and peak loads is equivalent to the incremental maintenance attributable to three hours of base load operation.

- Z  =  Total unit operating hours at any load level

- Y  =  Hours above base load temperature limit

Note: ESH must be calculated using all available history in the Maintenance Period. QSEs may propose alternative cyclic starting factors or cyclic peaking factors for individual units. Such alternative factor proposals should include supporting documentation (e.g., manufacturer recommendation) of this new factor, to be reviewed for approval by ERCOT, on a case-by-case basis.

### How to Calculate Equivalent Hourly Maintenance Cost (EHMC)

EHMC = TMD / ESH

Where:

- TMD = Total maintenance dollars as approved by ERCOT

- ESH = Equivalent service hours

- EHMC is in terms of $/hour

### Applying Equivalent Hours Maintenance Costs to Calculate Maintenance Rates:

Starting Maintenance Cost ($/Start) = A *  EHMC

Where:

- A  =  Cyclic starting factor as defined above

- EHMC  =  Hourly Maintenance Cost ($/Hour)

This hourly value is assigned as a capacity cost and is independent of unit loading.

- Peak Incremental Maintenance Rate ($/MWh) = [B/peak pickup] x EHMC

Peak pickup is equal to the difference between a CT's energy outputs at base and at peak loading levels.

Comments:

- The above formulations are applicable for determination of Maintenance Cost Adder for both aircraft and industrial type combustion turbines.

- CT incremental costs may only be developed and applied on a unit-by-unit basis

### Example Calculation of the Maintenance Adder for a Combustion Turbine

If:

- Peak Hours = Y = 200hr

- Service Hours = Z = 2000hr (Total Base Peak Hours)

- Number of Starts = 300

- Peak Pickup = 5 MW

- A = 10, B = 3 (Note: A = 5 for aircraft engine CT's)

- TMD  =  $100,000 (Actual historical maintenance data escalated to present value)

Then Equivalent Hourly Maintenance Cost:

EHMC = $100,000 / [ (10*300) + 2,000 + (3*200) ] = $17.86/hr

And Maintenance Rates:

Starting Maintenance Rate = A * EHMC = 10 * $17.86/hr = $178.60/start

Peak Incremental Maintenance Rate = B / [peak pickup] * EHMC

= 3/5 * $17.86 = $10.72/MWh

# Appendix 2:  Auxiliary Data

### Figure 1:  Emission Factors

Source:  California Energy Commission, Comparative Costs of California Central Station Electricity Generation Technologies, June 2007 CEC-200-2007-011-SD Report, p. 18 available at http://www.energy.ca.gov/2007publications/CEC-200-2007-011/CEC-200-2007-011-SD.PDF.

### Figure 2:  Cost Escalation Factors

Chronology of Maintenance Escalation Numbers

| Year | Escalation Factors (1) | $ spent on Maintenance (before escalation factor applied) | Escalated Maintenance Amounts |
| --- | --- | --- | --- |
| 1997 | 1.4210 | $0 | $0 |
| 1998 | 1.3920 | $0 | $0 |
| 1999 | 1.3700 | $0 | $0 |
| 2000 | 1.2840 | $0 | $0 |
| 2001 | 1.2540 | $0 | $0 |
| 2002 | 1.2170 | $0 | $0 |
| 2003 | 1.2090 | $0 | $0 |
| 2004 | 1.1460 | $0 | $0 |
| 2005 | 1.0810 | $0 | $0 |
| 2006 | 1.035 | $0 | $0 |
| 2007 | 1 | $0 | $0 |

Source:  PJM, PJM Manual 15:  Cost Development Guidelines, p. 24-25, available at http://www.pjm.com/contributions/pjm-manuals/pdf/m15.pdf.

# Figure 3:  Data to Support Verifiable Costs

Time

Time

Time

MBTU/hr

MBTU/MWh

MW

Heat-Rate Over Time

Fuel-Rate

MW over Time

Breaker Close

Breaker Open

LSL

# Appendix 3:  Examples of Calculations of Power Purchase  & Tolling Agreements Verifiable Cost Caps

Example 1:  PPAs indicating only one (1) cost for the Resource; no fuel volume shown

Example 1 consists of Tables 1, 2, 3 and 4 and demonstrates the cases where the PPA has filed only the total cost for the Resource and shows the calculations for Startup costs under cold, hot and intermediate conditions as well as costs at LSL (Table 4).  This example demonstrates how the PPA is capped under these conditions.

In all the cases where the PPA contains only the total cost for the Resource, the verifiable costs are capped by the highest total cost calculated from similar non-PPA units.  The verifiable costs established for the capped PPA Resources are deemed to have the same fuel and O&M costs as the highest non-PPA unit.

Table 1:  Cold Start-Up Cost for Simple Cycle > 90 MW Resources

| Cold Type | Unit 1 | Unit 2 | Unit 3 | Unit 4 | Unit 5 | Unit 6 | Unit 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PPA Cost ($) |  |  |  |  | 9,600(1) | 15,000(1) | 10,000(1) |
| Fuel to LSL (MMBtu) | 100 | 120 | 90 | 80 |  |  |  |
| Average FIP over the last 30 days ($/MMBtu) | 10 | 10 | 10 | 10 |  |  |  |
| Total Fuel Cost ($) Fuel x FIP | 1,000 | 1,200 | 900 | 800 |  |  |  |
| Total O&M Cost ($) | 8,700 | 7,000 | 6,000 | 9,000 |  |  |  |
| Total Fuel + O&M Cost ($) | 9,700 | 8,200 | 6,900 | 9,800 |  |  |  |
| Approved VC Fuel Cap (MMBtu) | N/A | N/A | N/A | N/A | N/A | 80 | 80 |
| Approved VC O&M Cap ($) | N/A | N/A | N/A | N/A | 9,600 | 9,000 | 9,000 |

(1) Approved VC = Minimum {Reference Resource or PPA}

In Table 1, the largest cost for the four Resources (Units 1 through 4) is $9,800 which is that for Unit 4.  Thus PPAs for Units 6 and 7 are capped at $9,800 while the cost for PPA Unit 5 is set at $9,600 (the filed amount).

Table 2:  Hot Start-Up Cost for Simple Cycle > 90 MW Resources

| Hot Type | Unit 1 | Unit 2 | Unit 3 | Unit 4 | Unit 5 | Unit 6 | Unit 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PPA Cost* ($) |  |  |  |  | 4800(1) | 7500(1) | 5000(1) |
| Fuel to LSL (MMBtu) | 65 | 80 | 70 | 60 |  |  |  |
| Average FIP over the last 30 days ($/MMBtu) | 10 | 10 | 10 | 10 |  |  |  |
| Total Fuel Cost ($) Fuel x FIP | 650 | 800 | 700 | 600 |  |  |  |
| Total O&M Cost ($) | 6000 | 4000 | 3500 | 6000 |  |  |  |
| Total Fuel + O&M Cost ($) | 6,650 | 4,800 | 4,200 | 6,600 |  |  |  |
| Approved VC Fuel Cap (MMBtu) | N/A | N/A | N/A | N/A | N/A | 65 | N/A |
| Approved VC O&M Cap ($) | N/A | N/A | N/A | N/A | 4,800 | 6,000 | 5,000 |
| (1) Costs for a hot start is calculated by multiplying a Relative Weight 0.5 by the cost for a cold start |  |  |  |  |  |  |  |

In Table 2 the highest cost of the four non-PPA Resources is that of Unit 1 ($6,650); thus the Cap for the PPA Units is set at this level.  Note that the verifiable costs for PPA Units 5 and 7 are set at their filed amounts since they are less than the Cap.

Table 3:  Intermediate Start-Up Cost for Simple Cycle > 90 MW Resources

| Inter Type | Unit 1 | Unit 2 | Unit 3 | Unit 4 | Unit 5 | Unit 6 | Unit 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PPA Cost* ($) |  |  |  |  | 6720(1) | 10500(1) | 7000(1) |
| Fuel to LSL (MMBtu) | 75 | 100 | 80 | 70 |  |  |  |
| Average FIP over the last 30 days ($/MMBtu) | 10 | 10 | 10 | 10 |  |  |  |
| Total Fuel Cost ($) Fuel x FIP | 750 | 1000 | 800 | 700 |  |  |  |
| Total O&M Cost ($) | 7000 | 5000 | 4500 | 7000 |  |  |  |
| Total Fuel + O&M Cost ($) | 7750 | 6000 | 5300 | 7700 |  |  |  |
| Approved VC Fuel Cap (MMBtu) | N/A | N/A | N/A | N/A | N/A | 75 | N/A |
| Approved VC O&M Cap ($) | N/A | N/A | N/A | N/A | 6,720 | 7,000 | 7,000 |
| (1) Costs for an intermediate start is calculated by multiplying a Relative Weight 0.7 by the cost for a cold start |  |  |  |  |  |  |  |

Similarly in Table 3, the highest cost for Resources 1 through 4 is that of Unit 1 and the Cap for the PPAs is $7,750.  Please note again that the verifiable costs for PPA units 5 and 7 are set at the filed amounts (less than the calculated Cap).

Table 4:  Minimum Energy {@ LSL} PPA Cap for Simple Cycle > 90MW Resources

| Resource | PPA Cost ($/MWh) | Fuel at LSL (MMBtu/MWhr) | Average FIP over the last 30 days ($/MMBtu) | Resource Fuel MMBtu/MWh = PPA Cost/FIP | Resource Fuel Cost ($/MWh) = Fuel x FIP | O&M ($/MWh) | Total Cost ($/MWh) Fuel +O&M | Approved VC Fuel Cap (MMBtu) | Approved VC O&M Cap ($) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | N/A | 19 | 10 | N/A | 190 | 17 | 207 | N/A | N/A |
| 2 | N/A | 21 | 10 | N/A | 210 | 20 | 230 | N/A | N/A |
| 3 | N/A | 18 | 10 | N/A | 180 | 14 | 194 | N/A | N/A |
| 4 | N/A | 15 | 10 | N/A | 150 | 15 | 165 | N/A | N/A |
| 5 | 300 |  |  |  |  |  |  | 21 | 20 |
| 6 | 130 |  |  |  |  |  |  |  | 130 |
| 7 | 200 |  |  |  |  |  |  |  | 200 |

Table 4 demonstrates the calculation of the Cap for Minimum Energy @ LSL when the verifiable costs filed by the QSE representing the PPA include only a single cost.  In this Table the verifiable total costs are shown for Non-PPA units 1 through 4.  The highest of these is Unit 2 with a total verifiable cost of $230/MWh which becomes the Cap for the PPA units 5, 6 and 7.  As before, if the costs of the PPA units are more than the Cap, the fuel and O&M components for the PPA Resource are set equal to the same components in the Reference Resource (Resource 2).  In cases where the PPA filed costs are less than those of the Reference Resource, the verifiable costs will be treated as O&M only and will not have a fuel component.

Example 2:  PPAs indicating both Fuel and O&M costs

Example 2 below demonstrates the calculation of the PPA Cap when the PPA verifiable cost filing by the QSE representing the PPA includes fuel and O&M costs stated separately.  The Cap for the PPAs is set by calculating the total O&M cost for the set of similar units and choosing the highest of those as the Cap.  This example also shows (Table 5) how ERCOT is going to determine the Reference Resource and the PPA cap for operations above LSL.

Table 1:  Cold Start-Up Cost for Simple Cycle > 90 MW Resources

Note: Units 5, 6 and 7 are under a PPA, therefore, their O&M are capped

| Cold Type | Unit 1 | Unit 2 | Unit 3 | Unit 4 | Unit 5 | Unit 6 | Unit 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel to LSL (MMBtu) | N/A | N/A | N/A | N/A | 120 | 80 | 140 |
| Total O&M Cost ($) | 8,700 | 7,000 | 6,000 | 9,000 | 7,000 | 8,000 | 10,000 |
| Approved VC Fuel (MMBtu) | N/A | N/A | N/A | N/A | 120 | 80 | 140 |
| Approved VC O&M Cap ($) | N/A | N/A | N/A | N/A | 7,000 | 8,000 | 9,000 |

In Table 1 above, the Cap is established by Unit 4 at $9,000.  Of the three PPA Resources, only unit 7 is higher than the Cap at $10,000.  Thus the verifiable O&M cost for PPA Unit 7 is set at $9,000 since Unit 4 establishes the O&M cap.

Table 2:  Hot Start-Up Cost for Simple Cycle > 90 MW Resources

| Hot Type | Unit 1 | Unit 2 | Unit 3 | Unit 4 | Unit 5 | Unit 6 | Unit 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel to LSL (MMBtu) | N/A | N/A | N/A | N/A | 55 | 80 | 90 |
| Total O&M Cost ($) | 6,000 | 4,000 | 3,500 | 6,000 | 5,000 | 5,900 | 8,000 |
| Approved VC Fuel (MMBtu) | N/A | N/A | N/A | N/A | 55 | 80 | 90 |
| Approved VC O&M Cap ($) | N/A | N/A | N/A | N/A | 5,000 | 5,900 | 6,000 |

The same principle applies in Table 2.  The PPA costs are capped by Unit 1.  As in Table 1, the O&M costs for the Unit establishing the Cap are applied to the PPA units which filed higher total O&M costs than the Cap.

Table 3:  Intermediate Start-Up Cost for Simple Cycle > 90 MW Resources

| Inter Type | Unit 1 | Unit 2 | Unit 3 | Unit 4 | Unit 5 | Unit 6 | Unit 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel to LSL (MMBtu) | N/A | N/A | N/A | N/A | 100 | 65 | 120 |
| Total O&M Cost ($) | 6,700 | 7,000 | 4,500 | 7,000 | 6,500 | 7,000 | 8,000 |
| Approved VC Fuel (MMBtu) | N/A | N/A | N/A | N/A | 100 | 65 | 120 |
| Approved VC O&M Cap ($) | N/A | N/A | N/A | N/A | 6,500 | 7,000 | 7,000 |

In Table 3 the PPA costs are capped by Unit 4.  As in Table 1, the O&M costs for the Unit establishing the Cap are applied to the PPA units which filed higher total costs than the Cap.

Table 4:  Minimum Energy {@ LSL} PPA Cap for Simple Cycle > 90MW Resources

| Resource | HSL (MW) | Fuel at LSL (MMBtu/MWhr) | O&M ($/MWh) | Approved VC Fuel ($/MWh)(1) | Approved VC O&M ($/MWh)(1) |
| --- | --- | --- | --- | --- | --- |
| 1 | 300 |  | 17 |  |  |
| 2 | 250 |  | 20 |  |  |
| 3 | 280 |  | 14 |  |  |
| 4 | 320 |  | 15 |  |  |
| 5 | 290 | 25 | 25 | 25 | 20 |
| 6 | 310 | 30 | 21 | 30 | 20 |
| 7 | 300 | 15 | 19 | 15 | 19 |
| (1) Approved VC = Minimum {Highest O&M without PPA or O&M with PPA} |  |  |  |  |  |

Table 4 contains an example for the calculation of the Cap for Minimum Energy Cost @ LSL when the QSE files verifiable costs which include both fuel and O&M costs.  In this example, the highest Total O&M Cost for the non-PPA Resources is that for Resource 2 at $20/MWh.  There are two PPAs (Resources 5 & 6) which have total filed costs greater than this calculated Cap.  The verifiable O&M costs components for these capped PPAs will be set equal to those components in Reference Resource 2.

Table 5:  Costs for Operation Above LSL, PPA Cap for Simple Cycle > 90MW Resources (This cost is optional)

| Resource | Average(1) O&M ($/MWh) | Approved VC O&M ($/MWh) |
| --- | --- | --- |
| 1 | 17 |  |
| 2 | 20 |  |
| 3 | 14 |  |
| 4 | 15 |  |
| 5 | 25 | 20 |
| 6 | 21 | 20 |
| 7 | 19 | 19 |

(1) Simple average for all of the (MMBtu/MWh versus MW) pairs along the IHR curve

Example 3:  PPAs indicating only one (1) cost for the Resource; no fuel volume shown and no Reference Unit available

Table 1:  Test for Reference Unit

| Simple Cycle > 90 MW Resources |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Unit 1 | Unit 2 | Unit 3 | Unit 4 | Unit 5 (with PPA) |
| HSL | 200 | 210 | 180 | 160 | 250 |
| HSL Difference(1) | 50% | 40% | 70% | 90% |  |
| Initial Commercial Operation Date | 1996 | 2005 | 2006 | 2000 | 1990 |
| Differences in years between PPA and non-PPA Resources | 6 | 15 | 16 | 10 |  |
| Meets Criteria(2) | No | No | No | No |  |
| (1) Percent difference with respect to unit with PPA |  |  |  |  |  |
| (2) Criteria: HSL difference +- 30% and year less than or equal to 5 years |  |  |  |  |  |

Table 2:  Cold, Hot, and Intermediate Start-Up Cost for Simple Cycle > 90 MW Resources

| Cold Type | Unit 5 |  | Hot Type | Unit 5 |  | Inter Type | Unit 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PPA Cost ($) | 6,000 |  | PPA Cost ($) | 4,500 |  | PPA Cost ($) | 3,000 |
| Approved VC Fuel (MMBtu) | N/A |  | Approved VC Fuel (MMBtu) | N/A |  | Approved VC Fuel (MMBtu) | N/A |
| Generic O&M ($) | 5,000 |  | Generic O&M ($) | 5,000 |  | Generic O&M ($) | 5,000 |
| Approved VC O&M Cap ($) | 5,000 |  | Approved VC O&M Cap ($) | 4,500 |  | Approved VC O&M Cap ($) | 3,000 |

Since there is no Reference Resource, the approved verifiable costs will be set at the minimum of the PPA costs or the Generic O&M costs set forth in the Protocols.

Table 3:  Minimum Energy {@ LSL} PPA Cap for Simple Cycle > 90MW Resources

| Resource | PPA Cost ($/MWh) | Generic Fuel at LSL (MMBtu/MWhr) | Approved VC Fuel (MMBtu/MWh) | Approved VC O&M Cap ($) |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 | 300 | 15 | 15 | 0 |
| 6 | 140 | 15 | 15 | 0 |
| 7 | 200 | 15 | 15 | 0 |

In this example the filed PPA costs for Units 6 and 7 were equal to or exceeded the Generic fuel costs and were therefore capped and approved at the Generic level.  Since Generic costs at LSL do not include an O&M component, PPA Resources cannot received any O&M costs and their fuel rate is capped at Generic values.

Example 4:  PPAs indicating both Fuel and O&M costs but without a Reference Unit

Table 1:  Cold, Hot, and Intermediate Start-Up Cost for Simple Cycle > 90 MW Resources

| Cold Type | Unit 5 |  | Hot Type | Unit 5 |  | Inter Type | Unit 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel to LSL (MMBtu) | 120 |  | Fuel to LSL (MMBtu) | 55 |  | Fuel to LSL (MMBtu) | 100 |
| Total O&M Cost ($) | 7,000 |  | Total O&M Cost ($) | 5,000 |  | Total O&M Cost ($) | 6,500 |
| Generic O&M ($) | 5,000 |  | Generic O&M ($) | 5,000 |  | Generic O&M ($) | 5,000 |
| Approved VC Fuel (MMBtu) | 120 |  | Approved VC Fuel (MMBtu) | 55 |  | Approved VC Fuel (MMBtu) | 100 |
| Approved VC O&M Cap ($) | 5,000 |  | Approved VC O&M Cap ($) | 5,000 |  | Approved VC O&M Cap ($) | 5,000 |

Table 2:  Minimum Energy {@ LSL} PPA Cap for Simple Cycle > 90MW Resources

| Resource | O&M ($/MWh) | Fuel at LSL (MMBtu/MWhr) | Approved VC Fuel (MMBtu/MWh) | Approved VC O&M Cap ($) |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 | 15 | 25 | 25 | 0 |
| 6 | 14 | 30 | 30 | 0 |
| 7 | 18 | 15 | 15 | 0 |

In Table 2 above, Resources 5, 6 and 7 provided O&M costs and fuel rate at LSL, however, ERCOT did not approve the O&M costs since there were no Reference Units and no additional documentation was provided

# Appendix 4:  Operating and Maintenance Cost Categories

### Maintenance Cost Categories

Maintenance Costs that fall within the categories listed in NERC’s Data Reporting Instructions or as shown in Tables 4-6 below may be included as a Resource’s Verifiable Maintenance Costs.  Submitted Maintenance Costs may include a reference to an appropriate GADS Code.  Additionally, submitted maintenance costs will only be approved to the extent that they comply with all other relevant provisions within this Manual.  This means, for example, fixed maintenance costs, maintenance costs which cannot be verified, and Event Specific Verifiable Maintenance Costs will not be approved, even if they fall within a GADS category.

NERC’s Data Reporting Instructions with an effective date of January 2007 may be found at the following link:

ftp://www.nerc.com/pub/sys/all_updl/gads/dri/2007_GADS_DRI.pdf

Currently, the Data Reporting Instructions may be found by following these steps:

Step 1:  Connect to www.nerc.com

Step 2;  From the drop down menu, choose GADS Services

Step 3:  From the title menu, select Data Reporting Instructions

Step 4:  Choose Appendix B1-B9

### Operating Cost Categories

Other permissible categories of Operating Costs are listed tables 4-6 below.  Additionally, submitted Operating Costs will only be approved to the extent that they comply with all other relevant provisions within this Manual.

### Table 4:  O&M Categories applicable to all generating plants

- Water consumption in plant operations.

- Emission credits.

- Incremental operating labor.

- Auxiliary equipment repair.

- Replacement of consumables and normal wear-and-tear items (e.g., seals, lockplates, nuts, bolts, gaskets, etc.).

- Mechanical parts replacement.

- On-line running maintenance.

- Performance testing.

- Vibration analysis monitoring.

- Waste water treatment.

- Auxiliary equipment maintenance.

- Filter changes.

- Oil changes.

- Oil and water level checks.

- Condensate Pump Inspection and Overhaul.

- Circulation Pump Inspection and Overhaul.

- Replacement of Cooling Tower Fill and Drift Eliminators.

- Steam Turbine Generator inspection.

- Preventive maintenance tests.

- Auxiliary power consumption.

- Auxiliary fuels/lubricants.

- Compressor and turbine rotors inspection.

- Minor Generators repairs.

- Fuel metering equipment replacement.

- Gas turbine auxiliaries.

- Controls and accessories replacement.

- Pre-air heater test.

- Transformer maintenance.

- Relay cleaning.

- Battery system service.

- Oxygen boiler test.

- Condenser inspections.

- Condenser cleaning.

- Water demineralization and treatment.

- On-line performance testing.

- Condenser tube cleaning.

- Boiler tube repair resulting from cycling operations.

- Generator field rewinds.

- Stop valve inspection.

- Control valve inspection.

- Boiler casing leak repair (where applicable).

### Table 5:  O&M Categories applicable to Coal-Fired Generating Plants

- Boiler safety valve testing

- Steam drum repairs

- Water wall repairs

- Seals replacement

- Coal hopper repair

- Coal crusher repair

- Coal freeze proofing

- Chemical cleaning of internal heating surfaces

- Repair or stud surfaces

- Evaporator tubing repairs

- Superheater and reheater tubing and headers repairs

- Heat exchanger cleaning

- Conveyor belt repair

- Coal drying

- Coal-handling and distribution equipment repair

- Stack, fans and draft repair

- Seal welding of tubes

- Coal pulverizing equipment repair

### Table 6:  O&M Categories applicable to Combustion Turbine/Combined-Cycle Generating Plants

- Alignment check of the gas turbine to the generator, as well as of the gas turbine to the accessory gear.

- BFW Pump Inspection and Overhaul.

- Borescope inspections of compressor casings and turbine shells.

- Casings, shells, and frames/diffusers inspected for cracks and erosion.

- Checks of alignment between gas turbine and generator; gas turbine and accessory gear.

- Radial and axial clearances check.

- Seals for rubs and deterioration of clearance checks.

- Device calibrations check.

- Chemical Cleaning or Hydro-Blasting of Heat Transfer Surfaces.

- Fluorescent penetrant inspection of bucket vane sections.

- Combustion Turbine Generator Evaporative cooling system media replacement.

- Combustion Turbine Generator Inspections.

- Compressor inlet and flow-path inspection for fouling, erosion, corrosion, and leakage.

- Cooling Tower Fan Motor and Gearbox Inspection and Overhaul.

- Compressor wash systems repair.

- Distillate Fuel Pumps Inspection and Overhaul.

- Electric generator inspection and overhaul.

- Environmental:  SCR replacement.

- Evaporative cooling system media.

- Inlet Air Filter Replacement.

- Inspection of bearing liners and seals for clearance and wear.

- Buckets inspection.

- Fuel Gas Compressors Inspection and Overhaul.

- Fuel System replacement.

- Heat Transfer Surface Replacements.

- Inspection of compressor blades for rubs.

- Hydrogen embrittlement testing.

- Inspection of flow sleeve welds for cracking.

- Inspection of fuel nozzles for plugging and erosion of tip holes.

- Detectors, combustor flow sleeves, flow sleeve welds, combustion system and discharge casing.

- Maintenance of fuel treatment system.

- Mechanical inlet air cooling chiller and pump inspection and overhaul.

- Sampling of turbine lube oil for viscosity, chemical composition, contamination, particulate, and water-contamination.

- Water Treatment:  Resin Replacement.

- Impact damage, corrosion, and buildup of deposits.

- Inspection of cross-fire tube, retainer, and combustion liner for cracking, oxidation, corrosion, and erosion.

- Inspection of fluid, air, and gas passages in the nozzle assembly for plugging, erosion, corrosion, etc.

- Inspection of fuel nozzles, liners, transition pieces, crossfire tubes and retainers, spark plug assemblies, flame.

- Inspection of inlet systems for corrosion, cracked silencers, and loose parts.

- Maintenance of inlet filtration systems and inlet evaporative coolers for compressors.

- Refurbishing bucket coatings.

- Turning and ratchet gear maintenance.

- Water Treatment:  RO Cartridges Replacement.

- Inspection of combustion chamber interior.

- Inspection of fuel nozzles.

- Inspection of spark plug assembly.

- Inspection of later-stage diaphragm packing.

- Inspection of bucket seals for clearances, rubs, and deterioration.

- Inspection of turbine stationary shrouds.

- Inspection of wheelspace instrumentation.

- Inspection of variable inlet guide vanes (VIGVs).

- Repair and refurbishment of second and third-stage nozzles.

- Recoating of turbine buckets.

- Replacement or refurbishment of hot gas parts.

Appendix 5:  Specification of Relevant Equations

Equation 1: Verifiable Start-up Offer Cap (Hot Start) ($/Start)

Verifiable Startup Offer Cap ($/Start) = Fuel_Hot_Start (MMBtu/Start) * [(Startup_FIP_PRC*FIP + Startup_FOP_PRC*FOP)/100] +  OM_Cost_Hot_Startup

Where:	Fuel_Hot_Start = the verified fuel consumption for a hot start (MMBtu/Startup)

Startup_FIP_PRC = Percentage of natural gas used for a start

FIP = Fuel Index Price from Platts ($/MMBtu)

Startup_FOP_PRC = Percentage of oil used for a start

FOP = Fuel Oil Price from Platts ($/MMBtu)

OM_Cost_Hot_Startup = the verified O&M cost for a hot start ($/Startup)

Similar equations apply for Verifiable Cold and Intermediate Startup Offer Caps

Equation 2: Verifiable Minimum-Energy Offer Cap ($/MWh)

Verifiable Minimum-Energy Offer Cap ($/MWh) = Fuel_LSL*[(Mingen_FIP_PRC*FIP + Mingen_FOP_PRC*FOP)/100] + OM_Cost_LSL

Where:	Fuel_LSL = Fuel consumed at LSL (MMBtu/MWh) = Fuel Rate at LSL (MMBtu/Hour) divided by LSL (MW)

Mingen_FIP_PRC = Percentage of natural gas used at LSL

FIP = Fuel Index Price from Platts ($/MMBtu)

Mingen_FOP_PRC = Percentage of oil used at LSL

FOP = Fuel Oil Price from Platts ($/MMBtu)

OM_Cost _LSL = the verified O&M cost at Minimum-Energy ($/MWh)

Equation 3: Calculation of Composite Unit Parameters using Alternate Unit Specifications

Composite Unit Parameter = [Alt_Unit_Par*Alt_Unit_HSL +  Non_Alt_Unit_Par* Non_Alt_Unit_HSL] / [Alt_Unit_HSL + Non_Alt_Unit_HSL]

Where:		Alt_Unit_Par = Relevant parameter of Alternate Unit

Alt_Unit_HSL = High Sustained Limit of Alternate Unit

Non_Alt_Unit_Par = Relevant parameter of non-Alternate Unit

Non_Alt_Unit_HSL = High Sustained Limit of non-Alternate Unit

This calculation would be executed for all relevant parameters of the alternate and non-alternate units.  This would include for example Startup Cost data, Minimum-Energy Cost data and heat rate data. _

Equation 4: Equation for Calculation of Verifiable Startup Emission Costs

Verifiable Startup Emission Cost ($/Start) = AFCRS * ∑Emission Rate i * Emission Cost Index i

Where            AFCRS = Quantity of startup fuel consumed by Resource (including         fuel  used to shutdown Resource (MMBtu/Start)

- Emission Rate i =  Quantity of emission i emitted by resource (lbs/MMBtu)

- Emission Cost Index i = Published cost index of emission i ($/lb)

- i = Index for each emittent approved for inclusion in Startup Cost

Equation 5:  Equation for Calculation of Verifiable Minimum-Energy Emission Costs

Verifiable Minimum-Energy Emission Costs ($/MWh) =

[VFCLSL/LSL] * ∑Emission Rate i * Emission Cost Index i

Where		VFCLSL = Verified fuel consumption at Minimum Energy (MMBtu/Hr)

LSL = Low Sustained Limit of Resource (MW)

Emission Rate i = Quantity of emission i emitted by resource (lbs/MMBtu)

Emission Cost Index i = Published cost index of emission i

i = Index of each emittent approved for inclusion in Minimum-Energy Cost

Equation 6: Equation for Calculation of Verifiable Startup Costs (VERISU)  ($/Start)

VERISU = SFC + VOMS

Where		VERISU = Verifiable Startup Costs

SFC = Verifiable Startup Fuel Costs ($/Start)

VOMS = Verifiable Operations and Maintenance Costs ($/Startup)

SFC = [FuelStartup-BC + FuelBC-LSL + FuelBO-Shutdown] * [FIP*GASPERSU(%) + FOP*OILPERSU(%) + SFP*SFPERSU(%)]

Where	FuelStartup-BC = Fuel Quantity required to bring Resource from Startup to Breaker Close (MMBtu)

FuelBC-LSL = Fuel Quantity required to bring Resource from Breaker Close to Minimum Energy at LSL (MMBtu)

FuelBO-Shutdown = Fuel Quantity required to take Resource from Breaker Open to Shutdown (MMBtu)

FIP = Fuel Price Index for gas ($/MMBtu)

FOP = Fuel Price Index for oil ($/MMBtu)

SFP = Fuel Price Index for solid fuel = $1.50/MMBtu

GASPERSU = Percent of gas used during startup

OILPERSU = Percent of oil used during startup

SFPERSU = Percent of solid fuel used during startup

VOMS = IO&MStart-LSL +IO&MBO-Shutdown + Verifiable Startup Emission Costs

Where	IO&MStart-LSL = Incremental O&M costs incurred to bring Resource from Start to LSL ($/Start)

IO&MBO-Shutdown = Incremental O&M costs incurred to take Resource from Breaker Open to Shutdown ($/Start)

Verifiable Startup Emission Costs = The allowable costs of acquiring emission credits required to start up Resource and defined in Equation 4 above .

Equation 7: The Equation for calculating Verifiable Minimum Energy Costs ($/MWh)

VERIME = FCLSL + VOMLSL

Where 		VERIME = Verifiable Minimum Energy Costs

FCLSL = Verifiable Fuel Costs at Minimum Energy

VOMLSL = Verifiable variable O&M costs at Minimum Energy

FCLSL = [(VFCLSL/LSL)] * [FIP*GASPERME(%) + FOP*OILPERME(%) + SFP*SFPERME(%)]

Where		VFCLSL = Verified fuel consumption at Minimum Energy(MMBtu/Hr)

LSL = Low Sustained Limit of Resource (MW)

FIP = Fuel Price Index for gas ($/MMBtu)

FOP = Fuel Price Index for oil ($/MMBtu)

SFP = Fuel Price Index for solid fuel = $1.50/MMBtu

GASPERME = Percent of gas used at minimum energy

OILPERME = Percent of oil used at minimum energy

SFPERME = Percent of solid fuel used at minimum energy

VOMLSL = IO&MLSL + Verifiable Emission Costs at Minimum Energy + Nodal Implementation Surcharge

Where		IO&MLSL = Incremental O&M costs at minimum energy

Verifiable Emission Costs at Minimum Energy = The allowable costs of acquiring emission credits required to operate Resource at minimum energy and defined in Equation 5 above.

Nodal Implementation Surcharge = Charge approved by the Texas PUC and imposed by ERCOT to cover the costs of implementing the Texas Nodal Market.

# Table of References

| GE Energy, Heavy-Duty Gas Turbine Operating and Maintenance Considerations, 2004, available at http://www.gepower.com/prod_serv/products/tech_docs/en/downloads/ger3620k.pdf . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | p. 67 |
| --- | --- |
| PJM, PJM Manual 15: Cost Development Guidelines Revision 07, 08/03/2006, available at http://www.pjm.com/contributions/pjm-manuals/pdf/m15.pdf. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | p. 42, 50-53, 66 |

At this time, it is unknown how many levels of appeal will be available or all parties that will be involved.  Specific details in this section that are unknown are represented by a generic description enclosed in brackets.  ERCOT is awaiting responses from key parties and is unable to fully detail the appeal process until these are received.  ERCOT nevertheless included this incomplete and general framework in the Manual in order to inform Market Participants and to encourage Market Participants to submit feedback and suggestions.

The Methodology detailed within this Appendix is based upon the one currently used by PJM.  See PJM, PJM Manual 15:  Cost Development Guidelines, p.  24-25, available at http://www.pjm. com/contributions/pjm-manuals/pdf/m15.pdf.

Both the methodology and the example detailed within this Appendix are based upon ones used by PJM.  See PJM, PJM Manual 15:  Cost Development Guidelines, p. 25-29, available at http://www.pjm.com/contributions/pjm-manuals/pdf/m15.pdf.