NMMS:

ERCOT Modeling Guidelines

Version 0.03

| Document Revisions |  |  |  |
| --- | --- | --- | --- |
| Date | Version | Description | Author(s) |
| 1/13/09 | 0.01 | Initial draft | Terry Nielsen |
| 1/29/09 | 0.02 | Updated rules, edited to support for draft release to NDWSG. | Terry Nielsen |
| 2/17/09 | 0.03 | Updated with section changes based upon NDSWG input and NMMS team input. Also updated to add draft completed section for switch. | Terry Nielsen, Keith Smith |

Table of Contents

1	OVERVIEW	4

1.1	Document Organization	4

1.1.1	Major Sections	4

1.1.2	Modeling Approach Subsection	4

1.1.3	Classes Used, Not Used and Not Allowed Subsection	4

1.1.4	Appendix A - Validation Rules for Required Fields	4

1.1.5	Appendix B - Validation Rules for Required Associations	5

1.1.5.1	Automatic Validation Performed by NMMS Based Upon Data Dictionary	5

1.1.5.2	Further Clarification on Interpreting Data Dictionary and How Data is Used in NMMS IMM	5

1.1.6	Appendix C – Complex Validation Rules	7

1.1.7	Relevant reference documents:	7

2	Modeling Switches	9

2.1	Modeling Approach	9

2.1.1	Modeling Approach for Disconnector	9

2.1.1.1	Required Attributes Disconnector	10

2.1.1.2	Default Attributes Disconnector	10

2.1.1.3	Required Associations Disconnector	10

2.1.2	Modeling Approach for Breaker	10

2.1.2.1	Required Attributes Breaker	11

2.1.2.2	Default Attributes Breaker	11

2.1.2.3	Required Associations Breaker	11

2.2	Classes Used, Classes Not Used, and Not Allowed	12

Appendix A - Validation Rules – Required Fields	13

Appendix B - Validation Rules – Required associations	27

Appendix C - Validation Rules – COMPLEX rules	30

# OVERVIEW

This document provides guidelines for the creation of network models that are to be submitted to ERCOT. The models may be either created using the Information Model Management (IMM) editor component of the Network Model Management System (NMMS) or by submitting a file that is in Common Information Model (CIM) XML format.

## Document Organization

Since the ERCOT network model is based upon extensions to the CIM standard, this document focuses primarily on clarifying how to use the new data attributes, associations and classes defined in the ERCOT extensions.

### Major Sections

The document is broken down into major sections (Section 2, 3, etc.) that map to a single CIM package or a subsection of a package. This breaks down the document into logical subsections that cover areas such as generation, loads, etc.

### Modeling Approach Subsection

Within each major section, the document focuses on aspects of the model that are not part of the CPSM profile of the CIM modeling standard. These are documented, either class by class or as logical group in subsections titled “Modeling approach for <X>”. The modeling approach section will describe the approach a user should take for modeling this object type. It covers what associations and attributes are needed and how they should be defined. For more complex modeling concepts, it may include flowcharts or other diagrams necessary to explain the modeling approach.

The document will further elaborate on how standard CIM classes, attributes and associations are used if they are used in a non-standard way. These will also be documented in a modeling approach section.

### Classes Used, Not Used and Not Allowed Subsection

If a particular CIM class is not used in the ERCOT systems, but can be included in a model submitted to ERCOT with no adverse effects it will be listed in the subsection titled “classes not used”. The complete list of classes that are not used are also listed in Appendix D.

If a particular CIM class is not used in the ERCOT systems and would potentially break the ERCOT model, the class will be listed in the subsection titled “classes not allowed”. The complete list of classes that are not used are also listed in Appendix E.

The appendix of this document contains the complete listing of validation rules that run against each NMMS NOMCR submission. If any of these rules do not pass, the NOMCR will not be allowed to be submitted and the user must make the necessary corrections. These rules document any minimally required attributes, valid data ranges. The validation also ensures that the submission meets the cardinality defined in the latest ERCOT data dictionary.

### Appendix A - Validation Rules for Required Fields

Appendix A lists all of the validation rules for attributes that must be defined for a specific class. It also lists any validation for attributes that must be within a specific value range.

### Appendix B - Validation Rules for Required Associations

Appendix B lists all of the validation rules for associations (links) that must be defined for a specific class.

#### Automatic Validation Performed by NMMS Based Upon Data Dictionary

The list of validation rules does not include the validation that is automatically performed by NMMS for links between objects. These links are validated based upon the data dictionary definition of the cardinality between the associated objects. There are two cardinality definitions that are validated, the first is the CardinalityRoleA and the second CardinalityRoleB. CardinalityRoleA defines the number of objects of the first Class that may exist related to the second Class.

For instance, in the data dictionary, the link between ThermalGeneratingUnit to CombinedCyclePlant CardinalityRoleA has a value of “0..n”. This means that there may be anything between 0 and n ThermalGeneratingUnits associated with a CombinedCyclePlant. The same association has a CardinalityRoleB of “0..1” which means that there may be between 0 and 1 CombinedCyclePlants associated with a ThermalGeneratingUnit.

This validation performed by NMMS follows the inheritance tree and the same validation is done for all objects that inherit from the object which has the cardinality definition.

For instance, in the data dictionary, a GeneratingUnit has a link to a ResourceNode that has a CardinalityRoleA defined as 1..n. This means that validation would be performed checking that there are one or more GeneratingUnits associated with a ResourceNode. It also means that validation would be performed for HydroGeneratingUnit, ThermalGeneratingUnit, WindGeneratingUnit and NuclearGeneratingUnit because these classes inherit from GeneratingUnit. Note: that the cardinality is not defined explicitly for each inherited class.

#### Further Clarification on Interpreting Data Dictionary and How Data is Used in NMMS IMM

The rules used when dealing with associations/links may help with understanding these relationships. In general, the direction is defined in the data dictionary by the A and B (e.g. Class A and Class B, A --> B). The key, however, is that while the IMM UI uses the direction of the link to determine which side to display the links, the CIM/RDF export standard uses the cardinality to determine which side to export links.

Sample association definition:

| Class A | Cardinality A | Class B | Cardinality B | NMMS Label |
| --- | --- | --- | --- | --- |
| GeneratingUnit | 0..1 | DCTie | 1..1 | PartOfDCTie |

The NMMS IMM UI displays links in the direction A --> B.

By default the IMM UI instance editor will show links to instances of Class B on the Class A side. This means you will see links in the direction of A --> B. In the definition above, the IMM UI will show the link to DCTie on the "link tab" of the GeneratingUnit. If the user wishes to see the links in the opposite direction, they can do so by pressing the [Options] button and choosing the appropriate setting.

The NMMS RDF Export lists the links to the Class having lower cardinality under the Class having higher cardinality.

The CIM/RDF standard recommends that the side with the lower cardinality be exported under the side having higher cardinality. In case the cardinalities are equal, NMMS is implemented so that it export the links to Class B under the Class A instance.

This explains why the user sees an inconsistency between IMM UI and RDF export with the GeneratingUnit to DCTie relationship. IMM UI shows the links to DCTie (Class B) under the GeneratingUnit (Class A). However, RDF Export lists the links pointing to GeneratingUnit (cardinality 0-1) under the DCTie (cardinality 1-1).

The cardinality order is shown in the following table:

Cardinality Order (Least to Greatest)

0..1

1..1

0..n

1..n

Below is another example:

| Class A | Cardinality A | Class B | Cardinality B | NMMS Label |
| --- | --- | --- | --- | --- |
| ConductingEquipment | 0..n | BaseVoltage | 0..1 | BaseVoltage |
| SynchronousMachine | 0..1 | GeneratingUnit | 0..1 | GeneratingUnit |
| Reservoir | 0..1 | HydroPowerPlant | 0..n | Provides Water To(HydroPowerPlants) |

The links to BaseVoltage (lower cardinality) will be exported under ConductingEquipment (higher cardinality). Likewise, IMM UI will list the links to BaseVoltage (Class B) under the ConductingEquipment (Class A).

The links to GeneratingUnit (cardinality 0..1) will be exported under the SynchronousMachine (cardinality 0..1). Likewise, IMM UI will list the links to GeneratingUnit (Class B) under the SynchronousMachine (Class A). The links to Reservoir (lower cardinality) will be exported under HydroPowerPlant (higher cardinality). However, IMM UI will list the links to HydroPowerPlant (Class B) under the Reservoir (Class A).

### Appendix C – Complex Validation Rules

Appendix B lists all of the validation rules that are more complex than required fields and range checks. These include validation rules that are conditional based upon certain conditions, involve associations with other attributes, and those that describe specific data relationships. The validation rule is explained descriptively and not in a formal language.

### Relevant reference documents:

TBD

# Modeling Switches

## Modeling Approach

In the ERCOT CIM model Switch is defined as a generic device designed to close, or open, or both, one or more electric circuits.   Switch is an abstract type, meaning that it represents a class of several similar devices and not a corresponding real device.

The following real devices inherit from the Switch class and are used in the ERCOT CIM model:  Disconnector and Breaker.  These devices should be modeled under a voltage level in a substation.  Every disconnector/breaker should be modeled with two terminals as connection points.  Instance Diagram 1 below shows where a switch fits into the model hierarchy.  The discrete values provide the open or close status which may be provided through SCADA.  Please refer to the Section 18 on SCADA values for more information.

Diagram 1

### Modeling Approach for Disconnector

The ERCOT CIM model defines a disconnector as a manually operated or motor operated mechanical switching device used for changing the connections in a circuit, or for isolating a circuit or equipment from a source of power. It is required to open or close circuits when negligible current is broken or made.

#### Required Attributes Disconnector

The required attributes for a disconnector are shown in the table below.

| Attribute | Description | Data Type | Default |
| --- | --- | --- | --- |
| TEID | Transmission Element ID | Integer | Auto Populated |
| normalOpen | The attribute is used in cases when no Measurement for the status value is present. | Boolean | False |
| initiallyOpen | T = The initial breaker state for the online switching device model is open. | Boolean | False |

#### Default Attributes Disconnector

There are many Boolean attribute fields that are not required, but will be populated with default values.  The default values will be used in the model unless changed by the user.  The attributes and their default values can be found in the ERCOT Data Dictionary.

#### Required Associations Disconnector

The required associations for a disconnector are shown in the table below.

| AssociationName | Class | Description |
| --- | --- | --- |
| ConductingEquipment.BaseVoltage | BaseVoltage | Use association to ConductingEquipment only when there is no VoltageLevel container used. |
| PowerSystemResource.Operatorship | Operatorship | The operator of equipment has the right to edit the equipment. An operator can only be assigned by the creator or owner of the equipment. |
| PowerSystemResource.Ownership | Ownership | The owner of equipment has editing and administrative rights. An owner can only be assigned by the creator of the equipment. |

### Modeling Approach for Breaker

The ERCOT CIM model defines a breaker as a mechanical switching device capable of making, carrying, and breaking currents under normal circuit conditions and also making, carrying for a specified time, and breaking currents under specified abnormal circuit conditions e.g. those of short circuit.  Breaker inherits from the ProtectedSwitch class.

#### Required Attributes Breaker

The required attributes for a breaker are shown in the table below.

| Attribute | Description | Data Type | Default |
| --- | --- | --- | --- |
| TEID | Transmission Element ID | Integer | Auto Populated |
| normalOpen | The attribute is used in cases when no Measurement for the status value is present. | Boolean | False |
| initiallyOpen | T = The initial breaker state for the online switching device model is open. | Boolean | False |
| InTransitTime | The transition time from open to close, in seconds. | Float | 0 |

#### Default Attributes Breaker

There are many boolean attribute fields that are not required, but will be populated with default values.  The default values will be used in the model unless changed by the user.  The attributes and their default values can be found in the ERCOT Data Dictionary.

#### Required Associations Breaker

The required associations for a breaker are shown in the table below.

| AssociationName | Class | Description |
| --- | --- | --- |
| ConductingEquipment.BaseVoltage | BaseVoltage | Use association to ConductingEquipment only when there is no VoltageLevel container used. |
| PowerSystemResource.Operatorship | Operatorship | The operator of equipment has the right to edit the equipment. An operator can only be assigned by the creator or owner of the equipment. |
| PowerSystemResource.Ownership | Ownership | The owner of equipment has editing and administrative rights. An owner can only be assigned by the creator of the equipment. |

## Classes Used, Classes Not Used, and Not Allowed

The table below shows which classes are supported and not supported in the ERCOT CIM model.

| Category | Description | Classes |
| --- | --- | --- |
| Classes Used | These classes are real devices that are supported in the ERCOT CIM Model. | Disconnector, Breaker |
| Classes Not Used | These classes may be used in a model submission but will be ignored. They are abstract types that do not correspond to real devices | Switch, ProtectedSwitch |
| Classes Not Allowed | These classes may NOT be used in a model submission. They are currently not supported in the ERCOT CIM model. | Fuse, Ground Disconnector, Jumper, Load Break Switch |

# Appendix A - Validation Rules – Required Fields

This appendix lists all of the validation rules for attributes that must be defined for a specific class. It also lists any validation for attributes that must be within a specific value range.

| ClassName | Attributes | Required | Range |
| --- | --- | --- | --- |
| Accumulator | maxValue |  | >= 0 |
| AccumulatorLimit | value | x | >=0 |
| AccumulatorValue | cstatus | x |  |
| ACLineSegment | b0ch |  | >= 0 and <= 5 |
| ACLineSegment | bch |  | >= 0 and <= 50 |
| ACLineSegment | length |  | can be zero if x=0 and r=0 |
| ACLineSegment | r | x | >= -500 and <= 500 |
| ACLineSegment | r0 |  | >= -500 and <= 500 |
| ACLineSegment | x | x | >= -1000 and <= 1000 |
| ACLineSegment | x0 |  | >= -1000 and <= 1000 |
| ACLineSegment | teid | x | >=0 and <= 999999999 |
| Address | City | x |  |
| Address | PostalCode | x |  |
| Address | State | x |  |
| Address | Street1 | x |  |
| AggregateHub | hubCalculationMethod | x |  |
| AirCompressor | teid | x | >=0 and <= 999999999 |
| AlternateLimit | highLimit | x |  |
| AlternateLimit | lowLimit | x |  |
| Analog | teid | x | >=0 and <= 999999999 |
| AnalogValue | abnormalAlarmDelayTime |  | >= 0 |
| AnalogValue | historicalRecordingDeadband |  | >= 0 and <= 100 |
| AnalogValue | returnToNormalAlarmDelayTime |  | >= 0 |
| AnalogValue | timeDelay |  | >= 0 |
| AnalogValue | cstatus | x |  |
| AreaReserveSpec | lowerRegMarginReqt | x | >= 0 |
| AreaReserveSpec | opReserveReqt | x | >= 0 |
| AreaReserveSpec | primaryReserveReqt | x | >= 0 |
| AreaReserveSpec | raiseRegMarginReqt | x | >= 0 |
| AreaReserveSpec | spinningReserveReqt | x | >= 0 |
| BasePower | basePower | x |  |
| BaseVoltage | nominalVoltage | x | >=0 and <= 1100 |
| BasicIntervalSchedule | startTime | x |  |
| BasicIntervalSchedule | value1Unit | x |  |
| BasicIntervalSchedule | value2Unit | x |  |
| Bay | teid | x | >=0 and <= 999999999 |
| BranchGroup | maximumActivePower | x |  |
| BranchGroup | maximumReactivePower | x |  |
| BranchGroup | minimumReactivePower | x |  |
| BranchGroup | monitorActivePower | x |  |
| BranchGroup | teid | x | >=0 and <= 999999999 |
| BranchGroupTerminal | positiveFlowIn | x |  |
| Breaker | InTransitTime | x |  |
| Breaker | teid | x | >=0 and <= 999999999 |
| BusbarSection | teid | x | >=0 and <= 999999999 |
| BWRSteamSupply | teid | x | >=0 and <= 999999999 |
| CAESPlant | teid | x | >=0 and <= 999999999 |
| CogenerationPlant | teid | x | >=0 and <= 999999999 |
| CombinedCyclePlant | LogicalResourceNodeName | x |  |
| CombinedCyclePlant | teid | x | >=0 and <= 999999999 |
| CombinedCyclePlant | aliasName | x |  |
| CombustionTurbine | teid | x | >=0 and <= 999999999 |
| CommunicationLink | teid | x | >=0 and <= 999999999 |
| Company | companyType | x |  |
| Company | DUNS | x |  |
| Company | PrimaryPhone | x |  |
| Company | teid | x | >=0 and <= 999999999 |
| Company | aliasName | x |  |
| CompositeSwitch | teid | x | >=0 and <= 999999999 |
| ConductingEquipment | teid | x | >=0 and <= 999999999 |
| Conductor | teid | x | >=0 and <= 999999999 |
| Configuration | highReasonabilityLimit | x | >= lowReasonabilityLimit |
| Configuration | highReasonabilityRampRateLimit | x | >= lowReasonabilityRampRateLimit |
| Configuration | hotintTime | x |  |
| Configuration | hotStartTime | x | <= intStartTime |
| Configuration | intColdTime | x |  |
| Configuration | intStartTime | x | <= coldStartTime |
| Configuration | lowReasonabilityLimit | x | >= 0 |
| Configuration | lowReasonabilityRampRateLimit | x |  |
| Configuration | maxDailyStarts | x |  |
| Configuration | maxONTime | x |  |
| Configuration | maxWeeklyEnergy | x |  |
| Configuration | maxWeeklyStarts | x | >= maxDailyStarts |
| Configuration | minOFFTime | x |  |
| Configuration | minONTime | x | <= maxONTime |
| Configuration | primaryConfiguration | x |  |
| Configuration | qualificationStatus | x |  |
| Configuration | seasonalMaxEmergencyRating | x |  |
| Configuration | seasonalMaxSustainableRating | x | >= seasonalMinSustainableRating |
| Configuration | seasonalMinEmergencyRating | x |  |
| Configuration | seasonalMinSustainableRating | x |  |
| ConformLoad | conformingLoadFlag | x |  |
| ConformLoad | pfixedPct | x |  |
| ConformLoad | pnomPct | x |  |
| ConformLoad | powerFactor | x |  |
| ConformLoad | qfixed | x |  |
| ConformLoad | qnomPct | x |  |
| ConformLoad | teid | x | >=0 and <= 999999999 |
| ConformLoadSchedule | startTime | x |  |
| ConformLoadSchedule | value1Unit | x |  |
| ConformLoadSchedule | value2Unit | x |  |
| ConformLoadSchedule | aliasName | x |  |
| ConnectivityNode | PSSEBusNumber |  | >= 1 and <= 99997 |
| ConnectivityNode | teid | x | >=0 and <= 999999999 |
| ConnectivityNodeGroup | PSSEBusNumber | x | can not be between 9400-9999 and 94000-99999 |
| ConnectivityNodeGroup | teid | x | >=0 and <= 999999999 |
| Connector | teid | x | >=0 and <= 999999999 |
| Contact | EmailAddress | x |  |
| Contact | FirstName | x |  |
| Contact | LastName | x |  |
| Contact | PrimaryPhone | x |  |
| Contact | City | x |  |
| Contact | PostalCode | x |  |
| Contact | State | x |  |
| Contact | Street1 | x |  |
| Contingency | DeviceName | x |  |
| Contingency | DeviceType | x |  |
| Contingency | probabilityOfOccurance |  | >= 0 and <= 1 |
| ControlHouseEquipment | teid | x | >=0 and <= 999999999 |
| ControllableLoadResource | maxDeploymentTime | x |  |
| ControllableLoadResource | maxWeeklyEnergy | x |  |
| ControllableLoadResource | minNoticeTime | x |  |
| ControllableLoadResource | rampRateSlopeDownLimit | x |  |
| ControllableLoadResource | rampRateSlopeUpLimit | x |  |
| ControllableLoadResource | maxDailyDeployment | x |  |
| ControllableLoadResource | maxInterruptionTime | x |  |
| ControllableLoadResource | maxWeeklyDeployment | x |  |
| ControllableLoadResource | minInterruptionTime | x |  |
| ControllableLoadResource | minRestorationTime | x |  |
| ControllableLoadResource | teid | x | >=0 and <= 999999999 |
| ControllableLoadResource | aliasName | x |  |
| CurrentRelay | teid | x | >=0 and <= 999999999 |
| Curve | curveStyle | x |  |
| Curve | xUnit | x |  |
| Curve | y1Unit | x |  |
| Curve | y2Unit | x |  |
| CurveData | xvalue | x |  |
| CurveData | y1value | x |  |
| CurveData | y2value | x |  |
| CustomerLoad | conformingLoadFlag | x |  |
| CustomerLoad | pfixedPct | x |  |
| CustomerLoad | pnomPct | x |  |
| CustomerLoad | qfixed | x |  |
| CustomerLoad | qnomPct | x |  |
| CustomerLoad | teid | x | >=0 and <= 999999999 |
| DayType | FriFlag | x |  |
| DayType | MonFlag | x |  |
| DayType | SatFlag | x |  |
| DayType | SunFlag | x |  |
| DayType | ThursFlag | x |  |
| DayType | TuesFlag | x |  |
| DayType | WedFlag | x |  |
| DCLineSegment | r | x |  |
| DCLineSegment | teid | x | >=0 and <= 999999999 |
| Disconnector | initiallyOpen | x |  |
| Disconnector | normalOpen | x |  |
| Disconnector | teid | x | >=0 and <= 999999999 |
| Discrete | teid | x | >=0 and <= 999999999 |
| DiscreteValue | abnormalAlarmDelayTime |  | >= 0 |
| DiscreteValue | returnToNormalAlarmDelayTime |  | >= 0 |
| DiscreteValue | tapChangerTimeout |  | >= 0 |
| DiscreteValue | timeDelay |  | >= 0 |
| DiscreteValue | cstatus | x |  |
| DrumBoiler | teid | x | >=0 and <= 999999999 |
| EndCap | teid | x | >=0 and <= 999999999 |
| EnergyConsumer | conformingLoadFlag | x |  |
| EnergyConsumer | pfixedPct | x |  |
| EnergyConsumer | pnomPct | x |  |
| EnergyConsumer | qfixed | x |  |
| EnergyConsumer | qnomPct | x |  |
| EnergyConsumer | teid | x | >=0 and <= 999999999 |
| EnergyConsumerAltConnection | percentTransfer | x |  |
| EnergyConsumerResource | maxDailyDeployment | x | >= maxDailyDeployment |
| EnergyConsumerResource | maxInterruptionTime | x | <= maxInterruptionTime |
| EnergyConsumerResource | maxWeeklyDeployment | x |  |
| EnergyConsumerResource | minInterruptionTime | x |  |
| EnergyConsumerResource | minRestorationTime | x |  |
| EnergyConsumerResource | teid | x | >=0 and <= 999999999 |
| EPSMeter | AcceptedStatus | x |  |
| EPSMeter | RID | x |  |
| EPSMeter | teid | x | >=0 and <= 999999999 |
| Equipment | teid | x | >=0 and <= 999999999 |
| EquipmentContainer | teid | x | >=0 and <= 999999999 |
| EquivalentSource | teid | x | >=0 and <= 999999999 |
| ERCOTVersion | ContactEmail | x |  |
| ERCOTVersion | ContactPhoneNumber | x |  |
| ERCOTVersion | date | x |  |
| ERCOTVersion | ModelAdmin | x |  |
| ERCOTVersion | version | x |  |
| Feeder | teid | x | >=0 and <= 999999999 |
| FlowgateElement | biddable | x |  |
| FlowgateElement | endDate | x |  |
| FlowgateElement | monitored | x |  |
| FlowgateElement | rating | x |  |
| FlowgateElement | startDate | x |  |
| FlowgateGroup | infB | x |  |
| FlowgateGroup | infC | x |  |
| FlowgateGroup | teid | x | >=0 and <= 999999999 |
| FossilSteamSupply | teid | x | >=0 and <= 999999999 |
| FrequencyConverter | teid | x | >=0 and <= 999999999 |
| FrequencyRelay | teid | x | >=0 and <= 999999999 |
| Fuse | initiallyOpen | x |  |
| Fuse | normalOpen | x |  |
| Fuse | teid | x | >=0 and <= 999999999 |
| Gate | limitPercentage |  | >=0 and <= 100 |
| Gate | thresholdValue | x |  |
| GeneratingUnit | coldStartTime | x |  |
| GeneratingUnit | controlPulseHigh | x |  |
| GeneratingUnit | controlPulseLow | x |  |
| GeneratingUnit | controlResponseRate | x |  |
| GeneratingUnit | efficiency | x | >= 0.001 and <= 1.2 |
| GeneratingUnit | freqBias | x |  |
| GeneratingUnit | freqDevDeadband | x |  |
| GeneratingUnit | governorMPL | x |  |
| GeneratingUnit | governorSCD | x |  |
| GeneratingUnit | highReasonabilityLimit | x | >= lowReasonabilityLimit |
| GeneratingUnit | highReasonabilityRampRateLimit | x | >= lowReasonabilityRampRateLimit |
| GeneratingUnit | hotintTime | x |  |
| GeneratingUnit | hotStartTime | x | <= intStartTime |
| GeneratingUnit | intColdTime | x |  |
| GeneratingUnit | intStartTime | x | <= coldStartTime |
| GeneratingUnit | lowReasonabilityLimit | x | >= 0 |
| GeneratingUnit | lowReasonabilityRampRateLimit | x |  |
| GeneratingUnit | maxDailyStarts | x |  |
| GeneratingUnit | maximumEconomicMW |  | >=0 and <= 9999 |
| GeneratingUnit | maximumOperatingMW | x | equal to highReasonabilityLimit |
| GeneratingUnit | maxONTime | x |  |
| GeneratingUnit | maxWeeklyEnergy | x |  |
| GeneratingUnit | maxWeeklyStarts | x | >= maxDailyStarts |
| GeneratingUnit | minimumOperatingMW | x | >=0 and <= 999 and equal to lowReasonabilityLimit |
| GeneratingUnit | minOFFTime | x |  |
| GeneratingUnit | minONTime | x | <= maxONTime |
| GeneratingUnit | modelDetail | x |  |
| GeneratingUnit | penaltyFactor | x |  |
| GeneratingUnit | seasonalMaxSustainableRating | x | >= seasonalMinSustainableRating |
| GeneratingUnit | seasonalMinEmergencyRating | x |  |
| GeneratingUnit | seasonalMinSustainableRating | x |  |
| GeneratingUnit | stepChange | x |  |
| GeneratingUnit | teid | x | >=0 and <= 999999999 |
| GenericEquipment | teid | x | >=0 and <= 999999999 |
| GenericUnderfrequencyRelay | relayDelayTime | x |  |
| GenericUnderfrequencyRelay | teid | x | >=0 and <= 999999999 |
| Ground | teid | x | >=0 and <= 999999999 |
| GroundDisconnector | initiallyOpen | x |  |
| GroundDisconnector | normalOpen | x |  |
| GroundDisconnector | teid | x | >=0 and <= 999999999 |
| HeatExchanger | teid | x | >=0 and <= 999999999 |
| HeatRecoveryBoiler | teid | x | >=0 and <= 999999999 |
| HostControlArea | areaControlMode | x |  |
| HostControlArea | forGeneration | x |  |
| HostControlArea | forNetwork | x |  |
| HostControlArea | teid | x | >=0 and <= 999999999 |
| HydroGeneratingUnit | coldStartTime | x |  |
| HydroGeneratingUnit | controlPulseHigh | x |  |
| HydroGeneratingUnit | controlPulseLow | x |  |
| HydroGeneratingUnit | controlResponseRate | x |  |
| HydroGeneratingUnit | efficiency | x | >= 0.001 and <= 1.2 |
| HydroGeneratingUnit | freqBias | x |  |
| HydroGeneratingUnit | freqDevDeadband | x |  |
| HydroGeneratingUnit | governorMPL | x |  |
| HydroGeneratingUnit | governorSCD | x |  |
| HydroGeneratingUnit | highReasonabilityLimit | x | >= lowReasonabilityLimit |
| HydroGeneratingUnit | highReasonabilityRampRateLimit | x | >= lowReasonabilityRampRateLimit |
| HydroGeneratingUnit | hotintTime | x |  |
| HydroGeneratingUnit | hotStartTime | x | <= intStartTime |
| HydroGeneratingUnit | intColdTime | x |  |
| HydroGeneratingUnit | intStartTime | x | <= coldStartTime |
| HydroGeneratingUnit | lowReasonabilityLimit | x | >= 0 |
| HydroGeneratingUnit | lowReasonabilityRampRateLimit | x |  |
| HydroGeneratingUnit | maxDailyStarts | x |  |
| HydroGeneratingUnit | maximumEconomicMW |  | >=0 and <= 9999 |
| HydroGeneratingUnit | maximumOperatingMW | x | equal to highReasonabilityLimit |
| HydroGeneratingUnit | maxONTime | x |  |
| HydroGeneratingUnit | maxWeeklyEnergy | x |  |
| HydroGeneratingUnit | maxWeeklyStarts | x | >= maxDailyStarts |
| HydroGeneratingUnit | minimumOperatingMW | x | >=0 and <= 999 and equal to lowReasonabilityLimit |
| HydroGeneratingUnit | minOFFTime | x |  |
| HydroGeneratingUnit | minONTime | x | <= maxONTime |
| HydroGeneratingUnit | modelDetail | x |  |
| HydroGeneratingUnit | penaltyFactor | x |  |
| HydroGeneratingUnit | seasonalMaxSustainableRating | x | >= seasonalMinSustainableRating |
| HydroGeneratingUnit | seasonalMinEmergencyRating | x |  |
| HydroGeneratingUnit | seasonalMinSustainableRating | x |  |
| HydroGeneratingUnit | stepChange | x |  |
| HydroGeneratingUnit | teid | x | >=0 and <= 999999999 |
| HydroPowerPlant | teid | x | >=0 and <= 999999999 |
| HydroPump | teid | x | >=0 and <= 999999999 |
| HydroTurbine | teid | x | >=0 and <= 999999999 |
| InductionMotorLoad | conformingLoadFlag | x |  |
| InductionMotorLoad | pfixedPct | x |  |
| InductionMotorLoad | pnomPct | x |  |
| InductionMotorLoad | qfixed | x |  |
| InductionMotorLoad | qnomPct | x |  |
| InductionMotorLoad | teid | x | >=0 and <= 999999999 |
| IrregularIntervalSchedule | startTime | x |  |
| IrregularIntervalSchedule | value1Unit | x |  |
| IrregularIntervalSchedule | value2Unit | x |  |
| IrregularTimePoint | time | x |  |
| IrregularTimePoint | value1 | x |  |
| IrregularTimePoint | value2 | x |  |
| Jumper | initiallyOpen | x |  |
| Jumper | normalOpen | x |  |
| Jumper | teid | x | >=0 and <= 999999999 |
| Junction | teid | x | >=0 and <= 999999999 |
| LevelVsVolumeCurve | curveStyle | x |  |
| LevelVsVolumeCurve | xUnit | x |  |
| LevelVsVolumeCurve | y1Unit | x |  |
| LevelVsVolumeCurve | y2Unit | x |  |
| Line | teid | x | >=0 and <= 999999999 |
| Load | conformingLoadFlag | x |  |
| Load | pfixedPct | x |  |
| Load | pnomPct | x |  |
| Load | qfixedPct | x |  |
| Load | qnomPct | x |  |
| Load | teid | x | >=0 and <= 999999999 |
| LoadBasedRegulationCurve | curveStyle | x |  |
| LoadBasedRegulationCurve | xUnit | x |  |
| LoadBasedRegulationCurve | y1Unit | x |  |
| LoadBasedRegulationCurve | y2Unit | x |  |
| LoadBasedRegulationCurve | aliasName | x |  |
| LoadBreakSwitch | initiallyOpen | x |  |
| LoadBreakSwitch | normalOpen | x |  |
| LoadBreakSwitch | teid | x | >=0 and <= 999999999 |
| LoadDemandModel | startTime | x |  |
| LoadDemandModel | value1Unit | x |  |
| LoadDemandModel | value2Unit | x |  |
| LoadResource | maxWeeklyEnergy | x |  |
| LoadResource | minNoticeTime | x |  |
| LoadResource | maxDailyDeployment | x |  |
| LoadResource | maxInterruptionTime | x |  |
| LoadResource | maxWeeklyDeployment | x |  |
| LoadResource | minInterruptionTime | x |  |
| LoadResource | teid | x | >=0 and <= 999999999 |
| LoadResource | aliasName | x |  |
| MeasurementArgument | referenceName | x | Required if ReferenceFlag = True |
| MeasurementValue | cstatus | x |  |
| MeasurementValueQuality | hasCOV | x |  |
| MeasurementValueQuality | hasQuality | x |  |
| MeasurementValueQuality | hasTimestamp | x |  |
| MeasurementValueSource | biverAus | x |  |
| MeasurementValueSource | biverTay | x |  |
| MeasurementValueSource | BufferTime | x |  |
| MeasurementValueSource | defaultDataSetPeriod | x |  |
| MeasurementValueSource | GraceTime | x |  |
| MeasurementValueSource | iccpDomainAus | x |  |
| MeasurementValueSource | iccpDomainTay | x |  |
| MeasurementValueSource | IntegrityPeriod | x |  |
| MeasurementValueSource | lnkapp | x |  |
| MeasurementValueSource | MaxDisPerDataset | x |  |
| MeasurementValueSource | maxScanRate | x |  |
| MeasurementValueSource | protocolVersion | x |  |
| MeasurementValueSource | remoteIdAus | x |  |
| MeasurementValueSource | remoteIdTay | x |  |
| MeasurementValueSource | savecaseType | x |  |
| MeasurementValueSource | trsfr | x |  |
| ModelVersion | date | x |  |
| ModelVersion | information | x |  |
| ModelVersion | version | x |  |
| MVArCapabilityCurve | curveStyle | x |  |
| MVArCapabilityCurve | xUnit | x |  |
| MVArCapabilityCurve | y1Unit | x |  |
| MVArCapabilityCurve | y2Unit | x |  |
| MVArCapabilityCurve | aliasName | x |  |
| NonConformLoad | conformingLoadFlag | x |  |
| NonConformLoad | pfixedPct | x |  |
| NonConformLoad | pnomPct | x |  |
| NonConformLoad | qfixedPct | x |  |
| NonConformLoad | qnomPct | x |  |
| NonConformLoad | teid | x | >=0 and <= 999999999 |
| NuclearGeneratingUnit | coldStartTime | x |  |
| NuclearGeneratingUnit | controlPulseHigh | x |  |
| NuclearGeneratingUnit | controlPulseLow | x |  |
| NuclearGeneratingUnit | controlResponseRate | x |  |
| NuclearGeneratingUnit | efficiency | x | >= 0.001 and <= 1.2 |
| NuclearGeneratingUnit | freqBias | x |  |
| NuclearGeneratingUnit | freqDevDeadband | x |  |
| NuclearGeneratingUnit | governorMPL | x |  |
| NuclearGeneratingUnit | governorSCD | x |  |
| NuclearGeneratingUnit | highReasonabilityLimit | x | >= lowReasonabilityLimit |
| NuclearGeneratingUnit | highReasonabilityRampRateLimit | x | >= lowReasonabilityRampRateLimit |
| NuclearGeneratingUnit | hotintTime | x |  |
| NuclearGeneratingUnit | hotStartTime | x | <= intStartTime |
| NuclearGeneratingUnit | intColdTime | x |  |
| NuclearGeneratingUnit | intStartTime | x | <= coldStartTime |
| NuclearGeneratingUnit | lowReasonabilityLimit | x | >= 0 |
| NuclearGeneratingUnit | lowReasonabilityRampRateLimit | x |  |
| NuclearGeneratingUnit | maxDailyStarts | x |  |
| NuclearGeneratingUnit | maximumEconomicMW |  | >=0 and <= 9999 |
| NuclearGeneratingUnit | maximumOperatingMW | x | equal to highReasonabilityLimit |
| NuclearGeneratingUnit | maxONTime | x |  |
| NuclearGeneratingUnit | maxWeeklyEnergy | x |  |
| NuclearGeneratingUnit | maxWeeklyStarts | x | >= maxDailyStarts |
| NuclearGeneratingUnit | minimumOperatingMW | x | >=0 and <= 999 and equal to lowReasonabilityLimit |
| NuclearGeneratingUnit | minOFFTime | x |  |
| NuclearGeneratingUnit | minONTime | x | <= maxONTime |
| NuclearGeneratingUnit | modelDetail | x |  |
| NuclearGeneratingUnit | penaltyFactor | x |  |
| NuclearGeneratingUnit | seasonalMaxSustainableRating | x | >= seasonalMinSustainableRating |
| NuclearGeneratingUnit | seasonalMinEmergencyRating | x |  |
| NuclearGeneratingUnit | seasonalMinSustainableRating | x |  |
| NuclearGeneratingUnit | stepChange | x |  |
| NuclearGeneratingUnit | teid | x | >=0 and <= 999999999 |
| OwnerShareLimits | highReasonabilityLimit | x | >= lowReasonabilityLimit |
| OwnerShareLimits | lowReasonabilityLimit | x |  |
| OwnerShareLimits | maxWeeklyEnergy | x |  |
| OwnerShareLimits | teid | x | >=0 and <= 999999999 |
| OwnerShareLimits | aliasName | x |  |
| OwnerShareRating | teid | x | >=0 and <= 999999999 |
| Ownership | ownPercent | x |  |
| Ownership | teid | x | >=0 and <= 999999999 |
| PenstockLossCurve | curveStyle | x |  |
| PenstockLossCurve | xUnit | x |  |
| PenstockLossCurve | y1Unit | x |  |
| PenstockLossCurve | y2Unit | x |  |
| PhaseShifterImpedanceCurve | curveStyle | x |  |
| PhaseShifterImpedanceCurve | xUnit | x |  |
| PhaseShifterImpedanceCurve | y1Unit | x |  |
| PhaseShifterImpedanceCurve | y2Unit | x |  |
| PhaseShifterImpedanceCurve | aliasName | x |  |
| PhysicalTiePoint | teid | x | >=0 and <= 999999999 |
| PlanningBay | teid | x | >=0 and <= 999999999 |
| Plant | teid | x | >=0 and <= 999999999 |
| PowerCutZone | teid | x | >=0 and <= 999999999 |
| PowerSystemResource | teid | x | >=0 and <= 999999999 |
| PowerTransformer | bmagSat | x |  |
| PowerTransformer | transformerType | x |  |
| PowerTransformer | voltageSensitivity | x |  |
| PowerTransformer | teid | x | >=0 and <= 999999999 |
| PricingVector | teid | x | >=0 and <= 999999999 |
| PrimeMover | teid | x | >=0 and <= 999999999 |
| ProtectedSwitch | initiallyOpen | x |  |
| ProtectedSwitch | normalOpen | x |  |
| ProtectedSwitch | teid | x | >=0 and <= 999999999 |
| ProtectionEquipment | teid | x | >=0 and <= 999999999 |
| PseudoCompany | teid | x | >=0 and <= 999999999 |
| PWRSteamSupply | teid | x | >=0 and <= 999999999 |
| RampRateCurve | Emergency | x |  |
| RampRateCurve | curveStyle | x |  |
| RampRateCurve | xUnit | x |  |
| RampRateCurve | y1Unit | x |  |
| RampRateCurve | y2Unit | x |  |
| RampRateCurve | aliasName | x |  |
| Rating | ratingType | x |  |
| Rating | TempPoint | x |  |
| Rating | forNetwork | x |  |
| RectifierInverter | teid | x | >=0 and <= 999999999 |
| RegularIntervalSchedule | startTime | x |  |
| RegularIntervalSchedule | value1Unit | x |  |
| RegularIntervalSchedule | value2Unit | x |  |
| RegulatingCondEq | teid | x | >=0 and <= 999999999 |
| RegulationSchedule | startTime | x |  |
| RegulationSchedule | value1Unit | x |  |
| RegulationSchedule | value2Unit | x |  |
| RemedialActionScheme | active | x |  |
| RemedialActionScheme | isRAP | x |  |
| RemedialActionScheme | isRAS | x |  |
| RemoteUnit | teid | x | >=0 and <= 999999999 |
| Reservoir | teid | x | >=0 and <= 999999999 |
| ResourceController | externalJointUnit | x |  |
| ResourceController | frequencyBias | x |  |
| ResourceController | gross | x |  |
| ResourceController | jointUnit | x |  |
| ResourceController | physical | x |  |
| ResourceController | teid | x | >=0 and <= 999999999 |
| SeasonDayTypeSchedule | startTime | x |  |
| SeasonDayTypeSchedule | value1Unit | x |  |
| SeasonDayTypeSchedule | value2Unit | x |  |
| SeriesCompensator | r | x |  |
| SeriesCompensator | x | x |  |
| SeriesCompensator | teid | x | >=0 and <= 999999999 |
| ShuntCompensator | aVRDelay | x |  |
| ShuntCompensator | maximumkV | x |  |
| ShuntCompensator | minimumkV | x |  |
| ShuntCompensator | nominalkV | x |  |
| ShuntCompensator | nominalMVAr | x |  |
| ShuntCompensator | switchingPriority | x |  |
| ShuntCompensator | voltSensitivity | x |  |
| ShuntCompensator | avrEnabled | x |  |
| ShuntCompensator | teid | x | >=0 and <= 999999999 |
| StaticVarCompensator | b | x |  |
| StaticVarCompensator | capacitiveRating | x | > 0 |
| StaticVarCompensator | fixedMVAr | x | < 0 |
| StaticVarCompensator | highVoltageChangeThreshold | x |  |
| StaticVarCompensator | inductiveRating | x | < 0 |
| StaticVarCompensator | lowVoltageChangeThreshold | x |  |
| StaticVarCompensator | maximumkV | x |  |
| StaticVarCompensator | minimumkV | x |  |
| StaticVarCompensator | postContingencyMVARMax |  | postContingencyMVARMax >steadyStateMVARMax > steadyStateMVARMin > postContingencyMVARMin |
| StaticVarCompensator | postContingencyMVARMin |  | postContingencyMVARMax >steadyStateMVARMax > steadyStateMVARMin > postContingencyMVARMin |
| StaticVarCompensator | steadyStateMVARMax | x | postContingencyMVARMax >steadyStateMVARMax > steadyStateMVARMin > postContingencyMVARMin |
| StaticVarCompensator | steadyStateMVARMin | x | postContingencyMVARMax >steadyStateMVARMax > steadyStateMVARMin > postContingencyMVARMin |
| StaticVarCompensator | voltageSetPoint |  | >= 0 and <= 1100 |
| StaticVarCompensator | teid | x | >=0 and <= 999999999 |
| StationSupply | conformingLoadFlag | x |  |
| StationSupply | pfixedPct | x |  |
| StationSupply | pnomPct | x |  |
| StationSupply | qfixed | x |  |
| StationSupply | qnomPct | x |  |
| StationSupply | teid | x | >=0 and <= 999999999 |
| StationSupplyWRCurve | aliasName | x |  |
| SteamSupply | teid | x | >=0 and <= 999999999 |
| SteamTurbine | teid | x | >=0 and <= 999999999 |
| SubControlArea | ControlAreaNumber | x |  |
| SubControlArea | teid | x | >=0 and <= 999999999 |
| Subcritical | teid | x | >=0 and <= 999999999 |
| SubGeographicalRegion | isWeatherZone | x |  |
| SubGeographicalRegion | aliasName | x | if isWeatherZone=true |
| Substation | forNetwork | x |  |
| Substation | forScada | x |  |
| Substation | teid | x | >=0 and <= 999999999 |
| Supercritical | teid | x | >=0 and <= 999999999 |
| Switch | teid | x | >=0 and <= 999999999 |
| SynchrocheckRelay | teid | x | >=0 and <= 999999999 |
| SynchronousMachine | baseMVAr | x |  |
| SynchronousMachine | inertia | x |  |
| SynchronousMachine | maximumKV | x | >=0 and <= 1100 |
| SynchronousMachine | maximumMVAr | x |  |
| SynchronousMachine | minimumKV | x | >=0 and <= 1100 |
| SynchronousMachine | minimumMVAr | x |  |
| SynchronousMachine | r | x |  |
| SynchronousMachine | ratedMVA | x | >= maximumMVAr |
| SynchronousMachine | x | x | >=0 and <= 500 |
| SynchronousMachine | x0 |  | >=0.001 and <= 500 |
| SynchronousMachine | xSubtrans | x |  |
| SynchronousMachine | xTrans | x |  |
| SynchronousMachine | teid | x | >=0 and <= 999999999 |
| TapChanger | avrEnabled | x |  |
| TapChanger | awrEnabled | x |  |
| TapChanger | highStep | x | >=0 and <= 99 |
| TapChanger | initialDelay | x |  |
| TapChanger | lowStep | x | >=0 and <= 99 |
| TapChanger | maximumkV | x |  |
| TapChanger | minimumkV | x |  |
| TapChanger | neutralAngle | x |  |
| TapChanger | neutralKV | x | >=0 and <= 1100 |
| TapChanger | neutralStep | x | >=0 and <= 99 |
| TapChanger | normalStep | x | >=-99 and <= 99 , >= lowStep and <= highStep |
| TapChanger | stepVoltageIncrement | x | >=0 and <= 100 |
| TapChanger | subsequentDelay | x |  |
| TapChanger | teid | x | >=0 and <= 999999999 |
| ThermalGeneratingUnit | coldStartTime | x |  |
| ThermalGeneratingUnit | controlPulseHigh | x |  |
| ThermalGeneratingUnit | controlPulseLow | x |  |
| ThermalGeneratingUnit | controlResponseRate | x |  |
| ThermalGeneratingUnit | efficiency | x | >= 0.001 and <= 1.2 |
| ThermalGeneratingUnit | freqBias | x |  |
| ThermalGeneratingUnit | freqDevDeadband | x |  |
| ThermalGeneratingUnit | governorMPL | x |  |
| ThermalGeneratingUnit | governorSCD | x |  |
| ThermalGeneratingUnit | highReasonabilityLimit | x | >= lowReasonabilityLimit |
| ThermalGeneratingUnit | highReasonabilityRampRateLimit | x | >= lowReasonabilityRampRateLimit |
| ThermalGeneratingUnit | hotintTime | x |  |
| ThermalGeneratingUnit | hotStartTime | x | <= intStartTime |
| ThermalGeneratingUnit | intColdTime | x |  |
| ThermalGeneratingUnit | intStartTime | x | <= coldStartTime |
| ThermalGeneratingUnit | lowReasonabilityLimit | x | >= 0 |
| ThermalGeneratingUnit | lowReasonabilityRampRateLimit | x |  |
| ThermalGeneratingUnit | maxDailyStarts | x |  |
| ThermalGeneratingUnit | maximumEconomicMW |  | >=0 and <= 9999 |
| ThermalGeneratingUnit | maximumOperatingMW | x | equal to highReasonabilityLimit |
| ThermalGeneratingUnit | maxONTime | x |  |
| ThermalGeneratingUnit | maxWeeklyEnergy | x |  |
| ThermalGeneratingUnit | maxWeeklyStarts | x | >= maxDailyStarts |
| ThermalGeneratingUnit | minimumOperatingMW | x | >=0 and <= 999 and equal to lowReasonabilityLimit |
| ThermalGeneratingUnit | minOFFTime | x |  |
| ThermalGeneratingUnit | minONTime | x | <= maxONTime |
| ThermalGeneratingUnit | modelDetail | x |  |
| ThermalGeneratingUnit | penaltyFactor | x |  |
| ThermalGeneratingUnit | seasonalMaxSustainableRating | x | >= seasonalMinSustainableRating |
| ThermalGeneratingUnit | seasonalMinEmergencyRating | x |  |
| ThermalGeneratingUnit | seasonalMinSustainableRating | x |  |
| ThermalGeneratingUnit | stepChange | x |  |
| ThermalGeneratingUnit | teid | x | >=0 and <= 999999999 |
| TieCorridor | teid | x | >=0 and <= 999999999 |
| TieEnd | teid | x | >=0 and <= 999999999 |
| TieLine | teid | x | >=0 and <= 999999999 |
| TransformerWinding | b | x | >=-100 and <= 100 |
| TransformerWinding | connectionType | x |  |
| TransformerWinding | emergencyMVA | x | <= shortTermMVA |
| TransformerWinding | g | x | >=0 and <= 999 |
| TransformerWinding | grounded | x |  |
| TransformerWinding | r | x | >=-50 and <= 500 |
| TransformerWinding | r0 | x | >=-5000 and <= 5000 |
| TransformerWinding | ratedKV |  | >= 0.1 and <= 1100 |
| TransformerWinding | ratedMVA | x |  |
| TransformerWinding | rground | x | >=0 and <= 10 |
| TransformerWinding | shortTermMVA | x |  |
| TransformerWinding | x | x | >= -1000 and <= 1000 |
| TransformerWinding | x0 | x | >= -10000 and <= 10000 |
| TransformerWinding | teid | x | >=0 and <= 999999999 |
| VoltageControlZone | teid | x | >=0 and <= 999999999 |
| VoltageLevel | highVoltageLimit |  | >= 0 and <= 1100 |
| VoltageLevel | lowVoltageLimit |  | >= 0 and <= 1100 |
| VoltageLevel | teid | x | >=0 and <= 999999999 |
| VoltageRelay | teid | x | >=0 and <= 999999999 |
| WindGeneratingUnit | coldStartTime | x |  |
| WindGeneratingUnit | controlPulseHigh | x |  |
| WindGeneratingUnit | controlPulseLow | x |  |
| WindGeneratingUnit | controlResponseRate | x |  |
| WindGeneratingUnit | efficiency | x | >= 0.001 and <= 1.2 |
| WindGeneratingUnit | freqBias | x |  |
| WindGeneratingUnit | freqDevDeadband | x |  |
| WindGeneratingUnit | governorMPL | x |  |
| WindGeneratingUnit | governorSCD | x |  |
| WindGeneratingUnit | highReasonabilityLimit | x | >= lowReasonabilityLimit |
| WindGeneratingUnit | highReasonabilityRampRateLimit | x | >= lowReasonabilityRampRateLimit |
| WindGeneratingUnit | hotintTime | x |  |
| WindGeneratingUnit | hotStartTime | x | <= intStartTime |
| WindGeneratingUnit | intColdTime | x |  |
| WindGeneratingUnit | intStartTime | x | <= coldStartTime |
| WindGeneratingUnit | lowReasonabilityLimit | x | >= 0 |
| WindGeneratingUnit | lowReasonabilityRampRateLimit | x |  |
| WindGeneratingUnit | maxDailyStarts | x |  |
| WindGeneratingUnit | maximumEconomicMW |  | >=0 and <= 9999 |
| WindGeneratingUnit | maximumOperatingMW | x | equal to highReasonabilityLimit |
| WindGeneratingUnit | maxONTime | x |  |
| WindGeneratingUnit | maxWeeklyEnergy | x |  |
| WindGeneratingUnit | maxWeeklyStarts | x | >= maxDailyStarts |
| WindGeneratingUnit | minimumOperatingMW | x | >=0 and <= 999 and equal to lowReasonabilityLimit |
| WindGeneratingUnit | minOFFTime | x |  |
| WindGeneratingUnit | minONTime | x | <= maxONTime |
| WindGeneratingUnit | modelDetail | x |  |
| WindGeneratingUnit | penaltyFactor | x |  |
| WindGeneratingUnit | seasonalMaxSustainableRating | x | >= seasonalMinSustainableRating |
| WindGeneratingUnit | seasonalMinEmergencyRating | x |  |
| WindGeneratingUnit | seasonalMinSustainableRating | x |  |
| WindGeneratingUnit | stepChange | x |  |
| WindGeneratingUnit | teid | x | >=0 and <= 999999999 |

# Appendix B - Validation Rules – Required associations

This appendix lists all of the validation rules for associations that must be defined for a specific class.

| ClassA | ClassB |
| --- | --- |
| Accumulator | MeasurementType |
| Accumulator | Operatorship |
| Accumulator | Ownership |
| Accumulator | Terminal |
| Accumulator | Unit |
| AccumulatorLimitSet | Accumulator |
| AccumulatorValue | MeasurementGroup |
| AccumulatorValue | MeasurementValueSource |
| AccumulatorValue | PermissionArea |
| AccumulatorValue | RemoteSource |
| ACLineSegment | BaseVoltage |
| ACLineSegment | Operatorship |
| ACLineSegment | Ownership |
| AlternateLimit | AlternateLimitSet |
| Analog | Operatorship |
| Analog | Ownership |
| Analog | Unit |
| AnalogLimit | AnalogLimitSet |
| AnalogLimitSet | Analog |
| AngleDifference | MonitoredGroup |
| Bay | Operatorship |
| Bay | Ownership |
| BranchGroup | MonitoredGroup |
| Breaker | Operatorship |
| Breaker | Ownership |
| BusbarSection | BaseVoltage |
| BusbarSection | Operatorship |
| BusbarSection | Ownership |
| BWRSteamSupply | Operatorship |
| BWRSteamSupply | Ownership |
| CAESPlant | Operatorship |
| CAESPlant | Ownership |
| CogenerationPlant | Operatorship |
| CogenerationPlant | Ownership |
| CombinedCyclePlant | Operatorship |
| CombinedCyclePlant | Ownership |
| CombustionTurbine | Operatorship |
| CombustionTurbine | Ownership |
| ConformLoad | ConformLoadGroup |
| ConnectivityNode | EquipmentContainer |
| Connector | Operatorship |
| Connector | Ownership |
| ControllableLoadResource | Operatorship |
| ControllableLoadResource | Ownership |
| CurrentRelay | Operatorship |
| CurrentRelay | Ownership |
| CurveData | Curve |
| CustomerLoad | ConformLoadGroup |
| CustomerLoad | Operatorship |
| CustomerLoad | Ownership |
| Disconnector | Operatorship |
| Discrete | MeasurementType |
| Discrete | Operatorship |
| Discrete | Ownership |
| Discrete | Unit |
| EndCap | EquipmentContainer |
| EndCap | Operatorship |
| EndCap | Ownership |
| EnergyConsumerResource | Operatorship |
| EnergyConsumerResource | Ownership |
| EPSMeter | PricingVector |
| FlowgateGroup | MonitoredGroup |
| HydroGeneratingUnit | Operatorship |
| HydroGeneratingUnit | Ownership |
| Junction | Operatorship |
| Load | ConformLoadGroup |
| Load | EquipmentContainer |
| Load | Operatorship |
| Load | Ownership |
| LoadResource | EnergyConsumer |
| LoadResource | Operatorship |
| Measurement | Unit |
| Monitored | MonitoredGroup |
| MonitoredRating | MonitoredGroup |
| MVArCapabilityCurve | SynchronousMachine |
| NuclearGeneratingUnit | Operatorship |
| NuclearGeneratingUnit | Ownership |
| OwnerShareLimits | Operatorship |
| OwnerShareLimits | Ownership |
| PowerTransformer | Operatorship |
| SeriesCompensator | Operatorship |
| SettlementLoadZone | Substation |
| ShuntCompensator | Operatorship |
| StaticVarCompensator | Operatorship |
| StationSupply | Operatorship |
| StringMeasurement | Unit |
| StringMeasurementValue | PermissionArea |
| SubGeographicalRegion | GeographicalRegion |
| Substation | SubGeographicalRegion |
| Substation | Operatorship |
| SynchronousMachine | Operatorship |
| TapChanger | Operatorship |
| ThermalGeneratingUnit | CombinedCyclePlant |
| ThermalGeneratingUnit | Operatorship |
| ThermalGeneratingUnit | Ownership |
| TransformerWinding | Operatorship |
| TransmissionPath | ServicePoint |
| TransmissionPath | ServicePoint |
| VLSBusBarMonitored | MonitoredGroup |
| VoltageLevel | Substation |
| VoltageLevel | Operatorship |
| VoltageMonitored | MonitoredGroup |
| WindGeneratingUnit | Operatorship |
| WindGeneratingUnit | Ownership |

# Appendix C - Validation Rules – COMPLEX rules

This appendix lists all of the validation rules that are more complex than required fields and range checks. These include validation rules that are conditional based upon certain conditions, involve associations with other attributes, and those that describe specific data relationships. The validation rule is explained descriptively and not in a formal language.

| RuleNo. | Validation Rule |
| --- | --- |
| R1 | The abstract classes GeneratingUnit, Switch, EnergyConsumer,EnergyCosumerResource should not be used. Only the classes representing the actual classes should be used. For ex: Only instances of ThermalGeneratingUnit, NuclearGeneratingUnit, HydroGeneratingUnit or WindGeneratingUnit should be created instead of the generic GeneratingUnit Class |
| R2 | Each PowerTransformer and its associated TransformerWindings and TapChangers must be contained within one substation. |
| R3 | A PowerTransformer must be contained by a Substation. A TransformerWinding must be contained by a PowerTransformer. A TapChanger must be contained by a TransformerWinding( Check for associations between transformer, winding and tap. |
| R4 | A Measurement must be associated with a PowerSystemResource to convey containment information for the Measurement. |
| R5 | Transmission line measurements should be associated with an ACLineSegment or SeriesCompensator not with a Line. Transmission line measurements must be associated with a terminal. |
| R6 | Transformer measurements should be associated with a PowerTransformer. Transformer Windings should not have measurements. |
| R7 | A Measurement can be associated with at most one Terminal. |
| R8 | Combined Cycle configuration names for a given combined cycle plant should be unique |
| R9 | ConnectivityNodes may only be placed within ConnectivityNodeGroup. |
| R10 | SynchronousMachine's minimumMVAr and maximumMVAr attributes and the minimum/maximum MVAr value specified in the associated MVArCapabilityCurve data should match. |
| R11 | Terminals connected to the same piece of Equipment can not be connected to the same connectivitynode |
| R12 | Voltage Levels can't be empty. There should always be classes associated to voltage levels. |
| R13 | The normal and neutral tap of the transformer must be within low and high steps |
| R14 | If there are 2 identical transformers in parallel with one marked Master and the other as Follower , the tap changer of the follower should always point towards the master tapchanger and not to itself. |
| R15 | Connectivity nodes must be associated to atleast 2 terminals. |
| R16 | Terminal associated to a any given equipment should have unique names. |
| R17 | ACLinesegment cannot go between different voltage levels. |
| R18 | Seriescompensator cannot go between different voltage levels and should be contained within one substation |
| R19 | The regulated terminal of a shuntcompensator must be in the same substation as the shuntcompensator's terminal (RegulatingCondEq->Terminal) |
| R20 | A shuntcompensator's SwitchedBy switch's terminal must be connected to shuntcompensator's terminal through a connectivitynode. |
| R21 | Transformer and Line ratings should follow these rules. NormalRating<=twohourrating<=15minrating |
| R22 | Sum of Ownership.ownershippercent should total 100% for SplitGenerationUnits(SGR). An SGR will have 1 record in one of the GeneratingUnit classes and "n" number of OwnerShareLimits that are associated to that GeneratingUnit. Each OwnerShareLimit will point to a OwnerShip record. The sum of all these ownership.ownershippercent should be equal to 100. |
| R23 | DCTie Psuedo Load breaker default value must be "Closed". |
| R24 | BLT(BlockLoadTransfer) must only group loads and only zero or one psuedo GeneratingUnit per BLT. The breaker for this psuedo gen if modeleld must have a default value of OPEN. |
| R25 | HubBus must have atleast one electrical bus. |
| R26 | The ConnectivityNodes in a ConnectivityNodeGroup must be in the SAME STATION i.e. ConnectivityNodes in a given ConnectivityNodeGroup cannot span multiple Stations. |
| R27 | Deleted. |
| R28 | SynchronousMachine's 'ratedMVA' attribute should be greater than or equal to the 'baseMW' attribute of the GeneratingUnit it is associated to. |
| R29 | Check the baseVoltage (if not null) of each conducting equipment connected to a connectivityNode to be the same. |
| R30 | Only AnalogLimit instances that are children of an AnalogLimitSet instance with the LimitSet.forNetwork = False/Null should have the LimitAlarmCategory and AnalogLimitName associations populated. OR If the parent AnalogLimitSet instance’s the LimitSet.forNetwork = True of if the AnalogLimit instance’s parent is a Rating instance, the two associations do not need to be populated. |
| R31 | The energy consumer that is part of the DCTie must have ONE and ONLY one controllable load resource mapped to it. This energyconsumer can NOT be associated to a "non-controllable load resource". It has to be associated only to "controllable load resource". |
| R32 | Only AnalogLimit instances that are children of an AnalogLimitSet instance with the LimitSet.forNetwork = False/Null should have the lower and upper thresholds |
| R33 | Substation names should not exceed 8 chars. |
| R34 | The target voltage setting specified by the regulation schedules must be within 80% to 120% of the rated kV |
| R35 | The number of CurveData associated with the MVArCapabilityCurves and RampRateCurve should not exceed 10 |
| R36 | Two RampRateCurves (one with Emergency = True) should be associated with a ControllableLoadResource, a Configuration and a ThermalGeneratingUnit / HydroGeneratingUnit / NuclearGeneratingUnit / WindGeneratingUnit which has no association to a DCTie and a PrivateAreaNetwork (NetworkType: Non Modeled Generation, Mothballed, Retired) or a CombinedCyclePlant. |
| R37 | The FromConfig and ToConfig of a TransState shouldn't be same |
| R38 | FromConfig and ToConfig of a TransState must belong to the same CombinedCyclePlant |
| R39 | A Contingency can’t contain duplicate ContingencyElements |
| R40 | The controllable load resource must be associated with exactly two RampRateCurves, one for normal condition and the other for emergency condition |
| R41 | There is no duplicated CC configuration, i.e. there should not be 2 CC configurations that contain the same CCU combination with exactly same status of Primary/Alternate flag |
| R42 | Each CC plant must have at least one CC configuration to startup/shutdown |
| R43 | If a ThermalGeneratingUnit is a unit in a CombinedCyclePlant, then it cannot be a SplitGenerationResource. |
| R44 | If any Measurement instance has more than one (MeasurementValue instance that is associted with MeasurementValueSource instance) then only one can have Company != ERCOT |
| R45 | Each CustomerLoad instance should be associated with One instance of SettlementNOIELoadZone, Or One instance of DCTie, Or Its parent Substation should be associated with one instance of SettlementLoadZone |
| R46 | For any instance of MeasurementArgument with ReferenceFlag = True, the Calculation link on the associated MeasurementValue instance should not be same as the parent Calculation instance of the MeasurementArgument instance. |
| R47 | MeasurementValue instances that are sourced from or output of a calculation should have its Primary Site as “DE”. |
| R48 | Every ElectricalBus should be in a LoadZone or a NOIE load zone |
| R49 | PsuedoTerminal shoud not be Associated to a connectivityNode |
| R50 | Psuedoterminal should not be associated to a RegulatingconductingEquipment |
| R51 | EnergyConsumerResource(LoadResource and ControllableLoadResource) should not have any association to ResourceNode |
| R52 | If PAN.networktype = retired or mothbolled, then there should be only one genunit associated to it. |
| R53 | x/r ratio for ACLineSegment should follow these rules. For 345kv x/r > 7, 138kv x/r> 1.5, 69kv x/r > 1 |
| R54 | A resourceNode that is associated to a PrivateUseNetwork(PUN), should not be associated to any Physical Generating Unit |
| R55 | Every GeneratingUnit(Thermal, Hydro, Nuclear, Wind) should have an association to a ResourceNode unless the GeneratingUnit is associated to a DCTie, NonModelledGeneration or BlockLoadType resource. |
| R56 | If a Load is assigned to a NOIE load zone, then its electrical bus can ONLY have loads that are assigned to the same NOIE load zone, i.e. cannot have other loads at the same electrical bus assigned to different load zone In other words, all loads that connect to the same EB must belong to the same load zone. All those loads must either be associated to the same NOIE load zone, or none of them has this association. |
| R57 | DCTIE ElectricalBus can only connect to DCTIE load. The ElectricalBus connecting to terminal of DCTIE pseudo load (DCTIE attribute of CUSTOMERLOAD instance) cannot have other loads connect to it |
| R58 | Each CombinedCyclePlant Configuration instance should have at least one association to a ConfigurationMember instance |
| R59 | PricingVector's should have a unique name |
| R60 | MeasurementGroup and Calculation instances associated to a Zonal MeasurementLocation should have unique names. Similarly MeasurementGroup and Calculation instances associated to a Nodal MeasurementLocation should have unique names |
| R61 | Measurementargument and ConstantArgument instances associated to a Calculation should have unique names |
| R62 | Zonal Measurements(Analog, Discrete) associated to a terminal should have unique names. |
| R63 | Nodal Measurements(Analog, Discrete) associated to a terminal should have unique names. |
| R64 | At least one Ownersharelimit instance should be associated with every LoadResource, ControllableLoadResource, ThermalGeneratingUnit, HydroGeneratingUnit, WindGeneratingUnit, NuclearGeneratingUnit, |
| R65 | Every zonal AnalogLimitSet should be associated with a zonal Analog - Every Nodal AnalogLimitSet should be associated with Nodal Analog. |
| R66 | Every Zonal AnalogValue with NO association to a AnalogName should be associated with a zonal MeasurementValueQuality and MeasurementValueSource. Every Nodal AnalogValue with no association to a AnalogName should be associated with a Nonal MeasurementValueQuality and MeasurementValueSource |
| R67 | Every zonal AnalogValue with association to a AnalogName should be associated with a zonal site and should be associated to atleast one zonal MeasurementGroup or one zonal Calculation - Every Nodal AnalogValue with association to a AnalogName should be associated with a zonal site and should be associated to atleast one Nodal MeasurementGroup or one Nodal Calculation |
| R68 | Every zonal DiscreteValue with association to a DiscreteName should be associated with a zonal site and should be associated to atleast one zonal MeasurementGroup or one zonal Calculation - Every Nodal discreteValue with association to a discreteName should be associated with a zonal site and should be associated to atleast one Nodal MeasurementGroup or one Nodal Calculation |
| R69 | Every zonal Analog should be associated with a zonal AnalogValue - Every Nodal Analog should be associated with a Nodal AnalogValue |
| R70 | Every zonal AnalogLimit should be associated with a zonal AnalogLimitSet - Every Nodal AnalogLimit should be associated with a Nodal AnalogLimitSet. Check for only AnalogLimit's with association to AnalogLimitName |
| R71 | Every zonal MeasurementArgument and ConstantArgument should be associated with a zonal Calculation - Every Nodal MeasurementArgument and ConstantArgument should be associated with a Nodal Calculation |
| R72 | Every zonal ConstantValue should be associated with a zonal ConstantArgument - Every Nodal ConstantValue should be associated with a Nodal ConstantArgument |
| R73 | Every configuration should either be able to StartUp or Shutdown or transition into another configuration, else it is invalid. |
| R74 | Attributes, HightReasonabilityLimit, LowReasonabilityLimit and MaxWeeklyenergy should be populated for any ownersharelimits associated with a LoadResource, a ControllableLoadResource, and a ThermalGeneratingUnit / HydroGeneratingUnit / NuclearGeneratingUnit / WindGeneratingUnit which has no association to a DCTie and a PrivateAreaNetwork (NetworkType: Non Modeled Generation Retired, Mothballed, BlockLoadTransfer). |
| R75 | Attributes (Operational Resource Parameters), MinOnlineTime, minOfflineTime, hotstartTime, IntStartTime, ColdStartTime, Maxweeklystarts, maxonlinetime, maxdailystarts, maxweeklystarts, hotinttime, intcoldtime, syscondensercapable should be populated for any Configuration and any ThermalGeneratingUnit / HydroGeneratingUnit / NuclearGeneratingUnit / WindGeneratingUnit which has no association to a CombinedCyclePlant, a DCTie and a PrivateAreaNetwork (NetworkType: Non Modeled Generation, Retired, Mothballed, BlockLoadTransfer) . |
| R76 | Attributes (Resource Parameters), HighReasonabilityLimit, LowReasonabilityLimit, HighReasonabilityRampRateLimit, LowReasonabilityRampRateLimit, MaxSustainableRating, MinSustainableRating, MaxEmergencyRating, MinEmergencyRating should be populated for any Configuration and any ThermalGeneratingUnit / HydroGeneratingUnit / NuclearGeneratingUnit / WindGeneratingUnit which has no association to a DCTie and a PrivateAreaNetwork (NetworkType: Non Modeled Generation, Retired, Mothballed, BlockLoadTransfer). |
| R77 | Each Configuration must have at least one PRIMARY CCU (defined as CONFIGURATIONMEMBER with PRIMARY flag = ‘TRUE’ ) |
| R78 | Each CombinedCyclePlant must have at least one configuration that can start up (i.e. STARTUPFLAG = ‘TRUE’) |
| R79 | Each CombinedCyclePlant must have at least one configuration that can shut down (i.e. SHUTDOWNFLAG = ‘TRUE’) |
| R80 | Each Configuration must satisfy at least one of the two conditions a) have at least one in-transstate (i.e. transstate with that configuration as TOCONFIG_CONFIGURATION) or b) its STARTUPFLAG is set to ‘True’ |
| R81 | Each Configuration must have at least one of the two conditions a) have at least one out-transstate (i.e. transstate with that configuration as FROMCONFIG_CONFIGURATION) or b) its SHUTDNFLAG is set to ‘True’ |
| R82 | All Configurations must have path to transit from start-up configuration AND transit to shut-down figuration in some way. This can be direct transstate or through other configurations. This is to prevent CC Plant being trapped ONLINE or OFFLINE |
| R83 | Validate the UpTransition Attribute on a TransState: If UpTransition = True, count of primary members in the FromConfiguration <= count of primary members in the ToConfiguration and if UpTransition = False, count of primary members in the FromConfiguration > count of primary members in the ToConfiguration. |
| R84 | Every Zonal DiscreteValue with no association to a DiscreteName should be associated with a zonal MeasurementValueQuality and a zonal MeasurementValueSource. Every Nodal DiscreteValue with no association to a DiscreteName should be associated with a Nodal MeasurementValueQuality and a nodal MeasurementValueSource. |
| R85 | All x_axisdata data points in a MVArCapabilityCurve should be unique |
| R86 | Only one y1_axisdata point in a MVArCapabilityCurve can be 0. |
| R87 | Only one y2_axisdata point in a MVArCapabilityCurve can be 0. |
| R88 | For every ICCPQualityConversion instance, a. Only One of the following attributes ICalculated, IManual, IEstimated, ITelemetered should be true, and b. Only One of the following attributes IValid, INotValid, ISuspect, IHeld should be true, and c. Only One of the following attributes OValid, ONotValid, OSuspect, OHeld should be true, and d. Only One of the following attributes OCalculated, OManual, OEstimated, ITelemetered should be true. |
| R89 | There should be atleast one swing bus in the model. i.e. atleast one connectivity node with SlackBusPriority = True |
| R90 | There should be no Pricingvector associated to a BusbarSection or a Endcap. |
| R91 | For an EnergyConsumer (CustomerLoad or StationSupply), if conformingLoadFlag = True, then the EnergyConsumer should be associated to a ConformLoadGroup |
| R92 | When a MeasurementLocation has more than one children (MeasurementGroup or Calculation), (All zonal analogvalues and discretevalues associated to the child MeasurementGroup) and (All zonal Analogvalues and discretevalues associated with measurementargument having referencefalg = False within the child calculation) should be associated to the same terminal. All Nodal analogvalues and discretevalues associated to the child MeasurementGroup) and (All Nodal Analogvalues and discretevalues associated with measurementargument having referencefalg = False within the child calculation) should be associated to the same terminal. For example 1. MeasLoc1 -> MeasGrp1 -> AnalogValue1 ->Analog1 ->Terminal1 2. MeasLoc1 -> MeasGrp1 -> AnalogValue2 ->Analog2->Terminal1 3. MeasLoc1 -> Calc1 -> MeasArg1 (referenceflag = false) -> AnalogValue3 ->Analog3 ->Terminal1 4. MeasLoc1 -> Calc1 -> MeasArg1 (referenceflag = false) -> AnalogValue4 ->Analog4->Terminal1 |
| R93 | A TriggerCondition may have multiple Gates associated with it. One of them must be its resulting Gate, (i.e. the Gate determing the trigger condition). This resulting Gate must be of type: or/and/nand/nor |
| R94 | If a Gate is of the following types: gt/ge/ lt/le/eq/ne, then its input must be exactly One PinFlow or PinBranchGroup or PinVoltage. |
| R95 | If a Gate is of the following types: or/and/nand/nor, then its input must be one or more PinGate or PinSwitch. |
| R96 | If a Gate is of the following types: not, then its input must be exactly one PinGate or PinSwitch. |
| R97 | Deleted. |
| R98 | Any Company with CompanyType = QSE can Own and Operate Zero Or One Substation with ForNetwork = False and ForSCADA = True. |
| R99 | For any GeneratingUnit, each QSE operator of the GeneratinUnit needs to Own a Substation with ForNetwork = False and ForSCADA = True. Ex. GU->OSL->Operatorship->Company->Ownership ->Substation (forNetwork = False and ForSCADA = True) |
| R100 | The owner of a GeneratingUnit should always be an RE. Operator should be a QSE. |
| R101 | For every generating unit that’s not associated to a combined cycle plant, maximumOperatingMW = HRL and minimumOperatingMW = LRL |