NMMS:

ERCOT Modeling Guidelines

for Market Participant

Network Model NOMCR Submissions

Version 0.02

March 19, 2009

| Document Revisions |  |  |  |
| --- | --- | --- | --- |
| Date | Version | Description | Author(s) |
| 1/13/09 | 0.01 | Initial draft | Terry Nielsen |
| 3/15/09 | 0.02 | Revisions | Keith Smith |

Table of Contents

1	OVERVIEW	3

1.1	Document Organization	3

1.1.1	Major Sections	3

1.1.2	Modeling Approach Subsection	3

1.1.3	Standard Approach Subsection	3

1.1.4	Classes Not Used Subsection	3

1.1.5	Classes Not Allowed	3

1.1.6	Appendix A - Validation Rules for Required Fields	3

1.1.7	Appendix B – Complex Validation Rules	3

1.1.8	Relevant reference documents:	3

2.0	PROJECT TRACKER AND COORDINATOR	3

2.1	PTC PROJECT	3

2.2	Network Operations model change Request	3

3.0	INFORMATION MODEL MANAGER	3

3.1	Model Changes	3

3.2	IMM Tools	3

4.0	Modeling Breakers	3

4.1	Modeling Approach	3

4.2	Attributes breaker	3

4.3	Linkage breaker	3

Appendix A - Validation Rules – Required Fields	3

Appendix B - Validation Rules – Required Linkages	3

Appendix C - Validation Rules – COMPLEX rules	3

Appendix D – CLASSES modeled According to STANDARD	3

Appendix E – COMPLETE LIST OF CLASSES NOT USED	3

Appendix F – COMPLETE LIST OF CLASSES NOT ALLOWED	3

# OVERVIEW

This document provides guidelines for the creation of network models that are to be submitted to ERCOT. The models may be either created using the Information Model Management (IMM) editor component of the Network Model Management System (NMMS) or by submitting a file that is in Common Information Model (CIM) XML format.

## Document Organization

Since the ERCOT network model is based upon extensions to the CIM standard, this document focuses primarily on clarifying how to use the new data attributes, associations and classes defined in the ERCOT extensions.

### Major Sections

The document is broken down into major sections (Section 2, 3, etc.) that map to a single CIM package or a subsection of a package. This breaks down the document into logical subsections that cover areas such as generation, loads, etc.

### Modeling Approach Subsection

The modeling approach section will describe the approach a user should take for modeling this object type. It covers what associations and attributes are needed and how they should be defined. For more complex modeling concepts, it may include flowcharts or other diagrams necessary to explain the modeling approach.

The document will further elaborate on how standard CIM classes, attributes and associations are used if they are used in a non-standard way. These will also be documented in a modeling approach section.

### Standard Approach Subsection

If a particular class is modeled with no deviations from the standard approach documented in the CPSM profile, the class is listed in the subsection titled “classes modeled with standard approach”.

### Classes Not Allowed

If a particular CIM class is not used in the ERCOT systems and would potentially break the ERCOT model, the class will be listed in the subsection titled “classes not allowed”. The complete list of classes that are not used are also listed in Appendix F.

The appendix of this document contains the complete listing of validation rules that run against each NMMS NOMCR submission. If any of these rules do not pass, the NOMCR will not be allowed to be submitted and the user must make the necessary corrections. These rules document any minimally required attributes, valid data ranges. The validation also ensures that the submission meets the cardinality defined in the latest ERCOT data dictionary.

### Appendix A - Validation Rules for Required Fields

Appendix A lists all of the validation rules for attributes that must be defined for a specific class. It also lists any validation for attributes that must be within a specific value range.

#### Automatic Validation Performed by NMMS Based Upon Data Dictionary

The list of validation rules does not include the validation that is automatically performed by NMMS for links between objects. These links are validated based upon the data dictionary definition of the cardinality between the associated objects. There are two cardinality definitions that are validated, the first is the CardinalityRoleA and the second CardinalityRoleB. CardinalityRoleA defines the number of objects of the first Class that may exist related to the second Class.

For instance, in the data dictionary, the link between ThermalGeneratingUnit to CombinedCyclePlant CardinalityRoleA has a value of “0..n”. This means that there may be anything between 0 and n ThermalGeneratingUnits associated with a CombinedCyclePlant. The same association has a CardinalityRoleB of “0..1” which means that there may be between 0 and 1 CombinedCyclePlants associated with a ThermalGeneratingUnit.

This validation performed by NMMS follows the inheritance tree and the same validation is done for all objects that inherit from the object which has the cardinality definition.

For instance, in the data dictionary, a GeneratingUnit has a link to a ResourceNode that has a CardinalityRoleA defined as 1..n. This means that validation would be performed checking that there are one or more GeneratingUnits associated with a ResourceNode. It also means that validation would be performed for HydroGeneratingUnit, ThermalGeneratingUnit, WindGeneratingUnit and NuclearGeneratingUnit because these classes inherit from GeneratingUnit. Note: that the cardinality is not defined explicitly for each inherited class.

### Appendix C – Complex Validation Rules

Appendix B lists all of the validation rules that are more complex than required fields and range checks. These include validation rules that are conditional based upon certain conditions, involve associations with other attributes, and those that describe specific data relationships. The validation rule is explained descriptively and not in a formal language.

### Relevant reference documents:

IEC CIM docs, 61970-?, CPSM, etc. ???

ERCOT CIM Data Dictionary 1.21

ICCP Handbook ???

NMMS user guide?

NMMS process documents?

???

# 2.0	PROJECT TRACKER AND COORDINATOR

The Project Tracker and Coordinator(PTC) is the single entry point for Operations Model changes.  Modeling data and change requests are initially received in the PTC, where each submission is tracked through any corrections, resubmission, validation, testing and review.

Upon login to the PTC, the user will see that Project Summary screen shown below in Figure 1.  The Project Summary screen is the base display for the PTC and provides the user the ability to view, edit, and create both change requests and projects.

Figure 1: Project Summary Display

## PTC PROJECT

A PTC project is a container that is used to group similar change requests.  It is a requirement that every change request has belongs to a project, even if it is the only change request in the project.  To create a new project the user will click the New Project button on the Project Summary Display screen.  The Create New Project screen shown below in Figure 2 will appear. The user must designate the project as a NOMCR(Network Operations Model Change Request) project or a SAMR(Special Action Model Request) project.  Required fields are marked with an asterisk.   For modeling purposes, we will only cover a NOMCR project.   After the required fields are populated, the project can be saved.

Figure 2: Create New Project Display

Once a project is created, the Edit Project screen can be opened by clicking on the Project ID on the Project Summary Screen.  The Edit Project screen is shown below in Figure 3.  Here the user can make changes to the project, create a new change request, or edit an existing change request.

Figure 3: Edit Project Display

## Network Operations model change Request

Transmission Service Providers(TSPs) will submit all types of modeling changes using a Network Operations Model Change Request(NOMCR).  These changes include topological changes, limit/rating changes, telemetry mapping, and telemetry definitions.  The NOMCR process is defined in the Nodal Protocols and outlines submission deadlines, formats, and procedures.

A new NOMCR can be created from the Project Summary Screen, the Create New Project Screen, or the Edit Project Screen.  Once a NOMCR is created, the Edit NOMCR screen shown in Figure 4 will appear.  All NOMCR information can be added/edited from this screen.

Figure 4:  Edit NOMCR Display

Once data is entered into the required fields in the Edit NOMCR screen, the user can make changes to the model.  The most common method for making changes to the model is by clicking the Launch Model Editor button on the Edit NOMCR screen, which will open the Information Model Manager(IMM).  Once model changes have been made in the IMM, the NOMCR must be validated and submitted for the model changes to be applied to the Network Operations Model.  Validation is performed by clicking the Validate IMM Data button.  If the model changes pass validation, the NOMCR can be submitted by clicking the Submit NOMCR.  If the model changes fail validation, an error message will appear in the bottom left corner of the screen on the status bar.  The validation errors can be viewed by clicking on the Validation Log button.  These errors must be corrected before the NOMCR can be submitted.

Another option for making changes to the model is importing a CIM XML file.   This can be done by clicking the Browse button next to the CIM XML data field in the Upload section of the Edit NOMCR screen.  Once a CIM XML file is selected, the file is imported by clicking the Import button next to the Browse button.  After the file is imported successfully, the NOMCR can be validated and submitted.

# INFORMATION MODEL MANAGER

The Information Model Manager(IMM) is the user interface used to add and modify data in the Network Operations Model database.  The IMM can be accessed by clicking the “Launch Model Editor” button on the Edit NOMCR screen seen in Figure 4.   The IMM, as shown below in Figure 5 will appear.

Figure 5: IMM Main Screen

Market Participants will be responsible for maintaining only a small number of the instances that are seen in the IMM.  A majority of these instances will be in the IMM under the path Network/Ercot Texas Network.  Figure 6 shows the IMM model expanded to show the full ERCOT Texas Network.  The ERCOT Texas Network includes substations, AC lines, and series compensators, all of which will be modeled by the Market Participants.

Figure 6: ERCOT Texas Network

# 3.1	Model Changes

This section will cover the process of making model changes in the IMM by creating and editing a generic device.  For simplicity purposes we will use a substation as an example at this time.  The modeling of more complex scenarios and devices will be covered in Section 4.

3.1.1  Create New Instance

In order to find where in the model hierarchy a device is modeled it is best to look at the section on Substations.  Here we see that a substation is modeled under a weather zone.  To create a new substation right-click on a weather zone and select New, as seen below in Figure 7.

Figure 7:  Create New Instance

The screen seen in Figure 8 will appear in the IMM prompting the user to select an instance type.  Select substation and click the Create Instance button.

Figure 8:  Create New Substation

Once the substation is created the General Edit Instance screen shown below in Figure 9 will appear.  A name must be entered for the instance before the instance can be saved in the model.  Upon saving the new instance, the substation will appear in the model hierarchy.

Figure 9: General Edit Instance Screen

The updated model hierarchy with new substation __SUB1 is shown below in Figure 10.

Figure 10:  __SUB1 in Model Hierarchy

3.1.1.1 Add Attributes

Once the substation has been created and saved, attributes for the substation must be entered into the required attribute fields.  Required attributes for each device can be found in a table in the device section.  Looking at the section on Substations, it is found that the required attributes for a substation are forNetwork and forScada.  These fields must be populated for the model changes to pass IMM validation.

Many of the attribute fields are auto-populated by the IMM.  Figure 11 shows the attributes screen for a newly created substation.  The auto-populated fields are populated in this figure. The IMM will prevent data that is not of the correct data type or size from being saved in the model.

Figure 11:  Substation Attributes

3.1.1.2  Add Links

As with attributes, for every device there are required link fields that must be populated for the model changes to pass the IMM validation.  The required links for a substation can be found in the table named required links in the substation section of this document.  Looking at this table, it is found that the required links for a substation are Ownership, Operatorship, and PermissionArea.  Figure 12 shows the links screen for a newly created substation.

Figure 12:  Substation Links

Links cannot be manually entered into the value column.  All of the links must be added using the drag and drop method or the copy/paste method.  The path for all of the required links can be found in the Required Links table.  With the links screen shown in Figure 12 up in the IMM, navigate to the desired link in the IMM and left-click on the link holding down the mouse button.  Drag the selected link into the desired field and release the mouse button.  The value field will be populated with the link.  Save the change and move forward populating all of the required links in a similar manner.  The screen shot in Figure 13 shows the ERCOT_Operator and ERCOT_Owner links in the __SUB1 Operatorship and Ownership fields.

Figure 13:  __SUB1 Links Populated

    - Rename Instance

To rename an instance right-click on the instance and selecting Rename.  A popup screen will appear prompting the user to enter a new instance name.  This popup screen is shown in Figure 14.  After the name is entered and the user selects the OK button, the instance name will be updated in the model hierarchy.

Figure 14:  Rename Substation

    - Delete Instance

To delete an instance right-click on the instance and select Delete.  The warning message shown in Figure 15 will appear.  If the instance or children of the instance are modified in a later job, the user will not be able to delete the instance.

Figure 15:  Delete Substation

# IMM Tools

    - Templates

    - Auto Layout

# Modeling Breakers

## Modeling Approach

The ERCOT CIM model defines a breaker as a mechanical switching device capable of making, carrying, and breaking currents under normal circuit conditions and also making, carrying for a specified time, and breaking currents under specified abnormal circuit conditions e.g. those of short circuit.

A breaker is modeled under a VoltageLevel in a Substation.  Two terminals are modeled under the breaker as connection points to the circuit.  SCADA values modeled under the terminals provide the Open/Close status of the breakers to the model.  Figure 2 below shows a breaker modeled in the IMM model hierarchy with parent/child associations labeled.    For more information on modeling SCADA values, refer to Section 18 of this document.

Figure 2

## Attributes breaker

The attributes for a breaker are shown in the table below.  Required attributes are listed at the top of the table in bold text.  Most of the Boolean values will have default values upon creation.  These values are listed in the table along with sample data for the field.

| Attribute | Description | Data Type | Default Value | Sample Data |
| --- | --- | --- | --- | --- |
| TEID | Transmission Element ID | Integer | Auto-populated |  |
| normalOpen | The attribute is used in cases when no Measurement for the status value is present. | Boolean | False |  |
| initiallyOpen | T = The initial breaker state for the online switching device model is open. | Boolean | False |  |
| inTransitTime | The transition time from open to close, in seconds. | Float | 0 |  |
| Alias Name | Free text name of the object or instance. | String | None |  |
| Current Flow | Fault interrupting rating in amperes. |  |  |  |
| DAM Monitored | The Boolean flags to indicate which process(s) monitor the branch devices and by which process the devices is secured. | Boolean | False |  |
| DAM Secured | The Boolean flags to indicate which process(s) monitor the branch devices and by which process the devices is secured. | Boolean | True |  |
| Enable Manual Switching | T = Enables manual switching capability for the device. | Boolean | True |  |
| Latitude | Latiitude coordinates. | Float | None |  |
| Local Name | The localName is a human readable name of the object. It is only used with objects organized in a naming hierarchy. The simplest naming hierarchy has just one parent (the root) giving a flat naming hierarchy. However, the naming hierarchy usually has several levels, e.g. Substation, VoltageLevel, Equipment etc. Children of the same parent have names that are unique among them. If the uniqueness requirement cannot be met IdentifiedObject.localName shall not be used, use IdentifiedObject.name instead. | String | None |  |
| Longitude | Longitude coordinates. | Float | None |  |
| Model Resource ID | A Model Authority issues mRIDs. Given that each Model Authority has a unique id and this id is part of the mRID, then the mRID is globally unique. | Integer | None |  |
| Normal Open | The attribute is used in cases when no Measurement for the status value is present. If the Switch has a status measurment the Measurement.normalValue is expected to match with the Switch.normalOpen. | Boolean | False |  |
| OUS Priority | An Outage Scheduler priority is included in all ConductingEquipment classes except BusbarSection and TransformerWinding. |  |  |  |
| Path Name | The pathname is a system unique name composed from all IdentifiedObject.names in a standard naming hierarchy path from the object to the root. |  |  |  |
| Phases | Describes the phases carried by a conducting equipment. Possible values { ABCN , ABC, ABN, ACN, BCN, AB, AC, BC, AN, BN, CN, A, B, C, N }. |  |  |  |
| PSS®E ID | Attribute will give IMM modelers control over the PSS®E ID that the Topology Processor uses to properly name units, loads, branches, and transformers |  |  |  |
| Remove | The equipment is removed from the network. | Boolean | False |  |
| Remove Enable | The equipment can be removed from the network. | Boolean | False |  |
| RUC Monitored | The Boolean flags to indicate which process(s) monitor the branch devices and by which process the devices is secured. | Boolean | False |  |
| RUC Secured | The Boolean flags to indicate which process(s) monitor the branch devices and by which process the devices is secured. | Boolean | True |  |
| Switch On Count | The switch on count since the switch was last reset or initialized. | Integer | None |  |
| Switch On Date | The date and time when the switch was last switched on. | Date/Time | None |  |
| Switch Operation Over | Over Limit Violation Switch Operation (Open/Close). | Boolean | None |  |
| Switch Operation Under | Under Limit Violation Switch Operation (Open/Close). | Boolean | None |  |
| Switched By Schedule | T = The switch will follow a switching schedule to change state in the network applications. | Boolean | False |  |
| Trigger Network Sequence | T = The switch will trigger the real?time network application sequence when the switch changes state in SCADA. | Boolean | False |  |
| Trip Lockout | Lockout after trip. | Boolean | None |  |
| Zero Impedance Branch? | Mark Switching devices that should be converted to Zero Impedance Branches in Connectivity Node to Bus Processing Option 1. | Boolean | False |  |

## Linkage breaker

The required links for a breaker are defined in Table 2 below.  The path name shows where the links can be found in the model.  Definitions for all of the links can be found by clicking on the help button at the bottom of the links screen.

| Link Name | Description | Path Name |
| --- | --- | --- |
| BaseVoltage | A link to a BaseVoltage is only necessary when there is no VoltageLevel container used. If a VoltageLevel container is used the disconnector will inherit a BaseVoltage from the VoltageLevel. | Network/Base Voltage |
| Operatorship | The Operatorship link is used for permission rights. The Operator of equipment has the right to edit the equipment. An operator can only be assigned by the creator or owner of the equipment. | Network/Companies |
| Ownership | The Ownership link is used for permission rights. The owner of equipment has editing and administrative rights. An owner can only be assigned by the creator of the equipment. | Network/Companies |

# Appendix A - Validation Rules – Required Fields

This appendix lists all of the validation rules for attributes that must be defined for a specific class. It also lists any validation for attributes that must be within a specific value range.

| ClassName | Attributes | Range |
| --- | --- | --- |
| AccumulatorLimit | value | >=0 |
| AccumulatorValue | cstatus |  |
| ACLineSegment | r | >= -500 and <= 500 |
| ACLineSegment | x | >= -1000 and <= 1000 |
| ACLineSegment | teid | >=0 and <= 999999999 |
| Address | City |  |
| Address | PostalCode |  |
| Address | State |  |
| Address | Street1 |  |
| AggregateHub | hubCalculationMethod |  |
| AirCompressor | teid | >=0 and <= 999999999 |
| AlternateLimit | highLimit |  |
| AlternateLimit | lowLimit |  |
| Analog | teid | >=0 and <= 999999999 |
| AnalogValue | cstatus |  |
| AreaReserveSpec | lowerRegMarginReqt | >= 0 |
| AreaReserveSpec | opReserveReqt | >= 0 |
| AreaReserveSpec | primaryReserveReqt | >= 0 |
| AreaReserveSpec | raiseRegMarginReqt | >= 0 |
| AreaReserveSpec | spinningReserveReqt | >= 0 |
| BasePower | basePower |  |
| BaseVoltage | nominalVoltage | >=0 and <= 1100 |
| BasicIntervalSchedule | startTime |  |
| BasicIntervalSchedule | value1Unit |  |
| BasicIntervalSchedule | value2Unit |  |
| Bay | teid | >=0 and <= 999999999 |
| BranchGroup | maximumActivePower |  |
| BranchGroup | maximumReactivePower |  |
| BranchGroup | minimumReactivePower |  |
| BranchGroup | monitorActivePower |  |
| BranchGroup | teid | >=0 and <= 999999999 |
| BranchGroupTerminal | positiveFlowIn |  |
| Breaker | InTransitTime |  |
| Breaker | teid | >=0 and <= 999999999 |
| BusbarSection | teid | >=0 and <= 999999999 |
| BWRSteamSupply | teid | >=0 and <= 999999999 |
| CAESPlant | teid | >=0 and <= 999999999 |
| CogenerationPlant | teid | >=0 and <= 999999999 |
| CombinedCyclePlant | LogicalResourceNodeName |  |
| CombinedCyclePlant | teid | >=0 and <= 999999999 |
| CombinedCyclePlant | aliasName |  |
| CombustionTurbine | teid | >=0 and <= 999999999 |
| CommunicationLink | teid | >=0 and <= 999999999 |
| Company | companyType |  |
| Company | DUNS |  |
| Company | PrimaryPhone |  |
| Company | teid | >=0 and <= 999999999 |
| Company | aliasName |  |
| CompositeSwitch | teid | >=0 and <= 999999999 |
| ConductingEquipment | teid | >=0 and <= 999999999 |
| Conductor | teid | >=0 and <= 999999999 |
| Configuration | highReasonabilityLimit | >= lowReasonabilityLimit |
| Configuration | highReasonabilityRampRateLimit | >= lowReasonabilityRampRateLimit |
| Configuration | hotintTime |  |
| Configuration | hotStartTime | <= intStartTime |
| Configuration | intColdTime |  |
| Configuration | intStartTime | <= coldStartTime |
| Configuration | lowReasonabilityLimit | >= 0 |
| Configuration | lowReasonabilityRampRateLimit |  |
| Configuration | maxDailyStarts |  |
| Configuration | maxONTime |  |
| Configuration | maxWeeklyEnergy |  |
| Configuration | maxWeeklyStarts | >= maxDailyStarts |
| Configuration | minOFFTime |  |
| Configuration | minONTime | <= maxONTime |
| Configuration | primaryConfiguration |  |
| Configuration | qualificationStatus |  |
| Configuration | seasonalMaxEmergencyRating |  |
| Configuration | seasonalMaxSustainableRating | >= seasonalMinSustainableRating |
| Configuration | seasonalMinEmergencyRating |  |
| Configuration | seasonalMinSustainableRating |  |
| ConformLoad | conformingLoadFlag |  |
| ConformLoad | pfixedPct |  |
| ConformLoad | pnomPct |  |
| ConformLoad | powerFactor |  |
| ConformLoad | qfixed |  |
| ConformLoad | qnomPct |  |
| ConformLoad | teid | >=0 and <= 999999999 |
| ConformLoadSchedule | startTime |  |
| ConformLoadSchedule | value1Unit |  |
| ConformLoadSchedule | value2Unit |  |
| ConformLoadSchedule | aliasName |  |
| ConnectivityNode | teid | >=0 and <= 999999999 |
| ConnectivityNodeGroup | PSSEBusNumber | can not be between 9400-9999 and 94000-99999 |
| ConnectivityNodeGroup | teid | >=0 and <= 999999999 |
| Connector | teid | >=0 and <= 999999999 |
| Contact | EmailAddress |  |
| Contact | FirstName |  |
| Contact | LastName |  |
| Contact | PrimaryPhone |  |
| Contact | City |  |
| Contact | PostalCode |  |
| Contact | State |  |
| Contact | Street1 |  |
| Contingency | DeviceName |  |
| Contingency | DeviceType |  |
| ControlHouseEquipment | teid | >=0 and <= 999999999 |
| ControllableLoadResource | maxDeploymentTime |  |
| ControllableLoadResource | maxWeeklyEnergy |  |
| ControllableLoadResource | minNoticeTime |  |
| ControllableLoadResource | rampRateSlopeDownLimit |  |
| ControllableLoadResource | rampRateSlopeUpLimit |  |
| ControllableLoadResource | maxDailyDeployment |  |
| ControllableLoadResource | maxInterruptionTime |  |
| ControllableLoadResource | maxWeeklyDeployment |  |
| ControllableLoadResource | minInterruptionTime |  |
| ControllableLoadResource | minRestorationTime |  |
| ControllableLoadResource | teid | >=0 and <= 999999999 |
| ControllableLoadResource | aliasName |  |
| CurrentRelay | teid | >=0 and <= 999999999 |
| Curve | curveStyle |  |
| Curve | xUnit |  |
| Curve | y1Unit |  |
| Curve | y2Unit |  |
| CurveData | xvalue |  |
| CurveData | y1value |  |
| CurveData | y2value |  |
| CustomerLoad | conformingLoadFlag |  |
| CustomerLoad | pfixedPct |  |
| CustomerLoad | pnomPct |  |
| CustomerLoad | qfixed |  |
| CustomerLoad | qnomPct |  |
| CustomerLoad | teid | >=0 and <= 999999999 |
| DayType | FriFlag |  |
| DayType | MonFlag |  |
| DayType | SatFlag |  |
| DayType | SunFlag |  |
| DayType | ThursFlag |  |
| DayType | TuesFlag |  |
| DayType | WedFlag |  |
| DCLineSegment | r |  |
| DCLineSegment | teid | >=0 and <= 999999999 |
| Disconnector | initiallyOpen |  |
| Disconnector | normalOpen |  |
| Disconnector | teid | >=0 and <= 999999999 |
| Discrete | teid | >=0 and <= 999999999 |
| DiscreteValue | cstatus |  |
| DrumBoiler | teid | >=0 and <= 999999999 |
| EndCap | teid | >=0 and <= 999999999 |
| EnergyConsumer | conformingLoadFlag |  |
| EnergyConsumer | pfixedPct |  |
| EnergyConsumer | pnomPct |  |
| EnergyConsumer | qfixed |  |
| EnergyConsumer | qnomPct |  |
| EnergyConsumer | teid | >=0 and <= 999999999 |
| EnergyConsumerAltConnection | percentTransfer |  |
| EnergyConsumerResource | maxDailyDeployment | >= maxDailyDeployment |
| EnergyConsumerResource | maxInterruptionTime | <= maxInterruptionTime |
| EnergyConsumerResource | maxWeeklyDeployment |  |
| EnergyConsumerResource | minInterruptionTime |  |
| EnergyConsumerResource | minRestorationTime |  |
| EnergyConsumerResource | teid | >=0 and <= 999999999 |
| EPSMeter | AcceptedStatus |  |
| EPSMeter | RID |  |
| EPSMeter | teid | >=0 and <= 999999999 |
| Equipment | teid | >=0 and <= 999999999 |
| EquipmentContainer | teid | >=0 and <= 999999999 |
| EquivalentSource | teid | >=0 and <= 999999999 |
| ERCOTVersion | ContactEmail |  |
| ERCOTVersion | ContactPhoneNumber |  |
| ERCOTVersion | date |  |
| ERCOTVersion | ModelAdmin |  |
| ERCOTVersion | version |  |
| Feeder | teid | >=0 and <= 999999999 |
| FlowgateElement | biddable |  |
| FlowgateElement | endDate |  |
| FlowgateElement | monitored |  |
| FlowgateElement | rating |  |
| FlowgateElement | startDate |  |
| FlowgateGroup | infB |  |
| FlowgateGroup | infC |  |
| FlowgateGroup | teid | >=0 and <= 999999999 |
| FossilSteamSupply | teid | >=0 and <= 999999999 |
| FrequencyConverter | teid | >=0 and <= 999999999 |
| FrequencyRelay | teid | >=0 and <= 999999999 |
| Fuse | initiallyOpen |  |
| Fuse | normalOpen |  |
| Fuse | teid | >=0 and <= 999999999 |
| Gate | thresholdValue |  |
| GeneratingUnit | coldStartTime |  |
| GeneratingUnit | controlPulseHigh |  |
| GeneratingUnit | controlPulseLow |  |
| GeneratingUnit | controlResponseRate |  |
| GeneratingUnit | efficiency | >= 0.001 and <= 1.2 |
| GeneratingUnit | freqBias |  |
| GeneratingUnit | freqDevDeadband |  |
| GeneratingUnit | governorMPL |  |
| GeneratingUnit | governorSCD |  |
| GeneratingUnit | highReasonabilityLimit | >= lowReasonabilityLimit |
| GeneratingUnit | highReasonabilityRampRateLimit | >= lowReasonabilityRampRateLimit |
| GeneratingUnit | hotintTime |  |
| GeneratingUnit | hotStartTime | <= intStartTime |
| GeneratingUnit | intColdTime |  |
| GeneratingUnit | intStartTime | <= coldStartTime |
| GeneratingUnit | lowReasonabilityLimit | >= 0 |
| GeneratingUnit | lowReasonabilityRampRateLimit |  |
| GeneratingUnit | maxDailyStarts |  |
| GeneratingUnit | maximumOperatingMW | equal to highReasonabilityLimit |
| GeneratingUnit | maxONTime |  |
| GeneratingUnit | maxWeeklyEnergy |  |
| GeneratingUnit | maxWeeklyStarts | >= maxDailyStarts |
| GeneratingUnit | minimumOperatingMW | >=0 and <= 999 and equal to lowReasonabilityLimit |
| GeneratingUnit | minOFFTime |  |
| GeneratingUnit | minONTime | <= maxONTime |
| GeneratingUnit | modelDetail |  |
| GeneratingUnit | penaltyFactor |  |
| GeneratingUnit | seasonalMaxSustainableRating | >= seasonalMinSustainableRating |
| GeneratingUnit | seasonalMinEmergencyRating |  |
| GeneratingUnit | seasonalMinSustainableRating |  |
| GeneratingUnit | stepChange |  |
| GeneratingUnit | teid | >=0 and <= 999999999 |
| GenericEquipment | teid | >=0 and <= 999999999 |
| GenericUnderfrequencyRelay | relayDelayTime |  |
| GenericUnderfrequencyRelay | teid | >=0 and <= 999999999 |
| Ground | teid | >=0 and <= 999999999 |
| GroundDisconnector | initiallyOpen |  |
| GroundDisconnector | normalOpen |  |
| GroundDisconnector | teid | >=0 and <= 999999999 |
| HeatExchanger | teid | >=0 and <= 999999999 |
| HeatRecoveryBoiler | teid | >=0 and <= 999999999 |
| HostControlArea | areaControlMode |  |
| HostControlArea | forGeneration |  |
| HostControlArea | forNetwork |  |
| HostControlArea | teid | >=0 and <= 999999999 |
| HydroGeneratingUnit | coldStartTime |  |
| HydroGeneratingUnit | controlPulseHigh |  |
| HydroGeneratingUnit | controlPulseLow |  |
| HydroGeneratingUnit | controlResponseRate |  |
| HydroGeneratingUnit | efficiency | >= 0.001 and <= 1.2 |
| HydroGeneratingUnit | freqBias |  |
| HydroGeneratingUnit | freqDevDeadband |  |
| HydroGeneratingUnit | governorMPL |  |
| HydroGeneratingUnit | governorSCD |  |
| HydroGeneratingUnit | highReasonabilityLimit | >= lowReasonabilityLimit |
| HydroGeneratingUnit | highReasonabilityRampRateLimit | >= lowReasonabilityRampRateLimit |
| HydroGeneratingUnit | hotintTime |  |
| HydroGeneratingUnit | hotStartTime | <= intStartTime |
| HydroGeneratingUnit | intColdTime |  |
| HydroGeneratingUnit | intStartTime | <= coldStartTime |
| HydroGeneratingUnit | lowReasonabilityLimit | >= 0 |
| HydroGeneratingUnit | lowReasonabilityRampRateLimit |  |
| HydroGeneratingUnit | maxDailyStarts |  |
| HydroGeneratingUnit | maximumOperatingMW | equal to highReasonabilityLimit |
| HydroGeneratingUnit | maxONTime |  |
| HydroGeneratingUnit | maxWeeklyEnergy |  |
| HydroGeneratingUnit | maxWeeklyStarts | >= maxDailyStarts |
| HydroGeneratingUnit | minimumOperatingMW | >=0 and <= 999 and equal to lowReasonabilityLimit |
| HydroGeneratingUnit | minOFFTime |  |
| HydroGeneratingUnit | minONTime | <= maxONTime |
| HydroGeneratingUnit | modelDetail |  |
| HydroGeneratingUnit | penaltyFactor |  |
| HydroGeneratingUnit | seasonalMaxSustainableRating | >= seasonalMinSustainableRating |
| HydroGeneratingUnit | seasonalMinEmergencyRating |  |
| HydroGeneratingUnit | seasonalMinSustainableRating |  |
| HydroGeneratingUnit | stepChange |  |
| HydroGeneratingUnit | teid | >=0 and <= 999999999 |
| HydroPowerPlant | teid | >=0 and <= 999999999 |
| HydroPump | teid | >=0 and <= 999999999 |
| HydroTurbine | teid | >=0 and <= 999999999 |
| InductionMotorLoad | conformingLoadFlag |  |
| InductionMotorLoad | pfixedPct |  |
| InductionMotorLoad | pnomPct |  |
| InductionMotorLoad | qfixed |  |
| InductionMotorLoad | qnomPct |  |
| InductionMotorLoad | teid | >=0 and <= 999999999 |
| IrregularIntervalSchedule | startTime |  |
| IrregularIntervalSchedule | value1Unit |  |
| IrregularIntervalSchedule | value2Unit |  |
| IrregularTimePoint | time |  |
| IrregularTimePoint | value1 |  |
| IrregularTimePoint | value2 |  |
| Jumper | initiallyOpen |  |
| Jumper | normalOpen |  |
| Jumper | teid | >=0 and <= 999999999 |
| Junction | teid | >=0 and <= 999999999 |
| LevelVsVolumeCurve | curveStyle |  |
| LevelVsVolumeCurve | xUnit |  |
| LevelVsVolumeCurve | y1Unit |  |
| LevelVsVolumeCurve | y2Unit |  |
| Line | teid | >=0 and <= 999999999 |
| Load | conformingLoadFlag |  |
| Load | pfixedPct |  |
| Load | pnomPct |  |
| Load | qfixedPct |  |
| Load | qnomPct |  |
| Load | teid | >=0 and <= 999999999 |
| LoadBasedRegulationCurve | curveStyle |  |
| LoadBasedRegulationCurve | xUnit |  |
| LoadBasedRegulationCurve | y1Unit |  |
| LoadBasedRegulationCurve | y2Unit |  |
| LoadBasedRegulationCurve | aliasName |  |
| LoadBreakSwitch | initiallyOpen |  |
| LoadBreakSwitch | normalOpen |  |
| LoadBreakSwitch | teid | >=0 and <= 999999999 |
| LoadDemandModel | startTime |  |
| LoadDemandModel | value1Unit |  |
| LoadDemandModel | value2Unit |  |
| LoadResource | maxWeeklyEnergy |  |
| LoadResource | minNoticeTime |  |
| LoadResource | maxDailyDeployment |  |
| LoadResource | maxInterruptionTime |  |
| LoadResource | maxWeeklyDeployment |  |
| LoadResource | minInterruptionTime |  |
| LoadResource | teid | >=0 and <= 999999999 |
| LoadResource | aliasName |  |
| MeasurementArgument | referenceName | Required if ReferenceFlag = True |
| MeasurementValue | cstatus |  |
| MeasurementValueQuality | hasCOV |  |
| MeasurementValueQuality | hasQuality |  |
| MeasurementValueQuality | hasTimestamp |  |
| MeasurementValueSource | biverAus |  |
| MeasurementValueSource | biverTay |  |
| MeasurementValueSource | BufferTime |  |
| MeasurementValueSource | defaultDataSetPeriod |  |
| MeasurementValueSource | GraceTime |  |
| MeasurementValueSource | iccpDomainAus |  |
| MeasurementValueSource | iccpDomainTay |  |
| MeasurementValueSource | IntegrityPeriod |  |
| MeasurementValueSource | lnkapp |  |
| MeasurementValueSource | MaxDisPerDataset |  |
| MeasurementValueSource | maxScanRate |  |
| MeasurementValueSource | protocolVersion |  |
| MeasurementValueSource | remoteIdAus |  |
| MeasurementValueSource | remoteIdTay |  |
| MeasurementValueSource | savecaseType |  |
| MeasurementValueSource | trsfr |  |
| ModelVersion | date |  |
| ModelVersion | information |  |
| ModelVersion | version |  |
| MVArCapabilityCurve | curveStyle |  |
| MVArCapabilityCurve | xUnit |  |
| MVArCapabilityCurve | y1Unit |  |
| MVArCapabilityCurve | y2Unit |  |
| MVArCapabilityCurve | aliasName |  |
| NonConformLoad | conformingLoadFlag |  |
| NonConformLoad | pfixedPct |  |
| NonConformLoad | pnomPct |  |
| NonConformLoad | qfixedPct |  |
| NonConformLoad | qnomPct |  |
| NonConformLoad | teid | >=0 and <= 999999999 |
| NuclearGeneratingUnit | coldStartTime |  |
| NuclearGeneratingUnit | controlPulseHigh |  |
| NuclearGeneratingUnit | controlPulseLow |  |
| NuclearGeneratingUnit | controlResponseRate |  |
| NuclearGeneratingUnit | efficiency | >= 0.001 and <= 1.2 |
| NuclearGeneratingUnit | freqBias |  |
| NuclearGeneratingUnit | freqDevDeadband |  |
| NuclearGeneratingUnit | governorMPL |  |
| NuclearGeneratingUnit | governorSCD |  |
| NuclearGeneratingUnit | highReasonabilityLimit | >= lowReasonabilityLimit |
| NuclearGeneratingUnit | highReasonabilityRampRateLimit | >= lowReasonabilityRampRateLimit |
| NuclearGeneratingUnit | hotintTime |  |
| NuclearGeneratingUnit | hotStartTime | <= intStartTime |
| NuclearGeneratingUnit | intColdTime |  |
| NuclearGeneratingUnit | intStartTime | <= coldStartTime |
| NuclearGeneratingUnit | lowReasonabilityLimit | >= 0 |
| NuclearGeneratingUnit | lowReasonabilityRampRateLimit |  |
| NuclearGeneratingUnit | maxDailyStarts |  |
| NuclearGeneratingUnit | maximumOperatingMW | equal to highReasonabilityLimit |
| NuclearGeneratingUnit | maxONTime |  |
| NuclearGeneratingUnit | maxWeeklyEnergy |  |
| NuclearGeneratingUnit | maxWeeklyStarts | >= maxDailyStarts |
| NuclearGeneratingUnit | minimumOperatingMW | >=0 and <= 999 and equal to lowReasonabilityLimit |
| NuclearGeneratingUnit | minOFFTime |  |
| NuclearGeneratingUnit | minONTime | <= maxONTime |
| NuclearGeneratingUnit | modelDetail |  |
| NuclearGeneratingUnit | penaltyFactor |  |
| NuclearGeneratingUnit | seasonalMaxSustainableRating | >= seasonalMinSustainableRating |
| NuclearGeneratingUnit | seasonalMinEmergencyRating |  |
| NuclearGeneratingUnit | seasonalMinSustainableRating |  |
| NuclearGeneratingUnit | stepChange |  |
| NuclearGeneratingUnit | teid | >=0 and <= 999999999 |
| OwnerShareLimits | highReasonabilityLimit | >= lowReasonabilityLimit |
| OwnerShareLimits | lowReasonabilityLimit |  |
| OwnerShareLimits | maxWeeklyEnergy |  |
| OwnerShareLimits | teid | >=0 and <= 999999999 |
| OwnerShareLimits | aliasName |  |
| OwnerShareRating | teid | >=0 and <= 999999999 |
| Ownership | ownPercent |  |
| Ownership | teid | >=0 and <= 999999999 |
| PenstockLossCurve | curveStyle |  |
| PenstockLossCurve | xUnit |  |
| PenstockLossCurve | y1Unit |  |
| PenstockLossCurve | y2Unit |  |
| PhaseShifterImpedanceCurve | curveStyle |  |
| PhaseShifterImpedanceCurve | xUnit |  |
| PhaseShifterImpedanceCurve | y1Unit |  |
| PhaseShifterImpedanceCurve | y2Unit |  |
| PhaseShifterImpedanceCurve | aliasName |  |
| PhysicalTiePoint | teid | >=0 and <= 999999999 |
| PlanningBay | teid | >=0 and <= 999999999 |
| Plant | teid | >=0 and <= 999999999 |
| PowerCutZone | teid | >=0 and <= 999999999 |
| PowerSystemResource | teid | >=0 and <= 999999999 |
| PowerTransformer | bmagSat |  |
| PowerTransformer | transformerType |  |
| PowerTransformer | voltageSensitivity |  |
| PowerTransformer | teid | >=0 and <= 999999999 |
| PricingVector | teid | >=0 and <= 999999999 |
| PrimeMover | teid | >=0 and <= 999999999 |
| ProtectedSwitch | initiallyOpen |  |
| ProtectedSwitch | normalOpen |  |
| ProtectedSwitch | teid | >=0 and <= 999999999 |
| ProtectionEquipment | teid | >=0 and <= 999999999 |
| PseudoCompany | teid | >=0 and <= 999999999 |
| PWRSteamSupply | teid | >=0 and <= 999999999 |
| RampRateCurve | Emergency |  |
| RampRateCurve | curveStyle |  |
| RampRateCurve | xUnit |  |
| RampRateCurve | y1Unit |  |
| RampRateCurve | y2Unit |  |
| RampRateCurve | aliasName |  |
| Rating | ratingType |  |
| Rating | TempPoint |  |
| Rating | forNetwork |  |
| RectifierInverter | teid | >=0 and <= 999999999 |
| RegularIntervalSchedule | startTime |  |
| RegularIntervalSchedule | value1Unit |  |
| RegularIntervalSchedule | value2Unit |  |
| RegulatingCondEq | teid | >=0 and <= 999999999 |
| RegulationSchedule | startTime |  |
| RegulationSchedule | value1Unit |  |
| RegulationSchedule | value2Unit |  |
| RemedialActionScheme | active |  |
| RemedialActionScheme | isRAP |  |
| RemedialActionScheme | isRAS |  |
| RemoteUnit | teid | >=0 and <= 999999999 |
| Reservoir | teid | >=0 and <= 999999999 |
| ResourceController | externalJointUnit |  |
| ResourceController | frequencyBias |  |
| ResourceController | gross |  |
| ResourceController | jointUnit |  |
| ResourceController | physical |  |
| ResourceController | teid | >=0 and <= 999999999 |
| SeasonDayTypeSchedule | startTime |  |
| SeasonDayTypeSchedule | value1Unit |  |
| SeasonDayTypeSchedule | value2Unit |  |
| SeriesCompensator | r |  |
| SeriesCompensator | x |  |
| SeriesCompensator | teid | >=0 and <= 999999999 |
| ShuntCompensator | aVRDelay |  |
| ShuntCompensator | maximumkV |  |
| ShuntCompensator | minimumkV |  |
| ShuntCompensator | nominalkV |  |
| ShuntCompensator | nominalMVAr |  |
| ShuntCompensator | switchingPriority |  |
| ShuntCompensator | voltSensitivity |  |
| ShuntCompensator | avrEnabled |  |
| ShuntCompensator | teid | >=0 and <= 999999999 |
| StaticVarCompensator | b |  |
| StaticVarCompensator | capacitiveRating | > 0 |
| StaticVarCompensator | fixedMVAr | < 0 |
| StaticVarCompensator | highVoltageChangeThreshold |  |
| StaticVarCompensator | inductiveRating | < 0 |
| StaticVarCompensator | lowVoltageChangeThreshold |  |
| StaticVarCompensator | maximumkV |  |
| StaticVarCompensator | minimumkV |  |
| StaticVarCompensator | steadyStateMVARMax | postContingencyMVARMax >steadyStateMVARMax > steadyStateMVARMin > postContingencyMVARMin |
| StaticVarCompensator | steadyStateMVARMin | postContingencyMVARMax >steadyStateMVARMax > steadyStateMVARMin > postContingencyMVARMin |
| StaticVarCompensator | teid | >=0 and <= 999999999 |
| StationSupply | conformingLoadFlag |  |
| StationSupply | pfixedPct |  |
| StationSupply | pnomPct |  |
| StationSupply | qfixed |  |
| StationSupply | qnomPct |  |
| StationSupply | teid | >=0 and <= 999999999 |
| StationSupplyWRCurve | aliasName |  |
| SteamSupply | teid | >=0 and <= 999999999 |
| SteamTurbine | teid | >=0 and <= 999999999 |
| SubControlArea | ControlAreaNumber |  |
| SubControlArea | teid | >=0 and <= 999999999 |
| Subcritical | teid | >=0 and <= 999999999 |
| SubGeographicalRegion | isWeatherZone |  |
| SubGeographicalRegion | aliasName | if isWeatherZone=true |
| Substation | forNetwork |  |
| Substation | forScada |  |
| Substation | teid | >=0 and <= 999999999 |
| Supercritical | teid | >=0 and <= 999999999 |
| Switch | teid | >=0 and <= 999999999 |
| SynchrocheckRelay | teid | >=0 and <= 999999999 |
| SynchronousMachine | baseMVAr |  |
| SynchronousMachine | inertia |  |
| SynchronousMachine | maximumKV | >=0 and <= 1100 |
| SynchronousMachine | maximumMVAr |  |
| SynchronousMachine | minimumKV | >=0 and <= 1100 |
| SynchronousMachine | minimumMVAr |  |
| SynchronousMachine | r |  |
| SynchronousMachine | ratedMVA | >= maximumMVAr |
| SynchronousMachine | x | >=0 and <= 500 |
| SynchronousMachine | xSubtrans |  |
| SynchronousMachine | xTrans |  |
| SynchronousMachine | teid | >=0 and <= 999999999 |
| TapChanger | avrEnabled |  |
| TapChanger | awrEnabled |  |
| TapChanger | highStep | >=0 and <= 99 |
| TapChanger | initialDelay |  |
| TapChanger | lowStep | >=0 and <= 99 |
| TapChanger | maximumkV |  |
| TapChanger | minimumkV |  |
| TapChanger | neutralAngle |  |
| TapChanger | neutralKV | >=0 and <= 1100 |
| TapChanger | neutralStep | >=0 and <= 99 |
| TapChanger | normalStep | >=-99 and <= 99 , >= lowStep and <= highStep |
| TapChanger | stepVoltageIncrement | >=0 and <= 100 |
| TapChanger | subsequentDelay |  |
| TapChanger | teid | >=0 and <= 999999999 |
| ThermalGeneratingUnit | coldStartTime |  |
| ThermalGeneratingUnit | controlPulseHigh |  |
| ThermalGeneratingUnit | controlPulseLow |  |
| ThermalGeneratingUnit | controlResponseRate |  |
| ThermalGeneratingUnit | efficiency | >= 0.001 and <= 1.2 |
| ThermalGeneratingUnit | freqBias |  |
| ThermalGeneratingUnit | freqDevDeadband |  |
| ThermalGeneratingUnit | governorMPL |  |
| ThermalGeneratingUnit | governorSCD |  |
| ThermalGeneratingUnit | highReasonabilityLimit | >= lowReasonabilityLimit |
| ThermalGeneratingUnit | highReasonabilityRampRateLimit | >= lowReasonabilityRampRateLimit |
| ThermalGeneratingUnit | hotintTime |  |
| ThermalGeneratingUnit | hotStartTime | <= intStartTime |
| ThermalGeneratingUnit | intColdTime |  |
| ThermalGeneratingUnit | intStartTime | <= coldStartTime |
| ThermalGeneratingUnit | lowReasonabilityLimit | >= 0 |
| ThermalGeneratingUnit | lowReasonabilityRampRateLimit |  |
| ThermalGeneratingUnit | maxDailyStarts |  |
| ThermalGeneratingUnit | maximumOperatingMW | equal to highReasonabilityLimit |
| ThermalGeneratingUnit | maxONTime |  |
| ThermalGeneratingUnit | maxWeeklyEnergy |  |
| ThermalGeneratingUnit | maxWeeklyStarts | >= maxDailyStarts |
| ThermalGeneratingUnit | minimumOperatingMW | >=0 and <= 999 and equal to lowReasonabilityLimit |
| ThermalGeneratingUnit | minOFFTime |  |
| ThermalGeneratingUnit | minONTime | <= maxONTime |
| ThermalGeneratingUnit | modelDetail |  |
| ThermalGeneratingUnit | penaltyFactor |  |
| ThermalGeneratingUnit | seasonalMaxSustainableRating | >= seasonalMinSustainableRating |
| ThermalGeneratingUnit | seasonalMinEmergencyRating |  |
| ThermalGeneratingUnit | seasonalMinSustainableRating |  |
| ThermalGeneratingUnit | stepChange |  |
| ThermalGeneratingUnit | teid | >=0 and <= 999999999 |
| TieCorridor | teid | >=0 and <= 999999999 |
| TieEnd | teid | >=0 and <= 999999999 |
| TieLine | teid | >=0 and <= 999999999 |
| TransformerWinding | b | >=-100 and <= 100 |
| TransformerWinding | connectionType |  |
| TransformerWinding | emergencyMVA | <= shortTermMVA |
| TransformerWinding | g | >=0 and <= 999 |
| TransformerWinding | grounded |  |
| TransformerWinding | r | >=-50 and <= 500 |
| TransformerWinding | r0 | >=-5000 and <= 5000 |
| TransformerWinding | ratedMVA |  |
| TransformerWinding | rground | >=0 and <= 10 |
| TransformerWinding | shortTermMVA |  |
| TransformerWinding | x | >= -1000 and <= 1000 |
| TransformerWinding | x0 | >= -10000 and <= 10000 |
| TransformerWinding | teid | >=0 and <= 999999999 |
| VoltageControlZone | teid | >=0 and <= 999999999 |
| VoltageLevel | teid | >=0 and <= 999999999 |
| VoltageRelay | teid | >=0 and <= 999999999 |
| WindGeneratingUnit | coldStartTime |  |
| WindGeneratingUnit | controlPulseHigh |  |
| WindGeneratingUnit | controlPulseLow |  |
| WindGeneratingUnit | controlResponseRate |  |
| WindGeneratingUnit | efficiency | >= 0.001 and <= 1.2 |
| WindGeneratingUnit | freqBias |  |
| WindGeneratingUnit | freqDevDeadband |  |
| WindGeneratingUnit | governorMPL |  |
| WindGeneratingUnit | governorSCD |  |
| WindGeneratingUnit | highReasonabilityLimit | >= lowReasonabilityLimit |
| WindGeneratingUnit | highReasonabilityRampRateLimit | >= lowReasonabilityRampRateLimit |
| WindGeneratingUnit | hotintTime |  |
| WindGeneratingUnit | hotStartTime | <= intStartTime |
| WindGeneratingUnit | intColdTime |  |
| WindGeneratingUnit | intStartTime | <= coldStartTime |
| WindGeneratingUnit | lowReasonabilityLimit | >= 0 |
| WindGeneratingUnit | lowReasonabilityRampRateLimit |  |
| WindGeneratingUnit | maxDailyStarts |  |
| WindGeneratingUnit | maximumOperatingMW | equal to highReasonabilityLimit |
| WindGeneratingUnit | maxONTime |  |
| WindGeneratingUnit | maxWeeklyEnergy |  |
| WindGeneratingUnit | maxWeeklyStarts | >= maxDailyStarts |
| WindGeneratingUnit | minimumOperatingMW | >=0 and <= 999 and equal to lowReasonabilityLimit |
| WindGeneratingUnit | minOFFTime |  |
| WindGeneratingUnit | minONTime | <= maxONTime |
| WindGeneratingUnit | modelDetail |  |
| WindGeneratingUnit | penaltyFactor |  |
| WindGeneratingUnit | seasonalMaxSustainableRating | >= seasonalMinSustainableRating |
| WindGeneratingUnit | seasonalMinEmergencyRating |  |
| WindGeneratingUnit | seasonalMinSustainableRating |  |
| WindGeneratingUnit | stepChange |  |
| WindGeneratingUnit | teid | >=0 and <= 999999999 |

# Appendix B - Validation Rules – Required Linkages

This appendix lists all of the validation rules for associations that must be defined for a specific class.

| ClassA | ClassB |
| --- | --- |
| Accumulator | MeasurementType |
| Accumulator | Operatorship |
| Accumulator | Ownership |
| Accumulator | Terminal |
| Accumulator | Unit |
| AccumulatorLimit | AccumulatorLimitSet |
| AccumulatorLimitSet | Accumulator |
| AccumulatorValue | Accumulator |
| AccumulatorValue | MeasurementGroup |
| AccumulatorValue | MeasurementValueSource |
| AccumulatorValue | PermissionArea |
| AccumulatorValue | RemoteSource |
| AccumulatorValue | SitePermission |
| ACLineSegment | BaseVoltage |
| ACLineSegment | EquipmentContainer |
| ACLineSegment | Operatorship |
| ACLineSegment | Ownership |
| AlternateLimit | AlternateLimitSet |
| Analog | Operatorship |
| Analog | Ownership |
| Analog | Unit |
| AnalogLimit | AnalogLimitSet |
| AnalogLimitSet | Analog |
| AnalogValue | AccuracyClass |
| AnalogValue | Analog |
| AnalogValue | AnalogLimit |
| AnalogValue | AnalogName |
| AnalogValue | IccpQualityConversionSet |
| AnalogValue | SitePermission |
| AngleDifference | MonitoredGroup |
| Bay | VoltageLevel |
| Bay | Operatorship |
| Bay | Ownership |
| BranchGroup | Terminal |
| BranchGroup | MonitoredGroup |
| Breaker | EquipmentContainer |
| Breaker | Operatorship |
| Breaker | Ownership |
| BusbarSection | BaseVoltage |
| BusbarSection | EquipmentContainer |
| BusbarSection | Operatorship |
| BusbarSection | Ownership |
| BWRSteamSupply | Operatorship |
| BWRSteamSupply | Ownership |
| CAESPlant | Operatorship |
| CAESPlant | Ownership |
| Calculation | Function |
| Calculation | MeasurementLocation |
| Calculation | MeasurementValue |
| CogenerationPlant | Operatorship |
| CogenerationPlant | Ownership |
| CombinedCyclePlant | Operatorship |
| CombinedCyclePlant | Ownership |
| CombustionTurbine | Operatorship |
| CombustionTurbine | Ownership |
| Configuration | LogicalConfiguration |
| ConfigurationMember | Configuration |
| ConfigurationMember | ThermalGeneratingUnit |
| ConformLoad | ConformLoadGroup |
| ConformLoadGroup | SubLoadArea |
| ConformLoadSchedule | ConformLoadGroup |
| ConformLoadSchedule | DayType |
| ConformLoadSchedule | Season |
| ConnectivityNode | ConnectivityNodeGroup |
| ConnectivityNode | EquipmentContainer |
| ConnectivityNodeGroup | VoltageLevel |
| Connector | Operatorship |
| Connector | Ownership |
| ConstantArgument | ConstantValue |
| ConstantArgument | Calculation |
| ConstantArgument | FunctionArgument |
| ConstantValue | Calculation |
| ContingencyElement | Contingency |
| ContingencyGroupMember | Contingency |
| ContingencyGroupMember | ContingencyGroup |
| ControllableLoadResource | EnergyConsumer |
| ControllableLoadResource | HostControlArea |
| ControllableLoadResource | Operatorship |
| ControllableLoadResource | Ownership |
| ControllableLoadResource | EquipmentContainer |
| CurrentRelay | ProtectedSwitch |
| CurrentRelay | Unit |
| CurrentRelay | EquipmentContainer |
| CurrentRelay | Operatorship |
| CurrentRelay | Ownership |
| CurveData | Curve |
| CustomerLoad | ConformLoadGroup |
| CustomerLoad | EquipmentContainer |
| CustomerLoad | Operatorship |
| CustomerLoad | Ownership |
| Disconnector | EquipmentContainer |
| Disconnector | Operatorship |
| Disconnector | Ownership |
| Discrete | MeasurementType |
| Discrete | Operatorship |
| Discrete | Ownership |
| Discrete | PowerSystemResource |
| Discrete | Terminal |
| Discrete | Unit |
| DiscreteValue | ConversionTable |
| DiscreteValue | Discrete |
| DiscreteValue | IccpQualityConversionSet |
| ElectricalBus | ConnectivityNode |
| EndCap | EquipmentContainer |
| EndCap | Operatorship |
| EndCap | Ownership |
| EnergyConsumerAltConnection | EnergyConsumer |
| EnergyConsumerResource | Operatorship |
| EnergyConsumerResource | Ownership |
| EPSMeter | GeneratingUnit |
| EPSMeter | PricingVector |
| FlowgateElement | ConductingEquipment |
| FlowgateElement | FlowgateGroup |
| FlowgateElement | Terminal |
| FlowgateGroup | MonitoredGroup |
| FunctionArgument | Function |
| Gate | TriggerCondition |
| GateInputPin | Gate |
| HostControlArea | Operatorship |
| HostControlArea | Ownership |
| HUBBus | ConnectivityNodeGroup |
| HUBBus | SettlementHUB |
| HydroGeneratingUnit | ResourceNode |
| HydroGeneratingUnit | EquipmentContainer |
| HydroGeneratingUnit | Operatorship |
| HydroGeneratingUnit | Ownership |
| InductionMotorLoad | NonConformLoadGroup |
| InductionMotorLoad | EquipmentContainer |
| InductionMotorLoad | Operatorship |
| InductionMotorLoad | Ownership |
| IrregularTimePoint | IrregularIntervalSchedule |
| Junction | Operatorship |
| Line | SubGeographicalRegion |
| Load | ConformLoadGroup |
| Load | LoadArea |
| Load | LoadType |
| Load | EquipmentContainer |
| Load | Operatorship |
| Load | Ownership |
| LoadResource | EnergyConsumer |
| LoadResource | Operatorship |
| LoadResource | Ownership |
| LoadResource | EquipmentContainer |
| LogicalConfiguration | CombinedCyclePlant |
| Measurement | Unit |
| MeasurementArgument | MeasurementValue |
| MeasurementArgument | Calculation |
| MeasurementArgument | FunctionArgument |
| MeasurementGroup | MeasurementLocation |
| MeasurementLocation | PermissionArea |
| MeasurementLocation | PowerSystemResource |
| Monitored | MonitoredGroup |
| MonitoredRating | MonitoredGroup |
| MVArCapabilityCurve | SynchronousMachine |
| NonConformLoadGroup | SubLoadArea |
| NonConformLoadSchedule | NonConformLoadGroup |
| NonConformLoadSchedule | DayType |
| NonConformLoadSchedule | Season |
| NuclearGeneratingUnit | ResourceNode |
| NuclearGeneratingUnit | EquipmentContainer |
| NuclearGeneratingUnit | Operatorship |
| NuclearGeneratingUnit | Ownership |
| Operatorship | Company |
| OwnerShareLimits | Operatorship |
| OwnerShareLimits | Ownership |
| Ownership | Company |
| PhysicalTiePoint | Terminal |
| PhysicalTiePoint | Operatorship |
| PhysicalTiePoint | Ownership |
| PinBranchGroup | BranchGroup |
| PinBranchGroup | Gate |
| PinFlow | Terminal |
| PinFlow | Gate |
| PinGate | Gate |
| PinSwitch | Switch |
| PinTriggerCondition | TriggerCondition |
| PinVoltage | ConnectivityNode |
| PinVoltage | Gate |
| PlanningBay | VoltageLevel |
| PlanningZone | PlanningZoneGroup |
| Plant | Operatorship |
| Plant | Ownership |
| PowerCutZone | Operatorship |
| PowerCutZone | Ownership |
| PowerTransformer | EquipmentContainer |
| PowerTransformer | EquipmentType |
| PowerTransformer | Operatorship |
| PowerTransformer | Ownership |
| PricingVector | ElectricalBus |
| PrimeMover | Operatorship |
| PrimeMover | Ownership |
| ProtActSeriesCompensator | SeriesCompensator |
| ProtActSwitch | Switch |
| ProtActSynchronousMachine | SynchronousMachine |
| ProtActTransformerWinding | TransformerWinding |
| ProtectedSwitch | EquipmentContainer |
| ProtectedSwitch | EquipmentType |
| ProtectedSwitch | Operatorship |
| ProtectedSwitch | Ownership |
| ProtectionEquipment | EquipmentContainer |
| ProtectionEquipment | EquipmentType |
| ProtectionEquipment | Operatorship |
| ProtectionEquipment | Ownership |
| Rating | OwnerShareRating |
| RegularTimePoint | RegularIntervalSchedule |
| RegulatingCondEq | RegulationSchedule |
| RegulatingCondEq | Terminal |
| RegulatingCondEq | EquipmentContainer |
| RegulatingCondEq | EquipmentType |
| RegulatingCondEq | Operatorship |
| RegulatingCondEq | Ownership |
| RegulationSchedule | DayType |
| RemedialActionScheme | Contingency |
| RemoteUnit | CommunicationLink |
| ResourceController | Operatorship |
| ResourceController | Ownership |
| ResourceNode | ElectricalBus |
| SeasonDayTypeSchedule | DayType |
| SeriesCompensator | EquipmentContainer |
| SeriesCompensator | EquipmentType |
| SeriesCompensator | Operatorship |
| SeriesCompensator | Ownership |
| SettlementLoadZone | Substation |
| ShuntCompensator | Switch |
| ShuntCompensator | Terminal |
| ShuntCompensator | EquipmentContainer |
| ShuntCompensator | EquipmentType |
| ShuntCompensator | Operatorship |
| ShuntCompensator | Ownership |
| Stage | RemedialActionScheme |
| StageTrigger | Stage |
| StageTrigger | TriggerCondition |
| StaticVarCompensator | Terminal |
| StaticVarCompensator | EquipmentContainer |
| StaticVarCompensator | EquipmentType |
| StaticVarCompensator | Operatorship |
| StaticVarCompensator | Ownership |
| StationSupply | EquipmentContainer |
| StationSupply | EquipmentType |
| StationSupply | Operatorship |
| StationSupply | Ownership |
| StringMeasurement | Unit |
| StringMeasurementValue | PermissionArea |
| SubGeographicalRegion | GeographicalRegion |
| SubGeographicalRegion | HostControlArea |
| Substation | SubGeographicalRegion |
| Substation | Operatorship |
| Substation | Ownership |
| SynchronousMachine | GeneratingUnit |
| SynchronousMachine | MVArCapabilityCurve |
| SynchronousMachine | EquipmentContainer |
| SynchronousMachine | EquipmentType |
| SynchronousMachine | Operatorship |
| SynchronousMachine | Ownership |
| TapChanger | LoadBasedRegulationCurve |
| TapChanger | Terminal |
| TapChanger | TransformerWinding |
| TapChanger | Operatorship |
| TapChanger | Ownership |
| TapSchedule | DayType |
| Terminal | ConductingEquipment |
| Terminal | ConnectivityNode |
| ThermalGeneratingUnit | CombinedCyclePlant |
| ThermalGeneratingUnit | ResourceNode |
| ThermalGeneratingUnit | EquipmentContainer |
| ThermalGeneratingUnit | EquipmentType |
| ThermalGeneratingUnit | Operatorship |
| ThermalGeneratingUnit | Ownership |
| TieCorridor | HostControlArea |
| TieEnd | Operatorship |
| TieEnd | Ownership |
| TieLine | Terminal |
| TieLine | TieCorridor |
| TransformerKluge | PowerTransformer |
| TransformerWinding | PowerTransformer |
| TransformerWinding | BaseVoltage |
| TransformerWinding | EquipmentContainer |
| TransformerWinding | EquipmentType |
| TransformerWinding | Operatorship |
| TransformerWinding | Ownership |
| TransmissionPath | ServicePoint |
| TransmissionPath | ServicePoint |
| TransState | Configuration |
| VLSBusBarMonitored | MonitoredGroup |
| VoltageLevel | BaseVoltage |
| VoltageLevel | Substation |
| VoltageLevel | Operatorship |
| VoltageLevel | Ownership |
| VoltageMonitored | MonitoredGroup |
| WindGeneratingUnit | ResourceNode |
| WindGeneratingUnit | EquipmentContainer |
| WindGeneratingUnit | EquipmentType |
| WindGeneratingUnit | Operatorship |
| WindGeneratingUnit | Ownership |

# Appendix C - Validation Rules – COMPLEX rules

This appendix lists all of the validation rules that are more complex than required fields and range checks. These include validation rules that are conditional based upon certain conditions, involve associations with other attributes, and those that describe specific data relationships. The validation rule is explained descriptively and not in a formal language.

| Rule No. | Validation Rule Description |
| --- | --- |
| 1 | The abstract classes GeneratingUnit, Switch, EnergyConsumer,EnergyCosumerResource etc should not be used. Only the classes representing the actual classes should be used. For ex: Only instances of ThermalGeneratingUnit, NuclearGeneratingUnit, HydroGeneratingUnit or WindGeneratingUnit should be created instead of the generic GeneratingUnit Class |
| 2 | Each PowerTransformer and its associated TransformerWindings and TapChangers must be contained within one substation. |
| 3 | A PowerTransformer must be contained by a Substation. A TransformerWinding must be contained by a PowerTransformer. A TapChanger must be contained by a TransformerWinding( Check for associations between transformer, winding and tap. |
| 4 | A Measurement must be associated with a PowerSystemResource to convey containment information for the Measurement. |
| 5 | Transmission line measurements should be associated with an ACLineSegment or SeriesCompensator not with a Line. Transmission line measurements must be associated with a terminal. |
| 6 | Transformer measurements should be associated with a PowerTransformer. Transformer Windings should not have measurements. |
| 7 | A Measurement can be associated with at most one Terminal. |
| 8 | Combined Cycle configuration names for a given combined cycle plant should be unique |
| 9 | ConnectivityNodes may only be placed within ConnectivityNodeGroup. |
| 10 | SynchronousMachine's minimumMVAr and maximumMVAr attributes and the minimum/maximum MVAr value specified in the associated MVArCapabilityCurve data should match. |
| 11 | Classes associated to BaseVoltage should be unique |
| 12 | Terminals connected to the same piece of Equipment can not be connected to the same connectivitynode |
| 13 | Voltage Levels can't be empty. |
| 14 | The normal and neutral tap of the transformer must be within low and high steps |
| 15 | TapChanger master associations can't point to each other. |
| 16 | Winding's associated basevoltage must be equal to connectivitynodes base voltage |
| 17 | Connectivity nodes must be associated to atl east 2 terminals. |
| 18 | Terminal associated to a any given equipment should have unique names. |
| 19 | ACLinesegment cannot go between different voltage levels. |
| 20 | Seriescompensator cannot go between different voltage levels and should be contained within one substation |
| 21 | The regulated terminal of a shuntcompensator must be in the same substation as the shuntcompensator's terminal (RegulatingCondEq->Terminal) |
| 22 | A shuntcompensator's SwitchedBy switch's terminal must be connected to shuntcompensator's terminal through a connectivitynode. |
| 23 | Limits must follow nested limit rules. Limit1 <= Limit2 |
| 24 | Sum of Ownership.ownershippercent should total 100% for SplitGenerationUnits(SGR). An SGR will have 1 record in one of the GeneratingUnit classes and "n" number of OwnerShareLimits that are associated to that GeneratingUnit. Each OwnerShareLimit will point to a OwnerShip record. The sum of all these ownership.ownershippercent should be equal to 100. |
| 25 | DCTie Psuedo Load breaker default value must be "Closed". |
| 26 | BLT(BlockLoadTransfer) must only group loads and only zero or one psuedo GeneratingUnit per BLT. The breaker for this psuedo gen if modeleld must have a default value of OPEN. |
| 27 | HubBus must have atleast one electrical bus. |
| 28 | The ConnectivityNodes in a ConnectivityNodeGroup must be in the SAME STATION i.e. ConnectivityNodes in a given ConnectivityNodeGroup cannot span multiple Stations. |
| 29 | If a HubBus is associated to more than 1 ConnectivityNodeGroup then ALL the associated ConnectivityNodeGroups must belong to the same station. |
| 30 | SynchronousMachine's 'ratedMVA' attribute should be greater than or equal to the 'baseMW' attribute of the GeneratingUnit it is associated to. |
| 31 | Check the baseVoltage (if not null) of each conducting equipment connected to a connectivityNode to be the same. |
| 32 | Only AnalogLimit instances that are children of an AnalogLimitSet instance with the LimitSet.forNetwork = False/Null should have the LimitAlarmCategory and AnalogLimitName associations populated. OR If the parent AnalogLimitSet instance’s the LimitSet.forNetwork = True of if the AnalogLimit instance’s parent is a Rating instance, the two associations do not need to be populated. |
| 33 | The energy consumer that is part of the DCTie must have ONE and ONLY one controllable load resource mapped to it. This energyconsumer can NOT be associated to a "non-controllable load resource". It has to be associated only to "controllable load resource". |
| 34 | Only AnalogLimit instances that are children of an AnalogLimitSet instance with the LimitSet.forNetwork = False/Null should have the lower and upper thresholds |
| 35 | Substation names should not exceed 8 chars. |
| 36 | The target voltage setting specified by the regulation schedules must be within 80% to 120% of the rated kV |
| 37 | The number of CurveData associated with the MVArCapabilityCurves and RampRateCurve should not exceed 10 |
| 38 | Two RampRateCurves (one with Emergency = True) should be associated with a ControllableLoadResource, a Configuration and a ThermalGeneratingUnit / HydroGeneratingUnit / NuclearGeneratingUnit / WindGeneratingUnit which has no association to a DCTie and a PrivateAreaNetwork (NetworkType: Non Modeled Generation, Mothballed, Retired) or a CombinedCyclePlant. |
| 39 | The FromConfig and ToConfig of a TransState shouldn't be same |
| 40 | FromConfig and ToConfig of a TransState must belong to the same CombinedCyclePlant |
| 41 | A Contingency can’t contain duplicate ContingencyElements |
| 42 | The controllable load resource must be associated with exactly two RampRateCurves, one for normal condition and the other for emergency condition |
| 43 | There is no duplicated CC configuration, i.e. there should not be 2 CC configurations that contain the same CCU combination with exactly same status of Primary/Alternate flag |
| 44 | Each CC plant must have at least one CC configuration to startup/shutdown |
| 45 | If a ThermalGeneratingUnit is a unit in a CombinedCyclePlant, then it cannot be a SplitGenerationResource. |
| 46 | If any Measurement instance has more than one (MeasurementValue instance that is associted with MeasurementValueSource instance) then only one can have Company != ERCOT |
| 47 | Each CustomerLoad instance should be associated with One instance of SettlementNOIELoadZone, Or One instance of DCTie, Or Its parent Substation should be associated with one instance of SettlementLoadZone |
| 48 | For any instance of MeasurementArgument with ReferenceFlag = True, the Calculation link on the associated MeasurementValue instance should not be same as the parent Calculation instance of the MeasurementArgument instance. |
| 49 | MeasurementValue instances that are sourced from or output of a calculation should have its Primary Site as “DE”. |
| 50 | Every ElectricalBus should be in a load zone or a NOIE load zone |
| 51 | PsuedoTerminal shoud not be Associated to a connectivityNode |
| 52 | Psuedoterminal should not be associated to a RegulatingconductingEquipment |
| 53 | EnergyConsumerResource(LoadResource and ControllableLoadResource) should not have any association to ResourceNode |
| 54 | If PAN.networktype = retired or mothbolled, then there should be only one genunit associated to it. |
| 55 | x/r ratio for ACLineSegment should follow these rules. For 345kv x/r > 7, 138kv x/r> 1.5, 69kv x/r > 1 |
| 56 | A resourceNode that is associated to a PrivateUseNetwork(PUN), should not be associated to any Physical Generating Unit |
| 57 | Every GeneratingUnit(Thermal, Hydro, Nuclear, Wind) should have an association to a ResourceNode unless the GeneratingUnit is associated to a DCTie, NonModelledGeneration or BlockLoadType resource. |
| 58 | If a Load is assigned to a NOIE load zone, then its electrical bus can ONLY have loads that are assigned to the same NOIE load zone, i.e. cannot have other loads at the same electrical bus assigned to different load zone In other words, all loads that connect to the same EB must belong to the same load zone. All those loads must either be associated to the same NOIE load zone, or none of them has this association. |
| 59 | DCTIE ElectricalBus can ONLY connect to DCTIE load. The EB connecting to terminal of DCTIE pseudo load (DCTIE attribute of CUSTOMERLOAD instance) cannot have other loads connect to it |
| 60 | Each CombinedCyclePlant Configuration instance should have at least one association to a ConfigurationMember instance |
| 61 | PricingVector's should have a unique name |
| 62 | MeasurementGroup and Calculation instances associated to a Zonal MeasurementLocation should have unique names. Similarly MeasurementGroup and Calculation instances associated to a Nodal MeasurementLocation should have unique names |
| 63 | Measurementargument and ConstantArgument instances associated to a Calculation should have unique names |
| 64 | Zonal Measurements associated to the same terminal need a unique name. |
| 65 | Nodal Measurements associated to the same terminal need a unique name. |
| 66 | At least one Ownersharelimits instance should be associated with every LoadResource, ControllableLoadResource, ThermalGeneratingUnit, HydroGeneratingUnit, WindGeneratingUnit, NuclearGeneratingUnit, |
| 67 | Every zonal AnalogLimitSet should be associated with a zonal analog - Every Nodal AnalogLimitSet should be associated with Nodal Analog. |
| 68 | Every Zonal AnalogValue with no association to a AnalogName should be associated with a zonal MeasurementValueQuality. Every Nodal AnalogValue with no association to a AnalogName should be associated with a Nonal MeasurementValueQuality. |
| 69 | Every zonal AnalogValue with no association to a AnalogName should be associated with a zonal MeasurementValueSource - Every Nodal AnalogValue with no association to a AnalogName should be associated with a Nodal MeasurementValueSource |
| 70 | Every zonal AnalogValue with association to a AnalogName should be associated with a zonal site and should be associated to atleast one zonal MeasurementGroup or one zonal Calculation - Every Nodal AnalogValue with association to a AnalogName should be associated with a zonal site and should be associated to atleast one Nodal MeasurementGroup or one Nodal Calculation |
| 71 | Every zonal DiscreteValue with association to a DiscreteName should be associated with a zonal site and should be associated to atleast one zonal MeasurementGroup or one zonal Calculation - Every Nodal discreteValue with association to a discreteName should be associated with a zonal site and should be associated to atleast one Nodal MeasurementGroup or one Nodal Calculation |
| 72 | Every zonal Analog should be associated with a zonal AnalogValue - Every Nodal Analog should be associated with a Nodal AnalogValue |
| 73 | Every zonal AnalogLimit should be associated with a zonal AnalogLimitSet - Every Nodal AnalogLimit should be associated with a Nodal AnalogLimitSet. Check for only AnalogLimit's with association to AnalogLimitName |
| 74 | Every zonal MeasurementArgument and ConstantArgument should be associated with a zonal Calculation - Every Nodal MeasurementArgument and ConstantArgument should be associated with a Nodal Calculation |
| 75 | Every zonal ConstantValue should be associated with a zonal ConstantArgument - Every Nodal ConstantValue should be associated with a Nodal ConstantArgument |
| 76 | Every configuration should either be able to StartUp or Shutdown or transition into another configuration, else it is invalid. |
| 77 | Attributes, HightReasonabilityLimit, LowReasonabilityLimit and MaxWeeklyenergy should be populated for any ownersharelimits associated with a LoadResource, a ControllableLoadResource, and a ThermalGeneratingUnit / HydroGeneratingUnit / NuclearGeneratingUnit / WindGeneratingUnit which has no association to a DCTie and a PrivateAreaNetwork (NetworkType: Non Modeled Generation Retired, Mothballed, BlockLoadTransfer). |
| 78 | Attributes (Operational Resource Parameters), MinOnlineTime, minOfflineTime, hotstartTime, IntStartTime, ColdStartTime, Maxweeklystarts, maxonlinetime, maxdailystarts, maxweeklystarts, hotinttime, intcoldtime, syscondensercapable should be populated for any Configuration and any ThermalGeneratingUnit / HydroGeneratingUnit / NuclearGeneratingUnit / WindGeneratingUnit which has no association to a CombinedCyclePlant, a DCTie and a PrivateAreaNetwork (NetworkType: Non Modeled Generation, Retired, Mothballed, BlockLoadTransfer) . |
| 79 | Attributes (Resource Parameters), HighReasonabilityLimit, LowReasonabilityLimit, HighReasonabilityRampRateLimit, LowReasonabilityRampRateLimit, MaxSustainableRating, MinSustainableRating, MaxEmergencyRating, MinEmergencyRating should be populated for any Configuration and any ThermalGeneratingUnit / HydroGeneratingUnit / NuclearGeneratingUnit / WindGeneratingUnit which has no association to a DCTie and a PrivateAreaNetwork (NetworkType: Non Modeled Generation, Retired, Mothballed, BlockLoadTransfer). |
| 80 | Each Configuration must have at least one PRIMARY CCU (defined as CONFIGURATIONMEMBER with PRIMARY flag = ‘TRUE’ ) |
| 81 | Each CombinedCyclePlant must have at least one configuration that can start up (i.e. STARTUPFLAG = ‘TRUE’) |
| 82 | Each CombinedCyclePlant must have at least one configuration that can shut down (i.e. SHUTDOWNFLAG = ‘TRUE’) |
| 83 | Each Configuration must satisfy at least one of the two conditions a) have at least one in-transstate (i.e. transstate with that configuration as TOCONFIG_CONFIGURATION) or b) its STARTUPFLAG is set to ‘True’ |
| 84 | Each Configuration must have at least one of the two conditions a) have at least one out-transstate (i.e. transstate with that configuration as FROMCONFIG_CONFIGURATION) or b) its SHUTDNFLAG is set to ‘True’ |
| 85 | All Configurations must have path to transit from start-up configuration AND transit to shut-down figuration in some way. This can be direct transstate or through other configurations. This is to prevent CC Plant being trapped ONLINE or OFFLINE |
| 86 | Validate the UpTransition Attribute on a TransState: If UpTransition = True, count of primary members in the FromConfiguration < count of primary members in the ToConfiguration and if UpTransition = False, count of primary members in the FromConfiguration > count of primary members in the ToConfiguration. |
| 87 | For both TransState_ToConfig and TransState_FromConfig association, loosen the cardinalityA to "ZeroToMany" instead of "OnetoMany" in light of the “valid” configuration definition (or points 5 and 6 in the email below) |
| 88 | Every Zonal DiscreteValue with no association to a DiscreteName should be associated with a zonal MeasurementValueQuality. Every Nodal DiscreteValue with no association to a DiscreteName should be associated with a Nonal MeasurementValueQuality. |
| 89 | Every zonal DiscreteValue with no association to a DiscreteName should be associated with a zonal MeasurementValueSource - Every Nodal DiscreteValue with no association to a DiscreteName should be associated with a Nodal MeasurementValueSource |
| 90 | All x_axisdata data points in a MVArCapabilityCurve should be unique |
| 91 | Only one y1_axisdata point in a MVArCapabilityCurve can be 0. |
| 92 | Only one y2_axisdata point in a MVArCapabilityCurve can be 0. |
| 93 | For every ICCPQualityConversion instance, a. Only One amongst ICalculated, IManual, IEstimated, ICalculated should be true, and b. Only One amongst IValid, INotValid, ISuspect, IHeld should be true, and c. Only One amongst OValid, ONotValid, OSuspect, OHeld should be true, and d. Only One amongst OCalculated, OManual, OEstimated, OCalculated should be true. |
| 94 | There should be atleast one swing bus in the model. i.e. atleast one connectivity node with SlackBusPriority = True |
| 95 | There should be no Pricingvector associated to a BusbarSection or a Endcap. |
| 96 | For an EnercyConsumer (CustomerLoad or StationSupply), if conformingLoadFlag = True, then the EnergyConsumer should be associated to a ConformLoadGroup |
| 97 | For every MeasurementGroup, the associated AnalogValue and DiscreteValue instances should be associated to the same terminal. For example 1. MeasGroup1 -> AnalogValue1 –> Analog1–> Terminal1 2. MeasGroup1 -> AnalogValue2 –> Analog2–> Terminal1 |
| 98 | For every Calculation, when associated with more than one MeasurementArgument with referenceFlag = Flag, the measurementvalues associated to such MeasurementArguments should be linked to the same terminal. For example 1. Calc1 -> MeasArg1 (ref = False) ->AnalogValue1 –> Analog1–> Terminal1 2. Calc1 -> MeasArg2 (ref = False) -> AnalogValue2 –> Analog2–> Terminal1 |
| 99 | When a MeasurementLocation has more than one children (MeasurementGroup or Calculation), (All analogvalues and discretevalues associated to the child MeasurementGroup) and (All Analogvalues and discretevalues associated with measurementargument having referencefalg = False within the child calculation) should be associated to the same terminal. For example 1. MeasLoc1 -> MeasGrp1 -> AnalogValue1 ->Analog1 ->Terminal1 2. MeasLoc1 -> MeasGrp1 -> AnalogValue2 ->Analog2->Terminal1 3. MeasLoc1 -> Calc1 -> MeasArg1 (referenceflag = false) -> AnalogValue3 ->Analog3 ->Terminal1 4. MeasLoc1 -> Calc1 -> MeasArg1 (referenceflag = false) -> AnalogValue4 ->Analog4->Terminal1 |

# Appendix D – CLASSES modeled According to STANDARD

This appendix lists all of the classes that are modeled with the standard CIM approach.

# Appendix E – COMPLETE LIST OF CLASSES NOT USED

This appendix lists all of the CIM classes in the ERCOT CIM model that are not used.

# Appendix F – COMPLETE LIST OF CLASSES NOT ALLOWED

This appendix lists all of the CIM classes in the ERCOT CIM model that are not allowed.