| NPRR Number | 141 | NPRR Title | TSP Energy Storage for Reliability (formerly “TSP and/or DSP Energy Storage for Reliability”) |
| --- | --- | --- | --- |
| Date |  | May 28, 2009 |  |
| Submitter’s Information |  |  |  |
| Name |  | Jonathan Levine |  |
| E-mail Address |  | jlevine@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | (512) 248-6464 |  |
| Cell Number |  | (860) 539-7269 |  |
| Market Segment |  | Not applicable |  |

Comments

| Overall Market Benefit |  |
| --- | --- |
| Overall Market Impact |  |
| Consumer Impact |  |

ERCOT proposes the following revision to improve the structure of the proposed additional language.  This proposed revision does not substantively change the proposed additional language and has been reviewed and approved by the submitter of Nodal Protocol Revision Request (NPRR) 141.

Revised Proposed Protocol Language

10.3.3	TSP or DSP Metered Entities

10.3.3.1	Data Responsibilities

Each TSP and DSP shall be responsible for the following:

(a)	Providing consumption data for each ESI ID and RID on a monthly basis according to the data timeliness and accuracy standards defined in this Section and in the SMOG;

(b)	Providing start date, stop date, ESI ID or RID, and consumption data in kWh as well as an identifier for “estimated” reads as applicable;

(c)	Submitting a single Demand value for each non-IDR ESI ID that has a demand register to ERCOT if, and only if, a Demand value is required for TSP or DSP tariffs or for CR Customer billing.  If the CR and TSP or DSP do not require a Demand value, then the TSP or DSP shall not submit a Demand value to ERCOT even if the meter has a demand register;

(d)	Validating, Editing, and Estimating (VEE) meter data according to the standards in this Section before submitting data to the settlement process;

(e)	Calculating consumption for any unmetered services by ESI ID and submitting such data monthly to ERCOT, subject to ERCOT audit.  These calculations must be made pursuant to TSP and DSP-approved tariffs; and,

(f)	Metering all Loads, unless the Load meets one of the following criteria:

(i)	Energy consumption by substation Facilities and equipment for the purpose of transporting electricity (e.g., substation transformers, fans, etc.);

(ii)	Unmetered energy consumption represented by an ERCOT-approved Load Profile; or

(iii)	Energy charge and discharge and associated losses for the ERCOT Board-approved storage devices installed as part of a transmission reliability project for the Presidio substation Facilities.