| NPRR Number | 075 | NPRR Title | Advanced Meter Interval Data File Format and Submission |
| --- | --- | --- | --- |
| Date |  | May 12, 2008 |  |
| Submitter’s Information |  |  |  |
| Name |  | Chuck Moore on behalf of the Commercial Operations Subcommittee (COPS) |  |
| E-mail Address |  | chuck.moore@directenergy.com |  |
| Company |  | Direct Energy |  |
| Phone Number |  | 512-320-7915 |  |
| Cell Number |  | 512-627-6114 |  |
| Market Segment |  |  |  |

Comments

On May 12, COPS endorsed RMGRR075 with ERCOT comments. COPS also proposes grammatical changes and supports TDSPs on discontinuing posting interval data to their FTP sites after functionality is available on the Common Web Portal for CRs to retrieve. COPS discussed the current 12pm requirement for TDSPs having their settlement quality interval data posted to the FTP site, the TDSP proposed 11pm requirement and Reliant’s 70% proposed requirement.  COPS discussed changing the timeline from 12pm to 5pm, but did not make a motion for that change.  COPS defers to the expertise at RMS regarding this timeline.

.

| Overall Market Benefit |  |
| --- | --- |
| Overall Market Impact |  |
| Consumer Impact |  |

Revised Proposed Guide Language

7.15        Advanced Meter Interval Data File Format and Submission

7.15.1          Ad Hoc Connectivity Test of Advanced Metering System (AMS) Interval Data

Transmission Distribution Service Providers (TDSPs) will contact the ERCOT Flight Test Administrator to perform an ad hoc connectivity test with ERCOT to ensure that they can successfully send and ERCOT receive the ERCOT Specified File Format (Section 9, Appendix F7, File Layout for ERCOT Specified File Format).  ERCOT will send a response to the submitting TDSP via North American Energy Standards Board (NAESB).  The acceptable file layout can be found in Section 9, Appendix F7.

7.15.2         Submission of Interval Data on ESI ID(s) with AMS Meters

All TDSPs shall submit fifteen (15) minute settlement quality interval data to ERCOT daily for provisioned Advanced Metering System (AMS) meters.  Each file shall contain up to, but not to exceed, 50,000 data records.  For optimum processing at ERCOT, it is suggested that the file contain a minimum of 10,000 data records.  Files shall be zipped prior to Pretty Good Privacy (PGP) encryption and compression.  See Section 9, Appendix F7, File Layout for ERCOT Specified File Format.

The recommended file naming convention is <DUNS><ReportName><DateTime><Counter>.lse <.optional data>in addition to any application file naming conventions used in transmitting the file.  For example, “999999999IntervalData20081227113001123.lse<.optional data>” where:

| Element | Explanation | Format |
| --- | --- | --- |
| DUNS | TDSP DUNS Number | Numeric (9 or 13) |
| ReportName | ‘IntervalData’ | Alphanumeric (12) |
| DateTime | File transmission date/time stamp | Datetime format = ccyymmddhhmmss |
| Counter | Counter with no specified value | Numeric (3) |
| .lse | Value of .lse in file extension |  |
| <.optional data> | Any optional data, if necessary | Cannot contain csv |

At a minimum the filename must contain .lse after decryption otherwise the file will be rejected by ERCOT.  The filename cannot contain .csv after decryption.  Files will be sent with a NAESB input-format of “FF”.  ERCOT will send a response to the submitting TDSP via NAESB indicating receipt of the file.

After receipt, ERCOT will validate that all mandatory data elements are present and meet formatting requirements.  ERCOT will inform the submitting TDSP of the success or failure of their file via the Interval Data LSE Activity Report.  The layout of this report can be found on the ERCOT website.

7.15.2.1	Missing Data or Gaps in Data

TDSPs will provide estimated data for any missing data or gaps in the interval data on a provisioned AMS meter prior to posting the file to the TDSP’s File Transfer Protocol (FTP) site or sending the file to ERCOT.

7.15.3  	Posting Data to TDSP FTP Site

TDSPs will provide on their FTP site, settlement quality interval data no later than 12:00 PM CPT of the next calendar day using the ERCOT specified file format (Section 9, Appendix F7, - File Layout for ERCOT Specified File Format) for each Electric Service Identifier (ESI ID) with a provisioned AMS meter.  Competitive Retailers (CRs) will access the TDSP's FTP site to retrieve the daily fifteen (15) minute interval data associated with a provisioned AMS meter for their ESI IDs.

TDSPs will discontinue posting interval data to their FTP sites after this functionality is available on the Common Web Portal for CRs to retrieve.

7.15.4     Availability of Interval Data for Provisioned AMS Meters

CRs will access ERCOT’s Market Information System (MIS) for interval data for their ESI IDs to allow them to shadow settle.  CR disputes or disagreements of interval data obtained from the AMS provisioned meter should be based on the data used by ERCOT in settlement and not the data provided to CRs on the TDSP’s FTP Site.

TDSPs will retain the daily interval data on their FTP site for ESI ID(s) with a provisioned AMS meter for ten (10) calendar days from the date that the file was initially posted to the FTP site.

Appendix F7

File Layout for ERCOT Specified File Format

(Same file layout used for TDSP’s File Transfer Protocol (FTP) site)

Note:  The correct number of commas must be included even if optional elements are not provided.

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row One |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000001 | Must be 00000001 | Numeric (8) |
| 2 | ESI ID | Mandatory |  |  | Alphanumeric (64) |
| 3 | Channel | Mandatory | 1 4 | Indicates type of data contained in the detailed rows. 1 = generation 4 = load | Numeric (1) |
| 4 | Start Time | Mandatory |  |  | YYYYMMDDHHMMSS (24-hour) (14) |
| 5 | Stop Time | Mandatory |  |  | YYYYMMDDHHMMSS (24-hour) (14) |
| 6 | DST Participation | Mandatory | Y | Y = DST Participant Data must be DST adjusted and must be in local prevailing time | Alphanumeric (1) |
| 7 | Invalid Record Flag | Mandatory | N | N indicates that the data is VEE and does not need to be pre-validated. | Alphanumeric (1) |

Note:  The correct number of commas must be included even if optional elements are not provided.

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row Two |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000002 | Must be 00000002 | Numeric (8) |
| 2 | Meter Start Reading | Optional |  | Meter Start Reading Not used by ERCOT Default = 0 | Non-negative numeric Max = 999999999999999.9999 00:00:00 |
| 3 | Meter Stop Reading – Register Read for the day | Optional |  | Meter Stop Reading Not used by ERCOT Default = 0 | Non-negative numeric Max = 999999999999999.9999 23:59:59 |
| 4 | Meter Multiplier | Conditional |  | Must be included if meter start and meter stop is included. Not used by ERCOT Default = 0 | Non-negative numeric Max = 999999999999999.9999 |
| 5 | Empty value | Mandatory | No value provided | Must be Null. See example. | The correct number of commas must be included. |
| 6 | Pulse multiplier | Optional |  | Not used by ERCOT Default = 0 | Non-negative numeric Max = 999999999999999.9999 |
| 7 | Empty value | Mandatory | No value provided | Must be Null. See example. | The correct number of commas must be included. |
| 8 | Seconds Per Interval | Mandatory | 900 | 900 = 15 minute intervals | Numeric (3) |
| 9 | Lodestar Unit of Measure | Mandatory | 01 | 01 = kWh | Numeric (2) |
| 10 | Basic Unit Code | Optional |  | Not used by ERCOT Default = 1 | Positive numeric. Max 9999. |
| 11 | Time Zones West of GMT | Optional |  | Not used by ERCOT Default = -1 | Numeric. Min = -1 Max = 47 |
| 12 | Population | Optional |  | Not used by ERCOT Default = 0.0 | Positive numeric. Max = 999999999999999.9999 |
| 13 | Weight | Optional |  | Not used by ERCOT Default = 0.0 | Max = 999999999999999.9999 |
| 14 | Time Zone Standard Name | Mandatory | CST | Not used by ERCOT | Alphanumeric (3) |

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row Three |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000003 | Must be 00000003 | Numeric (8) |
| 2 | Descriptor | Mandatory |  | Unique tran id | Alphanumeric (80) |

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row Four |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000004 | Must be 00000004 | Numeric (8) |
| 2 | Timestamp | Mandatory |  | Timestamp of read. This value will determine which read will ‘win’ for a day if there are multiple reads. | YYYYMMDDHHMMSS |
| 3 | Origin | Mandatory | M | M=Metered | Alphanumeric (1) |

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Header Row Thirty |  |  |  |  |  |
| 1 | Sort code | Mandatory | 00000030 | Must be 00000030 | Numeric (8) |
| 2 | Name Value Pairs | Mandatory | ATTRIBUTE_VALUE_PAIRS | Must be ATTRIBUTE_VALUE_PAIRS | Alphanumeric (21) |
| 3 | MRE DUNS Number | Mandatory | MRE=<required MRE DUNS> | Insert MRE DUNS Number | MRE= + Numeric (9 or 13) |
| 4 | TDSP DUNS Number | Mandatory | Sender=<required Sender DUNS> | Insert Sender DUNS Number | Sender= + Numeric (9 or 13) |
| 5 | ERCOT DUNS Number | Mandatory | Receiver= 183529049 | Must contain ERCOT’s DUNS number. | Receiver= + Numeric (9) |
| 6 | CR DUNS Number | Attribute is Mandatory Value is optional | REP=<optional CR DUNS> | Insert CR DUNS number The attribute (REP=) is required. The value (the CR DUNS number) is optional. Not Used by ERCOT | REP= + Numeric (9 or 13) |

Note:  The correct number of commas must be included even if optional elements are not provided.

| Element | Description | Mandatory / Optional | Valid Values | Comment | Format |
| --- | --- | --- | --- | --- | --- |
| Detailed Record |  |  |  |  |  |
| 1 | Sort code | Mandatory | 10000000 through 10000024 | Each row must contain four 15 minute interval sets. For a 92 interval day, the data records will go through row 22 (10000022). For a 96 interval day the data records will go through row 23 (10000023). For a 100 interval day will go through row 24 (10000024). | Numeric (8) |
| 2 | Interval value | Mandatory |  |  | Numeric Maximum of 3 significant digits to the right of the decimal. Must be a positive value. |
| 3 | Lodestar Status Code | Mandatory | A = actual E = estimate | Indicates whether the interval is an actual or estimate. | Alphanumeric (1) |
| 4 | Empty value | Mandatory | No value provided | Must be Null. See example. | The correct number of commas must be included. |