| NOGRR Number | 025 | NOGRR Title | Monitoring Programs for QSEs, TSPs, and ERCOT |
| --- | --- | --- | --- |

| Date | June 8, 2009 |
| --- | --- |

| Submitter’s Information |  |
| --- | --- |
| Name | Don Blackburn |
| E-mail Address | don.blackburn@luminant.com |
| Company | Luminant |
| Phone Number | 214-762-6159 |
| Cell Number |  |
| Market Segment | Investor-Owned Utility (IOU) |

Comments

Luminant proposes the language changes below based on discussions at the NOGRR025 Workshop held on June 1, 2009.

Additionally, Luminant withdraws the 3/24/09 Luminant comments.

| Overall Market Benefit |  |
| --- | --- |
| Overall Market Impact |  |
| Consumer Impact |  |

Revised Proposed Guide Language

ERCOT Nodal Operating Guides

Section 9: Monitoring Program

<date approved>

(Effective Upon Texas Nodal Market Implementation)

9	Monitoring Programs	3

9.1	QSE and Resource Monitoring Program	3

9.1.2	QSE Unannounced Testing	3

9.1.3	Reactive Testing	3

9.1.4	Real-Time Data	3

9.1.5	Compliance with Valid Dispatch Instructions	3

9.1.6	Supplying and Validating Data for Generator Models 	3

9.1.7	Resource Outage Scheduling and Reporting	3

9.1.8	Backup Control for Resource Energy Deployment	3

9.1.9	Qualified Staffing Requirement	3

9.1.10	Automatic Voltage Regulator (AVR)	3

9.1.11	Current Operating Plan Metrics for QSEs	3

9.2	TSP Monitoring Program	3

9.2.1	Rating Methodology Reporting Program	3

9..2	Real-Time Data Monitor	3

9.2.1	Outage Scheduling and Reporting	3

9.2.3	Transmission Service Provider (TSP) Network Operations Model Update Implementation Monitor	3

9.2.4	Compliance with Valid Dispatch Instructions Monitoring	3

9.3	ERCOT Monitoring Program	3

9.3.1	Transmission Control	3

9.3.2	Resource Control	3

9.3.3	Load Forecasting	3

9.3.4	System Operating Constraints	3

9.3.5	Network Operations Model Update Implementation Statistics	3

9.3.6	Back-up Control Plan	3

9.3.6	ERCOT Black Start Plan	3

9.3.7	Computer and Communication Systems Real-Time Availability and Systems Security	3

9.3.8	Voltage and Reactive Control Performance Monitoring	3

9.4	Ancillary Services Monitoring Program	3

9.4.1	Regulation Control Performance	3

9.4.2	Hydro Responsive Testing	3

9.4.3	Black Start Service Requirements for TSPs, QSEs, and Generation Resources	3

9.4.4	Resource-Specific Responsive Reserve Performance	3

9.4.5	 Constant Frequency Control	3

9.4.6	Resource-specific Non-Spinning Reserve	3

# Monitoring Programs

9.1	QSE and Resource Monitoring Program

This Section complies with the requirement outlined in Protocol Section 8, Performance Monitoring and Compliance, for the development of a Qualified Scheduling Entity (QSE), Resource, Transmission Service Provider (TSP) and ERCOT monitoring program.  Reports defined in this Section will be posted on the Market Information System (MIS) Secure Area unless otherwise stated.

9.1.2	QSE Unannounced Testing

ERCOT shall produce reports annually which summarize the unannounced test results provided by Qualified Scheduling Entities (QSEs).  The QSE shall provide these results to ERCOT upon completion of its test.

9.1.3	Reactive Testing

(1)	The Qualified Scheduling Entity (QSE) shall provide the following information:

(a)	Unit name;

(b)	QSE;

(c)	Date;

(d)	Time;

(e)	Tested generation capability;

(f)	Reported time; and

(g)	Corrected Unit Reactive Limit (CURL) and Unit Reactive Limit (URL).

(2)	ERCOT shall produce reports annually which describe when the last reactive capability test took place, what the MW and MVAr outputs were during the test period, how long the output was held, and a minimum of four MW/MVAr pairs describing the CURL of the unit.  This data shall be provided by the QSE to ERCOT when reactive testing indicates a change in unit capability and at a minimum of once every two years.

9.1.4	Real-Time Data

ERCOT shall produce reports describing Real-Time data performance of Qualified Scheduling Entities (QSEs) in the following areas.  ERCOT shall post the summary report on the MIS Secure Area.  Individual point performance shall be posted to the MIS Certified Area.

(a)	Telemetry Performance:

(i)	ERCOT shall produce quarterly reports describing general and critically important telemetry performance as defined in the Protocols and the Technical Advisory Committee (TAC) approved Telemetry Standards.

(b)	Communication System Performance:

(i)	ERCOT shall produce monthly reports describing the  reliability of each participants Inter-Control Center Communications Protocol (ICCP) data link to ERCOT as defined in the Protocols and the TAC approved Telemetry Standards.

(ii)	ERCOT shall produce monthly reports describing ICCP link up/down statistics.

(c)	Operational Data Requirements:

(i)	ERCOT shall produce monthly telemetry availability reports for Dynamically Scheduled Resource (DSR) Loads.  A DSR Load telemetry failure greater than five minutes will be recorded as an unavailable period.

9.1.5	Compliance with Valid Dispatch Instructions

ERCOT shall produce monthly reports describing Qualified Scheduling Entity (QSE) and Resource energy performance with respect to ERCOT’s valid Dispatch Instruction over the 15-minute Settlement Interval. The report would specify the Resource energy performance above the ERCOT Dispatch Instruction as a percentage of the Resource’s High Sustained Limit (HSL) and only when system frequency is above the frequency target; and the Resource energy performance below the ERCOT Dispatch Instruction as a percentage of the Resource’s HSL and only when frequency is below the frequency target.  A Resource’s energy performance is the integration of the Net MW output telemetry over the Settlement Interval for that Resource. An ERCOT valid Dispatch Instruction is the amount of energy expected from the Security-Constrained Economic Dispatch (SCED) Base Point instruction adjusted by the Resource’s share of the QSE portfolio Regulation Up (Reg-Up) and Regulation Down (Reg-Down) service instructions

9.1.6	Supplying and Validating Data for Generator Models

(1)	ERCOT shall prepare monthly reports summarizing the Network Operations Model update modeling process.  These reports shall include the

number of Notices of interim updates, delineated by category, submitted to the Public Utility Commission of Texas (PUCT) and the Independent Market Monitor (IMM) as accumulated by Qualified Scheduling Entity (QSE) due to the actions or inactions of the associated QSE.  Interim updates caused by ERCOT software, hardware, or communication infrastructure shall not be attributed to the QSE.

(2)	ERCOT shall post reports on the Market Information System (MIS) Secure Area.

9.1.7	Resource Outage Scheduling and Reporting

This Section describes the reporting of data for Resource Outage scheduling by individual owner or Resource and ERCOT for informational purposes.  There are no performance metrics for this data.

(1)	ERCOT shall post a report of total Outages considered on the Market Information System (MIS) Secure Area including:

(a)	Number of Outage requests submitted in the ERCOT Outage Scheduler greater than 335 days (11 months) in advance;

(b)	Number of Outage requests submitted in the ERCOT Outage Scheduler between 90 and 334 days in advance of the desired Outage date;

(c)	Number of Outage requests submitted in the ERCOT Outage Scheduler between eight and 89 days in advance of the desired Outage date;

(d)	Number of Outage requests submitted in the ERCOT Outage Scheduler between three and seven days in advance of the desired Outage date;

(e)	Number of Outage requests submitted less than three days in advance;

(f)	Number of Outages by Outage type; and

(g)	Total number of Outages that were requested, accepted, approved, cancelled, and withdrawn.

(2)	ERCOT shall post reports showing for each Qualified Scheduling Entity (QSE) the total number of Outages requested and the percentage of the total Outages for the individual owner or Resource by type described in paragraph (1) above on the MIS Secure Area.

9.1.8	Backup Control for Resource Energy Deployment

ERCOT shall produce a report identifying the date of backup control plan testing and the success or failure of that test for ERCOT and Qualified Scheduling Entities (QSEs).  A test would include Real-Time operation.  Results shall be provided by the QSE to ERCOT at the completion of all tests.  Backup control plan tests shall be conducted at least annually.

9.1.9	Qualified Staffing Requirement

A Qualified Scheduling Entity (QSE) shall maintain a 24x7 scheduling center with qualified personnel with the authority to commit and bind the QSE.  ERCOT shall report to the Texas Regional Entity (TRE), instances of suspected non-performance following ERCOT’s investigation.

9.1.10	Automatic Voltage Regulator (AVR)

(1)	ERCOT shall record Qualified Scheduling Entity (QSE) provided Automatic Voltage Regulator (AVR) test reports which will include:

(a) 	The minimum and maximum excitation limiters settings and associated time limits;

(b)	Volts/hertz settings;

(c)	Gain and time constants;

(d)	Date tested; and

(e)	Voltage regulator control setting;

(2)	ERCOT shall produce a monthly report that will identify Resources for which test reports have not been submitted within the last 60 months.

(3)	ERCOT shall produce a monthly report listing the Generation Resources that are not meeting AVR availability for periods in which they are required to provide Voltage Support Service (VSS) as described in paragraph (4) of Protocol Section 3.15.3, QSE Responsibilities Related to Voltage Support.  The AVR criteria are described in paragraph (2) of Section 2.7.4.1, Maintaining System Voltage.

9.1.11	Current Operating Plan Metrics for QSEs

(1)	ERCOT shall report when a seven day Current Operating Plan (COP) has not been provided by the representing QSE for a Resource by 1500 each day.  An event occurs when a QSE has not provided at least 153 hours of a Resources Operating Plan to ERCOT by 1500.  This report shall list Resources with three or more events in a calendar month.  The report will be prepared monthly and posted on the MIS)Secure Area.

(2) 	ERCOT shall report in the Day-Ahead when the reserved capacity of a QSE’s Resources in the Operating Day COP at 1430 is not sufficient to supply Ancillary Service requirements for the upcoming Operating Day, including regulation, Non-Spinning Reserve (Non-spin), and Responsive Reserve (RRS).  ERCOT shall provide a monthly summary of the total hours failed and total hours analyzed when the Resource’s reserves are insufficient to supply for more than three hours during an Operating Day pursuant to paragraph (2) of Protocol Section 8.1.2, Current Operating Plan (COP) Performance Requirements and not excused due to exemptions contained in the Protocols.

9.2	TSP Monitoring Program

9.2.1	Rating Methodology Reporting Program

Transmission Service Providers (TSPs) shall submit their equipment rating methodology to ERCOT in the first quarter of each year or provide notice that the rating methodology has not changed since the previous year.  ERCOT shall keep records describing the rating methodologies of each TSP in the ERCOT Region. ERCOT shall prepare an annual report summarizing these rating methodologies and submit this information to the Texas Regional Entity (TRE).

9..2	Real-Time Data Monitor

ERCOT shall produce reports describing Real-Time data performance of TSPs in the following areas.  ERCOT shall post the summary report on the Market Information System (MIS) Secure Area.  Individual point performance shall be posted to the MIS Certified Area.

(a)	Telemetry Performance:

(i)	ERCOT shall produce quarterly reports describing general and critically important telemetry performance as defined in the Protocols and the Technical Advisory Committee (TAC) approved Telemetry Standards.

(b)	Communication System Performance:

(i)	ERCOT shall produce monthly reports describing the reliability of each participants Inter-Control Center Communications Protocol (ICCP) data link to ERCOT as defined in the Protocols and the TAC approved Telemetry Standards.

(ii)	ERCOT shall produce monthly reports describing ICCP link up/down statistics.

9.2.1	Outage Scheduling and Reporting

This Section describes the reporting data for the transmission Outage scheduling provided for informational purposes.  There are no performance metrics for this data.

(1)	ERCOT shall post a monthly report of Outages considered on the MIS Secure Area including:

(a)	Number of Outage requests submitted in the ERCOT Outage Scheduler greater than 335 days (11 months) in advance;

(b)	Number of Outage requests submitted in the ERCOT Outage Scheduler between 90 and 334 days in advance of the desired Outage date;

(c)	Number of Outage requests submitted in the ERCOT Outage Scheduler between eight and 89 days in advance of the desired Outage date;

(d)	Number of Outage requests submitted in the ERCOT Outage Scheduler between three and seven days in advance of the desired Outage date;

(e)	Number of Outage requests submitted less than three days in advance;

(f)	Number of Outages by Outage type; and

(g)	Total Number of Outages that were requested, accepted, approved, cancelled, and withdrawn.

(2)	ERCOT shall post reports for each owner showing the percentage of the total number of Outages, by type, described in paragraph (1) above.

9.2.3	Transmission Service Provider (TSP) Network Operations Model Update Implementation Monitor

(1)	ERCOT shall prepare monthly reports summarizing the Network Operations Model update modeling process utilized by TSPs.  The report shall include the

number of Notices of interim reports, delineated by category, submitted to the Public Utility Commission of Texas (PUCT) and the Independent Market Monitor (IMM) as accumulated by TSP due to the actions or inactions of the associated TSP.  Interim updates caused by ERCOT software, hardware, or communication infrastructure shall not be attributed to the TSP.

(2)	ERCOT shall post reports on the MIS Secure Area.

9.2.4	Compliance with Valid Dispatch Instructions Monitoring

ERCOT shall produce monthly reports describing TSP valid Dispatch Instruction performance.

9.3	ERCOT Monitoring Program

9.3.1	Transmission Control

(1)	ERCOT shall prepare a report describing the operating violations on every transmission line and autotransformer operated at voltages greater than 60 kV.

(a)	ERCOT shall prepare monthly reports on the following:

(i)	Transmission equipment ratings that are exceeded;

(ii)	Violations of system operating limits or power transfer limitations set by ERCOT to guard against post-contingency stability exceedance for over ten minutes;

(iii)	Violations of system operating limits or power transfer limitations set by ERCOT to guard against post-contingency stability exceedance for over 30 minutes;

(iv)	N-1 violations of equipment ratings provided by Transmission Service Providers (TSPs) and Resources for over 30 minutes; and

(v)	N-0 violations detected during the month, the time duration of the loading above normal operating rating and the maximum exceedence.

(2)	Other transmission monitoring and control metrics:

(a)	ERCOT shall prepare reports describing the number of Forced Outages by owner by month.

(b)	ERCOT shall post on the Market Information System (MIS) Secure Area its performance in processing Outage requests in accordance with the following table including justification for rolling Outages from one timeline to another:

| Amount of time between the request for approval of the proposed Outage and the scheduled start date of the proposed Outage: | ERCOT shall approve or reject no later than: |
| --- | --- |
| Three days | 1800 hours, two days before the start of the proposed Outage |
| Between three and eight days | 1800 hours, three days before the start of the proposed Outage |
| Between nine days and 45 days | Four days before the start of the proposed Outage |
| Between 46 and 90 days | 40 days before the start of the proposed Outage |
| Greater than 90 days | 75 days before the start of the proposed Outage |

(c)	ERCOT shall post on the MIS Secure Area, by owner, the number of rejection Notices .  The report will include specific concerns that caused the rejection.

(d)	ERCOT shall post on the MIS Secure Area, by owner, the number of withdrawals of approved Outages.

(e)	ERCOT shall post on the MIS Secure Area, by owner, the number of approved Outages that were rescheduled by ERCOT.

(3)		State Estimator (SE) Performance:

(a)	ERCOT shall report State Estimator (SE) performance in accordance with the Protocols and the Technical Advisory Committee (TAC)-approved SE standard and post such report on the MIS Secure Area.

(i)	ERCOT shall produce monthly reports describing SE convergence and valid SE solution rates as described in Protocol Section 3.10.9.2, Telemetry and State Estimator Performance Monitoring.

(ii)	ERCOT shall produce monthly reports describing the MW differences between SE results and power flow results for congested Transmission Elements as approved by TAC.

(iii)	ERCOT shall produce monthly reports describing the MW differences between the SE results and telemetry for congested Transmission Elements as approved by TAC.

(iv)	ERCOT shall produce monthly reports describing the voltage differences between the SE results and telemetry for the 20 most voltage critical buses as approved by TAC.

(v)	ERCOT shall produce monthly reports describing the MW differences between the SE results and telemetry for all Transmission Elements greater than 100kV.

(vi)	ERCOT shall produce monthly reports identifying the sum of MW flows around telemetered Electrical Buses as described in paragraph (5) of Protocol Section 3.10.7.5.2, Continuous Telemetry of the Real-Time Measurements of Bus Load, Voltages, Tap Position, and Flows.

9.3.2	Resource Control

(1)	Resource Control Metrics:

(a)	Average sum of Regulation Up (Reg-Up) and Regulation Down (Reg-Down) energy - ERCOT shall report for each month the difference between the five-minute average monthly sum of Reg-Up energy deployed and the five-minute average monthly sum of Reg-Down energy deployed for each five-minute interval of the Operating Day.

(b)	Total Reg–Up and Reg-Down per interval - ERCOT shall develop a monthly report detailing the total amount of Reg-Up energy deployed in the Settlement Interval and the total amount of Reg-Down energy deployed for each Settlement Interval of the Operating Day.

(2)	Reserve Monitoring:

(a)	ERCOT shall prepare monthly reports describing amounts of time that it operated without sufficient operating reserves as defined in the Protocols.

(3)	RUC commitments and deployments:

(a)	For each month, ERCOT shall report, Generation Resources committed in the RUC process, including Resource name, intervals deployed, process in which the Resource was committed, and any deviation from Resources committed and actual Resource performance.

(4)	Dynamically Scheduled Resource (DSR) Performance:

(a)	ERCOT shall produce monthly reports for DSR Load signal failures of more than five minutes.

(5)	Conflicting Instructions to Generation Resources from interval to interval:

(a)	ERCOT shall record and report, on a monthly basis, instances of Dispatch Instructions to Resources not providing Regulation Service in which there is an inflection in the instruction for four consecutive Security-Constrained Economic Dispatch (SCED) intervals for validation and review.  Inflection is defined as a reversal in ramp direction between consecutive intervals.

(6)	ERCOT-wide governor response to Measurable Events:

(a)	ERCOT shall develop monthly reports detailing ERCOT’s governor response to each Measureable Event that month.  ERCOT shall meet at all times the governor response criteria as described in Protocol Section 8.5.2, Primary Frequency Control Measurements.

9.3.3	Load Forecasting

ERCOT shall report the Mean Absolute Percent Error (MAPE) each month for the following:

(a)	The accuracy of each day’s system Load forecast posted at 0600 in the Day Ahead of the Operating Day as compared with the actual average ERCOT Load for each hour of the Operating Day;

(b)	Accuracy of the system Load forecast used for Day-Ahead Reliability Unit Commitment (DRUC) compared to the actual average ERCOT Load for each hour of the Operating Day; and

(c)	The accuracy of the Load forecast for the following items compared to the average of the State Estimated Load at each Electrical Bus with a modeled Load for each hour:

(a)	Hourly Load forecast used in the DRUC by Weather Zone;

(b)	Hourly Load forecast used in the Hourly Reliability Unit Commitment (HRUC) by Weather Zone; and

(c)	The accuracy of the Load forecast used in the DRUC for the largest MW and MVA differences between the hourly Bus Load Forecast and the Real-Time Load at each Electrical Bus, by Weather Zone.

9.3.4	System Operating Constraints

ERCOT shall report the following comparisons with respect to Real-Time operations as they occur:

(a)	Three consecutive days of a particular congestion constraint (constraints passed to SCED from Network Security Analysis (NSA)) in Real-Time not identified in the Day Ahead Market (DAM);

(b)	Two consecutive hours of a particular congestion constraint in Real-Time not identified in the HRUC process; and

(c)	Two consecutive days of a particular congestion constraint in Real-Time not identified in the DRUC process.

9.3.5	Network Operations Model Update Implementation Statistics

ERCOT shall report monthly on the Network Operations Model update implementation performance.  The report shall include the total number of Network Operations Model Change Requests (NOMCRs) submitted, approved, rejected, withdrawn and any other status contained in the Protocols or defined by ERCOT.  The report shall also include the number of interim updates, delineated by category and owner, including reasons for the interim updates.   NOMCRs not meeting the time line pursuant to Protocol Section 3.10.1, Time Line for Network Operations Model Change Requests, due to ERCOT related issues (i.e. server unavailability, Network Model Management System (NMMS) component failure, site failover, loss of data, staff overload, weather, etc.) shall be reported and attributed to ERCOT.  The time line defined in paragraph (1) of Protocol Section 3.10.1 shall be used in the performance evaluation.

9.3.6	Back-up Control Plan

ERCOT shall report, by exception, the Entities that have not submitted back-up plans by the end of the first quarter of each year in accordance with the following:

(1)	ERCOT’s back-up plan as required by paragraph (h) of Protocol Section 8.2, ERCOT Performance Monitoring;

(2)	Qualified Scheduling Entity (QSE) back-up plans submitted to ERCOT pursuant to Section 3.2.1, Operating Obligations; and

(3)	TSPs back-up plans submitted to ERCOT and the date that the plan was last updated pursuant to Section 3.9, Transmission Operators.

9.3.6	ERCOT Black Start Plan

ERCOT shall develop an annual Black Start Plan and post it on the MIS Certified Area.

9.3.7	Computer and Communication Systems Real-Time Availability and Systems Security

ERCOT shall report monthly the availability of its computer and communications systems.

9.3.8	Voltage and Reactive Control Performance Monitoring

ERCOT in coordination with the TSPs shall conduct studies to determine the normally desired predetermined distribution of desired nominal voltage set points across the ERCOT System Voltage Profile for all Electrical Buses used for Voltage Support in the ERCOT System and shall post all Voltage Profiles annually on the MIS Secure Area.

Transmission owners shall provide switching plans for automatically controlled reactors, capacitors, and other reactive controlled sources to ERCOT.  For manually switched reactive devices, the transmission owner shall provide its guidelines for the operation of these devices.  These plans and guidelines shall be posted annually to the MIS Secure Area and must be provided in accordance with the NOMCR or other prescribed process.

9.4	Ancillary Services Monitoring Program

ERCOT shall monitor Ancillary Service energy deployment according to the criteria outlined in Protocol Section 8, Performance Monitoring.  Reports required by Protocol Section 8.1.1.4, QSE Ancillary Service Energy Deployment Monitoring Criteria, will be posted on the Market Information System (MIS) Certified Area.

9.4.1	Regulation Control Performance

ERCOT shall develop monthly reports detailing Resource-specific and Dynamically Scheduled Resources (DSR) Portfolio Regulation and Security-Constrained Economic Dispatch (SCED) Base Point following performance as described in Protocol Section 8.1.1.4.1, Regulation Service and Generation Resource Energy Deployment Performance.

9.4.2	Hydro Responsive Testing

ERCOT shall produce monthly reports of hydro responsive tests and verify results submitted.

9.4.3	Black Start Service Requirements for TSPs, QSEs, and Generation Resources

ERCOT shall produce annual reports containing record(s) of if and when a black start plan was last received at ERCOT including the Qualified Scheduling Entity (QSE) or Transmission Service Provider (TSP) name and the date the plan was received.  The QSEs participating in Black Start Service and TSPs shall provide an updated black start plan to ERCOT by the end of the 1st quarter and when the plan has changed.

9.4.4	Resource-Specific Responsive Reserve Performance

ERCOT shall develop monthly reports detailing Resource-specific Responsive Reserve (RRS) deployments, including Load Resources, capacity released deployments for SCED and the amount of time it took the QSE to release RRS capacity.

If a Resource does not release capacity reserved for RRS within 60 seconds and is not excused due to exemptions contained in the Protocols, the Resource will have failed the performance criterion.

9.4.5	 Constant Frequency Control

ERCOT shall survey QSEs qualified to provide Regulation Services to attest that the QSEs have the capability to operate in constant frequency control mode.  ERCOT shall develop annual summaries of QSEs surveyed.

9.4.6	Resource-specific Non-Spinning Reserve

ERCOT shall develop a monthly summary for Non-Spinning Reserve (Non-Spin) deployed on On-Line Resources, Load response and DSR, and on Off-Line Resources and the amount of time it took to release the Non-Spin capacity.

If an On-Line Resource does not release its reserved Non-Spin capacity via telemetry within five minutes or an Off-Line Resource does not come On-Line and release its reserved Non-Spin capacity within 30 minutes and is not excused due to exemptions contained in the Protocols, the Resource will have failed the performance criterion.