| PRR Number | 815 | PRR Title | CSC Process Clarification |  |
| --- | --- | --- | --- | --- |
| Timeline | Urgent | Action |  | Approved |
| Date of Decision |  | July 21, 2009 |  |  |
| Protocol Sections Requiring Revision |  | 7.2, CSC Zone Determination Process 7.2.1, Process for Determining CSCs 7.2.2, Congestion Zone & Zonal Shift Factor Determination Methodology 7.2.3, Determining Closely Related Elements (CREs) |  |  |
| Effective Date |  | August 1, 2009 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Revision Description |  | This Protocol Revision Request (PRR) clarifies the process used to determine Commercially Significant Constraints (CSCs). This PRR also proposes non-substantive administrative changes, such as correcting acronym and defined term usage and section references. These are shown below with the author “ERCOT Market Rules.” |  |  |
| Overall Market Benefit |  | Provides clarity of process used in determining CSCs. |  |  |
| Overall Market Impact |  | Unknown. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed PRR815 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 6/10/09, PRR815 and its associated Impact Analysis and CEO Revision Request Review were posted. - On 6/12/09, the motion to grant PRR815 Urgent status passed via email vote. - On 6/17/09, PUCT Staff comments were posted. - On 6/17/09, WMS comments were posted. - On 6/18/09, PRS considered PRR815. - On 6/25/09, AEP comments were posted. - On 7/2/09, ERCOT comments were posted. - On 7/9/09, TAC considered PRR815. - On 7/21/09, the ERCOT Board considered PRR815. |  |  |
| PRS Decision |  | On 6/18/09, PRS voted to recommend approval of PRR815 as amended by PUCT Staff comments. There was one opposing vote from the Investor Owned Utility (IOU) Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 6/18/09, ERCOT Staff explained that the revision in paragraph (1)(a)(i) of Section 7.2.1 recommended by the 6/17/09 WMS comments were intended to allow ERCOT to consider as Study Zones limiting elements that do not reach 100% of their emergency rating at high transfer levels, but that ERCOT was amenable to the PUCT Staff version, which restricted consideration to limiting elements that reached 100%, which is consistent with how ERCOT conducts its studies. One participant noted several concerns with PRR815 that would be included in forthcoming comments and suggested delaying consideration of PRR815. Concern was expressed over delaying consideration of PRR815, since the Public Utility Commission of Texas (PUCT) indicated that it wanted the ERCOT Board to consider this PRR no later than July. |  |  |
| TAC Decision |  | On 7/9/09, TAC voted to recommend approval of PRR815 as revised by TAC. There was one opposing vote from the IOU Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 7/9/09, the 6/25/09 AEP comments and 7/2/09 ERCOT comments were reviewed, with AEP explaining the questions it posed to ERCOT and ERCOT clarifying its responses. Protocol language revisions were proposed to clarify that TAC’s recommendations to the ERCOT Board regarding constraints and Congestion Zones may differ from the recommendations of the appropriate TAC subcommittee and that the pre-contingency and post-contingency system models that may be used in the clustering analysis required by paragraph (2) of Section 7.2.2 are actually different system representations in the same model. |  |  |
| Board Decision |  | On 7/21/09, the ERCOT Board approved PRR815 as recommended by TAC. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | Unknown | Unknown |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Information transparency. | Unknown |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| PUCT Staff 061709 | Proposed that the second “may” found in paragraph (1)(a)(i) of Section 7.2.1 and the “may” found in paragraph (1)(e) be replaced with the word “shall” to make the designation process more predictable, and that the phrase “and proposed congestion zones” be added immediately after the phrase “candidate CSCs” each time the latter phrase appears in paragraph (2) to clarify ERCOT’s recommendations and concerns with regard to both candidate CSCs and Congestion Zones. |
| WMS 061709 | Indicated that WMS voted to endorse PRR815 as amended by PUCT Staff comments and as revised by WMS to restore the word “may” in paragraph (1)(a)(i) of Section 7.2.1. |
| AEP 062509 | Stated that the proposed changes simply document what ERCOT did during the 2009 CSC approval process and do not reflect an evaluation of the policy decision and application of that evaluation to the overall process. Posed questions to ERCOT regarding proposed process clarification. |
| ERCOT 070209 | Responded to questions raised in 6/25/09 AEP comments. |

| Original Sponsor |  |
| --- | --- |
| Name | Isabel Flores |
| Company | ERCOT |
| Market Segment | Not applicable |

Proposed Protocol Language Revision

7.2	CSC Zone Determination Process

By November 1 of each year, the appropriate ERCOT subcommittee will report to the Technical Advisory Committee (TAC) and ERCOT Board with recommended Commercially Significant Constraint (CSC) designations, resulting Congestion Zone boundaries with any granted exemptions noted, Closely Related Element (CRE) designations and associated Boundary Generation Resources for ERCOT Board review and approval.  Shortly after ERCOT Board approval, CSCs and resulting Congestion Zone boundaries and CRE designations will be posted on the Market Information System (MIS).  This posting will include a station and bus-by-bus identification of each Congestion Zone.

7.2.1	Process for Determining CSCs

CSCs and resulting Congestion Zones will be reassessed annually by November 1 of each year.  ERCOT will use the following process to identify CSCs:

(1)	By August 1, of each year, ERCOT will complete an analysis of Load flow data using the latest Steady State Working Group (SSWG) Data Set A summer peak case and will determine expected operating limits, candidate CSCs and associated constraints to be used in the designation of CSCs for the upcoming calendar year using the following steps:

(a)	Determination of new CSC candidates:

(i)	Candidate CSCs may be determined using transfer analysis between Study Zones.  Study Zones are groupings of buses used to test transfer limits using power system simulation tools.  These Study Zones can be either of the following: (1) reasonable approximations of the final Congestion Zones, Weather Zones, SSWG Areas or Zones; or (2) any grouping of buses by geographic region or generation/Load pocket.  Candidate CSCs   shall be: (1) the contingency that  causes limiting elements to reach one hundred percent (100%) of their emergency rating at high transfer levels; (2) the transmission line that is the limiting element under contingencies; or (3) a stability interface limit.

(ii)	Candidate CSCs may also be identified through an evaluation of the actual annual Congestion costs from the prior year.  For this purpose, Congestion costs include the cost of any zonal Balancing Energy Service, Reliability Must-Run (RMR) Service, Out of Merit Capacity (OOMC) Service, Replacement Reserve Service (RPRS), Out of Merit Energy (OOME) Service, or Resource-specific Dispatch Instructions used to resolve Congestion on that particular transmission path in that particular direction during the prior year.  Candidate CSCs may also be identified by considering projected annual Congestion costs for the CSC designation year.  Since this type of data reflects past system topology and generation, generation and transmission system changes expected to be in service in the CSC designation year shall also be considered.

(iii)	Candidate CSCs may also be identified based on ERCOT’s operational experience.

(b)	If no new candidate CSCs are considered other than the immediately preceding year’s CSCs, then ERCOT shall use the Congestion Zones from the immediately preceding year as Study Zones.

(c)	If new candidate CSCs are considered, then ERCOT shall calculate Shift Factors and perform cluster analysis based on the candidate CSCs to create Study Zones.

(d)	ERCOT shall perform system simulation studies with transfers between the Study Zones and find constraints on the transmission system to determine if candidate CSCs are appropriate for designation as CSCs.

(e)	ERCOT shall recommend whether the candidate CSCs qualify for CSC designation.  The following factors shall be considered:

(i)	Whether there is a sufficiently competitive market to resolve Congestion on the transmission path to be considered for CSC designation.  One factor that will be considered when assessing whether a sufficiently competitive market exists is the concentration of capacity ownership or control by Market Participants or their Affiliates, compared to the distribution of Shift Factor impacts;

(ii) 	Whether the candidate CSC is an adequate indicator to be used to operationally manage and measure inter-zonal transfers; and

(iii) 	Whether deployment of zonal balancing will be effective for managing the post-contingency flow on the limiting element.

(2)	ERCOT shall present all candidate CSCs and proposed Congestion Zones to the appropriate TAC subcommittee.  ERCOT shall identify the candidate CSCs and proposed Congestion Zones it believes are the most appropriate CSCs and proposed Congestion Zones and shall explain the basis for its position.  ERCOT’s justification shall include any operational or reliability concerns that it has identified with any candidate CSCs and proposed Congestion Zones.

(3)	CSC approval process:  The appropriate TAC subcommittee will review the results and the process followed above to determine the list of constraints to be recommended for approval to the TAC.  TAC shall then make a recommendation to the ERCOT Board.

7.2.2	Congestion Zone & Zonal Shift Factor Determination Methodology

ERCOT will take the following steps to determine Congestion Zones and Zonal Shift Factors:

(1)	Shift Factors are developed using a linearized (Direct Current (DC)) model to identify the impact of each transmission bus on each CSC relative to an ERCOT reference bus.

(2)	A statistical clustering analysis will be used to aggregate transmission buses into zones based upon similar Shift Factors relative to all CSCs.  The clustering must meet the following criteria: (i) each CSC must straddle a zonal boundary (however, not every zonal boundary need be straddled by a CSC); and (ii) station IDs as provided by Transmission and/or Distribution Service Providers (TDSPs) in Section 15.1.1.5, Response from TDSP to Registration Notification Request, can be assigned only into one (1) Congestion Zone.

The following measurements will be calculated for each combination of zones: (i) the maximum difference between the Zonal Shift Factor and the Shift Factor of any generation bus within the zone; and (ii), the R2 (coefficient of determination) from the cluster analysis.

The system model used to conduct the clustering analysis required by this Section shall be either a pre-contingency or post-contingency system model.  A pre-contingency system representation in the model shall be used when the candidate CSC is the contingency element or a stability interface limit.  A post-contingency system representation in the model shall be used when the candidate CSC is the limiting element.

(3)	If the clustering process described in paragraph (2), above, results in the splitting of two or more buses within any single station into different zones, then the following process will be used to adjust the zone selection to place all buses within a single station in the same zone:

(a)	For Generation Resource stations, determine which zone contains the largest amount of generating capacity of the station and assign all buses of that station to that zone;

(b)	For all other stations, determine which zone is assigned the largest total amount of Customer Load of the buses in the station and assign all buses of that station to that zone;

(c)	If there is no Load or generation in the station, or if the amount of generation and/or Load assigned to individual station buses are equal, then the Load or generation shall be assigned to the zone containing the lowest bus number of the station.

(4)	Zonal Shift Factors for each CSC will be determined by averaging the individual bus Shift Factors within each Congestion Zone for the corresponding CSC, weighted by the megawatt levels on the bus in the Load flow base case used to determine CSC limits from Generation Resources deemed likely to vary their output.

The appropriate TAC subcommittee will review the results and the process followed above to determine the Congestion Zones to be recommended for approval to the TAC. TAC shall then make a recommendation to the ERCOT Board.

(5)	Thirty (30) days prior to TAC review, a Non Opt-In Entity (NOIE) may request an exemption to its zonal placement by submitting a request to ERCOT for consideration.  ERCOT will evaluate the merits of the request and authorize the exemption based on the following criteria:

(a)	Exemptions shall not cause significant operational impact to the ERCOT System.  This will normally be satisfied if the request will not result in a change in the annual average Shift Factor for the original or requested Congestion Zone by more than five percent (5%).  After the proposed move, the Zonal Shift Factor in each of the original and requested Congestion Zones shall be no less than ninety-five percent (95%), and no more than one hundred five percent (105%), of its original Zonal Shift Factor before the proposed move.

Because only buses with generation have an impact on the calculation of Zonal Shift Factors, if no generation exists at a bus that is part of an exemption request, a token generator of one (1) MW will be placed at that bus to test the impact of moving that bus from one (1) zone to another.  This will be done to all buses that make up a request for exemption.  For example, if an Entity is requesting an exemption for five (5) buses and only one (1) bus has generation On-line in the Load flow case used to test the impact, one (1) MW of generation will be placed at the four (4) other buses and the impact of all five (5) buses on Zonal Shift Factors will be determined as a group as opposed to individually.  The Load flow case used to determine this impact will be the same Load flow case used to calculate annual Transmission Congestion Right (TCR) quantities.

(b)	Exemptions shall not have a commercial impact/gain for the requestor.  This will be measured by whether or not the scheduling which would otherwise result would be eligible for Pre-assigned Congestion Rights (PCRs).

7.2.3	Determining Closely Related Elements (CREs)

For each year, ERCOT shall identify potential CREs using, at a minimum, the following process:

(1)	Determine the Zonal Shift Factor for a particular CSC (Xz) for each Zone (z).

(2)	Determine the Zonal Shift Factor for the candidate CRE (Yz) for each Zone (z) using the same generation weighting as in paragraph (1) above but ignoring Boundary Generation Resource buses that would cluster into a different Congestion Zone with respect to the CRE.

(3)	Determine positive “a” applying least-square curve fitting to the following equation:

Yz = a (Xz) + bz  	for all Zones z.

(4)	Using “a” from paragraph (3) above, determine the maximum absolute value of bz.

(5)	Also determine the total capacity (MW) of Boundary Generation Resources that would cluster into a different Congestion Zone.

(6)	If the maximum absolute value of bz is less than a threshold set by the appropriate TAC subcommittee, not to exceed 0.2, and the total capacity of Boundary Generation Resources that would cluster into a different Congestion Zone is less than one thousand five hundred (1,500) MW, then the element is a CRE for the particular CSC.

ERCOT may identify potential CREs in addition to the potential CREs identified through the process described above as it determines appropriate.  As part of the CRE identification process, ERCOT shall evaluate each contingency element/limiting element pair combination to determine if deployment of zonal balancing will be effective for managing the post-contingency flow on the limiting element.  Based on this evaluation, ERCOT shall develop its proposed CREs and list of contingencies for managing CSC and CRE related Congestion.  ERCOT shall present all proposed CREs and list of contingencies for managing CSC and CRE related Congestion, including justification for their inclusion, to the appropriate TAC subcommittee for approval.

During the effective year, ERCOT may propose modifications to the list of CREs or the approved list of contingencies for managing CSC and CRE related congestion, including the expected duration of those modifications, as needed to more closely represent actual system constraints that can be effectively resolved by Zonal Balancing Energy deployments.  Any modifications to the list of CREs or the approved list of contingencies for managing CSC and CRE related congestion will not affect Congestion Zone definition or composition, nor will they affect CSC definitions.  ERCOT shall present modifications proposed to the list of CREs or the approved list of contingencies for managing CSC and CRE related congestion after the start of the year to TAC for approval.  TAC will have seven (7) days to take action on the proposed modification.  If TAC takes no action within seven (7) days, the proposed modification shall be deemed approved.

7.2.4	Determining Generation Resources Deemed Likely to Vary Their Output

At the beginning of each year, ERCOT will compile a list of all Generation Resources fueled by nuclear fuel, coal or lignite.  ERCOT will exclude such units from the calculation of Zonal Shift Factors for the rest of the year.  All other Generation Resources will be deemed likely to vary their output.