| NOGRR Number | 025 | NOGRR Title | Monitoring Programs for QSEs, TSPs, and ERCOT |
| --- | --- | --- | --- |

| Date | August 25, 2009 |
| --- | --- |

| Submitter’s Information |  |
| --- | --- |
| Name | Barbara Clemenhagen on behalf of the Wholesale Market Subcommittee (WMS) |
| E-mail Address | bclemenhagen@topazpowergroup.com |
| Company | Topaz Power Group |
| Phone Number | (512) 314-8621 |
| Cell Number | (512) 670-6702 |
| Market Segment | Independent Generator |

Comments

At the August 19, 2009 WMS meeting, WMS considered Nodal Operating Guide Revision Request (NOGRR) 025.  After discussion, WMS unanimously voted to endorse NOGRR025 as amended by the 7/30/09 ERCOT comments and as revised by WMS, including the selection of “Option 2” in Section 9.4.5, Resource-specific Non-Spinning Reserve.

As requested by TAC, WMS also reviewed the list the reports provided by ERCOT in its 8/11/09 comments.  WMS unanimously voted to recommend that TAC be added to the list of parties that may request the reports marked in yellow on a limited basis for specific intervals.  WMS unanimously voted to endorse the list with the request that three of the reports that ERCOT recommended be delayed until after the Texas Nodal Market Implementation Date (TNMID) instead be made available for testing during market trials (i.e., green), or upon request on a limited basis if ERCOT has concerns about making such reports available for testing during market trials (i.e., yellow).  A mark-up of the list is provided below with these three reports shown in pink.  WMS also requests that ERCOT provide additional information regarding the priorities and cuts it has chosen in making its report availability determination.

Mark-up of List from 8/11/09 ERCOT Comments

The items highlighted in green represent the 16 reports which will be available for testing during market trials.  The items in yellow represent the 18 reports that can be queried by ERCOT on a limited basis for specific intervals as requested by TAC, the Public Utility Commission of Texas (PUCT), Texas Regional Entity (TRE), or the Independent Market Monitor (IMM).  The remaining reports are recommended to be delayed until after the Texas Nodal Market Implementation Date (TNMID).

|  |  |  | Monitoring Program |  |  |
| --- | --- | --- | --- | --- | --- |
| Program Report Product | Operating Guide | Protocol Section | QSE | TSP | ERCOT |
| Annual QSE Testing Results | 9.1.1 | 8.1.1.1 | X |  |  |
| Annual Reactive Testing Results | 9.1.2(2) | 8.1(3)(b) | X |  |  |
| Critically Important Telemetry Availability | 9.1.3(a)(i) 9.2.2(a)(i) | 8.1.(3)(c)(i) 8.3(b)(i) | X | X |  |
| Critically Important Telemetry Availability Detail | 9.1.3(a)(i) 9.2.2(a)(i) | 8.1.(3)(c)(i) 8.3(b)(i) | X | X |  |
| Monthly Market Participant Real Time Availability Statistics | 9.1.3(a)(i) 9.2.2(a)(i) | 8.1.(3)(c )(i) 8.3(b)(i) | X | X |  |
| Monthly Market Participant Real Time Availability Statistics Detail | 9.1.3(a)(i) 9.2.2(a)(i) | 8.1.(3(c )(i) 8.3(b)(i) | X | X |  |
| Quarterly Point Availability Statistics | 9.1.3(a)(i) 9.2.2(a)(i) | 8.1.(3) (c )(i) 8.3(b)(i) | X | X |  |
| Quarterly Point Availability Statistics Detail | 9.1.3(a)(i) 9.2.2(a)(i) | 8.1.(3) (c )(i) 8.3(b)(i) | X | X |  |
| ICCP Link Availability Statistics | 9.1.3(b)(i) 9.2.2(b)(i) | 8.1.(3)(c )(ii) 8.3(b)(ii) | X | X |  |
| ICCP Link Uptime Statistics | 9.1.3(b)(ii) 9.2.2(b)(ii) | 8.1.(3)( c) (ii) 8.3(b)(ii) | X | X |  |
| Monthly Resource Energy Deployment Performance | 9.1.4(1) | 8.1.1.4.1 | X |  |  |
| Annual Resource Energy Deployment Performance | 9.1.4(1) |  | X |  |  |
| Emergency Interruptable Load Service Event | 9.1.4(2) |  | X |  |  |
| Monthly Network Operations Model Updates | 9.1.5(1) 9.2.4(2) 9.3.5 | 8.1(3)(i) 8.2 8.3(d) | X | X | X |
| Monthly Resource Advanced Outage Schedule Statisitics | 9.1.6 (1) 9.2.3(1) | 8.1(3)(j) 8.1(3)(q) 8.2(2)(B)(i) 8.3(c) | X | X | X |
| Annual Backup Control Plan Results | 9.1.7 9.2.5 | 8.1(3)(l) 8.3(e) | X | X |  |
| Qualified Staffing Non-Performance Notice | 9.1.8 | 8.1(3)(n) | X |  |  |
| Monthly Automatic Voltage Regulator Test Submissions | 9.1.9(2) |  | X |  |  |
| Monthly Automatic Voltage Regulator Availability Compliance | 9.1.9(3) | 8.1(3)(0) | X |  |  |
| Monthly Current Operating Plan Submission Compliance | 9.1.10(1) | 8.1.2 | X |  |  |
| COP | 9.1.10(3) | 8.1(3)(r) 8.1.2 | X |  |  |
| Annual Rating Methodology Summary | 9.2.1 | 8.3(1)(a) |  | X | X |
| Monthly ERCOT Transmission Violations | 9.3.1(1) | 8.2(2)(a)(i) | X |  | X |
| Monthly Forced Outage | 9.3.1(2)(a) |  |  |  |  |
| ERCOT Monitoring and Control Metrics | 9.3.1(2)(b) 9.3.1(2)(c) 9.3.1(2)(d) 9.3.1(2)(e) | 8.2(2)(a)(ii) |  |  | X |
| State Estimator Convergence Rate | 9.3(3)(a)(i) | 8.2(2)(a)(iii) |  |  | X |
| MW Residuals vs RTCA Base Case for Congested Transmission Elements | 9.3.1(3)(a)(ii) | 8.2(2)(a)(iii) |  |  | X |
| MW Residuals vs Telemetry for Congested Transmission Elements | 9.3.1(3)(a)(iii) | 8.2(2)(a)(iii) |  |  | X |
| Voltage Residuals vs Telemetry for Critical Buses | 9.3.1(3)(a)(iv) | 8.2(2)(a)(iii) |  |  | X |
| MW Residuals vs Telemetry for Major Transmission Elements | 9.3.1(3)(a)(v) | 8.2(2)(a)(iii) |  |  | X |
| Buses Not Meeting Telemetry Standards | 9.3.1(3)(a)(vi) | 8.2(2)(a)(iii) | X | X | X |
| Monthly Total Regulation | 9.3.2(1)(a) | 8.1.1.3.18.2(2)(b)(ii) |  |  | X |
| Monthly Reserve Monitoring | 9.3.2(2)(a) | 8.2 8.2(2)(b)(iii) |  |  | X |
| Monthly RUC Deployments | 9.3.2(3)(a) | 8.1(3)(s) 8.2(2)(b)(iv) |  |  | X |
| Monthly DSR Performance | 9.3.2(4)(a) | 8.2(2)(b)(v) | X |  | X |
| Monthly Conflicting Instructions | 9.3.2(5)(a) | 8.2(2)(b)(vi) |  |  | X |
| Actual Governor Response |  | 8.5.1.2(2) | X |  |  |
| Monthly Event Governor Response | 9.3.2(6)(a) | 8.5.1.2(2) | X |  | X |
| Monthly Mean Absolute Percent Error | 9.3.3(1) | 8.2 |  |  | X |
| Monthly Wind Forcast and Actuals | 9.3.3(2) | 8.2 |  |  | X |
| System Operations 3-Day Congestion Event | 9.3.4(a) | 8.2 |  |  | X |
| System Operations 2-Hour Congestion Event | 9.3.4(b) | 8.2 |  |  | X |
| System Operations 2-Day Congestion Event | 9.3.4 ( c) | 8.2 |  |  | X |
| Annual Back-up Control Plan Compliance | 9.3.6 | 8.1(3)(p) |  |  | X |
| Annual Black Start Procedure Submittal | 9.3.7 9.4.2 | 8.1 (3)(d) | X | X | X |
| Annual Black Start Qualification | 9.3.7 9.4.2 | 8.1.2.1.5(2) 8.1(3)(h) | X |  | X |
| Monthly ERCOT Communications Availability | 9.3.8(1) |  |  |  | X |
| Monthly SCED Violations | 9.3.8(2) |  |  |  | X |
| Monthly ERCOT SCED Metrics | 9.3.8(3) | 8.1.1.4.1(4) |  |  | X |
| Annual Voltage Profiles and Reactive Control Guidelines | 9.3.9 | 8.2(2)(a)(iv) |  |  | X |
| Monthly Hydro Responsive Testing Results | 9.4.1 | 8.1(3)(g) | X |  |  |
| Monthly Responsive Reserve | 9.4.3 | 8.1.1.4.2 8.1(3)(k) |  | X | X |
| Annual Constant Frequency Control Results | 9.4.4 |  | X |  |  |
| Monthly Non-Spin Performance | 9.4.5 | 8.1.1.4.3 8.1(3)(n) | X |  |  |

Revised Proposed Guide Language Revision

ERCOT Nodal Operating Guides

Section 9: Monitoring Program

<date approved>

(Effective Upon Texas Nodal Market Implementation)

9	Monitoring Programs	3

9.1	QSE and Resource Monitoring Program	3

9.1.1	Testing	3

9.1.2	Reactive Testing for Generation Resources	3

9.1.3	Real-Time Data	3

9.1.4	Compliance with Valid Dispatch Instructions	3

9.1.5	Resource Facilities Network Operations Model Update Implementation Monitor 	3

9.1.6	Resource Outage Reporting	3

9.1.7	Backup Control for Resource Energy Deployment	3

9.1.8	Qualified Staffing Requirement	3

9.1.9	Automatic Voltage Regulator (AVR)	3

9.1.10	Current Operating Plan Metrics for QSEs	3

9.2	TSP Monitoring Program	3

9.2.1	Rating Methodology Reporting Program	3

9.2.2	Real-Time Data Monitor	3

9.2.3	Transmission Outage Reporting	3

9.2.4	Transmission Service Provider (TSP) Network Operations Model Update Implementation Monitor	3

9.2.5	Backup Control for TSPs	3

9.3	ERCOT Monitoring Program	3

9.3.1	Transmission Control	3

9.3.2	System and Resource Control	3

9.3.3	Forecasting	3

9.3.4	System Operating Constraints	3

9.3.5	Network Operations Model Update Implementation Statistics	3

9.3.6	Back-up Control Plan	3

9.3.6	ERCOT Black Start Plan	3

9.3.7	Computer and Communication Systems Real-Time Availability and Systems Security	3

9.3.8	Voltage and Reactive Control Performance Monitoring	3

9.4	Ancillary Services Monitoring Program	3

3

9.4.1	Hydro Responsive Testing	3

9.4.2	Black Star Service Requirements for TSPs, QSEs, and Generation Resources	3

9.4.3	Resource-Specific Responsive Reserve Performance	3

9.4.4	 Constant Frequency Control	3

9.4.5	Resource-specific Non-Spinning Reserve	3

# Monitoring Programs

9.1	QSE and Resource Monitoring Program

This Section sets forth formats and data needed for reporting to comply with Protocol Section 8, Performance Monitoring and Compliance.   These performance monitoring and compliance requirements apply as set forth below to Qualified Scheduling Entities (QSEs), Resources, Transmission Service Providers (TSPs) and ERCOT.  Reports defined in this Section will be posted on the Market Information System (MIS) Secure Area unless otherwise stated.

9.1.1	Testing

ERCOT shall produce reports annually which summarize:

(1)	Capability test results for the applicable Resources.  These results must be provided to ERCOT by the  QSE for the Resource.  The QSE shall provide these results to ERCOT upon completion of capability testing of the Resource.

(2)	Resource specific Non-Spinning Reserve (Non-Spin) performance results for QSEs and Resources;

(3)	Test results of Emergency Interruptible Load Service (EILS) Loads by Resource and QSE; and

(4)	Test results for Load Resources.

9.1.2	Reactive Testing for Generation Resources

(1)	The QSE shall provide the following information:

(a)	Unit name;

(b)	QSE;

(c)	Date;

(d)	Time;

(e)	Tested generation real power capability;

(f)	Reported time; and

(g)	Corrected Unit Reactive Limit (CURL) and Unit Reactive Limit (URL).

(2)	ERCOT shall produce reports annually which describe when the last Lagging reactive capability test took place, what the MW and MVAr outputs were during the test period, how long the output was held, and a minimum of four MW/MVAr pairs describing the CURL of the unit and any limiting conditions that created the need for a CURL.  This data shall be provided by the QSE to ERCOT when reactive testing indicates a change in unit reactive capability and at a minimum of once every two years.

9.1.3	Real-Time Data

ERCOT shall produce reports describing Real-Time data performance of QSEs in the following areas.  ERCOT shall post the summary report on the MIS Secure Area.  Individual point performance shall be posted to the MIS Certified Area.

(a)	Telemetry Performance:

(i)	ERCOT shall produce quarterly reports describing telemetry performance as defined in the Protocols and the Technical Advisory Committee (TAC) approved Telemetry Standards.

(b)	Communication System Performance:

(i)	ERCOT shall produce monthly reports describing the  reliability of each participants Inter-Control Center Communications Protocol (ICCP) data link to ERCOT as defined in the Protocols and the TAC approved Telemetry Standards.

(ii)	ERCOT shall produce monthly reports describing ICCP link up/down statistics.

9.1.4	Compliance with Valid Dispatch Instructions

(1)	ERCOT shall produce monthly reports detailing Resource-specific Regulation Service and Energy deployment performance, including Load Resources, based on the criteria described in Protocol Section 8.1.1.4.1, Regulation Service and Generation Resource Energy Deployment Performance.  ERCOT shall produce yearly reports of QSE and Resource-specific regulation performance metrics as required in the Protocols.

(2)	ERCOT shall produce a report for any system-wide deployment of Load Resources or EILS, on an event basis, within one month after the event occurs and shall post it to the MIS Secure Area.

9.1.5	Resource Facilities Network Operations Model Update Implementation Monitor

(1)	ERCOT shall prepare monthly reports summarizing the Network Operations Model updates  by Resource Facilities.  These reports shall include the

number of Notices of interim updates submitted to the Public Utility Commission of Texas (PUCT) and the Independent Market Monitor (IMM) as accumulated by QSE due to the actions or inactions of the associated QSE for Network Operations Model Change Requests (NOMCRs) not meeting the time line pursuant to Protocol Section 3.10.1, Time Line for Network Operations Model Change Requests.  These reports shall be delineated by category and owner including reasons for the interim updates.   Interim updates caused by ERCOT (i.e. server unavailability, Network Model Management System (NMMS) component failure, site failover, loss of data, staff overload, weather, etc.) shall be reported and attributed to ERCOT.

(2)	ERCOT shall post reports on the MIS Secure Area.

9.1.6	Resource Outage Reporting

This Section describes the reporting of data for Resource Outage scheduling by individual Resource for informational purposes.  There are no performance metrics for this data.

(1)	ERCOT shall post a confidential report of total number of Outages reported through the Texas Market Link (TML) including Monthly Resource Outage reporting by Resource:

(a)	Number of Outage requests submitted in the ERCOT Outage Scheduler greater than 335 days (11 months) in advance;

(b)	Number of Outage requests submitted in the ERCOT Outage Scheduler between 90 and 334 days in advance of the desired Outage date;

(c)	Number of Outage requests submitted in the ERCOT Outage Scheduler between eight and 89 days in advance of the desired Outage date;

(d)	Number of Outage requests submitted in the ERCOT Outage Scheduler between three and seven days in advance of the desired Outage date;

(e)	Number of Outage requests submitted less than three days in advance;

(f)	Number of Outages by Outage type; and

(g)	Total number of Outages that were requested, accepted, approved, cancelled, and withdrawn.

9.1.7	Backup Control for Resource Energy Deployment

ERCOT shall produce a report identifying the date of backup control plan testing and the success or failure of that test for ERCOT and QSEs.  Backup control plan tests shall be conducted at least annually.

9.1.8	Qualified Staffing Requirement

A QSE shall maintain a continuously operating scheduling center staffed with qualified personnel with the authority to commit and bind the QSE.  ERCOT shall report to the Texas Regional Entity (TRE), instances of suspected non-performance.

9.1.9	Automatic Voltage Regulator (AVR)

(1)	ERCOT shall record QSE provided Automatic Voltage Regulator (AVR) test reports for Resources which must include:

(a) 	The minimum and maximum excitation limiters settings and associated time limits;

(b)	Volts/hertz settings;

(c)	Gain and time constants;

(d)	Date tested; and

(e)	Voltage regulator control mode;

(2)	ERCOT shall produce a monthly report that will identify Resources for which test reports have not been submitted within the last 60 months.

(3)	ERCOT shall produce a monthly report listing the Generation Resources that are not meeting AVR availability for periods in which they are required to provide Voltage Support Service (VSS) as described in paragraph (4) of Protocol Section 3.15.3, QSE Responsibilities Related to Voltage Support.  The AVR criteria are described in paragraph (2) of Section 2.7.4.1, Maintaining System Voltage.

9.1.10	Current Operating Plan Metrics for QSEs

(1)	ERCOT shall report when a seven day Current Operating Plan (COP) has not been provided by the representing QSE for a Resource by 1500 each day. An event occurs when a QSE has not provided at least 153 hours of a Resource’s Operating Plan to ERCOT by 1500.  This report will be prepared monthly and posted on the MIS Secure Area.

(3) 	ERCOT shall report in the Day-Ahead when the reserved capacity of a QSE’s Resources in the Operating Day COP at 1430 is not sufficient to supply Ancillary Service requirements for the upcoming Operating Day.  ERCOT shall provide a monthly summary of the total days failed and total hours analyzed when the Resource’s reserves are insufficient for any hours during an Operating Day pursuant to paragraph (2) of Protocol Section 8.1.2, Current Operating Plan (COP) Performance Requirements, and not excused due to exemptions contained in the Protocols.

9.2	TSP Monitoring Program

9.2.1	Rating Methodology Reporting Program

Transmission Service Providers (TSPs) shall submit their equipment rating methodology to ERCOT in the first quarter of each year or provide notice to ERCOT that the rating methodology has not changed since the previous year.  ERCOT shall keep records describing the rating methodologies of each TSP in the ERCOT Region. ERCOT shall prepare an annual report summarizing these rating methodologies and submit this information to the Texas Regional Entity (TRE) and the appropriate Technical Advisory Committee (TAC) subcommittee.

9.2.2	Real-Time Data Monitor

ERCOT shall produce reports describing Real-Time data performance of TSPs in the following areas.  ERCOT shall post the summary report on the Market Information System (MIS) Secure Area.  Individual point performance shall be posted to the MIS Certified Area.

(a)	Telemetry Performance:

(i)	ERCOT shall produce quarterly reports describing telemetry performance as defined in the Protocols and the TAC approved Telemetry Standards.

(b)	Communication System Performance:

(i)	ERCOT shall produce monthly reports describing the reliability of each participant’s Inter-Control Center Communications Protocol (ICCP) data link to ERCOT as defined in the Protocols and the TAC approved Telemetry Standards.

(ii)	ERCOT shall produce monthly reports describing ICCP link up/down statistics.

9.2.3	Transmission Outage Reporting

This Section describes the reporting data for the transmission Outage scheduling and is provided for informational purposes.  There are no performance metrics for this data.

(1)	ERCOT shall post a monthly report of Outages considered on the MIS Secure Area including:

(a)	Number of Outage requests submitted in the ERCOT Outage Scheduler greater than 335 days (11 months) in advance;

(b)	Number of Outage requests submitted in the ERCOT Outage Scheduler between 90 and 334 days in advance of the desired Outage date;

(c)	Number of Outage requests submitted in the ERCOT Outage Scheduler between eight and 89 days in advance of the desired Outage date;

(d)	Number of Outage requests submitted in the ERCOT Outage Scheduler between three and seven days in advance of the desired Outage date;

(e)	Number of Outage requests submitted less than three days in advance;

(f)	Number of Outages by Outage type; and

(g)	Total Number of Outages that were requested, accepted, approved, cancelled, and withdrawn.

(2)	ERCOT shall post reports for each transmission owner showing the percentage of the total number of Outages, by type, described in paragraph (1) above.

9.2.4	Transmission Service Provider (TSP) Network Operations Model Update Implementation Monitor

(1)	ERCOT shall prepare monthly reports summarizing the Network Operations Model updates  by TSPs.  The report shall include the

number of Notices of interim reports submitted to the Public Utility Commission of Texas (PUCT) and the Independent Market Monitor (IMM) as accumulated by TSP due to the actions or inactions of the associated TSP for Network Operations Model Change Requests (NOMCRs) not meeting the time line pursuant to Protocol Section 3.10.1, Time Line for Network Operations Model Change Requests.  These reports shall be delineated by category and owner including reasons for the interim updates.  Interim updates caused by ERCOT (i.e. server unavailability, Network Model Management System (NMMS) component failure, site failover, loss of data, staff overload, weather, etc.) shall be reported and attributed to ERCOT.

(2)	ERCOT shall post reports on the MIS Secure Area.

9.2.5	 Backup Control for TSPs

- ERCOT shall produce a report identifying the date of backup control plan testing and the success or failure of that test for  TSPs.  Backup control plan tests shall be conducted at least annually.

9.3	ERCOT Monitoring Program

9.3.1	Transmission Control

(1)	ERCOT shall prepare a report describing the operating violations on every transmission line and autotransformer operated at voltages greater than 60 kV.  These reports shall be provided to the Texas Regional Entity (TRE) and the appropriate Technical Advisory Committee (TAC) subcommittee.

(a)	ERCOT shall prepare monthly reports on the following:

(i)	Exceedance of system operating limits or power transfer limitations set by ERCOT to guard against post-contingency stability exceedance for over ten minutes;

(ii)	N-1 exceedance of the applicable equipment ratings provided by Transmission Service Providers (TSPs), Qualified Scheduling Entities (QSEs), and Resources for over 30 minutes; and

(v)	N-0 exceedance detected during the month, the time duration of the loading above the applicable operating rating and the maximum exceedence level that occurred.

(2)	Other transmission monitoring and control metrics:

(a)	ERCOT shall prepare reports describing the number of Forced Outages by transmission owner by month.

(b)	ERCOT shall post on the Market Information System (MIS) Secure Area its performance in processing Outage requests in accordance with the following table including justification for rolling Outages from one timeline to another:

| Amount of time between the request for approval of the proposed Outage and the scheduled start date of the proposed Outage: | ERCOT shall approve or reject no later than: |
| --- | --- |
| Three days | 1800 hours, two days before the start of the proposed Outage |
| Between three and eight days | 1800 hours, three days before the start of the proposed Outage |
| Between nine days and 45 days | Four days before the start of the proposed Outage |
| Between 46 and 90 days | 40 days before the start of the proposed Outage |
| Greater than 90 days | 75 days before the start of the proposed Outage |

(c)	ERCOT shall post on the MIS Secure Area, by transmission owner, the number of rejection Notices.  The report will include specific concerns that caused the rejection.

(d)	ERCOT shall post on the MIS Secure Area, by transmission owner, the number of withdrawals of approved Outages.

(e)	ERCOT shall post on the MIS Secure Area, by transmission owner, the number of approved Outages that were rescheduled by ERCOT.

(3)		State Estimator (SE) Performance:

(a)	ERCOT shall report State Estimator (SE) performance in accordance with the Protocols and the TAC-approved SE standard and post such report on the MIS Secure Area.

(i)	ERCOT shall produce monthly reports describing SE convergence and valid SE solution rates as described in Protocol Section 3.10.9.2, Telemetry and State Estimator Performance Monitoring.

(ii)	ERCOT shall produce monthly reports describing the MW differences between SE results and power flow results for identified congested Transmission Elements as approved by TAC.

(iii)	ERCOT shall produce monthly reports describing the MW differences between the SE results and telemetry for identified congested Transmission Elements as approved by TAC.

(iv)	ERCOT shall produce monthly reports describing the voltage differences between the SE results and telemetry for the voltage critical buses identified in accordance with the SE standard.

(v)	ERCOT shall produce monthly reports describing the MW differences as defined in the SE standard.

(vi)	ERCOT shall produce monthly reports identifying the sum of MW flows around telemetered Electrical Buses as described in paragraph (5) of Protocol Section 3.10.7.5.2, Continuous Telemetry of the Real-Time Measurements of Bus Load, Voltages, Tap Position, and Flows.

9.3.2	System and Resource Control

The following reports shall be posted on the MIS Secure Area:

(1)	Resource Control Metrics:

(a)	Total Regulation Up (Reg–Up) Service and Regulation Down (Reg-Down) Service per interval - ERCOT shall develop a monthly report detailing the total amount of Reg-Up energy deployed in the Settlement Interval and by hour and the total amount of Reg-Down energy deployed for each Settlement Interval and by hour of the Operating Day.

(2)	Reserve Monitoring:

(a)	ERCOT shall prepare monthly reports describing the dates and associated duration that ERCOT operated without sufficient operating reserves as defined in the Protocols.

(3)	Reliability Unit Commitments (RUCs) and Deployments:

(a)	For each month, ERCOT shall report, Generation Resources committed in each RUC process, the reason for the commitment, Resource name and intervals deployed, and the hours committed for Voltage Support Service (VSS).

(4)	Dynamically Scheduled Resource (DSR) Performance and Resource with Output Schedule:

(a)	ERCOT shall produce monthly reports for DSR Load signal failures of more than five minutes.

(b)	ERCOT shall produce monthly reports noting the number of Resources on Output Schedule, by QSE.

(5)	Reversal of Base Point Instructions to Generation Resources from Interval to Interval:

(a)	ERCOT shall record and report, on a monthly basis, instances of Dispatch Instructions to Resources not providing Regulation Service in which there is a directional change in Base Point instructions for four consecutive Security-Constrained Economic Dispatch (SCED) intervals for validation and review.

(6)	ERCOT-wide Governor Response to Measurable Events:

(a)	ERCOT shall develop monthly reports detailing ERCOT’s System-wide governor response to each Measureable Event.  ERCOT shall meet at all times the governor response criteria as described in Protocol Section 8.5.2, Primary Frequency Control Measurements.

9.3.3	Forecasting

(1)	ERCOT shall report the Mean Absolute Percent Error (MAPE) each month for the following:

(a)	The accuracy of each day’s hourly system Load forecast posted at 0600 in the Day-Ahead of the Operating Day as compared with the actual average ERCOT Load for each hour of the Operating Day;

(b)	Accuracy of the system hourly Load forecast used for Day-Ahead Reliability Unit Commitment (DRUC) compared to the actual average ERCOT Load for each hour of the Operating Day; and

(c)	The accuracy of the hourly Load forecast for the following items compared to the average of the State Estimated Load at each Electrical Bus with a modeled Load for each hour:

(a)	Hourly Load forecast used in the DRUC by Weather Zone;

(b)	Hourly Load forecast used in the Hourly Reliability Unit Commitment (HRUC) by Weather Zone; and

(c)	The accuracy of the Load forecast used in the DRUC for the largest MW and MVA differences between the hourly Bus Load Forecast and the Real-Time Load at each Electrical Bus, by Weather Zone.

(2)	ERCOT shall prepare monthly reports detailing:

(a)	Day-Ahead forecast wind output, by Load Zone, by hour for the Operating Day, used in DRUC and at 0600 at the Day-Ahead.

(b)	Actual wind output, aggregated by Load Zone, by hour of the corresponding Operating Day by Load Zone.

9.3.4	System Operating Constraints

ERCOT shall report the following comparisons with respect to Real-Time operations as they occur:

(a)	Three consecutive days of a particular congestion constraint (constraints passed to SCED from Network Security Analysis (NSA)) in Real-Time not identified in the Day-Ahead Market (DAM);

(b)	Two consecutive hours of a particular congestion constraint in Real-Time not identified in the HRUC process; and

(c)	Two consecutive days of a particular congestion constraint in Real-Time not identified in the DRUC process.

9.3.5	Network Operations Model Update Implementation Statistics

ERCOT shall report monthly on the Network Operations Model update implementation statistics.  The report shall include the total number of Network Operations Model Change Requests (NOMCRs) submitted, approved, rejected, withdrawn and any other status contained in the Protocols or defined by ERCOT.

9.3.6	Back-up Control Plan

ERCOT shall report, by exception, the Entities that have not submitted back-up plans by the end of the first quarter of each year in accordance with the following:

(1)	ERCOT’s back-up plan as required by paragraph (2) (h) of Protocol Section 8.2, ERCOT Performance Monitoring;

(2)	QSE back-up plans submitted to ERCOT pursuant to Section 3.2.1, Operating Obligations; and

(3)	TSPs’ back-up plans submitted to ERCOT and the date that the plan was last updated pursuant to Section 3.9, Transmission Operators.

9.3.7	ERCOT Black Start Plan

ERCOT shall develop an annual Black Start Plan and post it on the MIS Certified Area.

9.3.8	Computer and Communication Systems Real-Time Availability and Systems Security

(1)	ERCOT shall report each month the availability of its computer and communications systems to the appropriate TAC subcommittee.  This report shall include availability statistics for ERCOT’s Inter-Control Center Communications Protocol (ICCP) data links.

(2)	ERCOT shall report each month the number of times a SCED run was requested but failed to provide a valid result in less time than normal SCED interval.

(3)	ERCOT shall also report each month the number of times the execution of SCED is terminated, by either manual action of the operator or program failure.  The number of times of each event will be posted on the MIS Certified Area.

9.3.9	Voltage and Reactive Control Performance Monitoring

ERCOT in coordination with the TSPs shall conduct studies to determine the nominal voltage set points across the ERCOT System  for all Electrical Buses used for voltage support and shall post all Voltage Profiles annually on the MIS Secure Area.

Transmission owners shall provide switching plans for automatically controlled reactors, capacitors, and other reactive controlled sources to ERCOT.  For manually switched reactive devices, the transmission owner shall provide its guidelines for the operation of these devices.  These plans and guidelines shall be posted annually to the MIS Secure Area and must be provided in accordance with the NOMCR or other prescribed process.

9.4	Ancillary Services Monitoring Program

ERCOT shall monitor Ancillary Service energy deployment according to the criteria outlined in Protocol Section 8, Performance Monitoring.  Reports required by Protocol Section 8.1.1.4, QSE Ancillary Service Energy Deployment Monitoring Criteria, will be posted on the Market Information System (MIS) Certified Area.

9.4.1	Hydro Responsive Testing

ERCOT shall produce monthly reports of hydro responsive tests and verify results submitted.

9.4.2	Black Start Service Requirements for TSPs, QSEs, and Generation Resources

ERCOT shall produce annual reports containing record(s) of if and when a Black Start Plan was last received at ERCOT including Resources participating in Black Start Service and Transmission Service Provider (TSP) name and the date the plan was received.  The Resource participating in Black Start Service and TSPs shall provide an updated Black Start Plan to ERCOT by the end of the 1st quarter and when the plan has changed.

9.4.3	Resource-Specific Responsive Reserve Performance

ERCOT shall develop monthly reports detailing Resource-specific Responsive Reserve (RRS) performance during deployments, including Load Resources, based on criteria described in Protocol Section 8.1.1.4.2, Regulation Service and Generation Resource Energy Deployment Performance.

9.4.4	 Constant Frequency Control

ERCOT shall survey Qualified Scheduling Entities (QSEs) qualified to provide Regulation Services to attest that the QSEs have the capability to operate in constant frequency control mode.  ERCOT shall post to the MIS Secure Area annual summaries of QSEs surveyed.

Option Two:  07/09/09 ERCOT Comments

9.4.5	Resource-specific Non-Spinning Reserve

ERCOT shall develop monthly reports detailing Resource-specific Non-Spinning Reserve (Non-Spin) performance during deployments, including Load Resources, based on the criteria described in Protocol Section 8.1.1.4.3, Non-Spinning Reserve Service Energy Deployment Criteria.