| OGRR Number | 226 | OGRR Title | Generation Resource Response Time Requirement |
| --- | --- | --- | --- |

| Date | September 25, 2009 |
| --- | --- |

| Submitter’s Information |  |
| --- | --- |
| Name | Barbara Clemenhagen on behalf of the Wholesale Market Subcommittee (WMS) |
| E-mail Address | bclemenhagen@topazpowergroup.com |
| Company | Topaz Power Group |
| Phone Number | (512) 314-8621 |
| Cell Number | (512) 670-6702 |
| Market Segment | Independent Generator |

Comments

At its August 13, 2009 meeting, ROS requested input from the QSE Managers Working Group (QMWG) regarding the language related to the Generation Resource staffing requirement and the language issues in paragraph (2)(b) of Section 3.1.4.9, Generation Resource Response Time Requirements, in regards to a possible inadvertent removal of Private Use Networks.  At the September 16, 2009 WMS meeting, QMWG leadership explained that QMWG discussed Operating Guide Revision Request (OGRR) 226 at its meeting on September 14, 2009.   QMWG leadership expressed support for OGRR226 as recommended for approval by the Operations Working Group (OWG) in its 9/15/09 OWG Recommendation Report.  It was also noted that QMWG did not address the language issues in paragraph (2)(b) of Section 3.1.4.9 due to the absence of subject matter experts at the QMWG meeting.

After discussion, WMS unanimously voted to endorse OGRR226 as reflected in the 9/15/09 OWG Recommendation Report (language below).

Revised Proposed Guide Language

- 3.1.4.9  	Generation Resource Response Time Requirements

This Section shall be effective for all QSEs representing a Generation Resource as of April 1, 2010, if feasible.  If compliance by July 1, 2010 is not feasible then the Generation Resource or the QSE representing the Generation Resource, shall submit, by February 1, 2010, a plan for compliance to ERCOT.  The compliance plan for paragraph (1) below may include TSP supervisory control of the needed Breaker if mutually acceptable by the generator and the TSP.

(1) 	All Generation Resources providing Voltage Support Service (VSS) as described in Protocol Section 6.5.7, Voltage Support Service, shall maintain the necessary procedures and processes plus communications, telemetry, remote control, automation, and staffing in order to normally comply with the response times listed below when a valid VSS Dispatch Instruction as described in Protocol Section 6.7.6, Deployment of Voltage Support Service, is given to the Generation Resource or the QSE representing a Generation Resource. Compliance is based upon normal operating conditions where VSS Dispatch Instructions respect all equipment operating limits and other restrictions that are periodically placed on equipment according to Protocol Section 5.2.3, Equipment Operating Limits.  The response time to valid instruction shall commence with the successful receipt by the appropriate Entity.

In a Force Majeure Event, as defined in paragraph (1) (c) of Protocol Section 5.4.3, Dispatch Instruction Procedure, compliance with VSS Dispatch Instructions shall not be required.  TSPs and QSEs shall follow Dispatch Instructions for VSS except under those exemptions described in paragraph (1) and (2) of Protocol Section 5.4.4, Compliance with Dispatch Instructions.

The required VSS response times for a Generation Resources are:

(a)	For automated response, when voltage or reactive measurements at the point of interconnection are outside of the limits as specified in paragraph (4) of Protocol Section 6.10.9, Reactive Power Supply from Generation Resources Required to Provide VSS Performance Criteria; then the response (within the operating Reactive Power capability  of the generator) must be sufficient to initiate response in no more than one (1) minute and return the measurement within the required range in no more than five (5) minutes.

(b)	Response to valid VSS Dispatch Instructions from ERCOT or authorized VSS Dispatch Instructions from a TSP, within the Reactive Power capability of the generator shall be completed in no more than five (5) minutes of the receipt of a valid instruction;

(2) 	Shutting down and disconnecting Generation Resources from the ERCOT System:

(a)	On-line Generation Resources must be able to commence their shutdown sequence within five (5) minutes of receipt of a valid instruction from ERCOT.  Nuclear fueled Generation Resources shall comply with the procedural requirements of the Nuclear Regulatory Commission (NRC) when receiving instructions from ERCOT to disconnect the Generation Resource from the ERCOT Transmission Grid.

(b)	If system conditions require, Breaker or switch operations needed to disconnect a non-MW producing generator  from the system  shall be completed as soon as practical, but no longer than fifteen (15) minutes of the receipt of a valid instruction from ERCOT.  Once disconnected from the ERCOT Transmission Grid, a Generation Resource shall complete as soon as practical, but no longer than fifteen (15) minutes, the required switching to return the system to a normal configuration except under those exemptions described in paragraph (1) and (2) of Protocol Section 5.4.4.  Nuclear fueled Generation Resources shall comply with the procedural requirements of the NRC when receiving instructions from ERCOT to disconnect the Generation Resource from the ERCOT Transmission Grid.