ERCOT Protocols

Section 7:  Congestion Management

November 1, 2009

7	Congestion Management	7-1

7.1	Overview of ERCOT Congestion Management	7-1

7.2	CSC Zone Determination Process	7-2

7.3	Congestion Management for CSCs/Zonal Congestion	7-10

7.4	Congestion Management for Local Congestion	7-14

7.5	Transmission Congestion Rights	7-25

7.6	Incorporation of Reliability Must-Run and Out of Merit Order Resources	7-35

7.7	Remedies to Congestion Through Transmission Expansion	7-35

7.8	[RESERVED]	7-35

7.9	Trading Hubs	7-36

# Congestion Management

7.1	Overview of ERCOT Congestion Management

ERCOT employs a Zonal Congestion management scheme that is flow-based, whereby the ERCOT Transmission Grid, including attached Generation Resources and Load, will be divided into a predetermined number of Congestion Zones.  Each Congestion Zone is defined such that each Generation Resource or Load within the Congestion Zone boundaries has a similar effect on the loading (Shift Factor) of Transmission Facilities between Congestion Zones.  For purposes of solving Zonal Congestion the Shift Factor will be assumed the same for: (i) all Generation Resources deemed likely to vary their output and (ii) Loads within a Congestion Zone.  Therefore any imbalance between Loads and Generation Resources in a Congestion Zone will be deemed to have the same impact on a given loading between Congestion Zones.

This Congestion management scheme applies zonal Shift Factors, determined by ERCOT, to predict potential Congestion on Commericially Significant Constraints (CSCs) under the known topology of the ERCOT System.  The zonal Shift Factors determined by ERCOT should most closely represent the effect of Generation Resources deemed likely to vary their output and Loads in the Congestion Zone on the CSCs with the current topology of ERCOT System.  This scheme is used in the Day Ahead and Adjustment Periods to evaluate potential Congestion and notify Market Participants accordingly.  ERCOT also uses this scheme, along with other factors, to determine if it should procure Replacement Reserve Service in a Congestion Zone to provide additional Balancing Energy Service to solve expected Congestion.  ERCOT will use monthly zonal Shift Factors to post and analyze the Schedule impact on CSCs until it implements calculations of interval Shift Factors in the system.

ERCOT will manage transmission Congestion and categorize the cost of Congestion management as either Zonal Congestion management costs or Local Congestion management Costs.  Zonal Congestion management costs are those costs attributable to managing Congestion on CSCs or predefined Closely Related Elements (CRE).  The costs of managing Zonal Congestion will be directly assigned to Qualified Scheduling Entities (QSEs) based on the impact of each QSE’s portfolio on CSCs.  All other Congestion management costs are considered Local Congestion management costs.

ERCOT will use the Zonal Congestion management model with Shift Factors of Generation Resources and Loads on CSCs and zonal Balancing Energy Service Market Clearing Price for Energy (MCPE) to determine the Shadow Prices of energy across corresponding CSCs.   In addition, ERCOT will directly assign to QSEs the costs of any Replacement Reserve Service (RPRS) procured on a zonal basis for Congestion management purposes.  ERCOT will use these Shadow Prices in settlement to directly assign to QSEs the cost of managing Zonal Congestion to QSEs.

ERCOT will set the Shadow Price cap at a level that ERCOT calculates to be in alignment with the prevailing system-wide offer cap defined in paragraph (g)(6) of PUCT Subst. R. 25.505, Resource Adequacy in the Electric Reliability Council of Texas Power Region, and Section 6.11, System-Wide Offer Caps.   In some cases, such as where two (2) or more CSCs are simultaneously binding, MCPEs may be generated that are at levels higher than the prevailing system-wide offer cap or lower than the Balancing Energy offer floor.  When that occurs, ERCOT shall make Real Time or ex-post adjustments to zonal MCPEs to ensure that MCPEs do not exceed the system-wide offer cap or fall below the Balancing Energy offer floor, and adjust CSC Shadow Prices accordingly.  ERCOT shall describe the ERCOT Board-approved process for making price adjustments in a Market Operations Bulletin that will be posted on the Market Information System (MIS) Public Area.   ERCOT shall send a Notification to Market Participants informing them of any change to the Shadow Price cap at least ten (10) days prior to implementation of the new system-wide offer cap.

The Local Congestion management scheme relies on a more detailed Operational Model to determine how each particular Resource or Load impacts the transmission system.  This model does not use portfolios and uses the current known topology of the transmission system.

ERCOT will Uplift the cost to solve Local Congestion pro rata to each QSE based on its Load Ratio Share.

ERCOT will manage Congestion by:

(1)	Evaluating the levels of Zonal Congestion and any other Congestion during the Day Ahead, the Adjustment Period and the Operating Period using appropriate models of the ERCOT Transmission Grid;

(2)	Examining the impacts of QSE energy schedules on CSCs;

(3)	Posting on the MIS, the total megawatt quantity impacts on every CSC, and allowing QSEs to adjust schedules to mitigate potential Congestion on the CSCs;

(4)	Procuring during the Adjustment Period, as needed, RPRS to use with other Resources for which QSEs have submitted Balancing Energy bids to provide sufficient capacity for Balancing Energy flows in the Operating Hour while respecting operational limits of the ERCOT Transmission Grid;

(5)	Determining settlement for QSEs providing RPRS procured to manage Congestion; and

(6)	Determining settlement for QSEs providing Balancing Energy associated with resolving Zonal Congestion.

ERCOT will carry out these steps in accordance with this Section and the scheduling and Ancillary Service scheduling and selection requirements in Section 4, Scheduling, and Section 6, Ancillary Services, respectively.

7.2	CSC Zone Determination Process

By November 1 of each year, the appropriate ERCOT subcommittee will report to the Technical Advisory Committee (TAC) and ERCOT Board with recommended Commercially Significant Constraint (CSC) designations, resulting Congestion Zone boundaries with any granted exemptions noted, Closely Related Element (CRE) designations and associated Boundary Generation Resources for ERCOT Board review and approval.  Shortly after ERCOT Board approval, CSCs and resulting Congestion Zone boundaries and CRE designations will be posted on the Market Information System (MIS).  This posting will include a station and bus-by-bus identification of each Congestion Zone.

7.2.1	Process for Determining CSCs

CSCs and resulting Congestion Zones will be reassessed annually by November 1 of each year.  ERCOT will use the following process to identify CSCs:

(1)	By August 1, of each year, ERCOT will complete an analysis of Load flow data using the latest Steady State Working Group (SSWG) Data Set A summer peak case and will determine expected operating limits, candidate CSCs and associated constraints to be used in the designation of CSCs for the upcoming calendar year using the following steps:

(a)	Determination of new CSC candidates:

(i)	Candidate CSCs may be determined using transfer analysis between Study Zones.  Study Zones are groupings of buses used to test transfer limits using power system simulation tools.  These Study Zones can be either of the following: (1) reasonable approximations of the final Congestion Zones, Weather Zones, SSWG Areas or Zones; or (2) any grouping of buses by geographic region or generation/Load pocket.  Candidate CSCs shall be:  (1) the contingency that causes limiting elements to reach one hundred-percent (100%) of their emergency rating at high transfer levels; (2) the transmission line that is the limiting element under contingencies; or (3) a stability interface limit.

(ii)	Candidate CSCs may also be identified through an evaluation of the actual annual Congestion costs from the prior year.  For this purpose, Congestion costs include the cost of any zonal Balancing Energy Service, Reliability Must-Run (RMR) Service, Out of Merit Capacity (OOMC) Service, Replacement Reserve Service (RPRS), Out of Merit Energy (OOME) Service, or Resource-specific Dispatch Instructions used to resolve Congestion on that particular transmission path in that particular direction during the prior year.  Candidate CSCs may also be identified by considering projected annual Congestion costs for the CSC designation year.  Since this type of data reflects past system topology and generation, generation and transmission system changes expected to be in service in the CSC designation year shall also be considered.

(iii)	Candidate CSCs may also be identified based on ERCOT’s operational experience.

(b)	If no new candidate CSCs are considered other than the immediately preceding year’s CSCs, then ERCOT shall use the Congestion Zones from the immediately preceding year as Study Zones.

(c)	If new candidate CSCs are considered, then ERCOT shall calculate Shift Factors and perform cluster analysis based on the candidate CSCs to create Study Zones.

(d)	ERCOT shall perform system simulation studies with transfers between the Study Zones and find constraints on the transmission system to determine if candidate CSCs are appropriate for designation as CSCs.

(e)	ERCOT shall recommend whether the candidate CSCs qualify for CSC designation.  The following factors shall be considered:

(i)	Whether there is a sufficiently competitive market to resolve Congestion on the transmission path to be considered for CSC designation.  One factor that will be considered when assessing whether a sufficiently competitive market exists is the concentration of capacity ownership or control by Market Participants or their Affiliates, compared to the distribution of Shift Factor impacts;

(ii)	Whether the candidate CSC is an adequate indicator to be used to operationally manage and measure inter-zonal transfers; and

(iii) 	Whether deployment of zonal balancing will be effective for managing the post-contingency flow on the limiting element.

(2)	ERCOT shall present all candidate CSCs and proposed Congestion Zones to the appropriate TAC subcommittee.  ERCOT shall identify the candidate CSCs and proposed Congestion Zones it believes are the most appropriate CSCs and proposed Congestion Zones and shall explain the basis for its position.  ERCOT’s justification shall include any operational or reliability concerns that it has identified with any candidate CSCs and proposed Congestion Zones.

(3)	CSC approval process:  The appropriate TAC subcommittee will review the results and the process followed above to determine the list of constraints to be recommended for approval to the TAC.  TAC shall then make a recommendation to the ERCOT Board.

7.2.2	Congestion Zone & Zonal Shift Factor Determination Methodology

ERCOT will take the following steps to determine Congestion Zones and Zonal Shift Factors:

(1)	Shift Factors are developed using a linearized (Direct Current (DC)) model to identify the impact of each transmission bus on each CSC relative to an ERCOT reference bus.

(2)	A statistical clustering analysis will be used to aggregate transmission buses into zones based upon similar Shift Factors relative to all CSCs.  The clustering must meet the following criteria: (i) each CSC must straddle a zonal boundary (however, not every zonal boundary need be straddled by a CSC); and (ii) station IDs as provided by Transmission and/or Distribution Service Providers (TDSPs) in Section 15.1.1.5, Response from TDSP to Registration Notification Request, can be assigned only into one (1) Congestion Zone.

The following measurements will be calculated for each combination of zones: (i) the maximum difference between the Zonal Shift Factor and the Shift Factor of any generation bus within the zone; and (ii) the R2 (coefficient of determination) from the cluster analysis.

The system model used to conduct the clustering analysis required by this Section shall be either a pre-contingency or post-contingency system model.  A pre-contingency system representation in the model shall be used when the candidate CSC is the contingency element or a stability interface limit.  A post-contingency system representation in the model shall be used when the candidate CSC is the limiting element.

(3)	If the clustering process described in paragraph (2) above, results in the splitting of two (2) or more buses within any single station into different zones, then the following process will be used to adjust the zone selection to place all buses within a single station in the same zone:

(a)	For Generation Resource stations, determine which zone contains the largest amount of generating capacity of the station and assign all buses of that station to that zone;

(b)	For all other stations, determine which zone is assigned the largest total amount of Customer Load of the buses in the station and assign all buses of that station to that zone;

(c)	If there is no Load or generation in the station, or if the amount of generation and/or Load assigned to individual station buses are equal, then the Load or generation shall be assigned to the zone containing the lowest bus number of the station.

(4)	Zonal Shift Factors for each CSC will be determined by averaging the individual bus Shift Factors within each Congestion Zone for the corresponding CSC, weighted by the megawatt levels on the bus in the Load flow base case used to determine CSC Limits from Generation Resources deemed likely to vary their output.

The appropriate TAC subcommittee will review the results and the process followed above to determine the Congestion Zones to be recommended for approval to the TAC.  TAC shall then make a recommendation to the ERCOT Board.

(5)	Thirty (30) days prior to TAC review, a Non Opt-In Entity (NOIE) may request an exemption to its zonal placement by submitting a request to ERCOT for consideration.  ERCOT will evaluate the merits of the request and authorize the exemption based on the following criteria:

(a)	Exemptions shall not cause significant operational impact to the ERCOT System.  This will normally be satisfied if the request will not result in a change in the annual average Shift Factor for the original or requested Congestion Zone by more than five-percent (5%).  After the proposed move, the Zonal Shift Factor in each of the original and requested Congestion Zones shall be no less than ninety-five percent (95%), and no more than one hundred and five percent (105%), of its original Zonal Shift Factor before the proposed move.

Because only buses with generation have an impact on the calculation of Zonal Shift Factors, if no generation exists at a bus that is part of an exemption request, a token generator of one (1) MW will be placed at that bus to test the impact of moving that bus from one zone to another.  This will be done to all buses that make up a request for exemption.  For example, if an Entity is requesting an exemption for five (5) buses and only one (1) bus has generation On-line in the Load flow case used to test the impact, one (1) MW of generation will be placed at the four (4) other buses and the impact of all five (5) buses on Zonal Shift Factors will be determined as a group as opposed to individually.  The Load flow case used to determine this impact will be the same Load flow case used to calculate annual Transmission Congestion Right (TCR) quantities.

(b)	Exemptions shall not have a commercial impact/gain for the requestor.  This will be measured by whether or not the scheduling which would otherwise result would be eligible for Pre-assigned Congestion Rights (PCRs).

7.2.3	Determining Closely Related Elements (CREs)

ERCOT shall evaluate each contingency element/limiting element pair (constraint) combination to determine if deployment of zonal Balancing Energy will be effective for managing the post-contingency flow on the limiting element.  Based on this evaluation, ERCOT shall develop proposed CREs and a list of contingencies for managing CSC and CRE related Congestion.  ERCOT shall present all proposed CREs and a list of contingencies for managing CSC and CRE related Congestion, including justification for their inclusion, to the appropriate ERCOT TAC subcommittee for approval.  For each year, ERCOT shall identify potential CREs using the following process:

(1)	Using the same Load flow data and flow limits used to determine CSCs, determine the transfer capability between Congestion Zones that are Terminal Zones for CSCs.  “Terminal Zones” means the pair of zones containing the endpoints of the CSC for which a particular candidate CRE is proposed.

(2)	Identify contingency/limiting element pairs that are binding for transfers between zones for up to one hundred fifty percent (150%) of the transfer capability identified in paragraph (1) above.

(3)	The limiting elements that meet the CRE criteria below for all contingency/limiting pair elements may be submitted to TAC for approval.

ERCOT may propose additional CREs as provided under this Section, provided the results of the tests below are submitted as well as a description of a need for the CRE.  If a proposed CRE fails any of the tests below, ERCOT shall provide an explanation of the reason ERCOT believes the proposed element should nevertheless be designated a CRE.

7.2.3.1	CRE Criteria

Each of the following criteria will use the post-contingency generation Shift Factors of the candidate contingency/limiting element pairs.  In order for a CRE to pass, all constraints identified in Section 7.2.3, Determining Closely Related Elements (CREs), associated with the CRE should pass the tests below.

7.2.3.1.1	Power Transfer Distribution Factor (PTDF) Threshold

(1)	The Power Transfer Distribution Factor (PTDF) measures the impact of power flow between a pair of locations on a particular transmission element.

(2)	“Terminal Zones” means the pair of zones containing the endpoints of the CSC for which a particular candidate CRE is proposed.

(3)	Determine the weighted average PTDF between the Terminal Zones of the candidate CRE by calculating the Shift Factor difference between the source and sink zones.  The weighted averages will only consider generation technologies that would be responsive to price signals.  For the source zone, the technologies excluded are:

(a)	Cogeneration;

(b)	Nuclear;

(c)	Hydro; and

(d)	DC Ties.

For the sink zone, the technologies excluded are:

(a)	Wind;

(b)	Cogeneration;

(c)	Nuclear;

(d)	Hydro;

(e)	Coal; and

(f)	DC Ties.

(4)	In order for the limiting element of the contingency limiting element pair to qualify as a CRE, the weighted average PTDF between the Terminal Zones for that element should be greater than 0.05, and the ratio of the PTDF to the line rating of the limiting element shall be greater than 0.0002 (e.g., the threshold PTDF for a line rated at 250 MVA is 250 * 0.0002 or 0.05).  However, if the weighted average PTDF between the Terminal Zones for the candidate CRE is greater than 0.2, then the candidate CRE would satisfy this criterion.

7.2.3.1.2	Generation Concentration

ERCOT shall determine the concentration of effective generation responsive to the constraint on both the incremental and decremental sides of the constraint.

(1)	Calculate the PTDF of all generators as the Shift Factor of the generator relative to the proposed CRE minus the generation weighted average Shift Factor of each CSC Terminal Zone relative to the proposed CRE.  For example, for a CRE candidate for the West to North CSC, the PTDFs on the CRE of all generators would be calculated relative to the North Zone for the decremental side of the CRE, and the PTDFs on the CRE of all generators would be calculated relative to the West Zone for the incremental side of the CRE.

(2)	For movable generation capacity (in any zone) equal to twice the CRE rating and having the greatest impact on the CRE on incremental side, calculate their generation weighted average PTDF with respect to the decremental Terminal Zone of the CSC.  This calculation is done by sorting the moveable generation capacity list from greatest to least absolute value PTDF on the CRE, summing the moveable capacity down to the first generator where the sum of the moveable capacity is just greater than twice the rating of the proposed CRE, and then calculating the generation weighted average PTDF for that amount of moveable capacity with respect to the decremental Terminal Zone of the CSC.  In the example above, the decremental zone is the West Zone.

(3)	For movable generation capacity (in any zone) equal to twice the CRE rating (using the same flow limits as for TCR modeling) and having the greatest impact on the CRE on decremental side, calculate their generation weighted average PTDF with respect to the incremental Terminal Zone of the CSC.  This calculation is done by sorting the moveable generation capacity list from greatest to least absolute value PTDF on the CRE, summing the moveable capacity down to the first generator where the sum of the moveable capacity is just greater than twice the rating of the proposed CRE, and then calculating the generation weighted average PTDF for that amount of moveable capacity with respect to the incremental Terminal Zone of the CSC.  In the example above, the incremental zone is the North Zone.

(4)	In order for the limiting element of the contingency limiting element pair to qualify as a CRE, incremental movable generation capacity equal to twice the CRE’s rating should not have a generation weighted average PTDF on the CRE greater than twice the zonal average PTDF on the CRE of the incremental Terminal Zone of the CSC.  Similarly, decremental movable generation capacity equal to twice the CRE’s rating should not have a generation weighted average PTDF on the CRE greater than twice the zonal average PTDF on the CRE of the decremental Terminal Zone of the CSC.

7.2.3.1.3	Boundary Generation Resources

ERCOT shall determine the impact error due to Boundary Generation Resources as follows:

(1)	For each generator, find the generator’s PTDF on the CRE to each Congestion Zone and select the Congestion Zone to which the generator’s PTDF on the CRE has the lowest absolute value;

(2)	If the Congestion Zone selected in paragraph (1) above is different from the generator’s designated Congestion Zone, then that generator is a Boundary Generation Resource;

(3)	Determine the impact error of a Boundary Generation Resource by multiplying the difference between the absolute value of the generator’s PTDF on the CRE to its designated Congestion Zone minus the absolute value of the generator’s PTDF on the CRE to the Congestion Zone selected in paragraph (1) above times its maximum capacity.

In order for the limiting element of the contingency-limiting element pair to qualify as a CRE, the sum of the impact errors of all Boundary Generation Resources should be less than or equal to thirty percent (30%) of the rating of the CRE (using the same flow limits as for TCR modeling).

7.2.3.1.4	CRE Correlation to CSC

ERCOT shall determine the correlation of the CRE to its designated CSC by calculating the percentage error in deployment as follows:

(1)	For the CRE and its CSC, determine their PTDFs for each pair of Congestion Zones using the method described in paragraph (3) of Section 7.2.3.1.1, Power Transfer Distribution Factor (PTDF) Threshold.

(2)	Calculate a scaling factor “A” by dividing the CSC PTDF for the CSC’s Terminal Zones by the CRE PTDF for the CSC’s Terminal Zones, i.e., A = (CSC PTDF)/(CRE PTDF), both PTDFs being for the CSC Terminal Zones.

(3)	Determine the error in deployment for each pair of Congestion Zones, Bp, by subtracting the product of A times the CRE PTDF from the CSC PTDF for the same pair of Congestion Zone, i.e. Bp = (CSC PTDF)p – A*(CRE PTDF)p, where p is one pair of Congestion Zones.

(4)	Determine the percentage error in deployment by dividing Bp by the CSC PTDF for the CSC’s Terminal Zones.

In order for the limiting element of the contingency limiting element pair to qualify as a CRE, the maximum absolute value of percentage error of deployment should be less than fifty percent (50%).

7.2.3.1.5	Intra-Year CRE Recommendation and Approval

During the effective year, ERCOT may propose modifications to the list of CREs or the approved list of contingencies for managing CSC and CRE related congestion, including the expected duration of those modifications, as needed to more closely represent actual system constraints that can be effectively resolved by zonal Balancing Energy deployments.  Any modifications to the list of CREs or the approved list of contingencies for managing CSC and CRE related congestion will not affect Congestion Zone definition or composition, nor will they affect CSC definitions.  ERCOT shall present modifications proposed to the list of CREs or the approved list of contingencies for managing CSC and CRE related Congestion after the start of the year to TAC for approval.  TAC will have seven (7) days to take action on the proposed modification.  If TAC takes no action within seven (7) days, the proposed modification shall be deemed approved.

7.2.4	Determining Generation Resources Deemed Likely to Vary Their Output

At the beginning of each year, ERCOT will compile a list of all Generation Resources fueled by nuclear fuel, coal or lignite.  ERCOT will exclude such units from the calculation of Zonal Shift Factors for the rest of the year.  All other Generation Resources will be deemed likely to vary their output.

7.3	Congestion Management for CSCs/Zonal Congestion

7.3.1	Determination of Zonal Congestion

ERCOT will analyze energy schedules to determine the existence and extent of Zonal Congestion as part of the Day Ahead Scheduling Process.

7.3.2	Resolution of Zonal Congestion

ERCOT will resolve Zonal Congestion by the following means:

(1)	Using adjusted Balanced Schedules received from Qualified Scheduling Entities (QSEs) after ERCOT’s posting of Commercially Significant Contraints (CSC) impacts greater than the CSC Limit, ERCOT will reassess the resulting level of Zonal Congestion.  ERCOT will take no further Zonal Congestion actions if the adjusted Balanced Schedules resolve initial Zonal Congestion.

(2)	If Zonal Congestion still exists following receipt of adjusted Balanced Schedules, ERCOT may procure Replacement Reserve Service (RPRS) in accordance with Section 6.6.3.2, ERCOT Ancillary Services Procurement during Adjustment Period (AP), provided that sufficient Resources are available to provide Balancing Energy in the Operating Period, in accordance with Section 6.6.3.2.  ERCOT will then balance the energy within the ERCOT System in the Operating Period respecting all operational limitations of the ERCOT System.

(3)	Zonal Balancing Energy can be deployed only for Congestion management purposes to resolve flow limit violations on CSCs and Closely Related Elements (CREs) where the base case or post-contingency limiting element is the CSC or CRE, unless an Emergency Condition is declared by ERCOT.  ERCOT must manage CSC and CRE related Congestion using the approved list of contingencies for managing CSC and CRE related Congestion.

(4)	Congestion is resolved in Real Time as follows:

(a)	Using Zonal Shift Factors, estimate zonal Balancing Energy Service deployments needed to maintain flows within CSC Limits.  If ERCOT applies any offsets to the zonal Balancing Energy Service deployments based on its operational judgment and experience, it will post a report on the Market Information System (MIS) listing the adjustment in MW, by zone, for each interval.

(b)	Based on item (4)(a) above, deploy unit specific instructions to manage Local Congestion.  The Resource specific instructions will be defined as instructed deviations using the following criteria:

(i)	When a Resource is instructed to operate at or above an instructed output level and the Resource Plan output level is below the instructed output level, the instructed deviation will be defined as the difference between the instructed output level and Resource Plan output level for the target interval.

(ii)	When a Resource is instructed to operate at or below an instructed output level and Resource Plan output level is above the instructed output level, the instructed deviation will be defined as the difference between the instructed output level and Resource Plan output level for the target interval.

(iii)	When a Resource is requested to go to an instructed output level, the instructed deviation will be defined as the difference between the instructed output level and the Resource Plan output level.

(iv)	Otherwise, the Resource specific deployment will not be defined as instructed deviation.

(c)	Clear the Balancing Energy Service market to maintain system power balance and the CSC flow within the CSC Limit.

(d)	In the event of a contingency, estimate Balancing Energy Service deployment to maintain flows within pre-second contingency CSC Limits and deploy unit-specific instructions to manage any resulting Local Congestion and clear the Balancing Energy Service market to maintain CSC flows within pre-second contingency CSC Limits.

(5)	The Shift Factors for CSCs and CREs shall be at the same value.

The result of Congestion Management should ensure that:

(1)	Balancing Energy Market Clearing Prices for Energy (MCPEs) are the same in all zones if no CSC or CRE is constrained.

(2)	Balancing Energy MCPEs will be different in two (2) or more Congestion Zones only if one (1) or more CSCs/CREs are constrained.

(3)	The Shadow Price of a constrained CSC will be calculated by taking the difference in Balancing Energy MCPEs between any two (2) zones divided by the difference in Zonal Shift Factors on the corresponding CSC of the two (2) zones even in the event of an Outage of the CSC.  If there is no feasible solution, then these criteria may not be met.

7.3.3	[Expired 2/15/2002]

7.3.3.1	[Expired 2/15/2002]

7.3.3.2	[Expired 2/15/2002]

7.3.3.3	[Expired 2/15/2002]

7.3.3.3.1	[Expired 2/15/2002]

7.3.4	Settlement of Zonal Congestion

7.3.4.1	Balancing Energy CSC Congestion Charge

The calculation for Zonal Congestion Management Charge is as follows:

ICSCiq	=	Σ ((QSSiqz - SOiqz) * SFzcsc )z

If ICSCiq > 0 then,

CSCBECSCiq	=	SPCSCi	* MAX (0, ICSCiq)

Else (counterflow)

CSCBECSCiq	=	SPCSCi	* ICSCiq

End

CSCBEiq	=	SUM (CSCBECSCiq)CSC

i	Interval being calculated

q	QSE

z	Congestion Zone

CSC	Commercially Significant Constraint

CSCBECSCiq 	CSC Energy Related Congestion Charge, per interval, per CSC, per QSE

CSCBEiq 	CSC Energy Related Congestion Charge, per interval of that QSE

SPCSCi    	Shadow Price per CSC, per interval

ICSCiq    	Scheduled MW Impact per CSC, per interval, per QSE

SFzcsc	Commercial Model Shift Factor of Generation Resources deemed likely to vary their output per CSC, per zone

QSSiqz	QSE Supply Schedule per interval, per zone, per QSE

SOiqz	Scheduled Obligation (prior to Real Time) per interval, per zone, per QSE

ERCOT will compute the Zonal Shift Factors for settlement purposes using Shift Factors of Generation Resources within the zone deemed likely to vary their output, weighted by the generation level in the monthly base case.  The monthly base case will be the most recent base case used by ERCOT but no less current than the base case used by ERCOT to determine the number of TCRs available for sale for the corresponding month.

7.3.4.2	Replacement Reserve Service Zonal Congestion Charge

Replacement Reserve Zonal Congestion costs will be allocated directly to the QSE impacting the CSC.  The Replacement Reserve Zonal Congestion costs per interval per QSE will be determined by multiplying the Shadow Price of CSC capacity by the QSE’s CSC impact and in that interval.

The CSC impact is determined by taking for each Congestion Zone, the maximum difference, in any of the four intervals in an hour, between each QSE’s Supply schedules and the QSE Obligation schedules at the time of each round of Replacement Reserve capacity procurement multiplied by the Zonal Shift Factor.  The calculation of the Replacement Reserve Service Zonal Congestion Charge will be as follows:

CSCRPhq	=	SPCCSCh* MAX (0, ICSChq))

ICSChq	=	MAX [0,MAX [Σ ((QSSTOPiqz – QOSTOPiqz) * SFzcsc)z ]n]

Where:

h	hour being calculated

i	interval within hour h

n	The number of times that RPRS is procured.

q	QSE

z	Congestion Zone

CSC	Commercially Significant Constraint

CSCRPiq 	CSC Replacement Reserve related Congestion charge, per interval, per QSE

SPCCSCi	Capacity Shadow Price per CSC, per interval

ICSCiq	Scheduled MW Impact per CSC, per interval, at the time of purchase, per QSE

SFzcsc	Zonal Shift Factor per CSC, per zone

QSSTOPiqz	QSE Supply Schedule per interval, per zone, at the time of purchase, per QSE

QOSTOPiqz	QSE Obligation Schedule per interval, per zone, at the time of purchase, per QSE

7.4	Congestion Management for Local Congestion

7.4.1	Determination of Local Congestion

ERCOT will use the Operational Model of the ERCOT System in combination with QSE energy schedules and ERCOT forecasted Load to determine the extent of Local Congestion.

When ERCOT identifies Local Congestion on a repeated basis ERCOT shall have procedures established to contact the appropriate TDSP to:

(1)	Verify that ratings of Transmission Facilities in the ERCOT Operational Model causing the event are current and correctly represented;

(2)	Verify, when the TDSP’s analysis results differ from those of ERCOT, that the configuration of the transmission system in the ERCOT Operational Model matches that in use by the TDSP.  To recognize operational time constraints, such verification will focus on transmission elements believed to have impacted the event; and

(3)	Mutually identify with the TDSP any additional operational intervention or system monitoring that could be implemented to manage recurring Congestion due to a recurring cause.

If it is determined that rating(s) in the ERCOT Operational Model or configuration of the Transmission Facilities are not correct, then the TDSP will provide, via submittal of a Service Request on the ERCOT MIS, written confirmation of the correction.  Once written confirmation is completed, ratings in the ERCOT Operational Model will be updated in the Real Time systems by the close of the next Business Day, and such rating changes and any needed configuration changes will be included in the next scheduled Operational Model database update.  Once additional operational intervention to the transmission system has been identified (if any) to relieve Congestion and mutually accepted by both ERCOT and TDSP operators, the necessary operational instructions shall be prepared and implemented as soon as practicable.

ERCOT will use an interim practice between the identification of a problem and the final authorized submittal of a Service Request in order to implement corrections as soon as practicable.  The interim practice will allow an authorized or alternate contact to provide corrected ratings to ERCOT by email.  ERCOT will enter that rating into the Real Time systems to be used in current operating activities on a temporary basis.  After the Service Request documentation is received by ERCOT, permanent changes will be made in the Operational Model and loaded into the operating systems.

7.4.2	Resolution of Local Congestion

ERCOT will procure RPRS if necessary to provide sufficient capacity to resolve forecasted Local Congestion.  ERCOT will purchase RPRS from the Resource(s) having the lowest cost per megawatt based on each Resource’s Resource Category Generic Cost and its ability to resolve the Congestion given its Shift Factor on the Local Congestion.

ERCOT will instruct and Dispatch Resources in Real Time in a manner that does not exceed the units’ operational limits known to ERCOT.  The Resources that have been instructed to operate within a range to resolve Local Congestion are paid or pay a price, in accordance with Section 7.4.3.1, Balancing Energy Up from a Specific Resource, Section 7.4.3.2, Balancing Energy Down from a Specific Resource, and Section 6.8, Compensation for Services Provided.

The Local Congestion Costs are embedded in each service deployed to resolve Local Congestion such as OOMC, OOME, and unit specific premiums.

The formula to calculate Local Congestion Replacement Reserve payment can be found in Section 6.8.1.11, Local Congestion Replacement Reserve Payment to QSE, of these Protocols.

7.4.3	Settlement of Local Congestion Costs

Net Local Congestion costs will be settled in the Balancing Energy settlement process.

7.4.3.1	Balancing Energy Up from a Specific Resource

(1)	In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, whenever a specific Resource with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Up, is deployed incrementally to solve Local Congestion, the QSE will be paid the difference between the MCPE of the Congestion Zone in which the specific Resource is located and the incremental bid premium specified.  Aggregated Unit rules as described in Section 6.8.2, Capacity and Energy Payments for Out of Merit Service, apply to Aggregated Units approved by ERCOT.

The calculation of the payment for Balancing Energy Up from a specific Resource is as follows:

LPCRSUquiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (MRqiuz -OLqiuz, IOLqiuz - OLqiuz)))

LPCRSUqviz	=	 -1 * Σ ((PMqivz – MCPEiz )* Max (0, Min (MRqivz -OLqivz, NETUEQivq)) * LBEAGRivq)

For gas-fired units:

FBPMqiuz = (BPMqiuz  / FIPd-1) * FIPd

PMqiuz	= Max (FBPMqiuz, MCPEiz )

PMqivz	= Min(PMqiuz)

For all other Resource categories:

PMqiuz	= Max (BPMqiuz, MCPEiz )

PMqivz	= Min(PMqiuz)

NETUEQivq	=	Max [0, ((NETOOMUEQivq + NETLBEUQivq) –(NETOOMDEQivq + NETLBEDQivq))]

NETOOMUEQivq	=	Max(0, (SUM(IOOMUPqui)u – SUM(IOOMDNqui)u))

NETLBEUQivq	=	Max (0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u))

NETOOMDEQivq	=	Max(0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u))

NETLBEDQivq 	=	Max(0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u))

LBEAGRivq 	=	[(SUM(LBEUQqui)u + SUM(LBEDQqui)u)] /
[SUM(LBEUQqui)u + SUM(LBEDQqui)u +
SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

d	Operating Day of interval i

i	interval

q	QSE

u	individual Generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

BPMqiuz	Incremental bid premium for that specific unit per interval

FBPMqiuz	Incremental bid premium, adjusted for fuel price, for that specific unit per interval

FIPd	Fuel Index Price for Operating Day of interval i

FIPd-1	Fuel Index Price used to calculate Resource category bid limits effective for Operating Day of interval i

IOLqiuz	The Instructed output level for the unit given the unit-specific instruction

IOOMDNqui	OOM Energy Down Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMUPqui	OOM Energy Up Instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

LPCRSUquiz	Payment to QSE for providing Balancing Energy Up from a specific unit to solve Local Congestion per QSE per interval per zone

LPCRSUqviz	Payment to QSE for providing Balancing Energy Up from Aggregated Units to solve Local Congestion per QSE per interval per zone

LBEAGRivq	The percentage of the aggregated instructions to the Aggregated Unit that represents the OOM Energy market

LBEDQqui	Local Balancing Energy Down Instructions for that interval per specific unit

LBEUQqui	Local Balancing Energy Up Instructions for that interval per specific unit

MCPEiz	Market Clearing Price for Energy per interval per zone

MRqiuz	Meter reading per unit per interval

MRqivz	Meter reading per Aggregated Unit per interval

NETLBEDQivq	The net of all unit specific Local Balancing Energy Instructions

(Down minus Up) that result in a positive value

NETLBEUQivq	The net of all unit specific Local Balancing Energy Instructions (Up minus Down) that result in a positive value

NETOOMDEQivq	The net of all unit specific OOM instructions (Down minus Up) that result in a positive value

NETOOMUEQivq	The net of all unit specific OOM instructions (Up minus Down) that result in a positive value

NETUEQivq	The overall net direction per the instructions given to the Aggregated Unit results in an up direction

OLqiuz	Resource Plan output level submitted by the QSE for the specific unit given the unit-specific instruction for that interval

OLqivz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval

PMqiuz	Incremental Premium Specified for that specific unit per interval

PMqivz	Incremental Premium Specified for that specific Aggregated Unit per interval

(a)	The Nodal Market implementation requires Early Delivery System (EDS) testing.  During the EDS 3 R6.3 Load Frequency Control (LFC) testing as documented by ERCOT’s issuance of Dispatch Instructions to perform the test, the formula in Section 7.4.3.1(1) will be modified in the following way:

IOOMUPiuq	=	¼ * (Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y))

IOOMDNiuq	=	¼ * (Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y))

LBEUQqui	=	¼ * (Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y))

LBEDQqui	=	¼ * (Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y))

Where:

IOOMUPiuq	OOM Energy Up Instructions - The Generation Resource’s aggregated Base Point for the 15-minute Settlement Interval corresponding with an OOME Up deployment.

IOOMDNiuq	OOM Energy Down Instructions - The Generation Resource’s aggregated Base Point for the 15-minute Settlement Interval corresponding with an OOME Down deployment.

LBEUQqui	Local Balancing Energy Up Instructions - The Generation Resource’s aggregated Base Point for the 15-minute Settlement Interval corresponding with a Local Balancing Energy Up deployment.

LBEDQqui	Local Balancing Energy Down Instructions - The Generation Resource’s aggregated Base Point for the 15-minute Settlement Interval corresponding with a Local Balancing Energy Down deployment.

BP y	Base Point by interval - The Base Point generated from SCED test system for the Generation Resource for the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred.

TLMP y	Duration of SCED interval per interval - The duration of the portion of the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred.

y	A SCED interval in the Settlement Interval.  The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred.

i	15-minute Settlement Interval.

u	Individual Generation unit (including units within an Aggregated Unit).

q	QSE.

(i)	During EDS 3 R6.3 LFC testing, when a combined cycle Resource receives SCED Base Points at the Combined Cycle Configuration (as defined in the ERCOT Nodal Protocols) level, ERCOT will use MW telemetry based splitting percentages to allocate the OOME or Local Balancing Energy Instruction, as calculated in Section 7.4.3.1(1)(a) above, to each individual unit making up the Combined Cycle Configuration for each 15-minute Settlement Interval.

(ii)	If, after receiving the Initial Statement for the Operating Day on which EDS 3 R6.3 LFC testing occurred as documented by Dispatch Instruction to the QSE for a specific test period, a QSE does not agree with the updated Instructed Output level or payment for OOME or Local Balancing Energy, a QSE may submit a settlement and billing dispute in accordance with the process outlined in Section 9.5, Settlement and Billing Dispute Process.  In addition to the standard information required on the dispute form, the dispute should clearly reference the EDS 3 R6.3 LFC test and indicate the disputed Settlement Intervals involved in the test.

(2)	Whenever a LaaR with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Up, is deployed to solve Local Congestion, the QSE will be paid the incremental premium specified as defined below:

LPCLAARquiz 	= 	-1 * Σ ((PMqiuz – MCPEiz )* Max (0, Min (OLqiuz -LAARTOTqiuz, OLqiuz - IOLqiuz)))

PMqiuz	= Max (FBPMqiuz, MCPEiz )

FBPMqiuz	= (BPMqiuz / FIPd-1) * FIPd

Where:

i	interval

q	QSE

u	individual LAAR unit

z	zone

BPMqiuz	Incremental bid premium for that specific LaaR unit per interval

FBPMqiuz	Incremental bid premium, adjusted for fuel price, for that specific unit per interval

FIPd	Fuel Index Price for Operating Day of interval i

FIPd-1	Fuel Index Price used to calculate Resource category bid limits effective for Operating Day of interval i

IOLqiuz	The Instructed output level for the LaaR unit given the unit-specific instruction

LAARTOTqiuz	Actual ESI ID usage per unit per interval

LPCLAARquiz	Payment to QSE for providing Balancing Energy Up from specific LaaR unit to solve Local Congestion per QSE per interval per zone

MCPEiz	Market Clearing Price for Energy per interval per zone

OLqiuz	Resource Plan output level submitted by the QSE for the specific LaaR unit given the unit-specific instruction for that interval

PMqiuz	Incremental Premium specified for that specific LaaR unit per interval

7.4.3.2	Balancing Energy Down from a Specific Resource

In accordance with Section 6.7.1.2, Deployment of Balancing Energy when Congestion Occurs, whenever a specific Generation Resource with a submitted bid premium, in accordance with the Resource Category Generic bid limit for Balancing Energy Down, is deployed decrementally to solve Local Congestion, the QSE will be paid the difference between the MCPE of the Congestion Zone in which the specific Generation Resource is located, and the decremental bid premium specified.

The calculation of the payment for Balancing Energy Down from a specific Resource is as follows:

LPCRSDquiz 	= 	-1 * Σ(MAX(0,(MCPEiz - PMqiuz )) * Max(0, Min(OLqiuz -MRqiuz, OLqiuz - IOLqiuz)))u

LPCRSDqviz 	=	-1 * Σ(MAX(0,(MCPEiz - PMqivz )) * Max(0, Min(OLqivz -MRqivz  NETDEQivq)))v * LBEAGRivq

For gas-fired units:

FBPMqiuz	= (BPMqiuz  / FIPd-1) * FIPd

PMqiuz	= FBPMqiuz

PMqivz	= Max(PMqiuz)

For all other Resource categories:

PMqiuz	= BPMqiuz

PMqivz	= Max(PMqiuz)

NETDEQivq	=	Max [0, ((NETOOMDEQivq + NETLBEDQivq) – (NETOOMUEQivq + NETLBEUQivq))]

NETOOMUEQivq	=	Max [0, (SUM (IOOMUPqui)u – SUM(IOOMDNqui)u)]

NETLBEUQivq 	=	Max [0, (SUM(LBEUQqui)u – SUM(LBEDQqui)u)]

NETOOMDEQivq	=	Max [0, (SUM(IOOMDNqui)u – SUM(IOOMUPqui)u)]

NETLBEDQivq 	=	Max[0, (SUM(LBEDQqui)u – SUM(LBEUQqui)u)]

LBEAGRivq	=	[SUM(LBEUQqui)u + SUM(LBEDQqui)u] /
[SUM(LBEUQqui)u + SUM(LBEDQqui)u +
SUM(IOOMUPqui)u + SUM(IOOMDNqui)u]

Where:

d	Operating Day of interval i

i	interval

q	QSE

u	individual Generation unit (including units within an Aggregated Unit)

v	Aggregated Unit

z	zone

BPMqiuz	Decremental bid premium specified for that specific unit per interval

FBPMqiuz	Decremental bid premium, adjusted for fuel price, for that specific unit per interval

FIPd	Fuel Index Price for Operating Day of interval i

FIPd-1	Fuel Index Price used to calculate Resource category bid limits effective for Operating Day of interval i

IOLqiuz	The instructed output level for the unit given a unit-specific instruction

IOOMDNqui	OOM Energy Down instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

IOOMUPqui	OOM Energy Up instructions for that interval for that unit converted to MWh for that interval by dividing by the interval count per hour (4 per hour for 15 minute Settlement Interval)

LBEAGRivq	The percentage of the aggregated instructions to the Aggregate Unit that represents the OOM Energy market.

LBEDQqui	Local Balancing Energy Down instructions for that interval per specific unit

LBEUQqui	Local Balancing Energy Up instructions for that interval per specific unit

LPCRSDquiz	Payment to QSE for providing Balancing Energy Down from a specific unit to solve Local Congestion per QSE per interval per zone

LPCRSDqviz	Payment to QSE for providing Balancing Energy Down from Aggregated Units to solve Local Congestion per QSE per interval per zone

MCPEiz	Market Clearing Price of Energy per interval per zone

MRqiuz	Meter reading per unit per interval.

MRqivz	Meter reading per Aggregated Unit per interval.

NETDEQivq	The overall net direction per the instructions given to the Aggregated Unit results in a down direction

NETLBEDQivq	The net of all unit specific Local Balancing Energy instructions (Down minus Up) that result in a positive value.

NETLBEUQivq	The net of all unit specific Local Balancing Energy instructions (Up minus Down) that result in a positive value.

NETOOMDEQivq	The net of all unit specific OOM instructions (Down minus Up) that result in a positive value.

NETOOMUEQivq	The net of all unit specific OOM instructions (Up minus Down) that result in a positive value.

OLqiuz	Resource Plan output level submitted by the QSE for the unit given the unit-specific instruction for that interval

OLqivz	Resource Plan output level submitted by the QSE for the Aggregated Unit containing the unit given the unit-specific instruction for that interval

PMqiuz	Decremental Premium for that specific unit per interval

PMqivz	Decremental Premium for that specific Aggregated Unit per interval

(a)	The Nodal Market implementation requires Early Delivery System (EDS) testing.  During the EDS 3 R6.3 Load Frequency Control (LFC) testing documented by Dispatch Instruction to the QSE for a specific test period, the formula in Section 7.4.3.2 will be modified in the following way:

IOOMUPiuq	=	¼ * (Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y))

IOOMDNiuq	=	¼ * (Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y))

LBEUQqui	=	¼ * (Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y))

LBEDQqui	=	¼ * (Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y))

Where:

IOOMUPiuq	OOM Energy Up Instructions - The Generation Resource’s aggregated Base Point for the 15-minute Settlement Interval corresponding with an OOME Up deployment.

IOOMDNiuq	OOM Energy Down Instructions - The Generation Resource’s aggregated Base Point for the 15-minute Settlement Interval corresponding with an OOME Down deployment.

LBEUQqui	Local Balancing Energy Up Instructions - The Generation Resource’s aggregated Base Point for the 15-minute Settlement Interval corresponding with a Local Balancing Energy Up deployment.

LBEDQqui	Local Balancing Energy Down Instructions - The Generation Resource’s aggregated Base Point for the 15-minute Settlement Interval corresponding with a Local Balancing Energy Down deployment.

BP y	Base Point by interval - The Base Point generated from SCED test system for the Generation Resource for the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred.

TLMP y	Duration of SCED interval per interval - The duration of the portion of the SCED interval y within a 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred.

y	A SCED interval in the Settlement Interval.  The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval where an OOME or Local Balancing Energy deployment occurred.

i	15-minute Settlement Interval.

u	Individual Generation unit (including units within an Aggregated Unit).

q	QSE.

(i)	During EDS 3 R6.3 LFC testing, when a combined cycle plant receives SCED Base Points at the Combined Cycle Configuration (as defined in the ERCOT Nodal Protocols) level, ERCOT will use MW telemetry based splitting percentages to allocate the OOME or Local Balancing Energy Instructions, as calculated in Section 7.4.3.2(1)(a) above, to each individual unit making up the Combined Cycle Configuration for each 15-minute Settlement Interval.

(ii)	If, after receiving the Initial Statement for the Operating Day on which EDS 3 R6.3 LFC testing occurred as documented by Dispatch Instruction(s) to the QSE for a specific test period, a QSE does not agree with the updated Instructed Output level or payment for OOME or Local Balancing Energy, a QSE may submit a settlement and billing dispute in accordance with the process outlined in Section 9.5, Settlement and Billing Dispute Process.  In addition to the standard information required on the dispute form, the dispute should clearly reference the EDS 3 R6.3 LFC test and indicate the disputed Settlement Intervals involved in the test.

7.4.3.3	Balancing Energy Charge to Solve Local Congestion

In accordance with this section, all the cost associated with Balancing Energy from a specific Resource to solve Local Congestion will be allocated on a QSE Load Ratio Share.

LCCiq	=	- 1* Σ ((LPCRSUqiz + LPCRSDqiz) * LRSiq)

Where:

LPCRSUqiz	Payment to QSE for providing Balancing Energy Up from specific Resource to solve Local Congestion per QSE per interval per zone

LPCRSDqiz	Payment to QSE for providing Balancing Energy Down from specific Resource to solve Local Congestion per QSE per interval per zone

LCCiq	Local Congestion Load allocation charge per QSE per interval

LRSqi	Load Ratio Share

7.4.4	Direct Assignment of Local Congestion Costs

ERCOT shall directly assign Local Congestion costs beginning six months after ERCOT’s incurred costs of clearing Local Congestion during a rolling twelve month period reaches 20 million dollars.

[PRR492 & PRR532: Add Section 7.4.5 upon system implementation and filling staffing requirements:] 7.4.5 Plan to Alleviate Chronic Local Congestion Charges (1) ERCOT shall monitor Local Congestion Area costs and post a report on the MIS ten (10) days following the end of the month. The report shall include the amount of Local Congestion Area costs by type of payment to Resources (e.g., OOME Up, OOME Down, and OOMC) including discussion of the limiting transmission element(s) or other significant events that may have contributed to Local Congestion as available. This information will also be reported monthly to the ERCOT Board of Directors, PUCT Market Oversight Division, and appropriate ERCOT subcommittees. (2) For the purposes of this section, “Local Congestion Area” is defined as a sub-region of a Congestion Zone in which ERCOT dispatches OOME, OOMC, or local Balancing Energy Service on a regular basis as described in the ERCOT monthly report of Local Congestion Cost. (3) If the amount of OOME, OOMC, or local Balancing Energy Service in a Local Congestion Area exceeds a threshold amount of either ten million dollars ($10 million) during the most current twelve (12) consecutive months or five million dollars ($5 million) in the most current three (3) consecutive months, ERCOT shall report such to the ERCOT Board, PUC Market Oversight Division, and appropriate ERCOT subcommittees and shall initiate a study to identify feasible alternatives that may, at a future time, be more economically efficient than the continued use of OOME, OOMC, or local Balancing Energy Service. (4) ERCOT shall develop feasible alternatives and their associated costs to reduce or replace the use of OOME, OOMC, or local Balancing Energy Service. At a minimum, the list of feasible alternatives that ERCOT shall include consideration of a new CSC for the following year under Section 7.2.1.1, Process for Determining CSCs, item (2), a change in ERCOT operations affecting the Local Congestion, construction of new or expansion of existing Transmission Facilities, installation of voltage control devices, more accurate modeling of transmission line or transformer MVA Real Time limits, solicitation or auctions for interruptible Load from Load Serving Entities when such Protocols are available, or continued use of OOME, OOMC, or local Balancing Energy Service. (5) ERCOT shall prepare a report, no later than forty-five (45) days after the threshold is exceeded, on the feasible alternatives available to reduce Local Congestion below the threshold amount. The report shall include: (a) The root cause of the Local Congestion; (b) Current actions of the TDSP and/or ERCOT to relieve such Local Congestion; (c) A review of ERCOT’s operations (e.g., Dispatch Instructions, transmission line limits, etc.); (d) A determination of short term mitigation measures that could reduce the use of OOME, OOMC or local Balancing Energy Service; and (e) The amount of market impact caused by the Local Congestion. The report shall also include an implementation plan, timeline, and recommendations regarding short-and long-term solutions to the Local Congestion as appropriate. The implementation plan may identify studies to be conducted prior to recommendations for short- and long-term solutions to the Local Congestion. These additional studies will be conducted through the regional planning process, as appropriate. (6) This report shall be posted on the MIS and provided to the PUCT Market Oversight Division, the appropriate ERCOT subcommittee(s), and ERCOT Board of Directors. (7) Subsequent to the initial report, ERCOT staff shall provide monthly updates on the schedule and planning status to the PUCT Market Oversight Division, appropriate ERCOT subcommittee(s), and the Board. ERCOT will use its best efforts to implement short- and long-term solutions as quickly as practical.

7.5	Transmission Congestion Rights

Pre-assigned Congestion Rights (PCRs) are priced differently than Transmission Congestion Rights (TCRs), and PCRs are allocated rather than awarded as a result of the annual TCR auction process.  The eligibility, pricing and annual allocation of PCRs are addressed in Section 7.5.6, Direct Allocation of Pre-assigned Congestion Rights.  For all other purposes, PCRs are functionally and financially equivalent to TCRs.

7.5.1	Function of Transmission Congestion Rights

TCRs and PCRs function as financial hedges (similar to financial options with zero strike price) against the marginal costs to resolve Zonal Congestion.  The total costs of Zonal Congestion comprise those portions of Replacement Reserve Service (RPRS) costs and the Balancing Energy service costs associated with managing Zonal Congestion.  The TCR and PCR holder will receive an amount equal to the directly assigned Congestion costs for an equivalent quantity of scheduled flow.  TCRs are not deratable once they are sold in the auction(s), and allocated PCRs are not deratable.

A TCR is a financial right on a specified directional Commercially Significant Constraint (CSC) for a particular hour that entitles the holder of record to receive remuneration equal to the sum of non-negative Balancing Energy service and RPRS Shadow Prices ($/MW/15-minute) over all 15-minute intervals within the hour for the corresponding CSC multiplied by 1 MW.

7.5.2	Procedure for Computation of Transmission Congestion Rights Quantities

The ERCOT System will be modeled and the CSC interfaces determined in accordance with Section 7.2, CSC Zone Determination Process, using steady-state and dynamic power system simulation software.  The capacity of the CSCs will be fully used and there will be no reservation of capacity for any particular segment of Load.

ERCOT will determine CSC Limits as follows:

(1)	Determine the pre-contingency flow limit on the directional CSC that results in the CSC or any directional Clostely Related Element (CRE) corresponding to the CSC becoming the base case limiting element, for all Congestion Zone-to-Congestion Zone transfer studies.

(2)	Determine the pre-contingency flow limit on the directional CSC to withstand any single contingency that results in any directional CRE, including elements comprising the CSC, corresponding to the CSC to become the post-contingency limiting element for all Congestion Zone-to-Congestion Zone transfer studies.

(3)	The lower of the CSC flow limits from (1) and (2) above sets the CSC Limit.

7.5.2.1	Annual TCR Quantities

Annually determined Summer ERCOT peak flow limits for CSCs will provide the starting basis for determining the number of TCRs to be auctioned.  The flow limit established for each CSC on an annual basis may vary from time to time based on changes in system conditions.  The ultimate number of TCRs to be auctioned for the CSCs will take into account forecasted changes in system conditions.

ERCOT will determine the total annual quantity of TCRs for the annual distribution by adjusting the annual summer peak base case CSC Limits by the following amount for each CSC: the sum over all nodes of the product of the expected level of Load at each node from the annual summer peak base case multiplied by the difference between the corresponding Shift Factor for that node and CSC and a modification of the Zonal Shift Factor calculated using all Generation Resources.

7.5.2.2	Monthly TCR Quantities

ERCOT will determine the total quantity of TCRs for the monthly distribution by adjusting the corresponding modified seasonal base case CSC Limits by the following amount for each CSC: the sum over all nodes of the product of the expected level of Load at each node from the modified seasonal base case multiplied by the difference between the corresponding Shift Factor for that node and CSC and a modification of the Zonal Shift Factor calculated using all Generation Resources.  The modified seasonal base case for a particular month will be based on the most limiting situation within the corresponding month.  This analysis takes into account all transmission outages of significant duration and impact with the objective of collecting sufficient revenues from direct assigned Congestion costs to pay all TCR holders.

ERCOT may distribute TCRs in amounts less than the total quantities based on expectations of the frequency and magnitude of potential CSC de-rates that could occur during any particular month.

7.5.2.3	ERCOT Posting of TCR Quantities

Upon review and approval of the TCR amount, ERCOT will determine and post on the Market Information System (MIS) the total quantity, the amount of PCRs, and the remaining available quantity of TCRs for each CSC.  The remaining available quantity of TCRs for auction will be the total amount of TCRs less the amount of PCRs.

7.5.3	Annual and Monthly Auction of Transmission Congestion Rights

7.5.3.1	Annual and Monthly Auction Summary

Thirty-percent (30%) of the total annual quantity of TCRs less PCRs for any given CSC will be awarded to Market Participants based on the results of an initial annual auction.  These TCRs will be made available in the annual auction for each hour of the year.  For the annual auction, the quantity of TCRs used to calculate the twenty-five percent (25%) limit, pursuant to Section 7.5.9, Limitation on Ownership or Control of TCRs, is based on the total annual quantity of TCRs (which includes PCRs).  Each annual auction will be completed no later than the 15th of December, on a date set by ERCOT.

Prior to each monthly auction, ERCOT will determine the total monthly quantity of TCRs available for that month based upon forecasted system conditions for the month.  The amount of TCRs that will be auctioned in the monthly auction will be the total monthly quantity of TCRs, less any PCRs, and less any TCRs for that month sold in the annual auction.  For that monthly auction, the quantity of TCRs used to calculate the twenty-five percent (25%) limit, pursuant to Section 7.5.9, is the greater of (A) the total annual quantity of TCRs, or (B) the total monthly quantity of TCRs.  However, in determining the amount of TCRs available in each monthly auction, ERCOT shall seek to achieve monthly revenue neutrality to avoid a revenue shortfall due to payment to TCR holders for the month.  In an effort to achieve revenue neutrality, ERCOT shall manually adjust the quantity of monthly TCRs auctioned.  The manual adjustment made by ERCOT shall not exceed forty-percent (40%) of the Total Transmission Capacity (TTC) of each CSC for the month.  In developing the monthly TCR auction model CSC transfer capability, ERCOT shall strive to achieve monthly TCR revenue neutrality.  Monthly TCR revenue neutrality is achieved by ensuring that the number of TCRs sold for each CSC is not greater than the average actual Real Time flow capability of that CSC for the month.  ERCOT shall strive to ensure that monthly TCR payouts for each CSC, match as closely as possible, total CSC charges for the month for that CSC.  Monthly revenue neutrality does not include annual TCR auction revenues.

To achieve this objective, ERCOT may use manual adjustments and/or a scaling factor.  In developing these adjustments or scaling factors, ERCOT may consider:

(1)	Combinations of Outages known at the time of development of the monthly TCR auction model;

(2)	Outages expected to be entered for the month, subsequent to the development of the TCR auction model; and

(3)	Historical CSC operational flow capability for the month or Season.

In no event shall ERCOT sell more TCRs than the calculations show to be available.  ERCOT shall report the Outages included in the TCR case and the scaling factor(s) and the data used as the basis for developing the scaling factor(s) with the monthly TCR auction announcement.

The TCRs auctioned in the monthly auction will be awarded to Market Participants based on the results of monthly auctions.  These TCRs will be made available in the monthly auction for all hours of each day of each month for which the auction is being held.  Each monthly auction will be completed no later than the 15th of each month prior to the month that the TCRs are effective, on a date set by ERCOT.

7.5.3.2	Auction Procedures

ERCOT will develop a procedure for authenticating ownership for each TCR.

ERCOT, or its designee, will conduct a single-round, simultaneous combinatorial auction for selling the TCRs available for each annual or monthly auction for all CSCs.  ERCOT shall use a standard linear programming model to maximize the auction revenue subject to the constraints that ERCOT and the bidders provide prior to the start of the auction.  The clearing-price for each TCR will be equal to the corresponding Shadow Price of the marginal TCR awarded on that CSC.

The auction process will be open to any qualified TCR Account Holder.  To participate in the auctions, a bidder must meet financial security requirements established by ERCOT and must have access to the computer hardware, software and communications equipment required to participate.

7.5.3.2.1	TCR Auction Notices

No less than twenty-days (20) prior to the annual auction and ten (10) days prior to the monthly auction, ERCOT will post:

(1)	The Zonal Shift Factor and zone-to-zone impact matrix (to determine the megawatt impact on each CSC);

(2)	The number of TCRs to be issued for each period (annual, monthly) in that auction;

(3)	The number of PCRs that have been allocated for each CSC;

(4)	Deadline for bidders to satisfy financial requirements;

(5)	Specifications for the equipment necessary to participate in the auction;

(6)	The date and time by which bids must be submitted;

(7)	The bid format;

(8)	CSCs and CREs;

(9)	Any other information of commercial significance to bidders; and

(10)	The total number of TCRs available and used to determine the 25% limit, pursuant to Section 7.5.9, Limitation on Ownership or Control of TCRs.

7.5.3.2.2	Auction

The auction will be a single-round, simultaneous combinatorial auction for selling the TCRs available for an annual or monthly auction for CSCs, with the following steps:

(1)	The following constraints shall be entered into ERCOT’s auction engine model:

(a)	The maximum number of TCRs for any given CSC available for auction.

(b)	The maximum number of TCRs for each CSC that a bidder may purchase in the auction that will ensure the bidder will not exceed the lower of the TCR limitation described in Section 7.5.9, Limitation on Ownership or Control of TCRs, or a bidder’s self-imposed TCR limit set by the bidder prior to the auction.

(c)	The bidder’s credit limit approved by ERCOT or, if provided, the bidder’s self-imposed maximum credit Obligation will be enforced as a constraint such that the summation over all bids of the product of the awarded TCRs and the bidder’s bid price is less than or equal to the lower of the bidder’s self-imposed maximum Obligation or the credit limit approved by ERCOT.  The bidder’s self-imposed maximum credit Obligation cannot exceed the credit limit approved by ERCOT.

(d)	Bids may be submitted for TCRs for any individual CSC and for weighted combinations of CSCs.  Each bid must be non-negative and shall contain the maximum quantity of TCRs, the maximum price, and the distribution weights of the TCRs among the CSCs.  The distribution weight shall be submitted with non-negative ratios (with up to three decimal places) that sum to one (1.000), as illustrated in the table below.

- Examples of Allowable Bids

| Bidders and Bids | CSC 1 Weighting | CSC 2 Weighting | CSC 3 Weighting | Bid Price ($ per MWh) | Maximum Quantity (TCRs) |
| --- | --- | --- | --- | --- | --- |
| A1 | 0.2 | 0.3 | 0.5 | $10.00 | 300 |
| A2 | 1.0 | 0.0 | 0.0 | $5.00 | 185 |
| B | 0.2 | 0.5 | 0.3 | $11.25 | 250 |
| C1 | 0.6 | 0.3 | 0.1 | $7.50 | 240 |
| C2 | 1.0 | 0.0 | 0.0 | $1.00 | 100 |
| D1 | 0.0 | 0.5 | 0.5 | $9.50 | 320 |
| D2 | 0.0 | 1.0 | 0.0 | $3.00 | 140 |
| D3 | 0.0 | 0.0 | 1.0 | $2.50 | 170 |

- NOTE: 3 decimal places for weightings and bid price allowable.

(2)	The auction system will award a combination of TCRs that maximizes the sum of bid prices times the awarded MW for the respective bids while simultaneously observing all applicable constraints.  ERCOT shall award TCRs in quantities rounded to three decimal places.

(3)	The TCR clearing price for a CSC will be equal to the corresponding Shadow Price of the marginal TCR awarded on that CSC.  The Shadow Price is representative of the decremental bid-based revenue that would be produced from the TCR auction if one less TCR were available to be auctioned on that CSC.

(4)	If all of the annual TCRs available for a CSC in the annual auction are not awarded (excluding fractional rounding), the TCR clearing price for that CSC shall be determined to be zero (0.0) dollars per MWh.  The TCRs not awarded/purchased in the annual auction will be included in the monthly auction.

(5)	If all of the TCRs available for a CSC in the monthly auction are not awarded (excluding fractional rounding), the TCR Clearing Price for that CSC shall be determined to be zero (0.0) dollars per MWh.  The TCRs not awarded/purchased in the monthly auction will not be sold.  Payments for TCR and PCR Invoices shall be due five (5) Bank Business Days after issuance of the Invoice.  Any unpaid TCRs and PCRs that remain outstanding after their Invoice date shall be forfeited by the awardees and, with respect to TCRs and PCRs from the annual auction, included in the following monthly auctions.

(6)	The linear programming solution will solve for one optimal solution, which may result in some bids at the clearing price not being awarded a TCR.  This situation may be minimized because bid prices and bid ratios can be submitted with up to three decimal places.

(7)	The auctioneer shall announce the results of the auction simultaneously to all TCR bidders, by posting the results on the MIS.  For each CSC, the total quantity of TCRs awarded, the TCR clearing price, and list of all bids will be posted.  The identity of the bidders will not be posted.

(8)	When an Entity is awarded TCRs as a result of an auction, the TCRs will not become the property of the awardees and the TCRs will not be placed in their accounts until the TCR Invoices have been paid in full.  TCRs awarded are considered when determining ownership.  Ownership of TCRs in a secondary transfer does not change until the recipient accepts the transfer.  Until such time, the initiator of the secondary transfer is considered the owner of the TCRs.

(9)	The twenty-five percent (25%) limit shall be applied to secondary transfers.  When a recipient accepts a secondary transfer, the number of TCRs the recipient owns in any hour for a given CSC during the month will be added to the amount of the TCRs they will receive as a result of the secondary transfer.  If the total exceeds a bidder’s twenty-five percent (25%) limit for the CSC, the secondary transfer shall be cancelled.  Electronic Notice shall be given to the secondary transfer initiator and recipient of the cancellation.

7.5.4	Allocation Method and Timing for Distributing TCR Auction Revenues

The TCR auction revenues from each annual auction and PCR revenues will be divided into monthly revenues in the same proportion as ERCOT’s monthly energy forecast.  This amount will be added to the monthly auction revenue.  The total monthly revenues from the auctions will be credited to QSEs once per month on a monthly Load Ratio Share for the ERCOT peak operating interval during the month basis.  These revenues will be paid to QSEs no later than the 15th of the following month.  PCR holders will be issued invoices that are based on 15 percent of the TCR clearing price from the annual auction.  Payment of these invoices is required prior to the distribution of PCRs.

Any difference between the directly assigned Congestion costs charged by ERCOT and Zonal Congestion credits paid to TCR holders for any 15-minute interval will be credited or charged to all QSEs on a Load Ratio Share basis for the same 15-minute interval.  ERCOT shall provide a report to be published on its MIS which details by interval, any Zonal Congestion revenue shortfalls or surpluses.  The report shall be published on a monthly basis within seven (7) Business Days of the end of a month for the prior month’s activity.  The report shall provide a description of the methodology and billing determinants used to determine and assess any revenue shortfalls or surpluses, and shall include any billing determinants necessary to replicate the results in the report, to the extent such billing determinants are not already available.

7.5.5	TCR Settlement

To participate in ERCOT’s TCR settlements, a TCR holder need not be a QSE nor be represented by a QSE, but must be registered as a TCR Account Holder with ERCOT and have the ability to make and accept electronic transfer of funds.

TCRULnq=   -1* MAUCRn+AAUCR∗MEnergynAll MonthsMEnergyn∗LSRnq

Where:

n	Month

q	QSE

TCRULnq	TCR Auction Revenue Uplift per QSE per month

MAUCRn	Monthly TCR Auction Revenue

AAUCRn	Annual TCR Auction Revenue plus PCR revenues

Menergyn	Monthly Energy Forecast

LSRnq	QSE’s Load Share Ratio in that month’s peak interval

Congestion credits, calculated as described below, will be determined based on the record of ownership in ERCOT’s database, at the end of the day for which the TCRs are effective, for the relevant CSC, based on the published Shadow Price(s) for the same.  The TCR holders will be paid Congestion credits based on their TCR ownership the day after direct assigned Invoice payments are due in accordance with the Settlement Calendar in Section 9, Settlement and Billing.

TCRPAYhp	=	-1 * Σ (TCRCSChp  + PCRCSChp) * (Σ (SPCSCi /4)i + SPCCSCh))CSC

Where:

h	Hour being calculated

i	Intervals within the hour

p	TCR owner of record

CSC	Commercially Significant Constraint

CSCRPiq 	CSC Replacement Reserve related Congestion charge, per interval,  per QSE

TCRCSChp	Total number of TCRs owned per hour, per TCR owner of record

PCRCSChp	Total number of PCRs owned per hour, per PCR owner of record

SPCSCi	BES Shadow Price per CSC, per interval

SPCCSCh	RPRS Capacity Shadow Price per CSC, per hour

TCRPAYhp	TCR payment per hour, per TCR owner of record

7.5.6	Direct Allocation of Pre-assigned Congestion Rights

Upon the zonal method being implemented and once auctioned TCRs are available for use, Municipally Owned Utilities (MOUs) and Electric Cooperatives (ECs) which own or have a long-term (greater than five (5) years) contractual commitment for annual capacity and energy from a specific remote Generation Resource, and that commitment was entered into prior to September 1, 1999, are eligible for PCRs.  In addition, MOUs and ECs which have a long-term (greater than five (5) years) allocation from the federal government for annual capacity and energy produced at a federally-owned hydroelectric Generation Resource, and that allocation was in place prior to September 1, 1999, are eligible for PCRs.  The PCRs will be allocated as follows:

(1)	PCRs are available on an annual basis until the date upon which a MOU or EC implements retail Customer Choice, or, alternatively, until such other date as may be specified by Order of the Public Utility Commission of Texas (PUCT).  Non Opt-In Entities (NOIEs) that opt in after the first of the year shall pay a pro rata share of the full value for remaining months:  (Applicable Annual TCR Clearing Price * 85% * Quantity of PCRs * Number of hours remaining in the year).

(2)	The cost of PCRs shall be equal to fifteen percent (15%) of the applicable annual TCR auction clearing price for each CSC for which a PCR is allocated.

PCRINVyp	=	Σ ((ATCRCPCSCyp  * .15 ) * PCRCSCyp * H))CSC

Where:

p	PCR owner of record

y	Year

ATCRCP	Annual TCR Clearing Price ($/MW)

CSC	Commercially Significant Constraint

H	Number of Hours eligible for PCRs (hours/year)

PCRCSCyp	Total number of PCRs allocated for each of the CSCs, per PCR owner of record, for a given year (MW)

PCRINVyp	TCR payment per hour, per TCR owner of record ($/year)

(3)	The allocation of PCRs to any single Entity shall not exceed twenty-five percent (25%) of the total available TCRs at a CSC interface for any single direction.

(4)	The distribution of PCRs will be sufficient to isolate the MOUs and ECs from any direct-assigned costs associated with transporting the output of their remote Generation Resources to their native Load.

(5)	ERCOT shall distribute PCRs and accordingly populate the TCR ownership database for those months where the NOIE meets eligibility requirements.  PCRs will be included in applicable monthly TCR auctions when the NOIE does not meet eligibility requirements across the year.

(6)	PCRs may be traded in the secondary market.

(7)	Holders of PCRs shall not be precluded from participating in the market to purchase additional TCRs.

7.5.7	Secondary Market Exchange of Transmission Congestion Rights

Market Participants can exchange TCRs and PCRs in any secondary market.  ERCOT will initially populate a database of hourly TCR holders with the annual and monthly first-purchasers of TCRs and hourly PCR holders.  Subsequent holders or owners (other than first-purchasers) of the TCRs and PCRs who want the change in ownership to be recorded in the ERCOT TCR ownership database will be responsible for assuring that the sellers report the transactions to ERCOT’s satisfaction and that the sellers authorize the transaction within the ERCOT database in accordance to ERCOT’s procedures.  A TCR and PCR may change ownership in the ERCOT TCR ownership database through the end of the Adjustment Period of the Operating Hour for which it is designated to hedge Congestion costs.

7.5.8	Transmission Congestion Right Value Under Physical Curtailment or De-rating

TCRs and PCRs will retain their full value even in the event of physical curtailment or flow limitation below the megawatt quantity for which TCRs were defined on any given CSC path.

7.5.9	Limitation on Ownership or Control of TCRs

(1)	No Entity combined with its Affiliates may, either directly or indirectly, own, control, or receive the revenue from more than twenty-five percent (25%) of the total available TCRs and/or PCRs at a CSC interface for any single direction for a given hour.  The total available TCRs for the annual auction and each monthly auction are determined pursuant to Section 7.5.3.1, Annual and Monthly Auction Summary, and included in the Auction Notice posted pursuant to Section 7.5.3.2.1, TCR Auction Notices.

(2)	For purposes of this Section, the total number of TCRs available at a CSC interface for any single direction for a given hour is the greater of (a) the total quantity of TCRs available for annual distribution for that CSC interface, direction, and hour, as determined under Section 7.5.2.1, Annual TCR Quantities, or (b) the total quantity of TCRs for the monthly distribution for that CSC interface, direction, and hour, as determined under Section 7.5.2.2, Monthly TCR Quantities.

(3)	If an Entity engages in a transaction with a counter-party, whereby the total ownership or control of the combined Entities exceeds the twenty-five percent (25%) limitation, then any TCRs owned by the counter-parties are not, solely by virtue of that transaction, included in the TCRs that the Entity combined with its Affiliates owns, or controls.

7.5.10	Transition of Transmission Congestion Rights to Congestion Revenue Rights

ERCOT will refund to TCR/PCR owners the annual TCR/PCR auction revenues and monthly TCR auction revenues for all remaining days of the year that follow the Texas Nodal Market Implementation Date.  ERCOT will make refunds to TCR/PCR owners no later than thirty (30) Business Days following the start of the Texas Nodal Market.

The total refund of auction revenues to a TCR/PCR owner of record is calculated as follows:

TCRRAMTp = -1 * (TCRMRp + TCRARp + TCRRRp)

ERCOT shall refund each TCR owner of record its awarded monthly TCRs that are no longer effective for the month of Texas Nodal Market Implementation, including any monthly TCRs already awarded for a subsequent month, calculated as follows:

TCRMRp = (MAUCRm, p * MRP) + MAUCRn>m, p

ERCOT shall refund each TCR/PCR owner of record its awarded annual TCRs/PCRs that are no longer effective for the month of Texas Nodal Market Implementation, calculated as follows:

TCRARp = AAUCRp * (MEnergym / (All MonthsMEnergyn) * MRP

ERCOT shall refund each TCR owner of record any remaining annual TCRs/PCRs for all months following the month of the Texas Nodal Market Implementation, calculated as follows:

TCRRRp = AAUCRp * (Σn>mMEnergyn / (All MonthsMEnergyn))

Where:

TCRRAMT	TCR/PCR Refund Amount per TCR/PCR owner of record

TCRMR	TCR Monthly Refund amount per TCR owner of record for the month of Texas Nodal Market Implementation

TCRAR	TCR/PCR Annual Refund amount per TCR/PCR owner of record for the month of Texas Nodal Market Implementation

TCRRR	TCR/PCR Annual Remaining Refund amount per TCR/PCR owner of record for the months following Texas Nodal Market Implementation

MRP 	Monthly Refund Percentage calculated as the number of days remaining in the Texas Nodal Market Implementation month divided by the total number of days in the Texas Nodal Market Implementation month

MEnergy	Monthly Energy Forecast

MAUCR	Monthly TCR Auction Revenue

AAUCR 	Annual TCR/PCR Auction Revenue

n	Month

m	Month of Texas Nodal Market Implementation

p	TCR/PCR owner of record

7.6	Incorporation of Reliability Must-Run and Out of Merit Order Resources

RMR and OOM Resources will be used for Congestion Management as described in Sections 4, Scheduling, Section 5, Dispatch and Section 6, Ancillary Services.  RMR and OOM Resources will be used for Congestion Management only in the absence of bid-based solutions to alleviate Local Congestion.  RMR and OOM Resources will be compensated in accordance with Section 6.8, Compensation for Services Provided.

7.7	Remedies to Congestion Through Transmission Expansion

ERCOT will review the cost of correcting localized transmission limitations through new construction or other means, and will compare this to the costs incurred to correct those problems.  ERCOT shall publish the average cost of the constraint by period for each transmission limitation subtotaled by TDSP.  This information will be included in ERCOT’s Annual Transmission Planning Report.  If the projected or actual cost of ERCOT directed Congestion Management including all energy and capacity tools, is greater than the cost of correcting the transmission limitations through construction of Facilities, then ERCOT may recommend an upgrade of Facilities to the TDSP.

7.8	[RESERVED]

7.9	Trading Hubs

The purpose of this section is to define each Trading Hub, list the transmission buses that are included in each Trading Hub, describe how to calculate the Trading Hub Price for each Trading Hub, and establish the duties of ERCOT and Market Participants related to transactions involving Trading Hubs.

7.9.1	Definition of a Trading Hub

(1)	A Trading Hub is a specified group of Transmission Buses (60 kV and above) within the ERCOT System used for scheduling bilateral energy or capacity transactions between one or more Market Participants and/or ERCOT.

(2)	All Trading Hubs are defined in Section 7.9.2, ERCOT Trading Hubs, and the only way to create other Trading Hubs is by amending Section 7.9.2 through the Protocol Revision Request process, as described in Section 21, Process for Protocol Revision.

(3)	If a Transmission Bus that is included in a Trading Hub is physically removed from service, ERCOT shall file a Protocol Revision Request to revise the appropriate Trading Hub to reflect that event.

(4)	Transactions involving a Trading Hub shall be expressed in a minimum of one (1) MW increments, unless specified otherwise in Section 7.9.3.4, Scheduling of Transactions Using ERCOT Trading Hubs.

7.9.2	ERCOT Trading Hubs

7.9.2.1	North 345 kV Trading Hub

(1)	The North 345 kV Trading Hub is composed of the following Transmission Buses:

|  | ERCOT Operations |  | Steady State Working Group |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Station Mnemonic | kV | Bus Number | Name | kV | Trading Hub |
| 1 | ANASW | 345 | 2373 | ANNA SS | 345 | NORTH |
| 2 | CN345 | 345 | 2372 | COLLINSS | 345 | NORTH |
| 3 | WLSH | 345 | 5925 | DCEAST | 345 | NORTH |
| 4 | FMRVL | 345 | 1685 | FARM SW | 345 | NORTH |
| 5 | LPCCS | 345 | 1684 | LAMARPWR | 345 | NORTH |
| 6 | MNSES | 345 | 1695 | MOSES | 345 | NORTH |
| 7 | PRSSW | 345 | 1692 | PARIS SS | 345 | NORTH |
| 8 | SSPSW | 345 | 1697 | SULSP SS | 345 | NORTH |
| 9 | VLSES | 345 | 1690 | VALLEY | 345 | NORTH |
| 10 | ALNSW | 345 | 2513 | ALLEN1SS | 345 | NORTH |
| 11 | ALNSW | 345 | 2514 | ALLEN2SS | 345 | NORTH |
| 12 | ALLNC | 345 | 1855 | ALLIANCE | 345 | NORTH |
| 13 | BNDVS | 345 | 970 | BEN DV B | 345 | NORTH |
| 14 | BNBSW | 345 | 1869 | BENB A T | 345 | NORTH |
| 15 | BBSES | 345 | 3380 | BIGBRN | 345 | NORTH |
| 16 | BOSQUESW | 345 | 246 | BOSQUESW | 345 | NORTH |
| 17 | CDHSW | 345 | 2420 | C HILL | 345 | NORTH |
| 18 | CNTRY | 345 | 1929 | CENTURY1 | 345 | NORTH |
| 19 | CNTRY | 345 | 1930 | CENTURY3 | 345 | NORTH |
| 20 | CRLNW | 345 | 2361 | CLT NW | 345 | NORTH |
| 21 | CMNSW | 345 | 1440 | CMCHE SS | 345 | NORTH |
| 22 | CNRSW | 345 | 2453 | CNVIL | 345 | NORTH |
| 23 | CRTLD | 345 | 1931 | COURTLND | 345 | NORTH |
| 24 | DCSES | 345 | 1888 | DEC T | 345 | NORTH |
| 25 | EMSES | 345 | 1859 | EAGLE MT | 345 | NORTH |
| 26 | ELKTN | 345 | 3105 | ELKTON | 345 | NORTH |
| 27 | ELMOT | 345 | 3406 | ELM MOTT | 345 | NORTH |
| 28 | EVRSW | 345 | 1882 | EV WEST | 345 | NORTH |
| 29 | KWASS | 345 | 11690 | FANNIN | 345 | NORTH |
| 30 | FGRSW | 345 | 3130 | FOR GROV | 345 | NORTH |
| 31 | FORSW | 345 | 2437 | FORNEY | 345 | NORTH |
| 32 | FRNYPP | 345 | 12410 | FPLEFRB1 | 345 | NORTH |
| 33 | FRNYPP | 345 | 12420 | FPLEFRB2 | 345 | NORTH |
| 34 | GIBCRK | 345 | 967 | GIBCRK B | 345 | NORTH |
| 35 | HKBRY | 345 | 2387 | HACKBRY | 345 | NORTH |
| 36 | VLYRN | 345 | 2389 | IV VR | 345 | NORTH |
| 37 | JEWET | 345 | 3391 | JEWETT N | 345 | NORTH |
| 38 | JEWET | 345 | 3390 | JEWETT S | 345 | NORTH |

|  | ERCOT Operations |  | Steady State Working Group |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Station Mnemonic | kV | Bus Number | Name | kV | Trading Hub |
| 39 | KNEDL | 345 | 1932 | KENNDLE1 | 345 | NORTH |
| 40 | KLNSW | 345 | 3422 | KILL SS | 345 | NORTH |
| 41 | LCSES | 345 | 3409 | LAKE CRK | 345 | NORTH |
| 42 | LIGSW | 345 | 1916 | LIG2 T | 345 | NORTH |
| 43 | LEG | 345 | 46020 | LIMEST 5 | 345 | NORTH |
| 44 | LFKSW | 345 | 3117 | LUFKN SS | 345 | NORTH |
| 45 | LWSSW | 345 | 646 | LWSSWB | 345 | NORTH |
| 46 | MLSES | 345 | 3100 | MARTINLK | 345 | NORTH |
| 47 | MCCREE | 345 | 833 | MCCREE B | 345 | NORTH |
| 48 | MDANP | 345 | 1939 | MELP S | 345 | NORTH |
| 49 | MDANP | 345 | 1936 | MELP56NO | 345 | NORTH |
| 50 | ENTPR | 345 | 3116 | MT ENTRP | 345 | NORTH |
| 51 | NCDSE | 345 | 3119 | NACOG SE | 345 | NORTH |
| 52 | NORSW | 345 | 2406 | NORWD | 345 | NORTH |
| 53 | NUCOR | 345 | 3396 | NUCOR | 345 | NORTH |
| 54 | PKRSW | 345 | 1436 | PARKER | 345 | NORTH |
| 55 | KMCHI | 345 | 11697 | PITTSBRG | 345 | NORTH |
| 56 | PTENN | 345 | 2522 | PL TEN | 345 | NORTH |
| 57 | RENSW | 345 | 2355 | RENERTPL | 345 | NORTH |
| 58 | RCHBR | 345 | 3133 | RICHLND1 | 345 | NORTH |
| 59 | RCHBR | 345 | 3134 | RICHLND2 | 345 | NORTH |
| 60 | RNKSW | 345 | 1853 | ROANOKE | 345 | NORTH |
| 61 | RKCRK | 345 | 1880 | ROCKY CK | 345 | NORTH |
| 62 | RYSSW | 345 | 2461 | ROYSE S | 345 | NORTH |
| 63 | SGVSW | 345 | 2433 | SGVL SS | 345 | NORTH |
| 64 | SHBSW | 345 | 3103 | SHAMBRGR | 345 | NORTH |
| 65 | SHRSW | 345 | 1918 | SHERRY | 345 | NORTH |
| 66 | SHRTP | 345 | 1917 | SHERRY T | 345 | NORTH |
| 67 | SCSES | 345 | 3109 | STRYKER | 345 | NORTH |
| 68 | SYCRK | 345 | 1935 | SYCMR CK | 345 | NORTH |
| 69 | THSES | 345 | 3405 | T HOUSE | 345 | NORTH |
| 70 | TMPSW | 345 | 3413 | TEMPSSLT | 345 | NORTH |
| 71 | TNP_ONE | 345 | 39950 | TNP ONE | 345 | NORTH |
| 72 | TRCNR | 345 | 2432 | TRICORN | 345 | NORTH |
| 73 | TRSES | 345 | 3123 | TRINDAD1 | 345 | NORTH |
| 74 | TRSES | 345 | 3124 | TRINDAD2 | 345 | NORTH |
| 75 | TOKSW | 345 | 3400 | TWIN OAK | 345 | NORTH |
| 76 | VENSW | 345 | 1907 | VENUS N | 345 | NORTH |
| 77 | VENSW | 345 | 1906 | VENUS S | 345 | NORTH |

|  | ERCOT Operations |  | Steady State Working Group |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Station Mnemonic | kV | Bus Number | Name | kV | Trading Hub |
| 78 | WLVEE | 345 | 2398 | W LEVEE | 345 | NORTH |
| 79 | W_DENT | 345 | 988 | W.DENT B | 345 | NORTH |
| 80 | WTRML | 345 | 2427 | WATMILLW | 345 | NORTH |
| 81 | WCSWS | 345 | 1434 | WCPC | 345 | NORTH |
| 82 | WEBB | 345 | 1911 | WEBB 1 | 345 | NORTH |
| 83 | WHTNY | 345 | 240 | WHITNEY | 345 | NORTH |
| 84 | WCPP | 345 | 1421 | WILLOWCK | 345 | NORTH |

(2)	The North 345 kV Trading Hub Price is the simple average of the bus prices in each interval for each bus included in this Trading Hub.

7.9.2.2	South 345 kV Trading Hub

(1)	The South 345 kV Trading Hub is composed of the following Transmission Buses:

|  | ERCOT Operations |  | Steady State Working Group |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Station Mnemonic | kV | Bus Number | Name | kV | Trading Hub |
| 1 | AUSTRO | 345 | 7040 | AUSTRO34 | 345 | SOUTH |
| 2 | BLESSING | 345 | 8123 | BLESSNG6 | 345 | SOUTH |
| 3 | CAGNON | 345 | 5056 | CAGNON | 345 | SOUTH |
| 4 | COLETO | 345 | 8164 | COLETO 6 | 345 | SOUTH |
| 5 | CLEASP | 345 | 7050 | CRLSPG34 | 345 | SOUTH |
| 6 | NEDIN | 345 | 8383 | EDNBRG 6 | 345 | SOUTH |
| 7 | FAYETT | 345 | 7057 | FAYETT34 | 345 | SOUTH |
| 8 | FPPYD1 | 345 | 7056 | FPPYD134 | 345 | SOUTH |
| 9 | FPPYD2 | 345 | 7055 | FPPYD234 | 345 | SOUTH |
| 10 | GARFIE | 345 | 7048 | GARFIE34 | 345 | SOUTH |
| 11 | GUADG | 345 | 7047 | GPP | 345 | SOUTH |
| 12 | HAYSEN | 345 | 7043 | HAYSN 34 | 345 | SOUTH |
| 13 | HILLCTRY | 345 | 5211 | HILL CTY | 345 | SOUTH |
| 14 | HOLMAN | 345 | 9073 | HOLMAN | 345 | SOUTH |
| 15 | KENDAL | 345 | 7046 | KENDAL34 | 345 | SOUTH |
| 16 | LA_PALMA | 345 | 8317 | LAPALM 6 | 345 | SOUTH |
| 17 | LON_HILL | 345 | 8455 | LNHILL 6 | 345 | SOUTH |
| 18 | LOSTPI | 345 | 7041 | LOSTPN34 | 345 | SOUTH |
| 19 | LYTTON_S | 345 | 9074 | LYTTON | 345 | SOUTH |
| 20 | MARION | 345 | 7044 | MARION34 | 345 | SOUTH |
| 21 | PAWNEE | 345 | 5725 | PAWNESW6 | 345 | SOUTH |

|  | ERCOT Operations |  | Steady State Working Group |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Station Mnemonic | kV | Bus Number | Name | kV | Trading Hub |
| 22 | RIOHONDO | 345 | 8318 | RIOHND 6 | 345 | SOUTH |
| 23 | RIONOG | 345 | 7051 | RIONO 34 | 345 | SOUTH |
| 24 | SALEM | 345 | 7058 | SALEM 34 | 345 | SOUTH |
| 25 | SDSES | 345 | 3429 | SANDOW | 345 | SOUTH |
| 26 | SANMIGL | 345 | 52 | SANMIGEL | 345 | SOUTH |
| 27 | SKYLINE | 345 | 5371 | SKYLINE | 345 | SOUTH |
| 28 | STP | 345 | 5915 | SO TEX 5 | 345 | SOUTH |
| 29 | CALAVERS | 345 | 5400 | SPRUCE | 345 | SOUTH |
| 30 | BRAUNIG | 345 | 5475 | VON ROSE | 345 | SOUTH |
| 31 | WHITEPT | 345 | 8956 | WHITEPT | 345 | SOUTH |
| 32 | ZORN | 345 | 7042 | ZORN 34 | 345 | SOUTH |
| 33 | ZORN | 345 | 7045 | ZORN 34 | 345 | SOUTH |

(2)	The South 345 kV Trading Hub Price is the simple average of the bus prices in each interval for each bus included in this Trading Hub.

7.9.2.3	Houston 345 kV Trading Hub

(1)	The Houston 345 kV Trading Hub is composed of the following listed transmission buses:

|  | ERCOT Operations |  | Steady State Working Group |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Station Mnemonic | kV | Bus Number | Name | kV | Trading Hub |
| 1 | ADK | 345 | 45600 | ADICKS 5 | 345 | HOUSTON |
| 2 | _BI | 345 | 47000 | BELAIR 5 | 345 | HOUSTON |
| 3 | CBY | 345 | 40000 | CEDARP 5 | 345 | HOUSTON |
| 4 | CTR | 345 | 40240 | CENTER | 345 | HOUSTON |
| 5 | CHB | 345 | 40255 | CHAMBR 5 | 345 | HOUSTON |
| 6 | DPW | 345 | 40450 | DEPWTR 5 | 345 | HOUSTON |
| 7 | DOW | 345 | 42500 | DOW345 5 | 345 | HOUSTON |
| 8 | RNS | 345 | 40600 | FRONTR | 345 | HOUSTON |
| 9 | GBY | 345 | 40700 | GRNBYU 5 | 345 | HOUSTON |
| 10 | _JN | 345 | 47300 | JENETA 5 | 345 | HOUSTON |
| 11 | _KG | 345 | 40900 | KING 5 | 345 | HOUSTON |
| 12 | KDL | 345 | 45972 | KUYDAL 5 | 345 | HOUSTON |
| 13 | _NB | 345 | 46100 | N_BELT 5 | 345 | HOUSTON |
| 14 | _OB | 345 | 44500 | OBRIEN 5 | 345 | HOUSTON |
| 15 | PHR | 345 | 42000 | P_H_R_ 5 | 345 | HOUSTON |
| 16 | SDN | 345 | 41430 | SHELDN 5 | 345 | HOUSTON |
| 17 | SMITHERS | 345 | 44650 | SMTHRS 5 | 345 | HOUSTON |
| 18 | THW | 345 | 45500 | T_H_W_ 5 | 345 | HOUSTON |
| 19 | WAP | 345 | 44000 | W_A_P_ 5 | 345 | HOUSTON |
| 20 | _WO | 345 | 46600 | WHITOK 5 | 345 | HOUSTON |

(2)	The Houston 345 kV Trading Hub Price is the simple average of the bus prices in each interval for each bus included in this Trading Hub.

7.9.2.4	West 345 kV Trading Hub

(1)	The West 345 kV Trading Hub is composed of the following listed transmission buses:

|  | ERCOT Operations |  | Steady State Working Group |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Station Mnemonic | kV | Bus Number | Name | kV | Trading Hub |
| 1 | ABMB | 345 | 6230 | ABMULCE7 | 345 | WEST |
| 2 | BOMSW | 345 | 1422 | BOWMAN | 345 | WEST |
| 3 | OECCS | 345 | 1020 | EHV TIET | 345 | WEST |
| 4 | BTRCK | 345 | 1050 | ENRONIPP | 345 | WEST |
| 5 | FSHSW | 345 | 1425 | FISHRDSS | 345 | WEST |
| 6 | FLCNS | 345 | 1025 | FS COGEN | 345 | WEST |
| 7 | GRSES | 345 | 1430 | GRAHAM | 345 | WEST |
| 8 | JCKSW | 345 | 1429 | JXBRO SS | 345 | WEST |
| 9 | MDLNE | 345 | 1021 | MIDL E T | 345 | WEST |
| 10 | MOSSW | 345 | 1018 | MOSS | 345 | WEST |
| 11 | MGSES | 345 | 1030 | MRGN CRK | 345 | WEST |
| 12 | DCTM | 345 | 6096 | NORTHDC7 | 345 | WEST |
| 13 | ODEHV | 345 | 1026 | ODES EHV | 345 | WEST |
| 14 | OKLA | 345 | 6100 | OKLAEHV7 | 345 | WEST |
| 15 | SARC | 345 | 6444 | SAREDCK7 | 345 | WEST |
| 16 | SWCOG | 345 | 1420 | SWEETWTR | 345 | WEST |
| 17 | TWINBUTE | 345 | 6009 | TWBT7 | 345 | WEST |

(2)	The West 345 kV Trading Hub Price is the simple average of the bus prices in each interval for each bus included in this Trading Hub.

7.9.2.5	ERCOT Hub Average 345 kV Trading Hub

The ERCOT Hub Average 345 kV Trading Hub Price is the simple average of the North 345 kV Trading Hub Price, South 345 kV Trading Hub Price, Houston 345 kV Trading Hub Price, and West 345 kV Trading Hub Price.

7.9.2.6	ERCOT Bus Average 345 kV Trading Hub

(1)	The ERCOT Bus Average 345 kV Trading Hub is composed of the buses listed in Section 7.9.2.1,  North 345 kV Trading Hub, Section 7.9.2.2, South 345 kV Trading Hub, Section 7.9.2.3, Houston 345 kV Trading Hub and Section 7.9.2.4,  West 345 kV Trading Hub.

(2)	Until superseded by implementation of a different market design for the ERCOT market (e.g., a nodal market design), the ERCOT Bus Average 345 kV Trading Hub Price for each interval shall approximately consist of a fifty-six percent (56%) weighting applied to the North 345 kV Trading Hub price, a twenty-two percent (22%) weighting applied to the South 345 kV Trading Hub price, a thirteen percent (13%) weighting applied to the Houston 345 kV Trading Hub price, and a nine percent (9)% weighting applied to the West 345 kV Trading Hub price.  For the exact weights, please refer to Section 7.9.3.4, Scheduling of Transactions Using ERCOT Trading Hubs.

7.9.3	ERCOT Responsibilities

7.9.3.1	Posting of a List of Buses included in Trading Hubs

(1)	ERCOT shall post a list of all the buses included in each ERCOT Trading Hub on the public MIS.  The list must include the bus name and kV rating as used by ERCOT Operations, the bus number, bus name and kV rating as used in the Steady State Working Group (SSWG) base cases, and the Congestion Zone that the bus is assigned to.

(2)	ERCOT shall update the list of buses included in each ERCOT Trading Hub as soon as possible whenever a new Trading Hub is created under this Section 7.9 or when affected Transmission Buses are renamed in either the ERCOT Operations model or the SSWG model.

7.9.3.2	Calculation of Trading Hub Prices

ERCOT shall calculate Trading Hub Prices for each Settlement Interval. The bus price for each Transmission Bus for each Settlement Interval is the Market Clearing Price at that bus as determined elsewhere in these Protocols for that Settlement Interval.

7.9.3.3	Posting of Trading Hub Prices

ERCOT shall post the Trading Hub Prices for each ERCOT Trading Hub for each Settlement Interval as soon as practicable on the public MIS.

7.9.3.4	Scheduling of Transactions Using ERCOT Trading Hubs

(1)	Until ERCOT’s scheduling system is updated to automatically convert transactions involving Trading Hubs into transactions involving Congestion Zones, Qualified Scheduling Entities (QSEs) having any transaction involving an ERCOT Trading Hub shall convert each such transaction to one that can be expressed as a scheduled transaction between existing Congestion Zones.

(2)	Market Participants shall use the following matrix to convert transactions at Trading Hubs to existing Congestion Zones:

|  | 2009 Congestion Zone |  |  |  |
| --- | --- | --- | --- | --- |
| Trading Hub | North | South | Houston | West |
| North | .976 | 0.024 |  |  |
| South |  | 0.97 | 0.03 |  |
| Houston | 0.050 |  | 0.950 |  |
| West | 0.235 |  |  | 0.765 |
| Bus Average | 0.571 | 0.221 | 0.123 | 0.085 |
| ERCOT Average | 0.315 | 0.249 | 0.245 | 0.191 |