| NPRR Number | 190 | NPRR Title | Clarification of Resource Definitions and Resource Registration of Self-Serve Generators for Reliability Purposes |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Tabled |
| Date of Decision |  | December 17, 2009 |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms and Abbreviations 3.1.3.2, Resources 3.1.4.5, Management of Resource or Transmission Forced Outages or Maintenance Outages 3.1.4.6, Notice of Forced Outage or Unavoidable Extension of Planned or Maintenance Outage Due to Unforeseen Events 3.1.4.8, Deratings 3.1.6, Outages of Resources Other than Reliability Resources 3.1.6.1, Receipt of Resource Requests by ERCOT 3.1.6.2, Resource Outage Plan 3.10.7.2, Modeling of Resources and Transmission Loads 3.10.7.3, Modeling of Private Use Networks 6.5.1, ERCOT Activities 6.5.5.2, Operational Data Requirements 6.5.7, Energy Dispatch Methodology 6.5.7.8, Dispatch Procedures 6.5.8, Verbal Dispatch Instructions 6.5.9, Emergency Operations 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone 16.5, Registration of a Resource Entity 16.5.1.1, Designation of a Qualified Scheduling Entity |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) adds language to acknowledge the existence of Generation Resources that have no or minimal injection onto the ERCOT System that qualify either as distributed generation or are registered with the Public Utility Commission of Texas (PUCT) according to P.U.C. Subst. R. 25.109, Registration of Power Generation Companies and Self-Generators, as a self-generator but are required to participate with ERCOT for reliability purposes. |  |  |
| Overall Market Benefit |  | Inclusion of all self-generator types in the reliability obligation set forth by the Nodal Protocols and by North American Electric Reliability Corporation (NERC) standards. |  |  |
| Overall Market Impact |  | All generator types that impact operation and reliability planning of the ERCOT System will be required to register and participate in Real-Time data, Dispatching, and Outage Coordination with ERCOT for reliability purposes. |  |  |
| Consumer Impact |  | Enhanced reliability. |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - On 7/24/09, NPRR190 and a CEO Revision Request Review were posted. - On 8/24/09, TIEC comments were posted. - On 8/25/09, PRS considered NPRR190. - On 9/15/09, ERCOT comments were posted. - On 9/17/09, PRS again considered NPRR190. - On 10/22/09, PRS again considered NPRR190. - On 11/19/09, PRS again considered NPRR190. - On 12/17/09, PRS again considered NPRR190. |  |  |
| PRS Decision |  | On 8/25/09, PRS unanimously voted to table NPRR190 for one month. All Market Segments were present for the vote. On 9/17/09, PRS unanimously voted to table NPRR190 for one month. All Market Segments were present for the vote. On 10/22/09, PRS unanimously voted to table NPRR190 for one month. All Market Segments were present for the vote. On 11/19/09, PRS unanimously voted to table NPRR190. All Market Segments were present for the vote. On 12/17/09, PRS unanimously voted to table NPRR190 indefinitely. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 8/25/09, a representative of TIEC explained that discussions with ERCOT Staff are ongoing and that both TIEC and ERCOT Staff support tabling NPRR190 and the corresponding zonal Protocol Revision Request (PRR) 826, Clarification of Resource Definitions and Resource Registration of Self-Serve Generators for Reliability Purposes, for one month to refine the proposed language changes. ERCOT Staff explained that ERCOT submitted NPRR190 and PRR826 so that ERCOT will be able to coordinate with certain small generators for reliability purposes if necessary. On 9/17/09, there was no discussion. On 10/22/09, ERCOT Staff explained that it was working through some of the issues raised by Market Participants regarding NPRR190 and requested that NPRR190 be tabled for another month. On 11/19/09, ERCOT Staff explained that it was still working through some of the issues raised by Market Participants regarding NPRR190 and requested that NPRR190 be tabled. On 12/17/09, ERCOT Staff explained that it was still working through some of the issues raised by Market Participants regarding NPRR190, requested that NPRR190 be tabled indefinitely, and indicated that ERCOT should know by February 2010 whether ERCOT wants to proceed with this NPRR. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | All generator types that impact operation and reliability planning of the ERCOT System will be required to register and participate in Real-Time data, Dispatching, and Outage Coordination with ERCOT for reliability purposes. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Inclusion of all self-generator types in the reliability obligation set forth by the Nodal Protocols and by NERC standards. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| TIEC 082409 | Requested that NPRR190 be tabled pending discussions with ERCOT. |
| ERCOT 091509 | Requested that NPRR190 be tabled for one month. |

| Sponsor |  |
| --- | --- |
| Name | Joel Mickey |
| Company | ERCOT |
| Market Segment | Not applicable |

Proposed Protocol Language Revision

## 2.1	DEFINITIONS

Distributed Generation

As defined by P.U.C. Subst. R. 25.211, Interconnection of On-Site Distributed Generation (DG), an electrical generating facility located at a customer's point of delivery (point of common coupling) of ten megawatts (MW) or less and connected at a voltage less than 60 kilovolts (kV) which may be connected in parallel operation to the utility system.

Distributed Renewable Generation

Distributed Generation with a capacity of not more than 2,000 kW provided by a renewable energy technology that is installed on a retail electric Customer’s side of the meter.

Resource

The term is used to refer to both a Generation Resource and a Load Resource.  “Resource” used by itself in these Protocols does not include  Distributed Generation.

All-Inclusive Generation Resource

A term used to refer to both a Generation Resource, and Distributed Generation.

All-Inclusive Resource

A term used to refer to a Generation Resource, Load Resource, and Distributed Generation.

Controllable Load Resource

Load Resource capable of controllably reducing or increasing consumption under dispatch control (similar to AGC) and that immediately responds proportionally to frequency changes (similar to generator governor action).

Generation Resource

A generator that is capable of providing energy or Ancillary Service to the ERCOT System and is registered with ERCOT as a Generation Resource. “Generation Resource” used by itself in these Protocols does not include  Distributed Generation.

Isolated Generation Resource (IGR)

Generation Resource that produces energy within a Private Use Network and that is owned or operated by a registered Generation Entity with ERCOT approved procedure for isolating facilities from power injection onto the ERCOT System and that are registered with ERCOT as a Self-Serve Generator and excluded from ERCOT System reserves.

Mothballed Generation Resource

A Generation Resource for which a Generation Entity has submitted a Notification of Suspension of Operations, for which ERCOT has declined to execute an RMR Agreement, and for which the Generation Entity has not announced retirement of the Generation Resource.

Self-Serve Generator

A Generation Resource that is represented in the Network Operations Model and is:

(a)	Capable of providing net output of energy to the ERCOT System from a Private Use Network;

(b)	Registered with the PUCT according to P.U.C. Subst. R. 25.109, Registration of Power Generation Companies and Self-Generators, as a self-generator; and

(c)	Registered with ERCOT as a Self-Serve Generator.

Additionally, in accordance with these Protocols, a Self-Serve Generator may be required to perform certain operational and reliability responsibilities and may elect not to participate in the Ancillary Service or energy markets, RUC, or SCED.  Notwithstanding the above, Self-Serve Generator(s) on a Private Use Network with aggregate capacity of less than ten MW may be excluded from the Network Operations Model at the discretion of ERCOT.

Switchable Generation Resource

A Generation Resource that can be connected to either the ERCOT Transmission Grid or a non-ERCOT Control Area.

Wind-powered Generation Resource (WGR)

A Generation Resource that is powered by wind.

Load Resource

A load that is capable of providing Ancillary Service to the ERCOT System and is registered with ERCOT as a Load Resource.

## 2.2	ACRONYMS AND ABBREVIATIONS

IGR			Isolated Generation Resource

3.1.3.2	Resource Entities

(1)	Each Resource Entity shall provide to ERCOT a Planned Outage and Maintenance Outage plan in an ERCOT-provided format for the next 12-months updated monthly.  Planned Outage and Maintenance Outage scheduling data for Resource Facilities must be kept current.  Updates, through an electronic interface as specified by ERCOT, must identify any changes to previously proposed Planned Outages or Maintenance Outages and any additional Planned Outages or Maintenance Outages anticipated over the next 12 months.  Isolated Generation Resources (IGRs) are exempt from the rolling 12 month Planned Outage and Maintenance Outage plan.

(2)	ERCOT shall report statistics monthly on how Resource Planned Outages compare with actual Resource Outages, and post those statistics to the MIS Secure Area.

3.1.4.5	Management of Resource or Transmission Forced Outages or Maintenance Outages

(1)	In the event of a Forced Outage, after the affected equipment is removed from service, the Resource Entity or QSE, as appropriate, or TSP must notify ERCOT as soon as practicable of its action by:

(a)	For Resource Outages:

(i)	changing the telemetered Resource Status appropriately, including a text description when it becomes known, of the cause of the Forced Outage; and

(ii)	updating the COP; and

(iii)	updating the Outage Scheduler, if necessary.

(b)	For Transmission Facilities Forced Outages:

(i)	changing the telemetered status of the affected Transmission Elements; and

(ii)	updating the Outage Scheduler with the expected return-to-service time.

(c)	For IGR Outages, updating the Outage Scheduler, if necessary.

(2)	Forced Outages may require ERCOT to review and withdraw approval of previously approved or accepted, as applicable, Planned Outage or Maintenance Outage schedules to ensure reliability.

(3)	For Maintenance Outages, the Resource Entity or QSE, as appropriate, or TSP shall notify ERCOT of any Resource or Transmission Facilities Maintenance Outage according to the Maintenance Outage Levels by updating the Current Operating Plan and Outage Scheduler.  ERCOT shall coordinate the removal of facilities from service within the defined timeframes as specified by the TSP, QSE or Resource Entity in its notice to ERCOT.

(4)	ERCOT may require supporting information describing Forced Outages and Maintenance Outages.  ERCOT may reconsider and withdraw approvals of other previously approved Transmission Facilities Outage or an Outage of a Reliability Resource as a result of Forced Outages or Maintenance Outages, if necessary, in ERCOT’s determination to protect system reliability.  When ERCOT approves a Maintenance Outage, ERCOT shall coordinate timing of the appropriate course of action under these Protocols.

(5)	Removal of a Generation Resource or Transmission Facilities from service under Maintenance Outages must be coordinated with ERCOT.  To minimize harmful impacts to the system in urgent situations, the equipment may be removed immediately from service, provided notice is given immediately, by the Resource Entity or TSP, to ERCOT of such action.

3.1.4.6	Notice of Forced Outage or Unavoidable Extension of Planned or Maintenance Outage Due to Unforeseen Events

(1)	If a Planned or Maintenance Outage is not completed within the ERCOT-approved timeframe and the Transmission Facilities or Generation Resources are in such a condition that they cannot be restored at the Outage schedule completion date, the requesting party shall submit to ERCOT a Forced Outage (unavoidable extension) form describing the extension of the Outage and providing a revised return date.

(2)	Any Forced Outage that occurs in Real-Time must be entered into the Outage Scheduler if it is to remain an Outage for longer than two hours.

3.1.4.8	Deratings

The Resource Entity or its designee must enter material deratings that are expected to last more than 48 hours in the ERCOT Outage Scheduler.  A derating is considered to be material when the Generation Resource’s capability is reduced by the greater of ten percent or ten MW due to the loss of auxiliary equipment or other known conditions.  ERCOT will consider the Generation Resource’s capability as the lesser of the latest seasonal Net Dependable Capability test, or the asset registration submittal.

3.1.6	Outages of Generation Resources Other than Reliability Resources

(1)	ERCOT shall accept all Outage schedules and changes to Outage schedules for a Resource other than a Reliability Resource submitted to ERCOT more than eight days before the proposed start date of the Outage.  For purposes of this Section, the type of Resources required to be incorporated into Resource Entity’s Outage plan is limited to Generation Resources that are modeled in the Network Operations Model.

(2)	If a Resource Entity plans to start a Planned or Maintenance Outage within eight days that has not been previously included in the Resource’s written Planned Outage and Maintenance Outage plan, then the Resource Entity must immediately notify ERCOT and include in its notice whether the Outage is a Forced Outage, Maintenance (Level I, II, or III) Outage, or Planned Outage.  ERCOT’s response to this notification must comply with these requirements:

(a)	ERCOT shall accept Forced and Levels I, II, and III Maintenance Outages proposals, and ERCOT shall coordinate the Outages within the time frames specified in these Protocols.

(b)	ERCOT shall accept Planned Outage proposals, except that ERCOT shall reject an Outage proposal if it will impair ERCOT’s ability to meet applicable reliability standards and other solutions cannot be exercised.

(c)	ERCOT shall accept Forced and Maintenance Outage plans from a Qualifying Facility (QF) that result from the outage of the QF’s thermal host facility.

3.1.6.1	Receipt of Resource Entity Requests by ERCOT

ERCOT shall acknowledge each request for approval of a Resource Planned Outage schedule within two Business Hours of the receipt of the request.  ERCOT may request additional information or seek clarification from the Resource Entity regarding the information submitted for a proposed Planned Outage or Maintenance Outage for Resource Facilities.

3.1.6.2	Resource Entity’s Outage Plan

(1)	Resource Entity Outage requests shall include the following information:

(a)	The primary and alternate phone number of the Resource Entity’s Single Point of Contact for Outage Coordination;

(b)	The Resource identified by the name in the Network Operations Model;

(c)	The net megawatts of capacity the Resource Entity anticipates will be available during the Outage (if any);

(d)	The estimated start and finish dates for each Planned and Maintenance Outage;

(e)	An estimate of the acceptable deviation in the Outage schedule (i.e., the earliest start date and the latest finish date for the Outage); and

(f)	The nature of work to be performed during the Outage.

(2)	When ERCOT accepts a Maintenance Outage, ERCOT shall coordinate the timing of the appropriate course of action within the Resource Entity’s specified timeframe.  The QSE or Resource Entity’s designated agent shall notify ERCOT of the Outage and coordinate the time.

3.10.7.2	Modeling of Resources and Transmission Loads

(1)	Each Resource Entity shall provide ERCOT and TSPs with information describing each of its Generation Resources and Load Resources connected to the transmission system.  All Generation Resources greater than ten MW, Generation Resources less than ten MW but providing Ancillary Service, Split Generation Resources, Private Use Networks containing Generation Resources greater than ten MW, Direct Current Tie (DC Tie) Resources, and the non-TSP owned step-up transformers greater than ten MVA, must be modeled to provide equivalent generation injections to the ERCOT Transmission Grid or to provide for reliability coordination.  ERCOT shall coordinate the modeling of Generation Resources, Private Use Networks, DC Tie Resources and Load Resources with their owners to ensure consistency between TSP models and ERCOT models.

(2)	Each Resource Entity representing a Split Generation Resource shall provide information to ERCOT and TSPs describing an individual Split Generation Resource for its share of the Generation facility to be represented in the Network Operations Model in accordance with Section 3.8, Special Considerations for Split Generation Meters.  The Split Generation Resource must be modeled as connected to the ERCOT Transmission Grid on the low side of the generation facility main power transformer.

(3)	ERCOT shall create a DC Tie Resource to represent an equivalent generation injection to represent the flow into the ERCOT Transmission Grid from operation of DC Ties.  The actual injection flow on the DC Tie from telemetry provided by the facility owner(s) is the DC Tie Resource output.

(4)	TSPs shall provide ERCOT with information describing all transmission Load connections on the ERCOT Transmission Grid.  Individual Load connections may be combined, at the discretion of ERCOT, with other Load connections on the same transmission line to represent a Model Load to facilitate state estimation of Loads that do not telemeter Load measurements.  ERCOT shall define “Model Loads,” which may be one or more combined Loads, for use in its Network Operations Model.  A Model Load cannot be used to represent Load connections that are in different Load Zones.

(5)	ERCOT may require TSPs to provide additional Load telemetry to provide adequate modeling of the transmission system in accordance with Section 3.10.7.5, Telemetry Criteria.  When the TSP does not own the station for which additional Load telemetry is being requested, the TSP shall request that the owner make the telemetry available.  The TSP shall notify ERCOT if the owner does not comply with the request.

(6)	ERCOT shall create a DC Tie Load to represent an equivalent Load withdrawal to represent the flow from the ERCOT Transmission Grid from operation of DC Ties.  The actual withdrawal flow on the DC Tie from telemetry provided by the facility owner(s) is the DC Tie Load output.

(7)	Each TSP shall also provide information to ERCOT describing automatic Load transfer (rollover) plans and the events that trigger which Loads are switched to other Transmission Elements on detection of Outage of a primary Transmission Element.  ERCOT shall accommodate Load rollover plans in the Network Operations Model

(8)	Loads associated with a Generation Resource in a common switchyard as defined in Section 10.3.2.3, Generation Netting for EPS Meters, and served through a transformer owned by the Generation Entity is treated as an auxiliary Load and must be netted first against any generation meeting the requirements under Section 10.3.2.3.

3.10.7.3	Modeling of Private Use Networks

ERCOT shall create and use network models describing Private Use Networks according to the following:

(1)	A Generation Entity with a Generation Resource located within a Private Use Network shall provide data to ERCOT, for use in the Network Operations Model, for each of its individual generating unit(s) located within the Private Use Network in accordance with the Protocols if it meets any one of the following criteria:

(a)	Contains a generator greater than ten MW and is registered with the PUCT according to P.U.C. Subst. R. 25.109, Registration of Power Generation Companies and Self-Generators, as a power generation company; or

(b)	Is part of a Private Use Network which contains more than one connection to the ERCOT Transmission Grid; or

(c)	Contains generation registered to provide Ancillary Services,

(2)	A Generation Entity with a Generation Resource greater than ten MW located within a Private Use Network which does not meet any of the criteria of item (1) above shall provide to ERCOT annually, or more often upon change, the following information for ERCOT’s use in the Network Operations Model, for each of its individual generating unit(s) located within the Private Use Network:

(a)	Equipment owner(s);

(b)	Equipment operator(s);

(c)	TSP substation name connecting the Private Use Network to the ERCOT System;

(d)	At the request of ERCOT, a description of Transmission Elements within the Private Use Network that may be connected through breakers or switches;

(e)	Net energy delivery metering, as required by ERCOT, to and from a the Private Use Network and the ERCOT System at the point of interconnection with the TSP;

(f)	For each individual generator located within the Private Use Network, the gross capacity in MW and its reactive capability curve;

(g)	Maximum and minimum reasonability limits of the Load located within the Private Use Network.  Private Use Networks with only Isolated Generation Resources (IGRs) are exempt;

(h)	Outage schedule for each generation unit located within the Private Use Network, updated as changes occur from the annually submitted information; and

(i)	Other interconnection data as required by ERCOT.

(3)	Energy delivered to ERCOT from a Self-Serve Generator, and elects not to participate in the Ancillary Service or energy markets, RUC, or SCED, shall be settled in accordance with Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone.

(4)	ERCOT shall ensure the Network Operations Model properly models the physical effect of the loss of generators and Transmission Elements on the ERCOT Transmission Grid equipment loading, voltage, and stability.

(5)	ERCOT may require the owner or operator of a Private Use Network to provide information to ERCOT and the TSP on Transmission Facilities located within the Private Use Network for use in the Network Operations Model if the information is required to adequately model and determine the security of the ERCOT Transmission Grid, including data to perform loop flow analysis of Private Use Networks.

(6)	ERCOT shall review submittals of modeling data from owners or operators of Private Use Networks assure that it will result in correct analysis of ERCOT Transmission Grid security.

6.5.1	ERCOT Activities

ERCOT activities during Real-Time operations are summarized in the table located in Section 6.3.2, Activities for Real-Time Operations.  That table is intended to be only a general guide and not controlling language, and any conflict between the table and another section of the Protocols is controlled by the other section.  For purposes of this Section, the types of Resources referred to regarding ERCOT’s Dispatching activities are Generation Resources that are modeled in the Network Operations Model.

6.5.5.2	Operational Data Requirements

(1)	ERCOT shall use Operating Period data to monitor and control the reliability of the ERCOT Transmission Grid and shall use it in network analysis software to predict the short-term reliability of the ERCOT Transmission Grid.  Each TSP, at its own expense, may obtain that Operating Period data from ERCOT or directly from QSEs or Generation Resources.

(2)	A QSE representing a Generation Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time telemetry data to ERCOT for each Generation Resource.  ERCOT shall make that data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to requesting TSPs and DSPs operating within ERCOT.  With the exception of Isolated Generation Resources modeled in the Network Operations Model such data must be provided to the requesting TSP or DSP at the requesting TSP’s or DSP’s expense, including:

(a)	Net real power (in MW) as measured by installed power metering or as calculated in accordance with ERCOT Operating Guides based on metered gross real power and conversion constants determined by the Resource Entity and provided to ERCOT as a result of Section 3.7, Resource Parameters.  Net real power represents the actual generation of a Resource for all real power dispatch purposes, including use in Security-Constrained Economic Dispatch (SCED), determination of the High Ancillary Service Limit (HASL), High Dispatch Limit (HDL), Low Dispatch Limit (LDL) and Low Ancillary Service Limit (LASL), and is consistent with telemetered HSL and LSL;

(b)	Gross real power (in MW) as measured by installed power metering or as calculated in accordance with ERCOT Operating Guides based on metered real power, which may include Supervisory Control and Data Acquisition (SCADA) metering, and conversions constants determined by the Resource Entity and provided to ERCOT as a result of Section 3.7;

(c)	Gross Reactive Power (in Mega Volt-Amperes reactive (MVAr));

(d)	Net Reactive Power (in MVAr);

(e)	Power to standby transformers serving plant auxiliary Load;

(f)	Status of switching devices in the plant switchyard not monitored by the TSP or DSP affecting flows on the ERCOT Transmission Grid;

(g)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(h)	Generation Resource breaker and switch status;

(i)	HSL;

(j)	High Emergency Limit (HEL), under Section 6.5.9.2, Failure of the SCED Process;

(k)	Low Emergency Limit (LEL), under Section 6.5.9.2;

(l)	LSL;

(m)	Ancillary Service Schedule for each quantity of Regulation Up Service (Reg-Up), Regulation Down Service (Reg-Down), Responsive Reserve and Non-Spin:

(i)	For Responsive Reserve and Non-Spin, the Ancillary Service Schedule is equal to the Ancillary Service Resource Responsibility minus the amount of Ancillary Service deployment; and

(ii)	For Regulation Service, the Ancillary Service Schedule is equal to the Ancillary Service Resource Responsibility;

(n)	Ancillary Service Resource Responsibility for each quantity of Reg-Up, Reg-Down, Responsive Reserve and Non-Spin.  The sum of Ancillary Service Resource Responsibility for all Resources in a QSE is equal to the Ancillary Service Supply Responsibility for that QSE; and

(o)	Reg-Up and Reg-Down Services participation factors represent how a QSE is planning to deploy the Ancillary Service energy on a percentage basis to specific qualified Resource.

(3)	For each Wind-powered Generation Resource (WGR), the QSE shall set the HSL to the output capability of the facility based upon all available units and the current measured wind speed (HSL must be equal to or greater than the latest telemetered net real power of the WGR).

(4)	A QSE representing a Load Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time data to ERCOT for each Load Resource and ERCOT shall make the data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to the Load Resource’s host TSP or DSP at the TSP’s or DSP’s expense.  The Load Resource’s net real power consumption, Low Power Consumption (LPC) and Maximum Power Consumption (MPC) shall be telemetered to ERCOT using a positive (+) sign convention:

(a)	Load Resource net real power consumption (in MW);

(b)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(c)	Load Resource breaker status;

(d)	LPC (in MW);

(e)	MPC (in MW);

(f)	Ancillary Service Schedule (in MW) for each quantity of Reg-Up, Reg-Down, Responsive Reserve and Non-Spin;

(g)	Ancillary Service Resource Responsibility (in MW) for each quantity of Reg-Up, Reg-Down, Responsive Reserve and Non-Spin;

(h)	The status of the high-set under-frequency relay, if required for qualification;

(i)	For a Controllable Load Resource, the Scheduled Power Consumption that represents zero Ancillary Service deployments; and

(j)	For a Controllable Load Resource, net Reactive Power (in MVAr).

(5)	A QSE with Resources used in SCED shall provide communications equipment to receive ERCOT-telemetered control deployments.

(6)	A QSE providing any Regulation Service shall provide telemetry indicating the appropriate status of Resources providing Reg-Up or Reg-Down, including status indicating whether the Resource is temporarily blocked from receiving Reg-Up and/or Reg-Down deployments from the QSE.

(7)	Real-Time data for reliability purposes must be accurate to within 3%.  This telemetry may be provided from relaying accuracy instrumentation transformers.

(8)	Each QSE shall report the current configuration of combined-cycle Resources that it represents to ERCOT.

(a)	Each configuration for a power block of combined-cycle Resources is considered as a single Resource unless the different units of the combined-cycle Resource have separate injection points to the ERCOT Transmission Grid.  Units with separate injection points to the ERCOT Transmission Grid may be aggregated with ERCOT approval only after ERCOT implements software to accept and implement incremental and decremental generation disaggregation weightings for each combined cycle unit’s operating mode.

(b)	Each QSE shall use continuous telemetry to report changes to Combined-Cycle Configurations.  Changes must be reported by changing the Resource Status in Real-Time and in the COP for that Resource representing the desired Combined-Cycle Configuration.  Each QSE shall provide ERCOT with the elements comprising each Combined-Cycle Configuration for a Resource through Real-Time telemetry and by appropriate entries in the COP.

(c)	Each QSE shall provide individual telemetered generator output (MW and MVAr) and Resource Status that indicates the Combined-Cycle Configuration to be used in SCED and Reliability Unit Commitment (RUC).

(9)	A QSE representing combined-cycle Resources shall provide ERCOT with the possible operating configurations for each power block with accompanying limits and price points.  Power augmentation methods must be made available to ERCOT as part of one or more of the configurations.  Price points for the range of the curve represented by the power augmentation method must reflect the price of the added capability.  Such power augmentation methods may include:

(a)	Combustion turbine inlet air cooling (CTIAC) methods;

(b)	Duct firing;

(c)	Other ways of temporarily increasing the output of combined-cycle Resources; and

(d)	For Qualifying Facilities (QFs), an LSL that represents the minimum energy available, in MW, from the Resource for economic dispatch based on the minimum stable steam delivery to the thermal host plus a justifiable reliability margin that accounts for changes in ambient conditions.

(10)	A QSE or TSP with an operational agreement with an Isolated Generation Resource(s) (IGR(s)) within a Private Use Network connected to Transmission Facilities shall provide the following Real-Time telemetry data to ERCOT for each IGR.  ERCOT shall make that data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to requesting TSPs and DSPs operating within ERCOT.  Such data must be provided to the requesting TSP at the requesting TSP’s expense, including:

(a)	Gross real power (in MW) as measured by installed power metering or as calculated in accordance with ERCOT Operating Guides based on metered real power, which may include SCADA metering, and conversions constants determined by the Resource Entity and provided to ERCOT as a result of Section 3.7;

(b)	Gross Reactive Power (in Mega Volt-Amperes reactive (MVAr));

(c)	Status of switching devices in the plant switchyard not monitored by the TSP affecting flows on the ERCOT Transmission Grid;

(d)	Any data mutually agreed to by ERCOT and the QSE or TSP to adequately manage system reliability;

(e)	Generation Resource breaker and switch status;

6.5.7	Energy Dispatch Methodology

This Section outlines the programmatic and manual processes employed by ERCOT to simultaneously achieve power balance (minimizing the use of Regulation Service) and manage congestion while operating within the constraints of the system at economically optimized cost.  The Real-Time Sequence describes the key system components and inputs that are required to support the SCED process, which produces the Locational Marginal Prices (LMPs) and Base Points while meeting transmission system constraints.  Section 6.5.7.3, Security Constrained Economic Dispatch, provides further details regarding additional components and inputs and ex-ante mitigation.  For purposes of this Section, the types of Resources referred to in Section 6.5.7.1, Real-Time Sequence, and Section 6.5.7.3 are Generation Resources except Self-Serve Generators that are not participating in SCED.

6.5.7.8	Dispatch Procedures

(1)	ERCOT shall issue all Resource Dispatch Instructions to the QSE that represents the affected Resource.  ERCOT shall coordinate Dispatch Instructions with TSPs or QSEs which have an operational agreement with an IGR to coordinate the operations of the IGR’s transmission interconnection.  ERCOT may not issue Dispatch Instructions to the QSE for Resources with a Resource Status of ONTEST (which indicates that it is undergoing testing), except:

(a)	For Dispatch Instructions that are a part of the testing; or

(b)	During conditions when the Resource is the only alternative for solving a transmission constraint; or

(c)	During Force Majeure Events that threaten the reliability of the ERCOT System.

(2)	Each QSE shall immediately forward any valid Dispatch Instruction to the appropriate Resource or group of Resources or identify a reason for non-compliance with the Dispatch Instruction to ERCOT in accordance with Section 6.5.7.9, Compliance with Dispatch Instructions.  An IGR must comply with an authorized Dispatch Instruction from either the QSE or TSP registered as responsible for coordinating the IGR’s interconnection.  ERCOT shall not issue Dispatch Instructions affecting an IGR until market remedies have been exhausted.

(3)	If ERCOT believes that a Resource or IGR has inadequately responded to a Dispatch Instruction, ERCOT shall notify the QSE or TSP representing the Resource or IGR as soon as practicable.

(4)	The recipient of a Verbal Dispatch Instruction (VDI) shall confirm the Dispatch Instruction by providing the receiving operator’s identification and by repeating the VDI to ERCOT orally.

(5)	The recipient of an electronic Dispatch Instruction shall acknowledge receipt of the Dispatch Instruction to ERCOT electronically, within one minute.  The electronic acknowledgement must include the receiving operator’s identification.

(6)	The recipient of any Dispatch Instruction shall immediately request clarification of the Dispatch Instruction if the recipient fails to understand its responsibility under the Dispatch Instruction.

(7)	ERCOT shall record all voice conversations that occur in the communication of Dispatch Instructions.

(8)	ERCOT shall record and file all electronic Dispatch Instructions and acknowledgements as soon as practicable after the issuance of the Dispatch Instruction.

(9)	By mutual agreement of the TSP and ERCOT, Dispatch Instructions to the TSP may be provided to the TSP’s designated agent.  In that case, issuance of the Dispatch Instruction to the designated agent is considered issuance to the TSP, and the TSP must comply with the Dispatch Instruction exactly as if it had been issued directly to the TSP, whether or not the designated agent accurately conveys the Dispatch Instruction to the TSP.

6.5.8	Verbal Dispatch Instructions

A VDI must contain the following information:

(a)	Identification of the responsible Entity and instructing authority (to include ERCOT Operator’s and receiving operator’s names);

(b)	Specific Resources, Self-Serve Generators, or TSP facilities that are the subject of the Dispatch Instruction;

(c)	Specific action required;

(d)	Current operating level or state of the Resources, Self-Serve Generators, or TSP facilities that are the subject of the Dispatch Instruction;

(e)	Operating level or state to which such Resources, Self-Serve Generators, or facilities will be dispatched;

(f)	Time of notification of the Dispatch Instruction;

(g)	Time at which the QSE or TSP is required to initiate the Dispatch Instruction;

(h)	Time within which the QSE or TSP is required to complete the Dispatch Instruction;

(i)	VDI reference number; and

(j)	Other information relevant to that Dispatch Instruction.

6.5.9	Emergency Operations

(1)	ERCOT, based on ERCOT System reliability needs, may issue a Dispatch Instruction requiring a Resource to move to a specific output level (“Emergency Base Point”).

(2)	A QF may only be ordered Off-Line in the case of an ERCOT-declared Emergency Condition with imminent threat to the reliability of the ERCOT System.  ERCOT may only Dispatch a QF below its LSL when ERCOT has declared an Emergency Condition and the QF is the only Resource that can provide the necessary relief.

(3)	ERCOT shall honor all Resource operating parameters in Dispatch Instructions under normal conditions and Emergency Conditions.  During Emergency Conditions, ERCOT may verbally request QSEs to operate its Resources outside normal operating parameters.  If such request is received by a QSE, the QSE shall discuss the request with ERCOT in good faith and may choose to comply with the request.

(4)	A QSE may not self-arrange for Ancillary Services procured in response to Emergency Conditions.

(5) 	An IGR may only be ordered Off-Line in the case of an ERCOT-declared Emergency Condition with imminent threat to the reliability of the ERCOT System.

6.6.3.2	Real-Time Energy Imbalance Payment or Charge at a Load Zone

(1)	The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time Settlement Point Price for the following amounts at a particular Load Zone Settlement Point:

(a) 	The amount of its Self-Schedules with sink specified at the Settlement Point; plus

(b) 	The amount of its Energy Bids cleared in the DAM at the Settlement Point; plus

(c) 	The amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(d) 	The amount of its Self-Schedules with source specified at the Settlement Point; minus

(e) 	The amount of its Energy Offers cleared in the DAM at the Settlement Point; minus

(f) 	The amount of its Energy Trades at the Settlement Point where the QSE is the seller; minus

(g) 	Its AML at the Settlement Point; plus

(h)	The aggregated generation of its Self-Serve  Generators, electing Load Zone settlement, or Distributed Generation in the Load Zone.

(2)	The payment or charge to each QSE for Energy Imbalance Service at a Load Zone for a given 15-minute Settlement Interval is calculated as follows:

RTEIAMT q, p	=	(-1) * RTSPP p * {(SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼) – RTAML q, p +  RTMGNM q, p}

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for the Real-Time Energy Imbalance at Settlement Point p, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time SPP at Settlement Point p, for the 15-minute Settlement Interval. |
| RTAML q, p | MWh | Real-Time Adjusted Metered Load per QSE per Settlement Point—The sum of the Adjusted Metered Load at the Electrical Buses that are included in Settlement Point p represented by QSE q for the 15-minute Settlement Interval. |
| SSSK q, p | MW | Self-Schedule with Sink at Settlement Point per QSE per Settlement Point—The QSE q’s Self Schedule with sink at Settlement Point p, for the 15-minute Settlement Interval. |
| DAEP q, p | MW | Day-Ahead Energy Purchase per QSE per Settlement Point—The QSE q’s Energy Bids at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQEP q, p | MW | Real-Time QSE-to-QSE Energy Purchase per QSE per Settlement PointThe amount of MW bought by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSR q, p | MW | Self-Schedule with Source at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with source at Settlement Point p, for the 15-minute Settlement Interval. |
| DAES q, p | MW | Day-Ahead Energy Sale per QSE per Settlement Point—The QSE q’s Energy Offers at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQES q, p | MW | Real-Time QSE-to-QSE Energy Sale per QSE per Settlement PointThe amount of MW sold by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| RTMGNM q, p | MWh | Real-Time Metered Generation from Self-Serve or Distributed Generators per QSE per Settlement Point—The total Real-Time energy produced by Self-Serve Generators or Distributed Generation represented by QSE q in Load Zone Settlement Point p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Load Zone Settlement Point. |

(3)	The total net payments and charges to each QSE for Energy Imbalance Service at all Load Zones for the 15-minute Settlement Interval is calculated as follows:

RTEIAMTQSETOT q	=	ΣpRTEIAMT q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTEIAMTQSETOT q | $ | Real-Time Energy Imbalance Amount QSE Total per QSEThe total net payments and charges to QSE q for Real-Time Energy Imbalance at all Load Zone Settlement Points for the 15-minute Settlement Interval. |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The charge to QSE q for the Real-Time Energy Imbalance at Settlement Point p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Load Zone Settlement Point. |

16.5	Registration of a Resource Entity

A Resource Entity owns or controls an All-Inclusive Resource connected to the ERCOT System through Transmission Service Provider (TSP) or Distribution Service Provider (DSP) owned facilities or a Private Use Network.  Each Resource Entity operating in ERCOT must register with ERCOT.

To become registered as a Resource Entity, an Entity must execute a Standard Form Market Participant Agreement (using the form in Section 22, Agreements), designate Resource Entity Authorized Representatives, contacts, and a User Security Administrator (per the Application for Registration as a Resource Entity), and demonstrate to ERCOT’s reasonable satisfaction that it is capable of performing the functions of a Resource Entity under these Protocols.  The Resource Entity shall register each All-Inclusive Resource with ERCOT through ERCOT registration.

16.5.1.1	Designation of a Qualified Scheduling Entity

(1)	Each Resource Entity applicant within the ERCOT Region shall designate in its application the Qualified Scheduling Entity (QSE) that will represent the applicant with ERCOT.  Each applicant shall acknowledge in its application that it bears sole responsibility for selecting and maintaining a QSE as its representative.  The applicant shall include in its application a written statement from the designated QSE acknowledging that the QSE accepts responsibility for the applicant’s transactions pursuant to these Protocols.

(2)	A Resource Entity may be required to designate a backup QSE under this Section.

(3)	The Resource Entity of an Isolated Generation Resource (IGR) may designate a QSE or the TSP that operates the interconnection of the associated Private Use Network and is acknowledged by the QSE or TSP to provide reliability coordination with ERCOT.  The Resource Entity shall establish and maintain operational voice and data communications with the designated QSE or TSP.