| NPRR Number |  | NPRR Title |  |
| --- | --- | --- | --- |
| Date Posted |  |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms 10.2.2, TSP and DSP Metered Entities 11.4.4.2, Load Reduction for Excess PhotoVoltaic Distributed Renewable Generation 11.4.4.3, Load Reduction for Excess Non-PhotoVoltaic Distributed Generation 16.5, Registration of Generation Resources, Loads Acting as a Resource and Emergency Interruptible Load Service Resources 18.2, Methodology 18.2.2, Load Profiles for Non-Interval Metered Loads 18.2.3, Load Profiles for Non-Metered Loads 18.2.8, Adjustments and Changes to Load Profile Development |  |
| Revision Description |  | The Nodal Protocol Revision Request (NPRR) includes proposed language changes by the NOIE DRG Taskforce in regards to the registration requirements for Distributed Generation (DG) greater than 50 kW and less than or equal to 1 MW. In addition this NPRR synchronizes language approved through PRR756, Distributed Renewable Generation Modifications, with some recommended revisions consistent with the NOIE DRG Task force discussions. PRR756 was approved by ERCOT Board on 5/20/08 and was ungreyboxed upon system implementation on 11/21/09. |  |
| Reason for Revision |  | The reason for this revision is to incorporate changes approved in PRR 756, clarified registration requirements for DG to allow all DG to be settled as negative Load and raise the threshold for DG that can be settled as negative Load. PRR756 language established the threshold for Distributed Renewable Generation (DRG) negative Load Settlement at less than fifty (50) kW. This NPRR establishes a DG registration threshold. |  |
| Overall Market Benefit |  |  |  |
| Overall Market Impact |  |  |  |
| Consumer Impact |  |  |  |
| Credit Implications |  |  |  |
| Submitter Justification for Necessity Prior to the Texas Nodal Market Implementation Date |  |  |  |
| CEO Determination (To be completed by ERCOT) |  |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  |  |  |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name |  |
| E-mail Address |  |
| Company |  |
| Phone Number |  |
| Cell Number |  |
| Market Segment |  |

| Market Rules Staff Contact |  |
| --- | --- |
| Name |  |
| E-Mail Address |  |
| Phone Number |  |

Proposed Protocol Language Revision

  - Definitions

Advanced Metering System (AMS)

A system, including Advanced Meters and the associated hardware, software, and communications devices, that collects time-differentiated energy usage and is deployed pursuant to P.U.C. SUBST. R. 25.130.

Distributed Generation (DG)

Distributed Generation is an electrical generating facility located at a Customer’s point of delivery (point of common coupling) ten (10) megawatts (MW) or less and connected at a voltage less than or equal to sixty (60) kilovolts (kV) which may be connected in parallel operation to the utility system.

Distributed Renewable Generation (DRG)

Electric generation with a capacity of not more than 2,000 kW provided by a renewable energy technology that is installed on a retail electric Customer’s side of the meter.

PhotoVoltaic (PV)

Of or pertaining to a material or device in which electricity is generated as a result of exposure to light.

2.2	Acronyms

AMS				Advanced Metering System

DRG				Distributed Renewable Generation

PV				PhotoVoltaic

DG 				Distributed Generation

10.2.2	TSP and DSP Metered Entities

(1)	Each TSP and DSP is responsible for supplying ERCOT with meter data associated with:

(a)	All Loads using the ERCOT System;

(b)	Any All-Inclusive Generation Resource that delivers less than 10 MW to the ERCOT System and that is connected directly to the distribution system; a DSP may make some or all such meters ERCOT-Polled Settlement (EPS) compliant and may request that ERCOT poll the meters. Notwithstanding the foregoing sentence, meter data is not required from:

(i)	Generation owned by a NOIE and used for NOIE’s self-use (not serving Customer Load);

(ii)	Distributed Renewable Generation with a design capacity less than fifty (50) kW interconnected to a TDSP where the owner chooses not to have the out-flow measured in accordance with P.U.C. Subst. R. 25.213, Metering for Distributed Renewable Generation; and

(iii)	Distributed Generation interconnected to a DSP behind a registered NOIE boundary metering point, not registered as a Generation Resource and with an installed capacity below the Distributed Generation registration threshold, as determined in Protocol Section 16.5 and posted on the MIS.

(c)	NOIE points of delivery where metering points are radial Loads and are unidirectionally metered.  A TSP or DSP has the option of making some or all such meters EPS compliant and to request that ERCOT poll the meters.

(2)	Each TSP and DSP is responsible for the following:

(a)	Compliance with the procedures and standards in this Section, the Settlement Metering Operating Guides (SMOG) and the Operating Guides;

(b)	Installation, control, and maintenance of the Settlement Metering Facilities, as more fully described in this Section and SMOG, which includes meters, recorders, instrument transformers, wiring, and miscellaneous equipment required to measure electrical energy;

(c)	Costs incurred in the installation and maintenance of these Metering Facilities and communications except for incremental costs incurred for functions not required for the settlement of the Load or All-Inclusive Resource.  These incremental costs shall be borne by the Entities requesting the service pursuant to the TSP or DSP tariffs;

(d)	Installation, maintenance, data collection, and related communications, telemetry for the Metering Facilities, and related services necessary to meet the mandatory Interval Data Recorder (IDR) requirements detailed in this Section, Section 18, Load Profiling, and the SMOG; and

(e)	Provide ERCOT with any data required by ERCOT for reporting purposes on unregistered Distributed Generation.

(i)	NOIE TDSP reporting requirements:

(A)	Details of this data requirement are located in the COPS Market Guide.

(B)	Reporting is limited to distributed generation resources over 50 kW and capable of flowing excess energy onto the DSP system.

11.4.4.2	Load Reduction for Excess PhotoVoltaic Distributed Renewable Generation

Adjusted Metered Load (AML) for ESI IDs with PhotoVoltaic (PV) generation shall be adjusted as follows:

(1)	For ESI IDs with Non-IDRs installed, AML shall be reduced for excess generation from ESI IDs with PV generation equal to or lower than the Distributed Generation registration threshold behind the meter where there is a meter that measures excess energy flow into the ERCOT System in a separate register.  Only ESI IDs that have been assigned a PV profile segment as specified in Load Profiling Guide Appendix D, Profile Decision Tree, shall be eligible for this reduction.

Intervals beginning 11:00 A.M. and ending 3:00 P.M. CPT (spanning sixteen (16) 15-minute intervals) shall be reduced by the following amount:

PV_adjusti  =   kWh_Gen / (read_days * 16)

Where:

PV_adjusti	Reduction for PV excess generation for interval i

kWh_Gen		Actual (measured) kWh flowing into the Distribution System (out-flow from the Premise)

read_days		Number of days in meter read period

- The PV reduction adjustment for ESI IDs, which have PV generation of equal to or lower than the Distributed Generation registration threshold behind the meter and that have an Advanced Metering System (AMS) integrated meter that measures the excess energy flow into the ERCOT System in 15-minute intervals, shall be determined using the actual 15-minute interval data, if available.

11.4.4.3	Load Reduction for Excess Non-PhotoVoltaic Distributed Generation

AML for ESI IDs with non-PV Distributed Generation (DG) shall be adjusted as follows:

(1)	For ESI IDs with non-IDRs installed, AML shall be reduced for excess generation from ESI IDs with non-PV generation of equal to or lower than the Distributed Generation registration threshold behind the meter where there is a meter that measures excess energy flow into the ERCOT System in a separate register.  Only ESI IDs that have been assigned a DG profile segment as specified in Load Profiling Guide Appendix D, Profile Decision Tree, shall be eligible for this reduction.

All intervals in the meter read period shall be reduced by the following amount:

DG _adjusti  =   kWh_gen / read_ints

Where:

DG_adjusti	Reduction for excess DG for interval i

kWh_gen		Actual (measured) kWh flowing into the Distribution System (out-flow from the Premise)

read_ints		Number of 15-minute intervals in the meter read period

- The energy reduction adjustment for ESI IDs, which have DG equal to or lower than the Distributed Generation registration threshold behind the meter and have an Advanced Metering System (AMS) integrated meter that measures the excess energy flow into the ERCOT System in 15-minute intervals, shall be determined using the actual 15-minute interval data, if available.

16.5	Registration of a Resource Entity

A Resource Entity owns or controls an All-Inclusive Resource connected to the ERCOT System.  Each Resource Entity operating in ERCOT must register with ERCOT.  To become registered as a Resource Entity, an Entity must execute a Standard Form Market Participant Agreement (using the form in Section 22), designate Resource Entity Authorized Representatives, contacts, and a User Security Administrator (per the Application for Registration as a Resource Entity), and demonstrate to ERCOT’s reasonable satisfaction that it is capable of performing the functions of a Resource Entity under these Protocols.  The Resource Entity shall register each All-Inclusive Resource with ERCOT through ERCOT registration, except for Distributed Generation with an installed capacity lower than the Distributed Generation registration threshold.

Beginning on the Texas Nodal Market Implementation Date, Distributed Generation with an installed capacity greater than 1 MW must register with ERCOT.  ERCOT shall reduce the Distributed Generation registration threshold as required by these Protocols and post this requirement on the MIS.   ERCOT shall produce quarterly reports on the total unregistered installed capacity below the current threshold, summed by Load Zone. Such quarterly report shall contain the current Distributed Generation registration threshold. If the total installed capacity of Distributed Generation exceeds 10 MW in any Load Zone, then ERCOT shall issue a market notice that states a lower Distributed Generation registration threshold. Such registration threshold shall be calculated to reduce the total installed capacity of unregistered Distributed Generation to no more than 7 MW in any Load Zone.  This new Distributed Generation registration threshold shall become effective nine months after ERCOT has issued the market notice and posted the new requirement on the MIS.  Notwithstanding the above, at no time shall ERCOT reduce the Distributed Generation registration threshold below 50 kW.

Following this market notice, ERCOT shall inform the appropriate TAC subcommittee of the change in Distributed Generation registration requirements at its next meeting.  The subcommittee shall analyze the quarterly report, and may take any action it deems appropriate, including submitting a Protocol revision to alter this section.

18.2	Methodology

(1)	ERCOT has developed Load Profiles for both non-interval metered Loads and Non-Metered Loads.  A Load Profiling Methodology is the fundamental basis on which Load Profiles are created.  The implementation of a Load Profiling Methodology may require statistical sampling, engineering methods, econometric modeling, or other approaches.

(2)	The following Load Profiling methods are used:

| Type of Load | Load Profiling Methodology |
| --- | --- |
| non-Interval metered | adjusted static models |
| non-interval metered with Distributed Renewable Generation (DRG) | adjusted static models and engineering estimates |
| non-metered | engineering estimates |

(3)	Load Profiles have also been developed for Interval Data Recorders (IDRs) and Advanced Meters for use in Settlements when actual 15-minute data is not available.  All Load Profiles shall conform to the ERCOT-defined Settlement Interval length.

(4)	Adoption of a new methodology requires approval of the Technical Advisory Committee (TAC), without the necessity of complying with the procedures in Section 21, Process for Protocol Revision.  The TAC shall establish the implementation date for approved changes, recognizing the magnitude of the impacts on Market Participants.

18.2.2	Load Profiles for Non-Interval Metered Loads Without Distributed Renewable Generation

- Load Profiles for non-interval metered Loads are created using statistical models developed from appropriate Load research sample data.  These models are referred to as adjusted static. These model equations relate daily Settlement Interval Load patterns to relevant weather descriptors such as maximum and minimum dry-bulb temperature and humidity.  Other daily characteristics such as day-of-the-week and sunrise/sunset times are also employed.

18.2.3	Load Profiles for Non-Metered Loads With Distributed Renewable Generation

Load Profiles for non-metered Loads that utilize DRG (e.g., PhotoVoltaic (PV) or wind) will be created using a hybrid approach.  At least a portion of the Load Profile will be based on adjusted static models, while engineering estimates and/or generation models may be integrated as well or otherwise utilized.

18.2.8	Adjustments and Changes to Load Profile Development

(1)	ERCOT and the appropriate TAC subcommittee will conduct an ongoing evaluation of the current Load Profiling Methodology.  Together they will determine whether appropriate changes to the methodology should be made or whether another approach or combination of approaches is warranted.  Any Market Participant may request a review of the Load Profiling methodology.  Adoption of a new Load Profiling methodology must be approved by TAC, as provided in Section 18.2, Methodology.

(2)	Any Market Participant may petition ERCOT for adjustments to the existing Load Profiles and for development of new Load Profiles.  The Market Participant making the request shall submit their proposal in writing to ERCOT.  ERCOT will post to the Market Information System (MIS) Public Area the request and respond to such requests within 60 days.  ERCOT shall coordinate with the appropriate TAC subcommittee for each change request.  ERCOT shall strive to make the necessary changes within a reasonable period of time.

(3)	ERCOT, in coordination with the appropriate TAC subcommittee, may make changes to existing Load Profiles and establish additional Load Profiles.  All changes to Load Profiles shall adhere to these Protocols.  When additional Load Profiles are established, ERCOT shall evaluate the impact on existing Load Profiles and associated Load research samples.

(4)	A Market Participant may submit a request to ERCOT for conditional approval of a new Load Profile segment following the approval process as specified in the Load Profiling Guide Section 12, Request for Profile Segment Changes, Additions, or Removals.  In conjunction with this request, ERCOT shall specify the requirements for additional Load research sampling and shall define specific and objective criteria to be met by the analysis of this Load research data to meet the requirements for final approval.  Provided the request for conditional approval has received the appropriate ERCOT committee approval and ERCOT determines the specified criteria are met, the request shall be granted final approval.  If ERCOT determines the specified criteria are not met, the request shall be denied.

(5)	Section 9.18, Profile Development Cost Recovery Fee for a Non-ERCOT Sponsored Load Profile Segment, describes the process for compensating the originator of a profile segment change request by Retail Electric Providers (REPs) wishing to subscribe to the profile segment.

(6)	ERCOT shall give at least 150 days notice to all Market Participants prior to market implementation of any change in Load Profile Methodology, existing Load Profiles, or when any additional Load Profiles are developed.  This notice shall include a Load Profile change implementation timeline, which specifies dates on which key events during the Load Profile change process will take place.  Upon any change in Load Profile Types, TSPs and/or DSPs shall send any revised Load Profile ID assignments required by the change to the registration system within the implementation timeline.  After the new Load Profile(s) becomes available, changes to Load Profile Types will be effective on the next meter read date for each ESI ID.

(7)	If one or more Load Profiles require changes to reduce excessive UFE, as determined by the appropriate TAC subcommittee, TAC may provide a shorter notice period and implementation date, than otherwise provided herein, for such required changes to Load Profiles.  If the Load Profile Methodology requires changes to reduce excessive UFE, as determined by the appropriate TAC subcommittee, TAC may provide an expedited notice period and implementation date.  TAC may require the standard Load Profile revision process follow such expedited revisions for long-term resolution.