| NPRR Number | 169 | NPRR Title | Clarify the Calculation and Posting of LMPs for the Load Zone and LMPs for each Hub |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Tabled |
| Date of Decision |  | January 19, 2010 |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 6.3.2, Activities for Real-Time Operations 6.6.1.2, Real-Time Settlement Point Prices for a Load Zone 6.6.1.4, Load Zone LMPs (new) 6.6.1.5, Hub LMPs (new) |  |  |
| Proposed Effective Date |  | Upon System Implementation - Post Texas Nodal Market Implementation Date |  |  |
| Priority and Rank Assigned |  | Priority High |  |  |
| Revision Description |  | The Nodal Protocol Revision Request (NPRR) clarifies language that specifies the calculation and posting of Locational Marginal Prices (LMPs) for the Load Zone and “LMPs for each Hub.” |  |  |
| Overall Market Benefit |  | Calculation and posting of LMPs for the Load Zones and Hubs for each Security-Constrained Economic Dispatch (SCED) interval will be used by all Market Participants including those who need to project when demand response is needed. Used by retail Customers who are consuming power at Real-Time prices and used by generators who may desire to sell energy at Hub prices. |  |  |
| Overall Market Impact |  | Unknown. |  |  |
| Consumer Impact |  | Impacts Retail Electric Providers (REPs) and retail consumers who purchase energy at Real-Time prices. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR169 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 12/16/08, NPRR169 was posted. - On 12/16/08, CPS Energy comments were posted. - On 12/18/08, Public Utility Commission of Texas (PUCT) Staff comments were posted. - On 12/18/08, PRS considered NPRR169. - On 1/8/09, TAC considered NPRR169. - On 1/15/09, Transition Plan Task Force comments were posted. - On 1/22/09, PRS again considered NPRR169. - On 9/10/09, the CEO Revision Request Review was posted. - On 11/11/09, an Impact Analysis was posted. - On 11/19/09, PRS considered the 12/18/08 PRS Recommendation Report and Impact Analysis for NPRR169. - On 12/3/09, TAC again considered NPRR169. - On 1/6/10, TAC again considered NPRR169. - On 1/19/10, the ERCOT Board considered NPRR169. |  |  |
| PRS Decision |  | On 12/18/08, PRS unanimously voted to waive notice to consider NPRR169. Then, PRS unanimously voted to recommend approval of NPRR169 as amended by CPS Energy comments and as revised by PRS. The Independent Generator Market Segment was not present for the votes. On 1/22/09, PRS unanimously voted to table NPRR169. All Market Segments were present for the vote. On 11/19/09, PRS unanimously voted to endorse and forward the 12/18/08 PRS Recommendation Report for NPRR169 to TAC with a priority of High/Medium and rank of 1. |  |  |
| Summary of PRS Discussion |  | On 12/18/08, PRS discussed the reason that the Transition Plan Task Force (TPTF) requested that PRS consider NPRR169 at this PRS meeting although the NPRR was only submitted two days prior to the meeting. It was also noted that PUCT Staff comments express support for NPRR169. PRS also reviewed CPS Energy comments, and additional revisions were proposed and considered. On 1/22/09, there was no discussion. On 11/19/09, the priority and rank for NPRR169 was discussed. |  |  |
| TAC Decision |  | On 1/8/09, TAC unanimously voted to remand NPRR169 to PRS. All Market Segments were present for the vote. On 12/3/09, TAC unanimously voted to change the priority for NPRR169 to High and to table NPRR169 for one month. All Market Segments were present for the vote. On 1/6/10, TAC unanimously voted to recommend approval of NPRR169 as recommended by PRS in the 11/19/09 PRS Recommendation Report and with a priority of High. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 1/8/09, it was noted that the Impact Analysis for NPRR169 has not been completed. It was further noted that NPRR169 is not part of the approved scope for the Nodal program integrated schedule that is under development, and therefore, TPTF will be reconsidering whether this NPRR is needed for Nodal Go-Live. It was recommended that NPRR169 be remanded to PRS so that PRS and TPTF can work out how NPRR169 and other NPRRs that were not part of the approved scope for the integrated schedule should proceed. On 12/3/09, the priority for NPRR169 was discussed. On 1/6/10, it was noted that at the 12/16/09 PRS meeting, the decision was made to no longer recommend a rank for parking deck NPRRs. |  |  |
| Board Decision |  | On 1/19/10, the ERCOT Board tabled NPRR169 for one month. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 | None. |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | Unknown. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Calculation and posting of LMPs for the Load Zones and Hubs for each SCED interval will be used by all Market Participants including those who need to project when demand response is needed. Used by retail Customers who are consuming power at Real-Time prices and used by generators who may desire to sell energy at Hub prices. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| CPS Energy 121608 | Proposes eliminating change to definition of LMP; notes that calculation of zonal LMP is made purely for Market Participant self-use as a tool. |
| PUCT Staff 121808 | Expresses support for NPRR169; emphasizes that every effort should be made to develop markets that are designed to afford loads the opportunity to respond to high prices. |
| TPTF 011509 | Requests that NPRR169 be tabled and that TPTF be provided with the opportunity to resume its consideration of NPRR169 following posting of the formal Impact Analysis in Q3 2009. |

| Sponsor |  |
| --- | --- |
| Name | Joel Mickey on behalf of TPTF |
| Company | ERCOT |
| Market Segment | Not applicable. |

Proposed Protocol Language Revision

2.1	DEFINITIONS

Locational Marginal Price (LMP)

The offer-based marginal cost of serving the next increment of Load at an Electrical Bus, which marginal cost is produced by the DAM process or by the SCED process.  See Hub LMP and Load Zone LMP.

Hub LMP

The price calculated for a Hub for each SCED interval according to the formula in Section 6.6.1.5 using LMPs at the Electrical Buses included in the Hub.

Load Zone LMP

The price calculated for a Load Zone for each SCED interval according to the formula in Section 6.6.1.4 using State Estimator Load data and LMPs at the Electrical Buses included in the Load Zone.

6.3.2	Activities for Real-Time Operations

(1)	Activities for Real-Time operations begin at the end of the Adjustment Period and conclude at the close of the Operating Hour.

(2)	The following table summarizes the timeline for the Operating Period and the activities of QSEs and ERCOT during Real-Time operations where “T” represents any instant within the Operating Hour.  The table is intended to be only a general guide and not controlling language, and any conflict between this table and another section of the Protocols is controlled by the other section:

| Operating Period | QSE Activities | ERCOT Activities |
| --- | --- | --- |
| During the first hour of the Operating Period |  | Execute the Hour-Ahead Sequence, including HRUC, beginning with the second hour of the Operating Period Review and communicate HRUC commitments Snapshot the Scheduled Power Consumption for Controllable Load Resources |
| Before the start of each SCED run | Update Output Schedules for Dynamically Scheduled Resources (DSRs) | Validate Output Schedules for Dynamically Scheduled Resources (DSRs) Execute Real-Time Sequence |
| SCED run |  | Execute SCED |
| During the Operating Hour | Acknowledge receipt of Dispatch Instructions Comply with Dispatch Instruction Review Resource Status to assure current state of the Resources is properly telemetered Update Current Operating Plan with actual Resource Status and limits and Ancillary Service Schedules Communicate Resource Forced Outages to ERCOT | Communicate all Base Points, Dispatch Instructions and LMPs for energy and Ancillary Services using ICCP or Verbal Dispatch Instructions Monitor Resource Status and identify discrepancies between Current Operating Plan and telemetered Resource Status Restart Real-Time Sequence on major change of Resource or Transmission Element Status Monitor ERCOT total system capacity providing Ancillary Services; Validate COP information Monitor ERCOT control performance; Distribute by ICCP, and post to the MIS Public Area, the LMPs created by each SCED process for each Electrical Bus and Resource Node and, Hub LMP and Load Zone LMP. These prices shall be posted immediately subsequent to deployment of Base Points from SCED with the time stamp the prices are effective. Post SCED Shadow Prices via the MIS Public Area Post on the MIS Public Area active binding transmission constraints by Transmission Element name (contingency /overloaded element pairs) via the MIS Public Area Post the Settlement Point Prices for each Settlement Point immediately following the end of each Settlement Interval. Post parameters as required by Section 6.4.8, Ancillary Services Capacity During the Adjustment Period and in Real-Time, to the MIS Secure Area |

(3)	At the beginning of each hour, ERCOT shall post on the MIS Secure Area the following information:

(a)	Changes in ERCOT System conditions that could affect the security and dynamic transmission limits of the ERCOT System, including:

(i)	Changes or expected changes,  in the status of Transmission Facilities for the remaining hours of the current Operating Day and all hours of the next Operating Day, supplementing those that are already in the Outage Scheduler; and

(ii)	Any conditions such as adverse weather conditions as determined from the ERCOT-designated weather service;

(b)	Updated system-wide Load forecasts;

(c)	The quantities of RMR Services deployed by ERCOT for each previous hour of the current Operating Day;

(d)	Total ERCOT System Demand, from Real-Time operations, integrated over each Settlement Interval; and

(e)	Updated Electrical Bus Load distribution factors and other information necessary to forecast Electrical Bus Loads for each hour of the current Operating Day and all hours of the next Operating Day.

6.6.1.2	Real-Time Settlement Point Price for a Load Zone

The Real-Time SPP for a Load Zone Settlement Point is based on the state-estimated Load in MW and the time-weighted average Real-Time LMPs at Electrical Buses that are included in the Load Zone.  The Real-Time SPP for a Load Zone Settlement Point for a 15-minute Settlement Interval is calculated as follows:

RTSPP	=	ΣyΣb (RTLMP b, y * LZWF b, y)

For all Load Zones except Direct Current Tie (DC Tie) Load Zones:

LZWF b, y 	=	(SEL b, y * TLMP y) / [ΣyΣb(SEL b, y * TLMP y)]

For a DC Tie Load Zone:

LZWF y 	=	[Max (0.001, SEL y)* TLMP y)] / [Σy[Max (0.001, SEL y)* TLMPy)]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point PriceThe Real-Time SPP at the Settlement Point, for the 15-minute Settlement Interval. |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per intervalThe Real-Time LMP at Electrical Bus b in the Load Zone, for the SCED interval y. |
| LZWF b, y | none | Load Zone Weighting Factor per bus per intervalThe weight used in the Load Zone SPP calculation for Electrical Bus b, for the portion of the SCED interval y within the 15-minute Settlement Interval. |
| SEL b, y | MW | State Estimator Load at bus per intervalThe Load from State Estimator for Electrical Bus b in the Load Zone, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the portion of the SCED interval y within the Settlement Interval. |
| Y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| B | none | An Electrical Bus in the Load Zone. The summation is over all of the Electrical Buses in the Load Zone. |

6.6.1.3	Real-Time Settlement Point Price for a Hub

The Real-Time SPP at a Hub is determined according to the methodology included in the definition of that Hub in Section 3.5.2, Hub Definitions.

6.6.1.4	Load Zone LMPs

The Load Zone LMPs shall be made available through ICCP and posted on the MIS Public Area.  The Load Zone LMP is based on the state-estimated Loads in MW and the Real-Time LMPs at the Electrical Buses included in the Load Zone.  The Load Zone LMP for a Load Zone for a SCED Interval is calculated as follows:

LZLMP y	=	Σb (RTLMP b, y * LZWF b, y)

For all Load Zones except Direct Current Tie (DC Tie) Load Zones:

LZWF b, y 	=	SEL b, y / (ΣbSEL b, y)

For a DC Tie Load Zone:

LZWF b, y 	=	[Max (0.001, SEL b, y)]  /  [Max (0.001, SEL b, y)]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| LZLMP y | $/MWh | Load Zone Locational Marginal PriceThe Load Zone LMP for the Load Zone for the SCED Interval y. |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per SCED intervalThe Real-Time LMP at Electrical Bus b in the Load Zone, for the SCED interval y. |
| LZWF b, y | None | Load Zone State Estimator Load Weighting Factor per bus per SCED intervalThe weight used in the Load Zone LMP calculation for Electrical Bus b for the SCED interval y. |
| SEL b, y | MW | State Estimator Load at bus per SCED intervalThe Load from the State Estimator for Electrical Bus b in the Load Zone, for the SCED interval y. |
| Y | None | A SCED interval. |
| B | None | An Electrical Bus in the Load Zone. The summation is over all of the Electrical Buses in the Load Zone. |

6.6.1.5	Hub LMPs

The Hub LMPs shall be made available through ICCP and posted on the MIS Public Area.

For each defined Hub except for the ERCOT Hub Average 345 kV Hub, the Hub LMP is the arithmetic average of the Real-Time LMPs of the Hub Buses included in the Hub.  The Hub LMP for a SCED Interval is calculated as follows:

HUBLMP Hub, y	=	Σhb(HUBDF hb, Hub * RTHBP hb, Hub, y, if HB Hub ≠ 0

HUBLMP Hub, y	=	HUBLMPERCOT345Bus, if HB Hub = 0

Where:

RTHBP hb, Hub, y	=	Σb(HBDF b, hb, Hub * RTLMP b, hb, Hub, y)

HUBDF hb, Hub	=	 1 / HB Hub, if HB Hub ≠ 0

HUBDF hb, Hub	=	 0, if HB Hub = 0

HBDF b, hb, Hub	=	1 / B hb, Hub, if B hb, Hub ≠ 0

HBDF b, hb, Hub	=	0, if B hb, Hub = 0

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| HUBLMP Hub, y | $/MWh | Hub Locational Marginal PriceThe Hub LMP for the Hub for the SCED Interval y. |
| RTHBP hb, Hub, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per SCED intervalThe Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RTLMP b, hb, Hub, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per intervalThe Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb, for the SCED interval y. |
| HUBDF hb, Hub | none | Hub Distribution Factor per Hub BusThe distribution factor of Hub Bus hb. |
| HBDF b, hb, Hub | none | Hub Bus Distribution Factor per Electrical Bus of Hub BusThe distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| B hb, Hub | none | The total number of energized Electrical Buses in Hub Bus hb. |
| HB Hub | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus. |
| Hub | none | One of the following Hubs: ERCOT Bus Average 345 kV Hub, North 345 kV Hub, South 345 kV Hub, Houston 345 kV Hub, or the West 345 kV Hub |
| Hb | none | A Hub Bus that is a component of the Hub. |
| Y | none | A SCED interval. |
| B | none | An energized Electrical Bus that is a component of a Hub Bus. |

The Hub LMP for the ERCOT Hub Average 345 kV Hub (ERCOT 345) for a SCED Interval is calculated as follows:

HUBLMP ERCOT345, y	=	(HUBLMPNORTH345, y + HUBLMPSOUTH345, y + HUBLMPHOUSTON345, y  + HUBLMPWEST345, y) / 4

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| HUBLMPERCOT345, y | $/MWh | Hub Locational Marginal Price for the ERCOT345The Hub LMP for the ERCOT Hub Average 345 kV Hub (ERCOT 345), for the SCED Interval y. |
| HUBLMPNORTH345, y | $/MWh | Hub Locational Marginal Price for the NORTH345The Hub LMP for the North 345 kV Hub (NORTH 345), for the SCED Interval y. |
| HUBLMPSOUTH345, y | $/MWh | Hub Locational Marginal Price for the SOUTH345The Hub LMP for the South 345 kV Hub (SOUTH 345), for the SCED Interval y. |
| HUBLMPHOUSTON345, y | $/MWh | Hub Locational Marginal Price for the HOUSTON345The Hub LMP for the Houston 345 kV Hub (HOUSTON 345), for the SCED Interval y. |
| HUBLMPWEST345, y | $/MWh | Hub Locational Marginal Price for the WEST345The Hub LMP for the West 345 kV Hub (WEST 345), for the SCED Interval y. |