| PRR Number | 841 | PRR Title | Revise Total ERCOT Wind Power Forecast (TEWPF) |
| --- | --- | --- | --- |
| Date |  | January 20, 2010 |  |
| Submitter’s Information |  |  |  |
| Name |  | Barbara Clemenhagen on behalf of the Wholesale Market Subcommittee (WMS) |  |
| E-mail Address |  | bclemenhagen@topazpowergroup.com |  |
| Company |  | Topaz Power Group |  |
| Phone Number |  | (512) 314-8621 |  |
| Market Segment |  | Independent Generator |  |

Comments

At the January 20, 2010 WMS meeting, WMS considered Protocol Revision Request (PRR) 841.  It was noted that the Qualified Scheduling Entity (QSE) Managers Working Group (QMWG) reviewed PRR841 at its January 15, 2010 meeting and was in consensus to recommend approval of PRR841 as amended by the 12/22/09 ERCOT comments.  After discussion, WMS unanimously voted to endorse PRR841 as amended by the 12/22/09 ERCOT comments (reflected below).

Revised Proposed Protocol Language

2.1	Definitions

Short –Term Wind Power Forecast (STWPF)

An ERCOT produced, hourly, fifty percent (50%) probability of exceedance forecast of the generation in MWh per hour from each WGR that could be generated from all available units of that Resource.

2.2	Acronyms

STWPF			Short-Term Wind Power Forecast

4.4.15 	QSE Resource Plans

Each QSE that represents a Resource will present a Resource Plan to ERCOT at 1600.  These Resources may be specific Generation Resources and/or Loads acting as Resources (LaaRs).  The Resource Plan capacity should be sufficient to accommodate the combined quantity of energy and Ancillary Services scheduled by that QSE from the Resources that the QSE represents.  The Resource Plan shall indicate the availability of the Resources represented by the QSE, including a lead-time status code, and the planned operating level of each Resource, for each hour of the Operating Day.  The Resource Plan shall indicate the High Operating Limit (HOL) and Low Operating Limit (LOL), and High Sustainable Limit (HSL) and Low Sustainable Limit (LSL) by Resource.  A Resource may be listed as unavailable to ERCOT if the Resource’s capacity has been committed to markets in regions outside of ERCOT.  ERCOT shall use other Resource Dispatch options to maintain system reliability prior to Dispatching a Generation Resource below its LOL.  ERCOT shall request Qualifying Facilities (QFs), hydro units, and/or nuclear to operate below their LOLs only after other Resource Dispatch options have been exhausted.

ERCOT shall produce renewable production potential forecasts for Wind-powered Generation Resources (WGRs) to be used as the planned operating level in the Resource Plan during Replacement Reserve Service (RPRS) procurements.  The Short-Term Wind Power Forecast (STWPF) is an hourly fifty-percent (50%) probability of exceedance forecast of energy production for each WGR.  ERCOT shall use a probabilistic Total ERCOT Wind Power Forecast (TEWPF) and select the forecast that the actual total ERCOT WGR production is expected to exceed fifty-percent (50%) of the time (i.e., a fifty-percent (50%) probability of exceedance forecast).  To produce the STWPF, ERCOT will allocate the TEWPF fifty-percent (50%) probability of exceedance forecast to each WGR such that the sum of the individual STWPF forecasts equal the TEWPF forecast.  ERCOT shall produce these forecasts using information provided by WGRs to their QSEs including meteorological information or models, WGR power production curves and Supervisory Control and Data Acquisition (SCADA).  ERCOT shall provide forecasts for each WGR to the QSEs representing WGRs and shall deliver the forecasts before RPRS procurements to allow the QSEs to update the WGR Resource Plans.  QSEs shall use the ERCOT-provided STWPFs for WGRs as the planned operating level for the 1600 Resource Plan and prior to running an RPRS market in the Adjustment Period.  The QSE may submit a lower operating level than the STWPF in the WGR Resource Plan if the WGR has communicated that it will be unavailable or operating at a reduced capability during an Operating Period which the forecast did not anticipate.  QSEs representing only WGRs shall update their Resource Plans and schedules to reflect the expected wind-powered generation production after the close of the RPRS market.  The energy schedules submitted by QSEs representing only WGRs should correspond with the Resource Plan scheduled energy output in order for Real Time balancing and the operator entered offset to perform properly.  During Settlement Intervals in which QSEs representing only WGRs are using a Resource Plan modified due to insertion of the fifty-percent (50%) probability of exceedance forecast, ERCOT shall use the most recent available Resource Plan value prior to the ERCOT instruction to insert the fifty-percent (50%) probability of exceedance forecast.

QSEs shall use best efforts, consistent with Good Utility Practice, to continually update their Resource Plans to reflect the current and anticipated operating conditions of the Resources.  ERCOT will monitor the performance of QSEs with respect to the submission of accurate Resource Plans in accordance with the measures established in Section 4.10, Resource Plan Performance Metrics.  ERCOT will work with individual QSEs as necessary to improve the individual QSE performance.

4.5.12	Scheduling Requirements for a WGR-only QSE

A WGR-only QSE shall update each WGR’s Resource Plans and schedules each hour using their best forecast or the Short-Term Wind Power Forecast (STWPF) provided by ERCOT except as required by Section 4.4.15, QSE Resource Plans.  The updated Resource Plan shall not change or update the very first hour of the Operating Period immediately following the time of the update.  ERCOT may compare the actual average hourly WGR generation with the forecasted output for the Operating Hour of the Operating Period immediately following the time when the Resource Plan was updated.  For this calculation, ERCOT shall use the last STWPF forecast that was available before the close of the Adjustment Period for the calculation of each hourly forecast error.  A forecast error percentage shall be calculated using the WGR’s maximum rated output as the base for each hour that the WGR does not have a unit specific curtailment.  If the WGR-only QSE has received a Balancing Energy Service Down instruction, then all WGRs in the portfolio will be excluded from the error percentage calculation for that hour.  If, for two (2) consecutive months, the WGR’s monthly root mean square error of the forecasts for the hour specified above is greater than the monthly root mean square error for the STWPF for that same hour, the QSE will be required to use the STWPF adjusted for turbine outages when updating the Resource Plan, unless at some later date ERCOT approves the use of an alternative ERCOT-produced forecast.  A WGR-only QSE may resume using its own forecast to update Resource Plans and schedules if for two (2) consecutive months the WGR’s monthly root mean square error of the forecasts for the hour is less than the monthly root mean square error for the STWPF for that same hour.