| NPRR Number | 204 | NPRR Title | Update Generic Startup Cost for Reciprocating Engines |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | January 21, 2010 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Nodal Protocol Section Requiring Revision |  | 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) adds an Operations and Maintenance (O&M) cost for the “Reciprocating Engines” Resource category to the Resource Category Startup Offer Generic Cap determination. |  |  |
| Reason for Revision |  | The current Nodal Protocols utilize a $1 O&M cost for Reciprocating Engines. This is not representative of the O&M for this technology type. |  |  |
| Overall Market Benefit |  | Allows Reciprocating Engine Resources to submit a more realistic Startup Offer to recover normal O&M costs. This incents a valuable peaking technology to participate in the Nodal Market. |  |  |
| Overall Market Impact |  | More accurately reimburses startup costs and minimum energy costs for Reciprocating Engine Resources. |  |  |
| Consumer Impact |  | This incents a valuable peaking technology to participate in the Nodal Market. |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - On 12/22/09, NPRR204 and the associated CEO Revision Request Review were posted. - On 1/21/10, PRS considered NPRR204. |  |  |
| PRS Decision |  | On 1/21/10, PRS voted to recommend approval of NPRR204 as submitted. There were two opposing votes from the Independent Generator and Consumer Market Segments and one abstention from the Independent Power Marketer (IPM) Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 1/21/10, whether NPRR204 should be referred to the Verifiable Cost Working Group (VCWG) for review was discussed. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | More accurately reimburses startup costs and minimum energy costs for Reciprocating Engine Resources. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Allows Reciprocating Engine Resources to submit a more realistic Startup Offer to recover normal O&M costs. This incents a valuable peaking technology to participate in the Nodal Market. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Clif Lange |
| E-mail Address | clif@stec.org |
| Company | South Texas Electric Cooperative, Inc. |
| Phone Number | 361-485-6206 |
| Cell Number |  |
| Market Segment | Cooperative |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None. |  |

Proposed Protocol Language Revision

4.4.9.2.3	Startup Offer and Minimum-Energy Offer Generic Caps

(1)	The Resource Category Startup Offer Generic Cap, by applicable Resource category, is determined by the following Operations and Maintenance (O&M) costs by Resource category:

| Resource Category | O&M Costs ($) |
| --- | --- |
| Nuclear, Coal, Lignite and Hydro | 7,200 |
| Combined-Cycle greater than 90 MW with 5+ HRS off line | 6,810 |
| Combined-Cycle greater than 90 MW with less than 5 HRS off line | 5,310 |
| Combined-Cycle less than or equal to 90 MW with 5+ HRS off line | 6,810 |
| Combined-Cycle less than or equal to 90 MW with less than 5 HRS OffLine | 5,310 |
| Gas steam supercritical boiler | 4,800 |
| Gas steam reheat boiler | 3,000 |
| Gas steam non-reheat or boiler w/o air-preheater | 2,310 |
| Simple cycle greater than 90 MW | 5,000 |
| Simple cycle less than or equal to 90 MW | 2,300 |
| Reciprocating Engines | 487 |
| RMR Resource | Not Applicable |
| Wind Generation Resources | 0 |
| Any Resources not defined above | 0 |

(2)	The Resource Category Minimum-Energy Generic Cap is the cost per MWh of energy for a Resource to produce energy at the Resource’s LSL and is as follows:

(a)	Hydro = $10.00/MWh;

- (b)	Coal and lignite = $18.00/MWh;

- (c)	Combined-cycle greater than 90 MW = 10 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (d)	Combined-cycle less than or equal to 90 MW = 10 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (e)	Gas steam supercritical boiler = 16.5 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (f)	Gas steam reheat boiler = 17.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (g)	Gas steam non-reheat or boiler without air-preheater = 19.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (h)	Simple-cycle greater than 90 MW = 15.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (i)	Simple-cycle less than or equal to 90 MW = 15.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in Minimum-Energy Offer;

- (j)	Reciprocating engines = 16.0 MMBtu/MWh * ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Minimum-Energy Offer;

- (k)	RMR Resource = RMR contract estimated fuel cost using its contract I/O curve at its LSL times FIP;

- (l)	Nuclear = Not Applicable;

- (m)	Wind Generation Resources = $0; and

- (n)	Other Resources not defined above = $0.

- (3)	The FIP and FOP used to calculate the Resource Category Minimum-Energy Generic Cap shall be the FIP or FOP for the Operating Day.  In the event the Resource Category Minimum-Energy Generic Cap must be calculated before the FIP or FOP is available for the particular Operating Day, the FIP and FOP for the most recent preceding Operating Day shall be used.  Once the FIP and FOP are available for a particular Operating Day, those values shall be used in the calculations.  If the percentage fuel mix is not specified for Resource categories having the option to specify the fuel mix, then the minimum of FIP or FOP shall be used.

(4)	Items (2)(c) and (2)(d) above are determined by capacity of largest simple-cycle combustion turbine in the train.