| NPRR Number | 210 | NPRR Title | Wind Forecasting Change to P50, Synchronization with PRR841 |
| --- | --- | --- | --- |
| Date Posted |  | February 9, 2010 |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 4.2.2, Wind-Powered Generation Resource Production Potential |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) changes the wind forecasting methodology to use a 50% probability of exceedance calculation instead of an 80% for Reliability Unit Commitment (RUC) considerations. This NPRR would align the Nodal Protocols with changes made to the zonal Protocols pursuant to Protocol Revision Request (PRR) 841, Revise Total ERCOT Wind Power Forecast (TEWPF), which is proceeding through the stakeholder process. |  |
| Reason for Revision |  | The P80 forecast provides an unrealistically low prediction of wind output resulting in overcommitment of RUC Resources. Implementation of this NPRR should allow ERCOT to commit fewer RUC Resources and still maintain reliability. |  |
| Overall Market Benefit |  | Reduction in RUC commitments and related costs. |  |
| Overall Market Impact |  | None. |  |
| Consumer Impact |  | Lower RUC costs. |  |
| Credit Implications |  | No. |  |
| Submitter Justification for Necessity Prior to the Texas Nodal Market Implementation Date |  | If change is not made ERCOT may significantly overcommit RUC Resources that require later decommitment, increasing cost unnecessarily to the market. |  |
| CEO Determination (To be completed by ERCOT) |  | The ERCOT CEO has determined that NPRR210 is necessary prior to the Texas Nodal Market Implementation Date assuming that PRR841 is approved. |  |

| Quantitative Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 | Better forecast of wind generation will lower ancillary service cost uplift to the market. |  |
|  | 2 | Provides for more efficient procurement of Ancillary Services by use of market solutions rather than command and control actions by ERCOT. |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | No additional cost for market implementation. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Reduced RUC costs. | $10000/unit/day that has been overcommitted. |
|  | 2 |  | Depending on unit’s actual verifiable costs. |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Clayton Greer |
| E-mail Address | greercntgreer@aol.com |
| Company | Morgan Stanley |
| Phone Number | 512-497-2986 |
| Cell Number |  |
| Market Segment | Independent Power Marketer (IPM) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

Proposed Protocol Language Revision

2.1	DEFINITIONS

Short–Term Wind Power Forecast (STWPF)

An ERCOT produced hourly 50% probability of exceedance forecast of the generation in MWh per hour from each WGR that could be generated from all available units of that Resource.

4.2.2	Wind-Powered Generation Resource Production Potential

(1)	ERCOT shall produce and update hourly a Short-Term Wind Power Forecast (STWPF) that provides a rolling 48-hour hourly forecast of wind production potential for each Wind-powered Generation Resource (WGR).  ERCOT shall produce and update an hourly Total ERCOT Wind Power Forecast (TEWPF) providing a probability distribution of the hourly production potential from all wind-power in ERCOT for each of the next 48 hours.  Each Generation Entity that owns a WGR shall install and telemeter to ERCOT the site-specific meteorological information that ERCOT determines is necessary to produce the STWPF and TEWPF forecasts.  ERCOT shall establish procedures specifying the accuracy requirements of WGR meteorological information telemetry.

(2)	The STWPF is an hourly 50% probability of exceedance forecast of energy production for each WGR.  ERCOT shall use the probabilistic TEWPF and select the forecast that the actual total ERCOT WGR production is expected to exceed 50% of the time (50% probability of exceedance forecast).  To produce the STWPF ERCOT will allocate the TEWPF 50% probability of exceedance forecast to each WGR such that the sum of the individual STWPF forecasts equal the TEWPF forecast.  The updated STWPF forecasts for each hour for each WGR are to be used as input into each RUC process as per Section 5, Transmission Security Analysis and Reliability Unit Commitment (RUC).

(3)	ERCOT shall produce the WGRPP forecasts using the information provided by WGR owners including WGR availability, meteorological information, and Supervisory Control and Data Acquisition (SCADA).

(4)	Each hour, ERCOT shall provide, through the Messaging System, the STWPF forecasts for each WGR to the QSE that represents that WGR and shall post each STWPF forecast on the MIS Certified Area.

(5)	Each hour, ERCOT shall post the STWPF 50% probability of exceedance forecast on the MIS Secure Area.  ERCOT shall retain the STWPF for each hour.

(6)	ERCOT shall post to the MIS on a regional basis a rolling 48 hour actual wind power production and the forecasted amounts from the STWPF and the TEWPF.