ERCOT Load Profiling Guide

Appendix C

January 1, 2008

Appendix C	1

# Appendix C

Measuring Differences Between Load Profiles

- This Appendix describes and illustrates measures that may be used for assessing the differences between Load Profiles.

- Differences between Load Profiles are a consideration in many decisions regarding Load Profiling Methods and models, such as:

- Evaluating Load Profile Model performance or methodologies (Section 8, Load Profile Models, and Section 7, Requests for Changes to Load Profiling Methodology);

- Evaluating requests for changes to segments or Weather Zones (Section 12, Request for Profile Segment Changes, Additions, or Removals, and Section 13, Changes to Weather Zone Definitions); and

- Designing Load research samples (Section 15, Load Research Samples).

Target and Default Load Profiles

  - In most cases when Load Profile differences are measured, the question of concern is whether an existing or proposed method or model is adequate in a particular context, or an alternative is needed.  The alternative might be a finer segmentation or Weather Zone, use of more recent or more local data in estimating models, or an alternative Load Profiling Methodology.  In all these cases, the analysis compares a “target” Load Profile against a “default” Load Profile.

  - The default Load Profile is the one generated by the existing method or model, or the one that is used if the alternative is not accepted.  The Target Profile is the best available estimated Load shape for a particular proposed subgroup.  The default Load Profile is good enough to represent the target if the two are not significantly different.  Some examples of default and Target Profiles in addressing particular questions are given in the following Table C-1, Examples of Target and Default Profiles.

Table C-1
Examples of Target and Default Profiles

Load Shape Parameters

Load shapes may be compared in terms of several different parameters that characterize the Load shape.  Some of these parameters are a series of numbers that jointly characterize the shape.  Others are single parameters that represent a key characteristic.

General Notation

The following general notation is used in this Appendix, and elsewhere in the Load Profiling Guide (LPG).  General quantities are defined.  Suffixes and subscripts are used to signify specific quantities.  Explicit definitions are given in the formulas that follow.

General Quantities

e 	= 	elasticity of electricity Demand with respect to the commodity price

- E 	= 	energy

f 	= 	fraction

DWL 	= 	deadweight loss

L 	= 	Load

LF	= 	Load factor

N 	= 	number of intervals in a period for which the quantity is calculated

r 	=	ratio

ut 	=	market commodity price at interval t

T 	= 	Total across some calendar dimension (clock-hour, day-type, month, season, year, on- or off-peak period)

U	= 	Load-weighted average price

Suffixes

- D	= 	daily

H 	= 	clock-hourly

M 	= 	monthly

S 	= 	seasonal

Y 	= 	yearly

Subscripts

d 	=	day

m 	= 	month

h 	=	clock-hour

p	= 	day-type

ON 	= 	on-peak

OFF 	= 	off-peak

s 	= 	segment

t	= 	interval

z	=	Weather Zone

Series Parameters

- Series parameters include:

- Unitized Load shape, or interval fractions;

- Monthly fractions;

- Daily fractions; and

- Clock-hour fractions by day-type.

Unitized Load Shapes

  - A Load Profile defines the fraction of total energy use over a period that occurs in each time interval within the period.  For most comparisons of Load shapes for purposes of Load Profiling, the Load shapes shall first be unitized.  That is, the interval total or average Loads for the group are translated into interval fractions.

  - For most comparisons, the time period of interest is a year and the fractions are calculated as fractions of total annual energy consumption.  In some cases, shorter time periods such as a Season or month may be used.

  - The unitized Load ft for time interval t is calculated from the interval Loads Lt as

ft = (Load at interval t)/(sum of Loads over all intervals in the period).

= Lt/ T

where

t Lt

and the summation is over all intervals in the period.

  - Each of the other Load shape characteristics described below may be calculated using Loads Lt as indicated in the formulas, or using the unitized Loads ft in place of Lt. The same result shall be obtained either way.

Profile Totals

If interval data are finer than hourly, the hourly Loads may be averaged for each hour of the period to get the hourly Demand, equal to the total energy in each hour.  For quarter-hour data, each hour’s four quarters are averaged to give the hourly value.

Hourly totals

TH are the sum over days in the period for each of the 24 hours in a day.  Hourly totals may be calculated across a year, a month, or a day-type.

- Daily totals

- TD are the sum over hours in each day of the hourly values.

Day-type totals

TP are the sum over all hours in the day-type of the hourly values.

- Monthly totals

- TM are the sum over days in the month of the daily totals.

Seasonal totals

TS are the sum over all the months in a season of the monthly totals.

Yearly totals

TY are the sum over all the months in the year of the monthly totals.

On-peak totals

TON are the sum over all on-peak hours in the period of the hourly values.

Off-peak totals

TOFF are the sum over all off-peak hours in the period of the hourly values.

Profile Fractions

A year, Season, or month of Load data may be condensed into the fraction of total consumption occurring in each month, day, clock-hour, or day-type.

Monthly Fractions

- For each month m = 1 to 12, the monthly fraction fMm is calculated as

fMm	=	(sum of interval Loads in month m)/ (sum of all interval Loads in the year)

=	(tMm Lt)/ (t Lt ).

=	TMm/ TY.

where

t		= 	interval of time

Mm	= 	indicates month m

Lt	= 	Load at interval t

TMm and TY, respectively, are Load totals over month m and over the year.

Daily Fractions

- Daily fractions may be calculated similarly to monthly fractions.  Daily fractions may be calculated as fractions of the year or separately for each month.  The daily fraction for day d is calculated as

fDd	=	(sum of interval Loads in day d)/ (sum of all interval Loads in the period)

=	(tDd Lt)/ (t Lt ).

=	TDd/T

where

t	= interval of time

Dd	= day d

Lt	= Load at interval t

TDd and T, respectively are Load total sums over day d and over the entire period (e.g., year or month) for which the daily fractions are calculated.

- This calculation gives one daily fraction for every day of the month or year.

Clock-Hour Fractions by Day-type

- When hourly data are used, the unitized Load gives one hourly fraction for each hour of the month or year.  This information may be condensed to give the average clock-hour fraction for a period.  Each of the 24 hourly totals THph for the period is divided by the total for the period, TM, TP or TY.  For clock hour h, the clock-hour fraction for day-type p is calculated as

fHph = THph/TPp

where THph is the interval Load totals for day-type p and clock-hour h, and TPp is the total over all hours.

- Clock-hour fractions for a month or year are calculated analogously.

Single Parameters

- Single parameters include:

- Load factor;

- On-/off-peak ratio;

- Seasonal consumption ratio;

- Weekday fraction; and

- Load-weighted average price.

Load Factor

The Load factor for a period is the ratio of the average to the peak Load for the period:

LF = (average Load)/(maximum Load)

= (T/N) /max(Lt)

where

- Lt = Load at interval t

T = sum of interval Loads over all intervals in the period

N = the number of intervals in the period.

Average Load Factor

- Average Load factor is an average of monthly Load factors.  This average is computed as

AvgLF=m=112AHUsemm=112MaxKWm

where

- AHUsem = average hourly use in billing month m

= KWhm/(billing days x 24)

MaxKWm = maximum metered kW Demand in billing month m

- This definition and notation are consistent with the profile assignment decision tree.

- In terms of the notation used elsewhere in this appendix, if the interval t is hourly,

- AHUsem = Tm/Nm

and

MaxkWm = max(Lt)m.

AvgLF=m=112TmNmm=112maxt∈MmLtThus,

On-peak/Off-peak Ratio

The on-peak/off-peak ratio is the ratio of total consumption during on-peak periods to the consumption during off-peak periods.  The specific definition depends on the definitions of on-peak and off-peak periods.  The ratio is calculated as

ron/off	=	(on-peak consumption)/(off-peak consumption)

=	TON/TOFF

where

TON = sum of interval Loads over all intervals in on-peak periods

TOFF = sum of interval Loads over all intervals in off-peak periods.

Seasonal Consumption Ratios

Seasonal consumption ratios are the ratio of total consumption in one season to consumption in another season.  Most common is the ratio of summer to winter consumption.  The ratio of summer to annual or winter to annual consumption may also be used.  When using the seasonal consumption ratio calculation, the months of the seasons being used shall be defined.  The ratios are calculated as

rS1/S2	=	(total for season 1)/(total for season 2)

=	TS1/TS2

where

TS1 and TS2 denote totals of interval Loads over the two seasons being compared.

Load-Weighted Average or Annual Price

- The Load-weighted average price U for a period is calculated as

- U = Σt Lt ut /Σt Lt

where

Lt = Load at interval t

ut = market price for commodity at time interval t

the summation is over all intervals t in the period.

- Equivalently, the Load-weighted average price may be calculated from the unitized Loads as

- U = t ft ut.

where

ft is the unitized Load defined above.

- Load-weighted average price is most often considered on an annual basis.  When the period is annual the Load-weighted average price is also called the Load-weighted annual price.

- Unlike the other parameters defined, Load-weighted average price is not only a characteristic of a Load shape, but depends also on the price series ut.  When Load-weighted average price is used, the price series shall be specified.

Hourly and Quarter-Hourly Load Data

- Although ERCOT uses quarter-hourly data for settlement, hourly Load data may be used for supporting analysis to assess Load Profiling Methods.  When hourly data are used, the intervals t are hourly.  In this case, the Load Lt in Demand for each interval t is equal to the energy for the interval, and the sum of the Loads T is the total profile energy for the period.

- For calculations expressed as the ratio of Loads, the ratio may be calculated in the same way whether the Load data are hourly or quarter-hourly.  The same result will be obtained either way.  This rule applies to unitized Loads, Load factor, on-/off-peak ratio, and seasonal consumption ratios.

Measuring Differences Between Two Load Profiles

Direct Comparison of Simple Parameters

- Simple parameters are those that represent a Load shape in terms of a single number.  These parameters include the following:

- Load factor

- On-/off-peak ratios

- Seasonal consumption ratios

- Load-weighted average price.

  - Each simple parameter may be the basis for measuring differences between Load Profiles.  The value of the parameter for the Target Profile is subtracted from the value for the default profile.  For example, the difference in Load factors is expressed as:

LF = LFDefault – LFTarget

  - This difference may be thought of as the magnitude of the error if the default profile is used to represent the target.

Comparison of Two Series

- A Load Profile over a year may be characterized by:

- Its unitized Load shape;

- The separately unitized Load shapes for each month;

- The 12 monthly fractions;

- The 24 clock-hour fractions for the year; or

- The set of 24 clock-hour fractions for each month or day-type.

  - For any of these series, two Load Profiles may be compared in terms of various summary measures of the difference between their two series.  Each of these summary measures is called a measure of “error” or “deviation.”  In the context of comparing two Load Profiles, this error is a measure of how far the default profile is from the target.

Mean Deviation

The simplest measure of difference is the average difference in values or deviation between corresponding elements of the series.  The mean deviation is calculated by:

- Taking the difference between the default and target for each element in the series;

- Taking the mean of these differences over all terms in the series.

Mean Absolute Deviation (MAD)

The mean absolute deviation or MAD is calculated by:

- Taking the difference between the default and target for each element in the series;

- Taking the absolute value of each difference;

- Taking the mean of these absolute differences over all terms in the series.

Thus,

- MAD = (1/J) j |XTARGETj - XDEFAULTj|

where

J is the number of elements in the series (e.g., 12 for monthly fractions, 365 for daily fractions of the year, 24 for clock-hour fractions of a period).

Mean Absolute Percent Error (MAPE)

The mean absolute percent error or MAPE is calculated by:

- Taking the difference between the default and target for each element in the series

- Taking the absolute value of each difference expressing this absolute value as a percent of the target value

- Taking the mean of these absolute percent errors over all terms in the series.

Thus,

- MAPE = (1/J) j |XTARGETj - XDEFAULTj|/XTARGETj

where

J is the number of elements in the series (e.g., 12 for monthly fractions, 365 for daily fractions of the year, 24 for clock-hour fractions of a period).

Root Mean Square Error (RMSE)

The root mean square error or RMSE is calculated by:

- Taking the difference between the default and target for each element in the series

- Squaring each difference

- Taking the mean of the squared difference over all terms in the series

- Taking the square root of the mean squared difference.

Thus,

where

J is the number of elements in the series (e.g., 12 for monthly fractions, 365 for daily fractions of the year, 24 for clock-hour fractions of a period).

Measuring Differences for a Group of Load Profiles

Some decisions require comparisons across a group of profiles that are jointly affected by a possible change.  Key examples of such decisions are to either:

- Subdivide an existing segment into smaller segments,

- Subdivide an existing Weather Zone into smaller Weather Zones,

- Change segment definitions in a way that shall affect multiple segments, or

- Change the boundaries of Weather Zones in ways that shall affect multiple Weather Zones.

Deadweight Loss Reduction

One measure that reflects the combined effects of a change on several profiles is deadweight loss.  A reduction in deadweight loss is the gain in economic efficiency due to providing Customers with Load Profiles that are closer to their true Load shapes.  This reduction is a value to society, measured in dollars per year.  Given that ERCOT’s costs are ultimately paid by consumers through their electricity rates, this societal value is theoretically the maximum that it would be worth to implement a change paid for by ERCOT.  Changes that would cost more than this to implement shall cost more than the economic value of the benefit.

Deadweight Loss Reduction from Finer Subdivision

- When performing an analysis where a single group is divided into smaller groups, the Load Profiles for the smaller segments or zones are considered the Target Load Profiles. The profile for the single group is considered the default Load Profile.  The difference between the default and each of the Target Profiles may be measured by any of the difference measures described in this Appendix titled Measuring Differences between Two Load Profiles, as described in Section 12.6.2, Difference from Current Profiles.

- The combined effect of subdividing may be expressed as the deadweight loss reduction.  The calculation formula is

DWL = ½ e k Ek U0 ((Uk – U0)/U0)2

- Where

e	=	elasticity of electricity Demand with respect to the commodity price

Ek	=	total annual consumption for subgroup k of group 0 (kWh)

Uk	=	Load-weighted annual price for subgroup k of group 0 ($/kWh)

U0	=	Load-weighted annual price for the group 0 ($/kWh).

- For purposes of this calculation, elasticity estimates from secondary sources may be considered, and scenario analysis using a range of values may be used.  A value of 0.2 has been used in some studies.  Note that the annual consumption Ek is the total energy use of all Customers represented by the subgroup profile k.  If the profile is scaled so that the profile hourly value is an estimate of total Load for all Customer represented by the profile, the annual consumption Ek is the sum of the profile hourly values over all hours in the year.  If the profile is scaled in some other way, the group annual consumption Ek may be very different from the sum of the profile hourly values.

- To apply this formula to subdivisions of a Weather Zone, the combined group 0 is the entire Weather Zone.  Each subgroup k is a subdivision proposed as a new Weather Zone.  The formula is evaluated separately for each profile segment, and summed over segments to provide the total benefit of the Weather Zone subdivision.

- To apply this formula to subdivisions of a segment, the combined group 0 is the entire current segment.  Each subgroup k is a subdivision proposed as a new segment.  The formula is evaluated separately for each Weather Zone, and summed over Weather Zones to provide the total benefit of the segment subdivision.

- In either case, the subgroup profiles k defined by profile segment and Weather Zone are applied to the interval prices ut to produce distinct Load-weighted average annual prices Uk.  The Load-weighted average annual price for the existing segment or Weather Zone is U0.

Deadweight Loss Reduction if Only One Subdivision Changes

If only one proposed new segment shall have a new profile, while the remainder shall continue to have the existing profile, the deadweight loss formula reduces to

DWL = ½ e  Ek* U0 ((Uk* – U0)/U0)2

where the subscript k* indicates the proposed new segment.  All other terms in the summation for the full formula are zero.

Deadweight Loss Reduction by Creating a New Segment from Multiple Segments

- When performing an analysis where several groups are being combined to form a single group, the Load Profile for the single group is considered the Target Load Profile.  The Load Profiles for the groups contributing to the formation of the single group are considered the default Load Profiles. Examples where such analyses shall apply include:

- Changing a Weather Zone boundary so that a portion is removed from one zone and added to another

- Changing a set of profile segment definitions so that part of one segment is shifted from one to another

- Choosing between two alternative schemes for defining Weather Zones or profile segments.

  - Any of these choices may be assessed as a difference among possible subdivisions using the formula in this Appendix titled “Deadweight Loss from Finer Subdivision.”  If a single group may be divided into subgroups the deadweight loss reduction from each possible subdivision is calculated using the formula.  The method with the greatest deadweight loss reduction is the preferred method.

  - Thus, to request a change of the definitions of existing segments, a combined segment that includes all segments affected by the change is considered.  The deadweight loss reduction from “subdividing” the combined segment is then calculated.  Likewise, the deadweight loss reduction from subdividing the combined segment into the proposed new set of definitions is calculated.  The reduction from the current segmentation is subtracted from the reduction from the proposed segmentation.  Theoretically, the proposed segmentation is justified in terms of societal value if this difference is greater than the total cost of implementing the change.

  - A change that involves moving a part of a group into another group, or a combination of such moves, may be assessed using this same approach.  The combination of all affected subgroups is considered as the overall group.  The original and alternative groupings are then regarded as two possible subdivisions of this overall group.  The deadweight loss reduction compared to having the full combined group is evaluated for each of these “subdivisions.”  The preferred subdivision is the one with the greater deadweight loss reduction from the combined group.  The benefit of going from the original subdivision to the new one is the increase in the deadweight loss reduction.

  - For example, if the group A is to be moved from being included with group B to being included with group C, the combined group is the combination of A, B, and C.  Under one “subdivision” the subgroups are A+B and C.  Under the other, the subgroups are B and A+C.  The change is theoretically worth making if the deadweight loss reduction for (B, A+C) versus (A+B+C) is greater than that for (A+B, C) versus (A+B+C), by an amount greater than the cost of making the change.

Deadweight Loss Reduction from Revising the Load Profile Model Based on Current Load Research Data

(1)	The profile based on current Load research may be regarded as the best available estimate of the Load Profile for a particular segment.  Continuing to use the current model is then viewed as an approximation to this best available or Target Profile.  One way to measure the severity of the error in this approximation is in terms of the deadweight loss.  Revising the current profile to bring it closer to the target would reduce societal deadweight loss by at most this amount.

Table C-2

Hypothetical Loads and Prices for Illustration

(2)	The deadweight loss due to using the current profile model (default) rather than the target is calculated as

- DWL = ½ e E UDEFAULT ((UTARGET – UDEFAULT)/UDEFAULT)2

- Where

e 		= 	elasticity of electricity Demand with respect to the commodity price

E 		= 	total energy consumption for the profile segment (MWh)

UTARGET 	= 	Load-weighted average annual commodity price using the Target Profile for the segment ($/MWh)

UDEFAULT = Load-weighted average annual commodity price using the default profile for the segment ($/kWh).

Illustration of Measures of Differences

- To illustrate some of the measures of differences, following Table C-2, Hypothetical Loads and Prices for Illustration, shows hourly Loads and hourly market prices for a hypothetical period of two days.  Loads are shown for two sub segments that together make up an existing segment within a Weather Zone.  The highlighted hours are on-peak hours, which are defined as hour ending 0800 through1900 military time.

- Following Table C-3, Single Parameter Characteristics and Differences, shows some of the single-parameter characteristics statistics for each of the three Load Profiles.  Also shown are the differences between each subsegment and the existing combined segment.  These differences are the errors if the subsegments are considered as the targets and the combined is the default that estimates them if the finer segmentation is not adopted.

Table C-3

Single Parameter Characteristics and Differences

- Following Table C-4, Daily and Clock-Hour Totals and Fractions, shows the calculations of daily and clock-hour totals and fractions.

Table C-4
Daily and Clock-Hour Totals and Fractions

- Following Table C-5, Unitized Loads and Difference Measures, shows the unitized Load for the two-day period, and illustrates some of the difference measures based on this series.

Table C-5
Unitized Loads and Difference Measures

First, the unitized Loads are shown for each profile.  The differences or errors between each segment and the existing profile are then shown for the unitized Loads.  Also shown are the absolute difference, squared error, and absolute percent errors.  These are combined at the bottom to give the mean deviation, MAD, MAPE, and RMSE.

- Following Table C-6, Calculation of Deadweight Loss Reduction for Finer Segmentation, illustrates a calculation of the reduction in deadweight loss achieved by changing from the single existing profile to separate profiles for each subsegment.

Table C-6
Calculation of Deadweight Loss Reduction for Finer Segmentation