Texas Renewables Integration Plan

Quarterly Update for the 3-Month Period Ending March 31, 2010

Prepared by the Renewable Technologies Working Group of the ERCOT Technical Advisory Committee

April 2010

Status of Renewable Generation Resources in ERCOT

At the end of March 2010, ERCOT had 9,117 MW of “new” wind generation in operation, where “new” represents capacity added since September 1, 1999.  In addition, there were approximately 1.2 MW of new solar, 40.3 MW of new biomass, 33.1 MW of new hydro and 80.3 MW of new landfill gas renewable generation in operation.  Thus, the total new renewable capacity in ERCOT as of March 31, 2010 is approximately 9,272 MW.

For the three months ending March 31, 2010, the following new renewable resources became operational:

- Penescal Wind Farm in Kenedy County (201 MW)

Wind generation provided 8.4% of the total energy produced in ERCOT from January 1, 2010 through March 31, 2010.  The monthly ERCOT generation fuel mix is available at:

http://planning.ercot.com/reports/demand-energy/

In addition to wind generation capacity that is installed and operating, there are a substantial number of renewable generation projects in various stages of study by ERCOT as part of the Generation Interconnection Process as shown in Figure 1 below.

|  | Capacity (MW) |  |  |
| --- | --- | --- | --- |
| Project Description | Wind | Solar | Biomass |
| Projects with Interconnect Agreement/Public Letter | 5,936 | 0 | 145 |
| Projects Under Full Interconnect Study | 31,983 | 489 | 50 |
| Confidential Projects | 6,915 | 489 | 58 |
| Total | 44,834 | 978 | 253 |

Figure 1 – New Renewable Generation Capacity Under Study

There are 9 wind generation projects and 2 biomass projects with Interconnect Agreements or public letters.   In addition, there are 102 wind generation projects undergoing full interconnect studies.  There are 5 solar projects and 1 biomass project undergoing full interconnect studies.

For the 3-month period ending March 31, 2010, no renewable resources signed Generation Interconnection Agreements.

The annual ERCOT installed wind generating capacities since 1999 are shown in Figure 2 below.

Figure 2 – ERCOT Installed Wind Generating Capacity (End of Year)

Significant Events

At the March 11, 2010 Public Utility Commission of Texas (PUCT) Open meeting, the Commissioners discussed voiding the award of construction rights on the two 345 transmission lines previously awarded to the Lower Colorado River Authority (LCRA) in PUCT Project No. 36802 (which is a project related to the 345 kV transmission lines related to the Competitive Renewable Energy Zones – CREZ) and awarding them to a joint venture between the City of Garland and the South Texas Electric Cooperative (STEC).  This was in response to a successful appeal by the City of Garland and STEC to the state District Court related to the Commission’s original order in Project No. 35665 (Commission Staff’s Petition for Selection of Entities Responsible for Transmission Improvements Necessary to Deliver Renewable Energy from Competitive Renewable Energy Zones).  PUCT Chair Smitherman expressed hope that STEC and Garland could develop the joint venture agreement by the April 1st Open Meeting and hoped that it would be possible to set a new CCN deadline for the subsequent projects in early May, 2010.  A new docket, Docket No. 38045, was established to address potential assignment of joint responsibility for two specific transmission line projects: the McCamey A to Odessa 345 kV line (single circuit on double circuit capable structures) and the McCamey C to McCamey A 345 kV line (single circuit on double circuit capable structures).

On March 31, 2010, the Public Utility Commission of Texas held a workshop in Project No. 35792 – Rulemaking Related to the Goal for Renewable Energy.  The workshop was well attended and the participants heard several presentations on the following topics:

- PUC Rulemaking Authority

- Solar Energy

- Co-Firing, Landfill Gas and Biomass

- Distributed Renewable Generation

- Energy Storage

- Public Interest Issues

- Compliance with REC Requirements

- Implementation Issues

The Commission staff will use the input received from the workshop to revise and update their proposed draft rulemaking on the 500 MW non-wind Renewable Portfolio Standard (RPS).  A revised draft rule is likely to be published for public comment during the second quarter of 2010.

Completed Issues

During the 3-month period ending March 31, 2009, no Issues were completed by the RTWG.

For a detailed list of all Issues and their current status, see the Appendix.

In addition, the following Protocol Revision Request (PRR) impacting renewable resources in the ERCOT zonal market was approved and/or became effective during the first quarter of 2010:

- PRR 841 - Revise Total ERCOT Wind Power Forecast (TEWPF)

New Issues

During the 3-month period ending March 31, 2010, the RTWG began consideration of the following new issues related to the integration of renewable resources into the ERCOT market:

- WT 5 – Wind Workshop IV

- WT 6 – Solar Workshop

- WT 7 – Storage Workshop

Appendix