| NPRR Number | 207 | NPRR Title | Hour Start Unit Deselection and Half Hour Start Unit RUC Clawback |
| --- | --- | --- | --- |
| Date |  | April 22, 2010 |  |
| Submitter’s Information |  |  |  |
| Name |  | Barbara Clemenhagen on behalf of the Wholesale Market Subcommittee (WMS) |  |
| E-mail Address |  | bclemenhagen@topazpowergroup.com |  |
| Company |  | Topaz Power Group |  |
| Phone Number |  | (512) 314-8621 |  |
| Cell Number |  |  |  |
| Market Segment |  | Independent Generator |  |

Comments

At the April 21, 2010 WMS meeting, WMS voted to endorse NPRR207 as amended by the 4/21/10 Topaz comments and as revised by WMS with two abstentions from the Cooperative and Municipal Market Segments and one opposition from the Consumer Market Segment.

WMS also requested that Market Participants file comments prior to the May 6th TAC meeting to address concerns with changing bids between Reliability Unit Commitment (RUC) Notification and the Hourly Reliability Unit Commitment (HRUC).

Revised Cover Page Language

| NPRR Number | 207 | NPRR Title | Unit Deselection |
| --- | --- | --- | --- |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) would provide that ERCOT may deselect units from Reliability Unit Commitment (RUC) processes. The NPRR also creates the term “RUC Notification” to refer to an ERCOT instruction preceding a Resource’s Start-Up time. The NPRR enables the Qualified Scheduling Entity (QSE) to self-commit, without penalty, a Resource after receiving a RUC Notification but before the Resource’s start-uU time. |  |
| Reason for Revision |  | RUC processes consider all remaining Operating Hours for a given Operating Day and issue RUC Dispatch Instructions to ensure reliability. To issue a unit commitment decision well in advance of an unit’s capabilities, however, ignores the unit’s unique operational flexibility and may lead to over-commitment of Generation Resources, increasing uplifted RUC costs. This NPRR would provide ERCOT with discretion to “deselect” units from all RUC processes. In addition to creating ERCOT’s discretion to delay RUC instructions, the NPRR allows the QSE to self-commit the Resource, enabling market-based solutions to avoid uplifted RUC costs. |  |
| Overall Market Benefit |  | This NPRR has the potential to lower uplifted RUC costs. Load and generation forecasts have historically shifted within the Adjustment Period. By delaying the RUC commitments until the last possible time, over-commitment of capacity by the RUC process will be minimized without compromising ERCOT’s ability to utilize this significant quantity of capacity to ensure grid reliability. Furthermore, by enabling the QSE to self-commit the Resource in light of new information but before the Resource’s start-up time eliminates uplifted RUC costs altogether. |  |

Revised Proposed Protocol Language

2.1	DEFINITIONS

Reliability Unit Commitment (RUC) Notification

An ERCOT instruction, received in advance of a Generation Resource’s start-uU time for a given Operating Hour, indicating ERCOT’s intention to issue a RUC.

5.5.2	Reliability Unit Commitment (RUC) Process

(1)	The RUC process recommends commitment of Generation Resources, to match ERCOT’s forecasted Load, subject to all transmission constraints and Resource performance characteristics.  The RUC process takes into account Resources already committed in the Current Operating Plans (COPs), Resources already committed in previous RUCs, and Resource capacity already committed to provide Ancillary Service.  The formulation of the RUC objective function must employ penalty factors on violations of security constraints. The objective of the RUC process is to minimize costs based on Three-Part Supply Offers, substituting a proxy Energy Offer Curve for the Energy Offer Curve, over the RUC Study Period.

(2)	The RUC process can recommend Resource decommitment.  ERCOT may only decommit a Resource to resolve transmission constraints that are otherwise unresolvable. Qualifying Facilities (QFs) may be decommitted only after all other types of Resources have been assessed for decommitment.  In addition, the HRUC process provides decision support to ERCOT regarding a Resource decommitment requested by a Qualified Scheduling Entity (QSE).

(3)	ERCOT shall review the RUC-recommended Resource commitments to assess feasibility and shall make any changes that it considers necessary, in its sole discretion.  ERCOT may deselect units recommended in DRUC and in all HRUC processes if in its sole discretion there is enough time to commit those Resources in future HRUC processes to meet ERCOT System reliability.  At the final HRUC process that is a Generation Resource’s start-up time preceding an Operating Hour, a RUC Notification shall become a RUC instruction.  Prior to that time, QSEs may self-commit the Generation Resource without penalty.  ERCOT shall notify each QSE which of its Resources have been committed as a result of the RUC process.  ERCOT shall, within one day after making any changes to the RUC-recommended commitments, post to the MIS Secure Area any changes that ERCOT made to the RUC-recommended commitments with an explanation of the changes.

5.7.2	RUC Clawback Charge

(1)	A QSE for a Resource shall pay a RUC Clawback Charge for the Operating Day if the RUC Guarantee is less than the sum of:

(a)	RUC Minimum-Energy Revenue calculated in Section 5.7.1, RUC Make-Whole Payment;

(b)	Revenue Less Cost Above LSL During RUC-Committed Hours calculated in Section 5.7.1.3, Revenue Less Cost Above LSL During RUC-Committed Hours; and

(c)	Revenue Less Cost During QSE-Clawback Intervals calculated in Section 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals.

(2)	The amount of the RUC Clawback Charge is a percentage of the difference calculated in paragraph (1), above.  Whether or not the QSE submits a Three-Part Supply Offer for a Resource in the Day-Ahead Market (DAM) determines the clawback percentage.  If the QSE submitted a validated Three-Part Supply Offer for the Resource into the DAM, then the clawback percentage in RUC-Committed Hours is 50% and the clawback percentage in QSE Clawback Intervals is 0%.  If the QSE did not, then the clawback percentage in RUC-Committed Hours is 100% and the clawback percentage in QSE Clawback Intervals is 50%.

(3)	If an Energy Emergency Alert (EEA) is in effect for any hour that a Resource is RUC-committed, then in all RUC-Committed Hours of the Operating Day the clawback percentage is 0% if the QSE submitted a validated Three-Part Supply Offer for the Resource into the DAM and 50% otherwise.

(4)	The RUC Clawback Charge for a Resource, including RMR units, for each Operating Day is allocated evenly over the RUC-Committed Hours for that Resource.

(5)	For each RUC-committed Resource, the RUC Clawback Charge for each RUC-Committed Hour of the Operating Day is calculated as follows:

If (RUCMEREVq,r,d + RUCEXRRq,r,d – RUCGq,r,d) > 0,

Then, RUCCBAMTq,r,h	=	[(RUCMEREVq,r,d + RUCEXRRq,r,d – RUCGq,r,d) * RUCCBFRq,r,d + RUCEXRQCq,r,d * RUCCBFCq,r,d]/RUCHRq,r,d

Otherwise, RUCCBAMTq,r,h = [Max (0, RUCMEREVq,r,d + RUCEXRRq,r,d + RUCEXRQCq,r,d – RUCGq,r,d) * RUCCBFCq,r,d] / RUCHRq,r,d

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCCBAMTq,r,h | $ | RUC Clawback Charge––The RUC Clawback Charge to a QSE for a Resource as described in this Section, for each RUC-Committed Hour of the Operating Day for that Resource. |
| RUCGq,r,d | $ | RUC Guarantee—The sum of the Resource’s eligible Startup Costs and Minimum-Energy Costs during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.1, RUC Guarantee. |
| RUCMEREVq,r,d | $ | RUC Minimum-Energy Revenue—The sum of the energy revenues for the Resource’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.2, RUC Minimum-Energy Revenue. |
| RUCEXRRq,r,d | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for the Resource above the LSL less the cost during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.3. |
| RUCEXRQCq,r,d | $ | Revenue Less Cost from QSE-Clawback Intervals—The sum of the profits during QSE-Clawback Intervals, for the Operating Day. See Section 5.7.1.4. |
| RUCCBFRq,r,d | none | RUC Clawback Factor for RUC-Committed Hours—The Resource’s Clawback factor for RUC-Committed Hours, which is 50% if a Three-Part Supply Offer was submitted and 100% otherwise. During EEA conditions the Resource’s clawback factor for RUC-Committed Hours is 0% if a Three-Part Supply Offer was submitted and 50% otherwise. |
| RUCCBFCq,r,d | none | RUC Clawback Factor for QSE Clawback intervals—The Resource’s Clawback factor for QSE Clawback Intervals, which is 0% if a Three-Part Supply Offer was submitted and 50% otherwise. |
| RUCHRq,r,d | none | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| h | none | An hour in the RUC-commitment period. |