| NPRR Number | 218 | NPRR Title | Resolution of Alignment Item A71 - Add Protocol Description of the Power Balance Penalty Factor used in the SCED |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Tabled. |
| Date of Decision |  | April 22, 2010 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 6.5.7.1.11, Transmission Constraint Management 6.5.7.3, Security Constrained Economic Dispatch |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) will add a description of the power balance penalty factor to the Nodal Protocol sections that describe the implemented constraint set utilized in the Security-Constrained Economic Dispatch (SCED) optimization. |  |  |
| Reason for Revision |  | This change conforms the Nodal Protocols to the as-built Nodal system. |  |  |
| Overall Market Benefit |  | Alignment of the Nodal Protocols with the as-built Nodal system. |  |  |
| Overall Market Impact |  | Unknown. |  |  |
| Consumer Impact |  | Unknown. |  |  |
| Credit Impacts |  | None. |  |  |
| Procedural History |  | - On 3/23/10, NPRR218, a CEO Revision Request Review and Impact Analysis were posted. - On 4/22/10, PRS considered NPRR218. |  |  |
| PRS Decision |  | On 4/22/10, PRS unanimously voted to table NPRR218. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 4/22/10, ERCOT Staff explained the purpose of NPRR218 and committed to submitting comments for describing the process for setting maximum Shadow Prices. Concerns were raised regarding policy setting in the Nodal Protocols. |  |  |

| Quantitative Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | Unknown. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Alignment of Nodal Protocols with the Nodal system. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Mark Patterson |
| E-mail Address | mpatterson@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3912 |
| Cell Number |  |
| Market Segment | N/A |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None. |  |

Proposed Protocol Language Revision

6.5.7.1.11	Transmission and Power Balance Constraint Management

(1)	ERCOT may not allow any contingency anticipated to be active in SCED, identified by NSA, until it has verified that the contingency is accurate and appropriate given the current operating state of the ERCOT Transmission Grid.  ERCOT shall continuously post to the MIS Secure Area any active contingencies in SCED and any contingencies that it has determined to be inaccurate or inappropriate and thus excluded from SCED under Section 5.5.1, Security Sequence.  The ERCOT System Operator will flag for further review by ERCOT any contingencies deemed inaccurate or inappropriate.

(2)	ERCOT shall establish a maximum Shadow Price for each network constraint as part of the definition of contingencies.  The cost calculated by SCED to resolve an additional MW of congestion on the network constraint is limited to the maximum Shadow Price for the network constraint.  ERCOT shall develop a policy for setting maximum Shadow Prices for approval as prescribed in Section 21, Process for Nodal Protocol Revision.

(3)	ERCOT shall establish a maximum Shadow Price for the power balance constraint.  The cost calculated by SCED to resolve either the addition or reduction of one MW of dispatched generation on the power balance constraint is limited to the maximum Shadow Price for the power balance constraint.  ERCOT shall develop a policy for setting the maximum Shadow Price on the power balance constraint for approval as prescribed in Section 21, Process for Nodal Protocol Revision.

(4)	When ERCOT identifies a binding network constraint on a repeated basis ERCOT shall have procedures established to contact the appropriate TSP and validate the accuracy of the Network Operations Model according to paragraph (5) of Section 3.10.4, ERCOT Responsibilities.

(5)	If ERCOT determines that rating(s) in the Network Operations Model or configuration of the Transmission Facilities are not correct, then the TSP will provide the appropriate data submittals to ERCOT to correct the problem upon notification by ERCOT.

6.5.7.3	Security Constrained Economic Dispatch

(1)	The SCED process is designed to simultaneously manage energy, the system power balance and network congestion through Resource Base Points and calculation of LMPs every five minutes.  The SCED process uses a two-step methodology that applies mitigation prospectively to resolve network Non-Competitive Constraints for the current Operating Hour.  The SCED process evaluates Energy Offer Curves and Output Schedules to produce a least cost dispatch of On-Line Generation Resources to the total current generation requirement determined by LFC, subject to power balance and network constraints.  The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP.