| PRR Number | 847 | PRR Title | Additional Exemptions for Uninstructed Resource Charge |
| --- | --- | --- | --- |
| Date |  | May 13, 2010 |  |
| Submitter’s Information |  |  |  |
| Name |  | Formosa Utility Venture, Ltd. |  |
| E-mail Address |  | ewoelfel@fpcusa.com |  |
| Company |  | Formosa Utility Venture, Ltd. (QSE) |  |
| Phone Number |  | 973-716-7389 |  |
| Cell Number |  | 973-634-6107 |  |
| Market Segment |  | Independent Generator |  |

Comments

6.8.1.15.1	Uninstructed Resource Charge Process

Once ERCOT has determined the Congestion Zones in which the Uninstructed Deviations of the QSE have occurred, an uninstructed pricing differential will be applied to a QSE representing Resources whenever one (1) of the following two (2) conditions exists:

(1)	The QSE representing Resources has total metered Resources for any Settlement Interval that are greater than the larger of one hundred one and one-half percent (101.5%) of the QSE’s total of schedules plus instructions or five (5) MWh over the total of schedules plus instructions when the integrated amount of ERCOT-wide regulation is less than negative twenty-five (– 25) MWhs in the interval.  Upon one-day Notice, ERCOT can reduce the percentage and MWh tolerance to one hundred one percent (101%) and three (3) MWh, respectively, if significant price chasing exists.

(2)	The QSE representing Resources has total metered Resources for any Settlement Interval that are less than the lesser of ninety-eight and one-half percent (98.5%) of the total schedule plus instructions or five (5) MWh under the total of schedules plus instructions when the integrated amount of ERCOT-wide regulation is greater than positive twenty-five (+ 25) MWh in the interval.  Upon one day Notice, ERCOT can increase the percentage and MWh tolerance to ninety-nine percent (99%) and three (3) MWh, respectively, if significant price chasing exists.

(3)	A QSE that schedules Uncontrollable Renewable Resources will not be subject to Uninstructed Deviation.  The terms of this paragraph shall immediately apply to QSEs that represent only Uncontrollable Renewable Resources.  A QSE subject to the terms of this paragraph shall observe the requirements of Section 4.4.15, QSE Resource Plans, and use best efforts to continuously update its Resource Plan to reflect anticipated operating conditions.

(4)	A single resource QSE shall be exempt from an Uninstructed Resource Charge (URC) following a Verbal Dispatch Instruction (VDI) for each interval involving the VDI.

(5)	During a LaaR event, including a live LaaR qualification test, and for the three subsequent hours after a LaaR event has been terminated, a QSE representing a PUN will not be subject to URC up to the LaaR response deployment amount.

If any condition listed in items (1) or (2) above exists, then an Uninstructed Deviation pricing differential will be applied as follows:

(a)	QSEs would be paid (or pay) for instructed deviations and Uninstructed Deviations at the zonal MCPE under Resource Imbalance Charge.

(b)	An Uninstructed Deviation Charge Back would be calculated at a factor of the MCPE.  The Uninstructed Deviation Charge Back would be applied to each Congestion Zone that a QSE has a deviation from schedule.

When the two (2) conditions described in items (1) and (2) above are met, an Uninstructed Factor will be instituted to reduce the payment to the QSE.  The Uninstructed Factor will be adjusted in relation to the amount of RGS that was required during that interval on a sliding scale.

Once a regulation is determined to be over or under the defined bandwidth, a QSE would be subject to the uninstructed charge for over-generation when the MCPE is positive, and for under-generation when the MCPE price is negative.