- Registration Data Use in DAM/RUC/SASM/SCED

| ERCOT System Source | Data | MP Submission Source | Application Usage Notes (Refer to ERCOT Acronyms Attachment) |
| --- | --- | --- | --- |
| Registration System | - General Resource Parameters for each Generation Resource represented by the QSE: |  |  |
|  | - The Resource’s name | RARF Gen/CC/Wind Unit Information tab | DAM/SASM/RUC/SCED Optimization MMS Offer (3PO, ASO, INC/DEC, Output Schedule) & COP Validation |
|  | - High Reasonability Limit | RARF Gen/CC/CFG/Wind Parameters tab | MMS COP Validation; Aggregation/disaggregation between the CCT logical Resource Node and the CCT generation units in DAM/RUC. |
|  | - Low Reasonability Limit | RARF Gen/CC/CFG/Wind Parameters tab | MMS COP Validation |
|  | - Type of Resource – steam turbine, hydro, gas turbine, combined cycle, other | RARF Gen/CC/Wind Unit Info tab | DAM/RUC Startup & Min Energy Generic Cost limits (Pivotal Player identification in the Competitive Constraint Test ; Proxy Offer Curve creation and Mitigated Offer Cap/floor curve for SCED) |
|  | - QF Status | RARF Gen/CC/Wind Unit Information tab | COP Validation; Proxy Offer Curve creation for SCED |
|  | - AS Qualification | QSE submitted AS Attestation Form, or ERCOT internal notification upon the completion of a successful AS Qualification Test | MMS Offer & COP and AS Validation; RUC AS Resource selection list for the Operator. |
|  | - Resource Commercial Operations Date | RARF Gen/CC/Wind Unit Information tab | SCED Mitigated Cap Curve; RUC make whole cost calculation. |
|  | - Split Generation Resource data | RARF Ownership GEN and WIND tabs | MMS Offer & COP Validation; DAM/RUC SGR commitment constraints |
|  | - Load Resource Parameters for each Load Resource that is not a Controllable Load Resource represented by the QSE: | Load Asset Registration Form (LARF) |  |
|  | - The Resource’s name | LARF Load Resource Information tab | MMS AS Offers & COP Validation |
|  | - High Reasonability Limit | LARF Load Resource Information tab | MMS COP Validation |
|  | - Low Reasonability Limit | LARF Load Resource Information tab | MMS COP Validation |
|  | - Ancillary Service Qualification | QSE submitted AS Attestation Form, or ERCOT internal notification upon the completion of a successful AS Qualification Test | MMS AS Offer & COP Validation |
|  | - Load Resource Parameters for each Controllable Load Resource represented by the QSE: | Load Asset Registration Form (LARF) |  |
|  | - The Resource’s name | LARF Load Resource Information tab | MMS Offer & COP Validation |
|  | - CLR High Reasonability Ramp Rate Limit | LARF Load Resource Information tab | MMS Offer & COP Validation |
|  | - CLR Low Reasonability Limit Rate Limit | LARF Load Resource Information tab | MMS Offer & COP Validation |
|  | - QSE Parameters: |  |  |
|  | - QSE ID | ERCOT Manual update of Registration Database based on Qualified Scheduling Entity Application and Service Filing for Registration | MMS Offer & COP Validation |
|  | - QSE-to-Resource mapping | RARF General Information; Site Information and Unit Info (GEN, CC, Wind) tabs LARF, Load Resource Information tab Qualified Scheduling Entity Acknowledgement | MMS Offer & COP Validation |
|  | - Emergency QSE status | Entry by Wholesale Client Services & Qualified Scheduling Entity Acknowledgement | MMS Offer & COP Validation |
|  | - Virtual QSE status | Entry by Wholesale Client Services & Qualified Scheduling Entity Acknowledgement | MMS Offer & COP Validation |
|  | - Counter-Party to QSE Mapping Information: |  |  |
|  | - QSE ID | Credit Application | DAM/RUC Optimization |
|  | - Counter-Party ID | Credit Application | MMS Offer Validation (Phase II validations) prior to DAM execution CRR Optimization |
|  | - Combined Cycles Registration Data: |  |  |
|  | - Name of Combined Cycle Train | RARF Unit Info CC tab | MMS Offer (3PO and ASO) & COP Validation |
|  | - CCGR configurations | RARF Parameters-CFG tab | DAM/SASM/RUC/SCED Optimization MMS Offer & COP Validation |
|  | - Associated Physical Generation Units in each CCGR configuration | RARF CCx Config tab | DAM/RUC Optimization MMS Offer & COP Validation |
|  | - CCT Physical Generator Names | RARF Unit Info CC tab | DAM/RUC Optimization |
|  | - CCT cross reference between CCGR configurations and physical generation units | RARF CCx Config tab | DAM/RUC Optimization MMS Offer & COP Validation |
|  | - Transition matrix including transition direction, including Startup and shutdown transition flags | RARF CCx Transition tab | DAM/RUC Optimization MMS AS Offers & COP Validation for Off-line NS |
|  | - Primary/Alternate flag | RARF CCx Config tab | DAM/RUC Optimization |

| ERCOT System Source | Data | MP Submission Source | Application Usage Notes |
| --- | --- | --- | --- |
| Resource Parameter Update *** | Generation Resource Parameters |  |  |
|  | - Normal Ramp Rate Curve - Emergency Ramp Rate Curve | Market Manager Resource Parameters UI** | RLC/SCED Dispatch Limits; MMS DAM/RUC constraints for maximum awardable RRS quantity |
|  | - Minimum On-line Time - Minimum Off-line Time - Hot Start Time - Intermediate Start Time - Cold Start Time - Max On-Line Time - Max Daily Starts - Hot-to-intermediate Time - Intermediate-to-cold Time | Market Manager Resource Parameters UI** | DAM/RUC Optimization |
|  | Non-controllable Load Resource |  |  |
|  | - Minimum Interruption Time - Minimum Restoration Time - Maximum Weekly Deployments - Maximum Interruption Time - Maximum Daily Deployments - Maximum Weekly Energy - Minimum Notice Time | Market Manager Resource Parameters UI** | Performance Monitoring |
|  | Controllable Load Resource Parameters |  |  |
|  | - Normal Ramp Rate Curve - Emergency Ramp Rate Curve - Maximum Deployment Time - Maximum Weekly Energy | Market Manager Resource Parameters UI** | Performance Monitoring |

** Resource Parameter Market Interface

The MMS Market facing interface for Qualified Scheduling Entities (QSE) to update Resource Parameters is available on the MIS UI Market Manager.

The data submitted by the QSE through the Market Manager Resource Parameters interface will be effective as of the time it is submitted.  The Resource Parameter data is stored within MMS Market Interface system tables that are directly accessible by the EMS applications, for example Resource Limit Calculations. The MMS Market Interface System/EMS polling rate is configurable and currently set at 5 minutes.

The data entries are for a point in time going forward and remain in effect until changed by the QSE.  The system will be initialized from the ERCOT Registration System with data provided by the Resource Entity (RE).  Thereafter the RE’s QSE can manage the parameter changes and their return to default values through the Market Manager Resource Parameter UI.

The resource Parameters available for update through this process are:

- Resource Parameters for Generation (Protocol 3.7.1.1):

Normal Ramp Rate Curve;

Emergency Ramp Rate Curve;

Minimum On-line Time;

Minimum Off-line Time;

Hot start time;

Intermediate Start Time;

Cold start time;

Max Weekly Starts;

Max On-Line Time;

Max Daily Starts;

Max weekly Energy;

Hot-to-intermediate Time; and

Intermediate-to-cold Time

- Resource Parameters for Load Resource (not Controllable) (Protocol 3.7.1.2 (1)):

Minimum Interruption Time;

Minimum Restoration Time;

Maximum Weekly Deployments;

Maximum Interruption Time;

Maximum Daily Deployments;

Maximum Weekly Energy; and

Minimum Notice Time

- Resource Parameters for Controllable Load Resources (Protocol 3.7.1.2 (2)):

Normal Ramp Rate Curve;

Emergency Ramp Rate Curve;

Maximum Deployment Time; and

Maximum Weekly Energy

All QSE changes to Resource Parameters entered through the Market Manager application are logged.  The application also includes a field in which the QSE may provide the reason for the change to the parameters such as for seasonal purposes, equipment issues, or other needs.  The Reason for Change field is a free text field (256 character limit) with blank default.  Additionally, the application will not allow a QSE to enter Ramp Rates for a Resource that exceeds the Ramp Rate Reasonability Limits provided during the Registration Process by the Resource Entity.

The Market Manager Resource Parameter Update Application allows the QSE to query but not change the Resource’s High Reasonability Limit (HRL), Low Reasonability Limit (LRL) and Ramp Rate Reasonability Limits.  Changes to these reasonability limits must be submitted through the Registration update process.

The Market Manager Resource Parameter screen also includes a cancel function to allow the QSE to return all resource parameter values to the default values specified in the Registration System.

ERCOT ACRONYMS

This is a collection of acronyms we use at ERCOT. They include terms from the electric utility industry, information technology, project management and ERCOT.

Shaded Acronyms are used in this document.

Shaded rows indicate this term is no longer used, but may be found in older materials. Terms that are capitalized indicate they are Protocol-defined terms, names of systems, names of projects or formal names of organizations.

| ACRONYM | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| 3PO | Three Part Supply Offer |  |
| ACE | Area Control Error |  |
| ACL | available credit limit |  |
| ADR | Alternative Dispute Resolution |  |
| AGC | automatic generation control |  |
| ALDR | annual load data request |  |
| AMIT | Advanced Metering Implementation Team |  |
| AML | adjusted meter load |  |
| API | application programming interface |  |
| AREP | Affiliated Retail Electric Provider |  |
| ARI | asset registration interfaces |  |
| AS | Ancillary Service |  |
| ASD | ancillary service dispatch |  |
| ASO | Ancillary Service Offer |  |
| ARS | asset registration system |  |
| ATC | area transmission constraints |  |
| AVR | Automatic Voltage Regulator |  |
| BES | Balancing Energy Service |  |
| BLT | Block Load Transfer |  |
| BoD | Board of Directors |  |
| BR | business requirements |  |
| CAO | control area operator |  |
| CAF | Contract Approval Form |  |
| CCM | control center model |  |
| CCP | change control procedures |  |
| CCT | constraint competitiveness tests |  |
| CDR | Capacity Demand Reserve |  |
| CDR | Current Day Reports |  |
| CCGR | Combined Cycle Generation Resource |  |
| CEII | Critical Energy Infrastructure Information |  |
| CI | change item |  |
| CIM | common information model |  |
| CLR | controllable load resource |  |
| CMM | Credit Monitoring Management |  |
| CMWG | Congestion Management Working Group |  |
| COMG | Commercial Operations Market Guide |  |
| COMS | Commercial Systems |  |
| COP | Current Operating Plan |  |
| COPS | Commercial Operations Subcommittee |  |
| CPS | control performance standard (CPS1 or CPS2) |  |
| CPT | central prevailing time |  |
| CR | Competitive Retailer |  |
| CREZ | competitive renewable energy zone |  |
| CRR | Congestion Revenue Rights |  |
| CSA | Continuous Service Agreement |  |
| CSC | Commercially Significant Constraint |  |
| CSD | conceptual system design |  |
| CSI | Commercial Systems interface |  |
| CWG | Credit Work Group |  |
| DA | day-ahead |  |
| DAG | Delivery Assurance Group | Defunct nodal group |
| DAM | Day-Ahead Market |  |
| DAMCPE | DAM clearing price for energy |  |
| DAS | Data Aggregation System |  |
| DBA | Database administrator |  |
| DC | direct current |  |
| DC Tie | Direct Current Tie |  |
| DDL | data definition language |  |
| DEF | deliverability evaluation function |  |
| DEWG | Data Extracts Working Group |  |
| DGTF | Distributed Generation Task Force |  |
| DLC | Direct Load Control |  |
| DLF | Distribution Loss Factor |  |
| DRG | distributed renewable generation |  |
| DRS | down regulation service |  |
| DRUC | Day Ahead Reliability Unit Commitment |  |
| DSP | Distribution Service Provider |  |
| DSR | Dynamically Scheduled Resource |  |
| DSWG | Demand-Side Working Grup |  |
| DUNS | Data Universal Numbering System |  |
| DUNS # | DUNS Number |  |
| DWG | Dynamics Working Group |  |
| E2E | end-to-end (testing) |  |
| EAC | estimate at completion |  |
| EC | Electric Cooperative |  |
| ECI | Element Competitiveness Index |  |
| EDI | Electronic Data Interchange |  |
| EDS | Early Delivery Systems |  |
| EEA | Energy Emergency Alert | Was EECP |
| EECP | Emergency Electric Curtailment Plan | Changed to EEA |
| EDW | Enterprise Data Warehouse |  |
| EIF | enterprise integration framework |  |
| EILS | Emergency Interruptible Load Service |  |
| EIP | Enterprise Integration Project |  |
| EIS | Enterprise Information Systems |  |
| EMBP | emergency base point |  |
| EMIL | ERCOT Master Information List |  |
| EMS | Energy Management System |  |
| EMMS | Energy Management and Market Systems |  |
| EPS | ERCOT-Polled Settlement Meter |  |
| EPRI | Electric Power Research Institute |  |
| ERCOT | Electric Reliability Council of Texas |  |
| ERO | electric reliability organization |  |
| ERT | ERCOT Readiness & Transition |  |
| ESI ID | Electric Service Identifier |  |
| FAT | functional acceptance testing |  |
| FERC | Federal Energy Regulatory Commission |  |
| FGR | Flowgate Right |  |
| FIP | Fuel Index Price |  |
| FOD | forced outage detection |  |
| FOP | Fuel Oil Price |  |
| FT | Financial Transfer System |  |
| FTE | full time employee |  |
| FTP | file transfer protocol |  |
| GTL | Generic Transmission Limit |  |
| GUI | graphical user interface |  |
| HASL | High Ancillary Service Limit |  |
| HDL | High Dispatch Limit |  |
| HE | Hour Ending |  |
| HEL | High Emergency Limit |  |
| HRUC | Hourly Reliability Unit Commitment |  |
| HSL | High Sustained Limit |  |
| Hz | Hertz |  |
| IA | impact analysis |  |
| I/A | interface and adapter (testing) |  |
| ICCP | Inter-Control Center Communications Protocol |  |
| IDA | Integration & Design Authority | Defunct nodal project |
| IDR | interval data recorder |  |
| IEC | International Electrotechnical Commission |  |
| IG/IAG | inadvertent gains |  |
| ILM | Information Lifecycle Management |  |
| IMM | Independent Market Monitor |  |
| IMM | Information Model Manager |  |
| INC/DEC | Incremental/Decremental Energy Offer Curve |  |
| INF | Infrastructure (Project) |  |
| INT | Integration Testing (Project) | Defunct nodal project |
| IOU | investor-owned utility |  |
| IPP | independent power producers |  |
| IRR | Intermittent Renewable Resources |  |
| ISA | integrated subject area |  |
| ISO | independent system operator |  |
| iTest | integrated testing environment |  |
| KPI | key performance indicator |  |
| kV | Kilovolt |  |
| LaaRs | loads acting as resources |  |
| LARF | Load Asset Registration Form |  |
| LASL | Low Ancillary Service Limits |  |
| LDAP | lightweight Directory Access Protocol |  |
| LDL | Low Dispatch Limits |  |
| LEL | Low Emergency Limit |  |
| LF | load forecast / load forecasting |  |
| LFC | Load Frequency Control |  |
| LMP | Locational Marginal Pricing |  |
| LMS | Learning Management System |  |
| LOA | letter of authorization |  |
| LPGRR | Load Profiling Guide revision request |  |
| LRS | Load Ratio Share |  |
| LRS | load research sampling |  |
| LSE | Load Serving Entity |  |
| LSL | Low Sustained Limit |  |
| LTLF | Long-Term Load Forecast |  |
| MAP | Mitigation Action Plan |  |
| MARS | market advanced readings and settlement |  |
| McFRI | McCamey Area Flowgate Right |  |
| MCP | Market Clearing Price |  |
| MCPC | Market Clearing Price for Capacity |  |
| MCPE | Market Clearing Price for Energy |  |
| MDAS | Meter Data Acquisition System |  |
| MER | Market Engagement & Readiness (Project) | Defunct nodal project |
| MID | market information distribution |  |
| MIR | market information repository |  |
| MIS | Market Information System |  |
| MLRSZ | monthly load ratio share zonal |  |
| MMS | Market Management System |  |
| MMWG | Market Metrics Working Group |  |
| MOD | model on demand |  |
| MOMS | Market Oversight Monitoring System |  |
| MOS | Market Operating System |  |
| MOU | Municipally Owned Utility |  |
| MOTE | Market Operations Test Environment |  |
| MP | Market Participant |  |
| MPC | Maximum Power Consumption |  |
| MPI | market perceived inconsistencies |  |
| MPIM | Market Participant Identity Management (System) |  |
| MP USA | Market Participant user security administrators |  |
| MRE | Meter Reading Entity |  |
| MT | MarkeTrak (System) |  |
| MT | market trials |  |
| MTLF | Mid-Term Load Forecast |  |
| MVAr | Mega Volt-Amperes reactive |  |
| MW | Megawatt |  |
| MWG | Metering Working Group |  |
| MWh | Megawatt Hour |  |
| NATF | Nodal Advisory Task Force |  |
| NDSML | Nodal Data Services Master List | Replaced by EMIL |
| NDSWG | Network Data Support Working Group |  |
| NERC | North American Electric Reliability Corporation |  |
| N-iTest | nodal integrated testing environment |  |
| NIT | Nodal Implementation Team | Renamed NATF |
| NMG | Network Modeling Group |  |
| NMMS | Network Model Management System |  |
| NOIE | Non Opt-In Entity |  |
| NOM | Network Operations Model |  |
| NOMCR | Network Operations Model Change Request |  |
| Non-Spin | Non-Spinning Reserve |  |
| NMT | network model and telemetry |  |
| NPRR | nodal protocol revision request |  |
| NSA | network security analysis |  |
| NSLR | non-spin load resource |  |
| NSRS | Non-Spinning Reserve Service |  |
| OBL | other biomass liquids |  |
| OBS | other biomass solids |  |
| OCN | Operating Condition Notice |  |
| ODS | operational data store |  |
| OE | outage evaluation |  |
| OOM | Out of Merit Order |  |
| OOMC | Out of Merit Capacity |  |
| OOME | Out of Merit Energy |  |
| OPC | Office of Public Utility Counsel |  |
| OS | Outage Scheduler (System) |  |
| OTS | Operator Training Simulator |  |
| OUI | Operator User Interface |  |
| OWG | Operations Working Group |  |
| PC | program controls |  |
| PCRR | Pre-assigned Congestion Revenue Right |  |
| PDCWG | Performance, Disturbance, Compliance Working Group |  |
| PERTF | Potomac Economics Recommendations Task Force |  |
| PGC | Power Generation Company |  |
| PM | project manager |  |
| PM | Power Marketer |  |
| PMO | Program Management Office |  |
| PO | purchase order |  |
| POD | PTP option de-rating |  |
| POLR | Provider of Last Resort |  |
| POS | Power Operating System |  |
| PRF | Purchase Request Form |  |
| PRR | protocol revision request |  |
| PRS | Protocol Revision Subcommittee |  |
| PSM | power system model |  |
| PSS/E | power system simulator for engineering |  |
| PTB | Price-to-Beat |  |
| PTC | Project Tracker & Coordinator (System) |  |
| PTP | Point-To-Point |  |
| PUC | Public Utility Commission |  |
| PUCT | Public Utility Commission of Texas |  |
| PUN | Private Use Network |  |
| PURA | Public Utility Regulatory Act, Title II, Texas Utility Code |  |
| PURPA | Public Utility Regulatory Policy Act |  |
| QA | quality assurance |  |
| QC | quality control |  |
| QF | Qualifying Facility |  |
| QMWG | QSE Managers’ Working Group |  |
| QSE | Qualified Scheduling Entity |  |
| RAP | Remedial Action Plan |  |
| RARF | Resource Asset Registration Form |  |
| RDF | Reserve Discount Factor |  |
| REC | Renewable Energy Credit |  |
| Reg-Down | Regulation Down |  |
| Reg-Up | Regulation Up |  |
| RE | Resource Entity |  |
| REP | Retail Electric Provider |  |
| RFI | request for information |  |
| RFP | request for proposal |  |
| RGS | regulation service |  |
| RID | Resource ID |  |
| RIDR | Representative IDR |  |
| RLC | Resource Limit Calculator |  |
| RM | resource manager |  |
| RMG | Retail Market Guide |  |
| RMGRR | Retail Market Guide revision request |  |
| RMR | Reliability Must Run |  |
| RMS | Retail Market Subcommittee |  |
| ROS | Reliability and Operations Subcommittee |  |
| RPP | renewal production potential/ resource production potential |  |
| RPRS | Replacement Reserve Service |  |
| RRO | regional reliability organization |  |
| RRS | Responsive Reserve Service |  |
| RTC | real-time co-optimization |  |
| RT | Real-Time |  |
| RTM | Real-Time Market |  |
| RTO | reliability transmission operator |  |
| RTO | Real-Time operations |  |
| RTP | retail transaction processing |  |
| RTU | remote terminal unit |  |
| RTWG | Renewable Technologies Working Group |  |
| RUC | Reliability Unit Commitment |  |
| RUI | requestor user interface |  |
| RUP | Rational Unified Process |  |
| S&B | Settlements and Billing (System) |  |
| SASM | Supplemental Ancillary Service Market |  |
| SAT | site acceptance testing |  |
| SCADA | Supervisory Control and Data Acquisition |  |
| SCED | Security-Constrained Economic Dispatch |  |
| SCR | system change request |  |
| SCUC | Security-Constrained Unit Commitment |  |
| SDAWG | Settlement and Data Aggregation Working Group |  |
| SDRAMP | SCED down ramp rates |  |
| SE | State Estimator |  |
| SEM | single-entry model |  |
| SET | standard electronic transaction |  |
| SFT | simultaneous feasibility test |  |
| SGR | Split Generation Resource |  |
| SIG | System Implementation Guide |  |
| SLA | service level agreement |  |
| SME | subject matter expert |  |
| SOA | service-oriented architecture |  |
| SoSA | System of Systems Architecture |  |
| SOW | statement of work |  |
| SPP | Settlement Point Price |  |
| SPS | Special Protection Systems |  |
| SPWG | System Protection Working Group |  |
| SSWG | Steady State Working Group |  |
| STLF | Short-Term Load Forecast |  |
| SURAMP | SCED up ramp rates |  |
| TAC | Technical Advisory Committee |  |
| TCH | transaction clearinghouse |  |
| TCR | Transmission Congestion Rights |  |
| TDSP | Transmission/Distribution Service Provider |  |
| TDU | transmission and distribution utility |  |
| TDWG | Texas Data Working Group |  |
| TCR | Transmission Congestion Rights |  |
| TML | Texas Market Link |  |
| TNT | Texas Nodal Team | Defunct team |
| TNMID | Texas Nodal Market Implementation Date |  |
| TNMIP | Texas Nodal Market Implementation Program |  |
| TNP | Texas Nodal Project |  |
| TPO | Three-Part Supply Offer |  |
| TPTF | Transition Plan Task Force (Nodal) | Defunct task force replaced by NATF |
| TRE | Texas Regional Entity |  |
| TSAT | transient security assessment tool |  |
| TSP | Transmission Service Provider |  |
| TXMACS | Texas Energy Management and Control System |  |
| UAT | user acceptance test |  |
| UFE | Unaccounted for Energy |  |
| UI | user interface |  |
| URS | up regulation service |  |
| USA | user security administrator |  |
| VAr | Volt-Ampere reactive |  |
| VC/VCS | verifiable cost / verifiable cost submissions |  |
| VCWG | Verifiable Cost Working Group |  |
| VDI | Verbal Dispatch Instruction |  |
| VRT | voltage ride-through |  |
| VSAT | voltage stability analysis tool |  |
| VSS | voltage support service |  |
| WBS | work breakdown structure |  |
| WEMM | Wholesale Electric Market Monitor |  |
| WGR | Wind-powered Generation Resource |  |
| WMS | Wholesale Market Subcommittee |  |
| XML | Extensible Markup Language |  |
| XSD | XML schema definition |  |