| NPRR Number | 207 | NPRR Title | Unit Deselection (formerly “Hour Start Unit Deselection and Half Hour Start Unit RUC Clawback”) |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Tabled |
| Date of Decision |  | May 6, 2010 |  |  |
| Proposed Effective Date |  | Upon System Implementation - Post Texas Nodal Market Implementation Date (TNMID) |  |  |
| Priority and Rank Assigned |  | Priority High |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 4.4.9.2, Startup Offer and Minimum-Energy Offer 4.4.9.2.1, Startup Offer and Minimum-Energy Offer Criteria 4.4.9.3, Energy Offer Curve 5.5.2, Reliability Unit Commitment (RUC) Process 5.7.2, RUC Clawback Charge |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) would provide that ERCOT may deselect units from Reliability Unit Commitment (RUC) processes. The NPRR also creates the term “RUC Notification” to refer to an ERCOT instruction preceding a Resource’s startup time. The NPRR enables the Qualified Scheduling Entity (QSE) to self-commit, a Resource after receiving a RUC Notification but before the Resource’s startup time. |  |  |
| Reason for Revision |  | RUC processes consider all remaining Operating Hours for a given Operating Day and issue RUC Dispatch Instructions to ensure reliability. To issue a unit commitment decision well in advance of a unit’s capabilities, however, ignores the unit’s unique operational flexibility and may lead to over-commitment of Generation Resources, increasing uplifted RUC costs. This NPRR would provide ERCOT with discretion to “deselect” units from all RUC processes. In addition to creating ERCOT’s discretion to delay RUC instructions, the NPRR allows the QSE to self-commit the Resource, enabling market-based solutions to avoid uplifted RUC costs. |  |  |
| Overall Market Benefit |  | This NPRR has the potential to lower uplifted RUC costs. Load and generation forecasts have historically shifted within the Adjustment Period. By delaying the RUC commitments until the last possible time, over-commitment of capacity by the RUC process will be minimized without compromising ERCOT’s ability to utilize this significant quantity of capacity to ensure grid reliability. Furthermore, by enabling the QSE to self-commit the Resource in light of new information but before the Resource’s start-up time eliminates uplifted RUC costs altogether. |  |  |
| Overall Market Impact |  | Unknown. |  |  |
| Consumer Impact |  | Unknown. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR207 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 1/22/10, NPRR207 and a CEO Revision Request Review were posted. - On 2/11/10, Topaz comments were posted. - On 2/15/10, Reliant Energy comments were posted. - On 2/17/10, WMS comments were posted. - On 2/18/10, PRS considered NPRR207. - On 3/3/10, an Impact Analysis was posted. - On 3/4/10, TAC considered NPRR207. - On 3/31/10, a second set of WMS comments were posted. - On 4/2/10, a second set of Reliant Energy comments were posted. - On 4/7/10, ERCOT comments were posted. - On 4/8/10, TAC again considered NPRR207. - On 4/20/10, a second set of Topaz comments were posted. - On 4/21/10, a third set of Topaz comments were posted. - On 4/22/10, a third set of WMS comments were posted. - On 4/22/10, PRS again considered NPRR207. - On 5/3/10, a second set of ERCOT comments were posted. - On 5/6/10, TAC again considered NPRR207. |  |  |
| PRS Decision |  | On 2/18/10, PRS voted to recommend approval of NPRR207 as submitted, to forward NPRR207 to TAC, and to recommend a priority of High. There was one abstention from the Consumer Market Segment. All Market Segments were present for the vote. On 4/22/10, PRS voted to recommend approval of NPRR207 as amended by the 4/22/10 WMS comments and as revised by PRS. There was one abstention from the Consumer Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 2/18/10, ERCOT Staff explained that the impacts to the Commercial Systems (COMS) reported in the CEO Revision Request Review were related to both the Hour Start Unit deselection and Half Hour Start Unit RUC Clawback portions of NPRR207. Participants requested that the ERCOT CEO explain the extent of the system impacts related to each of the two portions of the NPRR and indicate whether either or both portions could be implemented prior to the TNMID. ERCOT Staff indicated that NPRR207 would be reevaluated by the ERCOT CEO for consideration of these issues. In considering what priority to recommend for NPRR207, it was suggested that the priority be High for the portions of NPRR207 that cannot be implemented prior to the TNMID. On 4/22/10, the 4/22/10 WMS comments were reviewed clarifying that ERCOT would not issue a RUC order until the last Hourly Reliability Unit Commitment (HRUC) prior to the selected unit’s startup time. Additional language was proposed to address concerns regarding the ability for QSEs to increase their Three-Part Supply Offer or Energy Offer Curve for any hour in which the Resource has received a RUC Notification. ERCOT Staff stated that they would be reviewing the proposed language for any Settlement implications. |  |  |
| TAC Decision |  | On 3/4/10, TAC unanimously voted to table NPRR207 for one month. All Market Segments were present for the vote. On 4/8/10, TAC unanimously voted to remand NPRR207 to PRS. All Market Segments were present for the vote. On 5/6/10, TAC unanimously voted to table NPRR207. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 3/4/10, ERCOT Staff noted that the ERCOT CEO reconsidered NPRR207. With respect to the RUC Clawback portion of NPRR207, it was explained that the ERCOT CEO has not changed the prior determination of “not necessary prior to the TNMID.” With respect to the Hour Start Unit deselection portion, ERCOT Staff explained that the proposed capability is available, but that ERCOT is currently reviewing options to improve operator screens to ensure operators have the ability to efficiently carry out this task. It was requested that NPRR207 be tabled while ERCOT continues this review, that ERCOT report its findings at the next WMS and TAC meetings, and that WMS provide input to TAC regarding ERCOT’s proposal. On 4/8/10, the 4/2/10 Reliant Energy comments and 4/7/10 ERCOT comments were discussed. It was requested that PRS consider these comments and address potential conflicts between the Nodal Protocol language and North American Electric Reliability Corporation (NERC) Reliability Standards. On 5/6/10, ERCOT Staff explained that the revisions proposed in the 5/3/10 ERCOT comments were an effort to address system impacts associated with the language recommended for approval by PRS on 4/22/10. TAC requested that WMS review the 5/3/10 ERCOT comments at the next WMS meeting. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | Unknown. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | See “Overall Market Benefit” above. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Seth Cochran and Tom Jackson, co-chairs, for the Quick Start Task Force (QSTF) on behalf of WMS |
| E-mail Address | scochran@sempratrading.com; tom.jackson@austinenergy.com |
| Company | Sempra Energy Trading; Austin Energy |
| Phone Number | 203-962-5188; 512-322-6324 |
| Cell Number |  |
| Market Segment | Independent Power Marketer (IPM); Municipal |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Topaz 021110 | Expressed support for NPRR207 as submitted, requested reconsideration of the CEO determination, and suggested possible means to mitigate system impacts. |
| Reliant Energy 021510 | Proposed revisions to place the RUC Clawback modifications in the parking deck while moving forward with the deselection revisions. |
| WMS 021710 | Endorsed NPRR207 as submitted and requested review by the ERCOT CEO of whether NPRR207 as amended by the 2/11/10 Topaz comments is necessary prior to the TNMID; stated that if the ERCOT CEO still determines that NPRR207 is “not necessary prior to the TNMID,” then WMS recommends that NPRR207 be amended as recommended in the 2/15/10 Reliant Energy comments so that the proposed revisions that can be implemented prior to the TNMID can move forward through the stakeholder process. |
| WMS 033110 | Restored the RUC Clawback language in Section 5.7.2 to the current Nodal Protocol language, and noted that WMS does not endorse this language, but instead will submit a new NPPR to be titled “Half-Hour Start Unit RUC Clawback” that will contain the RUC Clawback revisions originally proposed in NPRR207. |
| Reliant Energy 040210 | Proposed revisions to expand the applicability of the deselection provision from Hour Start Units to all units. |
| ERCOT 040710 | Expressed concern regarding potential conflict with NERC Reliability Standards and proposed revisions to the 3/31/10 WMS comments expanding the applicability of the deselection process from Hour Start Units to all units and providing ERCOT with discretion regarding deselection. |
| Topaz 042010 | Responded to 4/2/10 Reliant Energy comments and 4/7/10 ERCOT comments recommending rejection of the 4/7/10 ERCOT comments. |
| Topaz 042110 | Proposed language changes to avoid uplifted RUC costs. |
| WMS 042210 | Endorsed NPRR207 as amended by the 4/21/10 Topaz comments and as revised by WMS. |
| ERCOT 050310 | Proposed revisions to address Nodal Program impacts associated with language recommended for approval by PRS on 4/22/10. |

Comments

In the original submission form and prior reports for NPRR207, a small portion of text that the sponsors proposed to add is incorrectly shown as existing baseline Nodal Protocol language.  In paragraph (2) of Section 5.7.2, the phrase “the QSE did” appears as baseline Nodal Protocol language, but should have been shown as proposed new language authored by the sponsors, “WMS/QSTF.”  This report correctly shows this language as redline text authored by the sponsors and subsequently stricken as intended in the 3/31/10 WMS comments.

Proposed Protocol Language Revision

2.1	DEFINITIONS

Reliability Unit Commitment (RUC) Notification

An ERCOT notification for specified Operating Hour(s), taking into account a Generation Resource’s start-uU time findicating that the Resource was recommended but deselected in the RUC processER

4.4.9.2	Startup Offer and Minimum-Energy Offer

The Startup Offer component represents all costs incurred by a Generation Resource in starting up and reaching its LSL, minus the average energy produced during the time period between breaker close and LSL multiplied by the heat rate proxy “H” multiplied by the appropriate Fuel Index Price (FIP) or Fuel Oil Price (FOP).  The Minimum-Energy Offer component represents a proxy for the costs incurred by a Resource in producing energy at the Resource’s LSL.

4.4.9.2.1	Startup Offer and Minimum-Energy Offer Criteria

(1)	Each Startup Offer and Minimum-Energy Offer must be reported by a QSE and must include the following information:

(a)	The selling QSE;

(b)	The Resource represented by the QSE from which the offer would be supplied;

(c)	The Resource’s hot, intermediate, and cold Startup Offer in dollars;

(d)	The Resource’s Minimum-Energy Offer in dollars per MWh;

(e)	The first and last hour of the Startup and Minimum-Energy Offers

(f)	The expiration time and date of the offer;

(g)	Percentage of the Fuel Index Price (FIP) to the extent that the startup and minimum energy will be supplied by gas to determine the offer cap; and

(h)	Percentage of the Fuel Oil Price (FOP) to the extent that the startup and minimum energy will be supplied by oil to determine the offer cap.

(2)	Valid Startup Offers and Minimum-Energy Offers (which must be part of a Three-Part Supply Offer) must be received before 1000 for the effective DAM and DRUC.

(3)	A QSE may update and submit a Three-Part Supply Offer for a Resource during the Adjustment Period for any hours in which the Resource is not RUC-committed before the offer is updated or submitted.  A QSE may not increase a Three-Part Supply Offer for a Resource during the Adjustment Period for any hour in which the Resource has received a RUC Notification.

(4)	The Resource’s Startup Offer must be equal to or less than the Resource Category Generic Startup Cost for that type of Resource listed in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, unless ERCOT has approved verifiable Resource-specific startup costs for that Resource, under Section 4.4.9.2.4, Verifiable Startup Offer and Minimum-Energy Offer Caps, in which case the Resource’s Startup Offer must be equal to or less than those approved verifiable Resource-specific startup costs.

(5)	The Resource’s Minimum-Energy Offer must be equal to or less than the Resource Category Generic Minimum-Energy Cost for that type of Resource listed in Section 4.4.9.2.3 unless ERCOT has approved verifiable Resource-specific minimum-energy costs for that Resource, under Section 4.4.9.2.4 in which case the Resource’s Minimum-Energy Offer must be equal to or less than those approved verifiable Resource-specific minimum-energy costs.

4.4.9.3	Energy Offer Curve

(1)	The “Energy Offer Curve” represents the QSE’s willingness to sell energy at or above a certain price and at a certain quantity in the DAM or its willingness to be dispatched by SCED in Real-Time Operations.

(2)	A QSE may submit Resource-specific Energy Offer Curves to ERCOT.

(3)	Energy Offer Curves remain active for the offered period until either:

(a)	Selected by ERCOT, or

(b)	Automatically inactivated by the software at the offer expiration time selected by the QSE.

(4)	For any hour that is not a RUC-Committed Interval or a RUC Notification interval or a DAM-Committed Interval for a Resource, the QSE for that Resource may submit or change Energy Offer Curves in the Adjustment Period and a QSE may withdraw an Energy Offer Curve if:

(a)	An Output Schedule is submitted for all intervals for which an Energy Offer Curve is withdrawn, or

(b) 	The Resource is forced Off-Line and notifies ERCOT of the Forced Outage by changing the Resource Status appropriately and updating its COP.

(5)	For any hour that is a RUC-Committed Interval or a DAM-Committed Interval for a Resource, a QSE for that Resource may not change an Energy Offer Curve, except as specified in (a) and (b) below:

(a)	A QSE may change the Energy Offer Curve if the Resource is required, due to external fuel curtailments, to change fuel type or source during the Adjustment Period.  ERCOT shall develop reasonable procedures for QSEs to report and document such fuel curtailments.

(b)	A QSE may change the Energy Offer Curve if the Resource suffers a partial Forced Outage by truncating the Energy Offer Curve at the Resource’s HSL as modified by the partial Forced Outage.

(6)	For any hour that is a RUC Notification interval for a Resource, a QSE for that Resource may not increase an Energy Offer Curve, except as specified in (a) and (b) below:

(a)	A QSE may change the Energy Offer Curve if the Resource is required, due to external fuel curtailments, to change fuel type or source during the Adjustment Period.  ERCOT shall develop reasonable procedures for QSEs to report and document such fuel curtailments.

(b)		A QSE may change the Energy Offer Curve if the Resource suffers a partial Forced Outage by truncating the Energy Offer Curve at the Resource’s HSL as modified by the partial Forced Outage.

(7)	If a valid Energy Offer Curve or an Output Schedule does not exist for a Resource that has a status of On-Line at the end of the Adjustment Period, then ERCOT shall notify the QSE and set the Output Schedule equal to the then current telemetered output of the Resource until an Output Schedule or Energy Offer Curve is submitted in a subsequent Adjustment Period.

5.5.2	Reliability Unit Commitment (RUC) Process

(1)	The RUC process recommends commitment of Generation Resources, to match ERCOT’s forecasted Load, subject to all transmission constraints and Resource performance characteristics.  The RUC process takes into account Resources already committed in the Current Operating Plans (COPs), Resources already committed in previous RUCs, and Resource capacity already committed to provide Ancillary Service.  The formulation of the RUC objective function must employ penalty factors on violations of security constraints. The objective of the RUC process is to minimize costs based on Three-Part Supply Offers, substituting a proxy Energy Offer Curve for the Energy Offer Curve, over the RUC Study Period.

(2)	The RUC process can recommend Resource decommitment.  ERCOT may only decommit a Resource to resolve transmission constraints that are otherwise unresolvable. Qualifying Facilities (QFs) may be decommitted only after all other types of Resources have been assessed for decommitment.  In addition, the HRUC process provides decision support to ERCOT regarding a Resource decommitment requested by a Qualified Scheduling Entity (QSE).

(3)	ERCOT shall review the RUC-recommended Resource commitments to assess feasibility and shall make any changes that it considers necessary, in its sole discretion.  ERCOT may deselect Resources recommended in DRUC and in all HRUC processes if in ERCOT’s sole discretion there is enough time to commit those Resources in the future HRUC processes, taking into account the Resources’ start-up times, to meet ERCOT System reliability and shall issue a RUC Notification to the deselected Resources.  Prior to receiving the RUC Instruction, QSEs may self-commit the Generation Resource.  ERCOT shall issue RUC Instructions to each QSE specifying its Resources that have been committed as a result of the RUC process.  ERCOT shall, within one day after making any changes to the RUC-recommended commitments, post to the MIS Secure Area any changes that ERCOT made to the RUC-recommended commitments with an explanation of the changes.

5.7.2	RUC Clawback Charge

(1)	A QSE for a Resource shall pay a RUC Clawback Charge for the Operating Day if the RUC Guarantee is less than the sum of:

(a)	RUC Minimum-Energy Revenue calculated in Section 5.7.1, RUC Make-Whole Payment;

(b)	Revenue Less Cost Above LSL During RUC-Committed Hours calculated in Section 5.7.1.3, Revenue Less Cost Above LSL During RUC-Committed Hours; and

(c)	Revenue Less Cost During QSE-Clawback Intervals calculated in Section 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals.

(2)	The amount of the RUC Clawback Charge is a percentage of the difference calculated in paragraph (1), above.  Whether or not the QSE submits a Three-Part Supply Offer for a Resource in the Day-Ahead Market (DAM) determines the clawback percentage.  If the QSE submitted a validated Three-Part Supply Offer for the Resource into the DAM, then the clawback percentage in RUC-Committed Hours is 50% and the clawback percentage in QSE Clawback Intervals is 0%.  If not, then the clawback percentage in RUC-Committed Hours is 100% and the clawback percentage in QSE Clawback Intervals is 50%.

(3)	If an Energy Emergency Alert (EEA) is in effect for any hour that a Resource is RUC-committed, then in all RUC-Committed Hours of the Operating Day the clawback percentage is 0% if the QSE submitted a validated Three-Part Supply Offer for the Resource into the DAM and 50% otherwise.

(4)	The RUC Clawback Charge for a Resource, including RMR units, for each Operating Day is allocated evenly over the RUC-Committed Hours for that Resource.

(5)	For each RUC-committed Resource, the RUC Clawback Charge for each RUC-Committed Hour of the Operating Day is calculated as follows:

If (RUCMEREVq,r,d + RUCEXRRq,r,d – RUCGq,r,d) > 0,

Then, RUCCBAMTq,r,h	=	[(RUCMEREVq,r,d + RUCEXRRq,r,d – RUCGq,r,d) * RUCCBFRq,r,d + RUCEXRQCq,r,d * RUCCBFCq,r,d]/RUCHRq,r,d

Otherwise, RUCCBAMTq,r,h = [Max (0, RUCMEREVq,r,d + RUCEXRRq,r,d + RUCEXRQCq,r,d – RUCGq,r,d) * RUCCBFCq,r,d] / RUCHRq,r,d

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCCBAMTq,r,h | $ | RUC Clawback Charge––The RUC Clawback Charge to a QSE for a Resource as described in this Section, for each RUC-Committed Hour of the Operating Day for that Resource. |
| RUCGq,r,d | $ | RUC Guarantee—The sum of the Resource’s eligible Startup Costs and Minimum-Energy Costs during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.1, RUC Guarantee. |
| RUCMEREVq,r,d | $ | RUC Minimum-Energy Revenue—The sum of the energy revenues for the Resource’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.2, RUC Minimum-Energy Revenue. |
| RUCEXRRq,r,d | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for the Resource above the LSL less the cost during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.3. |
| RUCEXRQCq,r,d | $ | Revenue Less Cost from QSE-Clawback Intervals—The sum of the profits during QSE-Clawback Intervals, for the Operating Day. See Section 5.7.1.4. |
| RUCCBFRq,r,d | none | RUC Clawback Factor for RUC-Committed Hours—The Resource’s Clawback factor for RUC-Committed Hours, which is 50% if a Three-Part Supply Offer was submitted and 100% otherwise. During EEA conditions the Resource’s clawback factor for RUC-Committed Hours is 0% if a Three-Part Supply Offer was submitted and 50% otherwise. |
| RUCCBFCq,r,d | none | RUC Clawback Factor for QSE Clawback intervals—The Resource’s Clawback factor for QSE Clawback Intervals, which is 0% if a Three-Part Supply Offer was submitted and 50% otherwise. |
| RUCHRq,r,d | none | RUC Hour—The total number of RUC-Committed Hours, for the Resource for the Operating Day. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| h | none | An hour in the RUC-commitment period. |