| RMGRR Number | 086 | RMGRR Title | Submission of Distributed Generation Data for Advanced Meters |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | July 14, 2010 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Retail Market Guide Sections Requiring Revision |  | 7.14, Out-flow Power from Distributed Renewable Generation Facilities 7.14.1, Primary Requirements for Receiving a Settlement Adjustment for Out-Flow Power 7.14.2, TDSP Communication to ERCOT and the REP of Record Of Technical Information from Distributed Renewable Generation Interconnection Agreements 7.14.3, Metering Required for Measurement and Settlement of Out- flow Power 7.14.4, Transmittal of Out-flow Power Meter Data 7.14.5, ERCOT Processing of meter data for Out-Flow Power |  |  |
| Revision Description |  | This Retail Market Guide Revision Request (RMGRR) supplements the requirements outlined in Nodal Protocol Revision Request (NPRR) 208, Registration and Settlement of Distributed Generation (DG) Less Than One MW, and describes the submittal of DG for Advanced Meters. This RMGRR also proposes changing the term “out-flow power” to “out-flow energy”. |  |  |
| Reason for Revision |  | Provides market clarity on submission of DG data based on NPRR208. |  |  |
| Overall Market Benefit |  | Defines DG submission requirements. |  |  |
| Overall Market Impact |  | Exempts DG below a DG registration threshold from registering with ERCOT and allows the excess energy flows into the ERCOT System from such DG to be treated as negative Load. |  |  |
| Consumer Impact |  | None. |  |  |
| Procedural History |  | - On 3/18/10, RMGRR086 was posted. - On 4/14/10, RMS considered RMGRR086. - On 5/12/10, RMS again considered RMGRR086. - On 7/14/10, RMS again considered RMGRR086. |  |  |
| RMS Decision |  | On 4/14/10, RMS unanimously voted to table RMGRR086. All Market Segments were present for the vote. On 5/12/10, RMS unanimously voted to table RMGRR086 until the July 14, 2010 RMS meeting. All Market Segments were present for the vote. On 7/14/10, RMS unanimously voted to recommend approval of RMGRR086 as revised by RMS. All Market Segments were present for the vote. |  |  |
| Summary of RMS Discussion |  | On 4/14/10, it was noted that NPRR208 was tabled at the 4/8/10 TAC meeting to confirm that it was consistent with P.U.C. Subst. R. 25.213, Metering for Distributed Renewable Generation. RMS agreed to table RMGRR086 for one month. On 5/12/10, the sponsor of RMGRR086 explained the reason for tabling RMGRR086 was due to TAC’s recommendation on NPRR208, to delete item (a) in Section 11.4.4.3, Load Reduction for Excess Non-PhotoVoltaic Distributed Generation. ERCOT Staff noted potential Settlement gaps and the need to check for DG registration implications. RMS agreed to table this RMGRR until the July 14, 2010 RMS meeting. On 7/14/10, ERCOT Staff elaborated on the registration requirements and the process for settling out-flow of energy for DG. RMS also proposed administrative changes. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Defines DG submission requirements |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Original Sponsor |  |
| --- | --- |
| Name | Don Tucker on behalf of the Non-Opt In Entity (NOIE) Distributed Renewable Generation (DRG) Settlements Task Force |
| E-mail Address | dtucker@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3913 |
| Cell Number | 512-560-2378 |
| Market Segment | N/A |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sonja B. Mingo |
| E-Mail Address | Smingo@ercot.com |
| Phone Number | 512-248-6463 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None. |  |

Proposed Guide Language Revision

7.14	Out-flow Energy from Distributed Generation Facilities

Retail Electric Providers (REPs), via their Qualified Scheduling Entities (QSEs), can receive a wholesale Settlement adjustment for out-flow energy, according to the process and requirements outlined below.

7.14.1	Primary Requirements for Receiving a Settlement Adjustment for Out-flow Energy

In order for ERCOT to provide the wholesale Settlement adjustment for out-flow energy; the following requirements must be met:

(1)	A current and valid Interconnection Agreement must be in place with the Transmission and/or Distribution Service Provider (TDSP), as described in P.U.C. Subst. R. 25.211, Interconnection of On-Site Distributed Generation (DG), and P.U.C. Subst. R. 25.212, Technical Requirements for Interconnection and Parallel Operation Of On-Site Distributed Generation.

(2)	Customer requested metering that measures and reports separately, consumption from the distribution network and out-flow energy from the Customer’s side of the meter to the distribution network must be installed.

(3)	For non-Interval Data Recorder (IDR) metering, both the Load and out-flow energy measured at the point of common coupling must be settled with non-IDR data and the Electric Service Identifier (ESI ID) must be assigned to a DG Load Profile (see the Load Profiling Guide for further information on Load Profile requirements).

(4)	For IDR meters, both the Load and out-flow energy measured at the point of common coupling must be settled with IDR data and the facility must be registered with ERCOT as a Generation Resource and be assigned a Resource ID (RID).  Out-flow energy associated with the RID will be settled to the QSE associated with the RID.

(5)	For Advanced Meters, both the Load and out-flow energy measured at the point of common coupling must be settled with IDR data.

For more detailed information about the resource registration process, Market Participants (MPs) should contact their designated ERCOT account representative.

7.14.2	TDSP Communication to ERCOT and the REP of Record of Technical Information from Distributed Generation Interconnection Agreements

This section applies to non-IDR metering and Advanced Meters.  In order to assign a DG Load Profile, ERCOT requires that the information below from the Interconnection Agreement between the TDSP and the retail Customer be provided.  Once the TDSP establishes an Interconnection Agreement with a DG owner, the TDSP shall provide information from the Interconnection Agreement, specified in Load Profiling Guide, Appendix D, Profile Decision Tree, to ERCOT within ten (10) Retail Business Days of the effective date of the Interconnection Agreement.

7.14.3	Metering Required for Measurement and Settlement of Out-flow Energy

In order for out-flow energy to be measured, the Premise must have metering that measures in-flow and out-flow energy of electricity separately at the point of common coupling.  Customers choosing to have their out-flow energy measured, shall contact their TDSP to request the necessary metering if they have not already done so in conjunction with their interconnection activities.  TDSP charges may apply for the cost of the metering.  See P.U.C. Subst. R. 25.213, Metering for Distributed Renewable Generation, for further details.

7.14.4	Transmittal of Out-flow Energy Meter Data

For non-IDR metering and Advanced Meters, the out-flow energy value (kWh) will be transmitted on the Texas Standard Electronic Transaction (Texas SET) 867_03, Monthly Usage, and the 867_02, Historical Usage, transactions in the REF~JH~I segment (REF = Meter Role, PTD = Non-Interval Detail).  As of January 1, 2009, the REF~JH~I (REF = Meter Role, PTD = Non-Interval Detail) segment is dedicated for out-flow energy values only.  For instances where there has been no out-flow energy, the segment should either be omitted or included and populated with zero (“0”).  In the absence of a meter that measures out-flow energy, the REF~JH~I shall not be included on the 867_02 or 867_03 transactions.

For IDR meters, interval out-flow energy values will be transmitted on the Texas SET 867_03 transaction using the RID to identify the point of generation, as described in the Texas SET Implementation Guide.

For Advanced Meters, interval out-flow energy values will also be provided in the ERCOT specified file format.

7.14.5	ERCOT Processing of Meter Data for Out-flow Energy

For non-IDR metering, ERCOT will process out-flow energy values for Settlement received for any ESI ID, provided that the ESI ID is also assigned to a DG Load Profile.  Any 867_03, Monthly Usage, received by ERCOT that contains a value for out-flow energy in the REF~JH~I segment for an ESI ID that is not assigned to a DG Load Profile will be processed but the out-flow energy value will be rejected by ERCOT.

For a detailed description of the wholesale Settlement impact of out-flow energy values, see Protocol Sections 11.4.4.2, Load Reduction for Excess PhotoVoltaic Distributed Renewable Generation, and 11.4.4.3, Load Reduction for Excess Non-PhotoVoltaic Distributed Generation.

For IDR meters, ERCOT will process out-flow energy values for Settlement received for any RID, provided that the registration process for the Resource has been completed.  The RID meter data will be processed as part of the generation aggregation and Settlement process.

For Advanced Meters, ERCOT will process out-flow energy values for Settlement received for any ESI ID, provided that the ESI ID is also assigned a DG Load Profile. Any out-flow energy received by ERCOT via the ERCOT specified file format for any ESI ID that is not assigned a DG Load Profile will be rejected by ERCOT.