| NPRR Number | 228 | NPRR Title | Resolution of Alignment Items A2, A80, A83 and A93 - As-Built Treatment and Settlement of Combined Cycle Generation Resources in ERCOT Market Systems |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | August 5, 2010 |  |  |
| Proposed Effective Date |  | Upon the Nodal Protocol Transition Plan’s Texas Nodal Market Implementation Date as prescribed by zonal Protocol Section 21.12, Process for Transition to Nodal Market Protocol Sections. |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms 3.8, Special Considerations for Split Generation Meters 3.8.1, Split Generation Resources (new) 3.8.2, Combined Cycle Generation Resources (new) 3.9.1, Current Operating Plan (COP) Criteria 3.9.2, Current Operating Plan Validation 4.5.1, DAM Clearing Process 4.6.1.4, Day-Ahead Settlement Point Prices at the Logical Resource Node for a Combined Cycle Generation Resource (new) 4.6.2.3.1, Day-Ahead Make-Whole Payment 4.6.4.1.1, Regulation Up Service Payment 4.6.4.1.2, Regulation Down Service Payment 4.6.4.1.3, Responsive Reserve Service Payment 4.6.4.1.4, Non-Spin Reserve Service Payment 5.7.1, RUC Make-Whole Payment 5.7.1.1, RUC Guarantee 5.7.1.2, RUC Minimum-Energy Revenue 5.7.1.3, RUC Revenue Less Cost Above LSL During RUC-Committed Hours 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals 5.7.2, RUC Clawback Charge 5.7.3, Payment When ERCOT Decommits a QSE-Committed Resource 5.7.4, RUC Make-Whole Charges 5.7.4.1, RUC Capacity-Short Charge 5.7.4.1.1, Capacity Shortfall Ratio Share 5.7.5, RUC Clawback Payment 5.7.6, RUC Decommitment Charge 6.5.5.2, Operational Date Requirements 6.6.1.1, Real-Time Settlement Point Price for a Resource Node 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node 6.6.5, Generation Resource Base-Point Deviation Charge 6.6.5.1, General Generation Resource Base-Point Deviation Charge 6.6.5.1.1, Base Point Deviation Charge for Over Generation 6.6.5.1.2, Base Point Deviation Charge for Under Generation 6.6.5.2, IRR Generation Resource Base-Point Deviation Charge 6.6.5.3, Generators Exempt from Deviation Charges 6.6.5.4, Base Point Deviation Payment 6.6.6.1, RMR Standby Payment 6.6.6.2, RMR Payment for Energy 6.6.6.3, RMR Adjustment Charge 6.6.6.4, RMR Charge for Unexcused Misconduct 6.6.6.5, RMR Service Charge 6.6.7.1 Voltage Support Service Payments 6.6.8.1, Black Start Capacity Payment 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT 6.7.1, Payments for Ancillary Service Capacity Sold in a Supplemental Ancillary Service Market 7.9.3.1, DAM Congestion Rent |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) revises the Nodal Protocol language to include a description of the treatment and Settlement of Combined Cycle Generation Resources in the ERCOT Day-Ahead, reliability and Real-Time systems. The design change for the Combined Cycle upgrades are described in the documents located at the following URLs: “Combined Cycle Plants”, http://nodal.ercot.com/docs/pd/ida/sig/SIG0003/sig0003_combined_cycle_plants_v5_0.doc; “Transmission Constraint Modeling for RT CCP Dispatch”, http://nodal.ercot.com/docs/pd/ida/sig/SIG0022/sig0022_trans_constraint_mod_RT_ccp_disp_v4.doc; “Combined-Cycle Unit Modeling in the Nodal Design”, http://nodal.ercot.com/docs/pd/ida/wp/ccum/ida003_combined_cycle_unit_modeling_in_the_nodal_design_v3.doc. The Combined Cycle Train design change described in the above documentation has extensive impacts on ERCOT systems and processes in the areas of Registration, Management Activities (COP and Outage Scheduler), the Day-Ahead Market (DAM), the Supplemental Ancillary Services Market (SASM) and the Reliability Unit Commitment (RUC) processes; and Settlements and Billing. This NPRR adds clarifying language to explain the “as built” system implementation of the conceptual elements of the Combined Cycle Train design change consistent with the existing Protocol treatment of all other Generation Resource types. |  |  |
| Reason for Revision |  | This NPRR aligns the Nodal Protocol language with as-built system design for Combined Cycle Generation Resources. |  |  |
| Overall Market Benefit |  | Alignment of the Nodal Protocols with the as-built Nodal system. |  |  |
| Overall Market Impact |  | Unknown. |  |  |
| Consumer Impact |  | Unknown. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR228 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 4/23/10, NPRR228, a CEO Revision Request Review, and an Impact Analysis were posted. - On 5/18/10, ERCOT comments were posted. - On 5/20/10, PRS considered NPRR228. - On 6/17/10, PRS again considered NPRR228. - On 7/22/10, PRS considered the 6/17/10 PRS Report and Impact Analysis for NPRR228. - On 8/5/10, TAC considered NPRR228. |  |  |
| PRS Decision |  | On 5/20/10, PRS unanimously voted to table NPRR228 for one month. All Market Segments were present for the vote. On 6/17/10, PRS unanimously voted to recommend approval of NPRR228 as amended by the 5/18/10 ERCOT comments as revised by PRS. All Market Segments were present for the vote. On 7/22/10, PRS voted to endorse and forward the 6/17/10 PRS Report and Impact Analysis for NPRR228 to TAC. There was one abstention from the Consumer Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 5/20/10, ERCOT Staff explained that NPRR228 reflects the as-built Nodal system and that ERCOT was not opposed to PRS tabling NPRR228 to provide interested parties additional time for review and comment. On 6/17/10, a revision was made to clarify that power augmentation methods may only be included as part of one or more of the registered Combined Cycle Generation Resource configurations. On 7/22/10, there was no discussion. |  |  |
| TAC Decision |  | On 8/5/10, TAC unanimously voted to recommend approval of NPRR228 as recommended by PRS in the 7/22/10 PRS Report and as revised by TAC. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 8/5/10, it was explained that the baseline Nodal Protocol language in Section 5.7.2 has changed due to the 8/1/10 effectiveness of NPRR222, Half-Hour Start Unit RUC Clawback (Companion to NPRR207). Revisions were made to the new baseline language in Section 5.7.2 consistent with the NPRR228 proposed revisions to that Section. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Accuracy and clarity of Nodal Protocol language. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Mark Patterson and Mandy Bauld |
| E-mail Address | mpatterson@ercot.com and abauld@ercot.com |
| Company | ERCOT |
| Phone Number | Mark Patterson 512-248-3912, Mandy Bauld 512-248-6455 |
| Cell Number |  |
| Market Segment | N/A |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 051810 | Proposed deleting requirement to develop procedure controls for the primary/alternate Combined Cycle Train generation unit selection process from paragraph (2) of Section 3.8.2. |

Comments

Please note that the following NPRRs also propose revisions to the following sections that are included within this NPRR:

- NPRR236, Resolution of Alignment Item A83 partially, A86, A87 partially, A88 partially, and A142 - Clarify Default Actions for Missing Data and Range of Valid Data Entries for Energy Offers

  - Section 3.8

  - Section 3.9.1

- NPRR245, Protocol Synchronization and As-Built Clarification for RUC Shortfall Calculation

  - Section 5.7.4.1.1

Please also note that the baseline Nodal Protocol language has been updated due to the recent implementation of the following NPRRs.  Revisions are reflected in redline with the relevant NPRR number as the author.

- NPRR214, Wind-powered Generation Resource (WGR) High Sustained Limit (HSL) Update Process (implemented June 1, 2010)

  - Section 6.5.5.2

- NPRR210, Wind Forecasting Change to P50, Synchronization with PRR841 (implemented July 1, 2010)

  - Section 3.9.1

- NPRR207, Unit Deselection (formerly “Hour Start Unit Deselection and Half Hour Start Unit RUC Clawback”) (implemented August 1, 2010)

  - Section 5.7.2

- NPRR222, Half-Hour Start Unit RUC Clawback (Companion to NPRR207) (implemented August 1, 2010)

  - Section 5.7.2

Proposed Protocol Language Revision

2.1	DEFINITIONS

Resource

The term is used to refer to both a Generation Resource and a Load Resource.  The term “Resource” used by itself in these Protocols does not include a Non-Modeled Generator.

All-Inclusive Generation Resource

A term used to refer to both a Generation Resource and a Non-Modeled Generator.

All-Inclusive Resource

A term used to refer to a Generation Resource, Load Resource and a Non-Modeled Generator.

Dynamically Scheduled Resource (DSR)

A Resource that has been designated by the QSE, and approved by ERCOT, as a DSR status-type and that follows a DSR Load.

Generation Resource

A generator capable of providing energy or Ancillary Service to the ERCOT System and is registered with ERCOT as a Generation Resource.  The term “Generation Resource” used by itself in these Protocols does not include a Non-Modeled Generator.

Black Start Resource

A Generation Resource under contract with ERCOT to provide BSS.

Combined Cycle Train

The combinations of gas turbines and steam turbines in an electric generation plant that employs more than one thermodynamic cycle.  For example, a Combine Cycle Train refers to the combination of gas turbine generators (operating on the Brayton Cycle) with turbine exhaust waste heat boilers and steam turbine generators (operating on the Rankin Cycle) for the production of electric power.  In the ERCOT market, Combined Cycle Trains are each registered as a plant that can operate as a Generation Resource in one or more Combined Cycle Generation Resource configurations.

Combined Cycle Generation Resource

A specified configuration of physical Generation Resources (gas and steam turbines), with a distinct set of operating parameters and physical constraints, in a Combined Cycle Train registered with ERCOT.

Intermittent Renewable Resource (IRR)

A Generation Resource that can only produce energy from variable, uncontrollable Resources, such as wind, solar, or run-of-the-river hydroelectricity.

Mothballed Generation Resource

A Generation Resource for which a Generation Entity has submitted a Notification of Suspension of Operations, for which ERCOT has declined to execute an RMR Agreement, and for which the Generation Entity has not announced retirement of the Generation Resource.

Split Generation Resource

A Generation Resource that has been split to function as two or more independent Generation Resources in accordance with Section 10.3.2.1, Generation Meter Splitting, and Section 3.10.7.2, Modeling of Resources and Transmission Loads.

Switchable Generation Resource

A Generation Resource that can be connected to either the ERCOT Transmission Grid or a non-ERCOT Control Area.

Wind-powered Generation Resource (WGR)

A Generation Resource that is powered by wind.

Load Resource

A Load capable of providing Ancillary Service to the ERCOT System and registered with ERCOT as a Load Resource.

Controllable Load Resource

A Load Resource capable of controllably reducing or increasing consumption under dispatch control (similar to Automatic Generation Control (AGC)) and that immediately responds proportionally to frequency changes (similar to generator governor action).

Non-Modeled Generator

A generator that is:

(a)	Capable of providing net output of energy to the ERCOT System;

(b)	Ten MW or less in size; or greater than ten MW and registered with the PUCT according to P.U.C. Subst. R. 25.109, Registration of Power Generation Companies and Self-Generators, as a self-generator; and

(c)	Registered with ERCOT as a Non-Modeled Generator, which means that the generator may not participate in the Ancillary Service or energy markets, RUC, or SCED.

2.2	ACRONYMS AND ABBREVIATIONS

HRL	High Reasonability Limit

3.8	Special Considerations for Split Generation Meters and Combined Cycle Generation Resources

3.8.1	Split Generation Resources

(1)	When a generation meter is split, as provided for in Section 10.3.2.1, Generation Meter Splitting, two or more independent Generation Resources must be created in the ERCOT Network Operations Model according to Section 3.10.7.2, Modeling of Resources and Transmission Loads, to function in all respects as individual Split Generation Resources in ERCOT System operation.  A Combined Cycle Train may not be registered in ERCOT as a Split Generation Resource.

(2)	Each Qualified Scheduling Entity (QSE) representing an individual Split Generation Resource shall collect and shall submit to ERCOT the Resource Parameters defined under Section 3.7, Resource Parameters, for the individual Split Generation Resource it represents.  The parameters provided must be consistent with the parameters submitted by each other QSE that represents a Split Generation Resource from the same generation facility.  The parameters submitted for the individual Split Generation Resource for limits and ramp rates must be according to the capability of the individual Split Generation Resource represented by each QSE.  Startup and shutdown times, time to change status and number of starts must be identical for all the individual Split Generation Resources submitted by each QSE.  ERCOT shall review data submitted by each QSE representing Split Generation Resources for consistency and notify each QSE of any errors.

(3)	Each Split Generation Resource may be represented by a different QSE.  A Split Generation Resource must comply in all respects to the requirements of a Generation Resource specified under these Protocols.

(4)	Each QSE is responsible for representing its individual Split Generation Resource in its Current Operating Plan (COP).

(5)	If an individual Split Generation Resource is On-Line, then all individual Split Generation Resources for that generation facility are considered On-Line.  Each QSE representing a Split Generation Resource shall update its individual Resource Status appropriately.

(6)	Each QSE representing an individual Split Generation Resource may independently submit Energy Offer Curves and Three Part Supply Offers.  ERCOT shall treat each Split Generation Resource offer as a separate offer, except that all individual Split Generation Resources in a generation facility must be committed or decommitted together.

(7)	Each QSE submitting verifiable cost data to ERCOT shall coordinate among all owners of a generation facility to provide individual Split Generation Resource data consistent with the total verifiable cost of the entire generation facility.  ERCOT may compare the total verifiable costs with other similarly situated Generation Resources to determine the reasonability of the cost.

3.8.2	Combined Cycle Generation Resources

(1)	ERCOT shall assign a logical Resource Node for use in the Day-Ahead Market (DAM), Reliability Unit Commitment (RUC), Supplemental Ancillary Services Market (SASM), Security-Constrained Economic Dispatch (SCED) and Load Frequency Control (LFC) to each registered Combine Cycle Train.  Each Combined Cycle Generation Resource registered in the Combine Cycle Train will be mapped to the Combined Cycle Train logical Resource Node for the purposes of evaluating and settling each Combined Cycle Generation Resource’s Three Part Supply Offer and Ancillary Service Offer in the DAM, RUC and SCED.  Each generation unit identified in the Combined Cycle Train registration for a Combined Cycle Generation Resource configuration will be mapped to its designated Resource Node as determined in accordance with these Protocols and the Technical Advisory Committee (TAC) approved ERCOT Procedure for Identifying Resource Nodes.

(2)	If any of the generation units, designated in the Combined Cycle Train registration as a primary generation unit in a Combined Cycle Generation Resource, is isolated from the ERCOT Transmission Grid because of a transmission Outage reported in the Outage Scheduler, the DAM and RUC applications shall select an alternate generation unit for use in the application.

(3)	Three-Part Supply Offers and Resource-specific Ancillary Service Offers submitted for a Combined Cycle Generation Resource may only be made at the Resource’s logical Resource Node.  ERCOT shall use the logical Resource Node to settle these offers.

(4)	In the DAM and RUC, ERCOT shall model the energy injection from each generation unit registered to the Combine Cycle Generation Resource designated in a Three Part Supply Offer as follows:

(a)	The energy injection for each generation unit registered in the Combined Cycle Generation Resource designated in a Three Part Supply Offer shall be the offered energy injection for the selected price point on the Three Part Supply Offer’s Energy Offer Curve times a weight factor as determined in paragraph (4)(b) below.

(b)	The weight factor for each generation unit registered in a Combined Cycle Generation Resource shall be the generation unit’s High Reasonability Limit (HRL), as specified in its ERCOT approved Resource Asset Registration Form, divided by the total of all HRL values for the generation units registered in the designated Combine Cycle Generation Resource.

(5)	In the Network Operations Network Models used in the DAM, RUC and SCED applications, each generation unit identified in the Combined Cycle Train registration must be modeled at its designated point of interconnection.

(6)	For Ancillary Services offered and provided from Combined Cycle Generation Resources, ERCOT shall apply, without exception, the same rules and requirements specified in these Protocols for the DAM, RUC and Adjustment Period and Real-Time markets that apply to Ancillary Services provided from any other Generation Resources.

(a)	ERCOT systems shall determine the High and Low Ancillary Service Limits (HASL and LASL) for a Combined Cycle Generation Resource as follows:

(i)	In Real Time, relative to the telemetered HSL for the Combined Cycle Generation Resource, or

(ii)	During the DAM and RUC study periods, relative to the HSL in the COP.

(b)	The QSE shall assure that the Combined Cycle Generation Resource designated as On-Line through telemetry or in the COP can meet its Ancillary Service Resource Responsibility.

3.9.1	Current Operating Plan (COP) Criteria

(1)	Each QSE that represents a Resource must submit a COP to ERCOT that reflects expected operating conditions for each Resource for each hour in the next seven Operating Days.

(2)	Each QSE that represents a Resource shall update its COP reflecting changes in availability of any Resource as soon as reasonably practicable, but in no event later than 60 minutes after the event that caused the change.

(3)	The Resource capacity in a QSE’s COP must be sufficient to supply the Ancillary Service Supply Responsibility of that QSE.

(4)	A COP must include the following for each Resource represented by the QSE:

(a)	The name of the Resource;

(b)	The expected Resource Status:

(i)	Select one of the following for Generation Resources synchronized to the ERCOT System that best describes the Resource’s status:

(A)	ONRUC – On-Line and the hour is a RUC-Committed Hour;

(B)	ONREG – On-Line Resource with Energy Offer Curve providing Regulation Service;

(C)	ON – On-Line Resource with Energy Offer Curve;

(D)	ONDSR – On-Line Dynamically Scheduled Resource (DSR);

(E)	ONOS – On-Line Resource with Output Schedule;

(F)	ONOSREG – On-Line Resource with Output Schedule providing Regulation Service;

(G)	ONDSRREG – On-Line DSR providing Regulation Service;

(H)	ONTEST – On-Line test with Output Schedule;

(I)	ONEMR – On-Line EMR (available for commitment or dispatch only for ERCOT-declared Emergency Conditions; the QSE may appropriately set LSL and High Sustained Limit (HSL) to reflect operating limits); and

(J)	ONRR – On-Line as a synchronous condenser (hydro) providing Responsive Reserve (RRS) but unavailable for Dispatch by Security-Constrained Economic Dispatch (SCED) and available for commitment by RUC;

(ii)	Select one of the following for Off-Line Generation Resources not synchronized to the ERCOT System that best describes the Resource’s status:

(A)	OUT – Off-Line and unavailable;

(B)	OFFNS – Off-Line but reserved for Non-Spinning Reserve (Non-Spin);

(C)	OFF – Off-Line but available for commitment in the Day-Ahead Market (DAM) and RUC; and

(D)	EMR – Available for commitment only for ERCOT-declared Emergency Condition events; the QSE may appropriately set LSL and High Sustained Limit (HSL) to reflect operating limits; and

(iii)	Select one of the following for Load Resources:

(A)	ONRGL – Available for Dispatch of Regulation Service;

(B)	ONRRCLR – Available for Dispatch of Responsive Reserve Service as a Controllable Load Resource;

(C)	ONRL – Available for Dispatch of RRS Service or Non-Spin, excluding Controllable Load Resources; and

(D)	OUTL – Not available;

(c)	The HSL;

(d)	The LSL;

(e)	The High Emergency Limit (HEL);

(f)	The Low Emergency Limit (LEL); and

(g)	Ancillary Service Resource Responsibility capacity in MW for:

(i)	Regulation Up (Reg-Up);

(ii)	Regulation Down (Reg-Down);

(iii)	RRS Service; and

(iv)	Non-Spin.

(5)	For combined-cycle Resources, the above items are required for each operating configuration.  In each hour only one Combined Cycle Generation Resource in a Combined Cycle Train may be assigned one of the On-Line Resource Status codes described above.

(a)	During a RUC study period, if a QSE’s COP reports multiple Combined Cycle Generation Resources in a Combined Cycle Train to be On-Line for any hour, then until the QSE corrects its COP, the On-Line Combined Cycle Generation Resource with the largest HSL is considered to be On-Line and all other Combined Cycle Generation Resources in the Combined Cycle Train are considered to be Off-Line.  Furthermore, until the QSE corrects its COP, the Off-Line Combined Cycle Generation Resources as designated through the application of this process are ineligible for RUC commitment or de-commitment Dispatch Instructions.

(b)	For any hour in which QSE-submitted COP entries are used to determine the initial state of a Combined Cycle Generation Resource for a DAM or Day-Ahead Reliability Unit Commitment (DRUC) study and the COP shows multiple Combined-Cycle Generation Resources in a Combined Cycle Train to be in an On-line Resource Status, then until the QSE corrects its COP, the On-Line Combined Cycle Generation Resource that has been On-Line for the longest time from the last recorded start by ERCOT systems, regardless of the reason for the start, combined with the COP Resource Status for the remaining hours of the current Operating Day, is considered to be On-Line at the start of the DRUC study period and all other COP-designated Combined Cycle Generation Resources in the Combined Cycle Train are considered to be Off-Line.

(c)	ERCOT systems shall allow only one Combined Cycle Generation Resource in a Combined Cycle Train to offer Off-Line Non-Spin in the DAM or Supplemental Ancillary Services Market (SASM).

(i)	If there are multiple Non-Spin offers from different Combined Cycle Generation Resources in a Combined Cycle Train, then prior to execution of the DAM, ERCOT shall select the Non-Spin offer from the Combined Cycle Generation Resource with the highest HSL for consideration in the DAM and ignore the other offers.

(ii)	Combined Cycle Generation Resources offering Off-Line Non-Spin must be able to transition from the shutdown state to the offered Combined Cycle Generation Resource On-Line state and be capable of ramping to the full amount of the Non-Spin offered.

(d)	The DAM and RUC shall honor the registered hot, intermediate or cold Startup Costs for each Combined Cycle Generation Resource registered in a Combined Cycle Train when determining the transition costs for a Combined Cycle Generation Resource.  In the DAM and RUC, the Startup Cost for a Combined Cycle Generation Resource shall be determined by the positive transition cost from the On-Line Combined Cycle Generation Resource within the Combine Cycle Train or from a shutdown condition, whichever ERCOT determines to be appropriate.

(6)	ERCOT may accept COPs only from QSEs.

(7)	A QSE representing a Wind-powered Generation Resource (WGR) must enter an HSL value that is less than or equal to the amount for that Resource from the most recent Short-Term Wind Power Forecast (STWPF) provided by ERCOT.

(8)	A QSE representing a Resource that has a Resource Status of ONTEST must self-commit the Resource and must submit an Output Schedule for the Resource.

3.9.2	Current Operating Plan Validation

(1)	ERCOT shall verify that each COP, on its submission, complies with the criteria described in Section 3.9.1, Current Operating Plan (COP) Criteria.  ERCOT shall notify the QSE by means of the Messaging System if the QSE’s COP fails to comply with the criteria described in Section 3.9.1 and this Section 3.9.2 for any reason.  The QSE must then resubmit the COP within the appropriate market timeline.

(2)	ERCOT may reject a COP that does not meet the criteria described in Section 3.9.1.

(3)	If a Resource is designated in the COP to provide Ancillary Service, then ERCOT shall verify that the COP complies with Section 3.16, Standards for Determining Ancillary Service Quantities.  The Ancillary Service Supply Responsibilities as indicated in the Ancillary Service Resource Responsibility submitted immediately before the end of the Adjustment Period are physically binding commitments for each QSE for the corresponding Operating Period.

(4)	ERCOT shall notify the QSE if the sum of the Ancillary Service capacity designated in the COP for each hour, by service type, is less than the QSE’s Ancillary Service Supply Responsibility for each service type for that hour.  If the QSE does not correct the deficiency within one hour after receiving the notice from ERCOT, then ERCOT shall follow the procedures outlined in Section 6.4.8.1, Evaluation and Maintenance of Ancillary Service Capacity Sufficiency.

(5)	A QSE may change Ancillary Service Resource designations by changing its COP, subject to Section 6.4.8.1.

(6)	If ERCOT determines that it needs more Ancillary Service during the Adjustment Period, then the QSE’s allocated portion of the additional Ancillary Service may be self-arranged.

(7)	ERCOT systems must be able to detect a change in status of a Resource shown in the COP and must provide notice to ERCOT operators of changes that a QSE makes to its COP.

(8)	A QSE representing a Resource that has an Energy Offer Curve valid for an hour of the COP may not designate a Resource Status of ONTEST, ONOS or ONDSR for that hour for that Resource.

4.5.1	DAM Clearing Process

(1)	At 1000 in the Day-Ahead, ERCOT shall start the Day-Ahead Market (DAM) clearing process.

(2)	Prior to execution of the DAM, ERCOT shall complete a Day-Ahead Simultaneous Feasibility Test.  This test uses the Day-Ahead Updated Network Model topology and evaluates all Congestion Revenue Rights (CRRs) for feasibility to determine hourly oversold quantities.

(3)	The purpose of the DAM is to economically and simultaneously clear offers and bids described in Section 4.4, Inputs into DAM and Other Trades.

(4)	The DAM uses a multi-hour mixed integer programming algorithm to maximize bid-based revenues minus the offer-based costs over the Operating Day, subject to security and other constraints, and ERCOT Ancillary Service procurement requirements.

- The bid-based revenues include revenues from DAM Energy Bids and Point-to-Point (PTP) Obligation Bids.

(b)	The offer-based costs include costs from the Startup Offer, Minimum Energy Offer, and Energy Offer Curve of any Resource that submitted a Three-Part Supply Offer, DAM Energy-Only Offers, CRR Offers, and Ancillary Service Offers.

(c)	Security constraints specified to prevent DAM solutions that would overload the elements of the ERCOT Transmission Grid include the following:

(i)	Transmission constraints – Transfer limits on energy flows through the ERCOT Transmission Grid, e.g., thermal or stability limits.  These limits must be satisfied by the intact network and for certain specified contingencies.

These constraints may represent:

(A)	Thermal constraints – protect transmission facilities against thermal overload.

(B)	Generic constraints – protect the ERCOT Transmission Grid against transient instability, dynamic stability or voltage collapse.

(C)	Power flow constraints – the energy balance at required Electrical Buses in the ERCOT Transmission Grid must be maintained.

(ii)	Resource constraints – the physical and security limits on Resources that submit Three-Part Supply Offers:

(A)	Resource output constraints – the Low Sustained Limit (LSL) and High Sustained Limit (HSL) of each Resource; and

(B)	Resource operational constraints – includes minimum run time, minimum down time, and configuration constraints.

(iii)	Other constraints –

(A)	Linked offers –the DAM may not select any one part of that Resource capacity to provide more than one Ancillary Service or to provide both energy and an Ancillary Service in the same Operating Hour.  The DAM may, however, select part of that Resource capacity to provide one Ancillary Service and another part of that capacity to provide a different Ancillary Service or energy in the same Operating Hour, provided that a Generation Resource may not offer, and the DAM may not select, linked Energy and Off-Line Non-Spin Ancillary Service offers in the same Operating Hour.

(B)	The sum of the awarded Ancillary Service capacities for each Resource must be within the Resource limits specified in COP and Section 3.18, Resource Limits in Providing Ancillary Service, and the Resource parameters as described in Section 3.7, Resource Parameters.

(C)	Block Ancillary Service Offers for a Load Resource – blocks will not be cleared unless the entire quantity block can be awarded.

(D)	Block CRR Offers and PTP Obligation Bids – blocks will not be cleared unless the entire time block can be awarded.

(E)	Combined Cycle Generation Resources - The DAM may commit a Combined Cycle Generation Resource in a time period that includes the last hour of the Operating Day only if that Combine Cycle Generation Resource can be shutdown.

(d)	Ancillary Service needs for each Ancillary Service include the needs specified in the Ancillary Service Plan that are not part of the Self-Arranged Ancillary Service Quantity and that must be met from available DAM Ancillary Service Offers while co-optimizing with DAM Energy Offers.  ERCOT may not buy more of one Ancillary Service in place of the quantity of a different service.  See Section 4.5.2, Ancillary Service Insufficiency, for what happens if insufficient Ancillary Service Offers are received in the DAM.

[NPRR131: Replace paragraph (4)(d) above with the following upon system implementation:] (d) Ancillary Service needs for each Ancillary Service include the needs specified in the Ancillary Service Plan that are not part of the Self-Arranged Ancillary Service Quantity plus the quantity of Ancillary Service Trades with ERCOT, and that must be met from available DAM Ancillary Service Offers while co-optimizing with DAM Energy Offers. ERCOT may not buy more of one Ancillary Service in place of the quantity of a different service. See Section 4.5.2, Ancillary Service Insufficiency, for what happens if insufficient Ancillary Service Offers are received in the DAM.

(5)	ERCOT shall determine the appropriate Load distributions to allocate offers, bids, and source and sink of CRRs at a Load Zone across the Electrical Buses that are modeled with Load in that Load Zone. The default distribution is the State Estimator hourly distribution for the seven days before the Operating Day.  If ERCOT decides, in its sole discretion, to change this distribution for reasons such as anticipated weather events or holidays, ERCOT shall select a State Estimator distribution from a proxy day reasonably reflecting the anticipated distribution in the Operating Day.  ERCOT may also modify this distribution to account for predicted differences in network topology between the proxy day and Operating Day.  ERCOT shall develop a methodology, subject to Technical Advisory Committee (TAC) approval to describe the modification of the proxy day bus-load distribution for this purpose.

(6)	ERCOT shall allocate offers, bids, and source and sink of CRRs at a Hub using the distribution factors specified in the definition of that Hub in Section 3.5.2, Hub Definitions.

(7)	A Resource that has a Three-Part Supply Offer cleared in the DAM may be eligible for make whole payment of the Startup Offer and Minimum Energy Offer submitted by the Qualified Scheduling Entity (QSE) representing the Resource under Section 4.6, DAM Settlement.

(8)	The directional network element flows for PTP Options declared for settlement in Real-Time must be properly accounted for in determining available transmission network capacity in the DAM.  In the event the available transmission capability in the DAM cannot accommodate all PTP Options declared for settlement in Real-Time, any PTP Option declared for settlement in Real-Time that impacts overloaded directional network elements must be appropriately derated for DAM modeling purposes only, in proportion to that impact.  The derated MW of PTP Options declared for settlement in Real-Time will be settled in the DAM if their Minimum Reservation Prices are less than or equal to the DAM prices for corresponding PTP Options. Otherwise, the derated MW will be settled in Real-Time.

(9)	The DAM settlement is based on hourly MW awards and on Day-Ahead hourly Settlement Point Prices.  All PTP Options settled in the DAM are settled based on the Day-Ahead Settlement Point Prices.

(10)	The Day-Ahead Market Clearing Price for Capacity (MCPC) for each hour for each Ancillary Service is the Shadow Price for that Ancillary Service for the hour as determined by the DAM algorithm.

(11)	If the Day-Ahead MCPC cannot be calculated by ERCOT, the Day-Ahead MCPC for the particular Ancillary Service is equal to the Day-Ahead MCPC for that Ancillary Service in the same Settlement Interval of the preceding Operating Day.

(12)	If the Day-Ahead Settlement Point Prices cannot be calculated by ERCOT, all CRRs shall be settled based on Real-Time Prices.  Settlements for all CRRs shall be reflected on the RT Settlement Statement.

4.6.1.4	Day-Ahead Settlement Point Prices at the Logical Resource Node for a Combined Cycle Generation Resource

ERCOT shall calculate the Day-Ahead Settlement Point Price for each hour at the logical Resource Node for the Combined Cycle Generation Resource as follows:

(a)	The Day-Ahead Settlement Point Price at a logical Resource Node shall be the sum of a weight factor as determined in paragraph (b) below times the Day-Ahead LMP at each of the Resource Nodes of the generation units registered in the Combined Cycle Train registration for the Combined Cycle Generation Resource designated in the Three Part Supply Offer:

Where:

DASPP = ∑CCGR_PhyR DALMP CCGR_PhyR * DACCGRWF CCGR_PhyR

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP | $/MWh | Day-Ahead Settlement Point Price at a logical Resource Node for a Combined Cycle Train—The DAM Settlement Point Price at the logical Resource Node for a Combined Cycle Generation Resource for the hour. |
| DALMPCCGR_PhyR | $/MWh | Day-Ahead Locational Marginal Price at a Resource Node for a generation unit registered in the Combined Cycle Generation Resource—The Day-Ahead LMP at the Resource Node of a generation unit designated in the Combine Cycle Train registration for the Combined Cycle Generation Resource. |
| DACCGRWFCCGR_PhyR | none | Day-Ahead Combined Cycle Generation Resource Weighting Factor—The DAM Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the Combined Cycle Generation Resource. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train for the Combined Cycle Generation Resource. |

(b)	The weight factor for each generation unit designated in the Combine Cycle Train registration for the Combined Cycle Generation Resource shall be the generation unit’s High Reasonability Limit (HRL), as specified in its ERCOT approved Resource Asset Registration Form, divided by the total of all HRL values for the generation units designated in the Combine Cycle Generation Resource registration.

Where:

DACCGRWF CCGR_PhyR = HRL CCGR_PhyR / ∑CCGR_PhyR HRL CCGR_PhyR

The above variables are defined as follows:

| ###### Variable | ###### Unit | ###### Definition |
| --- | --- | --- |
| DACCGRWFCCGR_PhyR | none | Day-Ahead Combined Cycle Generation Resource Weighting Factor—The DAM Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the Combined Cycle Generation Resource. |
| HRLCCGR_PhyR | MW | High Reasonability Limit—The HRL as specified in the ERCOT-approved Resource Asset Registration Form for a generation unit designated in a Combined Cycle Train registration for the Combined Cycle Generation Resource. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train for the Combined Cycle Generation Resource. |

4.6.2.3.1	Day-Ahead Make-Whole Payment

(1)	ERCOT shall pay the QSE a Day-Ahead Make-Whole Payment for an eligible Resource, except that the Day-Ahead Make-Whole RMR Revenue amount is calculated but not paid for any RMR Unit, for each Operating Hour in a DAM-commitment period.

(2)	Any Ancillary Service Offer cleared for the same Operating Hour, QSE, and Generation Resource as a Three-Part Supply Offer cleared in the DAM shall be included in the calculation of the Day-Ahead Make-Whole Payment.

(3)	The guaranteed cost, energy revenue, and Ancillary Service revenue calculated for each Combined Cycle Generation Resource are each summed for the Combined Cycle Train, and the the Day-Ahead Make-Whole Amount is calculated for the Combined Cycle Train.

(4)	The Day-Ahead Make-Whole Payment to each QSE for each DAM-committed Generation Resource (excluding RMR Units) is calculated as follows:

DAMWAMT q, p, r, h	=	(-1) * Max (0, DAMGCOST q, p, r + DAEREV q, p, r, h

+ DAASREV q, r, h) * DAESR q, p, r, h / (DAESR q, p, r, h)

(5)	The Day-Ahead Make-Whole RMR Revenue to each QSE for each DAM-committed RMR Unit is calculated as follows:

DAMWRMRREV q, p,r, h  = (-1) * Max (0, DAMGCOST q, p, r +DAEREV q, p, r, h

+ DAASREV q, r, h) * DAESR q, p, r, h / (DAESR q, p, r, h)

(6)	The Day-Ahead Make-Whole Guaranteed Costs are calculated for each eligible DAM-Committed Generation Resource (including RMR Units) as follows:

For non-Combined Cycle Trains,

DAMGCOST q, p, r	=	DASUO q, p, r + (DAMEO q, p, r, h * DALSL q, p, r, h)

+ (DAAIEC q, p, r, h * (DAESR q, p, r, h – DALSL q, p, r, h))

For Combined Cycle Trains,

DAMGCOST q, p, r	=	DASUO q, p, r + (DAMEO q, p, r, h * DALSL q, p, r, h)

+ (Max (0, DASUO afterCCGR - DASUO beforeCCGR))

+ (DAAIEC q, p, r, h * (DAESR q, p, r, h – DALSL q, p, r, h))

(7)	The Day-Ahead Make-Whole Revenue is calculated for each DAM-Committed Generation Resource (including RMR Units) as follows:

DAEREV q, p, r, h	=	(-1) * DASPP p, h * DAESR q, p, r, h

DAASREV q, r, h 	=	((-1) * MCPCRU DAM, h * PCRUR r, q, DAM, h) +

((-1) * MCPCRD DAM, h  * PCRDR r, q, DAM, h) +

((-1) * MCPCRR DAM, h  * PCRRR r, q, DAM, h) +

((-1) * MCPCNS DAM, h  * PCNSR r, q, DAM, h)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAMWAMT q, p, r, h | $ | Day-Ahead Make-Whole Payment per QSE per Settlement Point per Resource per hourThe payment to QSE q to make-whole the Startup Cost and energy cost of Resource r committed in the DAM at Resource Node p for the hour h. When a Combined Cycle Generation Resource is committed in the DAM, payment is made to the Combined Cycle Train for the DAM-committed Combined Cycle Generation Resource. |
| DAMWRMRREV q, p, r, h | $ | Day-Ahead Make-Whole RMR Revenue per QSE per Settlement Point per RMR Resource per hourThe revenue calculated but not paid to QSE q to make-whole the Startup Cost and energy cost of the RMR Resource r committed in the DAM at Resource Node p for the hour h. When a Combined Cycle Generation Resource that is an RMR Resource is committed in the DAM, revenue is calculated for the Combined Cycle Train for the Combined Cycle Generation Resource. |
| DAMGCOST q, p, r | $ | Day-Ahead Market Guaranteed Amount per QSE per Settlement Point per ResourceThe sum of the Startup Cost and the operating energy costs of the DAM-committed Resource r at Resource Node p represented by QSE q, for the DAM-commitment period. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DAEREV q, p, r, h | $ | Day-Ahead Energy Revenue per QSE per Settlement Point per Resource by hourThe revenue received in the DAM for Resource r at Resource Node p represented by QSE q, based on the DAM Settlement Point Price, for the hour h. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DAASREV q, r, h | $ | Day-Ahead Ancillary Service Revenue per QSE per Resource by hourThe revenue received in the DAM for Resource r represented by QSE q, based on the Market Clearing Price for Capacity (MCPC) for each Ancillary Service in the DAM, for the hour h. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DASPP p, h | $/MWh | Day-Ahead Settlement Point Price by Settlement Point by hourThe DAM Settlement Point Price at Resource Node p for the hour h. |
| DAESR q, p, r, h | MW | Day-Ahead Energy Sale from Resource per QSE by Settlement Point per Resource by hourThe amount of energy cleared through Three-Part Supply Offers in the DAM for Resource r at Resource Node p represented by QSE q for the hour h. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| PCRUR r, q, DAM, h | MW | Procured Capacity for Reg-Up from Resource per Resource per QSE per hour in DAM—The Regulation Up (Reg-Up) capacity quantity awarded to QSE q in the DAM for Resource r for the hour h. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MCPCRU DAM, h | $/MW per hour | Market Clearing Price for Capacity for Reg-Up per hour in DAM—The DAM MCPC for Reg-Up for the hour h. |
| PCRDR r, q, DAM, h | MW | Procured Capacity for Reg-Down from Resource per Resource per QSE per hour in DAM—The Regulation Down (Reg-Down) capacity quantity awarded to QSE q in the DAM for Resource r for the hour h. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MCPCRD DAM, h | $/MW per hour | Market Clearing Price for Capacity for Reg-Down per hour in DAM—The DAM MCPC for Reg-Down for the hour h. |
| PCRRR r, q, DAM, h | MW | Procured Capacity for Responsive Reserve from Resource per Resource per QSE per hour in DAM—The Responsive Reserve capacity quantity awarded to QSE q in the DAM for Resource r for the hour h. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MCPCRR DAM, h | $/MW per hour | Market Clearing Price for Capacity for Responsive Reserve per hour in DAM—The DAM MCPC for Responsive Reserve for the hour h. |
| PCNSR r, q, DAM, h | MW | Procured Capacity for Non-Spin from Resource per Resource per QSE per hour in DAM—The Non-Spin capacity quantity awarded to QSE q in the DAM for Resource r for the hour h. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MCPCNS DAM, h | $/MW per hour | Market Clearing Price for Capacity for Non-Spin per hour in DAM—The DAM MCPC for Non-Spin for the hour h. |
| DASUO q, p, r | $/start | Day-Ahead Startup Offer per QSE per Settlement Point per Resource—The Startup Offer included in the Three-Part Supply Offer submitted in the DAM associated with Resource r at Resource Node p represented by QSE q, for the first hour of the DAM-commitment period. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DAMEO q, p, r, h | $/MWh | Day-Ahead Minimum-Energy Offer per QSE per Settlement Point per Resource per hour—The Minimum-Energy Offer included in the Three-Part Supply Offer submitted in the DAM associated with Resource r at Resource Node p represented by QSE q, for the hour h. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DALSL q, p, r, h | MW | Day-Ahead Low Sustained Limit per QSE per Settlement Point per Resource per hourThe Low Sustained Limit (LSL) of Resource r at Resource Node p represented by QSE q, for the hour h as seen in the 1000 Day-Ahead snapshot. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DAAIEC q, p, r, h | $/MWh | Day-Ahead Average Incremental Energy Cost per QSE per Settlement Point per Resource per hourThe average incremental energy cost, calculated according to the Energy Offer Curve capped by the generic energy price, for the output levels between the DAESR and the LSL of Resource r at Resource Node p represented by QSE q, for the hour h. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| Q | none | A QSE. |
| P | none | A Resource Node Settlement Point. |
| R | none | A DAM-committed Generation Resource. |
| H | none | An hour in the DAM-commitment period. |
| - afterCCGR | none | The Combined Cycle Generation Resource to which a Combined Cycle Train transitions. |
| - beforeCCGR | none | The Combined Cycle Generation Resource from which a Combined Cycle Train transitions. |

(8)	The calculation of the Day-Ahead Average Incremental Energy Cost for each Resource for each hour is illustrated with the picture below, where Pcap is the Energy Offer Curve Cap.  The method to calculate such cost is described in Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC).

$/

MWh

DASPP

Pcap

P3

P2

P1

Q (P1) 	Q (P2)	   Q (P3)	Q (Pcap)	        Q cleared	MW

[LSL]				     [DAESR]

Energy Offer Curve

The area under the capped Energy Offer Curve equals (DAAIEC * (DAESR – LSL))

(9)	The total of the Day-Ahead Make-Whole Payments to each QSE for non-RMR Generation Resources for a given hour is calculated as follows:

DAMWAMTQSETOT q	=	ΣpΣrDAMWAMT q, p, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAMWAMTQSETOT q | $ | Day-Ahead Make-Whole Payment QSE Total per QSEThe total of the Day-Ahead Make-Whole Payments to QSE q for the DAM-committed non-RMR Generation Resources represented by this QSE for the hour. |
| DAMWAMT q, p, r | $ | Day-Ahead Make-Whole Payment per QSE per Settlement Point per ResourceThe payment to QSE q to make-whole the Startup Cost and energy cost of Resource r committed in the DAM at Resource Node p for the hour. When a Combined Cycle Generation Resource is committed in the DAM, payment is made to the Combined Cycle Train for the DAM-committed Combined Cycle Generation Resource. |
| Q | none | A QSE. |
| P | none | A Settlement Point. |
| R | none | A DAM-committed non-RMR Generation Resource. |

Q (P1) 	Q (P2)	   Q (P3)	Q (Pcap)	        Q cleared	MW

[LSL]				     [DAESR]

Energy Offer Curve

The area under the capped Energy Offer Curve equals (DAAIEC * (DAESR – LSL))

(10)	The total of the Day-Ahead Make-Whole RMR Revenue for each QSE for RMR Units for a given hour is calculated as follows:

DAMWRMRREVQSETOT q	=	ΣpΣrDAMWRMRREV q, p, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DAMWRMRREVQSETOT q | $ | Day-Ahead Make-Whole RMR Revenue QSE Total per QSEThe total of the Day-Ahead Make-Whole Revenue calculated for QSE q for DAM-committed RMR Units represented by this QSE for the hour. |
| DAMWRMRREV q, p, r, | $ | Day-Ahead Make-Whole RMR Revenue per QSE per Settlement Point per RMR Resource per hour—The revenue calculated but not paid to QSE q to make-whole the Startup Cost and energy cost of the RMR Resource r committed in the DAM at Resource Node p for the hour. When a Combined Cycle Generation Resource that is an RMR Resource is committed in the DAM, revenue is calculated for the Combined Cycle Train for the Combined Cycle Generation Resource. |
| Q | none | A QSE. |
| P | none | A Settlement Point. |
| r | none | A DAM-committed RMR Unit. |

4.6.4.1	Payments for Ancillary Services Procured in the DAM

4.6.4.1.1	Regulation Up Service Payment

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Reg-Up to ERCOT were cleared in the DAM, for each hour as follows:

PCRUAMT q	=	(-1) * MCPCRU DAM * PCRU q

Where:

PCRU q	=	ΣrPCRUR r, q, DAM

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRUAMT q | $ | Procured Capacity for Reg-Up Amount per QSE in DAM—The DAM Reg-Up payment for QSE q for the hour. |
| PCRU q | MW | Procured Capacity for Reg-Up per QSE in DAM—The total Reg-Up Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by this QSE for the hour. |
| PCRUR r, q, DAM | MW | Procured Capacity for Reg-Up from Resource per Resource per QSE in DAM—The Reg-Up capacity quantity awarded to QSE q in the DAM for Resource r for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MCPCRU DAM | $/MW per hour | Market Clearing Price for Capacity for Reg-Up in DAM—The DAM MCPC for Reg-Up for the hour. |
| R | none | A Resource. |
| Q | none | A QSE. |

4.6.4.1.2	Regulation Down Service Payment

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Reg-Down to ERCOT were cleared in the DAM, for each hour as follows:

PCRDAMT q	=	(-1) * MCPCRD DAM * PCRD q

Where:

PCRD q	=	ΣrPCRDR r, q, DAM

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRDAMT q | $ | Procured Capacity for Reg-Down Amount per QSE in DAM—The DAM Reg-Down payment for QSE q for the hour. |
| PCRD q | MW | Procured Capacity for Reg-Down per QSE in DAM—The total Reg-Down Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by this QSE for the hour. |
| PCRDR r, q, DAM | MW | Procured Capacity for Reg-Down from Resource per Resource per QSE in DAM—The Reg-Down capacity quantity awarded to QSE q in the DAM for Resource r for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MCPCRD DAM | $/MW per hour | Market Clearing Price for Capacity for Reg-Down in DAM—The DAM MCPC for Reg-Down for the hour. |
| R | none | A Resource. |
| Q | none | A QSE. |

4.6.4.1.3	Responsive Reserve Service Payment

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Responsive Reserve to ERCOT were cleared in the DAM, for each hour as follows:

PCRRAMT q	=	(-1) * MCPCRR DAM * PCRR q

Where:

PCRR q	=	ΣrPCRRR r, q, DAM

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRRAMT q | $ | Procured Capacity for Responsive Reserve Amount per QSE in DAM—The DAM Responsive Reserve payment for QSE q for the hour. |
| PCRR q | MW | Procured Capacity for Responsive Reserve per QSE in DAM—The total Responsive Reserve Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by this QSE for the hour. |
| PCRRR r, q, DAM | MW | Procured Capacity for Responsive Reserve from Resource per Resource per QSE in DAM—The Responsive Reserve capacity quantity awarded to QSE q in the DAM for Resource r for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MCPCRR DAM | $/MW per hour | Market Clearing Price for Capacity for Responsive Reserve in DAM—The DAM MCPC for Responsive Reserve for the hour. |
| R | none | A Resource. |
| Q | none | A QSE. |

4.6.4.1.4	Non-Spinning Reserve Service Payment

ERCOT shall pay each QSE whose Ancillary Service Offers to provide Non-Spin to ERCOT were cleared in the DAM, for each hour as follows:

PCNSAMT q	=	(-1) * MCPCNS DAM * PCNS q

Where:

PCNS q	=	ΣrPCNSR r, q, DAM

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCNSAMT q | $ | Procured Capacity for Non-Spin Amount per QSE in DAM—The DAM Non-Spin payment for QSE q for the hour. |
| PCNS q | MW | Procured Capacity for Non-Spin per QSE in DAM—The total Non-Spin Service capacity quantity awarded to QSE q in the DAM for all the Resources represented by this QSE for the hour. |
| PCNSR r, q, DAM | MW | Procured Capacity for Non-Spin from Resource per Resource per QSE in DAM—The Non-Spin capacity quantity awarded to QSE q in the DAM for Resource r for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MCPCNS DAM | $/MW per hour | Market Clearing Price for Capacity for Non-Spin in DAM—The DAM MCPC for Non-Spin for the hour. |
| R | none | A Resource. |
| Q | none | A QSE. |

5.7	Settlement for RUC Process

5.7.1	RUC Make-Whole Payment

(1)	To make up the difference when the revenues that a Reliability Unit Commitment (RUC)-committed Resource receives are less than its costs as described in paragraph (2) below, ERCOT shall calculate a RUC Make-Whole Payment for that Operating Day for that Resource (whether committed by Day-Ahead RUC (DRUC) or Hourly RUC (HRUC)).

(2)	ERCOT shall pay to the QSE for the Resource a Make-Whole Payment if the RUC Guarantee calculated in Section 5.7.1.1, RUC Guarantee, is greater than the sum of:

(a)	RUC Minimum-Energy Revenue calculated in Section 5.7.1.2, RUC Minimum-Energy Revenue;

(b)	Revenue less cost above Low Sustained Limit (LSL) during RUC-Committed Hours calculated in Section 5.7.1.3, Revenue Less Cost Above LSL During RUC-Committed Hours; and

(c)	Revenue less cost during Qualified Scheduling Entity (QSE) Clawback Intervals calculated in Section 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals.

(3)	The RUC Make-Whole Payment to the QSE for each RUC-committed Resource, including Reliability Must-Run (RMR) Units, for each RUC-Committed Hour in an Operating Day is calculated as follows:

RUCMWAMT q, r, h	=	(-1) * Max (0, RUCG q ,r, d – RUCMEREVq, r, d – RUCEXRR q, r, d  – RUCEXRQC q, r, d) / RUCHR q, r, d

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCMWAMT q, r, h | $ | RUC Make-Whole Payment—The RUC Make-Whole Payment to the QSE for Resource r, for each RUC-Committed Hour of the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, payment is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCG q, r, d | $ | RUC Guarantee—The sum of eligible Startup Costs and minimum-energy costs for Resource r during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.1. When one or more Combined Cycle Generation Resources are committed by RUC, guaranteed costs are calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCMEREV q, r, d | $ | RUC Minimum-Energy Revenue—The sum of the energy revenues for Resource r’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.2. When one or more Combined Cycle Generation Resources are committed by RUC, minimum-energy revenue is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCEXRR q, r, d | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for Resource r operating above its LSL less the cost during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.3. When one or more Combined Cycle Generation Resources are committed by RUC, revenue less cost above LSL is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCEXRQC q, r, d | $ | Revenue Less Cost During QSE Clawback Intervals—The sum of the total revenue for Resource r less the cost during all QSE Clawback Intervals, for the Operating Day. See Section 5.7.1.4. When one or more Combined Cycle Generation Resources are committed by RUC, revenue less cost during QSE Clawback Intervals is calculated for the Combined Cycle Train for all Combined Cycle Generation Resources earning revenue in QSE Clawback Intervals. |
| RUCHR q, r, d | None | RUC Hour—The total number of RUC-Committed Hours, for Resource r for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, the total number of RUC-Committed Hours is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| Q | None | A QSE. |
| R | None | A RUC-committed Generation Resource. |
| D | None | An Operating Day containing the RUC-commitment. |
| H | None | An hour in the RUC-commitment period. |

5.7.1.1	RUC Guarantee

(1)	The allowable Startup Costs and minimum-energy costs of a Resource committed by RUC is the RUC Guarantee.  The RUC Guarantee minimum-energy costs are prorated according to the actual generation when the Resource’s average output during a 15-minute Settlement Interval is below the corresponding LSL.

(2)	The SUPR, MEPR and LSL used to calculate the RUC Guarantee for a Combined Cycle Train are the SUPR, MEPR and LSL that correspond to the Combined Cycle Generation Resource, within the Combined Cycle Train, that is RUC-committed for the hour.

(3)	The RUC Guarantee is calculated for non-Combined Cycle Trains as follows:

RUCG q, r, d	=	Σs(SUPR q ,r ,s * RUCSUFLAG q, r, s) +

Σi(MEPR q, r, i * Min ((LSL q, r, i * (¼)), RTMG q, r, i))

(4)	The RUC Guarantee is calculated for Combined Cycle Trains as follows:

RUCG q, r, d	=	(SUPR q, r, s * RUCSUFLAG q, r, s) +

Σt(MAX (0, SUPR - SUPR)) +

Σi(MEPR q, r, i * Min ((LSL q, r, i * (¼)), RTMG q, r, i))

(a)	If a Combined Cycle Train transitions to a RUC-committed configuration from a QSE-committed or other RUC-committed configuration, the transition is calculated as follows:

MAX (0, SUPR afterCCGR – SUPR beforeCCGR)

(b)	If a Combined Cycle Train transitions to a QSE-committed configuration from a RUC-committed configuration, the transition is calculated as follows:

MAX (0, SUPR beforeCCGR – SUPR afterCCGR)

(5)	If a validated Three-Part Supply Offer has been submitted for a Resource for the RUC, then the RUC Guarantee for that Resource is based on the Startup Offer and Minimum-Energy Offer in that validated Three-Part Supply Offer.  If a validated Three-Part Supply Offer has not been submitted for a Resource for the RUC and ERCOT has not yet approved verifiable unit-specific costs for the Resource, then the RUC Guarantee for a Resource is based on the Resource Category Startup Generic Cap and the Resource Category Minimum-Energy Generic Cap.  If a validated Three-Part Supply Offer has not been submitted for a Resource for the RUC and ERCOT has approved verifiable unit-specific costs for the Resource, then the RUC Guarantee for a Resource is based on the most recent ERCOT-approved verifiable unit-specific costs for that Resource.

(a)	If the QSE submitted a validated Three-Part Supply Offer,

Then, 		SUPR q, r, s	=	SUO q, r, s

MEPR q, r, i	=	MEO q, r, i

Otherwise, 	SUPR q, r, s 	= 	SUCAPq ,r ,s

MEPR q, r, i 	= 	MECAP q, r, i

(b)	If ERCOT has approved verifiable Startup Costs and minimum-energy costs for the Resource,

Then, 		SUCAP q, r, s	=	verifiable Startup Costs q, r, s

MECAP q, r, i	=	verifiable minimum-energy costs q, r, i

Otherwise, 	SUCAP q, r, s 	= 	RCGSC s

MECAP q, r, i	= 	RCGMEC i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCG q, r, d | $ | RUC Guarantee—The sum of eligible Startup Costs and minimum-energy costs for Resource r during all RUC-Committed Hours, for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, guaranteed costs are calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| SUPR q, r, s | $/Start | Startup Price per start—The settlement price for Resource r, for the start s. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUO q, r, s | $/Start | Startup Offer per start—Represents an offer for all costs incurred by Generation Resource r in starting up and reaching the Resource’s LSL, minus the average energy produced during the time period between breaker close and LSL multiplied by the heat rate proxy multiplied by the appropriate Fuel Index Price (FIP) or Fuel Oil Price (FOP), as described in the Verifiable Cost Manual. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUCAP q, r, s | $/Start | Startup Cap—The amount used for Resource r as Startup Costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGSC unless ERCOT has approved verifiable unit-specific Startup Costs for that Resource, in which case the startup cap is the verifiable unit-specific Startup Cost. See Section 5.6.1, Verifiable Costs, for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGSC s | $/Start | Resource Category Generic Startup Cost—The Resource Category Generic Startup Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| RUCSUFLAG q, r, s | none | RUC Startup Flag—The flag that indicates whether or not the start s for Resource r is eligible for RUC Make-Whole Payment. Its value is one if eligible; otherwise, zero <0See Section 5.6.2, RUC Startup Cost Eligibility, and Section 5.6.3, Forced Outage of RUC-Committed Resource, for more information on startup eligibility. For a Combined Cycle Train, the Resource r must be one of the registered Combined Cycle Generation Resources within the Combined Cycle Train. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Startup Flag is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| MEPR q, r, i | $/MWh | Minimum-Energy Price—The settlement price for Resource r for minimum energy for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MEO q, r, i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by Resource r in producing energy at the Resource’s LSL for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MECAP q, r, i | $/MWh | Minimum-Energy Cap—The amount used for Resource r for minimum-energy costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGMEC unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy Cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1 for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGMEC i | $/MWh | Resource Category Generic Minimum-Energy Cost—The Resource Category Generic Minimum Energy Cost cap for the category of the Resource, according to Section 4.4.9.2.3, for the Operating Day. |
| RTMG q, r, i | MWh | Real-Time Metered Generation—The Resource r’s metered generation for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| LSL q, r, i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the Current Operating Plan (COP). Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| Q | none | A QSE. |
| R | none | A RUC-committed Generation Resource. |
| D | none | An Operating Day containing the RUC-commitment. |
| I | none | A 15-minute Settlement Interval within the hour that includes a RUC-commitment. |
| S | none | A start that is eligible to have its costs included in the RUC Guarantee. |
| T | none | A transition that is eligible to have its costs included in the RUC Guarantee. |
| afterCCGR | none | The Combined Cycle Generation Resource to which a Combined Cycle Train transitions. |
| beforeCCGR | none | The Combined Cycle Generation Resource from which a Combined Cycle Train transitions. |

5.7.1.2	RUC Minimum-Energy Revenue

(1)	The energy revenue for a Resource’s generation up to LSL during all RUC-Committed Hours of the Operating Day is RUC Minimum-Energy Revenue.

(2)	The LSL used to calculate RUC Minimum-Energy Revenue for a Combined Cycle Train is the LSL that corresponds to the Combined Cycle Generation Resource, within the Combined Cycle Train, that is RUC-committed for the hour.

(3)	For each RUC-committed Resource, RUC Minimum-Energy Revenue is calculated as follows:

RUCMEREV q, r, d	=	Σi(RTSPP p, i * Min (RTMGq ,r ,i, (LSL q, r, i * (¼))))

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCMEREV q, r, d | $ | RUC Minimum-Energy Revenue—The sum of the energy revenues for Resource r’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, RUC Minimum-Energy Revenue is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource Node for the Settlement Interval i. |
| RTMG q, r, i | MWh | Real-Time Metered Generation—The Resource r’s metered generation for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| LSL q, r, i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| Q | none | A QSE. |
| R | none | A RUC-committed Generation Resource. |
| D | none | An Operating Day containing the RUC-commitment. |
| P | none | A Resource Node Settlement Point. |
| I | none | A 15-minute Settlement Interval within the hour that includes a RUC-commitment. |

5.7.1.3	Revenue Less Cost Above LSL During RUC-Committed Hours

(1)	The total revenue for a Resource operating above its LSL less the cost based on the Resource’s Energy Offer Curve capped by the Energy Offer Curve Cap (as described in Sections 4.4.9.3, Energy Offer Curve, and 4.4.9.3.3, Energy Offer Curve Caps for Make-Whole Calculation Purposes) or proxy Energy Offer Curve described in Section 6.5.7.3, Security Constrained Economic Dispatch, as applicable, during all RUC-Committed Hours of the Operating Day is Revenue Less Cost Above LSL During RUC-Committed Hours.

(2)	The LSL and RTAIEC used to calculate Revenue Less Cost Above LSL During RUC-Committed Hours for a Combined Cycle Train are the LSL and RTAIEC that correspond to the Combined Cycle Generation Resource, within the Combined Cycle Train, that is RUC-committed for the hour.

(3)	For each RUC-committed Resource, Revenue Less Cost Above LSL During RUC-Committed Hours is calculated as follows:

RUCEXRR q, r, d	=	Max {0, Σi[RTSPP p, i * Max (0, RTMG q, r, i – (LSL q, r, i * (¼)))

+ (-1) * (VSSVARAMT q, r, i + VSSEAMT q, r, i)

+ (-1) * EMREAMT q, r, I 
– RTAIEC q, r, i * Max (0, RTMG q, r, i – (LSL q, r, i * (¼)))]}

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCEXRR q, r, d | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for Resource r operating above its LSL less the cost during all RUC-Committed Hours, for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, revenue less cost above LSL is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource’s Settlement Point for the Settlement Interval i. |
| RTAIEC q, r, i | $/MWh | Real-Time Average Incremental Energy CostThe average incremental energy cost for Resource r, calculated using the Energy Offer Curve capped by the Energy Offer Curve Cap, for the Resource’s generation above the LSL for the Settlement Interval i. See Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC). Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTMG q, r, i | MWh | Real-Time Metered Generation—The Resource r’s metered generation for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| LSL q, r, i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| VSSVARAMT q, r, i | $ | Voltage Support Service VAr Amount by interval—The payment to the QSE for the Voltage Support Service (VSS) provided by Generation Resource r for the 15-minute Settlement Interval i. See Section 6.6.7.1, Voltage Support Service Payments. Payment for VSS is made to the Combined Cycle Train. |
| VSSEAMT q, r, i | $ | Voltage Support Service Energy Amount by interval—The lost opportunity payment to the QSE for ERCOT-directed VSS from the Generation Resource r for the 15-minute Settlement Interval i. See Section 6.6.7.1. Payment for VSS is made to the Combined Cycle Train. |
| EMREAMT q, r, i | $ | Emergency Energy Amount by interval—The payment to the QSE as additional compensation for the additional energy produced by the Generation Resource r in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval i. See Section 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT. Payment for emergency energy is made to the Combined Cycle Train. |
| Q | none | A QSE. |
| R | none | A RUC-committed Generation Resource. |
| D | none | An Operating Day containing the RUC-commitment. |
| P | none | A Resource Node Settlement Point. |
| I | none | A 15-minute Settlement Interval within the hour that includes a RUC instruction. |

5.7.1.4	Revenue Less Cost During QSE Clawback Intervals

(1)	The total revenue for a Resource less the cost based on the Resource’s Energy Offer Curve capped by the Energy Offer Curve Cap (as described in Sections 4.4.9.3, Energy Offer Curve, and 4.4.9.3.3, Energy Offer Curve Caps for Make-Whole Calculation Purposes) or proxy Energy Offer Curve described in Section 6.5.7.3, Security Constrained Economic Dispatch, as applicable, during all QSE Clawback Intervals of the Operating Day is Revenue Less Cost During QSE-Clawback Intervals.

(2)	The MEPR, LSL and RTAIEC used to calculate Revenue Less Cost During QSE Clawback Intervals for a Combined Cycle Train is the MEPR, LSL and RTAIEC that corresponds to the Combined Cycle Generation Resource, within a Combined Cycle Train, that operates in Real-Time for the QSE Clawback Interval.

(3)	For each QSE Clawback Interval, Revenue Less Cost During QSE Clawback Intervals is calculated as follows:

RUCEXRQC q, r, d	=	Max {0, Σi[(RTSPP p, i * RTMG q, r, i)
+ (-1) * (VSSVARAMT q, r, i + VSSEAMT q, r, i)
+ (-1) * EMREAMT q, r, i
– [MEPR q, r, i * Min (RTMG q, r, i, (LSL q, r, i * (¼)))] 
– [RTAIEC q, r, i * Max (0, RTMG q, r, i – (LSL q, r, i * (¼)))]]}

If the QSE submitted a validated Three-Part Supply Offer for the Resource,

Then, 		MEPR q, r, i	=	MEO q, r, i

Otherwise, 	MEPR q, r, i 	= 	MECAP q, r, i

If ERCOT has approved verifiable minimum-energy costs for the Resource,

Then,		MECAP q, r, i	=	verifiable minimum-energy costs q, r, i

Otherwise, 	MECAP q, r, i	= 	RCGMEC i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCEXRQC q, r, d | $ | Revenue Less Cost During QSE-Clawback Intervals—The sum of the total revenue for Resource r less the cost during all QSE-Clawback Intervals for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost During QSE-Clawback Intervals is calculated for the Combined Cycle Train for all Combined Cycle Generation Resources earning revenue in QSE-Clawback Intervals. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource’s Settlement Point for the Settlement Interval i. |
| MEPR q, r, i | $/MWh | Minimum-Energy Price—The Settlement price for Resource r for minimum energy for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MEO q, r, i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by Resource r in producing energy at the Resource’s LSL for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MECAP q, r, i | $/MWh | Minimum-Energy Cap—The amount used for Resource r for minimum-energy costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGMEC unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy Cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1, Verifiable Costs, for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGMEC i | $/MWh | Resource Category Generic Minimum-Energy Cost—The Resource Category Generic Minimum-Energy Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| RTAIEC q, r, i | $/MWh | Real-Time Average Incremental Energy CostThe average incremental energy cost for Resource r, calculated using the Energy Offer Curve capped by the Energy Offer Curve Cap, for the Resource’s generation above the LSL for the Settlement Interval i. See Section 4.6.5, Calculation of “Average Incremental Energy Cost” (AIEC). Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTMG q, r, i | MWh | Real-Time Metered Generation—The Resource r’s metered generation for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| LSL q, r, i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| VSSVARAMT q, r, i | $ | Voltage Support Service VAr Amount by interval—The payment to the QSE for the VSS provided by Generation Resource r for the 15-minute Settlement Interval i. See Section 6.6.7.1, Voltage Support Service Payments. Payment for VSS is made to the Combined Cycle Train. |
| VSSEAMT q, r, i | $ | Voltage Support Service Energy Amount by interval—The lost opportunity payment to the QSE for ERCOT-directed VSS from the Generation Resource r for the 15-minute Settlement Interval i. See Section 6.6.7.1. Payment for VSS is made to the Combined Cycle Train. |
| EMREAMT q, r, i | $ | Emergency Energy Amount by interval—The payment to the QSE as additional compensation for the additional energy produced by the Generation Resource r in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval i. See Section 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT. Payment for emergency energy is made to the Combined Cycle Train. |
| Q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval within the hour that is identified as a QSE-Clawback Interval. |

5.7.2	RUC Clawback Charge

(1)	A QSE for a Resource shall pay a RUC Clawback Charge for the Operating Day if the RUC Guarantee is less than the sum of:

(a)	RUC Minimum-Energy Revenue calculated in Section 5.7.1.2, RUC Minimum-Energy Revenue;

(b)	Revenue Less Cost Above LSL During RUC-Committed Hours calculated in Section 5.7.1.3, Revenue Less Cost Above LSL During RUC-Committed Hours; and

(c)	Revenue Less Cost During QSE-Clawback Intervals calculated in Section 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals.

(2)	The amount of the RUC Clawback Charge is a percentage of the difference calculated in paragraph (1) above.  Whether or not the QSE submits a Three-Part Supply Offer for a Resource in the Day-Ahead Market (DAM) determines the clawback percentage.  If the QSE submitted a validated Three-Part Supply Offer for the Resource into the DAM, then the clawback percentage in RUC-Committed Hours is 50% and the clawback percentage in QSE Clawback Intervals is 0%.  If not, then the clawback percentage in RUC-Committed Hours is 100% and the clawback percentage in QSE Clawback Intervals is 50%.

(3)	If an Energy Emergency Alert (EEA) is in effect for any hour that a Resource is RUC-committed, then in all RUC-Committed Hours of the Operating Day the clawback percentage is 0% if the QSE submitted a validated Three-Part Supply Offer for the Resource into the DAM and 50% otherwise.

(4)	For Combined Cycle Trains, if at least one Combined Cycle Generation Resource is offered into the DAM, then the Combined Cycle Train is considered to be offered into the DAM.

(5)	The RUC Clawback Charge for a Resource, including RMR Units, for each Operating Day is allocated evenly over the RUC-Committed Hours for that Resource.

(6)	For each RUC-committed Resource, the RUC Clawback Charge for each RUC-Committed Hour of the Operating Day is calculated as follows:

If (RUCMEREV q, r, d + RUCEXRR q, r, d – RUCG q, r, d) > 0,

Then, RUCCBAMT q, r, h	=	[(RUCMEREV q, r, d + RUCEXRR q, r, d – RUCG q, r, d) * RUCCBFR q, r, d + RUCEXRQC q, r, d * RUCCBFC q, r, d] / RUCHR q, r, d

Otherwise, RUCCBAMT q, r, h = [Max (0, RUCMEREV q, r, d + RUCEXRR q, r, d + RUCEXRQC q, r, d – RUCG q, r, d) * RUCCBFC q, r, d] / RUCHR q, r, d

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCCBAMT q, r, h | $ | RUC Clawback Charge––The RUC Clawback Charge to a QSE for Resource r as described in this Section, for each RUC-Committed Hour of the Operating Day for that Resource. When one or more Combined Cycle Generation Resources are committed by RUC, a charge is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCG q, r, d | $ | RUC Guarantee—The sum of eligible Startup Costs and Minimum-Energy Costs for Resource r during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.1, RUC Guarantee. When one or more Combined Cycle Generation Resources are committed by RUC, guaranteed costs are calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCMEREV q, r, d | $ | RUC Minimum-Energy Revenue—The sum of the energy revenues for Resource r’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.2. When one or more Combined Cycle Generation Resources are committed by RUC, RUC Minimum-Energy Revenue is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCEXRR q, r, d | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for Resource r above the LSL less the cost during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.3. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost Above LSL During RUC-Committed Hours is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCEXRQC q, r, d | $ | Revenue Less Cost from QSE-Clawback Intervals—The sum of the total revenue for Resource r less the cost during all QSE-Clawback Intervals for the Operating Day. See Section 5.7.1.4. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost from QSE-Clawback Intervals is calculated for the Combined Cycle Train for all Combined Cycle Generation Resources earning revenue in QSE Clawback Intervals. |
| RUCCBFR q, r, d | none | RUC Clawback Factor for RUC-Committed Hours—The Resource r’s Clawback factor for RUC-Committed Hours, which is 50% if a Three-Part Supply Offer was submitted and 100% otherwise. During EEA conditions, the Resource’s RUC Clawback Factor for RUC-Committed Hours is 0% if a Three-Part Supply Offer was submitted and 50% otherwise. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Clawback Factor for RUC-Committed Hours is determined for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCCBFC q, r, d | none | RUC Clawback Factor for QSE Clawback Intervals—The Resource r’s Clawback factor for QSE Clawback Intervals, which is 0% if a Three-Part Supply Offer was submitted and 50% otherwise. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Clawback Factor for QSE Clawback Intervals is determined for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCHR q, r, d | none | RUC Hour—The total number of RUC-Committed Hours, for Resource r for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, the total number of RUC-Committed Hours is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| h | none | An hour in the RUC-commitment period. |

[NPRR222: Replace the above Section 5.7.2, RUC Clawback Charge, with the following upon system implementation] 5.7.2 RUC Clawback Charge (1) A QSE for a Resource shall pay a RUC Clawback Charge for the Operating Day if the RUC Guarantee is less than the sum of: (a) RUC Minimum-Energy Revenue calculated in Section 5.7.1.2, RUC Minimum-Energy Revenue; (b) Revenue Less Cost Above LSL During RUC-Committed Hours calculated in Section 5.7.1.3, Revenue Less Cost Above LSL During RUC-Committed Hours; and (c) Revenue Less Cost During QSE-Clawback Intervals calculated in Section 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals. (2) The amount of the RUC Clawback Charge is a percentage of the difference calculated in paragraph (1) above. Whether or not the QSE submits a Three-Part Supply Offer for a Resource in the Day-Ahead Market (DAM) determines the clawback percentage. If the QSE submitted a validated Three-Part Supply Offer for the Resource into the DAM, then the clawback percentage in RUC-Committed Hours is 50% for Resources that are not Half-Hour Start Units, the clawback percentage in RUC-Committed Hours is 0% for Resources that are Half-Hour Start Units and the clawback percentage in QSE Clawback Intervals is 0%. If the QSE did not submit a validated Three-Part Supply Offer for the Resource that is not a Half-Hour Start Unit, then the clawback percentage in RUC-Committed Hours is 100% and the clawback percentage in QSE Clawback Intervals is 50%. If the QSE did not submit a validated Three-Part Supply Offer for the Resource that is a Half-Hour Start Unit, then the clawback percentage for RUC-Committed Hours is 50% and the clawback percentage in QSE Clawback Intervals is 0%. (3) If an Energy Emergency Alert (EEA) is in effect for any hour that a Resource that is not a Half-Hour Start Unit is RUC-committed, then in all RUC-Committed Hours of the Operating Day the clawback percentage is 0% if the QSE submitted a validated Three-Part Supply Offer for the Resource into the DAM and 50% otherwise. If an EEA is in effect for any hour that a Resource that is a Half-Hour Start Unit is RUC-Committed, then in all RUC-Committed Hours of the Operating Day the clawback percentage is 0%. (4) For Combined Cycle Trains, if at least one Combined Cycle Generation Resource is offered into the DAM, then the Combined Cycle Train is considered to be offered into the DAM. (5) The RUC Clawback Charge for a Resource, including RMR Units, for each Operating Day is allocated evenly over the RUC-Committed Hours for that Resource. (6) For each RUC-committed Resource, the RUC Clawback Charge for each RUC-Committed Hour of the Operating Day is calculated as follows: If (RUCMEREV q, r, d + RUCEXRR q, r, d – RUCG q, r, d) > 0, Then, RUCCBAMT q, r, h = [(RUCMEREV q, r, d + RUCEXRR q, r, d – RUCG q, r, d) * RUCCBFR q, r, d + RUCEXRQC q, r, d * RUCCBFC q, r, d] / RUCHR q, r, d Otherwise, RUCCBAMTq,r,h = [Max (0, RUCMEREV q, r, d + RUCEXRR q, r, d + RUCEXRQC q, r, d – RUCG q, r, d) * RUCCBFC q, r, d] / RUCHR q, r, d The above variables are defined as follows: Variable; Unit; Definition. RUCCBAMT q, r, h; $; RUC Clawback Charge––The RUC Clawback Charge to a QSE for Resource r as described in this Section, for each RUC-Committed Hour of the Operating Day for that Resource. When one or more Combined Cycle Generation Resources are committed by RUC, a charge is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCG q, r, d; $; RUC Guarantee—The sum of eligible Startup Costs and Minimum-Energy Costs for Resource r during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.1, RUC Guarantee. When one or more Combined Cycle Generation Resources are committed by RUC, guaranteed costs are calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCMEREV q, r, d; $; RUC Minimum-Energy Revenue—The sum of the energy revenues for Resource r’s generation up to LSL during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.2. When one or more Combined Cycle Generation Resources are committed by RUC, RUC Minimum-Energy Revenue is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCEXRR q, r, d; $; Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for Resource r above the LSL less the cost during all RUC-Committed Hours, for the Operating Day. See Section 5.7.1.3. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost Above LSL During RUC-Committed Hours is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCEXRQC q, r, d; $; Revenue Less Cost from QSE-Clawback Intervals—The sum of the total revenue for Resource r less the cost during all QSE-Clawback Intervals for the Operating Day. See Section 5.7.1.4. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost from QSE-Clawback Intervals is calculated for the Combined Cycle Train for all Combined Cycle Generation Resources earning revenue in QSE Clawback Intervals.. RUCCBFR q, r, d; none; RUC Clawback Factor for RUC-Committed Hours—The Resource r’s Clawback factor is defined in paragraphs (2) and (3) above. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Clawback Factor for RUC-Committed Hours is determined for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCCBFC q, r, d; none; RUC Clawback Factor for QSE Clawback Intervals—The Resource r’s Clawback factor is defined in paragraphs (2) and (3) above. When one or more Combined Cycle Generation Resources are committed by RUC, the RUC Clawback Factor for QSE Clawback Intervals is determined for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. RUCHR q, r, d; none; RUC Hour—The total number of RUC-Committed Hours, for Resource r for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, the total number of RUC-Committed Hours is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources.. q; none; A QSE.. r; none; A RUC-committed Generation Resource.. d; none; An Operating Day containing the RUC-commitment.. h; none; An hour in the RUC-commitment period..

5.7.3	Payment When ERCOT Decommits a QSE-Committed Resource

(1)	If ERCOT decommits a QSE-committed Resource during the RUC process earlier than its scheduled shutdown within the Operating Day, then no compensation is due to the affected QSE from ERCOT.

(2)	If ERCOT decommits a QSE committed Resource that is not scheduled to shutdown within the Operating Day, then ERCOT shall pay the affected QSE an amount as calculated below for the hours of decommitment.  The number of continuous decommitted hours used in the calculation are the hours beginning with the first decommitted hour until the earlier of:

(a)	The hour ERCOT determines that the Resource may again be at LSL; and

(b)	The end of the last hour of the Operating Day.

(3)	If ERCOT decommits a QSE-committed Resource not scheduled to shutdown within the Operating Day, and the decommitment period spans more than one Operating Day, the RUC Decommitment Payment Amount shall be calculated and paid in the Operating Day in which the RUC decommitment originated.  The number of continuous decommitted hours used in the calculation are the hours beginning with the first decommitted hour until the end of the last hour of the Operating Day in which the RUC decommitment originated.

(4)	The payment for a RUC Cancellation instruction for a Resource is settled for each hour through an adjustment in the RUC Decommitment Payment Amount as shown in paragraph (7) below.

(5)	ERCOT shall produce a report each April that provides the percentage of the RUC Decommitment Payment Amounts that are a result of RUC cancellations during the 12 months of the previous calendar year.  The report shall be based on the Final Settlements.  ERCOT shall present the results of this study to the appropriate Technical Advisory Committee (TAC) subcommittee.

(6)	The SUPR, MEPR and LSL used to calculate payment when ERCOT decommits a QSE-committed Combined Cycle Train is the SUPR, MEPR and LSL that corresponds to the Combined Cycle Generation Resource, within the Combined Cycle Train, that is RUC-decommitted in the first hour of a contiguous decommitted period.

(7)	The payment for a RUC decommitment instruction for a Resource, including RMR Units, is calculated for each hour as follows:

RUCDCAMT q, r, h	=	(-1) * Max (0, (SUPR q, r, s - Σi(Max (0, MEPR q, r, i – RTSPP p, i) * (LSL q, r, i * (¼))))) / NCDCHR q, r, h

Where:

If the QSE submitted a validated Three-Part Supply Offer for the Resource,

Then, 		SUPR q, r, s		=	SUO q, r, s

MEPR q, r, i	=	MEO q, r, i

Otherwise, 	SUPR q, r, s 	= 	SUCAP q, r, s

MEPR q, r, i 	= 	MECAP q, r, i

If ERCOT has approved verifiable Startup Costs and minimum-energy costs for the Resource,

Then, 		SUCAP q, r, s	=	verifiable Startup Costs q, r, s

MECAP q, r, i	=	verifiable minimum-energy costs q, r, i

Otherwise, 	SUCAP q, r, s 	= 	RCGSC s

MECAP q, r, i	= 	RCGMEC i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCDCAMT q, r, h | $ | RUC Decommitment Payment Amount—The payment to the QSE for the Resource that was decommitted by ERCOT but that was not scheduled to shut down in the Operating Day, for each decommited hour of the Operating Day. When one or more Combined Cycle Generation Resources are decommitted by RUC, payment is made to the Combined Cycle Train for all RUC-decommitted Combined Cycle Generation Resources. |
| SUPR q, r, s | $/Start | Startup Price per start—The settlement price for Resource r for the start s. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUO q, r, s | $/Start | Startup Offer per start—Represents an offer for all costs incurred by Generation Resource r in starting up and reaching the Resource’s LSL, minus the average energy produced during the time period between breaker close and LSL multiplied by the heat rate proxy multiplied by the appropriate FIP or FOP, as described in the Verifiable Cost Manual. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| SUCAP q, r, s | $/Start | Startup Cap—The amount used for Resource r as Startup Costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGSC unless ERCOT has approved verifiable unit-specific Startup Costs for that Resource, in which case the Startup Cap is the verifiable unit-specific Startup Cost. See Section 5.6.1, Verifiable Costs, for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGSC s | $/Start | Resource Category Generic Startup Cost—The Resource Category Startup Offer Generic Cap cost for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| MEPR q, r, i | $/MWh | Minimum-Energy Price—The settlement price for Resource r for minimum energy for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MEO q, r, i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by Resource r in producing energy at the Resource’s LSL for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MECAP q, r, i | $/MWh | Minimum-Energy Cap—The amount used for Resource r for minimum-energy costs if the QSE did not submit a validated Three-Part Supply Offer. The cap is the RCGMEC unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy Cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1 for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGMEC i | $/MWh | Resource Category Generic Minimum-Energy Cost—The Resource Category Minimum-Energy Generic Cap cost for the category of the Resource, according to Section 4.4.9.2.3. |
| LSL q, r, i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource’s Settlement Point for the Settlement Interval i. |
| NCDCHR q, r, h | none | Number of Continuous Decommitted Hours—The number of continuous decommitment hours for Resource r within an Operating Day. When one or more Combined Cycle Generation Resources are decommitted by RUC, the Number of Continuous Decommitted Hours is calculated for the Combined Cycle Train for all RUC-decommitted Combined Cycle Generation Resources. |
| Q | none | A QSE. |
| R | none | A RUC-decommitted Generation Resource. |
| H | none | An hour in the RUC decommitment period. |
| P | none | A Resource Node Settlement Point. |
| S | none | A start. |
| I | none | A 15-minute Settlement Interval within the contiguous decommitted period.. |

5.7.4	RUC Make-Whole Charges

(1)	All QSEs that were capacity-short in each RUC will be charged for that shortage, as described in Section 5.7.4.1, RUC Capacity-Short Charge.  If the revenues from the charges under Section 5.7.4.1 are not enough to cover all RUC Make-Whole Payments for a Settlement Interval, then the difference will be uplifted to all QSEs on a Load Ratio Share (LRS) basis, as described in Section 5.7.4.2, RUC Make-Whole Uplift Charge.

(2)	To determine whether a QSE is capacity-short, the Wind Generation Resource (WGR) Production Potential (WGRPP), as described in Section 4.2.2, Wind-Powered Generation Resource Production Potential, for a WGR used in the corresponding RUC is considered the available capacity of the WGR when determining responsibility for the corresponding RUC charges, regardless of the Real-Time output of the WGR.

(3)	On a monthly basis, within ten days after the Initial Settlement of the last day of the month has been completed, ERCOT shall post on the Market Information System (MIS) Secure Area the total RUC Make-Whole Charges and RUC Clawback Payment Amounts, by Settlement Interval, by QSE capacity-shortfall and by amount uplifted.

5.7.4.1	RUC Capacity-Short Charge

The dollar amount charged to each QSE, due to capacity shortfalls for a particular RUC, for a 15-minute Settlement Interval, is the QSE’s shortfall ratio share multiplied by the total RUC Make-Whole Payments, including amounts for RMR Units, to all QSEs for that RUC, subject to a cap.  The cap on the charge to each QSE is two multiplied by the total RUC Make-Whole Payments, including amounts for RMR Units, for all QSEs multiplied by that QSE’s capacity shortfall for that RUC process divided by the total capacity of all RUC-committed Resources during that Settlement Interval for the RUC process.  That dollar amount charged to each QSE is calculated as follows:

RUCCSAMT ruc, i , q	=	(-1) * Max [(RUCSFRS ruc, i, q * RUCMWAMTRUCTOT ruc, h), 
(2 * RUCSF ruc, i ,q * RUCMWAMTRUCTOT ruc, h / RUCCAPTOT ruc, h)] / 4

Where:

RUCMWAMTRUCTOT ruc, h	=	ΣqΣrRUCMWAMT ruc, q, r, h

RUCCAPTOT ruc, h		=	ΣrHSL ruc, h, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCCSAMT ruc, i, q | $ | RUC Capacity-Short Amount—The charge to a QSE, due to capacity shortfall for a particular RUC process, for the 15-minute Settlement Interval. |
| RUCMWAMTRUCTOT ruc, h | $ | RUC Make-Whole Amount Total per RUC—The sum of RUC Make-Whole Payments for a particular RUC process, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. |
| RUCMWAMT ruc, q ,r ,h | $ | RUC Make-Whole Payment—The RUC Make-Whole Payment to the QSE for Resource r, for a particular RUC process, for the hour that includes the 15-minute Settlement Interval. See Section 5.7.1, RUC Make-Whole Payment. When one or more Combined Cycle Generation Resources are committed by RUC, payment is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCSFRS ruc, i, q | none | RUC Shortfall Ratio Share—The ratio of the QSE’s capacity shortfall to the sum of all QSEs’ capacity shortfalls for a particular RUC process, for the 15-minute Settlement Interval. See Section 5.7.4.1.1, Capacity Shortfall Ratio Share. |
| RUCSF ruc, I, q | MW | RUC Shortfall—The QSE’s capacity shortfall for a particular RUC process for the 15-minute Settlement Interval. See formula in Section 5.7.4.1.1. |
| RUCCAPTOT ruc, h | MW | RUC Capacity Total—The sum of the High Sustained Limits (HSLs) of all RUC-committed Resources for a particular RUC process, for the hour that includes the 15-minute Settlement Interval. See formula in Section 5.7.4.1.1. |
| HSL ruc, h, r | MW | High Sustained Limit—The HSL of Generation Resource r as defined in Section 2, Definitions and Acronyms, for the hour that includes the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| Ruc | none | The RUC process for which the RUC Capacity-Short Charge is calculated. |
| I | none | A 15-minute Settlement Interval. |
| Q | none | A QSE. |
| H | none | The hour that includes the Settlement Interval i. |
| R | none | A Generation Resource that is RUC-committed for the hour that includes the Settlement Interval i, as a result of a particular RUC process. |

5.7.4.1.1	Capacity Shortfall Ratio Share

(1)	In calculating the amount short for each QSE, the QSE must be given a capacity credit for its WGRs based on the HSL values entered into the COP by the QSE just prior to the RUC execution.  For WGRs, ERCOT shall use for Settlement purposes the COP and Trades Snapshot prior to the RUC regardless of Real-Time capacity or actual generation.  Therefore, the HASLSNAP and HASLADJ variables used below shall be equal to the HSL values entered into the QSE’s COP submitted prior to the RUC for WGRs.

(2)	In calculating the amount short for each QSE, the QSE must be given a capacity credit for non-wind Resources that were given notice of decommitment within the two hours before the Operating Hour as a result of the RUC process by setting the HASLSNAP and HASLADJ variables used below equal to the HASLSNAP value for the Resource immediately before the decommitment instruction was given.

(3)	In calculating the short amount for each QSE, if the High Ancillary Service Limit (HASL) for a Resource was credited to the QSE during the RUC snapshot but the Resource experiences a Forced Outage within two hours before the start of the Settlement Interval, then the HASL for that Resource is also credited to the QSE in the HASLADJ.

(4)	In calculating the short amount for each QSE, if the DCIMPSNAP was credited to the QSE during the RUC snapshot but the entire Direct Current Tie (DC Tie) experiences a Forced Outage within two hours before the start of the Settlement Interval, then the DCIMPSNAP is also credited to the QSE in the DCIMPADJ.

(5)	For Combined Cycle Generation Resources, if more than one Combined Cycle Generation Resource is shown On-Line in its COP for the same Settlement hour, then the provisions of paragraph (5)(a) of Section 3.9.1, Current Operating Plan (COP) Criteria, apply in the determination of the On-Line Combined Cycle Generation Resource for that Settlement hour.

(6)	The capacity shortfall ratio share of a specific QSE for a particular RUC process is calculated, for a 15-minute Settlement Interval, as follows:

RUCSFRS ruc, i, q	=	RUCSF ruc, i ,q / RUCSFTOT ruc, i

Where:

RUCSFTOT ruc, i	=	ΣqRUCSF ruc, i, q

(7)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval is:

RUCSF ruc, i, q	=	Max (0, Max (RUCSFSNAP q, i, RUCSFADJ q, i) – Σz is prior to rucRUCCAPCREDIT q, i, z)

(8)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at the snapshot, is:

RUCSFSNAP q, i	=	Max (0, ((ΣpRTAML q, p, i * 4) + ΣpRTDCEXP q, p, i – RUCCAPSNAP q, i))

(9)	The amount of capacity that a QSE had according to the RUC snapshot for a 15-minute Settlement Interval is:

RUCCAPSNAP q, i =	ΣrHASLSNAP q, r, h + (RUCCPSNAP q, h – RUCCSSNAP q, h) + (ΣpDAEP q, p, h –ΣpDAES q, p, h) + (ΣpRTQQEPSNAP q, p, i – ΣpRTQQESSNAP q, p, i) + ΣpDCIMPSNAP q, p, i

(10)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at Real-Time, is:

RUCSFADJ q, i	=	Max (0, ((ΣpRTAML q, p, i) *4) + ΣpRTDCEXP q, p, i – RUCCAPADJ q, i)

(11)	The amount of capacity that a QSE had in Real-Time for a 15-minute Settlement Interval.

RUCCAPADJ q, i =	ΣrHASLADJ q, r, h + (RUCCPADJ q, h – RUCCSADJ q, h) + (ΣpDAEPq ,p ,h – ΣpDAES q, p, h) + (ΣpRTQQEPADJ q, p, i – ΣpRTQQESADJ q, p, i) + ΣpDCIMPADJ q, p, i

(12)	The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCSFRS ruc, i, q | none | RUC Shortfall Ratio Share—The ratio of the QSE’s capacity shortfall to the sum of all QSEs’ capacity shortfalls, for the RUC process, for the 15-minute Settlement Interval. |
| RUCSF ruc, i, q | MW | RUC Shortfall—The QSE q’s capacity shortfall for the RUC process for the 15-minute Settlement Interval. |
| RUCSFTOT ruc, i | MW | RUC Shortfall Total—The sum of all QSEs’ capacity shortfalls, for a RUC process, for a 15-minute Settlement Interval. |
| RUCSFSNAP q, i | MW | RUC Shortfall at Snapshot—The QSE q’s capacity shortfall according to the snapshot for the RUC process for the 15-minute Settlement Interval. |
| RUCSFADJ q, i | MW | RUC Shortfall at Adjustment Period—The QSE q’s Adjustment Period capacity shortfall for the 15-minute Settlement Interval. |
| RUCCAPCREDIT q, i, z | MW | RUC Capacity Credit by QSE—The capacity credit resulting from capacity paid through the RUC Capacity-Short Amount for the 15-minute Settlement Interval. |
| RTAML q, p, i | MWh | Real-Time Adjusted Metered Load—The QSE q’s Adjusted Metered Load (AML) at the Settlement Point p for the 15-minute Settlement Interval. |
| RUCCAPSNAP q, i | MW | RUC Capacity Snapshot at time of RUC—The amount of the QSE’s calculated capacity in the COP and Trades Snapshot for a 15-minute Settlement Interval. |
| HASLSNAP q, r, h | MW | High Ancillary Services Limit at Snapshot—The HASL of the Resource r represented by the QSE q, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTDCEXP q, p | MW | Real-Time DC Export per QSE per Settlement Point—The aggregated DC Tie Schedule through DC Tie p submitted by QSE q that is under the Oklaunion Exemption as an exporter from the ERCOT Region, for the 15-minute Settlement Interval. |
| DCIMPADJ q, p | MW | DC Import per QSE per Settlement Point—The approved aggregated DC Tie Schedule submitted by QSE q as an importer into the ERCOT System through DC Tie p according to the Adjustment Period snapshot, for the 15-minute Settlement Interval. |
| DCIMPSNAP q, p | MW | DC Import per QSE per Settlement Point—The approved aggregated DC Tie Schedule submitted by QSE q as an importer into the ERCOT System through DC Tie p, according to the snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RUCCPSNAP q, h | MW | RUC Capacity Purchase at Snapshot—The QSE q’s capacity purchase, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RUCCSSNAP q, h | MW | RUC Capacity Sale at Snapshot—The QSE q’s capacity sale, according to the COP and Trades Snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval. |
| RUCCAPADJ q, i | MW | RUC Capacity Snapshot during Adjustment Period—The amount of the QSE’s calculated capacity in the RUC according to the COP and Trades Snapshot at the end of the Adjustment Period for a 15-minute Settlement Interval |
| HASLADJ q, r, h | MW | High Ancillary Services Limit at Adjustment Period—The HASL of the Resource r represented by the QSE q, according to the Adjustment Period snapshot, for the hour that includes the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RUCCPADJ q, h | MW | RUC Capacity Purchase at Adjustment Period—The QSE q’s capacity purchase, according to the Adjustment Period COP and Trades Snapshot for the hour that includes the 15-minute Settlement Interval. |
| RUCCSADJ q, h | MW | RUC Capacity Sale at Adjustment Period—The QSE q’s capacity sale, according to the Adjustment Period COP and Trades Snapshot for the hour that includes the 15-minute Settlement Interval. |
| DAEP q, p, h | MW | Day-Ahead Energy Purchase—The QSE q’s energy purchased in the DAM at the Settlement Point p for the hour that includes the 15-minute Settlement Interval. |
| DAES q, p, h | MW | Day-Ahead Energy Sale—The QSE q’s energy sold in the DAM at the Settlement Point p for the hour that includes the 15-minute Settlement Interval. |
| RTQQEPSNAP q, p, i | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval, in the COP and Trades Snapshot. |
| RTQQESSNAP q, p, i | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval, in the COP and Trades Snapshot. |
| RTQQEPADJ q, p, i | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. |
| RTQQESADJq,p,i | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. |
| q | none | A QSE. |
| p | none | A Settlement Point. |
| r | none | A Generation Resource that is QSE-committed or RUC-decommitted (subject to paragraphs (1) and (2) above) for the Settlement Interval. |
| z | none | A previous RUC process for the Operating Day. |
| i | none | A 15-minute Settlement Interval. |
| h | none | The hour that includes the Settlement Interval i. |
| ruc | none | The RUC process for which this RUC Shortfall Ratio Share is calculated. |

5.7.5	RUC Clawback Payment

ERCOT shall pay the revenues from all RUC Clawback Charges, including amounts for RMR Units, in a 15-minute Settlement Interval to all QSEs, on an LRS basis, as the RUC Clawback Payment.  The RUC Clawback Payment is calculated as follows for each QSE for each 15-minute Settlement Interval:

LARUCCBAMT q, i	=	(-1) * (RUCCBAMTTOT h / 4 * LRS q, i)

Where:

RUCCBAMTTOT h	=	ΣqΣrRUCCBAMT q, r, h

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LARUCCBAMT q, i | $ | RUC Clawback Payment—The RUC make-whole clawback payment to a QSE to uplift RUC Make-Whole Clawback Charges received, for a 15-minute Settlement Interval. |
| RUCCBAMTTOT h | $ | RUC Clawback Charge Total—The sum of RUC Clawback Charges to all QSEs, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. |
| LRS q, i | none | Load Ratio Share—The LRS for the 15-minute Settlement Interval. See Section 6.6.2, Load Ratio Share. |
| RUCCBAMT q, r, h | $ | RUC Clawback Charge—The RUC Clawback Charge to the QSE q for the Resource r, for the hour that includes the 15-minute Settlement Interval. When one or more Combined Cycle Generation Resources are committed by RUC, a charge is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| q | None | A QSE. |
| i | none | A 15-minute Settlement Interval. |
| h | none | The hour that includes the Settlement Interval i. |
| r | none | A RUC-committed Generation Resource. |

5.7.6	RUC Decommitment Charge

ERCOT shall charge each QSE a RUC Decommitment Charge, on an LRS basis, all revenues paid as a result of RUC Decommitment Payments, including amounts for RMR Units.  The RUC Decommitment Charge for a 15-minute Settlement Interval is calculated as follows:

LARUCDCAMT q, i	=	(-1) * [(RUCDCAMTTOT h / 4) * LRS q, i]

Where:

RUCDCAMTTOT h	=	ΣqΣrRUCDCAMT q, r, h

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LARUCDCAMT q, i | $ | RUC Decommitment Charge—The RUC Decommitment Charge to a QSE, for a 15-minute Settlement Interval. |
| RUCDCAMTTOT h | $ | RUC Decommitment Charge Total—The sum of RUC Decommitment Payments to all QSEs, including amounts for RMR Units, for the hour that includes the 15-minute Settlement Interval. |
| LRS q, i | none | Load Ratio Share—The LRS for the 15-minute Settlement Interval. See Section 6.6.2, Load Ratio Share. |
| RUCDCAMT q, r, h | $ | RUC Decommitment Charge—The RUC Decommitment Charge to the QSE q for the Resource r, for the hour that includes the 15-minute Settlement Interval. When one or more Combined Cycle Generation Resources are decommitted by RUC, payment is made to the Combined Cycle Train for all RUC-decommitted Combined Cycle Generation Resources. |
| q | None | A QSE. |
| i | none | A 15-minute Settlement Interval. |
| h | none | The hour that includes the Settlement Interval i. |
| r | None | A RUC-decommitted Generation Resource. |

6.5.5.2	Operational Data Requirements

(1)	ERCOT shall use Operating Period data to monitor and control the reliability of the ERCOT Transmission Grid and shall use it in network analysis software to predict the short-term reliability of the ERCOT Transmission Grid.  Each TSP, at its own expense, may obtain that Operating Period data from ERCOT or directly from QSEs.

(2)	A QSE representing a Generation Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time telemetry data to ERCOT for each Generation Resource.  ERCOT shall make that data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to requesting TSPs and DSPs operating within ERCOT.  Such data must be provided to the requesting TSP or DSP at the requesting TSP’s or DSP’s expense, including:

(a)	Net real power (in MW) as measured by installed power metering or as calculated in accordance with the Operating Guides based on metered gross real power and conversion constants determined by the Resource Entity and provided to ERCOT as a result of Section 3.7, Resource Parameters.  Net real power represents the actual generation of a Resource for all real power dispatch purposes, including use in Security-Constrained Economic Dispatch (SCED), determination of the High Ancillary Service Limit (HASL), High Dispatch Limit (HDL), Low Dispatch Limit (LDL) and Low Ancillary Service Limit (LASL), and is consistent with telemetered HSL and LSL;

(b)	Gross real power (in MW) as measured by installed power metering or as calculated in accordance with the Operating Guides based on metered real power, which may include Supervisory Control and Data Acquisition (SCADA) metering, and conversions constants determined by the Resource Entity and provided to ERCOT as a result of Section 3.7;

(c)	Gross Reactive Power (in Megavolt-Amperes reactive (MVAr));

(d)	Net Reactive Power (in MVAr);

(e)	Power to standby transformers serving plant auxiliary Load;

(f)	Status of switching devices in the plant switchyard not monitored by the TSP or DSP affecting flows on the ERCOT Transmission Grid;

(g)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(h)	Generation Resource breaker and switch status;

(i)	HSL;

(j)	High Emergency Limit (HEL), under Section 6.5.9.2, Failure of the SCED Process;

(k)	Low Emergency Limit (LEL), under Section 6.5.9.2;

(l)	LSL;

(m)	Configuration identification for Combined Cycle Generation Resources;

(n)	Ancillary Service Schedule for each quantity of RRS and Non-Spin which is equal to the Ancillary Service Resource Responsibility minus the amount of Ancillary Service deployment;

(o)	Ancillary Service Resource Responsibility for each quantity of Regulation Up (Reg-Up), Regulation Down (Reg-Down), RRS and Non-Spin.  The sum of Ancillary Service Resource Responsibility for all Resources in a QSE is equal to the Ancillary Service Supply Responsibility for that QSE; and

(p)	Reg-Up and Reg-Down Services participation factors represent how a QSE is planning to deploy the Ancillary Service energy on a percentage basis to specific qualified Resource(s).

(3)	For each Wind-powered Generation Resource (WGR), the QSE shall set the HSL equal to the current net output capability of the facility within one SCADA cycle after receiving the last SCED Base Point Dispatch Instruction from ERCOT.  The HSL telemetered to ERCOT for the WGR shall not be changed until the receipt of the next Base Point.  If the Base Point received from ERCOT is less than the telemetered HSL minus two MW, then the WGR shall respond to the Base Point Dispatch Instruction and shall be considered curtailed.  During any curtailed SCED intervals, the WGR HSL shall be updated within one SCADA cycle after the receipt of a subsequent SCED Base Point with a value which represents the WGR’s estimated, non-curtailed output potential.  This estimated, non-curtailed output potential shall be based upon turbine availability, wind speed at the WGR and application of the appropriate wind/power curves.  During SCED intervals when the telemetered Base Point of the WGR exceeds the last HSL telemetered to ERCOT minus two MW, the WGR shall be considered as not curtailed.  HSLs telemetered to ERCOT for WGRs that are not curtailed shall be based on the WGRs’ actual net output as stated above in this Section 6.5.5.2.

(4)	A QSE representing a Load Resource connected to Transmission Facilities or distribution facilities shall provide the following Real-Time data to ERCOT for each Load Resource and ERCOT shall make the data available, in accordance with ERCOT Protocols, NERC standards and policies, and Governmental Authority requirements, to the Load Resource’s host TSP or DSP at the TSP’s or DSP’s expense.  The Load Resource’s net real power consumption, Low Power Consumption (LPC) and Maximum Power Consumption (MPC) shall be telemetered to ERCOT using a positive (+) sign convention:

(a)	Load Resource net real power consumption (in MW);

(b)	Any data mutually agreed to by ERCOT and the QSE to adequately manage system reliability;

(c)	Load Resource breaker status;

(d)	LPC (in MW);

(e)	MPC (in MW);

(f)	Ancillary Service Schedule (in MW) for each quantity of RRS and Non-Spin;

(g)	Ancillary Service Resource Responsibility (in MW) for each quantity of Reg-Up and Reg-Down for Controllable Load Resources, and RRS and Non-Spin for all Load Resources;

(h)	The status of the high-set under-frequency relay, if required for qualification;

(i)	For a Controllable Load Resource, the Scheduled Power Consumption that represents zero Ancillary Service deployments;

(j)	For a Controllable Load Resource, net Reactive Power (in MVAr);

(k)	Resource Status (Resource Status shall be ONRL if high-set under-frequency relay is active); and

(l)	Reg-Up and Reg-Down services participation factor, which represents how a QSE is planning to deploy the Ancillary Service energy on a percentage basis to specific qualified Resource.

(5)	A QSE with Resources used in SCED shall provide communications equipment to receive ERCOT-telemetered control deployments.

(6)	A QSE providing any Regulation Service shall provide telemetry indicating the appropriate status of Resources providing Reg-Up or Reg-Down, including status indicating whether the Resource is temporarily blocked from receiving Reg-Up and/or Reg-Down deployments from the QSE.

(7)	Real-Time data for reliability purposes must be accurate to within three percent.  This telemetry may be provided from relaying accuracy instrumentation transformers.

(8)	Each QSE shall report the current configuration of combined-cycle Resources that it represents to ERCOT.  The telemetered Resource Status for a Combined Cycle Generation Resource may only be assigned a Resource Status of OFFNS if no generation units within that Combined Cycle Generation Resource are On-Line.

(9)	A QSE representing combined-cycle Resources shall provide ERCOT with the possible operating configurations for each power block with accompanying limits.  Combined Cycle Train power augmentation methods may only be included as part of one or more of the registered Combined Cycle Generation Resource configurations.  The Energy Offer Curve in the Three Part Supply Offer that includes the offered power augmentation method may reflect the price of the added capability.  Such power augmentation methods may include:

(a)	Combustion turbine inlet air cooling (CTIAC) methods;

(b)	Duct firing;

(c)	Other ways of temporarily increasing the output of combined-cycle Resources; and

(d)	For Qualifying Facilities (QFs), an LSL that represents the minimum energy available, in MW, from the Combined Cycle Generation Resource for economic dispatch based on the minimum stable steam delivery to the thermal host plus a justifiable reliability margin that accounts for changes in ambient conditions.

6.6.1	Real Time Settlement Point Prices

6.6.1.1	Real-Time Settlement Point Price for a Resource Node

(1)	Except for a logical Resource Node for a Combined Cycle Train, the Real-Time Settlement Point Price for a Resource Node Settlement Point is a Base Point time-weighted average of the Real-Time Locational Marginal Prices (LMPs).  The Real-Time Settlement Point Price for a 15-minute Settlement Interval is calculated as follows:

RTSPP	=	Σy(RNWF y * RTLMP y)

Where the Resource Node weighting factor is:

RNWF y	=	[Max (0.001, ΣrBP r, y) * TLMP y] / [Σy(Max (0.001, ΣrBP r, y) * TLMP y)]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point PriceThe Real-Time Settlement Point Price at the Settlement Point for the 15-minute Settlement Interval. |
| RTLMP y | $/MWh | Real-Time Locational Marginal Price per intervalThe Real-Time LMP at the Settlement Point for the Security-Constrained Economic Dispatch (SCED) interval y. |
| BP r, y | MW | Base Point per Resource per intervalThe Base Point of Resource r, for the whole SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per intervalThe weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| r | none | A Resource at the Resource Node. The summation is taken over all Resources at that node. |

(2)	The Real-Time Settlement Point Price at the logical Resource Node for the On-Line Combined Cycle Generation Resource shall be determined in accordance with paragraph (1) above using a Real-Time LMP calculated for the logical Resource Node in each SCED Interval as follows:

(a)	The Real-Time LMP for the logical Resource Node for each SCED interval shall be the sum of the Real-Time LMP in each SCED interval at each of the Resource Nodes of the generation units registered in the On-Line (as determined by Real-Time telemetry) Combined Cycle Generation Resource times a weight factor determined as set forth in paragraph (b) below.

Where:

RTLMP = ∑CCGR_PhyR RTLMP CCGR_PhyR * RTCCGRWF CCGR_PhyR

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTLMP | $/MWh | Real-Time Locational Marginal Price at a logical Resource Node for a Combined Cycle TrainThe Real-Time LMP at the Combined Cycle Generation Resource logical Resource Node for a SCED Interval. |
| RTLMP CCGR_PhyR | $/MWh | Real-Time Locational Marginal Price at a generation unit Resource Node registered to the Combined Cycle Generation ResourceThe Real-Time LMP at the Resource Node of an On-Line generation unit designated in a Combined Cycle Train registration for the Combine Cycle Generation Resource for each SCED interval. |
| RTCCGRWF CCGR_PhyR | none | Real-Time Combined Cycle Generation Resource Weighting FactorThe Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the Combined Cycle Generation Resource. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train registration for a Combined Cycle Generation Resource. |

(b)	The weight factor for each generation unit registered in a Combined Cycle Generation Resource shall be the Real-Time net power output telemetry in each SCED interval for each generation unit registered in the Combined Cycle Generation Resource divided by the total Real-Time net power output telemetry for all of the generation units registered in the Combined Cycle Generation Resource.

Where:

RTCCGRWF CCGR_PhyR = TG CCGR_PhyR / ∑CCGR_PhyR TG CCGR_PhyR

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTCCGRWF CCGR_PhyR | none | Real-Time Combined Cycle Generation Resource Weighting FactorThe Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the Combined Cycle Generation Resource. |
| TG CCGR_PhyR | MW | Telemetered Generation for a Combined Cycle Generation Resource generation unitThe telemetered generation of a generation unit designated in a Combined Cycle Train registration for the Combined Cycle Generation Resource at the time of each SCED run. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train registration for a Combined Cycle Generation Resource. |

6.6.3	Real-Time Energy Charges and Payments

6.6.3.1	Real-Time Energy Imbalance Payment or Charge at a Resource Node

(1)	The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time Settlement Point Price for the following amounts at a particular Resource Node Settlement Point:

(a)	The energy produced by all its Generation Resources at the Settlement Point; plus

(b)	The amount of its Self-Schedules with sink specified at the Settlement Point; plus

(c)	The amount of its Energy Bids cleared in the Day-Ahead Market (DAM) at the Settlement Point; plus

(d)	The amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(e)	The amount of its Self-Schedules with source specified at the Settlement Point; minus

(f)	The amount of its energy offers cleared in the DAM at the Settlement Point; minus

(g)	The amount of its Energy Trades at the Settlement Point where the QSE is the seller

(2)	The payment or charge to each QSE for Energy Imbalance Service at a Resource Node Settlement Point for a given 15-minute Settlement Interval is calculated as follows:

If the Generation Resources at the Resource Node Settlement Point p are involved with a net metering scheme

RTEIAMT q, p	=	(-1) * {Σgsc(Σr(GSPLITPER q, r, gsc, p * NMSAMTTOT gsc)) + RTSPP p * [(SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)]}

Otherwise

RTEIAMTq, p	=	(-1) * RTSPP p * {ΣrRTMG q, p, r + (SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)}

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| RTMG q, r, p | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource—The Real-Time energy produced by the Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| SSSK q, p | MW | Self-Schedule with Sink at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with sink at Settlement Point p, for the 15-minute Settlement Interval. |
| DAEP q, p | MW | Day-Ahead Energy Purchase per QSE per Settlement Point—The QSE q’s DAM Energy Bids at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQEP q, p | MW | Real-Time QSE-to-QSE Energy Purchase per QSE per Settlement PointThe amount of MW bought by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSR q, p | MW | Self-Schedule with Source at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with source at Settlement Point p, for the 15-minute Settlement Interval. |
| DAES q, p | MW | Day-Ahead Energy Sale per QSE per Settlement Point—The QSE q’s energy offers at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQES q, p | MW | Real-Time QSE-to-QSE Energy Sale per QSE per Settlement PointThe amount of MW sold by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| NMSAMTTOT gsc | $ | Net Metering Settlement Payment—The total payment to the entire facility with a net metering arrangement. |
| GSPLITPER q, r, gsc, p | none | Generation Resource SCADA Splitting Percentage—The generation allocation percentage for Resource r that is part of a net metering arrangement. GSPLITPER is calculated by taking the Supervisory Control and Data Acquisition (SCADA) values (GSSPLITSCA) for a particular Generation Resource r that is part of a net metering configuration and dividing by the sum of all SCADA values for all Resources that are included in the net metering configuration for each interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |
| gsc | none | A generation site code. |

(3)	The total payments to a facility with a net metering arrangement, for each 15-minute Settlement Interval, shall be calculated as follows:

NMRTETOT gsc = Σme MEB gsc, b

If NMRTETOT gsc = 0

The Load is included in the Real-Time AML per QSE and is included in the Real-Time energy imbalance payment or charge at a Load Zone.

Otherwise

NMSAMTTOT gsc	=	Σb (RTRMPR b * MEB gsc, b)

Where the price for Settlement Meter is determined as follows:

For EBNRT b <= 0

RTRMPR b	=	Σy (RTLMP b, y * TLMP y) / Σy TLMP y

Otherwise RTRMPR b is determined as follows:

RTRMPR b =(RNWF b, y * RTLMP b, y)

Where the weighting factor for the bus associated with the meter is:

RNWF b, y = [Max(0.001, rBP y) * TLMP y] /

[yMax(0.001, ΣrBP r, y) * TLMP y].

The summation is over all Resources r associated to the individual meter.  The determination of which Resources are associated to an individual meter is static and based on the normal system configuration of the generation site code, gsc.

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| NMRTETOT gsc | MWh | Net Meter Real-Time Energy Total—The net sum for all Settlement Meters me included in generation site code gsc. A positive value indicates an injection of power to the ERCOT System. |
| NMSAMTTOT gsc | $ | Net Metering Settlement Payment—The total payment to the entire facility with a net metering arrangement. |
| RTRMPR b | $/MWh | Real-Time Price for the Energy Metered for each Resource meter at busThe Real-Time price for the Settlement Meter at Electrical Bus b, for the 15-minute Settlement Interval. |
| EBNRT me | MWh | Energy at bus near Real-TimeThe energy at the bus associated with the Settlement Meter gathered by the ERCOT Real-Time process for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy consumed. This is the integrated value for the 15 minute interval that is available shortly after the end of the 15 minute interval. |
| MEB gsc, b | MWh | Metered Energy at busThe metered energy by the Settlement Meter me for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy consumed. |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per intervalThe Real-Time LMP for the meter at Electrical Bus b, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per intervalThe duration of the SCED interval y. |
| RNWF b, y | none | Net meter Weighting Factor per intervalThe weight factor used in net meter price calculation for meters in Electrical Bus b, for the SCED interval y. The weighting factor used in the net meter price calculation shall not be recalculated after the fact due to revisions in the association of Resources to Settlement Meters. |
| BP r, y | MW | Base Point per Resource per interval The Base Point of Resource r, for the SCED interval y. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| gsc | none | A generation site code. |
| r | none | A Generation Resource that is located at the facility with net metering. The summation is over all the Generation Resources at the facility. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An Electrical Bus. |

(4)	The Generation Resource SCADA Splitting Percentage for each Resource within a net metering arrangement for the 15-minute Settlement Interval is calculated as follows:

GSPLITPER q,  r, gsc, p = GSSPLITSCA r / ∑r GSSPLITSCA r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| GSPLITPER q, r, gsc, p | none | Generation Resource SCADA Splitting Percentage—The generation allocation percentage for Resource r that is part of a generation site code gsc for the QSE q at Settlement Point p. GSPLITPER is calculated by taking the SCADA values (GSSPLITSCA) for a particular Generation Resource r that is part of a net metering configuration and dividing by the sum of all SCADA values for all Resources that are included in the net metering configuration for each interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| GSSPLITSCA r | MWh | Generation Resource SCADA Net Real Power provided via Telemetry—The net real power provided via telemetry per Resource within the net metering arrangement, integrated for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| gsc | none | A generation site code |
| r | none | A Generation Resource that is located at the facility with net metering. The summation is over all the Generation Resources at the facility. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |

(5)	The total net payments and charges to each QSE for Energy Imbalance Service at all Resource Node Settlement Points for the 15-minute Settlement Interval is calculated as follows:

RTEIAMTQSETOT q	=	Σp RTEIAMT q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTEIAMTQSETOT q | $ | Real-Time Energy Imbalance Amount QSE Total per QSEThe total net payments and charges to QSE q for Real-Time Energy Imbalance Service at all Resource Node Settlement Points for the 15-minute Settlement Interval. |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |

6.6.5	Generation Resource Base Point Deviation Charge

A QSE for a Generation Resource shall pay a Base Point Deviation Charge if the Resource did not follow Dispatch Instructions and Ancillary Services deployments within defined tolerances, except when the Dispatch Instructions and Ancillary Services deployments violate the Resource Parameters.  The Base Point Deviation Charge does not apply to Generation Resources between breaker close and the time at which the telemetered High Sustained Limit (HSL) becomes greater than Low Sustained Limit (LSL).  The desired output from a Generation Resource during a 15-minute Settlement Interval is calculated as follows:

AABP	=	Σy((BP y + BP y-1)/2 * TLMP y) / (ΣyTLMP y) +  TWAR

Where:

TWAR	=	Σy((ARI y * TLMP y) / (ΣyTLMP y)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| AABP | MW | Adjusted Aggregated Base Point—The Generation Resource’s aggregated Base Point adjusted for Ancillary Service deployments, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, AABP is calculated for the Combined Cycle Train considering all SCED Dispatch Instructions to any Combined Cycle Generation Resources within the Combined Cycle Train. |
| BP y | MW | Base Point by interval—The Base Point for the Generation Resource at the Resource Node, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. |
| TWAR | MW | Time-Weighted Average Regulation—The amount of regulation that the Generation Resource should have produced based on the deployment signals as calculated by the Load Frequency Control (LFC) within the 15-minute Settlement Interval. |
| ARI y | MW | Average Regulation Instruction—The amount of regulation that the Generation Resource should have produced based on the deployment signals as calculated by the LFC within the SCED interval. |
| Y | none | A SCED interval in the Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |

6.6.5.1	General Generation Resource Base Point Deviation Charge

(1)	Unless one of the exceptions specified in paragraphs (2) and (3) below applies, ERCOT shall charge a Generation Resource Base Point Deviation Charge for a Generation Resource other than those described in Section 6.6.5.2, IRR Generation Resource Base Point Deviation Charge, and Section 6.6.5.3, Generators Exempt from Deviation Charges, when:

(a)	The Settlement Point Price for the Resource Node is positive; and

(b)	The telemetered generation of the Generation Resource over the 15-minute Settlement Interval is outside the tolerances defined later in this Section 6.6.5.1.

(2)	ERCOT may not charge a QSE a Generation Resource Base Point Deviation Charge under paragraph (1) above when both of the following apply:

(a)	The generation deviation of the Generation Resource over the 15-minute Settlement Interval is in a direction that contributes to frequency corrections that resolve an ERCOT System frequency deviation; and

(b)	The ERCOT System frequency deviation is greater than +/-0.05 Hz at any time during the 15-minute Settlement Interval.

(3)	ERCOT may not charge a QSE a Generation Resource Base Point Deviation Charge under paragraph (1) above for any 15-minute Settlement Interval during which Responsive Reserve is deployed.

6.6.5.1.1	Base Point Deviation Charge for Over Generation

(1)	ERCOT shall charge a QSE for a Generation Resource for over-generation that exceeds the following tolerance.  The tolerance is the greater of:

(a)	Five percent of the average of the Base Points in the Settlement Interval adjusted for any Ancillary Service deployments; or

(b)	Five MW for metered generation above the average of the Base Points in the Settlement Interval adjusted for any Ancillary Service deployments.

(2)	The charge to each QSE for over-generation of each Generation Resource at each Resource Node Settlement Point, if the Real-Time metered generation is greater than the upper tolerance during a given 15-minute Settlement Interval, is calculated as follows:

BPDAMT q, r, p	=	Max (0, RTSPP p) * Max [0, (TWTG q, r, p – ¼ * Max (((1 + K1) * AABP q, r, p), (AABP q, r, p + Q1)))]

Where:

TWTG q, r, p	=	Σy(ATG q, r, p, y * TLMP y / 3600)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| BPDAMT q, r, p | $ | Base Point Deviation Charge per QSE per Settlement Point per Resource—The charge to QSE q for Generation Resource r at Resource Node p, for its deviation from Base Point, for the 15-minute Settlement Interval. The Base Point Deviation Charge is charged to the Combined Cycle Train for all Combined Cycle Generation Resources. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| TWTG q, r, p | MWh | Time-Weighted Telemetered Generation per QSE per Settlement Point per Resource—The telemetered generation of Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| AABP q, r, p | MW | Adjusted Aggregated Base Point per QSE per Settlement Point per Resource—The aggregated Base Point adjusted for Ancillary Service deployments, of Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, AABP is calculated for the Combined Cycle Train considering all SCED Dispatch Instructions to any Combined Cycle Generation Resources within the Combined Cycle Train. |
| ATG q, r, p, y | MW | Average Telemetered Generation—The average telemetered generation of Generation Resource r represented by QSE q at Resource Node p, for the SCED interval. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. |
| K1 | none | The percentage tolerance for over-generation, five percent. |
| Q1 | MW | The MW tolerance for over-generation, five MW. |
| q | none | A QSE. |
| 3600 | none | The number of seconds in one hour. |
| p | none | A Resource Node Settlement Point. |
| r | none | A non-exempt, non-Intermittent Renewable Resource (IRR) Generation Resource. |
| y | none | An Emergency Base Point interval or SCED interval that overlaps the 15-minute Settlement Interval. |

6.6.5.1.2	Base Point Deviation Charge for Under Generation

(1)	ERCOT shall charge a QSE for a Generation Resource for under generation if the metered generation is below the lesser of:

(a)	95% of the average of the Base Points in the Settlement Interval adjusted for any Ancillary Service deployments; or

(b)	The average of the Base Points in the Settlement Interval adjusted for any Ancillary Service deployments minus five MW.

(2)	The charge to each QSE for under-generation of each Generation Resource at each Resource Node Settlement Point for a given 15-minute Settlement Interval is calculated as follows:

BPDAMT q, r, p	=	Max (0, RTSPP p) * Min (1, KP) * Max {0, {Min [((1 – K2) * ¼( AABP q, r, p )) , ¼( AABP q, r, p – Q2 )] – TWTG q, r, p}}

Where:

TWTG q, r, p	=	Σy(ATG q, r, p, y * TLMP y /3600)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| BPDAMT q, r, p | $ | Base Point Deviation Charge per QSE per Settlement Point per Resource—The charge to QSE q for Generation Resource r at Resource Node p, for its deviation from Base Point, for the 15-minute Settlement Interval. A Base Point Deviation Charge is charged to the Combined Cycle Train for all Combined Cycle Generation Resources. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| TWTG q, r, p | MWh | Time-Weighted Telemetered Generation per QSE per Settlement Point per Resource—The telemetered generation of Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| AABP q, r, p | MW | Adjusted Aggregated Base Point—The aggregated Base Point adjusted for Ancillary Service deployments of Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, AABP is calculated for the Combined Cycle Train considering all SCED Dispatch Instructions to any Combined Cycle Generation Resources within the Combined Cycle Train. |
| ATG q, r, p, y | MW | Average Telemetered Generation—The average telemetered generation of Generation Resource r represented by QSE q at Resource Node p, for the SCED interval. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. |
| KP | None | The coefficient applied to the Settlement Point Price for under-generation charge, 1.0. |
| K2 | None | The percentage tolerance for under-generation, five percent. |
| Q2 | MW | The MW tolerance for under-generation, five MW. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | None | A non-exempt, non-IRR Generation Resource. |
| y | none | An Emergency Base Point interval or SCED interval that overlaps the 15-minute Settlement Interval. |

6.6.5.2	IRR Generation Resource Base Point Deviation Charge

(1)	ERCOT shall charge a QSE for an IRR a Base Point Deviation Charge if the IRR metered generation is more than ten percent above its Adjusted Aggregated Base Point and if the Adjusted Aggregated Base Point is two MW or more below the IRR’s HSL.  The deviation charge may be refunded if the IRR shows, to ERCOT’s satisfaction, that the IRR was taking the necessary control actions to produce at levels equal to or less than the Base Point but was unable to comply solely due to increasing renewable energy input.  The IRR must always take the necessary control actions, in its capability, to comply with Base Point Dispatch Instructions if the Base Point is two MW or more below the IRR’s HSL as soon as practicable.

(2)	The charge to each QSE for non-excused over-generation of each IRR at each Resource Node Settlement Point, if the Real-Time metered generation is greater than the upper tolerance during a 15-minute Settlement Interval, is calculated as follows:

If AABP q, r, p > (HSL q, r, p – QIRR)

BPDAMT q, r, p =	0

Otherwise

BPDAMT q, r, p =	Max (0, RTSPP p) *

Max (0, TWTG q, r, p – ¼ *AABP q, r, p * (1 + KIRR))

Where:

TWTG q, r, p	=	Σy(ATG q, r, p, y * TLMP y / 3600)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| BPDAMT q, r, p | $ | Base Point Deviation Charge per QSE per Settlement Point per Resource—The charge to QSE q for Generation Resource r at Resource Node p, for its deviation from Base Point, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Resource Node p, for the 15-minute Settlement Interval. |
| TWTG q, r, p | MWh | Time-Weighted Telemetered Generation per QSE per Settlement Point per Resource—The telemetered generation of Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. |
| AABP q, r, p | MW | Adjusted Aggregated Base Point Generation per QSE per Settlement Point per Resource—The aggregated Base Point adjusted for Ancillary Service deployments, of Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. |
| HSL q, r, p | MW | High Sustained Limit Generation per QSE per Settlement Point per Resource—The HSL of Generation Resource r represented by QSE q at Resource Node p for the hour that includes the 15-minute Settlement Interval. |
| ATG q, r, p, y | MW | Average Telemetered Generation—The average telemetered generation of Generation Resource r represented by QSE q at Resource Node p, for the SCED interval. |
| TLMP y | second | Duration of SCED interval per interval—The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. |
| KIRR |  | The percentage tolerance for over-generation of an IRR, ten percent. |
| QIRR | MW | The threshold to test the adjusted aggregated Base Point against the HSL for an IRR, two MW. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | An IRR Generation Resource. |
| y | none | An Emergency Base Point interval or SCED interval that overlaps the 15-minute Settlement Interval. |

6.6.5.3	Generators Exempt from Deviation Charges

Generation Resource Base Point Deviation Charges do not apply to RMR Units, Dynamically Scheduled Resources (DSRs) (except as described in Section 6.4.2.2, Output Schedules for Dynamically Scheduled Resources), or Qualifying Facilities (QFs) that do not submit an Energy Offer Curve for the Settlement Interval.

6.6.5.4	Base Point Deviation Payment

ERCOT shall pay the Base Point Deviation Charges collected from the QSEs representing Generation Resources to the QSEs representing Load based on LRS.  The payment to each QSE for a given 15-minute Settlement Interval is calculated as follows:

LABPDAMT q	=	(-1) * BPDAMTTOT * LRS q

Where:

BPDAMTTOT	=	ΣqBPDAMTQSETOT q

BPDAMTQSETOT q	=	ΣpΣrBPDAMT q, r, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LABPDAMT q | $ | Load-Allocated Base Point Deviation Amount per QSE—QSE q’s share of the total charge for all Generation Resources’ Base Point deviations, based on LRS for the 15-minute Settlement Interval. |
| BPDAMTTOT | $ | Base Point Deviation Amount Total—The total of Base Point Deviation Charges to all QSEs for all Generation Resources, for the 15-minute Settlement Interval. |
| BPDAMTQSETOT q | $ | Base Point Deviation Amount QSE Total per QSE—The total of Base Point Deviation Charges to QSE q for all Generation Resources represented by this QSE, for the 15-minute Settlement Interval. |
| BPDAMT q, r, p | $ | Base Point Deviation Charge per QSE per Settlement Point per Resource—The charge to QSE q for Generation Resource r at Resource Node p, for its deviation from Base Point, for the 15-minute Settlement Interval. A Base Point Deviation Charge is charged to the Combined Cycle Train for all Combined Cycle Generation Resources. |
| LRS q | none | The LRS calculated for QSE q for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource. |

6.6.6	Reliability Must-Run Settlement

6.6.6.1	RMR Standby Payment

(1)	The Standby Payment for RMR Service is paid to each QSE representing an RMR Unit for each RMR Unit for each contracted hour under performance requirements set forth in Section 22, Attachment B, Standard Form Reliability Must-Run Agreement, and other performance requirements in these Protocols.  For Initial Settlement, the Standby Payment is the “Estimated Standby Cost” stated in the RMR Agreement.  For Final and True-Up Settlements, the Standby Payment is based on the RMR Unit’s actual Eligible Cost.

(2)	The Standby Payment to each QSE for each RMR Unit for each hour is calculated as follows:

RMRSBAMT q, r	=	(-1) * RMRSBPR q, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRSBAMT q, r | $ | Reliability Must Run Standby Payment per QSE per Resource by hour—The Standby Payment to QSE q for RMR Unit r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRSBPR q, r | $ per hour | Reliability Must Run Standby Price per QSE per Resource by hour—The hourly standby cost for RMR Unit r represented by QSE q, for the hour. See item (3) below. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |

(3)	For the Initial Settlement and resettlements executed before true-up and before actual cost data is submitted, the standby price of an RMR Unit is the “Estimated Standby Cost” stated in the RMR Agreement.  For other resettlements, the standby price of an RMR Unit for each hour is calculated as follows:

RMRSBPR q, r	=	RMRMNFC q, r / MH q, r * (1 + RMRIF * RMRCRF q, r * RMRARF q, r)

Where:

RMR Capacity Reduction Factor

If (RMRTCAPA q, r + RMRTCAP q, r ≥ RMRCCAP q, r), then RMRCRF q, r	 =	 1

Otherwise

RMRCRF q, r	=	Max (0, 1 – 2 * (RMRCCAP q, r – RMRTCAP q, r) / RMRCCAP q, r)

RMR Availability Reduction Factor

If (RMRHREAF q, r ≥ RMRTA q, r), then RMRARF q, r	 =	 1

Otherwise

RMRARF q, r	=	Max (0, 1 - (RMRTA q, r – RMRHREAF q, r) * 2)

RMR Hourly Rolling Equivalent Availability Factor

If (RMREH q, r < 4380)

RMRHREAF q, r	=	1

Otherwise

RMRHREAF q, r	=	(Σhhr=h−4379 RMRAFLAG q, r, hr) / 4380

Availability for a Combined Cycle Train will be determined pursuant to contractual terms but no more than once per hour.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRSBPR q, r | $ per hour | Reliability Must-Run Standby Price per QSE per Resource by hour—The hourly standby cost for RMR Unit r represented by QSE q, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRARF q, r | none | Reliability Must-Run Availability Reduction Factor per QSE per Resource by hour—The availability reduction factor of RMR Unit r represented by QSE q, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRCRF q, r | none | - Reliability Must-Run Capacity Reduction Factor per QSE per Resource by hour—The capacity reduction factor of the RMR Unit, for the hour. See paragraph (2) of Section 3.14.1.13, Incentive Factor. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRCCAP q, r | MW | - Reliability Must-Run Contractual Capacity per QSE per Resource—The capacity of RMR Unit r represented by QSE q as specified in the RMR Agreement. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRTCAP q, r | MW | - Reliability Must Run Testing Capacity by hour—The testing capacity of RMR Unit r represented by QSE q, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRTA q, r | none | - Reliability Must-Run Target Availability per QSE per Resource—The Target Availability of RMR Unit r represented by QSE q, as specified in the RMR Agreement and divided by 100 to convert a percentage to a fraction. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRHREAF q, r | none | Reliability Must-Run Hourly Rolling Equivalent Availability Factor per QSE per Resource by hour—The equivalent availability factor of RMR Unit r represented by QSE q over 4380 hours, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMREH q, r | none | - Reliability Must-Run Elapsed number of Hours per QSE per Resource by hour—The number of the elapsed hours of the term of the RMR Agreement for RMR Unit r represented by QSE q, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRMNFC q, r | $ | - Reliability Must-Run Monthly Non-Fuel Cost per QSE per Resource—The actual non-fuel eligible cost of RMR Unit r represented by QSE q, for the month. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| MH q, r | hour | - Number of Hours in the Month per QSE per Resource—The total number of hours of the month, when RMR Unit r represented by QSE q is under an RMR Agreement. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRIF | none | Reliability Must Run Incentive Factor—The Incentive Factor of RMR Units under RMR Agreement. |
| RMRARF q, r | none | - Reliability Must-Run Availability Reduction Factor per QSE per Resource by hour—The availability reduction factor of RMR Unit r represented by QSE q, as calculated for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRAFLAG q, r, hr | none | RMR Availability Flag per QSE per Resource by hour—The flag of the availability of RMR Resource r represented by QSE q, 1 for available and 0 for unavailable, for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RMRTCAPA q, r | MW | Reliability Must-Run Testing Capacity Adjustment by hour—The testing capacity adjustment factor, in the event an ERCOT Operator has deemed that a RMR Unit’s Tested Capacity did not materially affect the reliability of the ERCOT System, of an RMR Unit r represented by QSE q, for the hour. See paragraph (2) of Section 3.14.1.13. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |
| hr | none | The index for a given hour and all the previous 4379 hours. |
| 4380 | none | The number of hours in a six-month period. |

(4)	The total of the Standby Payments to each QSE for all RMR Units represented by this QSE for a given hour is calculated as follows:

RMRSBAMTQSETOT q	=	ΣrRMRSBAMT q, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRSBAMTQSETOT q | $ | Reliability Must-Run Standby Amount QSE Total per QSEThe total of the Standby Payments to QSE q for all RMR Units represented by this QSE for the hour. |
| RMRSBAMT q, r | $ | Reliability Must-Run Standby Payment per QSE per Resource—The Standby Payment to QSE q for RMR Unit r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |

6.6.6.2	RMR Payment for Energy

(1)	Payment for energy on the Initial Settlement and settlements executed before true-up and before actual cost data is submitted must be calculated using the estimated input/output curve and startup fuel as specified in the RMR Agreement, the actual energy produced and the FIP.  The payment for energy for all other settlements must be based on actual fuel costs for the RMR Unit.  The payment for energy for each hour is calculated as follows:

RMREAMT q, r	=	(-1) * ((FIP + RMRCEFA q, r) * RMRSUFQ q, r / RMRH q, r) * RMRALLOCFLAG q, r + Σ4i=1(((FIP + RMRCEFA q, r) * RMRHR q, r, i + RMRVCC q, r) * RTMG q, r, i)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMREAMT q, r | $ | Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FIP | $/MMBtu | Fuel Index Price—The FIP for the Operating Day. |
| RMRSUFQ q, r | MMBtu | Reliability Must-Run Startup Fuel Quantity per QSE per ResourceThe Estimated Start Up Fuel specified in the RMR Agreement for RMR Unit r represented by QSE q. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRH q, r, h | hour | Reliability Must-Run Hours—The number of hours during which RMR Unit r represented by QSE q is instructed On-Line for the Operating Day. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRALLOCFLAG q, r | none | Reliability Must Run Startup Flag per QSE per Resource by hour—The number that indicates whether or not the startup fuel cost of RMR Unit r represented by QSE q is allocated to the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. The startup fuel cost will be allocated equally to all contiguous intervals for which there is an eligible start. The RMRALLOCFLAG q, r value is 1 if the startup fuel cost is allocated; otherwise, its value is 0. The RMRALLOCFLAG q, r for eligibility is determined in Sections 5.6.2, RUC Startup Cost Eligibility, and 5.6.3, Forced Outage of a RUC-Committed Resource, for startup payments and commitments in either the Reliability Unit Commitment (RUC) or DAM. |
| RMRHR q, r, i | MMBtu /MWh | Reliability Must-Run Heat Rate per QSE per Resource by Settlement Interval by hour—The multiplier determined based on the input/output curve and the Real-Time generation of RMR Unit r represented by QSE q, for the 15-minute Settlement Interval i in the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRVCC q, r | $/MWh | Reliability Must-Run Variable Cost Component per QSE per Resource—The monthly cost component that is used to adjust the energy cost calculation to reflect the actual fuel costs of RMR Unit r represented by QSE q. The value is initially set to zero. For resettlements, see item (2) below. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTMG q, r, i | MWh | Real-Time Metered Generation per QSE per Resource by Settlement Interval by hour—The Real-Time energy from RMR Unit r represented by QSE q, for the 15-minute Settlement Interval i in the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRCEFA q, r | $/MMBtu | Reliability Must-Run Contractual Estimated Fuel Adder—The Estimated Fuel Adder that is contractually agreed upon in Section 22, Attachment B, Standard Form Reliability Must-Run Agreement. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |
| i | none | A 15-minute Settlement Interval. |

(2)	If the RMR actual fuel cost is filed in accordance with the timeline in these Protocols, the monthly RMR variable cost component is calculated for the subsequent resettlements as follows:

RMRVCC q, r	=	(RMRMFCOST q, r + ΣhRMREAMT q, r, f, h) /

(ΣiRTMG q, r, i)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRVCC q, r | $/MWh | Reliability Must-Run Variable Cost Component per QSE per Resource—The monthly cost component that is used to adjust the energy cost calculation to reflect the actual fuel costs of RMR Unit r represented by QSE q. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRMFCOST q, r | $ | Reliability Must-Run Monthly actual Fuel Cost per QSE per Resource—The monthly actual fuel cost of RMR Unit r represented by QSE q, for the month. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTMG q, r, i | MWh | Real-Time Metered Generation per QSE per Resource by Settlement Interval—The Real-Time energy from RMR Unit r represented by QSE q for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMREAMT q, r, f, h | $ | Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour h from the former Settlement Statement f. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |
| h | none | An hour in the month. |
| i | none | A 15-minute Settlement Interval in the month. |
| f | none | Amount from former settlement run. |

(3)	The total of the payments for energy to each QSE for all RMR Units represented by this QSE for a given hour is calculated as follows:

RMREAMTQSETOT q	=	ΣrRMREAMT q, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMREAMTQSETOT q | $ | Reliability Must-Run Energy Amount QSE Total per QSEThe total of the energy payments to QSE q for all RMR Units represented by this QSE for the hour. |
| RMREAMT q, r | $ | Reliability Must-Run Energy Amount per QSE per Resource by hour—The energy payment to QSE q for RMR Unit r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |

6.6.6.3	RMR Adjustment Charge

(1)	Each QSE that represents an RMR Unit shall pay a charge designed to recover the net total revenues from RUC settlements, and from Real-Time settlements received by that QSE for all RMR Units that it represents, except that the charge does not include net revenues received by the QSE for the RMR Standby Payments calculated under Section 6.6.6.1, RMR Standby Payment, and the RMR energy payments calculated under Section 6.6.6.2, RMR Payment for Energy.

(2)	The charge for each QSE representing an RMR Unit for a given Operating Hour is calculated as follows:

RMRAAMT q	=	(-1) * [ΣpΣr(((-1) *Σ4i=1(RTMG q, r, p, i * RTSPP p, i)) + Σ4i=1EMREAMT q, r, p, i + RUCMWAMT q, r, p + RUCCBAMT q, r, p + RUCDCAMT q, r, p + Σ4i=1VSSEAMT q, r, p, i + Σ4i=1VSSVARAMT q, r, i)]

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRAAMT q | $ | RMR Adjustment Charge per QSE—The adjustment from QSE q Standby Payments and energy payments for all RMR Units represented by this QSE, for the revenues received for the same RMR Units from RUC and Real-Time operations, for the hour. |
| RTEIAMT q, p, i | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval. |
| EMREAMT q, r, p, i | $ | Emergency Energy Amount per QSE per Settlement Point per unit per interval—The payment to QSE q for the additional energy produced by RMR Unit r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval i. Payment for emergency energy is made to the Combined Cycle Train. |
| RUCMWAMT q, r, p | $ | RUC Make-Whole Amount per QSE per Settlement Point per unit—The amount calculated for RMR Unit r committed in RUC at Resource Node p to make whole the Startup Cost and minimum-energy cost of this unit, for the hour. See Section 5.7.1, RUC Make-Whole Payment. When one or more Combined Cycle Generation Resources are committed by RUC, payment is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCCBAMT q, r | $ | RUC Clawback Charge per QSE per unit—The RUC Clawback Charge to QSE q for RMR Unit r, for the hour. See Section 5.7.2, RUC Clawback Charge. When one or more Combined Cycle Generation Resources are committed by RUC, a charge is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCDCAMT q, r, p | $ | RUC Decommitment Amount per QSE per Settlement Point per unit—The amount calculated for RMR Unit r at Resource Node p represented by QSE q due to ERCOT de-commitment, for the hour. When one or more Combined Cycle Generation Resources are decommitted by RUC, payment is made to the Combined Cycle Train for all RUC-decommitted Combined Cycle Generation Resources. |
| VSSEAMT q, r, p, i | $ | Voltage Support Service Energy Amount per QSE per Settlement Point per unit per interval—The compensation to QSE q for ERCOT-directed power reduction from RMR Unit r at Resource Node p to provide Voltage Support Service (VSS), for the 15-minute Settlement Interval i. Payment for VSS is made to the Combined Cycle Train. |
| VSSVARAMT q, r, i | $ | Voltage Support Service VAr Amount per QSE per Unit—The payment to QSE q for the VSS provided by RMR Unit r, for the 15-minute Settlement Interval i. Payment for VSS is made to the Combined Cycle Train. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| RTMG q, r, p | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource—The Real-Time energy produced by the Generation Resource r represented by QSE q at Resource Node p, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | An RMR Unit. |
| i | none | A 15-minute Settlement Interval in the hour. |

6.6.6.4	RMR Charge for Unexcused Misconduct

(1)	If a Misconduct Event, as defined in the RMR Agreement, is not excused as provided in the RMR Agreement, then ERCOT shall charge the QSE that represents the RMR Unit an unexcused misconduct amount of $10,000 for each unexcused Misconduct Event as follows:

RMRNPAMT q, r	=	$10,000 * RMRNPFLAG q, r

The above variable is defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRNPAMT q, r | $ | Reliability Must Run Unexcused Misconduct Charge per QSE per Resource—The charge to QSE q for the unexcused Misconduct Event of RMR Unit r for an Operating Day. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RMRNPFLAG q, r | $ | Reliability Must-Run Non-Performance Flag per QSE per Resource—A flag for the QSE q for the unexcused Misconduct Event of RMR Unit r for an Operating Day. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |

(2)	The total of the charges to each QSE for unexcused Misconduct Events of all RMR Units represented by this QSE for a given Operating Day is calculated as follows:

RMRNPAMTQSETOT q	=	ΣrRMRNPAMT q, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRNPAMTQSETOT q | $ | Reliability Must-Run Unexcused Misconduct Amount QSE Total per QSEThe total of the charges to QSE q for unexcused Misconduct Events of the RMR Units represented by this QSE for the Operating Day. |
| RMRNPAMT q, r | $ | Reliability Must-Run Unexcused Misconduct Charge per QSE per Resource—The charge to QSE q for the unexcused Misconduct Event of RMR Unit r for the Operating Day. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | An RMR Unit. |

6.6.6.5	RMR Service Charge

The total RMR cost for all RMR Units less the amount received from DAM, RUC processes and Real-Time operations for all RMR Units is allocated to the QSEs representing Loads based on LRS.  The RMR Service charge to each QSE for a given hour is calculated as follows:

LARMRAMT q	=	(-1) * (RMRSBAMTTOT + RMREAMTTOT 
+ RMRAAMTTOT – Σ4i=1RMRDAESRTVTOT i – (RMRDAEREVTOT + RMRDAMWREVTOT) + RMRNPAMTTOT / H) * HLRS q

Where:

RMR Standby Amount Total

RMRSBAMTTOT	=	ΣqRMRSBAMTQSETOT q

RMR Energy Amount Total

RMREAMTTOT	=	ΣqRMREAMTQSETOT q

RMR Adjustment Charge Total

RMRAAMTTOT	=	ΣqRMRAAMT q

RMR Non-Performance Amount Total

RMRNPAMTTOT	=	ΣqRMRNPAMTQSETOT q

Total Day-Ahead energy revenue for all RMR Units

RMRDAEREVTOT	=	ΣqΣrΣpDAEREV q, r, p

DAEREV q, r, p		=	(-1) * DASPP p * DAESR q, r, p

Total Real-Time value of Day-Ahead energy for all RMR Units by interval

RMRDAESRTVTOT i	=	ΣqΣrΣpDAESRTV q, r, p, i

DAESRTV q, r, p, i	=	RTSPP p, i * (DAESR q, r, p * ¼)

Total Real-Time value of Day-Ahead Make-Whole Revenue for all RMR units by interval

RMRDAMWREVTOT i	=	DAMWRMRREVQSETOT

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LARMRAMT q | $ | Load-Allocated Reliability Must-Run Amount per QSE—The amount charged to QSE q based on its LRS of the difference between the amount paid to all QSEs for RMR Service under Section 6.6.6, Reliability Must-Run Settlement, and the amount that would have been paid to the QSEs for the same RMR Units if they were not providing RMR Service under the other parts of this Section 6, Adjustment Period and Real-Time Operations, Section 5, Transmission Security Analysis and Reliability Unit Commitment, and Section 4, Day-Ahead Operations, for the hour. |
| RMRSBAMTTOT | $ | RMR Standby Amount Total—The total of the Standby Payments to all QSEs for all RMR Units, for the hour. |
| RMREAMTTOT | $ | RMR Energy Amount Total—The total of the energy cost payments to all QSEs for all RMR Units, for the hour. |
| RMRAAMTTOT | $ | RMR Adjusted Amount Total—The total of the adjusted amounts from all QSEs representing RMR Units for the revenues received for these units from RUC, Real-Time operations and Ancillary Service markets, for the hour. |
| RMRNPAMTTOT | $ | RMR Non-Performance Amount Total—The total of the charges to all QSEs for unexcused Misconduct Events of all RMR Units, for the Operating Day. |
| RMRDAEREVTOT | $ | RMR Day-Ahead Energy Revenue Total—The total of the revenues for the offers cleared in the DAM for all RMR Units, for the hour. |
| RMRDAESRTVTOT | $ | RMR Day-Ahead Energy Sale Real-Time Value Total—The total of the Real-Time value of the offers cleared in the DAM for all RMR Units, for the hour. |
| RMRDAMWREVTOT | $ | RMR Day-Ahead Make-Whole Revenue TotalThe total of the RMR Day-Ahead Make-Whole Revenue for all DAM-committed RMR Units for the hour. |
| HLRS q | none | The hourly LRS calculated for QSE q for the hour. See Section 6.6.2.3, QSE Load Ratio Share for an Operating Hour. |
| RMRSBAMTQSETOT q | $ | Reliability Must-Run Standby Amount QSE Total per QSEThe total of the Standby Payments to QSE q for the RMR Units represented by the same QSE for the hour. |
| RMREAMTQSETOT q | $ | Reliability Must-Run Energy Amount QSE Total per QSEThe total of the energy payments to QSE q for the RMR Units represented by the same QSE for the hour. |
| RMRAAMT q | $ | RMR Adjusted Amount per QSE—The adjustment from QSE q Standby Payments and energy payments for all RMR Units represented by this QSE, for the revenues received for the same RMR Units from RUC and Real-Time operations, for the hour. |
| RMRNPAMTQSETOT q | $ | Reliability Must-Run Unexcused Misconduct Amount QSE Total per QSEThe total of the charges to QSE q for unexcused Misconduct Events of the RMR Units represented by the same QSE for the Operating Day. |
| DAEREV q, r, p | $ | Day-Ahead Energy Revenue per QSE by Settlement Point per unitThe revenue that ERCOT collects for the offer cleared in the DAM submitted for RMR Unit r at Resource Node p represented by QSE q, based on the DAM Settlement Point Price, for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DAESRTV q, r, p, i | $ | Day-Ahead Energy Sale Real-Time Value per QSE per Settlement Point per unit per interval—The Real-Time value of the energy sold in the DAM from RMR Unit r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DASPP p | $/MWh | Day-Ahead Settlement Point Price by Settlement PointThe DAM Settlement Point Price at Resource Node p for the hour. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price per Settlement Point per interval—The Real-Time Settlement Point Price at Resource Node p, for the 15-minute Settlement Interval i. |
| DAESR q, r, p | MW | Day-Ahead Energy Sale from Resource per QSE by Settlement Point per unitThe amount of energy cleared through Three-Part Supply Offers in the DAM and/or DAM Energy-Only Offer Curves for RMR Unit r at Resource Node p represented by QSE q for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DAMWRMRREVQSETOT | $ | Day-Ahead Make-Whole RMR Revenue QSE Total per QSEThe total of the Day-Ahead Make-Whole Revenue calculated for QSE q for DAM-committed RMR Units represented by this QSE for the hour. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | An RMR Unit. |
| i | none | A 15-minute Settlement Interval in the hour. |
| H | none | The number of hours of the Operating Day. |

6.6.7.1	Voltage Support Service Payments

(1)	All other Generation Resources shall be eligible for compensation for Reactive Power production in accordance with Section 6.5.7.7, Voltage Support Service, only if ERCOT issues a Dispatch Instruction that results in the following unit operation:

(a)	When ERCOT instructs the Generation Resource to exceed its Unit Reactive Limit (URL) and the Generation Resource provides additional Reactive Power, then ERCOT shall pay for the additional Reactive Power provided at a price that recognizes the avoided cost of reactive support Resources on the transmission network.

(b)	Any real power reduction directed by ERCOT through (VDIs) to provide for additional reactive capability for voltage support must be compensated as a lost opportunity payment

(2)	The payment for a given 15-minute Settlement Interval to each QSE representing a Generation Resource that operates in accordance with an ERCOT Dispatch Instruction is calculated as follows:

Depending on the Dispatch Instruction, payment for Volt Amps Reactive (var):

If VSSVARLAG q, r > 0

VSSVARAMT q, r	=	(-1) * VSSVARPR * VSSVARLAG q, r

If VSSVARLEAD q, r > 0

VSSVARAMT q, r	=	(-1) * VSSVARPR * VSSVARLEAD q, r

Where:

VSSVARLAG q, r	=	Max [0, Min (¼ * VSSVARIOL q, r, RTVAR q, r) – (¼ * URLLAG q, r)]

VSSVARLEAD q, r	=	Max {0, [(¼ * URLLEAD q, r ) – Max ((¼ * VSSVARIOL q, r), RTVAR q, r)]}

URLLAG q,r	=	0.32868 * HSL q,r

URLLEAD q,r	=	(-1) * 0.32868 * HSL q,r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| VSSVARAMT q, r | $ | Voltage Support Service var Amount per QSE per Generation Resource - The payment to QSE q for the VSS provided by Generation Resource r, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train. |
| VSSVARPR | $/Mvarh | Voltage Support Service var Price - The price for instructed Mvar beyond a Generation Resource’s URL currently is $2.65/Mvarh (based on $50.00/installed kvar). |
| VSSVARLAG q, r | Mvarh | Voltage Support Service var Lagging per QSE per Generation Resource - The instructed portion of the Reactive Power above the Generation Resource’s lagging URL for Generation Resource r represented by QSE q, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train. |
| VSSVARLEAD q, r | Mvarh | Voltage Support Service var Leading per QSE per Generation Resource - The instructed portion of the Reactive Power below the Generation Resource’s leading URL for Generation Resource r represented by QSE q, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train. |
| VSSVARIOL q, r | Mvar | Voltage Support Service var Instructed Output Level per QSE per Generation Resource—The instructed Reactive Power output level of Generation Resource r represented by QSE q, lagging Reactive Power if positive and leading Reactive Power if negative, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train. |
| RTVAR q, r | MVARh | Real-Time var per QSE per Resource—The netted Reactive Energy measured for Generation Resource r represented by QSE q, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train. |
| URLLAG q, r | Mvar | Unit Reactive Limit Lagging per QSE per Resource—The URL for lagging Reactive Power of the Generation Resource r represented by QSE q as determined in accordance with these Protocols. Its value is positive. Where for a combined cycle resource, r is a Combined Cycle Train. |
| URLLEAD q, r | Mvar | Unit Reactive Limit Leading per QSE per Resource—The URL for leading Reactive Power of the Generation Resource r represented by QSE q as determined in accordance with these Protocols. Its value is negative. Where for a combined cycle resource, r is a Combined Cycle Train. |
| HSL q,r | MW | High Sustained Limit— The (HSL) of a Generation Resource as defined in Section 2, Definitions, for the hour that includes the Settlement Interval i. Where for a combined cycle resource, r is a Combined Cycle Generation Resource. |
| q | none | A QSE. |
| r | none | A Generation Resource. |

(3)	The total additional compensation to each QSE for voltage support service for the 15-minute Settlement Interval is calculated as follows:

VSSVARAMTQSETOTq =  r VSSVARAMTq,r

| Variable | Unit | Definition |
| --- | --- | --- |
| VSSVARAMT q, r | $ | Voltage Support Service var Amount per QSE per Generation Resource - The payment to QSE q for the VSS provided by Generation Resource r, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train. |
| VSSVARAMTQSETOT q | $ | Voltage Support var Amount QSE total per QSE - The total of the payments to QSE q as compensation for VSS by this QSE for the 15-minute settlement interval. |
| q | none | A QSE. |
| r | none | A Generation Resource. |

(4)	The lost opportunity payment, if applicable:

VSSEAMT q, r		=	(-1) * Max (0, RTSPP p * Max (0, (HSL q, r * ¼ - RTMG q, r)) – (RTICHSL q, r – RTVSSAIEC q, r * (RTMG q, r - LSL q, r * ¼)))

Where:

RTICHSL q, r		=	RTHSLAIEC q, r * (¼ * HSL q, r – ¼ * LSL q, r)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| VSSEAMT q, r | $ | Voltage Support Service Energy Amount per QSE per Generation Resource—The lost opportunity payment to QSE q for ERCOT-directed VSS from Generation Resource r for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train. |
| RTMG q, r | MWh | Real-Time Metered Generation per QSE per Resource—The Real-Time metered generation of Generation Resource r represented by QSE q, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train. |
| RTSPPp | $ | Real-Time Settlement Point Price—The Real-Time SPP at the Resource Node for the 15-minute Settlement Interval. - |
| RTVSSAIEC q, r | $/MWh | Real-Time Average Incremental Energy Cost per QSE per Resource—The average incremental cost to operate (not subject to cost cap) the Generation Resource r represented by QSE q from its LSL to its metered MW output, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Generation Resource. |
| RTICHSL q, r | $ | Real-Time Incremental Cost Corresponding with HSL per QSE per Resource—The incremental cost to operate (not subject to cost cap) Generation Resource r represented by QSE q from its LSL to its HSL, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Generation Resource. |
| RTHSLAIEC q, r | $/MWh | Real-Time Average Incremental Energy Cost for the entire Energy Offer Curve through the HSL per QSE per Resource—The average incremental cost to operate (not subject to cost cap) the Generation Resource r represented by QSE q from its LSL to its HSL, for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Generation Resource. |
| HSL q, r | MW | High Sustainable Limit Generation per QSE per Settlement Point per Resource—The High Sustainable Limit of Generation Resource r represented by QSE q at Resource Node p for the hour that includes the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Generation Resource. |
| LSL q, r | MW | Low Sustainable Limit Generation per QSE per Settlement Point per Resource—The Low Sustainable Limit of Generation Resource r represented by QSE q at Resource Node p for the hour that includes the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Generation Resource. |
| q | none | A QSE. |
| r | none | A Generation Resource. |
| p | none | A Resource Node Settlement Point. |

(5)	The total of the payments to each QSE for ERCOT-directed power reduction to provide VSS for a given 15-minute Settlement Interval is calculated as follows:

VSSEAMTQSETOT q	=	rVSSEAMT q,r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| VSSEAMTQSETOT q | $ | Voltage Support Service Lost Opportunity Amount QSE Total per QSEThe total of the lost opportunity payments to QSE q for providing VSS for providing ERCOT-directed VSS for the 15-minute Settlement Interval. |
| VSSEAMT q, r | $ | Voltage Support Service Energy Amount per QSE per Settlement Point per Generation Resource—The lost opportunity payment to QSE q for ERCOT-directed VSS from Generation Resource r for the 15-minute Settlement Interval for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A Generation Resource. |

6.6.8	Black Start Capacity

6.6.8.1	Black Start Capacity Payment

(1)	ERCOT shall pay an hourly standby fee to QSEs representing Black Start Resources.  This standby fee is determined through a competitive annual bidding process, with an adjustment for reliability based on a six-month rolling availability equal to 85% in accordance with Section 22, Attachment D, Standard Form Black Start Agreement.

(2)	ERCOT shall pay a Black Start standby payment to each QSE for each Black Start Resource.  The payment for each hour is calculated as follows:

BSSAMT q, r	=	(-1) * BSSPR q, r * BSSARF q, r

Where:

Black Start Service Availability Reduction Factor

If (BSSHREAF q, r  0.85)

BSSARF q, r 	= 	1

Otherwise

BSSARF q, r 	= 	Max (0, 1 - (0.85 - BSSHREAF q, r) * 2)

Black Start Service Hourly Rolling Equivalent Availability Factor

If (BSSEH q, r < 4380)

BSSHREAF q, r	=	1

Otherwise

BSSHREAF q, r	=	(Σhhr=h−4379 BSSAFLAG q,r,hr) / 4380

Availability for a Combined Cycle Train will be determined pursuant to contractual terms but no more than once per hour.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| BSSAMT q, r | $ | Black Start Service Amount per QSE per Resource by hour—The standby payment to QSE q for the Black Start Service (BSS) provided by Resource r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| BSSPR q, r | $ per hour | Black Start Service Price per QSE per Resource—The standby price of BSS Resource r represented by QSE q, as specified in the Black Start Agreement. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| BSSARF q, r | none | Black Start Service Availability Reduction Factor per QSE per Resource by hour—The availability reduction factor of Resource r represented by QSE q under the Black Start Agreement, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| BSSHREAF q, r | none | Black Start Service Hourly Rolling Equivalent Availability Factor per QSE per Resource by hour—The equivalent availability factor of the BSS Resource r represented by QSE q over 4,380 hours, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| BSSEH q, r | none | Black Start Service Elapsed number of Hours per QSE per Resource by hour—The number of the elapsed hours of BSS Resource r represented by QSE q since the beginning of the BSS Agreement, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| BSSAFLAG q, r, hr | none | Black Start Service Availability Flag per QSE per Resource by hour—The flag of the availability of BSS Resource r represented by QSE q, 1 for available and 0 for unavailable, for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A BSS Resource. |
| hr | none | The index of a given hour and the previous 4379 hours. |
| 4380 | none | The number of hours in a six-month period. |

(3)	The total of the payments to each QSE for all BSS Resources represented by this QSE for a given hour is calculated as follows:

BSSAMTQSETOT q	=	ΣrBSSAMT q, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| BSSAMTQSETOT q | $ | Black Start Service Amount QSE Total per QSEThe total of the payments to QSE q for BSS provided by all the BSS Resources represented by this QSE for the hour h. |
| BSSAMT q, r | $ | Black Start Service Amount per QSE per Resource—The standby payment to QSE q for BSS provided by Resource r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A BSS Resource. |

6.6.9.1	Payment for Emergency Power Increase Directed by ERCOT

(1)	If the Emergency Base Point issued to a Generation Resource is higher than the SCED Base Point immediately before the Emergency Condition, then ERCOT shall pay the QSE an additional compensation for the Resource at its Resource Node Settlement Point.  The payment for a given 15-minute Settlement Interval is calculated as follows:

EMREAMT q, r, p	=	(-1) * EMREPR q, r, p * EMRE q, r, p

Where:

EMREPR q, r, p		=	Max (0, EBPWAPR q, r, p – RTSPP p)

EBPWAPR q, r, p	=	Σy(EBPPR q, r, p, y * EBP q, r, p, y * TLMP y) /

Σy(EBPq, r, p, y * TLMP y)

EMRE q, r, p	=	Max (0, Min (AEBP q, r, p , RTMG q, r, p) – ¼ * BP q, r, p)

AEBP q, r, p		=	ΣyΣy(EBP q, r, p, y * TLMPy / 3600)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EMREAMT q, r, p | $ | Emergency Energy Amount per QSE per Settlement Point per Resource—The payment to QSE q as additional compensation for the additional energy produced by Generation Resource r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| EMREPR q, r, p | $/MWh | Emergency Energy Price per QSE per Settlement Point per Resource—The compensation rate for the additional energy produced by Generation Resource r at Resource Node p represented by QSE q in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| EMRE q, r, p | MWh | Emergency Energy per QSE per Settlement Point per Resource—The additional energy produced by Generation Resource r at Resource Node p represented by QSE q in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| EBPWAPR q, r, p | $/MWh | Emergency Base Point Weighted Average Price per QSE per Settlement Point per Resource—The weighted average of the energy prices corresponding with the Emergency Base Points on the Energy Offer Curve for Resource r at Resource Node p represented by QSE q, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| BP q, r, p | MW | Base Point per QSE per Settlement Point per Resource—The Base Point of Resource r at Resource Node p represented by QSE q from the SCED prior to the Emergency Condition. For a Combined Cycle Train, the Resource r must be one of the registered Combined Cycle Generation Resources within the Combined Cycle Train. |
| AEBP q, r, p | MWh | Aggregated Emergency Base Point—The Generation Resource’s aggregated Emergency Base Point, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, AEBP is calculated for the Combined Cycle Train considering all emergency Dispatch Instructions to any Combined Cycle Generation Resources within the Combined Cycle Train. |
| EBP q, r, p, y | MW | Emergency Base Point per QSE per Settlement Point per Resource by interval—The Emergency Base Point of Resource r at Resource Node p represented by QSE q for the Emergency Base Point interval or SCED interval y. If a Base Point instead of an Emergency Base Point is effective during the interval y, its value equals the Base Point. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| EBPPR q, r, p, y | $/MWh | Emergency Base Point Price per QSE per Settlement Point per Resource by interval—The average incremental energy cost calculated per the Energy Offer Curve for the output levels between the SCED Base Point immediately before the Emergency Condition and the Emergency Base Point of Resource r at Resource Node p represented by QSE q for the Emergency Base Point interval or SCED interval y. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| RTMG q, r, p | MWh | Real-Time Metered Generation per QSE per Settlement Point per Resource—The metered generation of Resource r at Resource Node p represented by QSE q in Real-Time for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| TLMP y | second | Duration of Emergency Base Point interval or SCED interval per interval—The duration of the portion of the Emergency Base Point interval or SCED interval y within the 15-minute Settlement Interval. |
| Q | none | A QSE. |
| P | none | A Resource Node Settlement Point. |
| R | none | A Generation Resource. |
| Y | none | An Emergency Base Point interval or SCED interval that overlaps the 15-minute Settlement Interval. |
| 3600 | none | The number of seconds in one hour. |

(2)	The extension of the Energy Offer Curve is used to calculate the Emergency Base Point Price.  If the Emergency Base Point MW value is greater than the largest MW value on the Energy Offer Curve submitted by the QSE for the Resource, then the Energy Offer Curve is extended to the Emergency Base Point MW value with a $/MWh value that is the Mitigated Offer Cap (pursuant to Section 4.4.9.4.1, Mitigated Offer Cap) for the highest MW output on the Energy Offer Curve submitted by the QSE for the Resource.

Q1       Q2         SCED          Q3         EBP        MW

$/

MWh

P 3

P2

P1

The area under the capped Energy Offer Curve equals

(EBPPR * (EBP – SCED BP))

Mitigated Offer Cap

Extended portion of Energy Offer Curve

Q1       Q2        SCED             Q3         EBP        MW

$/

MWh

P 3

P2

P1

The area under the capped Energy Offer Curve equals

(EBPPR * (EBP – SCED BP))

Mitigated Offer Cap

Extended portion of Energy Offer Curve

(3)	The total additional compensation to each QSE for emergency power increases of Generation Resources for the 15-minute Settlement Interval is calculated as follows:

EMREAMTQSETOT q	=	ΣrΣpEMREAMT q, r, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| EMREAMTQSETOT q | $ | Emergency Energy Amount QSE Total per QSEThe total of the payments to QSE q as additional compensation for emergency power increases of the Generation Resources represented by this QSE for the 15-minute Settlement Interval. |
| EMREAMT q, r, p | $ | Emergency Energy Amount per QSE per Settlement Point per Resource—The payment to QSE q as additional compensation for the additional energy produced by Generation Resource r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| Q | none | A QSE. |
| P | none | A Resource Node Settlement Point. |
| R | none | A Generation Resource. |

6.7	Real-Time Settlement Calculations for the Ancillary Services

6.7.1	Payments for Ancillary Service Capacity Sold in a Supplemental Ancillary Service Market

If a Supplemental Ancillary Services Market (SASM) is executed for one or more Operating Hours for any reason, ERCOT shall pay Qualified Scheduling Entities (QSEs) for their Ancillary Service Offers cleared in the SASM, based on the Market Clearing Price for Capacity (MCPC) for that SASM and that service.  By service and by SASM, the payment to each QSE for a given Operating Hour is calculated as follows:

(1)	For Regulation Up (Reg-Up), if applicable:

RTPCRUAMT q, m	=	(-1) * MCPCRU m * RTPCRU q, m

Where:

RTPCRU q, m	=	ΣrPCRUR  q, r, m

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTPCRUAMT q, m | $ | Procured Capacity for Reg-Up Amount by QSE by market—The payment to QSE q for the Ancillary Service Offers cleared in the market m to provide Reg-Up, for the hour. |
| MCPCRU m | $/MW per hour | Market Clearing Price for Capacity for Reg-Up by market—The MCPC for Reg-Up from the market m, for the hour. |
| RTPCRU q, m | MW | Procured Capacity for Reg-Up by QSE by market—The portion of QSE q’s Ancillary Service Offers cleared in the market m to provide Reg-Up, for the hour. |
| PCRUR q, r, m | MW | Procured Capacity for Reg-Up from Resource per Resource per QSE by market—The Reg-Up capacity quantity awarded to QSE q in the market m for Resource r for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| m | none | A SASM. |
| q | none | A QSE. |
| r | none | A Generation Resource. |

(2)	For Regulation Down (Reg-Down), if applicable:

RTPCRDAMT q, m	=	(-1) * MCPCRD m * RTPCRD q, m

Where:

RTPCRD q, m	=	ΣrPCRDR r, q, m

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTPCRDAMT q, m | $ | Procured Capacity for Reg-Down Amount by QSE by market—The payment to QSE q for the Ancillary Service Offers cleared in the market m to provide Reg-Down, for the hour. |
| MCPCRD m | $/MW per hour | Market Clearing Price for Capacity for Reg-Down by market—The MCPC for Reg-Down from the market m, for the hour. |
| RTPCRD q, m | MW | Procured Capacity for Reg-Down by QSE by market—The portion of QSE q’s Ancillary Service Offers cleared in the market m to provide Reg-Down, for the hour. |
| PCRDR r, q, m | MW | Procured Capacity for Reg-Down from Resource per Resource per QSE by market—The Reg-Down capacity quantity awarded to QSE q in the market m for Resource r for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| m | none | A SASM. |
| q | none | A QSE. |
| r | none | A Generation Resource. |

(3)	For Responsive Reserve, if applicable:

RTPCRRAMT q, m	=	(-1) * MCPCRR m * RTPCRR q, m

Where:

RTPCRR q, m	=	ΣrPCRRR  q, r, m

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTPCRRAMT q, m | $ | Procured Capacity for Responsive Reserve Amount by QSE by market—The payment to QSE q for the Ancillary Service Offer cleared in the market m to provide Responsive Reserve, for the hour. |
| MCPCRR m | $/MW per hour | Market Clearing Price for Capacity for Responsive Reserve by market—The MCPC for Responsive Reserve from the market m, for the hour. |
| RTPCRR q, m | MW | Procured Capacity for Responsive Reserve by QSE by market—The portion of QSE q Ancillary Service Offers cleared in the market m to provide Responsive Reserve, for the hour. |
| PCRRR q,r, m | MW | Procured Capacity for Responsive Reserve from Resource per Resource per QSE by market—The Responsive Reserve capacity quantity awarded to QSE q in the market m for Resource r for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| m | none | A SASM. |
| q | none | A QSE. |
| r | none | A Generation Resource. |

(4)	For Non-Spin, if applicable:

RTPCNSAMT q, m	=	(-1) * MCPCNS m * RTPCNS q, m

Where:

RTPCNS q, m	=	ΣrPCNSR  q, r, m

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTPCNSAMT q, m | $ | Procured Capacity for Non-Spin Amount by QSE by market—The payment to QSE q for Ancillary Service Offer cleared in the market m to provide Non-Spin, for the hour. |
| MCPCNS m | $/MW per hour | Market Clearing Price for Capacity for Non-Spin by market—The MCPC for Non-Spin from the market m, for the hour. |
| RTPCNS q, m | MW | Procured Capacity for Non-Spin by QSE by market—The portion of QSE q’s Ancillary Service Offer cleared in the market m to provide Non-Spin, for the hour. |
| PCNSR q, r, m | MW | Procured Capacity for Non-Spin from Resource per Resource per QSE by market—The Non-Spin capacity quantity awarded to QSE q in the market m for Resource r for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| m | none | A SASM. |
| q | none | A QSE. |
| r | none | A Generation Resource. |

7.9.3.1	DAM Congestion Rent

(1)	The DAM congestion rent is calculated as the sum of the following payments and charges:

(a)	The total of payments to all QSEs for cleared DAM energy offers (this does not include any revenue calculated for an RMR Unit, even though its Three-Part Supply Offer was cleared in the DAM), whether through Three-Part Supply Offers or through DAM Energy-Only Offer Curves, calculated under Section 4.6.2.1, Day-Ahead Energy Payment;

(b)	The total of revenue for all RMR Units as calculated below;

(c)	The total of charges to all QSEs for cleared DAM Energy Bids, calculated under Section 4.6.2.2	, Day-Ahead Energy Charge; and

(d)	The total of charges or payments to all QSEs for PTP Obligation bids cleared in the DAM, calculated under Section 4.6.3, Settlement for PTP Obligations Bought in DAM.

(2)	The DAM congestion rent for a given Operating Hour is calculated as follows:

DACONGRENT	=	DAESAMTTOT + RMRDAEREVTOT + DAEPAMTTOT + DARTOBLAMTTOT

Where:

DAESAMTTOT	=	ΣqDAESAMTQSETOT q

DAEPAMTTOT	=	ΣqDAEPAMTQSETOT q

DARTOBLAMTTOT	=	ΣqDARTOBLAMTQSETOT q

RMRDAEREVTOT	=	ΣqΣpΣrDAEREV q, p, r

DAEREV q, p, r	=	(-1) * DASPP p * DAESR q, p, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DACONGRENT | $ | Day-Ahead Congestion RentThe congestion rent collected in the DAM for the hour. |
| DAESAMTTOT | $ | Day-Ahead Energy Sale Amount TotalThe total payment to all QSEs for cleared DAM energy offers, whether through Three-Part Supply Offers or through DAM Energy-Only Offer Curves for the hour. |
| RMRDAEREVTOT | $ | RMR Day-Ahead Energy Revenue TotalThe total of the RMR Day-Ahead Energy Revenue for all RMR Units for the hour. See Section 6.6.6, Reliability Must-Run Settlement. |
| DAEPAMTTOT | $ | Day-Ahead Energy Purchase Amount TotalThe total charge to all QSEs for cleared DAM Energy Bids for the hour. |
| DARTOBLAMTTOT | $ | Day-Ahead Real-Time Obligation Amount TotalThe net total charge or payment to all QSEs for cleared PTP Obligation bids in the DAM for the hour. |
| DAESAMTQSETOT q | $ | Day-Ahead Energy Sale Amount QSE Total per QSEThe total payment to QSE q for cleared DAM energy offers, whether through Three-Part Supply Offers or through DAM Energy-Only Offer Curves, for the hour. See item (2) of Section 4.6.2.1. |
| DAEREV q, p, r | $ | Day-Ahead Energy Revenue per QSE by Settlement Point per unitThe revenue received in the DAM for RMR Unit r at Resource Node p represented by QSE q, based on the DAM Settlement Point Price, for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DASPP p | $/MWh | Day-Ahead Settlement Point Price by Settlement PointThe DAM Settlement Point Price at Resource Node p for the hour. |
| DAESR q, p, r | MW | Day-Ahead Energy Sale from Resource per QSE by Settlement Point per unitThe amount of energy cleared through Three-Part Supply Offers in the DAM and/or DAM Energy-Only Offer Curves for RMR Unit r at Resource Node p represented by QSE q for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DAEPAMTQSETOT q | $ | Day-Ahead Energy Purchase Amount QSE Total per QSEThe total charge to QSE q for cleared DAM Energy Bids for the hour. See item (2) of Section 4.6.2.2, Day-Ahead Energy Charge. |
| DARTOBLAMTQSETOT q | $ | Day-Ahead Real-Time Obligation Amount QSE Total per QSE The total charge or payment to QSE q for PTP Obligation Bids cleared in the DAM for the hour. See item (2) of Section 4.6.3. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | An RMR Unit. |