| LPRR Number | 039 | LPGRR Title | Revisions for Texas Nodal Market Implementation Part Two and General Clean Up (formerly “Revisions for Texas Nodal Market Implementation Part Two”) |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | August 10, 2010 |  |  |
| Proposed Effective Date |  | October 1, 2010 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Load Profiling Guide (LPG) Sections Requiring Revision |  | 13, Changes to Weather Zone Definitions 14, Load Profile ID Dispute Procedure 15, Load Research Samples 16, Supplemental Load Profiling 17, Load Profile Metering 18, Access to Load Profiling Materials Appendix A Appendix B Appendix C Acronyms and Glossary (delete) 19, Definitions and Acronyms (new) |  |  |
| Revision Description |  | This Load Profiling Guide Revision Request (LPGRR) revises the Load Profiling Guide (LPG) in anticipation of Texas Nodal Market implementation. Revisions will be grey-boxed until the Texas Nodal Market Implementation Date (TNMID) where appropriate. |  |  |
| Reason for Revision |  | Preparation for Texas Nodal Market implementation. |  |  |
| Overall Market Benefit |  | Continuity of LPG with Nodal Protocols. |  |  |
| Overall Market Impact |  | None. |  |  |
| Consumer Impact |  | None. |  |  |
| Procedural History |  | - On 4/29/10, LPGRR039, a CEO Revision Request Review, and an Impact Analysis were posted. - On 5/26/10, the Profiling Working Group (PWG) considered LPGRR039. - On 7/28/10, PWG considered the 5/26/10 PWG Recommendation Report and Impact Analysis for LPGRR039. - On 8/10/10, COPS considered LPGRR039. |  |  |
| PWG Decision |  | On 5/26/10, the PWG was in consensus to recommend approval of LPGRR039 as revised by PWG. On 7/28/10, PWG was in consensus to endorse and forward the 5/26/10 PWG Recommendation Report and the Impact Analysis for LPGRR039 to COPS |  |  |
| Summary of PWG Discussion |  | On 5/26/10, PWG discussed LPGRR039 and revised the title to more accurately reflect the intent of the LPGRR. PWG also deleted language that no longer applies to Load Profiling and proposed additional administrative changes. On 7/28/10, there was no discussion. |  |  |
| COPS Decision |  | On 8/10/10, COPS unanimously voted to recommend approval of LPGRR039 as recommended by PWG in the 7/28/10 PWG Recommendation Report. All Market Segments were present for the vote. |  |  |
| Summary of COPS Discussion |  | On 8/10/10, there was no discussion. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Continuity of LPG with Nodal Protocols. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Sonja B. Mingo |
| E-mail Address | smingo@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6463 |
| Cell Number | 512-740-6030 |
| Market Segment | NA. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sonja B. Mingo |
| E-Mail Address | smingo@ercot.com |
| Phone Number | 512-248-6463 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Description |
| None. |  |

Proposed Guide Language Revision

ERCOT Load Profiling Guide

Section 13: Changes to Weather Zone Definitions

[DATE]

# 13	CHANGES TO WEATHER ZONE DEFINITIONS

(1)	Changes to Weather Zones and any combination thereof that may be requested include:

(a)	Changes in Weather Zone boundary definitions;

(b)	Changes in the boundaries of weather modeling regions;

(c)	Changes in the weather stations used; and

(d)	Changes in the weighting of weather stations used within a Weather Zone.

(2)	Any change to Load Profile ID assignments resulting from an approved modification to the definitions of Weather Zones shall not be retroactively applied.

(3)	A requested Weather Zone change shall be shown in the supporting documentation to be different from the existing Weather Zone definitions in ways that improve the accuracy of Settlement.

## 13.1	General Guidelines for Weather Zone Changes

### 13.1.1	Timeline for Processing a Request

Timing of requests, responses to requests, and change implementation shall be as defined for Segmentation in Section 12.4, Process Timing for Requesting Changes.

### 13.1.2	Uniformity

Weather Zone definitions shall be applied to all Electric Service Identifiers (ESI IDs) located within the geographic boundaries of the Weather Zone.  Zone Improvement Plan (ZIP) codes are mapped to Weather Zones and are defined by the ZIP-to-Zone mapping in Appendix D, Profile Decision Tree.

### 13.2	Changes to Weather Zone Boundaries

### 13.2.1	Types of Weather Zone Boundary Changes

(1)	Changes to Weather Zone boundaries and any combination thereof, may occur due to the following conditions:

(a)	Subdivision: An existing Weather Zone is divided into two or more zones;

(b)	Boundary shifting: Existing Weather Zone boundaries are moved so that areas are shifted between Weather Zones; and

(c)	Boundary collapsing: Existing Weather Zone boundaries are moved so that one Weather Zone is created from two or more existing Weather Zones.

(2)	When creating a new Weather Zone, the other zones affected by the boundary change shall satisfy the Weather Zone criteria in Section 13.2.2, Eligible Areas for Weather Zones.

Table 1
Weather Zones Assigned to Weather Modeling Regions as of January 1, 2002

(3)	Boundary shift considerations:

(a)	A shift within a modeling region is a boundary shift where all zones affected by the shift have the same zone constants; and

(b)	A shift across modeling regions is a boundary shift where zone constants are different between areas affected by the shift.

(4)	A shift across modeling regions is more complex to implement.  A subdivision of a Weather Zone is similar to a shift within a modeling region.  Therefore, all Weather Zones affected by the Weather Zone subdivision have the same zone constants.

### 13.2.2	Eligible Areas for Weather Zones

Each Weather Zone that results from a requested Weather Zone boundary change shall be a geographically contiguous area defined by identifiable physical, Transmission and/or Distribution Service Provider (TDSP) territory, or ZIP code boundaries.

#### 13.2.2.1	Size

The requested Weather Zone changes shall be shown in supporting documentation to be of sufficient size, both in number of Customers and in total energy consumption, to justify the changes.  While no explicit size threshold is set, the size of each proposed new or changed Weather Zone shall be considered in evaluating a Weather Zone change request.

#### 13.2.2.2	Weather Stations

(1)	Only weather data from National Oceanic and Atmospheric Administration (NOAA) first or second order weather stations shall be used in model calculations.  Each proposed new or changed weather station shall have at least two NOAA first or second order weather stations to represent it.

(2)	The change request shall propose the weights to be used for the weather stations in each Weather Zone to be created or changed.  No weather station is permitted to have more than 50% weight.

### 13.2.3	Supporting Data

A requested new Weather Zone created by subdividing an existing Weather Zone shall be different from the current Weather Zone assignment.  For requests of any boundary shift, the shifted area shall be different from the currently assigned Weather Zone and more similar to the proposed Weather Zone.  In each case, the difference (or similarity) shall be shown to result in important differences (or lack of important differences) in Load Profiles.  Important differences are those that materially affect the accuracy of Settlement.

#### 13.2.3.1	Calculated Load Profiles

(1)	Load Profile calculations should be provided on current Weather Zone definitions and proposed Weather Zone definitions.  The results of the change(s) should be significant enough to justify the proposed Weather Zone.

(a)	For a subdivision or a shift within a weather-modeling region, Load Profiles shall be calculated using the existing zone constants;

(i)	In the case of a subdivision, the Load Profiles for one or more of the new zones created by subdivision shall be different from the current set of profiles;

(ii)	In the case of a boundary shift or collapsing, the Load Profiles for the shifted area shall be different from those from the current assignment and more similar to those of the proposed new assignment.

(b)	For a shift across modeling regions, the following calculated Load Profiles shall be provided for each shifted area:

(i)	Load Profiles calculated using the zone constants of the currently assigned zone:

(A)	Using the weighted average for the current Weather Zone;

(B)	Using the weighted average weather of the current zone after the shifted area is removed, with the proposed weights;

(C)	Using the weather of the shifted area only.

(ii)	Load Profiles calculated using the zone constants of the receiving zone, to which the shifted area is proposed to be moved:

(A)	Using the weighted average for the receiving Weather Zone;

(B)	Using the weighted average weather of the receiving zone after the shifted area is added, with the proposed weights;

(C)	Using the weather of the shifted area only.

(2)	The Load Profile using the weather for the shifted area and its current assigned zone constants shall be different from the other two Load Profiles calculated with the current zone constants for the area.  The Load Profile developed by the weather of the shifted area and the zone constants of the receiving zone shall be similar to those of the other Load Profiles calculated with the zone constants of the current zone.  Differences of an area from its current zone and similarity of an area to a proposed receiving zone shall be assessed using the measures described in Appendix C, Measuring Differences Between Load Profiles.

#### 13.2.3.2	Additional Supporting Data for Shifts across Weather Modeling Regions

(1)	For a shift across weather modeling regions, evidence shall be provided that demonstrates the weather response of the affected area is likely to be more similar to the proposed new region than to the currently assigned region.  The types of evidence that may be offered for this purpose are the same as those types described in Section 12, Request for Profile Segment Changes, Additions, or Removals, and include:

(a)	Load research data from the affected area, from the current Weather Zone excluding the affected area, and from the proposed receiving Weather Zone;

(b)	Equipment operating data from each area;

(c)	End-use equipment saturation data from each area; and

(d)	Monthly consumption patterns from each area.

(2)	Based on the supporting data, the request shall indicate whether the zone constant(s) should be re-estimated.

#### 13.2.3.3	Basis for Assessing Differences

The difference in Load Profiles based on the proposed versus current Weather Zones shall be assessed similarly to an assessment of a new versus existing Load Profile Segment, by consideration of the same types of factors as described under Section 12.6.2, Difference from Current Load Profile Segments.  Only those Load Profile Models dependent on weather variables shall be used in the assessment of a Weather Zone change.

## 13.3	Changes to Weather Modeling Regions

A weather modeling region boundary shall be changed if shifting an area across weather modeling regions changes a Weather Zone boundary.  In some cases ERCOT and the Profiling Working Group (PWG) may recommend retaining current zone constants even though they shall be applied to a region different from the one for which the estimation was conducted.

### 13.3.1	Supporting Data Required

(1)	Any requested change to weather modeling regions shall be treated as a special case of a request for a change in Load Profile Segment definitions.  Supporting data required for such a request is described in Section 12.6, Information Required with Request for Change.  Specific supporting information required for a request to shift a Weather Zone boundary across weather modeling regions is described in Section 13.2.3, Supporting Data.  Corresponding information is required for other changes to weather modeling regions.

(2)	The requested Weather Zone shall be different from the current Weather Zone in ways that improve Load Profiles.  A change in Weather Zone requiring new coefficients for the new zone shall be considered as a special case of a request for a new Weather Zone segment.  Procedures for submitting and assessing requests are the same as the rules for requesting a change in Segmentation, as described in Section 12, Requests for Load Profile Segment Changes, Additions, or Removals. The assessment shall include the effect on the rest of the Weather Zone(s) of changing this area’s coefficients.

### 13.3.2	Basis for Assessing a Request

ERCOT shall assess the request based on the data and analysis submitted with the request as well as possible additional analysis by ERCOT.  Factors considered in assessing any request may include:

(a)	The quality of the supporting data provided;

(b)	The magnitude of differences indicated;

(c)	The size of the affected populations;

(d)	The complexity of the change required;

(e)	The effect on other Weather Zone(s) and other weather modeling regions if the change is accepted;

(f)	The effect on ERCOT systems; and

(g)	The enhancement of settlement accuracy.

## 13.4	Changing Weather Stations

## 13.4.1	Requests for Changes

(1)	A request may be made to change the weights assigned to weather stations within a Weather Zone.  Such a change would include adding a weather station that was not previously used, or deleting a station currently used.  Changing weather stations may require re-estimation of zone constants for weather modeling regions and model-based Load Profile Types.

(2)	A request for such a change shall be accompanied by evidence that the proposed new set of stations and weights are more representative of the population in each affected Weather Zone than the current ones.  An example of such evidence would be analysis of the distribution of population and Weather Zone patterns similar to that conducted for the initial development of the weather modeling procedures.  However, given the broad implications of changing the weather stations, the evidence shall also prove to be a substantial benefit to current specifications.

## 13.5	Weather Zone Definition or  Modeling Region Changes without a Change Request

### 13.5.1	Periodic Assessment

ERCOT may assess Weather Zone and weather modeling region boundaries in its periodic process of evaluating Load Profile Models.  In the event that ERCOT conducts such an assessment and determines that weather modeling boundaries shall be re-drawn, new zone constants may be estimated for all affected Weather Zones.  ERCOT shall present its proposed changes to the Profiling Working Group (PWG) for evaluation and implementation according to the procedures contained herein.

### 13.5.2	Changes Required Based on Changing Data Availability

#### 13.5.2.1	Changes in National Oceanic and Atmospheric Administration 							Weather Station

Weather station changes shall be necessary in the event that a station currently used is discontinued by National Oceanic and Atmospheric Administration (NOAA) or changed by downgrading from second order status.  In the event that NOAA makes such changes, ERCOT shall assess and propose reasonable adjustments.

#### 13.5.2.2	Changes in Zone Improvement Plan Codes

(1)	ERCOT’s Load Profiling Weather Zones are defined by the five digit ZIP codes.  ZIP code changes within a current Weather Zone shall not require any special adjustments.  The new ZIP code definitions shall be incorporated into profiling systems so that Electric Service Identifiers (ESI IDs) shall continue to be correctly assigned.

(2)	ZIP code changes that affect a Weather Zone boundary shall be incorporated into Weather Zone definitions with minimal change in definitions and assignments.  When a ZIP code overlaps two or more Weather Zones, the entire new ZIP code shall be assessed for the proper Weather Zone assignment.  A ZIP code shall be completely contained within only one Weather Zone.

ERCOT Load Profiling Guide

Section 14: Load Profile ID Dispute Procedure

[Date]

14	LOAD PROFILE ID DISPUTE PROCEDURE	1

14.1	Filing of a Load Profile ID Dispute	1

14.2	General Load Profile ID Dispute Resolution Guidelines	1

14.2.1	Disputes Involving ERCOT	1

14.2.2	Disputes Involving Transmission and/or Distribution Service Providers	1

14.2.3	Alternative Dispute Resolution	2

14.3	 Resolutions of Disputes	2

# 14	LOAD PROFILE ID DISPUTE PROCEDURE

ERCOT and Market Participants shall adhere to this procedure for disputing Load Profile ID assignments.

## 14.1	Filing of a Load Profile ID Dispute

ERCOT and any Market Participant, other than a retail Customer, may file disputes related to Load Profile ID assignments.  Retail Customers with disputes, related to Load Profile ID assignments, shall first request resolution from their Competitive Retailers (CRs).  The CR shall address the Customer’s issue, and if necessary, request changes or corrections from ERCOT related to the retail Customer’s request.  A retail Customer who is not satisfied with the CR’s response may appeal to the Public Utility Commission of Texas (PUCT) or the appropriate regulatory authority.  ERCOT does not resolve such disputes.

## 14.2	General Load Profile ID Dispute Resolution Guidelines

Transmission and/or Distribution Service Providers (TDSPs) and ERCOT share responsibility for the assignment of Load Profile IDs.  Competitive Retailers (CRs) may request a Load Profile ID assignment change as a dispute of an existing Load Profile ID assignment.  Requested changes to remove an Electric Service Identifier (ESI ID) from a default Load Profile ID should only be made after adequate monthly data becomes available.

### 14.2.1	Disputes Involving ERCOT

(1)	Disputes involving ERCOT should be submitted using the MarkeTrak system for any of the following cases:

(a)	Requests to remove an ESI ID from a default Load Profile ID - such requests should only be made after adequate monthly data becomes available;

(b)	Disputes regarding ERCOT calculations made as a part of Annual Validation; and

(c)	Disputes regarding ERCOT calculations relating to the weather sensitivity code.

(2)	ERCOT is responsible for all disputes defined in this Section, all Profile Decision Tree versions, and all Annual Validation years.

### 14.2.2	Disputes Involving Transmission and/or Distribution Service Providers

All disputes related to Load Profile ID assignments other than those described in the preceding section must be addressed with each TDSP in accordance with their individual processes.

### 14.2.3	Alternative Dispute Resolution

If attempts to clarify or resolve the issue using one of the processes listed above are unsuccessful, parties should refer to Protocol Section 20, Alternative Dispute Resolution Procedure.

## 14.3		Resolutions of Disputes

- When the resolution of a dispute requires a change in a Load Profile ID assignment, the change shall be implemented by the Transmission and/or Distribution Service Provider (TDSP) issuing the appropriate Texas Standard Electronic Transaction (TX SET).

ERCOT Load Profiling Guide

Section 15: Load Research Samples

[DATE]

15	LOAD RESEARCH SAMPLES	1

15.1	Transmission and/or Distribution Service Provider Samples	1

15.1.1	Maintenance of Existing Samples	1

15.1.2	Notification to ERCOT	2

15.1.3	Availability of Data	5

15.1.4	Creation of New Transmission and/or Distribution Service Provider Samples	5

15.2	ERCOT Samples	6

15.2.1	Maintenance	6

15.2.2	Availability of Data	6

15.2.3	Criteria of Standards	7

15.2.4	Creation of New Samples	8

15.2.5	Guidelines for Installing and Refreshing Load Research Samples	9

# 15	LOAD RESEARCH SAMPLES

(1)	Load research samples are required by ERCOT as the basis for developing and evaluating Load Profiles for most Load Profile Types.  Protocol Section 18.2.10, Responsibilities of Sampling in Support of Load Profiling, broadly defines the responsibilities of ERCOT and Transmission and/or Distribution Service Providers (TDSPs) regarding Load research samples.  This part of the Load Profiling Guide (LPG) also provides guidelines on communication and expectations between ERCOT and TDSPs in fulfilling those responsibilities.

(2)	TDSPs have provided their Load research data in the past and shall continue to provide available data in the future in the interests of keeping ERCOT’s costs down.  TDSPs may, at their own discretion, determine the overall level of Load research effort they will provide.  TDSP’s Load research is independent of ERCOT, except as specified in Protocol Section 18, Load Profiling.  ERCOT shall make use of TDSP and Non-Opt-In Entity (NOIE) Load research data to the extent such data are available and useful.  ERCOT shall attempt to minimize the burden to TDSPs of providing data to ERCOT.

(3)	TDSPs and NOIEs provided all Load research data to ERCOT used in the initial development of Load Profiles.  ERCOT shall periodically request current Load research data from all TDSPs and NOIEs for Load Profile Model evaluations.  ERCOT plans to develop and arrange for implementation of its own Load research samples.

## 15.1	Transmission and/or Distribution Service Provider Samples

### 15.1.1	Maintenance of Existing Samples

(1)	Transmission and/or Distribution Service Providers (TDSPs) with current Load research samples are required by Protocol Section 18.2, Methodology, to maintain these samples to the accuracy designed.  Maintaining accuracy means that as long as the sample is deployed, the TDSP is responsible for performing the following:

(a)	Replacing sample points as needed to compensate for sample attrition;

(b)	Replacing or repairing malfunctioning data collection equipment as needed;

(c)	Maintaining and operating data collection and processing systems; and

(d)	Providing annual reports to ERCOT as described in Section 15.1.2, Notification to ERCOT.

(2)	Subject to the one year notification requirement in Section 15.1.2, a TDSP may discontinue any sample at its own discretion.

### 15.1.2	Notification to ERCOT

#### 15.1.2.1	Types of Changes Given Advance Notice

(1)	For any major change to the design of a sample, the TDSP shall provide ERCOT with at least one year advance notice.  In particular, this amount of advance notice shall be given for taking an existing sample out of the field.

(2)	TDSPs shall also provide one year advance notice, whenever practical, for any of the following changes:

(a)	Putting a new sample into the field;

(b)	Rotating a sample by systematically replacing a subset of the current sample with new sample points;

(c)	Adding supplemental strata to account for new accounts added to the population of the class;

(d)	Bulk replacement of equipment or data collection systems with new types of equipment or systems; and

(e)	Other major changes to the Sample Design or implementation.

(3)	Notification to ERCOT is not intended to be a barrier to developing and implementing changes within less than a year.  If a TDSP determines a need to implement any of the above changes on a shorter timetable, the TDSP may do so at its own discretion, but shall notify ERCOT of its plans as soon as practical.  The sole exception is that a TDSP shall not discontinue an existing Load research sample with less than one year notice to ERCOT.

(4)	Changes involving routine sample maintenance, including replacement of dropped points or replacement or repair of problem equipment, do not require case-by-case notification to ERCOT.

#### 15.1.2.2	General Reporting Procedures

(1)	Each TDSP shall report to ERCOT by April 1st of each year the status of its Load research samples and future plans for these samples in addition to providing the Load research data.  The annual report on sample status shall include the information on each existing Load research sample as well as on any plans for new Load research samples.

(2)	TDSPs shall update ERCOT with a report by September 1st of each year regarding any major changes to samples planned for the next 12 months.

(3)	ERCOT shall ordinarily request data for each Load research sample once a year, on the schedule indicated above.  For new samples, requests may be made more frequently, enabling ERCOT to begin using the data before a full year of data is available.  ERCOT may also request data more frequently in special cases (e.g., Public Utility Commission of Texas (PUCT) mandate).  TDSPs shall provide requested data to ERCOT within 60 days.

(4)	Load research data shall be provided by the TDSP both at the individual Premise level and aggregated to TDSP class estimates.  The Load data and status codes delivered shall be in edited and validated form.

(5)	Specific required and desired information for each report is described below.  Where information is specified for each TDSP class Load research sample, the information shall be provided for each Load research sample that existed as of the last reporting period, as well as for all current Load research samples.  If a new sample shall be placed for a class that does not currently have one, this information should be provided as part of item (1)(f) of the following Section 15.1.2.3, Required Information.

#### 15.1.2.3	Required Information

(1)	The following information is required as part of the annual reporting and data transmittal.

(a)	Included as fields in the data files provided:

(i)	Data quality flags;

(ii)	Sample expansion weights; and

(iii)	Stratum identifiers.

(b)	A data dictionary providing the file layout(s) and codebook.

(c)	For each class sample, a description of the Sample Design, stratification, procedure for calculating expansion weights, and data validation procedures.

(d)	For each stratum in each sample:

(i)	The original and current sample sizes; and

(ii)	The original population number of Customers and annual energy in MWh.

(e)	For each class sample, the most recent available estimates of annual peak Load and whatever accuracy measures have been calculated for that estimate.

(i)	The date for which the analysis was conducted and the 	year when the analysis was completed shall also be reported; and

(ii)	If the Sample Design was based on accuracy criteria, other than annual peak Demand, a description of these criteria with the corresponding most recent estimates and accuracy measures and dates of these analyses shall be provided.

(f)	Plans for any major changes as described in Section 15.1.2, Notification to ERCOT, planned for the next 12 months.

(i)	The type of change planned; and

(ii)	The anticipated schedule.

(g)	Description of major changes during the preceding 12 months.  Major changes include the items under Section 15.1.2.1, Types of Changes Given Advance Notice.  For each change the TDSP shall indicate:

(i)	The type of changes made; and

(ii)	The timing of the changes.

(2)	Items (1)(a) through (1)(e) do not have to be re-submitted, if they have not changed since prior reports to ERCOT.  The report shall note that these items were previously submitted and have not changed.  Items (1)(f) and (1)(g) are required only with the regular (April and September) status reports, not as part of periodic reporting in response to special requests.

#### 15.1.2.4	Additional Requested Information

(1)	The following additional items are useful to ERCOT for analysis.  TDSPs should provide as much of this or related information as practical given their current practices and operations.

(a)	For each stratum in the sample:

(i)	The number of points removed and added in the past year, excluding direct replacements; and

(ii)	The fraction of intervals with missing or bad data.

(b)	For each class sample:

(i)	Distributions of energy; and

(ii)	Definitions of rate classes (TDSP tariffs) that the samples are applicable to along with the rate classes that the samples are assigned to.

(c)	Description of sample coverage:

(i)	Give Customer counts and annual energy for the portion of the population that is not represented by any of the samples;

(ii)	This information may be provided as a single total for each category or by identifying and quantifying specific subgroups that were not included in any of the sample frames; and

(iii)	Provide this information separately by Residential and Business categories.

(2)	ERCOT shall provide a standard reporting format for TDSP use for reporting on Load research.

### 15.1.3	Availability of Data

Load research data provided to ERCOT from the TDSP shall only be available to ERCOT for its use in Load Profiling.

### 15.1.4	Creation of New Transmission and/or Distribution Service Provider Samples

(1)	A TDSP may, at its discretion, develop new Load research samples.  These samples may be a replacement for existing samples or may represent a population not currently covered by an active Load research sample.

(2)	A TDSP that develops a new Load research sample shall inform ERCOT of the plan to develop the sample.  This information shall be provided as part of the reporting procedures described in Section 15.1.2, Notification to ERCOT.  Information the TDSP shall provide about a planned new sample shall include:

(a)	A description of the population to be represented by the sample;

(b)	The relationship between this population and classes represented by current samples or previously existing samples for which data have been provided to ERCOT; and

(c)	The approximate size of the population, in number of Customers and MWh.

(3)	When plans for a new sample are sufficiently developed, the TDSP shall provide in its report to ERCOT a description of the Sample Design.  This description shall include:

(a)	The Sample Design accuracy target;

(b)	The estimation method for which the sample accuracy is designed (typically, mean-per-unit or ratio estimation);

(c)	The stratification scheme;

(d)	The population size of each Sampling cell in number of Customers and annual MWh; and

(e)	The sample size of each Sampling cell.

## 15.2	ERCOT Samples

According to Protocol Section 18.2.10, Responsibilities for Sampling in Support of Load Profiling, ERCOT is responsible for developing new Load research samples if it determines that existing Load research data are insufficient for profile development and maintenance.  ERCOT or its designated agent shall develop Sample Designs, select samples, install metering equipment, collect, process, and validate data, and develop population estimates. ERCOT shall be responsible for the costs associated with the Sampling functions it directs.  ERCOT shall adhere to good professional practice in all these functions.  ERCOT shall utilize the Association of Edison Illuminating Companies (AEIC) Load Research Manual as a reference for standards of good practice.

### 15.2.1	Maintenance

As long as an ERCOT Load research sample is in the field, ERCOT shall maintain the sample to good standards.  Sample maintenance shall include the following:

(a)	Replace sample points as needed to compensate for sample attrition;

(b)	Replace or repair malfunctioning data collection equipment as needed;

(c)	Review incoming data on at least a monthly basis to identify problems of high rates of missing data, or anomalous values;

(d)	Repair or correct apparent equipment or system malfunctions on a timely basis; and

(e)	At least once a year, calculate class means for each class Load research sample, using the estimation procedure appropriate to the Sample Design, and calculate the accuracy of the estimated peak Load.  If criteria other than accuracy of Load at peak were used in designing the sample, calculate these accuracy measures.  If the sample no longer meets the design accuracy criteria, initiate steps to bring the sample into conformance with the design criteria.

### 15.2.2	Availability of Data

(1)	Load research data collected by ERCOT shall be available only to ERCOT or its designated agent.  Load Profiles developed from these data shall be made available through ERCOT’s standard profile reporting procedures.  ERCOT shall provide descriptive information available on Load research samples, in support of Load Profiling, according to Protocol Section 18.3.1, Methodology Information.  This data is strictly used for Load Profiling purposes.

(2)	In addition to the published Load Profiles, other aggregate data from the Load research samples shall also be made available to Market Participants by ERCOT.  Aggregate data that ERCOT shall provide for each Load research sample shall include:

(a)	ERCOT’s estimate of average kW per Electric Service Identifier (ESI ID) in each time interval based on the Load research sample;

(b)	Standard errors or other statistical accuracy measures for the estimated average kW per ESI ID in each interval; and

(c)	Sample Size.

(3)	The standard errors and sample sizes for each Load research sample may be provided as ranges or averages rather than providing individual values for each time interval.  ERCOT may provide additional aggregate information that it deems to be of value to the Market Participants.

### 15.2.3	Criteria of Standards

(1)	Load research samples developed by ERCOT shall be designed to meet a standard of  10% accuracy at 90% confidence. A discussion of the meaning of accuracy measures and procedures is in the AEIC Load Research Manual.

(2)	For Load research samples used for universally applicable Load Profiles, this accuracy standard shall be applied at the level of each Load Profile Segment definition.  It is preferred that this accuracy standard should be achieved at the level of a distinct Load Profile Segment and weather modeling region.   Universally applicable Load Profiles are defined in Section 12, Requests for Load Profile Segment Changes, Additions, or Removals.  Weather modeling region is defined in Section 13, Changes to Weather Zone Definitions.

(3)	Designing a sample to meet a particular accuracy standard requires information about the population, including the number and total Load by subgroup, and the variability in Load across the group.  Such information is typically not available before the data have been collected.  It is therefore standard practice to design samples initially using proxy measures.

(4)	Prior to collecting data for the designated population, the Sample Design shall be developed using characteristics from the Load research data already compiled by ERCOT.  After a year of data has been collected, ERCOT shall review the achieved accuracy of the samples.  If the achieved accuracy is worse than the design target, ERCOT shall consider increasing sample sizes or modifying the design to achieve the target accuracy.  For any such re-design efforts, the data from the current sample shall be used as the basis for estimating the population parameters needed to calculate sample requirements.

(5)	In reviewing the achieved accuracy of the initial Load research samples, ERCOT shall consider these possible metrics as well as conformance to the design accuracy standard:

(a)	Accuracy of the fraction of energy allocated into each of several Time Of Use (TOU) periods (several being about four time periods);

(b)	Accuracy of the ratio of on-peak to off-peak consumption;

(c)	Demand at the peak hour in each month;

(d)	Total energy consumption in each month; and

(e)	Accuracy of Load-weighted average price, using a standard price series.

(6)	Based on this review, ERCOT may recommend new standards based on one or more of these metrics for future Load research samples.

### 15.2.4	Creation of New Samples

(1)	ERCOT has the authority to determine the need for new Load research samples.  These samples may be a replacement for existing samples, or may represent a population not currently covered by an active Load research sample.

(2)	Samples developed by ERCOT may be regional spanning more than one TDSP.  The sampled populations may also be restricted to only a geographic subset of a TDSP’s service territory.

(3)	Information that ERCOT shall provide to the Profiling Working Group (PWG) about a planned new sample shall include:

(a)	A description of the population to be represented by the sample;

(b)	The relationship between this population and classes represented by current samples or previously existing samples for which data have been provided to ERCOT; and

(c)	The approximate size of the population, in number of Customers and MWh.

(4)	When plans for a new sample are sufficiently developed, ERCOT shall provide to the PWG a description of the Sample Design.  This description shall include:

(a)	The Sample Design accuracy target;

(b)	The estimation method for which the sample accuracy is designed (typically, mean-per-unit or ratio estimation);

(c)	The stratification scheme;

(d)	The population size of each Sampling cell in number of Customers and annual MWh; and

(e)	The Sample Size of each Sampling cell.

### 15.2.5	Guidelines for Installing and Refreshing Load Research Samples

The decision to develop a new Load research sample shall be based on ERCOT’s annual evaluation of models and methods.  This evaluation process is described in Section 8, Load Profile Models, and Section 7, Request for Changes to Load Profiling Methodology.  Circumstances that may trigger ERCOT’s decision to field a new Load research sample might include the following:

(a)	Indications that existing Load Profile Models do not perform well in areas that do not have recent Load research data.  Such indications could include:

(i)	Load Profiles whose monthly fractions are very different from those observed in monthly billing data for a particular area;

(ii)	Systematically high Unaccounted for Energy (UFE) for a particular area; and

(iii)	Other indicators that the equipment present or operating patterns are very different in a particular area from that for Load research data were available.

(b)	Reported plans by a TDSP to discontinue collecting Load research data for particular samples.

(c)	Determination that too much time has elapsed since the Load research data on which current models are based were collected.

(d)	Determination that current Load research samples do not meet accuracy standards for a particular population segment.

ERCOT Load Profiling Guide

Section 16: Supplemental Load Profiling

[DATE]

16	SUPPLEMENTAL LOAD PROFILING	1

16.1	Load Profiling for Time Of Use Schedules	1

16.1.1	Establishing New Time Of Use Schedules	1

16.1.2	Chunking Load Profiling Methodology Description	2

16.1.3	Evaluation of the Chunking Load Profiles	2

16.1.4	Revisions to Time Of Use Load Profile Methods if Changes Are Needed	5

16.2	 Direct Load Control	5

16.2.1	General Procedure	5

16.2.2	Requirements for Load Profiles for Direct Load Control	6

16.2.3	Responsibilities of the Competitive Retailer	6

16.2.4	ERCOT Responsibilities	8

16.2.4.3.3	Adherence to the Association of Edison Illuminating Companies       Load Research Manual	9

16.2.4.3.4	Annual Verification of Statistical Validity	9

16.2.4.4	Direct Load Control Participant Database	10

16.2.5	ERCOT Requested Data	11

16.2.6	Implementing Load Research Samples for Direct Load Control Programs	11

16.2.7	Processing Load Research Data	11

16.2.8	Direct Load Control Program Settlement and Balancing Up Load Baseline Calculation Methodology	12

16.2.9	Verifying Sample Validity	17

16.3	Other Supplemental Load Profiles	17

16.4	Requesting Direct Load Control or Other Supplemental Load Profile	18

16.4.1	Request for a Supplemental Load Profile	18

16.4.2	Procedure for Submitting a Request	18

16.4.3	Required Information	18

16.4.4	Process Timing for Requesting Changes	18

16.4.5	Response to the Request	19

SP16.1	Load Profiling for Time Of Use Schedules	1

.1	Et16.1.2	Chunking Load Profiling Methodology Description	2

16.1.4	Revisions to Time Of Use Load Profile Methods if Changes Are Needed	5

16.2.1	General Procedure	5

rol	516.2.3	Responsibilities of the Competitive Retailer	6

ERCOTR16.2.5	ERCOT Requested Data	10

ograms116.2.7	Processing Load Research Data	11

16.2.9	Verifying Sample Validity	16

16.4	Requesting Direct Load Control or Other Supplemental Load Profile	17

16.4.2	Procedure for Submitting a Request	17

16.4.4	Process Timing for Requesting Changes	17

16	SUPPLEMENTAL LOAD PROFILING	1

16.1.1	Establishing New Time-Of-Use Schedules	2

16.1.3	Evaluation of the Chunking Profiles	2

16.2	 Direct Load Control	5

16.2.2	Requirements for Load Profiles for Direct Load Control	6

16.2.4	ERCOT Responsibilities	7

16.2.6	Implementing Load Research Samples for Direct Load Control Programs	11

16.2.8	Direct Load Control Program Settlement and Balancing Up Load Baseline Calculation Methodology	11

16.3	Other Supplemental Load Profiles	17

16.4.1	Who May Submit a Request for a Supplemental Load Profile	17

16.4.3	Required Information	18

16.4.5	Response to the Request	18

# 16	SUPPLEMENTAL LOAD PROFILING

(1)	Protocol Section 18.7, Supplemental Load Profiling, requires that supplemental Load Profiles be developed for programs or pricing schemes that encourage a Demand Response (DR) to price in the retail market.  A DR program is designed to alter Load shape.  For such programs, methods other than Adjusted Static Methodology are necessary.  The supplemental Load Profiling Methodologies described in this Section of the Load Profiling Guide (LPG) are intended only for DR programs or pricing schemes.  Use of these methodologies for other applications requires approval of the Technical Advisory Committee (TAC).

(2)	The ERCOT Protocols allow Premises with Time Of Use (TOU) capable meters to be settled by a profiling method known as chunking, which is described below in Section 16.1.2, Chunking Profiling Methodology Description.  Only those Premises with TOU metered energy can utilize this capability.  The Protocols require that Direct Load Control (DLC) programs shall be profiled using Representative Interval Data Recorder (RIDR) profiles based on statistically representative Load research samples.  Other supplemental Load Profiles as described in Protocol Section 18.7.2, Other Load Profiling, are limited to segments that are subject to pricing schemes designed to encourage DR.  The appropriate methodology for other supplemental profiling shall be determined based on the characteristics of the DR program.

## 16.1	Load Profiling for Time Of Use Schedules

(1)	  All Competitive Retailers (CRs) have the right to offer Time Of Use Schedules (TOUSs) in all Transmission and/or Distribution Service Provider (TDSP) service territories, subject to the following restrictions (reference Protocol Section 18.7.1, Load Profiling of Time of Use Metered Electric Service Identifier):

(a)	Within each TDSP service territory, a CR may offer only TOUSs that are listed in Appendix D, Profile Decision Tree; and

(b)	Implementation of any new TOUS is subject to the ERCOT and Texas Standard Electronic Transaction (TX SET) change control process.

(2)	The right to use TOUSs, subject to restrictions in items (1)(a) and (1)(b) above, applies in both investor-owned TDSP service territories, in Non Opt-In Entities (NOIEs) territories if they opt in, and in any new TDSP territories.  For purposes of TOUS management, all TOUSs for NOIE or TDSP territories that opt into the market shall be considered new TOUS and therefore subject to the new TOUSs process in Section 16.1.1, Establishing New Time Of Use Schedules.

16.1.1	Establishing New Time Of Use Schedules

(1)	Available TOUSs are listed in Appendix D, Profile Decision Tree.  Any request for a new TOUS shall be submitted as a Load Profiling Guide Revision Request (LPGRR) in accordance with the process set forth in Section 2, Load Profiling Guide Revision Process.

(2)	Currently, the ERCOT Settlement system can only accommodate TOUSs that have up to four TOU periods (e.g., off-peak, mid-peak, on-peak, super-peak).  Requests for new TOUSs that have four or less buckets can be implemented in ERCOT systems within seven Business Days of approval.  Requests for TOUSs with more than these four buckets will require significant system changes and will subject the LPGRR requesting the new TOUSs to the project prioritization process within ERCOT to determine their ERCOT implementation time.

(3)	Since TOUSs also impact TDSP systems and these systems vary in their ability to support these TOUSs, ERCOT and the Profiling Working Group (PWG) will coordinate closely with TDSP(s) impacted by a new proposed TOUS.  Each impacted TDSP will be requested to submit comments to the proposed LPGRR regarding the system impacts and time frame required to implement the requested TOUS.  The PWG will incorporate this time frame in the expected implementation date for the LPGRR.

(4)	ERCOT shall issue a market notice once the new TOUS has been incorporated into ERCOT systems, and also once the affected TDSPs have implemented the new TOUS and notified ERCOT by sending an email to ERCOTLoadProfilingDepartment@ercot.com that the TOUS is ready.  At this point CRs may begin to request that the TOU meters be installed.

16.1.2	Chunking Load Profiling Methodology Description

The chunking method of Load Profiling for TOU Customers means that a standard Load Profile is applied to the Customer’s consumption data for a meter reading period.  However, the energy for each TOU period in the Load Profile is scaled so that it is equal to the metered energy (kWh) for the TOU period.  For each TOU period within a meter read cycle, the metered consumption during the TOU period is allocated to time intervals within the TOU period in proportion to the Load Profile level at each interval in that period.

16.1.3	Evaluation of the Chunking Load Profiles

(1)	Load Profiles that are adjusted by chunking shall be evaluated as part of the general evaluation procedures described in Section 6, Load Profiling Methodology, and Section 8, Load Profile Models.  Evaluation of the effects of chunking shall be included in the assessment of Unaccounted for Energy (UFE) described in Section 8.3, Evaluating Load Profile Models without Current Load Research Data.  If Load research data is available for a sample of TOU Customers, this data could also be used in the evaluation using methods discussed in Section 8.3.

(2)	Additional procedures that may be used to assess the adequacy of chunking include the following:

(a)	Assess chunking as a general method based on Load Profiles from other areas;

(b)	Assess chunking using Load research data collected in the ERCOT service territory; and

(c)	Assess chunking using data on Customer characteristics in the ERCOT service territory.

### 16.1.3.1	Assessment of Chunking as a General Method Based on Load Profiles 							from Other Areas

(1)	This assessment evaluates chunking as a general methodology, not necessarily specific to particular ERCOT Load Profiles.  Such an assessment could be conducted using Load Profiles from another source service area.  To be used in this analysis, the source service area shall have separate Load Profiles based on separate Load research samples for a TOU class and a corresponding Non Time Of Use (NOTOU) class.

(2)	The assessment compares the source service area’s TOU Load Profile based on the TOU sample to a chunked profile created by applying the chunking method to the source service area’s corresponding profile.  To create the chunked profile, the TOUS applicable from this TOU class in the source service area are used, with the chunking procedures described in Section 16.1.2, Chunking Load Profiling Methodology Description.

(3)	The two Load Profiles are compared using the comparison methods in Section 8, Load Profile Models.  Referencing the terminology in Section 8, the first Load Profile, based on the actual Load research data from the TOU class, is treated as the proposed Load Profile.  The second, chunked Load Profile is the existing Load Profile.  That is, the second Load Profile is the Load Profile the existing ERCOT methodology would use for the TOU class, if these classes were in ERCOT.

(4)	Such comparisons may be made for several different TOU classes, with corresponding NOTOU classes, in different regions, depending on available Load Profiles from other areas.  Substantial differences between the two Load Profiles for many of the classes studied would indicate problems with the general approach.  Substantial similarities between the two Load Profiles for most classes would indicate that the general method is reasonable.

16.1.3.2	Assessment of the Chunking Method Using ERCOT Load Research Data

(1)	This assessment relies on a limited ERCOT Load research sample to evaluate the TOU Load Profiles developed by chunking.  The goal is to compare two Load Profiles - the existing TOU Load Profile developed by the chunking method to:

(a)	The corresponding non-chunked Load Profile; and

(b)	The profile for the same population of TOU Electric Service Identifiers (ESI IDs) developed from a Load research sample of that population.

(2)	For purposes of this assessment, ERCOT may implement a limited Load research sample from each ERCOT TOUS and segment to be studied.  The Sampling criteria for each Profile Type and schedule do not have to adhere to the Sampling guidelines established in Section 15.2.3, Criteria of Standards, since these samples are not being used for Settlement purposes.

(3)	For each segment and TOUS sampled, ERCOT shall determine the average Load for each hour of the study period from the Load research sample data.  This Load-research-based Profile shall then be compared to the existing chunked Load Profile, using the comparison methods in Section 8, Load Profile Models.  Referencing the terminology in Section 8, the chunked Load Profile is the existing Load Profile, and the Load Profile developed from the Load research sample is the proposed Load Profile.

(4)	Substantial differences between the two Load Profiles for many of the classes studied would indicate problems with the general approach.  Substantial similarities between the two Load Profiles for most classes would indicate that the general method is reasonable.  The results might also indicate that the method is adequate for some classes but not for others.

16.1.3.3	Assessment of Chunking Method Based on Characteristics of the						 Time Of Use and Non Time Of Use Populations

(1)	This assessment is less direct than the previous two assessment methods.  The goal is to determine whether behavioral or operational differences between the TOU and NOTOU Customers are large enough to create substantial differences between the true Load shape for the TOU group and the chunked Load Profile.

(2)	For this assessment, ERCOT may examine data on appliance/equipment use patterns for ERCOT TOU and NOTOU Customers.   Such data may be obtained from appliance saturation studies conducted by TDSPs, if available, or from a new survey conducted by ERCOT.  If little difference is found between TOU and NOTOU Customers in the types of equipment in place and timing of its use, the chunking method may be considered adequate.

(3)	If substantial differences are found, ERCOT may develop rough adjustments to the NOTOU Load Profile that reflect these differences.  Such adjustments would require estimated end-use Load shapes, which may be provided by TDSPs if available, or obtained from commercial databases.

(4)	The adjusted Load Profile would then be chunked to provide a new estimate of the TOU Load Profile.  This new TOU Load Profile would then be compared with the original TOU Load Profile, using the comparison methods of Section 8, Load Profile Models.

(5)	Referencing the terminology in Section 8, the original chunked Load Profile is the existing Load Profile, and the new Load Profile developed from chunking the adjusted NOTOU Load Profile is the proposed Load Profile.  However, this new Load Profile based on rough adjustments would not in fact replace the existing TOU Load Profile if substantial differences are found.  Rather, these differences would be taken as an indication that the chunking method is inadequate for this segment.

(6)	Likewise, if substantial differences are found by this method for several segments, the chunking methodology as a whole may be questioned.  Conversely, if several segments are examined and no substantial differences are found, the general chunking methodology is supported.

16.1.4	Revisions to Time Of Use Load Profile Methods if Changes Are Needed

If the current chunking is determined to be an inadequate methodology for Load Profiling TOU Customers, the change to any other Load Profiling Methodology for these Customers would require the Technical Advisory Committee (TAC) approval in accordance with the Protocol Section 18.2, Methodology.  The primary alternative that would be considered is lagged dynamic Load Profiling.  Other alternatives may be proposed.

CKWHbase=NCKWHld×CKWHcomNCKWHcom

16.2	Other Supplemental Load Profiles

(1)	Other supplemental Load Profiles may be developed for other types of programs or pricing schemes that encourage a Demand Response (DR) to price in the retail market.  The only supplemental Load Profiles permitted by ERCOT system functionality are Time Of Use (TOU).

(2)	Methodologies for any other supplemental Load Profile will be evaluated on a program by program basis.  Procedures and requirements for developing these Load Profiles shall be the same as those described in Section 16.2, Direct Load Control, except where specified in Section 16.4, Requesting Direct Load Control or Other Supplemental Load Profile.

ERCOT Load Profiling Guide

Section 17: Load Profile Metering

[DATE]

17	LOAD PROFILE METERING	1

17.1	Introduction	1

17.2	Interval Data Recorder Requirement	1

17.3	Demand Meter Changes	2

17.4	Load Research Samples	2

17.5	Metering for Supplemental Load Profiling	3

# 17	LOAD PROFILE METERING

## 17.1	Introduction

(1)	This Section defines the requirements for metering with regard to Load Profiling and Interval Data Recorder (IDR) activities.  Meter reading data in this context encompasses monthly consumption, Demand and interval meter data.  The Transmission and/or Distribution Service Providers (TDSPs) are the only Entities authorized to provide Settlement meter data to ERCOT in accordance with Protocol Section 10, Metering.

(2)	Each Electric Service Identifier (ESI ID) in ERCOT shall be assigned to a Load Profile ID.  Meter reading data is necessary to perform this assignment because the information used for assignment of the Load Profile ID is energy and/or Demand data.  Only meter reading data provided to ERCOT shall be used to assign the Load Profile ID.  The other primary uses of meter reading data are:

(a)	To allocate daily Load for Settlement and aggregation process;

(b)	To allow validation for Load Profile ID assignments;

(c)	To ensure Load Profile Models are appropriately specified; and

(d)	To allow for Load Profile Model development.

(3)	If an Advanced Meter is installed on a Customer’s Premise and has the capability to function as an IDR or lower level metering, data shall be supplied to ERCOT in accordance with its intended purpose to meet the needs of ERCOT billing and Settlement activities.

(4)	This Section addresses the following topics:

(a)	IDR requirement;

(b)	Demand meter changes;

(c)	Load research samples; and

(d)	Supplemental Load Profiling.

(5)	Details for metering activities may be found in Protocol Section 10, Metering.

## 17.2	Interval Data Recorder Requirement

(1)	Interval Data Recorders (IDRs) shall be installed or removed in accordance with Protocol Section 18.6, Installation and Use of Interval Data Recorders and Retail Market Guide Section 7.13, Interval Data Recorder (IDR) Optional Removal/Installation Process.

(2)	Costs associated with mandatory installation of IDRs by Transmission and/or Distribution Service Providers (TDSPs) shall be the responsibility of the TDSP and be in accordance with approved TDSP tariffs.

## 17.3	Demand Meter Changes

(1)	Section 9.2.1, Load Profile ID Changes Initiated by Transmission and/or Distribution Service Providers, presents the procedure for changing Load Profile ID assignment.  The following provides brief discussion regarding the circumstances, which may involve a meter change.

(2)	When a Transmission and/or Distribution Service Provider (TDSP) determines that a Demand meter should be changed based on the TDSP metering tariff rules, the TDSP shall notify the Competitive Retailer (CR) prior to making the meter change.  If the CR requires Demand data to support Customer billing for the Electric Service Identifier (ESI ID) in question, then the CR shall notify the TDSP of its requirement for Demand data.  Upon CR notification, the TDSP shall not change the Demand meter.

(3)	If the Demand meter is no longer needed by TDSP tariff or CR billing requirements, the TDSP shall reassign the ESI ID to the appropriate Load Profile ID in accordance with Section 9.2, Processes to Change Load Profile ID Assignments.  It is at the discretion of the TDSP whether to physically remove the Demand register/meter or perform a virtual meter change in their system.  A virtual meter change means that no Demands shall be reported to ERCOT.

(4)	Conversely, the ESI ID’s Load growth may warrant the measurement of Demand.  TDSPs shall enforce appropriate thresholds and TDSP tariffs requiring the installation of a Demand meter.

(5)	Once it has been determined that a Demand meter change is warranted, the TDSP shall make appropriate changes in accordance with Protocol Section 18.4, Assignment of Load Profile ID.  The TDSP shall notify the CR of the completed changes as well.

(6)	CRs may request the installation of a Demand meter for their Customers, regardless of TDSP thresholds, when required for application of the CR billing.  The CR is responsible for any costs associated with the Demand meter installation and monthly meter reading in accordance with the approved TDSP tariffs.

## 17.4	Load Research Samples

(1)	Any Interval Data Recorders  (IDRs) installed as part of the Load research program, i.e., in support of ERCOT Load Profiling or Transmission and/or Distribution Service Provider (TDSP) cost allocation/rate design, are not subject to the IDR requirements stated in Protocol Section 18.6.1, Interval Data Recorder Installation and Use in Settlement.  These IDRs used for Load research may be moved as needed.

(2)	ERCOT has the responsibility to monitor and evaluate current Load research samples in the field.  For ERCOT sponsored sample sites, ERCOT may request additions, deletions, or a wholesale removal and installation of the IDRs.  The process shall follow the Section 15, Load Research Samples.

## 17.5	Metering for Supplemental Load Profiling

(1)	If a Competitive Retailer (CR) wants supplemental Load Profiling (i.e., Direct Load Control (DLC), Time Of Use (TOU), etc.), the CR shall follow procedures in Section 16, Supplemental Load Profiling.  Metering for supplemental Load Profiling shall be in accordance with Protocol Section 10, Metering, and Protocol Section 18, Load Profiling.

(2)	All Interval Data Recorder (IDR) installations for supplemental Load Profiling shall be consistent with IDR metering requirements in Protocol Section 10.9.2, TSP or DSP Metered Entities.  Additionally, any TOU metering for supplemental Load Profiling shall be able to collect and record meter data into specified TOU periods approved by the Public Utility Commission of Texas (PUCT).

ERCOT Load Profiling Guide

Section 18: Access to Load Profiling Materials

[Date]

18	ACCESS TO LOAD PROFILING MATERIALS	1

# 18	ACCESS TO LOAD PROFILING MATERIALS

(1)	The following Load Profiling related documents and materials

may be found on the ERCOT website:

(a)	Backcasted (Actual) Load Profiles – Extract files of Load Profiles for individual trade days. The trade day occurring one day prior to the current date will be the most current backcast available.;

(b)	Forecasted Load Profiles – Files include current day and three days forward of forecasted Load Profiles;

(c)	Profile File Format – Spreadsheets that illustrate the layout of the Load Profile extract files;

(d)	Final Profile Model Report – Report that describes ERCOT Load Profile Models used at Market Open;

(e)	Historical Backcasted Load Profiles – Multiple years of Load Profile history for each Load Profile Type and Weather Zone combination;

(f)	Historical Weather Data by Weather Zone – Five years of historical hourly weather data by Weather Zone, covering 1996-2000;

(g)	Load Profile Data Evaluation Report – Documents that provide an evaluation of the utility data used to generate the ERCOT Load Profile Models.

(2)	Profiling Working Group (PWG) information and meetings

may be found on the ERCOT website.

ERCOT Load Profiling Guide

Appendix A

[DATE]

Appendix A	1

# Appendix A

Appendix intentionally left blank

ERCOT Load Profiling Guide

Appendix B

[DATE]

Appendix B	1

ERCOT Load Profiling Guide

Appendix C

[DATE]

Appendix C	1

# Appendix C

- Measuring Differences Between Load Profiles

- This Appendix describes and illustrates measures that may be used for assessing the differences between Load Profiles.

- Differences between Load Profiles are a consideration in many decisions regarding Load Profiling Methods and models, such as:

- Evaluating Load Profile Model performance or methodologies (Section 8, Load Profile Models, and Section 7, Requests for Changes to Load Profiling Methodology);

- Evaluating requests for changes to Load Profile Segments or Weather Zones (Section 12, Request for Load Profile Segment Changes, Additions, or Removals, and Section 13, Changes to Weather Zone Definitions); and

- Designing Load research samples (Section 15, Load Research Samples).

Target and Default Load Profiles

  - In most cases when Load Profile differences are measured, the question of concern is whether an existing or proposed method or model is adequate in a particular context, or an alternative is needed.  The alternative might be a finer Segmentation or Weather Zone, use of more recent or more local data in estimating models, or an alternative Load Profiling Methodology.  In all these cases, the analysis compares a “target” Load Profile against a “default” Load Profile.

  - The default Load Profile is the one generated by the existing method or model, or the one that is used if the alternative is not accepted.  The Target Profile is the best available estimated Load shape for a particular proposed subgroup.  The default Load Profile is good enough to represent the target if the two are not significantly different.  Some examples of default and Target Profiles in addressing particular questions are given in the following Table C-1, Examples of Target and Default Profiles.

Table C-1
Examples of Target and Default Profiles

Load Shape Parameters

Load shapes may be compared in terms of several different parameters that characterize the Load shape.  Some of these parameters are a series of numbers that jointly characterize the shape.  Others are single parameters that represent a key characteristic.

General Notation

The following general notation is used in this Appendix, and elsewhere in the Load Profiling Guide (LPG).  General quantities are defined.  Suffixes and subscripts are used to signify specific quantities.  Explicit definitions are given in the formulas that follow.

General Quantities

e 	= 	Elasticity of electricity Demand with respect to the commodity price

- E 	= 	Energy

f 	= 	Fraction

DWL 	= 	Deadweight loss

L 	= 	Load

LF	= 	Load factor

N 	= 	Number of intervals in a period for which the quantity is calculated

r 	=	Ratio

ut 	=	Market commodity price at interval t

T 	= 	Total across some calendar dimension (clock-hour, day-type, month, Season, year, on- or off-peak period)

U	= 	Load-weighted average price

- Suffixes

- D	= 	Daily

H 	= 	Clock-hourly

M 	= 	Monthly

S 	= 	Seasonal

Y 	= 	Yearly

- Subscripts

d 	=	Day

m 	= 	Month

h 	=	Clock-hour

p	= 	Day-type

ON 	= 	On-peak

OFF 	= 	Off-peak

s 	= 	Segment

t	= 	Interval

z	=	Weather Zone

- Series Parameters

- (1)	Series parameters include:

- Unitized Load shape, or interval fractions;

- Monthly fractions;

- Daily fractions; and

- Clock-hour fractions by day-type.

Unitized Load Shapes

  - A Load Profile defines the fraction of total energy use over a period that occurs in each time interval within the period.  For most comparisons of Load shapes for purposes of Load Profiling, the Load shapes shall first be unitized.  That is, the interval total or average Loads for the group are translated into interval fractions.

  - For most comparisons, the time period of interest is a year and the fractions are calculated as fractions of total annual energy consumption.  In some cases, shorter time periods such as a Season or month may be used.

  - The unitized Load ft for time interval t is calculated from the interval Loads Lt as

ft = (Load at interval t)/(sum of Loads over all intervals in the period).

= Lt/ T

where

t Lt

and the summation is over all intervals in the period.

  - Each of the other Load shape characteristics described below may be calculated using Loads Lt as indicated in the formulas, or using the unitized Loads ft in place of Lt.  The same result shall be obtained either way.

Profile Totals

If interval data are finer than hourly, the hourly Loads may be averaged for each hour of the period to get the hourly Demand, equal to the total energy in each hour.  For quarter-hour data, each hour’s four quarters are averaged to give the hourly value.

Hourly totals

TH are the sum over days in the period for each of the 24 hours in a day.  Hourly totals may be calculated across a year, a month, or a day-type.

- Daily totals

- TD are the sum over hours in each day of the hourly values.

Day-type totals

TP are the sum over all hours in the day-type of the hourly values.

- Monthly totals

- TM are the sum over days in the month of the daily totals.

Seasonal totals

TS are the sum over all the months in a Season of the monthly totals.

Yearly totals

TY are the sum over all the months in the year of the monthly totals.

On-peak totals

TON are the sum over all on-peak hours in the period of the hourly values.

Off-peak totals

TOFF are the sum over all off-peak hours in the period of the hourly values.

Profile Fractions

A year, Season, or month of Load data may be condensed into the fraction of total consumption occurring in each month, day, clock-hour, or day-type.

Monthly Fractions

- For each month m = 1 to 12, the monthly fraction fMm is calculated as

fMm	=	(sum of interval Loads in month m)/ (sum of all interval Loads in the year)

=	(tMm Lt)/ (t Lt ).

=	TMm/ TY.

where

t		= 	interval of time

Mm	= 	indicates month m

Lt	= 	Load at interval t

TMm and TY, respectively, are Load totals over month m and over the year.

Daily Fractions

- Daily fractions may be calculated similarly to monthly fractions.  Daily fractions may be calculated as fractions of the year or separately for each month.  The daily fraction for day d is calculated as

fDd	=	(sum of interval Loads in day d)/ (sum of all interval Loads in the period)

=	(tDd Lt)/ (t Lt ).

=	TDd/T

where

t	= interval of time

Dd	= day d

Lt	= Load at interval t

TDd and T, respectively are Load total sums over day d and over the entire period (e.g., year or month) for which the daily fractions are calculated.

- This calculation gives one daily fraction for every day of the month or year.

Clock-Hour Fractions by Day-type

- When hourly data are used, the unitized Load gives one hourly fraction for each hour of the month or year.  This information may be condensed to give the average clock-hour fraction for a period.  Each of the 24 hourly totals THph for the period is divided by the total for the period, TM, TP or TY.  For clock hour h, the clock-hour fraction for day-type p is calculated as

fHph = THph/TPp

where THph is the interval Load totals for day-type p and clock-hour h, and TPp is the total over all hours.

- Clock-hour fractions for a month or year are calculated analogously.

Single Parameters

- Single parameters include:

- Load factor;

- On-/off-peak ratio;

- Seasonal consumption ratio;

- Weekday fraction; and

- Load-weighted average price.

Load Factor

The Load factor for a period is the ratio of the average to the peak Load for the period:

LF = (average Load)/(maximum Load)

= (T/N) /max(Lt)

where

- Lt = Load at interval t

T = sum of interval Loads over all intervals in the period

N = the number of intervals in the period.

Average Load Factor

- Average Load factor is an average of monthly Load factors.  This average is computed as

AvgLF=m=112AHUsemm=112MaxKWm

where

- AHUsem = average hourly use in billing month m

= KWhm/(billing days x 24)

MaxKWm = maximum metered kW Demand in billing month m

- This definition and notation are consistent with Appendix D, Profile Decision Tree.

- In terms of the notation used elsewhere in this Appendix, if the interval t is hourly,

- AHUsem = Tm/Nm

and

MaxkWm = max(Lt)m.

AvgLF=m=112TmNmm=112maxt∈MmLtThus,

On-peak/Off-peak Ratio

The on-peak/off-peak ratio is the ratio of total consumption during on-peak periods to the consumption during off-peak periods.  The specific definition depends on the definitions of on-peak and off-peak periods.  The ratio is calculated as

ron/off	=	(on-peak consumption)/(off-peak consumption)

=	TON/TOFF

where

TON = sum of interval Loads over all intervals in on-peak periods

TOFF = sum of interval Loads over all intervals in off-peak periods.

Seasonal Consumption Ratios

Seasonal consumption ratios are the ratio of total consumption in one Season to consumption in another Season.  Most common is the ratio of summer to winter consumption.  The ratio of summer to annual or winter to annual consumption may also be used.  When using the seasonal consumption ratio calculation, the months of the Seasons being used shall be defined.  The ratios are calculated as

rS1/S2	=	(total for Season 1)/(total for Season 2)

=	TS1/TS2

where

TS1 and TS2 denote totals of interval Loads over the two Seasons being compared.

Load-Weighted Average or Annual Price

- The Load-weighted average price U for a period is calculated as

- U = Σt Lt ut /Σt Lt

where

Lt = Load at interval t

ut = market price for commodity at time interval t

the summation is over all intervals t in the period.

- Equivalently, the Load-weighted average price may be calculated from the unitized Loads as

- U = t ft ut.

where

ft is the unitized Load defined above.

- Load-weighted average price is most often considered on an annual basis.  When the period is annual the Load-weighted average price is also called the Load-weighted annual price.

- Unlike the other parameters defined, Load-weighted average price is not only a characteristic of a Load shape, but depends also on the price series ut.  When Load-weighted average price is used, the price series shall be specified.

Hourly and Quarter-Hourly Load Data

- Although ERCOT uses quarter-hourly data for Settlement, hourly Load data may be used for supporting analysis to assess Load Profiling Methods.  When hourly data are used, the intervals t are hourly.  In this case, the Load Lt in Demand for each interval t is equal to the energy for the interval, and the sum of the Loads T is the total profile energy for the period.

- For calculations expressed as the ratio of Loads, the ratio may be calculated in the same way whether the Load data are hourly or quarter-hourly.  The same result will be obtained either way.  This rule applies to unitized Loads, Load factor, on-/off-peak ratio, and seasonal consumption ratios.

Measuring Differences Between Two Load Profiles

Direct Comparison of Simple Parameters

- Simple parameters are those that represent a Load shape in terms of a single number.  These parameters include the following:

- Load factor;

- On-/off-peak ratios;

- Seasonal consumption ratios; or

- Load-weighted average price.

  - Each simple parameter may be the basis for measuring differences between Load Profiles.  The value of the parameter for the Target Profile is subtracted from the value for the default profile.  For example, the difference in Load factors is expressed as:

LF = LFDefault – LFTarget

  - This difference may be thought of as the magnitude of the error if the default profile is used to represent the target.

Comparison of Two Series

- A Load Profile over a year may be characterized by:

- Its unitized Load shape;

- The separately unitized Load shapes for each month;

- The 12 monthly fractions;

- The 24 clock-hour fractions for the year; or

- The set of 24 clock-hour fractions for each month or day-type.

  - For any of these series, two Load Profiles may be compared in terms of various summary measures of the difference between their two series.  Each of these summary measures is called a measure of “error” or “deviation.”  In the context of comparing two Load Profiles, this error is a measure of how far the default Load profile is from the target.

Mean Deviation

The simplest measure of difference is the average difference in values or deviation between corresponding elements of the series.  The Mean deviation is calculated by:

- Taking the difference between the default and target for each element in the series; or

- Taking the Mean of these differences over all terms in the series.

Mean Absolute Deviation (MAD)

MAD is calculated by:

- Taking the difference between the default and target for each element in the series;

- Taking the absolute value of each difference; or

- Taking the mean of these absolute differences over all terms in the series.

Thus,

- MAD = (1/J) j |XTARGETj - XDEFAULTj|

where

J is the number of elements in the series (e.g., 12 for monthly fractions, 365 for daily fractions of the year, 24 for clock-hour fractions of a period).

Mean Absolute Percent Error (MAPE)

The MAPE is calculated by:

- Taking the difference between the default and target for each element in the series;

- Taking the absolute value of each difference expressing this absolute value as a percent of the target value; or

- Taking the Mean of these absolute percent errors over all terms in the series.

Thus,

- MAPE = (1/J) j |XTARGETj - XDEFAULTj|/XTARGETj

where

J is the number of elements in the series (e.g., 12 for monthly fractions, 365 for daily fractions of the year, 24 for clock-hour fractions of a period).

Root Mean Square Error (RMSE)

The RMSE is calculated by:

- Taking the difference between the default and target for each element in the series;

- Squaring each difference;

- Taking the Mean of the squared difference over all terms in the series; or

- Taking the square root of the Mean squared difference.

Thus,

where

J is the number of elements in the series (e.g., 12 for monthly fractions, 365 for daily fractions of the year, 24 for clock-hour fractions of a period).

Measuring Differences for a Group of Load Profiles

Some decisions require comparisons across a group of Load Profiles that are jointly affected by a possible change.  Key examples of such decisions are to either:

- Subdivide an existing segment into smaller segments,

- Subdivide an existing Weather Zone into smaller Weather Zones,

- Change segment definitions in a way that shall affect multiple segments, or

- Change the boundaries of Weather Zones in ways that shall affect multiple Weather Zones.

Deadweight Loss Reduction

One measure that reflects the combined effects of a change on several Load Profiles is deadweight loss.  A reduction in deadweight loss is the gain in economic efficiency due to providing Customers with Load Profiles that are closer to their true Load shapes.  This reduction is a value to society, measured in dollars per year.  Given that ERCOT’s costs are ultimately paid by consumers through their electricity rates, this societal value is theoretically the maximum that it would be worth to implement a change paid for by ERCOT.  Changes that would cost more than this to implement shall cost more than the economic value of the benefit.

Deadweight Loss Reduction from Finer Subdivision

- When performing an analysis where a single group is divided into smaller groups, the Load Profiles for the smaller segments or zones are considered the Target Load Profiles. The profile for the single group is considered the default Load Profile.  The difference between the default and each of the Target Profiles may be measured by any of the difference measures described in this Appendix titled Measuring Differences between Two Load Profiles, as described in Section 12.6.2, Difference from Current Load Profile Segments.

- The combined effect of subdividing may be expressed as the deadweight loss reduction.  The calculation formula is

DWL = ½ e k Ek U0 ((Uk – U0)/U0)2

- Where

e	=	Elasticity of electricity Demand with respect to the commodity price

Ek	=	Total annual consumption for subgroup k of group 0 (kWh)

Uk	=	Load-weighted annual price for subgroup k of group 0 ($/kWh)

U0	=	Load-weighted annual price for the group 0 ($/kWh).

- For purposes of this calculation, elasticity estimates from secondary sources may be considered, and scenario analysis using a range of values may be used.  A value of 0.2 has been used in some studies.  Note that the annual consumption Ek is the total energy use of all Customers represented by the subgroup profile k.  If the profile is scaled so that the profile hourly value is an estimate of total Load for all Customer represented by the profile, the annual consumption Ek is the sum of the profile hourly values over all hours in the year.  If the profile is scaled in some other way, the group annual consumption Ek may be very different from the sum of the profile hourly values.

- To apply this formula to subdivisions of a Weather Zone, the combined group 0 is the entire Weather Zone.  Each subgroup k is a subdivision proposed as a new Weather Zone.  The formula is evaluated separately for each Load Profile Segment, and summed over segments to provide the total benefit of the Weather Zone subdivision.

- To apply this formula to subdivisions of a segment, the combined group 0 is the entire current segment.  Each subgroup k is a subdivision proposed as a new segment.  The formula is evaluated separately for each Weather Zone, and summed over Weather Zones to provide the total benefit of the segment subdivision.

- In either case, the subgroup profiles k defined by profile segment and Weather Zone are applied to the interval prices ut to produce distinct Load-weighted average annual prices Uk.  The Load-weighted average annual price for the existing segment or Weather Zone is U0.

- Deadweight Loss Reduction if Only One Subdivision Changes

If only one proposed new segment shall have a new Load Profile, while the remainder shall continue to have the existing Load Profile, the deadweight loss formula reduces to

DWL = ½ e  Ek* U0 ((Uk* – U0)/U0)2

where the subscript k* indicates the proposed new segment.  All other terms in the summation for the full formula are zero.

Deadweight Loss Reduction by Creating a New Segment from Multiple Segments

- When performing an analysis where several groups are being combined to form a single group, the Load Profile for the single group is considered the Target Load Profile.  The Load Profiles for the groups contributing to the formation of the single group are considered the default Load Profiles. Examples where such analyses shall apply include:

- Changing a Weather Zone boundary so that a portion is removed from one zone and added to another

- Changing a set of Load Profile Segment definitions so that part of one segment is shifted from one to another

- Choosing between two alternative schemes for defining Weather Zones or Load Profile Segments.

  - Any of these choices may be assessed as a difference among possible subdivisions using the formula titled “Deadweight Loss from Finer Subdivision.”   If a single group may be divided into subgroups the deadweight loss reduction from each possible subdivision is calculated using the formula.  The method with the greatest deadweight loss reduction is the preferred method.

  - Thus, to request a change of the definitions of existing segments, a combined segment that includes all segments affected by the change is considered.  The deadweight loss reduction from “subdividing” the combined segment is then calculated.  Likewise, the deadweight loss reduction from subdividing the combined segment into the proposed new set of definitions is calculated.  The reduction from the current segmentation is subtracted from the reduction from the proposed segmentation.  Theoretically, the proposed segmentation is justified in terms of societal value if this difference is greater than the total cost of implementing the change.

  - A change that involves moving a part of a group into another group, or a combination of such moves, may be assessed using this same approach.  The combination of all affected subgroups is considered as the overall group.  The original and alternative groupings are then regarded as two possible subdivisions of this overall group.  The deadweight loss reduction compared to having the full combined group is evaluated for each of these “subdivisions.”  The preferred subdivision is the one with the greater deadweight loss reduction from the combined group.  The benefit of going from the original subdivision to the new one is the increase in the deadweight loss reduction.

  - For example, if the group A is to be moved from being included with group B to being included with group C, the combined group is the combination of A, B, and C.  Under one “subdivision” the subgroups are A+B and C.  Under the other, the subgroups are B and A+C.  The change is theoretically worth making if the deadweight loss reduction for (B, A+C) versus (A+B+C) is greater than that for (A+B, C) versus (A+B+C), by an amount greater than the cost of making the change.

Deadweight Loss Reduction from Revising the Load Profile Model Based on Current Load Research Data

(1)	The Load Profile based on current Load research may be regarded as the best available estimate of the Load Profile for a particular segment.  Continuing to use the current model is then viewed as an approximation to this best available or Target Profile.  One way to measure the severity of the error in this approximation is in terms of the deadweight loss.  Revising the current Load Profile to bring it closer to the target would reduce societal deadweight loss by at most this amount.

Table C-2

Hypothetical Loads and Prices for Illustration

(2)	The deadweight loss due to using the current Load Profile Model (default) rather than the target is calculated as

- DWL = ½ e E UDEFAULT ((UTARGET – UDEFAULT)/UDEFAULT)2

- Where

e 		= 	Elasticity of electricity Demand with respect to the commodity price

E 		= 	Total energy consumption for the profile segment (MWh)

UTARGET 	= 	Load-weighted average annual commodity price using the Target Profile for the segment ($/MWh)

UDEFAULT = Load-weighted average annual commodity price using the default profile for the segment ($/kWh).

- Illustration of Measures of Differences

- To illustrate some of the measures of differences, following Table C-2, Hypothetical Loads and Prices for Illustration, shows hourly Loads and hourly market prices for a hypothetical period of two days.  Loads are shown for two sub segments that together make up an existing segment within a Weather Zone.  The highlighted hours are on-peak hours, which are defined as hour ending 0800 through 1900.

- Following Table C-3, Single Parameter Characteristics and Differences, shows some of the single-parameter characteristics statistics for each of the three Load Profiles.  Also shown are the differences between each subsegment and the existing combined segment.  These differences are the errors if the subsegments are considered as the targets and the combined is the default that estimates them if the finer Segmentation is not adopted.

Table C-3

Single Parameter Characteristics and Differences

- Following Table C-4, Daily and Clock-Hour Totals and Fractions, shows the calculations of daily and clock-hour totals and fractions.

Table C-4
Daily and Clock-Hour Totals and Fractions

- Following Table C-5, Unitized Loads and Difference Measures, shows the unitized Load for the two-day period, and illustrates some of the difference measures based on this series.

Table C-5
Unitized Loads and Difference Measures

First, the unitized Loads are shown for each Load Profile.  The differences or errors between each segment and the existing Load Profile are then shown for the unitized Loads.  Also shown are the absolute difference, squared error, and absolute percent errors.  These are combined at the bottom to give the mean deviation, MAD, MAPE, and RMSE.

- Following Table C-6, Calculation of Deadweight Loss Reduction for Finer Segmentation, illustrates a calculation of the reduction in deadweight loss achieved by changing from the single existing Load Profile to separate Load Profiles for each subsegment.

Table C-6
Calculation of Deadweight Loss Reduction for Finer Segmentation

ERCOT Load Profiling Guide

Section 19: Definitions and Acronyms

[Date]

19	 Definitions and Acronyms	1

19.1	 Definitions	1

A	[Back to Top]	1

B	[Back to Top]	1

C	[Back to Top]	1

D    [Back to Top]	2

E    [Back to Top]	2

F	[Back to Top]	2

G	[Back to Top]	2

H	[Back to Top]	2

I	[Back to Top]	2

J	[Back to Top]	2

K	[Back to Top]	3

L	[Back to Top]	3

N	[Back to Top]	4

O	[Back to Top]	4

P	[Back to Top]	4

Q	[Back to Top]	4

R	[Back to Top]	4

S	[Back to Top]	4

T	[Back to Top]	4

U	[Back to Top]	5

V	[Back to Top]	5

W	[Back to Top]	5

X	[Back to Top]	5

Y	[Back to Top]	5

Z	[Back to Top]	5

19.2	 Acronyms	5

## 19		Definitions and Acronyms

## 19.1		Definitions

The defined terms in this Section are limited to those used specifically in the Load Profiling Guide (LPG).  Any additional defined terms used in the LPG can be found in Protocol Section 2, Definitions and Acronyms.

- Links to definitions:

- A, B, C, D, E, F, G, H, I, J, K, L, M, N, O, P, Q, R, S, T, U, V, W, X, Y, Z;

List of Acronyms

A	[Back to Top]

- Adjusted Static Models

Load Profiles that are generated from statistical models that are based on static historical Load data, and adjusted for conditions of the day (e.g., weather, Season, etc.)

Annual Validation

The formal process performed every year whereby ERCOT re-determines the first component of each Load Profile ID—the Load Profile Type—for Residential and Business Load Factor Electric Service Identifiers (ESI IDs).  ERCOT then works with the Transmission and/or Distribution Service Providers (TDSPs) to have them update ERCOT’s databases with the resulting Load Profile ID changes via Texas Standard Electronic Transactions (TX SET).

B	[Back to Top]

Business (BUS)

Load Profile Group designation for non-residential Electric Service Identifiers (ESI IDs) whose service is metered.  This encompasses rate classes for business ESI IDs, in addition to other classes.

C	[Back to Top]

- Cutover and Conversion

- Initial data transfer of Transmission and/or Distribution Service Providers’ (TDSPs’) Electric Service Identifier (ESI ID) data into the ERCOT systems.

D   [Back to Top]

E   [Back to Top]

- Electric Service Identifier (ESI ID)

See Protocol Section 2.1, Definitions.

Active ESI ID

ESI ID is presently receiving service (energized) and a Retail Electric Provider (REP) is currently assigned to it in ERCOT’s system.

De-Energized ESI ID

ESI ID does not have a REP assigned in ERCOT’s system, but has not been retired.  An 814_16, Move-In Request is necessary to change to active status.

Inactive ESI ID

ESI ID is retired and will never again receive service.

- Engineering Estimated

Estimated Loads based on engineering studies applied to unmetered Loads to allocate energy across specified periods of time.

F	[Back to Top]

G	[Back to Top]

H	[Back to Top]

I	[Back to Top]

- Interval Data Recorder (IDR) Requirement

The kW level at which the installation of interval data recorders are required for settlement purposes as set forth in Protocol Section 18.6.1, Interval Data Recorder Installation and Use in Settlement.

J	[Back to Top]

K	[Back to Top]

L	[Back to Top]

- Lagged Dynamic Profiling Methodology

The use of an active set of Load research sample sites to build an aggregated Load Profile for the sample group from actual metered usage processed after the target day.

- Load Profile Class

- A classification of a group of Customers having similar energy usage patterns and that are assigned the same Load Profile.  Load Profile Class is comprised of a Load Profile Group and a Load Profile Segment.  An example of a Load Profile Class:  Residential Low Winter Ratio (RESLOWR).  Load Profile Type and Load Profile Class are used interchangeably.

- Load Profile Group

- A high-level classification of a set of Customers who have similar characteristics.  The Load Profile Groups are:  Non-Metered, Residential, and Business.  Together, the Load Profile Group and the Load Profile Segment form the Load Profile Type.

- Load Profile ID

- The Load Profile designation string that contains, the Load Profile Type Code,  the Weather Zone Code, the Meter Data Type Code, the Weather Sensitivity Code, and the Time-Of-Use Schedule Code.  All Load Profile IDs are listed in Appendix D, Profile Decision Tree.

- Load Profile Models

Processes that use analytical modeling techniques to create Load Profiles.

Load Profile Segment

A sub-classification of a Load Profile Group.  High Winter Ratio (HIWR) is an example.  Together, the Load Profile Group and the Load Profile Segment form the Load Profile Type.

M	[Back to Top]

- Market Open

January 1, 2002

- Mean

A sample statistic or population parameter equal to the sum of all observations divided by the number of observations

Meter Data Type

The component of the Load Profile ID that identifies the type of meter data—either interval or non-interval—that is to be submitted to ERCOT by the Transmission and/or Distribution Service Provider (TDSP) and used for settlement.

.

N	[Back to Top]

O	[Back to Top]

Opt-In Entity

A Municipally-Owned Utility (MOU) or Electric Cooperative (EC) opting-in to Customer Choice

P	[Back to Top]

- Power Factor

The ratio of real power (kW) to the apparent power (kVA) for any given Load and time.

- Profile Decision Tree

- The document that contains the directions for determining the Load Profile ID to be assigned to an Electric Service Identifier (ESI ID)

- Profile Type

- See Load Profile Class

Q	[Back to Top]

R	[Back to Top]

- Representative Interval Data Recorder (RIDR)

The technique for profiling Premises participating in special pricing programs which consists of implementing a statistically representative Load research sample on the program population.  The sample data is then used to develop the RIDR for profiling these Premises.

Residential (RES)

- Load Profile Group designation for Electric Service Identifiers (ESI IDs) served within a residential rate class.

S	[Back to Top]

- Sample Design

The processes by which ERCOT determines the appropriate requirements for a sample of Customer Premises which requirements shall be used to create a Load Profile.

- Segmentation

The process of dividing a population into a number of sub-sets, according to certain parameters, for the purpose of creating Load Profiles for sub-sets of the population.

T	[Back to Top]

- Target Profile

The Target Profile is the best available estimated Load shape for a particular proposed subgroup.

U	[Back to Top]

Usage Month

- Each Usage Month corresponds with a calendar month and is a combination of one or more usage periods for the purpose of applying usage and Demand values in a consistent manner.

- Usage Profile

- See Load Profile

V	[Back to Top]

W	[Back to Top]

Winter Ratio

The proportion of usage in winter months to usage in the fall base and spring base months and is used to differentiate residential Electric Service Identifiers (ESI IDs).

X	[Back to Top]

Y	[Back to Top]

Z         [Back to Top]

19.2		Acronyms

The defined terms in this Section are limited to those used specifically in the Load Profiling Guide (LPG).  Any additional defined terms used in the LPG can be found in Protocol Section 2, Definitions and Acronyms.

BUL	Balancing Up Load

COPS	Commercial Operations Subcommittee

DR	Demand Response

HIWR	High Winter Ratio

LPG	Load Profiling Guide

LPGRR	Load Profiling Guide Revision Request

MAD	   Mean Absolute Deviation

MAPE	Mean Absolute Percent Error

NIDR	Non-Interval Data Recorder

NOAA	National Oceanic and Atmospheric Administration

NODEM	Non-Demand

NOTOU 	Non-Time-Of-Use

NWS	Non-Weather Sensitive

PWG	Profiling Working Group

LOWR	Low Winter Ratio

RIDR	Representative Interval Data Recorder

RMSE	Root Mean Square Error