| NOGRR Number | 045 | NOGRR Title | Synchronization of OGRR219, Time Error Correction |
| --- | --- | --- | --- |
| Date Posted |  | June 10, 2010 |  |

| Nodal Operating Guide Section Requiring Revision (Include Section No. and Title) | 2.2.9.1, Time Error |
| --- | --- |
| Protocol Section(s) Requiring Revision, if any | None. |
| Requested Resolution (Normal or Urgent, and justification for Urgent status) | Normal |
| Revision Description | This Nodal Operating Guide Revision Request (NOGRR) synchronizes the Nodal Operating Guide with Operating Guide Revision Request (OGRR) 219, Time Error Correction, which removes the requirement that ERCOT perform time error corrections for accumulated time errors exceeding five seconds. |
| Reason for Revision | Synchronizes the Nodal Operating Guide with Operating Guide Revision Request OGRR219. |
| Overall Market Benefit | Synchronized guides. |
| Overall Market Impact | None. |
| Consumer Impact | None. |
| Submitter Justification for Necessity Prior to the Texas Nodal Market Implementation Date | (1) Synchronization needed to have a clear transition from one set of rules to another for ERCOT and Market Participants to operate within at that time of Nodal go-live. (2) Synchronization will give ERCOT more discretionary authority regarding when to perform time error correction without impacting grid reliability and exposure to non-compliance with Nodal Operating Guide requirements. (a) Time error correction can increase the Area Control Error (ACE) significantly if performed when ERCOT’s magnitude of frequency excursion is high. (b) It was observed during current North American Electric Reliability Corporation (NERC) Balancing Authority ACE Limit (BAAL) field trial reporting that, the duration of exceedance due to time error correction was longer than it would have been without time error correction. Exceedance for longer than 30 consecutive minutes will result in violation once the draft BAL-007 standard becomes effective in its current form. |
| CEO Determination (To be completed by ERCOT) | The ERCOT CEO has determined that NOGRR045 is necessary prior to the Texas Nodal Market Implementation Date (TNMID). |

| Quantitative Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | None. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Synchronized guides. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | None. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | John Dumas/Sandip Sharma |
| E-mail Address | jdumas@ercot.com/ssharma@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3195/512-248-4298 |
| Cell Number |  |
| Market Segment | N/A. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sandra Tindall |
| E-Mail Address | stindall@ercot.com |
| Phone Number | 512-248-3867 |

Proposed Guide Language Revision

2.2.9.1	Time Error

Sustained frequency deviations from scheduled frequency result in time error.  Time error will be monitored and controlled in ERCOT as follows:

(1)	Time Error Monitoring - ERCOT will monitor accumulated time error and initiate time corrections.  The instantaneous time error is available to all ERCOT QSEs on the ERCOT MIS Public Area.  When time error is equal to or greater than ±3 seconds, ERCOT may initiate a time correction.  The correction may end when the error is less than ±0.5 seconds, or when system events mandate termination.  The time correction may be postponed if it is determined that load patterns in the immediate future will result in the desired time correction.

(2)	Time Error Correction - When a time correction is necessary, ERCOT will adjust scheduled frequency by arranging for more or less Resources by implementing a frequency offset.  Information will be communicated via hotline call to all QSEs, which will include the frequency offset (-.02 Hz for fast and +.02 Hz for slow) and the start time.  A time correction may be terminated after five hours, or after any hour without a 0.5 second error reduction.  ERCOT will provide adequate notice of the ending of a time correction to all QSEs in the ERCOT Region.