| PRR Number | 850 | PRR Title | Weather Responsiveness Determination for Interval Data Recorders |  |
| --- | --- | --- | --- | --- |
| Timeline | Urgent | Action |  | Approved |
| Date of Decision |  | September 21, 2010 |  |  |
| Effective Date |  | October 1, 2010 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Protocol Section Requiring Revision |  | 11.4.3, Interval Consumption Data Estimation 11.4.3.1, Weather Responsiveness Determination |  |  |
| Revision Description |  | This Protocol Revision Request (PRR) limits the applicability of the weather responsiveness test to Interval Data Recorder (IDR) Meters. |  |  |
| Reason for Revision |  | Current Protocols require ERCOT to perform a weather responsiveness test on Electric Service Identifiers (ESI IDs) with IDRs and does not differentiate between Advanced Meters and IDR Meters. Performing this test on Advanced Meters has a significant impact on ERCOT system processing times. ERCOT estimates that it would take approximately eight days of system processing time to perform this check on ESI IDs with an Advanced Meter. On Operating Day 6/01/2010, data was being processed for more than 900,000 Advanced Meters. Test runs have shown the data loading portion of this job to be ¾ of a second per ESI ID. - 900,000 ESI IDs * .75 seconds per ESI ID = 675,000 seconds - 24 hours/day * 60 min/hour * 60 sec/min = 86,400 sec/day - 675,000 sec / 86,400 sec/day = 7.8125 days This job running for nearly eight days would impact many core deliverables, including: - Timely completion of core wholesale aggregation and financial Settlements. - Timely loading of Advanced Meter System (AMS) interval data from the market. |  |  |
| Overall Market Benefit |  | Eliminates a process that has significant impact to ERCOT system processing time. |  |  |
| Overall Market Impact |  | Removes impediment that could impact the completion of core system deliverables. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed PRR850 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 8/11/10, PRR850 was posted. - On 8/13/10, the motion to grant PRR850 Urgent status passed via PRS e-mail vote. - On 8/19/10, the CEO Revision Request Review and the associated Impact Analysis were posted. - On 8/19/10, PRS considered PRR850. - On 9/2/10, TAC considered PRR850. - On 9/14/10, ERCOT comments were posted. - On 9/21/10, the ERCOT Board considered PRR850. |  |  |
| PRS Decision |  | On 8/19/10, PRS unanimously voted to recommend approval of PRR850 as submitted and forward PRR850 and the Impact Analysis to TAC. The Independent Power Marketer Market Segment was not present for the vote. |  |  |
| Summary of PRS Discussion |  | On 8/19/10, ERCOT Staff explained that without this Protocol change, the ERCOT system processing time required to perform weather sensitivity tests will jeopardize ERCOT’s ability to complete core Settlement runs. ERCOT Staff also confirmed that a Nodal Protocol Revision Request (NPRR) will be submitted to synchronize the Nodal Protocols with the Zonal Protocols. It was noted that the Profiling Working Group (PWG) reviewed the language prior to submission and stated that the intent of the market was to consider IDR Meters only for weather responsiveness tests. |  |  |
| TAC Decision |  | On 9/2/10, TAC unanimously voted to recommend approval of PRR850 as recommended by PRS in the 8/19/10 PRS Report. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 9/2/10, there was no discussion. |  |  |
| Board Decision |  | On 9/21/10, the ERCOT Board approved PRR850 as recommended by TAC in the 9/2/10 TAC Report and as amended by the 9/14/10 ERCOT comments. |  |  |

| Quantitative Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | Unknown. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Eliminate a process that has a significant impact to ERCOT system processing time. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Debbie McKeever on behalf of the Commercial Operations Subcommittee (COPS) |
| E-mail Address | Deborah.Mckeever@oncor.com |
| Company | Oncor |
| Phone Number | 214-486-5883 |
| Cell Number |  |
| Market Segment | Investor Owned Utility (IOU) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sonja Mingo |
| E-Mail Address | smingo@ercot.com |
| Phone Number | 512-248-6463 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 091410 | Corrected the baseline language. |

Proposed Protocol Language Revision

11.4.3	Interval Consumption Data Estimation

ERCOT will estimate all ESI IDs with IDRs for which consumption data has not been received for the Operating Day.  The method for estimating interval data for ESI IDs with IDR Meters is a “Weather Response Informed Proxy Day” technique.  This approach seeks to increase estimation accuracy by segmenting ESI IDs with IDR Meters into two groups based on a known indicator of Load, i.e. weather.  The classification of ESI IDs with IDR Meters into a weather-sensitive group and a non-weather-sensitive group determines the proxy day method used for estimation purposes.  The proxy day estimation method for each group captures the factors that best predict the ESI ID-specific Load shape for the Operating Day.

The Weather Sensitive Proxy Day Method will be used for estimating interval data for ESI IDs with Advanced Meters.

11.4.3.1	Weather Responsiveness Determination

ERCOT shall perform the weather responsiveness test for all ESI IDs with IDR Meters as specified below.

For each ESI ID with an IDR Meter, two (2) variables shall be calculated for each Business Day on which the ESI ID is active and for which actual interval data is available during the summer (June 1st - September 30th) immediately preceding the date the test is run:

(1)	Daily kWh; and

(2)	Average Weather Zone daily dry bulb temperature.

=	((MAX + MIN) / 2)

Where:

MAX	=	maximum Weather Zone daily dry bulb temperature

MIN	=	minimum Weather Zone daily dry bulb temperature

For each ESI ID an R-square (Pearson Product Moment Coefficient of Determination) shall be calculated between these two (2) variables, and all ESI IDs with R-square greater than or equal to 0.6 shall be classified as Weather Sensitive and all ESI IDs with an R-square less than 0.6 shall be classified as Non-Weather Sensitive.

The weather responsiveness determination shall be performed annually between November 1st and November 15th.

No later than November 20th, ERCOT shall produce a report that contains the ESI IDs that require profile code modifications as a result of the weather responsiveness test.  This report shall be published to Market Participants in a data extract via the Market Information System (MIS) by November 20th.

If an ESI ID is inactive or de-energized at the time the weather responsiveness test is performed, or if it is de-energized for fifty percent (50%) or more of the time period beginning June 1st and ending September 30th, it shall retain its current weather sensitivity classification and shall not be re-evaluated until the following year.

If, for a specific ESI ID, fifty percent (50%) or more of the data required for the calculations described above is missing, the ESI ID shall retain its current weather sensitivity classification.

Beginning on December 1st, and continuing monthly thereafter until May of the following year, ERCOT shall repeat the weather responsiveness test.  These tests shall be limited to ESI IDs that had some missing data during the previous summer period when the most recent weather responsiveness test was performed.  As above, ERCOT shall produce a report that contains the ESI IDs requiring profile code modifications and shall publish the report via the MIS.

TDSPs shall successfully complete Weather Sensitivity Code modifications (Load Profile ID changes) no later than sixty (60) days after the ESI ID appears on the ERCOT report.  Load Profile ID changes shall be effective as of the most current meter read date.

On a monthly basis, ERCOT shall produce a report of ESI IDs that are overdue in having their Weather Sensitivity Codes modified by the above referenced tests.

As a part of the Load Profile Class assignment, TDSPs will assign a Non-Weather Sensitive classification to all newly installed IDR Meters and a weather sensitive classification to all Advanced Meters.