| NPRR Number | 263 | NPRR Title | Resolution of Alignment Item A99 - Settlement Point Price Calculation When Busses are De-energized |
| --- | --- | --- | --- |
| Date |  | September 9, 2010 |  |
| Submitter’s Information |  |  |  |
| Name |  | Matt Mereness |  |
| E-mail Address |  | mmereness@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-3089 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable |  |

Comments

The current Nodal Protocols are silent on the assignment of Locational Marginal Prices (LMPs) in the case of de-energized Electrical Busses.  However, the Market Management System (MMS) was designed to assign LMPs to de-energized Electrical Busses for use in the calculation of Day-Ahead and Real-Time Settlement Point Prices.  Accordingly, ERCOT submitted Nodal Protocol Revision Request (NPRR) 263 for the purpose of informing interested parties that ERCOT would be assigning LMPs in these cases.

The current MMS design for determining the LMP at de-energized buses in the Day-Ahead Market (DAM) and Security-Constrained Economic Dispatch (SCED) is documented in the Overall MMS System and Other Processes Requirements document (09Q1, v3.0).  The relevant portion provides:

## LMP for Deenergized Electrical Buses:

| Requirement Id | FR36 |
| --- | --- |
| Source Mapping (Protocol/NERC/FERC and other binding documents Ref #) |  |
| Traceability to Sub-Process Maps |  |
| Description: For posting purposes, as a post processing for DAM and SCED, MMS shall assign LMPs to de-energized Electrical Buses by following heuristic rules applied in specified order as listed below: - Use average LMP for Electrical Buses within the same station having the same voltage level as de-energized Electrical Bus, if exist - Use average LMP for all Electrical Buses within the same station, if exist - Use system lambda. Note: If Network Topology processor determines that an Electrical Bus is not in an energized electrical island, then this Electrical Bus is a de-energized Electrical Bus. |  |

NPRR263 does not change the way in which ERCOT will calculate Day-Ahead and Real-Time Settlement Point Prices.  Rather, NPRR263 merely serves to notify stakeholders that ERCOT will assign LMPs for de-energized Electrical Busses, which was always the intent, as set forth in the Overall MMS System and Other Processes Requirements.  Disclosing this intent in the Nodal Protocols lowers the potential for Settlement disputes.

Revised Proposed Protocol Language

None.