| NPRR Number | 272 | NPRR Title | Definition and Participation of Quick Start Generation Resources |
| --- | --- | --- | --- |
| Date |  | October 4, 2010 |  |
| Submitter’s Information |  |  |  |
| Name |  | John Dumas |  |
| E-mail Address |  | jdumas@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-3195 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable |  |

Comments

ERCOT Response to comments filed by Topaz

- Topaz comment regarding changes to Section 3.8.3, paragraph (7)

ERCOT opposes the revision suggested by Topaz because it is not required.

The language in Section 3.8.3, Quick Start Generation Resources, paragraph (7) was included in order to address a specific situation that could occur until NPRR222 (Half-Hour Start Unit RUC Clawback (Companion to NPRR207)) is implemented post Texas Nodal Market Implementation Date (TNMID).  The Qualified Scheduling Entity (QSE) will only be subjected to Reliability Unit Commitment (RUC) Clawback Charges under these circumstances:

- A RUC commitment in which the QSE has not self-committed the Resources in any of the adjacent hours, the QSE may or may not have a Three-Part Supply Offer in the Day-Ahead, and the QSE has revenue in excess of the RUC Guarantee.  This is consistent will all Resources that are RUC committed.  Even after NPRR222 implementation, a Quick Start Generation Resource (QSGR) will be held to this rule if RUC committed without a Three-Part Supply Offer , even though clawback factors will be reduced.

- A RUC commitment in which the QSE self commits-in hours leading into or following a RUC instruction but ONLY after the QSE has received the RUC instruction.  If the QSE has revenue in excess of the RUC Guarantee during the RUC hours and the clawback hours combined, it would be subjected to clawback charges.  The QSE can avoid these charges by not self-committing in an hour adjacent to a RUC instruction and/or having a Three-Part Supply Offer in the Day-Ahead Market (DAM).

The QSGR is encouraged to follow practice described in (b) above until NPRR222 is implemented.

- Topaz comment regarding Security-Constrained Economic Dispatch (SCED) de-commitment

Following a SCED QSGR deployment, the QSGR is expected to follow the SCED Base Points.  If the QSGR’s SCED Base Point equals the QSGR’s physical Low Sustained Limit (LSL) provided by telemetry and the QSE desires to disconnect the QSGR from the ERCOT System, the QSE must obtain the approval of the ERCOT Operator before it removes the QSGR from the ERCOT System.  The QSE may place the QSGR in an OFF Resource Status or restore the QSGR to the conditions described in Section 3.8.3 (Resource Status = ON; Output Breaker Status = OPEN; LSL = 0).

If this request is not approved, the ERCOT Operator will manually set the QSGR’s Low Dispatch Limit (LDL) = High Dispatch Limit (HDL) until the request can be approved.  This action will result SCED issuing the QSGR a Base Point equal to its current net power output (i.e., net power = LSL).  These intervals are subject to Settlement in accordance with Section 6.6.9.1, Payment for Emergency Power Increase Directed by ERCOT.  Under these provisions, Resources which ERCOT has determined are necessary for reliability reasons according to paragraph (1) of Section 6.5.9, Emergency Operations, may be compensated in accordance with Section 6.6.9, Emergency Operations Settlement.  ECOT Settlements Staff will receive a flag from the operators that the Resource’s parameters were overridden.  Once the flag is received, if the Real-Time Settlement Point Price  is below the resource’s Energy Offer Curve price, the QSE may be compensated.

ERCOT is proposing to use this methodology to sequence the departure of the QSGRs that have been deployed by SCED and expects to move expeditiously to honor any QSEs’ requests.

- Topaz comment regarding the calculation of the Mitigated Offer Cap

ERCOT opposes the Topaz strike of paragraph (c) of Section 4.4.9.4.1, Mitigated Offer Cap, because the Mitigated Offer Cap adjustment included in this paragraph is intended to allow the QSGR to include a Startup Cost component in its Energy Offer Curve in lieu of a startup Make-Whole Payment.

- Topaz suggestion to utilize the DAM Make-Whole Payment to compensate the Startup Offer as set forth in the Three-Part Supply Offer.

ERCOT opposes this suggestion for the following reasons:

- QSEs that offer QSGRs into SCED are considered a self-commitment and therefore would not be considered for any Make-Whole Payment (RUC or DAM).

- The payments to Resources for Day-Ahead Make-Whole Payment are uplifted to Day-Ahead energy purchasers. Real-Time SCED dispatch is not a Day-Ahead process and therefore these charges should not be borne by those Market Participants.

- Any process outside of the current ERCOT proposal is not possible due to ERCOT system capabilities, until after the Texas Nodal Market Implementation Date (TNMID). The tools that would be necessary to determine startup compensation are currently not available outside of the SCED dispatch process.

- Topaz suggestion to exempt QSGRs from Base Point Deviation Charges in all Settlement Intervals during which the SCED Base Point is below the operational LSL telemetry (referred to as the “physical LSL” by Topaz).

Due to system limitations, ERCOT can implement an interim-solution system change to accomplish this exemption as follows.  If the net power output is equal to zero in the 15 minute Settlement Interval previous to the Settlement Interval in which the QSGR comes On-Line for a QSGR SCED deployment, then no Base Point Deviation Charges will be applied in the first two Settlement Intervals. While this may over-exempt in some circumstances, Settlements does not have the granularity in place to see SCED intervals until a post-TNMID enhancement can be implemented.

ERCOT Response to Comments Filed by Luminant

- Luminant comment regarding exemption of QSGRs from Base Point Deviation Charges.

Refer to ERCOT’s response to the similar Topaz comment Number 5 above.

- Luminant’s suggestion to use an average heat rate in the Mitigated Offer Cap calculation described in Section 4.4.9.4.1.

ERCOT opposes this change because it adds a new requirement to establish the Mitigated Offer Curve utilizing a new heat rate (18 MMBtu/MWh) in lieu of the existing values described in this section.  This new requirement will require a change to the Market Management System (MMS) and settlement systems.  In addition, utilizing the average heat rate instead of the incremental heat rate would, on average, increase the cap.

- Luminant suggests the use of a 60 day provisional qualification for a QSE that has demonstrated their ability on at least one QSGR of each technology.

In the Nodal Market, the Ancillary Service and, in particular, QSGR qualification is unit-specific.  Therefore, the NPRR requires that the QSGR demonstrated its ability to start, synchronize to the ERCOT System and increase its net power output in a ten minute test period to demonstrate its ability to achieve the qualification amount requested by the QSE.  ERCOT has revised the NPRR to indicate that ERCOT may apply the Nodal Ancillary Service provisional qualification requirements described in Section 8.1.1.2.1, Ancillary Service Technical Requirements and Qualification Criteria and Test Methods, for QSGR qualification.

Revised Proposed Protocol Language

## 2.1	DEFINITIONS

Resource

. . .

Generation Resource

. . .

Quick Start Generation Resource (QSGR)

A Generation Resource that is not physically On-Line but is available for SCED to deploy and has passed an ERCOT Quick Start Generation Resource test that establishes an amount of capacity that can be deployed within a ten minute period.

## 2.2	ACRONYMS AND ABBREVIATIONS

QSGR	Quick Start Generation Resource

3.8	Special Considerations for Split Generation Meters, Combined Cycle Generation Resources and Quick Start Generation Resources

. . .

3.8.3	Quick Start Generation Resources

(1)	The QSE for a Quick Start Generation Resource (QSGR) that is available for deployment by SCED shall set the COP Resource Status to ON, and the COP Low Sustained Limit (LSL) and COP HSL values to the expected sustainable LSL and HSL for the QSGR for the hour.  If the QSGR is providing Non-Spinning Reserve (Non-Spin) service, then the Ancillary Service Resource Responsibility for Non-Spin shall be set to Resource’s Non-Spin obligation in the COP.

[NPRR272: Replace paragraph (1) above with the following upon system implementation:] (1) The QSE for a Quick Start Generation Resource (QSGR) that is available for deployment by SCED shall set the COP Resource Status to OFFQS, and the COP Low Sustained Limit (LSL) and COP HSL values to the expected sustainable LSL and HSL for the QSGR for the hour. If the QSGR is providing Non-Spinning Reserve (Non-Spin) service, then the Ancillary Service Resource Responsibility for Non-Spin shall be set to Resource’s Non-Spin obligation in the COP.

(2)	The QSGR shall telemeter a Resource Status of ON and an LSL of zero prior to receiving a deployment instruction from SCED.  This status is necessary in order for SCED to recognize that the Resource can be Dispatched for market purposes.  The status of the breaker shall be open and the output of the Resource shall be zero in order for the State Estimator to correctly assess the state of the system.

(a)	After being deployed for energy from SCED, the Resource shall telemeter an LSL of zero until the Resource has ramped to its physical LSL.  After reaching its physical LSL, the QSGR shall telemeter an LSL that reflects its physical LSL.  The QSGR shall always telemeter an Ancillary Service Resource Responsibility for Non-Spin to reflect the Resource’s Non-Spin obligation and shall always telemeter an Ancillary Service Schedule for Non-Spin of zero to make the capacity available for SCED.  Following a SCED QSGR deployment, the QSGR is expected to follow the SCED Base Points.

(b)	If the QSGR’s SCED Base Point equals the QSGR’s physical LSL telemetry and the QSE desires to disconnect the QSGR from the ERCOT Grid, the QSE must obtain the approval of the ERCOT Operator before it removes the QSGR from the ERCOT System.  After completing the normal shutdown process, the QSE may place the QSGR in a OFF Resource Status or restore the QSGR to the conditions described in this Section 3.8.3 (Resource Status = ON; Output Breaker Status = OPEN; LSL = 0).

[NPRR272: Replace paragraph (2) above with the following upon system implementation:] (2) The QSGR shall telemeter a Resource Status of OFFQS and a LSL of zero prior to receiving a deployment instruction from SCED. This status is necessary in order for SCED to recognize that the Resource can be Dispatched for market purposes. The status of the breaker shall be open and the output of the Resource shall be zero in order for the State Estimator to correctly assess the state of the system. After being deployed for energy from SCED, the Resource shall telemeter an LSL of zero until the Resource has ramped to its physical LSL. After reaching its physical LSL, the QSGR shall telemeter an LSL that reflects its physical LSL. The QSGR shall always telemeter an Ancillary Service Resource Responsibility for Non-Spin to reflect the Resource’s Non-Spin obligation and shall always telemeter an Ancillary Service Schedule for Non-Spin of zero to make the capacity available for SCED.

(3)	A QSGR with a telemeter breaker status of open and a telemeter Resource Status of ON shall not provide Regulation Service or Responsive Reserve (RRS) Service

[NPRR272: Replace paragraph (3) above with the following upon system implementation:] (3) A QSGR with a telemeter breaker status of open and a telemeter Resource Status OFFQS shall not provide Regulation Service or Responsive Reserve (RRS) Service

(4)	ERCOT shall adjust the QSGR’s Mitigated Offer Cap curve as described in Section 4.4.9.4.1, Mitigated Offer Cap.

(5)	For a QSGR, the Resource Entity shall submit a Normal Ramp Rate curve and Emergency Ramp Rate curve indicating QSGR’s ability to reach its ten-minute tested output from zero output in five minutes.  This is necessary to prevent SCED from deploying multiple QSGRs due to ramp limitation in the first five minutes after being Dispatched by SCED.  QSGRs shall be exempt from Base Point Deviation Charges as described in Section 6.6.5.3, Generators Exempt from Deviation Charges.

(6)	Any hour in which the QSE for the QSGR has shown the Resource as available for SCED Dispatch as described in this Section 3.8.3 is a QSE-Committed Interval.

(7)	QSEs representing QSGRs may not recover RUC Clawback Charges when the QSGR has been Dispatched by SCED under conditions described in this Section 3.8.3.

(8)	QSEs must submit and maintain an Energy Offer Curve for their QSGRs for all hours in which the COP Resource Status is submitted as ON.  If a valid Energy Offer Curve or an Output Schedule does not exist for any QSGR for which a Resource Status of ON is telemetered at the end of the Adjustment Period, then ERCOT shall notify the QSE and set the Output Schedule equal to the then-current telemetered output of the Resource until an Output Schedule or Energy Offer Curve is submitted in a subsequent Adjustment Period.  For use as SCED inputs, ERCOT shall create proxy Energy Offer Curves for the Resource as described in paragraph (3) of Section 6.5.7.3, Security Constrained Economic Dispatch.

[NPRR272: Replace paragraph (8) above with the following upon system implementation:] (8) QSEs must submit and maintain an Energy Offer Curve for their QSGRs for all hours in which the COP Resource Status is submitted as OFFQS. If a valid Energy Offer Curve or an Output Schedule does not exist for any QSGR for which a Resource Status of OFFQS is telemetered at the end of the Adjustment Period, then ERCOT shall notify the QSE and set the Output Schedule equal to the then-current telemetered output of the Resource until an Output Schedule or Energy Offer Curve is submitted in a subsequent Adjustment Period. For use as SCED inputs, ERCOT shall create proxy Energy Offer Curves for the Resource as described in paragraph (3) of Section 6.5.7.3, Security Constrained Economic Dispatch.

3.9.1	Current Operating Plan (COP) Criteria

(1)	Each QSE that represents a Resource must submit a COP to ERCOT that reflects expected operating conditions for each Resource for each hour in the next seven Operating Days.

(2)	Each QSE that represents a Resource shall update its COP reflecting changes in availability of any Resource as soon as reasonably practicable, but in no event later than 60 minutes after the event that caused the change.

(3)	The Resource capacity in a QSE’s COP must be sufficient to supply the Ancillary Service Supply Responsibility of that QSE.

(4)	A COP must include the following for each Resource represented by the QSE:

(a)	The name of the Resource;

(b)	The expected Resource Status:

(i)	Select one of the following for Generation Resources synchronized to the ERCOT System that best describes the Resource’s status:

(A)	ONRUC – On-Line and the hour is a RUC-Committed Hour;

(B)	ONREG – On-Line Resource with Energy Offer Curve providing Regulation Service;

(C)	ON – On-Line Resource with Energy Offer Curve or Off-Line Quick Start Generation Resource (QSGR) available for SCED Dispatch;

[NPRR272: Replace paragraph (4)(b)(i)(C) above with the following upon system implementation:] (C) ON – On-Line Resource with Energy Offer Curve;

(D)	ONDSR – On-Line Dynamically Scheduled Resource (DSR);

(E)	ONOS – On-Line Resource with Output Schedule;

(F)	ONOSREG – On-Line Resource with Output Schedule providing Regulation Service;

(G)	ONDSRREG – On-Line DSR providing Regulation Service;

(H)	ONTEST – On-Line test with Output Schedule;

(I)	ONEMR – On-Line EMR (available for commitment or dispatch only for ERCOT-declared Emergency Conditions; the QSE may appropriately set LSL and High Sustained Limit (HSL) to reflect operating limits); and

(J)	ONRR – On-Line as a synchronous condenser (hydro) providing Responsive Reserve (RRS) but unavailable for Dispatch by Security-Constrained Economic Dispatch (SCED) and available for commitment by RUC;

(ii)	Select one of the following for Off-Line Generation Resources not synchronized to the ERCOT System that best describes the Resource’s status:

(A)	OUT – Off-Line and unavailable;

(B)	OFFNS – Off-Line but reserved for Non-Spinning Reserve (Non-Spin);

(C)	OFF – Off-Line but available for commitment in the Day-Ahead Market (DAM) and RUC; and

(D)	EMR – Available for commitment only for ERCOT-declared Emergency Condition events; the QSE may appropriately set LSL and HSL to reflect operating limits; and

[NPRR272: Insert paragraph (4)(b)(ii)(E) upon system implementation:] (E) OFFQS – Off-Line but available for SCED deployment. Only qualified Quick Start Generation Resources (QSGRs) may utilize this status.

(iii)	Select one of the following for Load Resources:

(A)	ONRGL – Available for Dispatch of Regulation Service;

(B)	ONRRCLR – Available for Dispatch of RRS Service as a Controllable Load Resource;

(C)	ONRL – Available for Dispatch of RRS Service or Non-Spin, excluding Controllable Load Resources; and

(D)	OUTL – Not available;

(c)	The HSL;

(d)	The LSL;

(e)	The High Emergency Limit (HEL);

(f)	The Low Emergency Limit (LEL); and

(g)	Ancillary Service Resource Responsibility capacity in MW for:

(i)	Regulation Up (Reg-Up);

(ii)	Regulation Down (Reg-Down);

(iii)	RRS Service; and

(iv)	Non-Spin.

(5)	For Combined Cycle Generation Resources, the above items are required for each operating configuration.  In each hour only one Combined Cycle Generation Resource in a Combined Cycle Train may be assigned one of the On-Line Resource Status codes described above.

(a)	During a RUC study period, if a QSE’s COP reports multiple Combined Cycle Generation Resources in a Combined Cycle Train to be On-Line for any hour, then until the QSE corrects its COP, the On-Line Combined Cycle Generation Resource with the largest HSL is considered to be On-Line and all other Combined Cycle Generation Resources in the Combined Cycle Train are considered to be Off-Line.  Furthermore, until the QSE corrects its COP, the Off-Line Combined Cycle Generation Resources as designated through the application of this process are ineligible for RUC commitment or de-commitment Dispatch Instructions.

(b)	For any hour in which QSE-submitted COP entries are used to determine the initial state of a Combined Cycle Generation Resource for a DAM or Day-Ahead Reliability Unit Commitment (DRUC) study and the COP shows multiple Combined-Cycle Generation Resources in a Combined Cycle Train to be in an On-line Resource Status, then until the QSE corrects its COP, the On-Line Combined Cycle Generation Resource that has been On-Line for the longest time from the last recorded start by ERCOT systems, regardless of the reason for the start, combined with the COP Resource Status for the remaining hours of the current Operating Day, is considered to be On-Line at the start of the DRUC study period and all other COP-designated Combined Cycle Generation Resources in the Combined Cycle Train are considered to be Off-Line.

(c)	ERCOT systems shall allow only one Combined Cycle Generation Resource in a Combined Cycle Train to offer Off-Line Non-Spin in the DAM or Supplemental Ancillary Services Market (SASM).

(i)	If there are multiple Non-Spin offers from different Combined Cycle Generation Resources in a Combined Cycle Train, then prior to execution of the DAM, ERCOT shall select the Non-Spin offer from the Combined Cycle Generation Resource with the highest HSL for consideration in the DAM and ignore the other offers.

(ii)	Combined Cycle Generation Resources offering Off-Line Non-Spin must be able to transition from the shutdown state to the offered Combined Cycle Generation Resource On-Line state and be capable of ramping to the full amount of the Non-Spin offered.

(d)	The DAM and RUC shall honor the registered hot, intermediate or cold Startup Costs for each Combined Cycle Generation Resource registered in a Combined Cycle Train when determining the transition costs for a Combined Cycle Generation Resource.  In the DAM and RUC, the Startup Cost for a Combined Cycle Generation Resource shall be determined by the positive transition cost from the On-Line Combined Cycle Generation Resource within the Combine Cycle Train or from a shutdown condition, whichever ERCOT determines to be appropriate.

(6)	ERCOT may accept COPs only from QSEs.

(7)	A QSE representing a Wind-powered Generation Resource (WGR) must enter an HSL value that is less than or equal to the amount for that Resource from the most recent Short-Term Wind Power Forecast (STWPF) provided by ERCOT.

(8)	A QSE representing a Resource that has a Resource Status of ONTEST must self-commit the Resource and must submit an Output Schedule for the Resource.

(9)	If a QSE has not  submitted a valid COP for any Generation Resource for any hour in the DAM or RUC Study Period, then the Generation Resource is considered to have a Resource Status as OUT thus not available for DAM awards or RUC commitments for those hours.

(10)	If a COP is not available for any Resource for any hour from the current hour to the start  of the DAM period or RUC study, then the Resource Status for those hours are considered equal to the last known Resource Status from a previous hour’s COP or from telemetry as appropriate for that Resource.

4.4.9.4.1	Mitigated Offer Cap

Energy Offer Curves may be subject to mitigation in Real-Time operations under Section 6.5.7.3, Security Constrained Economic Dispatch, using a Mitigated Offer Cap.  The “Mitigated Offer Cap” is:

(a)	For a Generation Resource that commences commercial operation after January 1, 2004, ERCOT shall construct an incremental Mitigated Offer Cap curve (Section 6.5.7.3) such that each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	14.5 MMBtu/MWh times the minimum of FIP or FOP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (d) below.

(b)	For all other Generation Resources, each point on the Mitigated Offer Cap curve (cap vs. output level) is the greater of:

(i)	10.5 MMBtu/MWh times the minimum of FIP or FOP; or

(ii)	The Resource’s verifiable incremental heat rate (MMBtu/MWh) for the output level multiplied by ((Percentage of FIP * FIP) + (Percentage of FOP * FOP))/100, as specified in the Energy Offer Curve, plus verifiable variable O&M cost ($/MWh) times a multiplier described in paragraph (d) below.

(c)	Notwithstanding paragraphs (a)(ii) and (b)(ii) above, the Mitigated Offer Cap verifiable variable O&M cost ($/MWh) for Quick Start Generation Resources (QSGRs) shall incorporate the cost to start the Resource from first fire to breaker close as described in the Verifiable Cost Manual.

(d)	The multipliers for paragraphs (a)(ii) and (b)(ii) above are as follows:

(i)	1.10 for Resources running at a ≥ 50% capacity factor for the previous 12 months;

(ii)	1.15 for Resources running at a ≥ 30 and < 50% capacity factor for the previous 12 months;

(iii)	1.20 for Resources running at a ≥ 20 and < 30% capacity factor for the previous 12 months;

(iv)	1.25 for Resources running at a ≥ 10 and < 20% capacity factor for the previous 12 months;

(v)	1.30 for Resources running at a ≥ 5 and < 10% capacity factor for the previous 12 months;

(vi)	1.40 for Resources running at a ≥ 1 and < 5% capacity factor for the previous 12 months; and

(vii)	1.50 for Resources running at a less than 1% capacity factor for the previous 12 months.

(e)	The previous 12 months’ capacity factor must be updated by ERCOT by the 20th day of each month using the most recent data for use in the next month.  ERCOT shall post to the MIS Secure Area the capacity factor for each Resource before the start of the effective month.

(f)	The process for developing the Mitigated Offer Cap in paragraphs (a), (b) and (c) above must be described by ERCOT in a procedure approved by the appropriate Technical Advisory Committee (TAC) subcommittee, and posted to the MIS Secure Area within one Business Day after initial approval, and after each approved change.

- 6.6.5.3	Generators Exempt from Deviation Charges

Generation Resource Base Point deviation charges do not apply to the following:

(a)	Reliability Must-Run (RMR) Units;

(b)	Dynamically Scheduled Resources (except as described in Section 6.4.2.2, Output Schedules for Dynamically Scheduled Resources);

(c)	Qualifying Facilities (QFs) that do not submit an Energy Offer Curve for the Settlement Interval; or

(d)	Quick Start Generation Resources (QSGRs) during the first 15-minute Settlement Interval that includes the SCED interval in which the QSGR is deployed and comes On-Line, and the immediately following Settlement Interval (a total of two Settlement Intervals).

- 8.1.1.2	General Capacity Testing Requirements

(1)	Within the first 15 days of each Season, each QSE shall provide ERCOT a seasonal HSL for any Generation Resource with a capacity greater than ten MW that will be operated during that Season.  ERCOT shall provide an appropriate form for QSEs to submit their seasonal HSL data.  The seasonal HSL form shall take into account auxiliary Load and gross and net real power capability of the Generation Resource.  Each QSE shall update its COP and telemetry, as necessary, to reflect the HSL of each of its Generation Resources in a given operating interval as well as other operational limitations.  The HSL shown in the COP for a Generation Resource may not be ramp rate-limited while the Real-Time telemetered value of HSL for the Generation Resource may be ramp rate-limited by the QSE representing the Generation Resource in order for the Generation Resource to meet its HSL using the testing process described in paragraph (2) below.

(2)	To verify that the HSL reported by telemetry is achievable, ERCOT may, at its discretion, conduct an unannounced Generation Resource test.  At a time determined solely by ERCOT, ERCOT will issue a Verbal Dispatch Instruction (VDI) to the QSE to operate the designated Generation Resource at its HSL as shown in the QSE’s telemetry at the time the test is initiated.  The QSE shall immediately upon receiving the VDI release all Ancillary Service obligations carried by the unit to be tested and shall telemeter Resource status as “ONTEST.”  The QSE shall not be required to start the designated Generation Resource if it is not already On-Line when ERCOT announces its intent to test the Resource.  If the designated Generation Resource is operating at its Low Sustained Limit (LSL) when ERCOT sends the VDI to begin the test, the QSE shall have up to 60 minutes to allow the Resource to reach 90% of its HSL as shown by telemetry and up to an additional 20 minutes for the Resource to reach the HSL shown by telemetry at the time the test is initiated.  This time requirement does not apply to nuclear-fueled Generation Resources.  If the designated Generation Resource is operating between its LSL and 50% of its HSL shown by telemetry when ERCOT begins the test, the QSE shall have 60 minutes for the Resource to reach its HSL.  If the Resource is operating at or above 50% of its HSL shown by telemetry when ERCOT begins the test, the QSE shall have 30 minutes for the Resource to reach its HSL.  Once the designated Generation Resource reaches its HSL, the QSE shall hold it at that output level for a minimum of 30 minutes.  The HSL for the designated Generation Resource shall be determined based on the Real-Time averaged MW telemetered by the Resource during the 30 minutes of constant output.  After each test, the QSE representing the Generation Resource will complete and submit the test form using the Net Dependable Capability and Reactive Capability (NDCRC) application located on the ERCOT Texas Market Link within two Business Days.

(3)	ERCOT may test multiple Generation Resources within a single QSE within a single 24-hour period.  However, in no case shall ERCOT test more than two Generation Resources within one QSE simultaneously.  All Resources On-Line in a Combined-Cycle Configuration will be measured on an aggregate capacity basis.  All QSEs associated with a jointly owned unit will be tested simultaneously.  Hydro and wind generation will be excluded from unannounced generation capacity testing.  ERCOT shall not perform an unannounced Generation Resource test during a Watch or Energy Emergency Alert (EEA) event.  If an unannounced Generation Resource test is underway when a Watch or EEA event commences, ERCOT may cancel the test.

(4)	Should the designated Generation Resource fail to reach its HSL shown in its telemetry within the time frame set forth herein, the Real-Time averaged MW telemetered during the test shall be the basis for the new HSL for the designated Generation Resource for that Season.  The QSE shall have the opportunity to request another test as quickly as possible (at a time determined by ERCOT) and may retest up to two times per month.  The QSE may also demonstrate an increased value of HSL by operating the Generation Resource at an Output Schedule for at least 30 minutes.  In order to raise an output schedule above the seasonal HSL, the QSE may set the Resource telemetered HSL equal to its output temporarily for the purposes of the demonstration tests.  After either a retest or a demonstration test, the MW capability of the Generation Resource based on the average of the MW production telemetered during the test shall be the basis for the new HSL for the designated Generation Resource for that Season.  Any requested retest must take place within three Business Days after the request for retest.

(5)	The telemetered value of HSL for the Generation Resource shall only be used for testing purposes as described in this Section or for system reliability calculations.  Under no circumstance shall the telemetered HSL be used for Settlement purposes.

(6)	A Resource Entity owning a hydro unit operating in the synchronous condenser fast response mode to provide hydro Responsive Reserve shall evaluate the maximum capability of the Resource each Season.

(7)	ERCOT shall maintain historical records of unannounced Generation Resource test results, using the information contained therein to adjust the Reserve Discount Factor (RDF) subject to the approval of the appropriate TAC subcommittee.  ERCOT shall report to the Reliability and Operations Subcommittee (ROS) annually or as requested by ROS the aggregated results of such unannounced testing (excluding retests), including, but not limited to, the number and total capacity of Resources tested, the percentage of Resources that met or exceeded their HSL reported by telemetry, the percentage that failed to meet their HSL reported by telemetry, and the total MW capacity shortfall of those Resources that failed to meet their HSL reported by telemetry.

(8)	QSEs who receive a VDI to operate the designated Generation Resource for an unannounced Generation Resource test may be considered for additional compensation under Section 6.6.9, Emergency Operations Settlement.  Any unannounced Generation Resource test VDI that ERCOT issues as a result of a QSE-requested retest will not be considered for additional compensation under Section 6.6.9.

(9)	All unannounced Generation Resource test VDIs will be considered as an instructed deviation for compliance purposes.

(10)	Before the start of each Season, a QSE shall provide ERCOT a list identifying each Controllable Load Resource that is expected to operate in a Season as a provider of Ancillary Service.  Prior to the beginning of each Season, QSEs shall identify the Controllable Load Resources to be tested during the Season and the specific week of the test if known.  Any Controllable Load Resource for which the QSE desires qualification to provide Ancillary Services shall have its Net Dependable Capability verified prior to providing Ancillary Services.

(11)	ERCOT shall annually verify the telemetry attributes of each qualified Load Resource.  In addition, once every two years, any Load Resource qualified to provide Responsive Reserve Service using a high-set under-frequency relay shall test the correct operation of the under-frequency relay or the output from the solid-state switch, whichever applies.  However, if a Load Resource’s performance has been verified through response to an actual event, the data from the event can be used to meet the annual telemetry verification requirement for that year and the biennial relay-testing requirement.

(12)	A specific Load Resource to be used for the first time to provide Regulation, Responsive Reserve or Non-Spin must be tested to ERCOT’s reasonable satisfaction using an actual interruption of its Load as part of its qualification to provide Ancillary Service.  The test must take place at a time mutually selected by the QSE representing the Load Resource and ERCOT.  ERCOT shall make available its standard test document for Load Resource qualification required under this Section on the Market Information System (MIS) Public Area.

(13)	Any changes to a Load Resource including changes to its capability to provide Ancillary Service requires updates by the Load Resource to the registration information detailing the change.  For Non-Opt-In Entities (NOIEs) representing specific Load Resources that are located behind the NOIE Settlement Metering points, the NOIE shall provide an alternative unique descriptor of the qualified Load Resource for ERCOT’s records.

(14)	Qualification of a Resource, including a Load Resource, remains valid for that Resource in the event of a change of QSE for the Resource, provided that the new QSE demonstrates to ERCOT’s reasonable satisfaction that the new QSE has adequate communications and control capability for the Resource.

(15)	For purposes of qualifying Quick Start Generation Resources (QSGRs), ERCOT shall issue a unit-specific VDI for the MW amount that the QSE is requesting to qualify its QSGR to provide.  The QSE shall telemeter an ONTEST Resource Status.  The QSGR will only be qualified to provide an amount not to exceed the observed output at the end of a ten minute test period.  ERCOT may grant a provisional qualification for the QSGR in accordance with the provisions described in Section 8.1.1.2.1, Ancillary Service Technical Requirements and Qualification Criteria and Test Methods.