| NPRR Number | 263 | NPRR Title | Resolution of Alignment Item A99 - Settlement Point Price Calculation When Busses are De-energized |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Approved |
| Date of Decision |  | October 19, 2010 |  |  |
| Effective Date |  | Upon the Nodal Protocol Transition Plan’s Texas Nodal Market Implementation Date as prescribed by zonal Protocol Section 21.12, Process for Transition to Nodal Market Protocol Sections. |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 4.5.1, DAM Clearing Process 6.6.1, Real-Time Settlement Point Prices |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) adds the requirement for ERCOT to assign a Locational Marginal Price (LMP) for use in the calculation of Day-Ahead and Real-Time Settlement Point Prices. |  |  |
| Reason for Revision |  | This change conforms the Nodal Protocols to the ERCOT Day-Ahead and Real-Time Security-Constrained Economic Dispatch (SCED) Market Management System (MMS) as-built implementation. |  |  |
| Overall Market Benefit |  | Alignment of the Nodal Protocols with the as-built Nodal System implementation. Adds certainty to market processes by clarifying the ERCOT process for the determination of LMPs in calculating Settlement Point Prices. |  |  |
| Overall Market Impact |  | None expected. |  |  |
| Consumer Impact |  | Adds certainty to the Settlement Point Price calculations. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR263 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 8/11/10, NPRR263, a CEO Revision Request Review, and an Impact Analysis were posted. - On 8/19/10, PRS considered NPRR263. - On 9/9/10, ERCOT comments were posted. - On 9/22/10, PSEG TX comments were posted. - On 9/23/10, PRS again considered NPRR263. - On 9/29/10, a second set of ERCOT comments were posted. - On 10/4/10, PRS again considered NPRR263. - On 10/7/10, TAC considered NPRR263. - On 10/19/10, the ERCOT Board considered NPRR263. |  |  |
| PRS Decision |  | On 8/19/10, PRS unanimously voted to table NPRR263 for one month. The Independent Power Marketer (IPM) Market Segment was not present for the vote. On 9/23/10, PRS unanimously voted to table NPRR263 until the next PRS meeting. All Market Segments were present for the vote. On 10/4/10, PRS voted to recommend approval of NPRR263 as amended by the 9/29/10 ERCOT comments and as revised by PRS and to forward NPRR263 and the Impact Analysis to TAC. There were two abstentions from the Independent Generator Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 8/19/10, participants requested that ERCOT provide additional details regarding how prices will be assigned to de-energized Electrical Busses. On 9/23/10, PRS requested that ERCOT incorporate the process described in the 9/23/10 ERCOT comments into the Nodal Protocols. On 10/4/10, in regards to baseline Nodal Protocol language incorporated into Section 4.5.1 upon the effectiveness of NPRR228, Resolution of Alignment Items A2, A80, A83 and A93 - As-Built Treatment and Settlement of Combined Cycle Generation Resources in ERCOT Market Systems (approved by the ERCOT Board on 8/17/10), an issue was raised regarding the open ended nature of Combined Cycle Generation Resource commitment in the Day-Ahead Market (DAM). ERCOT Staff stated that the purpose of NPRR263 was to align the Nodal Protocols with the as-built Nodal systems and that if revisions are necessary to the language in question, it may be more appropriate as the subject of a separate NPRR. ERCOT Staff agreed to consider whether this language can be clarified and to submit comments to TAC if appropriate. |  |  |
| TAC Decision |  | On 10/7/10, TAC unanimously voted to recommend approval of NPRR263 as recommended by PRS in the 10/4/10 PRS Report and as revised by TAC. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 10/7/10, there was no discussion. |  |  |
| Board Decision |  | On 10/19/10, the ERCOT Board approved NPRR263 as recommended by TAC in the 10/7/10 TAC Report. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | None expected. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Alignment of the Nodal Protocols with the as-built Nodal System implementation. |  |
|  | 2 | Adds certainty to market processes by clarifying the ERCOT process for the determination of LMPs in calculating Settlement Point Prices. |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Matt Mereness |
| E-mail Address | mmereness@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3089 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 090910 | Provided additional detail regarding the MMS design for determining the LMP at de-energized buses in the DAM and SCED. |
| PSEG 092210 | Proposed revisions to remove language inserted by NPRR228 in regards to Combined Cycle Generation Resource commitment in the DAM. |
| ERCOT 092910 | Proposed revisions to incorporate into the Nodal Protocols the description of how LMPs will be assigned to de-energized Electrical Buses for the calculation of Settlement Point Prices that was contained in the 9/9/10 ERCOT comments. |

Comments

Please note that the baseline Nodal Protocol language has been updated to reflect the September 1, 2010 implementation of NPRR228.  Revisions are reflected in redline with “NPRR228” as the author.

Proposed Protocol Language Revision

4.5	DAM Execution and Results

4.5.1	DAM Clearing Process

(1)	At 1000 in the Day-Ahead, ERCOT shall start the Day-Ahead Market (DAM) clearing process.

(2)	Prior to execution of the DAM, ERCOT shall complete a Day-Ahead Simultaneous Feasibility Test.  This test uses the Day-Ahead Updated Network Model topology and evaluates all Congestion Revenue Rights (CRRs) for feasibility to determine hourly oversold quantities.

(3)	The purpose of the DAM is to economically and simultaneously clear offers and bids described in Section 4.4, Inputs into DAM and Other Trades.

(4)	The DAM uses a multi-hour mixed integer programming algorithm to maximize bid-based revenues minus the offer-based costs over the Operating Day, subject to security and other constraints, and ERCOT Ancillary Service procurement requirements.

- The bid-based revenues include revenues from DAM Energy Bids and Point-to-Point (PTP) Obligation Bids.

(b)	The offer-based costs include costs from the Startup Offer, Minimum Energy Offer, and Energy Offer Curve of any Resource that submitted a Three-Part Supply Offer, DAM Energy-Only Offers, CRR Offers, and Ancillary Service Offers.

(c)	Security constraints specified to prevent DAM solutions that would overload the elements of the ERCOT Transmission Grid include the following:

(i)	Transmission constraints – transfer limits on energy flows through the ERCOT Transmission Grid, e.g., thermal or stability limits.  These limits must be satisfied by the intact network and for certain specified contingencies.

These constraints may represent:

(A)	Thermal constraints – protect Transmission Facilities against thermal overload.

(B)	Generic constraints – protect the ERCOT Transmission Grid against transient instability, dynamic stability or voltage collapse.

(C)	Power flow constraints – the energy balance at required Electrical Buses in the ERCOT Transmission Grid must be maintained.

(ii)	Resource constraints – the physical and security limits on Resources that submit Three-Part Supply Offers:

(A)	Resource output constraints – the Low Sustained Limit (LSL) and High Sustained Limit (HSL) of each Resource; and

(B)	Resource operational constraints – includes minimum run time, minimum down time, and configuration constraints.

(iii)	Other constraints –

(A)	Linked offers –the DAM may not select any one part of that Resource capacity to provide more than one Ancillary Service or to provide both energy and an Ancillary Service in the same Operating Hour.  The DAM may, however, select part of that Resource capacity to provide one Ancillary Service and another part of that capacity to provide a different Ancillary Service or energy in the same Operating Hour, provided that a Generation Resource may not offer, and the DAM may not select, linked Energy and Off-Line Non-Spinning Reserve (Non-Spin) Ancillary Service offers in the same Operating Hour.

(B)	The sum of the awarded Ancillary Service capacities for each Resource must be within the Resource limits specified in the Current Operating Plan (COP) and Section 3.18, Resource Limits in Providing Ancillary Service, and the Resource parameters as described in Section 3.7, Resource Parameters.

(C)	Block Ancillary Service Offers for a Load Resource – blocks will not be cleared unless the entire quantity block can be awarded.

(D)	Block CRR Offers and PTP Obligation Bids – blocks will not be cleared unless the entire time block can be awarded.

(E)	Combined Cycle Generation Resources – The DAM may commit a Combined Cycle Generation Resource in a time period that includes the last hour of the Operating Day only if that Combined Cycle Generation Resource can transition to a shutdown condition in the DAM Operating Day.

(d)	Ancillary Service needs for each Ancillary Service include the needs specified in the Ancillary Service Plan that are not part of the Self-Arranged Ancillary Service Quantity and that must be met from available DAM Ancillary Service Offers while co-optimizing with DAM Energy Offers.  ERCOT may not buy more of one Ancillary Service in place of the quantity of a different service.  See Section 4.5.2, Ancillary Service Insufficiency, for what happens if insufficient Ancillary Service Offers are received in the DAM.

[NPRR131: Replace paragraph (4)(d) above with the following upon system implementation:] (d) Ancillary Service needs for each Ancillary Service include the needs specified in the Ancillary Service Plan that are not part of the Self-Arranged Ancillary Service Quantity plus the quantity of Ancillary Service Trades with ERCOT, and that must be met from available DAM Ancillary Service Offers while co-optimizing with DAM Energy Offers. ERCOT may not buy more of one Ancillary Service in place of the quantity of a different service. See Section 4.5.2, Ancillary Service Insufficiency, for what happens if insufficient Ancillary Service Offers are received in the DAM.

(5)	ERCOT shall determine the appropriate Load distributions to allocate offers, bids, and source and sink of CRRs at a Load Zone across the Electrical Buses that are modeled with Load in that Load Zone.  The default distribution is the State Estimator hourly distribution for the seven days before the Operating Day.  If ERCOT decides, in its sole discretion, to change this distribution for reasons such as anticipated weather events or holidays, ERCOT shall select a State Estimator distribution from a proxy day reasonably reflecting the anticipated distribution in the Operating Day.  ERCOT may also modify this distribution to account for predicted differences in network topology between the proxy day and Operating Day.  ERCOT shall develop a methodology, subject to Technical Advisory Committee (TAC) approval, to describe the modification of the proxy day bus-load distribution for this purpose.

(6)	ERCOT shall allocate offers, bids, and source and sink of CRRs at a Hub using the distribution factors specified in the definition of that Hub in Section 3.5.2, Hub Definitions.

(7)	A Resource that has a Three-Part Supply Offer cleared in the DAM may be eligible for Make-Whole Payment of the Startup Offer and Minimum Energy Offer submitted by the Qualified Scheduling Entity (QSE) representing the Resource under Section 4.6, DAM Settlement.

(8)	The directional network element flows for PTP Options declared for Settlement in Real-Time must be properly accounted for in determining available transmission network capacity in the DAM.  In the event the available transmission capability in the DAM cannot accommodate all PTP Options declared for Settlement in Real-Time, any PTP Option declared for Settlement in Real-Time that impacts overloaded directional network elements must be appropriately derated for DAM modeling purposes only, in proportion to that impact.  The derated MW of PTP Options declared for Settlement in Real-Time will be settled in the DAM if their Minimum Reservation Prices are less than or equal to the DAM prices for corresponding PTP Options.  Otherwise, the derated MW will be settled in Real-Time.

(9)	The DAM Settlement is based on hourly MW awards and on Day-Ahead hourly Settlement Point Prices.  All PTP Options settled in the DAM are settled based on the Day-Ahead Settlement Point Prices.  ERCOT shall assign a Locational Marginal Price (LMP) to de-energized Electrical Buses for use in the calculation of the Day-Ahead Settlement Point Prices by using heuristic rules applied in the following order:

(a)	Use average LMP for Electrical Buses within the same station having the same voltage level as the de-energized Electrical Bus, if any exist.

(b)	Use average LMP for all Electrical Buses within the same station, if any exist.

(c)	Use system lambda.

(10)	The Day-Ahead Market Clearing Price for Capacity (MCPC) for each hour for each Ancillary Service is the Shadow Price for that Ancillary Service for the hour as determined by the DAM algorithm.

(11)	If the Day-Ahead MCPC cannot be calculated by ERCOT, the Day-Ahead MCPC for the particular Ancillary Service is equal to the Day-Ahead MCPC for that Ancillary Service in the same Settlement Interval of the preceding Operating Day.

(12)	If the Day-Ahead Settlement Point Prices cannot be calculated by ERCOT, all CRRs shall be settled based on Real-Time Prices.  Settlements for all CRRs shall be reflected on the Real-Time Settlement Statement.

- 6.6.1	Real-Time Settlement Point Prices

Real-Time energy Settlements use Real-Time Settlement Point Prices that are calculated for Resource Nodes, Load Zones, and Hubs.  ERCOT shall assign a Locational Marginal Price (LMP) to de-energized Electrical Buses for use in the calculation of the Real-Time Settlement Point Prices by using heuristic rules applied in the following order:

(a)	Use average LMP for Electrical Buses within the same station having the same voltage level as the de-energized Electrical Bus, if any exist.

(b)	Use average LMP for all Electrical Buses within the same station, if any exist.

(c)	Use system lambda.